3GPP TSG-RAN WG4 NR AH#3 Meeting                          R4-1709327

Nagoya, Japan, 18 - 21 September, 2017

Agenda item:	3.2.3.2

Source: 	ZTE

Title: 	Further discussion on NR spectrum utilization of single numerology case for sub-6GHz

Document for:	Approval

Introduction

In August RAN4#84 meeting R4-1709075 WF on spectrum utilization was approved for sub-6GHz as below [1]:

The RB values in the table above are agreed based on the assumption of symmetric guard band, and they are only meant to indicate the minimum guard band achievable for a given combination of SCS and channel BW. The RAN1 agreement for subcarrier alignment between different SCSs in a channel BW [2][3][4] could result in asymmetric guard band for some combinations of channel BW and a higher order SCS [5], and their actual maximum RB allocation should be 1 RB less than the values in the WF on spectrum utilization above. In this contribution, we calculate the minimum guard band of each combination of SCS and channel BW according to the WF above, and the guard band of each combination according to the RAN1 agreement to find out which combinations’ RB values should be reduced by 1 RB.

Asymmetric guard band considering the RAN1 agreement

- RAN1 sent R1-1706756 LS on subcarrier grid in NR to RAN4 in this April [3]:

Table 1 below summarizes the tentative agreed RB values (numbers in black) in the latest WF on spectrum utilization [1], minimum guard band (numbers in green), and guard band (numbers in red) after subcarrier 0 of the higher SCSs are aligned with the lowest SCS in the channel BW considering the RAN1 agreement. The asymmetric guard bands are highlighted with yellow colour in Table 1, and the corresponding RB values of the combinations of SCS and channel BW should be reduced with 1 RB since the low guard band is smaller than the minimum guard band.

Table 1: Tentative agreed RB values (numbers in black) in the latest WF on spectrum utilization [1], minimum guard band (numbers in green), and guard band (numbers in red) considering the RAN1 agreement

| Channel BW [MHz] |  | 5 | 10 | 15 | 20 | 25 | 40 | 50 | 60 | 80 | 100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SCS=15 KHz | 25 250k | 52 320k | 79 390k | 106 460k | 133 530k | 216 560k | 270 700k | N.A | N.A | N.A |
|  | SCS=30 KHz | 11 520k 430,610 | 24 680k 680,680 | 38 660k 570,750 | 51 820k 820,820 | 65 800k 710,890 | 106 920k 920,920 | 133 1060k 1060,1060 | 162 840k | 217 940k | 273 860k |
|  | SCS=60 KHz | N.A | 11 1040k 1040,1040 | 18 1020k 930,1110 | 24 1360k 1360,1360 | 31 1340k 1250,1430 | 51 1640k 1640,1640 | 65 1600k 1600,1600 | 79 1560k 1560,1560 | 107 1480k 1300,1660 | 135 1400k 1220,1580 |

Observation 1: When the smallest SCS’s RB value of a channel BW is an odd number, the guard band of the higher SCSs become asymmetric and does not meet the minimum guard band requirement.

Table 2 below shows the reduced RB values (highlighted with yellow colour), minimum guard band (numbers in green), and the guard band (number in red) after the RB values are reduced by 1 RB. After the RB values are reduced with 1 RB, the guard bands are bigger than the minimum guard bands.

Table 2: The reduced RB values (highlighted with yellow colour), minimum guard band (numbers in green), and the guard band (numbers in red) after the RB values are reduced by 1 RB

| Channel BW [MHz] |  | 5 | 10 | 15 | 20 | 25 | 40 | 50 | 60 | 80 | 100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SCS=15 KHz | 25 250k | 52 320k | 79 390k | 106 460k | 133 530k | 216 560k | 270 700k | N.A | N.A | N.A |
|  | SCS=30 KHz | 10 520k 610,790 | 24 680k 680,680 | 37 660k 750,930 | 51 820k 820,820 | 64 800k 890,1070 | 106 920k 920,920 | 133 1060k 1060,1060 | 162 840k | 217 940k | 273 860k |
|  | SCS=60 KHz | N.A | 11 1040k 1040,1040 | 17 1020k 1290,1470 | 24 1360k 1360,1360 | 30 1340k 1610,1790 | 51 1640k 1640,1640 | 65 1600k 1600,1600 | 79 1560k 1560,1560 | 106 1480k 1660,2020 | 134 1400k 1580,1940 |

Figure 1 shows the guard band after considering the RAN1 agreement, the black line is for SCS 15kHz, the blue line is for SCS 30kHz and the green line for SCS 60kHz. It could be seen that for channel BW ≥ 25MHz, the guard band is around 600kHz for SCS 15kHz, the guard band is around 950kHz for SCS 30kHz, and the guard band is around 1600kHz for SCS 60kHz.

Figure 1 The guard band considering the RAN1 agreement, the black line for SCS 15kHz,

the blue line for SCS 30kHz, and the green line for SCS 60kHz

Proposal 1: For the NR spectrum utilization of the single numerology case for sub-6GHz, considering the RAN1 agreement in R1-1706756 LS on subcarrier grid in NR, the tentative agreed RB values in R4-1709075 of the following 7 highlighted SCS and channel BW combinations should be reduced to the values shown in the table below in order to meet the minimum guard band requirement.

| Channel BW [MHz] |  | 5 | 10 | 15 | 20 | 25 | 40 | 50 | 60 | 80 | 100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SCS 15 | 25 | 52 | 79 | 106 | 133 | 216 | 270 | N.A | N.A | N.A |
|  | SCS 30 | 10 | 24 | 37 | 51 | 64 | 106 | 133 | 162 | 217 | 273 |
|  | SCS 60 | N.A | 11 | 17 | 24 | 30 | 51 | 65 | 79 | 106 | 134 |

Conclusions

Based on the discussions in this contribution, we have the following Observation and Proposal:

Observation 1: When the smallest SCS’s RB value of a channel BW is an odd number, the guard band of the higher SCSs become asymmetric and does not meet the minimum guard band requirement.

Proposal 1: For the NR spectrum utilization of the single numerology case for sub-6GHz, considering the RAN1 agreement in R1-1706756 LS on subcarrier grid in NR, the tentative agreed RB values in R4-1709075 of the following 7 highlighted SCS and channel BW combinations should be reduced to the values shown in the table below in order to meet the minimum guard band requirement.

| Channel BW [MHz] |  | 5 | 10 | 15 | 20 | 25 | 40 | 50 | 60 | 80 | 100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SCS 15 | 25 | 52 | 79 | 106 | 133 | 216 | 270 | N.A | N.A | N.A |
|  | SCS 30 | 10 | 24 | 37 | 51 | 64 | 106 | 133 | 162 | 217 | 273 |
|  | SCS 60 | N.A | 11 | 17 | 24 | 30 | 51 | 65 | 79 | 106 | 134 |

# References

- R4-1709075 WF on spectrum utilization, August 2017.

- R1-1706173 WF on subcarrier grid in NR, April 2017.

- R1-1706756 LS on subcarrier grid in NR, April 2017.

- R1-1708890 Final Report of 3GPP TSG RAN WG1 #88bis v1.0.0, May 2017.

- R4-1707599 Sub-carrier and resource block alignment in NR, August 2017.