3GPP TSG-RAN WG4 NR AH Meeting #3	R4-1709334

Nagoya, Japan, 18th – 21st September 2017

Agenda Item:	3.5.6.2.1

Source:	MediaTek Inc.

Title:	Link level simulation results for NR cell detection

Document for:		Discussion

- Introduction

In the RAN4#84 meeting the link level simulation assumptions for multi-cell NR cell detection with PSS/SSS were updated [1].

It could be found that there are large simulation results’ gap between each company’s submissions [2]-[4]. Therefore, in this contribution the link level simulation results for single cell NR PSS/SSS detection for sub 6GHz band are provided.

- Simulation assumption

The scenario for no Tx and Rx beamforming and single SS block are evaluated firstly. Only one cell configuration without interference cell is simulated. The detailed simulation parameters can be found in the appendix.

Considering the trade-off of detection performance and power consumption, 4 Rx is not supported. The following simulation is always based on 2 Rx.

Table 1 shows the simulation results of 90th percentile cell detection delay under AWGN, EPA5, ETU30, TDL-A, TDL-B, TDL-C channels with subcarrier spacing 15 kHz and 30 kHz. Table 2 is the one shot detection probability, and since the burst set periodicity is 5ms, one shot corresponds to 5ms.

Table 1: Cell detection delay (in number of required SS block)

| SINR (dB) Channel | SCS=15KHz |  |  | SCS=30KHz |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | -8 | -7 | -6 | -8 | -7 | -6 |
| AWGN | 1 | 1 | 1 | 1 | 1 | 1 |
| EPA5 | 2 | 2 | 2 | 2 | 2 | 2 |
| ETU30 | 4 | 3 | 2 | 6 | 4 | 3 |
| TDL-A(5Hz) | 2 | 2 | 2 | 2 | 2 | 2 |
| TDL-B(5Hz) | 2 | 2 | 2 | 2 | 2 | 2 |
| TDL-C(5Hz) | 2 | 2 | 1 | 2 | 2 | 2 |
| TDL-A(30Hz) | 3 | 2 | 2 | 3 | 2 | 2 |
| TDL-B(30Hz) | 3 | 2 | 2 | 3 | 2 | 2 |
| TDL-C(30Hz) | 3 | 2 | 2 | 3 | 2 | 2 |

Table 2: One shot detection probability

| SINR (dB) Channel | SCS=15KHz |  |  | SCS=30KHz |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | -8 | -7 | -6 | -8 | -7 | -6 |
| AWGN | 99.3% | 99.9% | 100% | 99.6% | 100% | 100% |
| EPA5 | 74.2% | 81.9% | 86.7% | 70.8% | 79.8% | 86.2% |
| ETU30 | 52.3% | 66.8% | 75.5% | 36.1% | 50.8% | 64.6% |
| TDL-A(5Hz) | 75.4% | 81.4% | 86.9% | 69.3% | 81.4% | 86.4% |
| TDL-B(5Hz) | 71.4% | 76.8% | 82.6% | 71.1% | 78.9% | 86.5% |
| TDL-C(5Hz) | 79.5% | 85.5% | 91.4% | 79.2% | 85% | 89.4% |
| TDL-A(30Hz) | 68.4% | 77.3% | 84.4% | 62.4% | 70.8% | 80% |
| TDL-B(30Hz) | 68.8% | 77% | 82.9% | 60.7% | 69.5% | 78.2% |
| TDL-C(30Hz) | 69.5% | 77% | 83.7% | 68.3% | 76.2% | 83% |

From Table 1 and 2, the followings are given:

In ETU channel with large RMS delay spread, the CP will be not enough for larger subcarrier spacing. Subcarrier spacing 30 kHz will have poorer performance than subcarrier spacing 15 kHz in ETU channel, while the performance seems not significantly impacted in other channel profile.

Observation 1: For sub6GHz, the impact of the subcarrier spacing on the NR-PSS/SSS detection time depends on the channel profile’s delay spread.

We tried different Doppler shifts for TDL channel profile. It could be found that different Doppler shift will result in different cell search performance in TDL channel profile. Therefore, we suggest RAN4 to further Align the TDL channel Doppler shift in simulation assumption.

Observation 2: Different Doppler shift will result in different cell search performance in TDL channel profile.

Proposal 1: Align the TDL channel Doppler shift in simulation assumption.

One shot simulation could reflects the performance changing more straightforward than 90th percentile detection delay for different channel profile in different company. Therefore, we think the one shot performance should also be provided during simulation alignment between companies.

Proposal 2: One shot performance should be provided during simulation alignment between companies.

Observation 3: The NR-PSS/SSS detection time seems to be accepted by using 2 receivers.

- Summary

In this paper, we provided initial results for single cell PSS/SS detection in NR sub-6G band, for a number of parameter combinations according to the simulation assumptions agreed in [1]. The results should be taken into account when RAN4 discusses the requirements.

Observation 1: For sub6GHz, the impact of the subcarrier spacing on the NR-PSS/SSS detection time depends on the channel profile’s delay spread.

Observation 2: Different Doppler shift will result in different cell search performance in TDL channel profile.

Proposal 1: Align the TDL channel Doppler shift in simulation assumption.

Proposal 2: One shot performance should be provided during simulation alignment between companies.

Observation 3: The NR-PSS/SSS detection time seems to be accepted by using 2 receivers.

- References

- R4-1708698, “Updated link level simulation assumption of cell detection in NR”, Huawei, HiSilicon, NTT DOCOMO, INC.

- R4-1707093, “Link level simulation results for PSS/SSS detection”, Samsung

- R4-1707356, “Link level simulation results for NR cell detection”, Intel Corporation

- R4-1708223,” Link level simulation results of PSS/SSS detection in NR”, Huawei, HiSilicon

- Annex A: Simulation Assumptions

Table A.1: Cell-specific Parameters for PSS/SSS Detection

| Parameter | Unit | Cell |
| --- | --- | --- |
| RF Channel number | - | Channel 1 |
| Carrier frequency |  | 4GHz |
| Subcarrier spacing | KHz | 15 kHz; 30 kHz for 4GHz; |
| RB Utilization | % | 100 |
| Power boosting for PSS and SSS | dB | 0 |
| Power boosting for Data and Control | dB | 0 |
| Data Modulation | - | QPSK |
| CP Length | - | Normal |
| Number of transmitted SS block within a SS burst set period (K) | - | 1(baseline) |
| SS burst set periodicity | ms | 5 |
| Frequency Offset relative to UE frequency reference | Hz | 0 |
| Eor/Ioc for K=1 | dB | SINR: -6dB, -7dB, -8dB |
| Number of Tx antennas | - | 1 |
| Cell ID |  | 674 |
| PSS Sequence ID | - | 2 |
| SSS Sequence ID | - | 224 |
| Propagation Condition | - | AWGN, EPA5, ETU30 TDL-A/B/C with 100ns for 4 GHz Note1 |
| Noc Model | - | AWGN noise |
| NOTE 1: The channel models of TDL are used for simplified link level evaluations, which are defined in TR 38.901. |  |  |

Table 2: UE-specific Parameters for Cell Detection

| Simulation parameters | Comments/values |
| --- | --- |
| False detect threshold | Required as in a real UE implementation |
| UE having apriority knowledge of system being synchronous or asynchronous | No |
| Performance criterion for comparison | 90th percentile acquisition time for “correct” cell detection of both PSS and SSS sequence IDs. |
| CP length detection | Not simulated |
| Receive antennas | 2 (Without receiver beamforming) |