3GPP TSG-RAN WG4 Meeting AH#03	                                   R4-1709345

Nogoya, Japan, 18-21 Sep, 2017

Agenda Item:   3.2.1

Source:        Samsung

Title:          Definition on NR channel bandwidth

Document for:  Discussion

Introduction

In RAN4#84 the WF on terminology of channel bandwidth was agreed as below:

- Separate terms for “UE channel bandwidth” and “BS channel bandwidth”

- The definition of UE channel bandwidth and BS channel bandwidth may remain as today or be revised. Proposals for definitions should be made in RAN4 AH#3

Following above agreement, the definition of UE channel bandwidth and BS channel bandwidth are discussed separately in this contribution.

Discussion

BS channel bandwidth

Several contribution provide corresponding discussion in previous meeting in [2] [3] [4] with the similar view to reuse the legacy channel bandwidth definition for BS as much as possible to avoid additional confusion. Based on this point, the BS channel bandwidth definition for NR could be as below:

BS Channel bandwidth: RF bandwidth supporting a single NR RF carrier with the transmission bandwidth configured in the uplink or downlink of a cell.

NOTE:	The channel bandwidth is measured in MHz and is used as a reference for transmitter and receiver RF requirements.

Furthermore, as indicated in [1] and [6] the BS Channel bandwidth set may differ with UE Channel bandwidth set, it should be considered to introduce the dedicated table for “BS channel bandwidths per operating band” in BS spec.

UE channel bandwidth

For UE channel bandwidth, the legacy definition is the same as BS side which perfectly matches with LTE condition. Even though in NR phase, new feature such as wideband operation makes the channel bandwidth more complicated, it is not expected to impact the UE channel bandwidth definition with the clarification in the sub-clause of UE channel bandwidths per operating band as that the specified UE channel bandwidths per operating band would be supported according UE capability.

UE Channel bandwidth: The RF bandwidth supporting a single NR RF carrier with the transmission bandwidth configured in the uplink or downlink of a cell. The channel bandwidth is measured in MHz and is used as a reference for transmitter and receiver RF requirements.

Conclusion

In this contribution we further discussed “BS channel bandwidth” and “UE channel bandwidth”, it is proposed to reuse the legacy LTE definition with some clarification as below:

- BS side: new table for BS channel bandwidths per operating band may need.

- UE side: clarification on potential UE capability on UE channel bandwidth

Reference

[1] R4-1708846, WF on terminology for channel bandwidth

[2] R4-1707225, BS channel bandwidth set for NR

[3] R4-1707694, NR BS bandwidth

[4] R4-1708137, BS Channel bandwidth Set

[5] R4-1708845, WF on UE mandatory channel bandwidth

[6] R4-1708847, Way forward on BS channel Bandwidth (CBW) Set