TSG-RAN Working Group 4 (Radio) NR Ad-hoc #3	R4-1709348

Nagoya, Japan, 18 - 21 Sep, 2017

Source:	Samsung

Title:	MPR evaluation results for sub 6GHz

Agenda item:	3.3.3.3.2

Document for:	Discussion

- Background

In RAN4 NR AdHoc2, MPR evaluation assumptions are agreed in [1].

In RAN4 #84, further agreements were reached for part of detailed simulation assumption in [4]. Based updated simulation assumptions, this contribution provide further MPR simulation results for sub 6GHz frequency range for calibration.

- Discussion on simulation model

Our simulation model is described as follows

- Windowing at baseband for spectrum confinement. Hann window is used with overlapping length equal to 2% of symbol length.

- MPR results are average value of 100 random waveform generations.

- Applied frequency offset to higher numerology to meet RB grid alignment. The frequency offset table is updated with final SU.

Table 1. Frequency offset of numerology for RB grid and SC0 alignment

| sub6 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCS [KHz]/ CHBW [MHz] | 5 | 10 | 15 | 20 | 25 | 40 | 50 | 60 | 80 | 100 |
| 15 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 30 | 82.5 | -7.5 | 82.5 | -7.5 | 82.5 | -7.5 | -7.5 | 0 | 0 | 0 |
| 60 | 0 | -22.5 | 67.5 | 157.5 | -112.5 | -22.5 | 157.5 | -15 | 165 | 165 |

- Frequency independent IQ mismatch of 28dBc and DC leakage of 28dBc. No CIM3

- PN model applies a CMOS model in [2], down-scaled to 5GHz carrier frequency.

- PA model uses the polynomial model provided in [3].

Regarding PA calibration point, in our evaluation, proposal 2 as in [4] was used here:

- The PA was calibrated to the conventional LTE reference i.e. 1 dB MPR for 100 RB QPSK DFT-s-OFDM (15KHz SCS) signal

Reusing LTE PA calibration point is helpful for NSA UE to share PA operation mode between NR and LTE, particularly when LTE and NR ULs are in the same band.

Other detailed simulation assumptions were followed the agreed WF in [4].

Regarding RB allocation, edge small RB allocation uses 3RB. For in channel partial allocation, 3RBs are assigned starting from RB6. This applies to all CHBWs and SCSs.

- Simulation results

Our summary results for PC3 RF requirements are provided in Table 2, where max MPR and min MPR for all CHBW and SCS are presented. Detail MPR results for each CHBW and SCS are provided in appendix. Some observations are

- Observation 1: Full-channel Pi/2 BPSK DFT-s-OFDM can only averagely raise output power by about 0.5dB compared to full-channel QPSK, though BPSK’s EVM is 35%. BPSK is limited by ACLR and SEM.

- Observation 2: For simulated cases, MPR difference (max MPR – min MPR) in each group (all SCSs and CHBWs) for full channel is <=1.3dB and for in channel is <= 1dB. Edge channel partial RB sees large MPR variation of 2.5dB. Using the MPR table format in the previous WF cannot be justified for edge channel partial RB transmission.

Table 2. Sub6 MPR summary results

| Waveform | Modulation | (max MPR, min MPR) for all CHBW and SCS |  |  |
| --- | --- | --- | --- | --- |
|  |  | full CH | edge partial RB | In-CH partial RB |
| DFT-s-OFDM | pi/2-BPSK | (0.8, -0.1) | (1.5, -1.0) | (-0.6, -1.6) |
| DFT-s-OFDM | QPSK | (1.4, 0.4) | (2.3, -0.2) | (-0.2, -1.0) |
| DFT-s-OFDM | 16QAM | (2.0, 1.1) | (2.2, 0.2) | (0.4, 0.2) |
| DFT-s-OFDM | 64QAM | (2.1, 1.6) | (2.3, 1.2) | (1.4, 1.2) |
| OFDM | QPSK | (3.2, 2.6) | (2.9, 0.8) | (0.9, 0.7) |
| OFDM | 16QAM | (3.2, 2.6) | (2.9, 1.7) | (1.8, 1.6) |
| OFDM | 64QAM | (3.9, 2.6) | (2.9, 2.7) | (2.9, 2.6) |

- References

[1]	R4-1706980, “WF on MPR evaluation assumption”, NTT DOCOMO, INC.

[2]	R4-1703528, “Phase noise model for above 6 GHz”, Huawei, Hisilicon

[3]	R4-164542, “Response LS on realistic power amplifier model for NR waveform evaluation”, Nokia

[4]	R4-1708912, “NR MPR simulation assumptions for sub-6 GHz operation”, Nokia

- Appendix: Full Sub6 MPR simulation results

| Waveform | Modulation | [RB start, RB Num] | SCS [KHz] | BW [MHz] | Limiting factor | PA out [dBm] | MPR [dB] | MPR difference [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | pi/2-BPSK | [0,25] | 15 | 5 | SEM&Spurious | 26 | 0.6 | 0.9 |
|  |  | [0,50] | 15 | 10 | SEM&Spurious | 26.1 | 0.5 |  |
|  |  | [0,75] | 15 | 15 | SEM&Spurious | 26.6 | 0 |  |
|  |  | [0,100] | 15 | 20 | SEM&Spurious | 26.2 | 0.4 |  |
|  |  | [0,128] | 15 | 25 | ACLR | 26 | 0.6 |  |
|  |  | [0,216] | 15 | 40 | ACLR | 26.7 | -0.1 |  |
|  |  | [0,270] | 15 | 50 | ACLR | 25.8 | 0.8 |  |
|  |  | [0,10] | 30 | 5 | SEM&Spurious | 26 | 0.6 |  |
|  |  | [0,24] | 30 | 10 | ACLR | 26.7 | -0.1 |  |
|  |  | [0,36] | 30 | 15 | SEM&Spurious | 26.7 | -0.1 |  |
|  |  | [0,50] | 30 | 20 | SEM&Spurious | 26.1 | 0.5 |  |
|  |  | [0,64] | 30 | 25 | SEM&Spurious | 26.3 | 0.3 |  |
|  |  | [0,100] | 30 | 40 | ACLR | 26.3 | 0.3 |  |
|  |  | [0,128] | 30 | 50 | ACLR | 26 | 0.6 |  |
|  |  | [0,162] | 30 | 60 | ACLR | 26.7 | -0.1 |  |
|  |  | [0,216] | 30 | 80 | ACLR | 26.7 | -0.1 |  |
|  |  | [0,270] | 30 | 100 | ACLR | 25.8 | 0.8 |  |
|  |  | [0,10] | 60 | 10 | ACLR | 26.2 | 0.4 |  |
|  |  | [0,18] | 60 | 15 | ACLR | 26.7 | -0.1 |  |
|  |  | [0,24] | 60 | 20 | ACLR | 26.7 | -0.1 |  |
|  |  | [0,30] | 60 | 25 | SEM&Spurious | 26.3 | 0.3 |  |
|  |  | [0,50] | 60 | 40 | ACLR | 26.3 | 0.3 |  |
|  |  | [0,64] | 60 | 50 | ACLR | 26.7 | -0.1 |  |
|  |  | [0,75] | 60 | 60 | ACLR | 26.7 | -0.1 |  |
|  |  | [0,100] | 60 | 80 | SEM&Spurious | 26.2 | 0.4 |  |
|  |  | [0,135] | 60 | 100 | ACLR | 26.7 | -0.1 |  |
|  |  | [0,3] | 15 | 5 | SEM&Spurious | 25.6 | 1 | 2.5 |
|  |  |  | 15 | 10 | SEM&Spurious | 26 | 0.6 |  |
|  |  |  | 15 | 15 | SEM&Spurious | 25.7 | 0.9 |  |
|  |  |  | 15 | 20 | SEM&Spurious | 25.2 | 1.4 |  |
|  |  |  | 15 | 25 | SEM&Spurious | 27 | -0.4 |  |
|  |  |  | 15 | 40 | SEM&Spurious | 27 | -0.4 |  |
|  |  |  | 15 | 50 | SEM&Spurious | 27.2 | -0.6 |  |
|  |  |  | 30 | 5 | SEM&Spurious | 27.1 | -0.5 |  |
|  |  |  | 30 | 10 | SEM&Spurious | 26.8 | -0.2 |  |
|  |  |  | 30 | 15 | SEM&Spurious | 26.5 | 0.1 |  |
|  |  |  | 30 | 20 | SEM&Spurious | 26.4 | 0.2 |  |
|  |  |  | 30 | 25 | SEM&Spurious | 25.4 | 1.2 |  |
|  |  |  | 30 | 40 | SEM&Spurious | 25.3 | 1.3 |  |
|  |  |  | 30 | 50 | SEM&Spurious | 27.6 | -1 |  |
|  |  |  | 30 | 60 | SEM&Spurious | 25.3 | 1.3 |  |
|  |  |  | 30 | 80 | SEM&Spurious | 25.1 | 1.5 |  |
|  |  |  | 30 | 100 | SEM&Spurious | 25.5 | 1.1 |  |
|  |  |  | 60 | 10 | SEM&Spurious | 26.4 | 0.2 |  |
|  |  |  | 60 | 15 | SEM&Spurious | 26.4 | 0.2 |  |
|  |  |  | 60 | 20 | SEM&Spurious | 27 | -0.4 |  |
|  |  |  | 60 | 25 | SEM&Spurious | 25.5 | 1.1 |  |
|  |  |  | 60 | 40 | SEM&Spurious | 26.4 | 0.2 |  |
|  |  |  | 60 | 50 | SEM&Spurious | 26.6 | 0 |  |
|  |  |  | 60 | 60 | SEM&Spurious | 26.1 | 0.5 |  |
|  |  |  | 60 | 80 | SEM&Spurious | 26.4 | 0.2 |  |
|  |  |  | 60 | 100 | SEM&Spurious | 26.4 | 0.2 |  |
|  |  | [6,3] | 15 | 5 | ACLR | 28.2 | -1.6 | 1 |
|  |  |  | 15 | 10 | SEM&Spurious | 27.2 | -0.6 |  |
|  |  |  | 15 | 15 | SEM&Spurious | 27.3 | -0.7 |  |
|  |  |  | 15 | 20 | SEM&Spurious | 27.2 | -0.6 |  |
|  |  |  | 15 | 25 | SEM&Spurious | 27.3 | -0.7 |  |
|  |  |  | 15 | 40 | SEM&Spurious | 27.2 | -0.6 |  |
|  |  |  | 15 | 50 | SEM&Spurious | 27.2 | -0.6 |  |
|  |  |  | 30 | 5 | ACLR | 28.2 | -1.6 |  |
|  |  |  | 30 | 10 | ACLR | 28.2 | -1.6 |  |
|  |  |  | 30 | 15 | SEM&Spurious | 27.7 | -1.1 |  |
|  |  |  | 30 | 20 | SEM&Spurious | 27.7 | -1.1 |  |
|  |  |  | 30 | 25 | SEM&Spurious | 27.8 | -1.2 |  |
|  |  |  | 30 | 40 | SEM&Spurious | 27.7 | -1.1 |  |
|  |  |  | 30 | 50 | SEM&Spurious | 27.7 | -1.1 |  |
|  |  |  | 30 | 60 | SEM&Spurious | 27.7 | -1.1 |  |
|  |  |  | 30 | 80 | SEM&Spurious | 27.7 | -1.1 |  |
|  |  |  | 30 | 100 | SEM&Spurious | 27.7 | -1.1 |  |
|  |  |  | 60 | 10 | ACLR | 28.2 | -1.6 |  |
|  |  |  | 60 | 15 | ACLR | 28.1 | -1.5 |  |
|  |  |  | 60 | 20 | ACLR | 28.2 | -1.6 |  |
|  |  |  | 60 | 25 | SEM&Spurious | 27.8 | -1.2 |  |
|  |  |  | 60 | 40 | SEM&Spurious | 28 | -1.4 |  |
|  |  |  | 60 | 50 | SEM&Spurious | 28 | -1.4 |  |
|  |  |  | 60 | 60 | SEM&Spurious | 28 | -1.4 |  |
|  |  |  | 60 | 80 | SEM&Spurious | 28 | -1.4 |  |
|  |  |  | 60 | 100 | SEM&Spurious | 28 | -1.4 |  |
|  | QPSK | [0,25] | 15 | 5 | SEM&Spurious | 25.4 | 1.2 | 1 |
|  |  | [0,50] | 15 | 10 | SEM&Spurious | 25.6 | 1 |  |
|  |  | [0,75] | 15 | 15 | SEM&Spurious | 25.6 | 1 |  |
|  |  | [0,100] | 15 | 20 | ACLR | 25.6 | 1 |  |
|  |  | [0,128] | 15 | 25 | ACLR | 25.3 | 1.3 |  |
|  |  | [0,216] | 15 | 40 | ACLR | 26.2 | 0.4 |  |
|  |  | [0,270] | 15 | 50 | ACLR | 25.2 | 1.4 |  |
|  |  | [0,10] | 30 | 5 | SEM&Spurious | 25.5 | 1.1 |  |
|  |  | [0,24] | 30 | 10 | ACLR | 26.2 | 0.4 |  |
|  |  | [0,36] | 30 | 15 | SEM&Spurious | 25.6 | 1 |  |
|  |  | [0,50] | 30 | 20 | SEM&Spurious | 25.6 | 1 |  |
|  |  | [0,64] | 30 | 25 | SEM&Spurious | 25.8 | 0.8 |  |
|  |  | [0,100] | 30 | 40 | ACLR | 25.6 | 1 |  |
|  |  | [0,128] | 30 | 50 | ACLR | 25.3 | 1.3 |  |
|  |  | [0,162] | 30 | 60 | ACLR | 26.2 | 0.4 |  |
|  |  | [0,216] | 30 | 80 | ACLR | 26.2 | 0.4 |  |
|  |  | [0,270] | 30 | 100 | ACLR | 25.2 | 1.4 |  |
|  |  | [0,10] | 60 | 10 | ACLR | 25.5 | 1.1 |  |
|  |  | [0,18] | 60 | 15 | SEM&Spurious | 25.6 | 1 |  |
|  |  | [0,24] | 60 | 20 | SEM&Spurious | 25.7 | 0.9 |  |
|  |  | [0,30] | 60 | 25 | ACLR | 25.7 | 0.9 |  |
|  |  | [0,50] | 60 | 40 | ACLR | 25.6 | 1 |  |
|  |  | [0,64] | 60 | 50 | ACLR | 25.9 | 0.7 |  |
|  |  | [0,75] | 60 | 60 | ACLR | 25.9 | 0.7 |  |
|  |  | [0,100] | 60 | 80 | ACLR | 25.6 | 1 |  |
|  |  | [0,135] | 60 | 100 | ACLR | 25.9 | 0.7 |  |
|  |  | [0,3] | 15 | 5 | SEM&Spurious | 25.3 | 1.3 | 2.5 |
|  |  |  | 15 | 10 | SEM&Spurious | 25.5 | 1.1 |  |
|  |  |  | 15 | 15 | SEM&Spurious | 24.8 | 1.8 |  |
|  |  |  | 15 | 20 | SEM&Spurious | 24.3 | 2.3 |  |
|  |  |  | 15 | 25 | SEM&Spurious | 25.7 | 0.9 |  |
|  |  |  | 15 | 40 | SEM&Spurious | 25.7 | 0.9 |  |
|  |  |  | 15 | 50 | SEM&Spurious | 26.7 | -0.1 |  |
|  |  |  | 30 | 5 | SEM&Spurious | 26.8 | -0.2 |  |
|  |  |  | 30 | 10 | SEM&Spurious | 26.4 | 0.2 |  |
|  |  |  | 30 | 15 | SEM&Spurious | 26 | 0.6 |  |
|  |  |  | 30 | 20 | SEM&Spurious | 25.6 | 1 |  |
|  |  |  | 30 | 25 | SEM&Spurious | 24.4 | 2.2 |  |
|  |  |  | 30 | 40 | SEM&Spurious | 24.4 | 2.2 |  |
|  |  |  | 30 | 50 | SEM&Spurious | 25.9 | 0.7 |  |
|  |  |  | 30 | 60 | SEM&Spurious | 24.3 | 2.3 |  |
|  |  |  | 30 | 80 | SEM&Spurious | 24.3 | 2.3 |  |
|  |  |  | 30 | 100 | SEM&Spurious | 24.4 | 2.2 |  |
|  |  |  | 60 | 10 | SEM&Spurious | 26.1 | 0.5 |  |
|  |  |  | 60 | 15 | SEM&Spurious | 26.1 | 0.5 |  |
|  |  |  | 60 | 20 | SEM&Spurious | 26.7 | -0.1 |  |
|  |  |  | 60 | 25 | SEM&Spurious | 24.9 | 1.7 |  |
|  |  |  | 60 | 40 | SEM&Spurious | 25.7 | 0.9 |  |
|  |  |  | 60 | 50 | SEM&Spurious | 25.7 | 0.9 |  |
|  |  |  | 60 | 60 | SEM&Spurious | 25.4 | 1.2 |  |
|  |  |  | 60 | 80 | SEM&Spurious | 25.8 | 0.8 |  |
|  |  |  | 60 | 100 | SEM&Spurious | 25.7 | 0.9 |  |
|  |  | [6,3] | 15 | 5 | IBE | 27.6 | -1 | 0.8 |
|  |  |  | 15 | 10 | SEM&Spurious | 27.1 | -0.5 |  |
|  |  |  | 15 | 15 | SEM&Spurious | 26.8 | -0.2 |  |
|  |  |  | 15 | 20 | SEM&Spurious | 26.8 | -0.2 |  |
|  |  |  | 15 | 25 | SEM&Spurious | 26.8 | -0.2 |  |
|  |  |  | 15 | 40 | SEM&Spurious | 26.8 | -0.2 |  |
|  |  |  | 15 | 50 | SEM&Spurious | 26.8 | -0.2 |  |
|  |  |  | 30 | 5 | IBE | 27.6 | -1 |  |
|  |  |  | 30 | 10 | IBE | 27.6 | -1 |  |
|  |  |  | 30 | 15 | IBE | 27.6 | -1 |  |
|  |  |  | 30 | 20 | SEM&Spurious | 27.2 | -0.6 |  |
|  |  |  | 30 | 25 | SEM&Spurious | 27.2 | -0.6 |  |
|  |  |  | 30 | 40 | SEM&Spurious | 27.3 | -0.7 |  |
|  |  |  | 30 | 50 | SEM&Spurious | 27.2 | -0.6 |  |
|  |  |  | 30 | 60 | SEM&Spurious | 27.2 | -0.6 |  |
|  |  |  | 30 | 80 | SEM&Spurious | 27.2 | -0.6 |  |
|  |  |  | 30 | 100 | SEM&Spurious | 27.2 | -0.6 |  |
|  |  |  | 60 | 10 | IBE | 27.6 | -1 |  |
|  |  |  | 60 | 15 | EVM | 27.5 | -0.9 |  |
|  |  |  | 60 | 20 | IBE | 27.5 | -0.9 |  |
|  |  |  | 60 | 25 | IBE | 27.6 | -1 |  |
|  |  |  | 60 | 40 | IBE | 27.6 | -1 |  |
|  |  |  | 60 | 50 | IBE | 27.6 | -1 |  |
|  |  |  | 60 | 60 | IBE | 27.6 | -1 |  |
|  |  |  | 60 | 80 | IBE | 27.6 | -1 |  |
|  |  |  | 60 | 100 | IBE | 27.6 | -1 |  |
|  | 16QAM | [0,25] | 15 | 5 | SEM&Spurious | 25 | 1.6 | 0.9 |
|  |  | [0,50] | 15 | 10 | SEM&Spurious | 24.6 | 2 |  |
|  |  | [0,75] | 15 | 15 | SEM&Spurious | 25.2 | 1.4 |  |
|  |  | [0,100] | 15 | 20 | ACLR | 25.1 | 1.5 |  |
|  |  | [0,128] | 15 | 25 | ACLR | 24.9 | 1.7 |  |
|  |  | [0,216] | 15 | 40 | ACLR | 25.5 | 1.1 |  |
|  |  | [0,270] | 15 | 50 | SEM&Spurious | 24.8 | 1.8 |  |
|  |  | [0,10] | 30 | 5 | SEM&Spurious | 25.2 | 1.4 |  |
|  |  | [0,24] | 30 | 10 | SEM&Spurious | 25.4 | 1.2 |  |
|  |  | [0,36] | 30 | 15 | SEM&Spurious | 25.3 | 1.3 |  |
|  |  | [0,50] | 30 | 20 | ACLR | 25.1 | 1.5 |  |
|  |  | [0,64] | 30 | 25 | SEM&Spurious | 25.2 | 1.4 |  |
|  |  | [0,100] | 30 | 40 | ACLR | 25 | 1.6 |  |
|  |  | [0,128] | 30 | 50 | ACLR | 24.9 | 1.7 |  |
|  |  | [0,162] | 30 | 60 | ACLR | 25.5 | 1.1 |  |
|  |  | [0,216] | 30 | 80 | ACLR | 25.5 | 1.1 |  |
|  |  | [0,270] | 30 | 100 | ACLR | 24.8 | 1.8 |  |
|  |  | [0,10] | 60 | 10 | ACLR | 25.1 | 1.5 |  |
|  |  | [0,18] | 60 | 15 | SEM&Spurious | 25.4 | 1.2 |  |
|  |  | [0,24] | 60 | 20 | SEM&Spurious | 25.4 | 1.2 |  |
|  |  | [0,30] | 60 | 25 | ACLR | 25.1 | 1.5 |  |
|  |  | [0,50] | 60 | 40 | ACLR | 25.1 | 1.5 |  |
|  |  | [0,64] | 60 | 50 | ACLR | 25.3 | 1.3 |  |
|  |  | [0,75] | 60 | 60 | ACLR | 25.3 | 1.3 |  |
|  |  | [0,100] | 60 | 80 | ACLR | 25 | 1.6 |  |
|  |  | [0,135] | 60 | 100 | ACLR | 25.3 | 1.3 |  |
|  |  | [0,3] | 15 | 5 | SEM&Spurious | 25 | 1.6 | 2 |
|  |  |  | 15 | 10 | SEM&Spurious | 25.2 | 1.4 |  |
|  |  |  | 15 | 15 | SEM&Spurious | 24.8 | 1.8 |  |
|  |  |  | 15 | 20 | SEM&Spurious | 24.5 | 2.1 |  |
|  |  |  | 15 | 25 | SEM&Spurious | 24.9 | 1.7 |  |
|  |  |  | 15 | 40 | SEM&Spurious | 24.9 | 1.7 |  |
|  |  |  | 15 | 50 | SEM&Spurious | 26.3 | 0.3 |  |
|  |  |  | 30 | 5 | SEM&Spurious | 26.4 | 0.2 |  |
|  |  |  | 30 | 10 | SEM&Spurious | 26.1 | 0.5 |  |
|  |  |  | 30 | 15 | SEM&Spurious | 25.7 | 0.9 |  |
|  |  |  | 30 | 20 | SEM&Spurious | 25.5 | 1.1 |  |
|  |  |  | 30 | 25 | SEM&Spurious | 24.5 | 2.1 |  |
|  |  |  | 30 | 40 | SEM&Spurious | 24.5 | 2.1 |  |
|  |  |  | 30 | 50 | SEM&Spurious | 26.1 | 0.5 |  |
|  |  |  | 30 | 60 | SEM&Spurious | 24.4 | 2.2 |  |
|  |  |  | 30 | 80 | SEM&Spurious | 24.4 | 2.2 |  |
|  |  |  | 30 | 100 | SEM&Spurious | 24.6 | 2 |  |
|  |  |  | 60 | 10 | SEM&Spurious | 25.8 | 0.8 |  |
|  |  |  | 60 | 15 | SEM&Spurious | 25.6 | 1 |  |
|  |  |  | 60 | 20 | SEM&Spurious | 26.3 | 0.3 |  |
|  |  |  | 60 | 25 | SEM&Spurious | 24.6 | 2 |  |
|  |  |  | 60 | 40 | SEM&Spurious | 25.6 | 1 |  |
|  |  |  | 60 | 50 | SEM&Spurious | 25.8 | 0.8 |  |
|  |  |  | 60 | 60 | SEM&Spurious | 25.2 | 1.4 |  |
|  |  |  | 60 | 80 | SEM&Spurious | 25.6 | 1 |  |
|  |  |  | 60 | 100 | SEM&Spurious | 25.5 | 1.1 |  |
|  |  | [6,3] | 15 | 5 | EVM | 26.4 | 0.2 | 0.2 |
|  |  |  | 15 | 10 | EVM | 26.4 | 0.2 |  |
|  |  |  | 15 | 15 | SEM&Spurious | 26.4 | 0.2 |  |
|  |  |  | 15 | 20 | SEM&Spurious | 26.4 | 0.2 |  |
|  |  |  | 15 | 25 | SEM&Spurious | 26.4 | 0.2 |  |
|  |  |  | 15 | 40 | SEM&Spurious | 26.3 | 0.3 |  |
|  |  |  | 15 | 50 | SEM&Spurious | 26.4 | 0.2 |  |
|  |  |  | 30 | 5 | EVM | 26.4 | 0.2 |  |
|  |  |  | 30 | 10 | EVM | 26.4 | 0.2 |  |
|  |  |  | 30 | 15 | EVM | 26.4 | 0.2 |  |
|  |  |  | 30 | 20 | EVM | 26.4 | 0.2 |  |
|  |  |  | 30 | 25 | EVM | 26.4 | 0.2 |  |
|  |  |  | 30 | 40 | EVM | 26.4 | 0.2 |  |
|  |  |  | 30 | 50 | EVM | 26.4 | 0.2 |  |
|  |  |  | 30 | 60 | EVM | 26.4 | 0.2 |  |
|  |  |  | 30 | 80 | EVM | 26.4 | 0.2 |  |
|  |  |  | 30 | 100 | EVM | 26.4 | 0.2 |  |
|  |  |  | 60 | 10 | EVM | 26.4 | 0.2 |  |
|  |  |  | 60 | 15 | EVM | 26.2 | 0.4 |  |
|  |  |  | 60 | 20 | EVM | 26.4 | 0.2 |  |
|  |  |  | 60 | 25 | EVM | 26.4 | 0.2 |  |
|  |  |  | 60 | 40 | EVM | 26.4 | 0.2 |  |
|  |  |  | 60 | 50 | EVM | 26.4 | 0.2 |  |
|  |  |  | 60 | 60 | EVM | 26.4 | 0.2 |  |
|  |  |  | 60 | 80 | EVM | 26.4 | 0.2 |  |
|  |  |  | 60 | 100 | EVM | 26.4 | 0.2 |  |
|  | 64QAM | [0,25] | 15 | 5 | EVM | 24.9 | 1.7 | 0.5 |
|  |  | [0,50] | 15 | 10 | SEM&Spurious | 24.5 | 2.1 |  |
|  |  | [0,75] | 15 | 15 | EVM | 24.9 | 1.7 |  |
|  |  | [0,100] | 15 | 20 | EVM | 25 | 1.6 |  |
|  |  | [0,128] | 15 | 25 | EVM | 24.7 | 1.9 |  |
|  |  | [0,216] | 15 | 40 | EVM | 24.8 | 1.8 |  |
|  |  | [0,270] | 15 | 50 | EVM | 24.7 | 1.9 |  |
|  |  | [0,10] | 30 | 5 | EVM | 24.6 | 2 |  |
|  |  | [0,24] | 30 | 10 | EVM | 24.8 | 1.8 |  |
|  |  | [0,36] | 30 | 15 | EVM | 24.9 | 1.7 |  |
|  |  | [0,50] | 30 | 20 | EVM | 24.9 | 1.7 |  |
|  |  | [0,64] | 30 | 25 | EVM | 24.9 | 1.7 |  |
|  |  | [0,100] | 30 | 40 | ACLR | 24.9 | 1.7 |  |
|  |  | [0,128] | 30 | 50 | EVM | 24.7 | 1.9 |  |
|  |  | [0,162] | 30 | 60 | EVM | 24.8 | 1.8 |  |
|  |  | [0,216] | 30 | 80 | EVM | 24.8 | 1.8 |  |
|  |  | [0,270] | 30 | 100 | ACLR | 24.7 | 1.9 |  |
|  |  | [0,10] | 60 | 10 | EVM | 24.6 | 2 |  |
|  |  | [0,18] | 60 | 15 | EVM | 24.8 | 1.8 |  |
|  |  | [0,24] | 60 | 20 | EVM | 24.8 | 1.8 |  |
|  |  | [0,30] | 60 | 25 | EVM | 25 | 1.6 |  |
|  |  | [0,50] | 60 | 40 | EVM | 24.9 | 1.7 |  |
|  |  | [0,64] | 60 | 50 | EVM | 24.8 | 1.8 |  |
|  |  | [0,75] | 60 | 60 | EVM | 25 | 1.6 |  |
|  |  | [0,100] | 60 | 80 | ACLR | 24.9 | 1.7 |  |
|  |  | [0,135] | 60 | 100 | EVM | 24.9 | 1.7 |  |
|  |  | [0,3] | 15 | 5 | SEM&Spurious | 24.9 | 1.7 | 1.1 |
|  |  |  | 15 | 10 | SEM&Spurious | 25.1 | 1.5 |  |
|  |  |  | 15 | 15 | SEM&Spurious | 24.7 | 1.9 |  |
|  |  |  | 15 | 20 | SEM&Spurious | 24.4 | 2.2 |  |
|  |  |  | 15 | 25 | SEM&Spurious | 24.8 | 1.8 |  |
|  |  |  | 15 | 40 | SEM&Spurious | 24.8 | 1.8 |  |
|  |  |  | 15 | 50 | EVM | 25.4 | 1.2 |  |
|  |  |  | 30 | 5 | EVM | 25.4 | 1.2 |  |
|  |  |  | 30 | 10 | EVM | 25.4 | 1.2 |  |
|  |  |  | 30 | 15 | EVM | 25.4 | 1.2 |  |
|  |  |  | 30 | 20 | EVM | 25.4 | 1.2 |  |
|  |  |  | 30 | 25 | SEM&Spurious | 24.5 | 2.1 |  |
|  |  |  | 30 | 40 | SEM&Spurious | 24.5 | 2.1 |  |
|  |  |  | 30 | 50 | EVM | 25.4 | 1.2 |  |
|  |  |  | 30 | 60 | SEM&Spurious | 24.4 | 2.2 |  |
|  |  |  | 30 | 80 | SEM&Spurious | 24.3 | 2.3 |  |
|  |  |  | 30 | 100 | SEM&Spurious | 24.6 | 2 |  |
|  |  |  | 60 | 10 | EVM | 25.4 | 1.2 |  |
|  |  |  | 60 | 15 | EVM | 25.4 | 1.2 |  |
|  |  |  | 60 | 20 | EVM | 25.4 | 1.2 |  |
|  |  |  | 60 | 25 | SEM&Spurious | 24.5 | 2.1 |  |
|  |  |  | 60 | 40 | EVM | 25.4 | 1.2 |  |
|  |  |  | 60 | 50 | EVM | 25.4 | 1.2 |  |
|  |  |  | 60 | 60 | SEM&Spurious | 25.2 | 1.4 |  |
|  |  |  | 60 | 80 | EVM | 25.4 | 1.2 |  |
|  |  |  | 60 | 100 | EVM | 25.4 | 1.2 |  |
|  |  | [6,3] | 15 | 5 | EVM | 25.4 | 1.2 | 0.2 |
|  |  |  | 15 | 10 | EVM | 25.4 | 1.2 |  |
|  |  |  | 15 | 15 | EVM | 25.4 | 1.2 |  |
|  |  |  | 15 | 20 | EVM | 25.4 | 1.2 |  |
|  |  |  | 15 | 25 | EVM | 25.4 | 1.2 |  |
|  |  |  | 15 | 40 | EVM | 25.4 | 1.2 |  |
|  |  |  | 15 | 50 | EVM | 25.4 | 1.2 |  |
|  |  |  | 30 | 5 | EVM | 25.4 | 1.2 |  |
|  |  |  | 30 | 10 | EVM | 25.4 | 1.2 |  |
|  |  |  | 30 | 15 | EVM | 25.4 | 1.2 |  |
|  |  |  | 30 | 20 | IBE | 25.4 | 1.2 |  |
|  |  |  | 30 | 25 | EVM | 25.4 | 1.2 |  |
|  |  |  | 30 | 40 | EVM | 25.4 | 1.2 |  |
|  |  |  | 30 | 50 | EVM | 25.4 | 1.2 |  |
|  |  |  | 30 | 60 | EVM | 25.4 | 1.2 |  |
|  |  |  | 30 | 80 | EVM | 25.4 | 1.2 |  |
|  |  |  | 30 | 100 | EVM | 25.4 | 1.2 |  |
|  |  |  | 60 | 10 | EVM | 25.4 | 1.2 |  |
|  |  |  | 60 | 15 | EVM | 25.2 | 1.4 |  |
|  |  |  | 60 | 20 | EVM | 25.4 | 1.2 |  |
|  |  |  | 60 | 25 | IBE | 25.4 | 1.2 |  |
|  |  |  | 60 | 40 | EVM | 25.4 | 1.2 |  |
|  |  |  | 60 | 50 | EVM | 25.4 | 1.2 |  |
|  |  |  | 60 | 60 | EVM | 25.4 | 1.2 |  |
|  |  |  | 60 | 80 | EVM | 25.4 | 1.2 |  |
|  |  |  | 60 | 100 | EVM | 25.4 | 1.2 |  |
| OFDM | QPSK | [0,25] | 15 | 5 | ACLR | 23.8 | 2.8 | 0.6 |
|  |  | [0,52] | 15 | 10 | ACLR | 23.7 | 2.9 |  |
|  |  | [0,79] | 15 | 15 | ACLR | 23.7 | 2.9 |  |
|  |  | [0,106] | 15 | 20 | ACLR | 23.6 | 3 |  |
|  |  | [0,133] | 15 | 25 | ACLR | 23.6 | 3 |  |
|  |  | [0,216] | 15 | 40 | ACLR | 23.8 | 2.8 |  |
|  |  | [0,270] | 15 | 50 | ACLR | 23.4 | 3.2 |  |
|  |  | [0,11] | 30 | 5 | ACLR | 23.9 | 2.7 |  |
|  |  | [0,24] | 30 | 10 | ACLR | 24 | 2.6 |  |
|  |  | [0,38] | 30 | 15 | ACLR | 23.7 | 2.9 |  |
|  |  | [0,51] | 30 | 20 | ACLR | 23.9 | 2.7 |  |
|  |  | [0,65] | 30 | 25 | ACLR | 23.6 | 3 |  |
|  |  | [0,106] | 30 | 40 | ACLR | 23.6 | 3 |  |
|  |  | [0,133] | 30 | 50 | ACLR | 23.5 | 3.1 |  |
|  |  | [0,162] | 30 | 60 | ACLR | 23.8 | 2.8 |  |
|  |  | [0,217] | 30 | 80 | ACLR | 23.4 | 3.2 |  |
|  |  | [0,273] | 30 | 100 | ACLR | 23.4 | 3.2 |  |
|  |  | [0,11] | 60 | 10 | ACLR | 24 | 2.6 |  |
|  |  | [0,18] | 60 | 15 | ACLR | 23.9 | 2.7 |  |
|  |  | [0,24] | 60 | 20 | ACLR | 23.9 | 2.7 |  |
|  |  | [0,31] | 60 | 25 | ACLR | 23.8 | 2.8 |  |
|  |  | [0,51] | 60 | 40 | ACLR | 23.8 | 2.8 |  |
|  |  | [0,65] | 60 | 50 | ACLR | 23.5 | 3.1 |  |
|  |  | [0,79] | 60 | 60 | ACLR | 23.7 | 2.9 |  |
|  |  | [0,107] | 60 | 80 | ACLR | 23.7 | 2.9 |  |
|  |  | [0,135] | 60 | 100 | ACLR | 23.6 | 3 |  |
|  |  | [0,3] | 15 | 5 | SEM&Spurious | 24.3 | 2.3 | 2.1 |
|  |  |  | 15 | 10 | SEM&Spurious | 24.3 | 2.3 |  |
|  |  |  | 15 | 15 | SEM&Spurious | 24.2 | 2.4 |  |
|  |  |  | 15 | 20 | SEM&Spurious | 24.1 | 2.5 |  |
|  |  |  | 15 | 25 | SEM&Spurious | 24.4 | 2.2 |  |
|  |  |  | 15 | 40 | SEM&Spurious | 24.4 | 2.2 |  |
|  |  |  | 15 | 50 | EVM | 25.8 | 0.8 |  |
|  |  |  | 30 | 5 | SEM&Spurious | 25.7 | 0.9 |  |
|  |  |  | 30 | 10 | SEM&Spurious | 25.4 | 1.2 |  |
|  |  |  | 30 | 15 | SEM&Spurious | 25 | 1.6 |  |
|  |  |  | 30 | 20 | SEM&Spurious | 24.9 | 1.7 |  |
|  |  |  | 30 | 25 | SEM&Spurious | 24.1 | 2.5 |  |
|  |  |  | 30 | 40 | SEM&Spurious | 24.1 | 2.5 |  |
|  |  |  | 30 | 50 | SEM&Spurious | 25.5 | 1.1 |  |
|  |  |  | 30 | 60 | SEM&Spurious | 23.9 | 2.7 |  |
|  |  |  | 30 | 80 | SEM&Spurious | 24 | 2.6 |  |
|  |  |  | 30 | 100 | SEM&Spurious | 24.1 | 2.5 |  |
|  |  |  | 60 | 10 | SEM&Spurious | 25.1 | 1.5 |  |
|  |  |  | 60 | 15 | SEM&Spurious | 24.9 | 1.7 |  |
|  |  |  | 60 | 20 | SEM&Spurious | 25.6 | 1 |  |
|  |  |  | 60 | 25 | SEM&Spurious | 23.7 | 2.9 |  |
|  |  |  | 60 | 40 | SEM&Spurious | 24.9 | 1.7 |  |
|  |  |  | 60 | 50 | SEM&Spurious | 25.3 | 1.3 |  |
|  |  |  | 60 | 60 | SEM&Spurious | 24.6 | 2 |  |
|  |  |  | 60 | 80 | SEM&Spurious | 25 | 1.6 |  |
|  |  |  | 60 | 100 | SEM&Spurious | 24.8 | 1.8 |  |
|  |  | [6,3] | 15 | 5 | EVM | 25.8 | 0.8 | 0.2 |
|  |  |  | 15 | 10 | EVM | 25.8 | 0.8 |  |
|  |  |  | 15 | 15 | EVM | 25.8 | 0.8 |  |
|  |  |  | 15 | 20 | EVM | 25.8 | 0.8 |  |
|  |  |  | 15 | 25 | EVM | 25.8 | 0.8 |  |
|  |  |  | 15 | 40 | EVM | 25.8 | 0.8 |  |
|  |  |  | 15 | 50 | EVM | 25.8 | 0.8 |  |
|  |  |  | 30 | 5 | EVM | 25.8 | 0.8 |  |
|  |  |  | 30 | 10 | EVM | 25.8 | 0.8 |  |
|  |  |  | 30 | 15 | EVM | 25.8 | 0.8 |  |
|  |  |  | 30 | 20 | EVM | 25.8 | 0.8 |  |
|  |  |  | 30 | 25 | EVM | 25.8 | 0.8 |  |
|  |  |  | 30 | 40 | EVM | 25.8 | 0.8 |  |
|  |  |  | 30 | 50 | EVM | 25.8 | 0.8 |  |
|  |  |  | 30 | 60 | EVM | 25.8 | 0.8 |  |
|  |  |  | 30 | 80 | EVM | 25.8 | 0.8 |  |
|  |  |  | 30 | 100 | EVM | 25.8 | 0.8 |  |
|  |  |  | 60 | 10 | EVM | 25.8 | 0.8 |  |
|  |  |  | 60 | 15 | EVM | 25.7 | 0.9 |  |
|  |  |  | 60 | 20 | EVM | 25.8 | 0.8 |  |
|  |  |  | 60 | 25 | EVM | 25.9 | 0.7 |  |
|  |  |  | 60 | 40 | EVM | 25.8 | 0.8 |  |
|  |  |  | 60 | 50 | EVM | 25.8 | 0.8 |  |
|  |  |  | 60 | 60 | EVM | 25.8 | 0.8 |  |
|  |  |  | 60 | 80 | EVM | 25.8 | 0.8 |  |
|  |  |  | 60 | 100 | EVM | 25.8 | 0.8 |  |
|  | 16QAM | [0,25] | 15 | 5 | ACLR | 23.8 | 2.8 | 0.6 |
|  |  | [0,52] | 15 | 10 | ACLR | 23.7 | 2.9 |  |
|  |  | [0,79] | 15 | 15 | ACLR | 23.7 | 2.9 |  |
|  |  | [0,106] | 15 | 20 | ACLR | 23.6 | 3 |  |
|  |  | [0,133] | 15 | 25 | ACLR | 23.6 | 3 |  |
|  |  | [0,216] | 15 | 40 | ACLR | 23.8 | 2.8 |  |
|  |  | [0,270] | 15 | 50 | ACLR | 23.5 | 3.1 |  |
|  |  | [0,11] | 30 | 5 | ACLR | 23.9 | 2.7 |  |
|  |  | [0,24] | 30 | 10 | ACLR | 24 | 2.6 |  |
|  |  | [0,38] | 30 | 15 | ACLR | 23.6 | 3 |  |
|  |  | [0,51] | 30 | 20 | ACLR | 23.8 | 2.8 |  |
|  |  | [0,65] | 30 | 25 | ACLR | 23.6 | 3 |  |
|  |  | [0,106] | 30 | 40 | ACLR | 23.6 | 3 |  |
|  |  | [0,133] | 30 | 50 | ACLR | 23.5 | 3.1 |  |
|  |  | [0,162] | 30 | 60 | ACLR | 23.8 | 2.8 |  |
|  |  | [0,217] | 30 | 80 | ACLR | 23.4 | 3.2 |  |
|  |  | [0,273] | 30 | 100 | ACLR | 23.4 | 3.2 |  |
|  |  | [0,11] | 60 | 10 | ACLR | 24 | 2.6 |  |
|  |  | [0,18] | 60 | 15 | ACLR | 23.9 | 2.7 |  |
|  |  | [0,24] | 60 | 20 | ACLR | 23.9 | 2.7 |  |
|  |  | [0,31] | 60 | 25 | ACLR | 23.8 | 2.8 |  |
|  |  | [0,51] | 60 | 40 | ACLR | 23.8 | 2.8 |  |
|  |  | [0,65] | 60 | 50 | ACLR | 23.5 | 3.1 |  |
|  |  | [0,79] | 60 | 60 | ACLR | 23.7 | 2.9 |  |
|  |  | [0,107] | 60 | 80 | ACLR | 23.7 | 2.9 |  |
|  |  | [0,135] | 60 | 100 | ACLR | 23.6 | 3 |  |
|  |  | [0,3] | 15 | 5 | SEM&Spurious | 24.3 | 2.3 | 1.2 |
|  |  |  | 15 | 10 | SEM&Spurious | 24.3 | 2.3 |  |
|  |  |  | 15 | 15 | SEM&Spurious | 24.2 | 2.4 |  |
|  |  |  | 15 | 20 | SEM&Spurious | 24.1 | 2.5 |  |
|  |  |  | 15 | 25 | SEM&Spurious | 24.3 | 2.3 |  |
|  |  |  | 15 | 40 | SEM&Spurious | 24.4 | 2.2 |  |
|  |  |  | 15 | 50 | EVM | 24.9 | 1.7 |  |
|  |  |  | 30 | 5 | EVM | 24.9 | 1.7 |  |
|  |  |  | 30 | 10 | EVM | 24.9 | 1.7 |  |
|  |  |  | 30 | 15 | EVM | 24.9 | 1.7 |  |
|  |  |  | 30 | 20 | EVM | 24.9 | 1.7 |  |
|  |  |  | 30 | 25 | SEM&Spurious | 24.1 | 2.5 |  |
|  |  |  | 30 | 40 | SEM&Spurious | 24.1 | 2.5 |  |
|  |  |  | 30 | 50 | EVM | 24.9 | 1.7 |  |
|  |  |  | 30 | 60 | SEM&Spurious | 23.9 | 2.7 |  |
|  |  |  | 30 | 80 | SEM&Spurious | 24 | 2.6 |  |
|  |  |  | 30 | 100 | SEM&Spurious | 24.1 | 2.5 |  |
|  |  |  | 60 | 10 | EVM | 24.9 | 1.7 |  |
|  |  |  | 60 | 15 | SEM&Spurious | 24.8 | 1.8 |  |
|  |  |  | 60 | 20 | EVM | 24.9 | 1.7 |  |
|  |  |  | 60 | 25 | SEM&Spurious | 23.7 | 2.9 |  |
|  |  |  | 60 | 40 | EVM | 24.9 | 1.7 |  |
|  |  |  | 60 | 50 | EVM | 24.9 | 1.7 |  |
|  |  |  | 60 | 60 | SEM&Spurious | 24.6 | 2 |  |
|  |  |  | 60 | 80 | EVM | 24.9 | 1.7 |  |
|  |  |  | 60 | 100 | SEM&Spurious | 24.8 | 1.8 |  |
|  |  | [6,3] | 15 | 5 | EVM | 25 | 1.6 | 0.2 |
|  |  |  | 15 | 10 | EVM | 25 | 1.6 |  |
|  |  |  | 15 | 15 | EVM | 24.9 | 1.7 |  |
|  |  |  | 15 | 20 | EVM | 24.9 | 1.7 |  |
|  |  |  | 15 | 25 | EVM | 24.9 | 1.7 |  |
|  |  |  | 15 | 40 | EVM | 24.9 | 1.7 |  |
|  |  |  | 15 | 50 | EVM | 24.9 | 1.7 |  |
|  |  |  | 30 | 5 | EVM | 24.9 | 1.7 |  |
|  |  |  | 30 | 10 | EVM | 24.9 | 1.7 |  |
|  |  |  | 30 | 15 | EVM | 25 | 1.6 |  |
|  |  |  | 30 | 20 | EVM | 25 | 1.6 |  |
|  |  |  | 30 | 25 | EVM | 24.9 | 1.7 |  |
|  |  |  | 30 | 40 | EVM | 24.9 | 1.7 |  |
|  |  |  | 30 | 50 | EVM | 25 | 1.6 |  |
|  |  |  | 30 | 60 | EVM | 24.9 | 1.7 |  |
|  |  |  | 30 | 80 | EVM | 24.9 | 1.7 |  |
|  |  |  | 30 | 100 | EVM | 24.9 | 1.7 |  |
|  |  |  | 60 | 10 | EVM | 24.9 | 1.7 |  |
|  |  |  | 60 | 15 | EVM | 24.8 | 1.8 |  |
|  |  |  | 60 | 20 | EVM | 24.9 | 1.7 |  |
|  |  |  | 60 | 25 | EVM | 25 | 1.6 |  |
|  |  |  | 60 | 40 | EVM | 25 | 1.6 |  |
|  |  |  | 60 | 50 | EVM | 25 | 1.6 |  |
|  |  |  | 60 | 60 | EVM | 25 | 1.6 |  |
|  |  |  | 60 | 80 | EVM | 25 | 1.6 |  |
|  |  |  | 60 | 100 | EVM | 24.9 | 1.7 |  |
|  | 64QAM | [0,25] | 15 | 5 | EVM | 22.8 | 3.8 | 1.3 |
|  |  | [0,52] | 15 | 10 | EVM | 22.8 | 3.8 |  |
|  |  | [0,79] | 15 | 15 | EVM | 22.8 | 3.8 |  |
|  |  | [0,106] | 15 | 20 | EVM | 22.8 | 3.8 |  |
|  |  | [0,133] | 15 | 25 | EVM | 22.8 | 3.8 |  |
|  |  | [0,216] | 15 | 40 | EVM | 22.8 | 3.8 |  |
|  |  | [0,270] | 15 | 50 | EVM | 22.9 | 3.7 |  |
|  |  | [0,11] | 30 | 5 | EVM | 22.7 | 3.9 |  |
|  |  | [0,24] | 30 | 10 | EVM | 22.8 | 3.8 |  |
|  |  | [0,38] | 30 | 15 | EVM | 22.8 | 3.8 |  |
|  |  | [0,51] | 30 | 20 | EVM | 22.7 | 3.9 |  |
|  |  | [0,65] | 30 | 25 | EVM | 22.9 | 3.7 |  |
|  |  | [0,106] | 30 | 40 | EVM | 22.8 | 3.8 |  |
|  |  | [0,133] | 30 | 50 | EVM | 22.8 | 3.8 |  |
|  |  | [0,162] | 30 | 60 | EVM | 22.8 | 3.8 |  |
|  |  | [0,217] | 30 | 80 | EVM | 22.9 | 3.7 |  |
|  |  | [0,273] | 30 | 100 | EVM | 22.8 | 3.8 |  |
|  |  | [0,11] | 60 | 10 | ACLR | 24 | 2.6 |  |
|  |  | [0,18] | 60 | 15 | EVM | 22.7 | 3.9 |  |
|  |  | [0,24] | 60 | 20 | EVM | 22.7 | 3.9 |  |
|  |  | [0,31] | 60 | 25 | EVM | 22.8 | 3.8 |  |
|  |  | [0,51] | 60 | 40 | EVM | 22.8 | 3.8 |  |
|  |  | [0,65] | 60 | 50 | EVM | 22.9 | 3.7 |  |
|  |  | [0,79] | 60 | 60 | EVM | 22.8 | 3.8 |  |
|  |  | [0,107] | 60 | 80 | EVM | 22.8 | 3.8 |  |
|  |  | [0,135] | 60 | 100 | EVM | 22.8 | 3.8 |  |
|  |  | [0,3] | 15 | 5 | EVM | 23.9 | 2.7 | 0.2 |
|  |  |  | 15 | 10 | EVM | 23.9 | 2.7 |  |
|  |  |  | 15 | 15 | EVM | 23.9 | 2.7 |  |
|  |  |  | 15 | 20 | EVM | 23.9 | 2.7 |  |
|  |  |  | 15 | 25 | EVM | 23.9 | 2.7 |  |
|  |  |  | 15 | 40 | EVM | 23.9 | 2.7 |  |
|  |  |  | 15 | 50 | EVM | 23.9 | 2.7 |  |
|  |  |  | 30 | 5 | EVM | 23.9 | 2.7 |  |
|  |  |  | 30 | 10 | EVM | 23.9 | 2.7 |  |
|  |  |  | 30 | 15 | EVM | 23.9 | 2.7 |  |
|  |  |  | 30 | 20 | EVM | 23.9 | 2.7 |  |
|  |  |  | 30 | 25 | EVM | 23.9 | 2.7 |  |
|  |  |  | 30 | 40 | EVM | 23.9 | 2.7 |  |
|  |  |  | 30 | 50 | EVM | 23.9 | 2.7 |  |
|  |  |  | 30 | 60 | EVM | 23.9 | 2.7 |  |
|  |  |  | 30 | 80 | EVM | 23.9 | 2.7 |  |
|  |  |  | 30 | 100 | EVM | 23.9 | 2.7 |  |
|  |  |  | 60 | 10 | EVM | 23.9 | 2.7 |  |
|  |  |  | 60 | 15 | EVM | 23.9 | 2.7 |  |
|  |  |  | 60 | 20 | EVM | 23.9 | 2.7 |  |
|  |  |  | 60 | 25 | SEM&Spurious | 23.7 | 2.9 |  |
|  |  |  | 60 | 40 | EVM | 23.9 | 2.7 |  |
|  |  |  | 60 | 50 | EVM | 23.9 | 2.7 |  |
|  |  |  | 60 | 60 | EVM | 23.9 | 2.7 |  |
|  |  |  | 60 | 80 | EVM | 23.9 | 2.7 |  |
|  |  |  | 60 | 100 | EVM | 23.9 | 2.7 |  |
|  |  | [6,3] | 15 | 5 | EVM | 23.9 | 2.7 | 0.3 |
|  |  |  | 15 | 10 | EVM | 23.9 | 2.7 |  |
|  |  |  | 15 | 15 | EVM | 23.9 | 2.7 |  |
|  |  |  | 15 | 20 | EVM | 23.9 | 2.7 |  |
|  |  |  | 15 | 25 | EVM | 23.9 | 2.7 |  |
|  |  |  | 15 | 40 | EVM | 23.9 | 2.7 |  |
|  |  |  | 15 | 50 | EVM | 23.9 | 2.7 |  |
|  |  |  | 30 | 5 | EVM | 24 | 2.6 |  |
|  |  |  | 30 | 10 | EVM | 23.9 | 2.7 |  |
|  |  |  | 30 | 15 | IBE | 23.9 | 2.7 |  |
|  |  |  | 30 | 20 | EVM | 24 | 2.6 |  |
|  |  |  | 30 | 25 | EVM | 23.9 | 2.7 |  |
|  |  |  | 30 | 40 | EVM | 23.9 | 2.7 |  |
|  |  |  | 30 | 50 | EVM | 23.9 | 2.7 |  |
|  |  |  | 30 | 60 | IBE | 24 | 2.6 |  |
|  |  |  | 30 | 80 | EVM | 23.9 | 2.7 |  |
|  |  |  | 30 | 100 | EVM | 24 | 2.6 |  |
|  |  |  | 60 | 10 | EVM | 24 | 2.6 |  |
|  |  |  | 60 | 15 | EVM | 23.7 | 2.9 |  |
|  |  |  | 60 | 20 | EVM | 23.9 | 2.7 |  |
|  |  |  | 60 | 25 | IBE | 23.9 | 2.7 |  |
|  |  |  | 60 | 40 | EVM | 23.9 | 2.7 |  |
|  |  |  | 60 | 50 | EVM | 23.9 | 2.7 |  |
|  |  |  | 60 | 60 | EVM | 24 | 2.6 |  |
|  |  |  | 60 | 80 | IBE | 23.9 | 2.7 |  |
|  |  |  | 60 | 100 | EVM | 23.9 | 2.7 |  |