3GPP TSG-RAN WG4 Meeting NR AH-3	    R4-1709349

Nagoya, Japan, 18 – 21 Sep., 2017

Agenda Item:	3.5.6.2.1

Source: 	Samsung

Title: 	Cell detection evaluation results

Document for:	Discussion

# Introduction

In RAN4 #NR Ad Hoc-2 meeting, both link level and system level simulations for RRM were discussed, and simulation assumptions were agreed in [1]-[3]. In RAN4#84 meeting, companies provided some initial simulation results for PSS/SSS detection. As observed, there existed some differences between the submitted simulation results. In addition, the simulation assumptions were updated in [4].

In this contribution, we provide the updated simulation results of 90%-tile cell search delay and one shot detection probability based on PSS and SSS according to the agreed assumptions in [4].

# Simulation Results

The detailed simulation assumptions are listed in Table A.1~4 in Annex A. m0, m1 in Table A.4 are updated according to RAN1’s latest agreements [4]. For PSS detection, time-domain correlation is performed, with false alarm probability set to be 0.01%. For SSS detection, correlation is done in the frequency domain. The FFT size is 512.

## SCS 15 kHz

Table 1: 90%-tile cell detection delay (number of SS burst periodicities), 15 kHz

| Eor/Ioc (dB) Channel | Case 1 |  |  | Case 2 |  |  | Case 3 |  |  | Case 4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | -8 | -7 | -6 | -8 | -7 | -6 | -8 | -7 | -6 | -8 | -7 | -6 |
| AWGN | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| EPA5 | 2 | 1 | 1 | 2 | 1 | 1 | 2 | 1 | 1 | 2 | 1 | 1 |
| ETU30 | 2 | 2 | 1 | 2 | 2 | 1 | 2 | 2 | 1 | 2 | 2 | 1 |
| TDL-A | 3 | 2 | 1 | 4 | 2 | 1 | 3 | 2 | 1 | 3 | 2 | 1 |
| TDL-B | 3 | 2 | 1 | 3 | 2 | 1 | 3 | 2 | 1 | 3 | 2 | 1 |
| TDL-C | 3 | 1 | 1 | 3 | 1 | 1 | 3 | 2 | 1 | 3 | 2 | 1 |
| TDL-D | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

Table 2: One shot detection probability, 15 kHz

| SINR (dB) Channel | Case 1 |  |  | Case 2 |  |  | Case 3 |  |  | Case 4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | -8 | -7 | -6 | -8 | -7 | -6 | -8 | -7 | -6 | -8 | -7 | -6 |
| AWGN | 100% | 100% | 100% | 100% | 100% | 100% | 98.4% | 99.9% | 100% | 98.6% | 99.8% | 100% |
| EPA5 | 88.3% | 91.2% | 93.6% | 84.9% | 90.1% | 93% | 86.6% | 90.8% | 93.5% | 85.8% | 90.2% | 93.7% |
| ETU30 | 78.8% | 85.9% | 91% | 81% | 86.7% | 91% | 79.2% | 85.9% | 91.4% | 78.8% | 86.6% | 92.7% |
| TDL-A | 80.9% | 87.5% | 91.7% | 81.2% | 87.7% | 92.2% | 82% | 87.1% | 92.8% | 79.9% | 87.3% | 92.6% |
| TDL-B | 84.5% | 88.8% | 92.8% | 83.5% | 89.9% | 94% | 81.3% | 88.1% | 92.4% | 80.4% | 86.1% | 91.4% |
| TDL-C | 84.8% | 90.1% | 93.2% | 85.2% | 90.4% | 93.6% | 82% | 89.1% | 93.2% | 83.2% | 88.3% | 93.1% |
| TDL-D | 98.8% | 99.6% | 99.9% | 99.6% | 100% | 100% | 94.5% | 98.4% | 99.7% | 95.6% | 98% | 99.4% |

Table 1 shows the simulation results of 90%-tile cell detection delay under AWGN, EPA5, ETU30, TDL-A, TDL-B, TDL-C, and TDL-D channels with subcarrier spacing 15 kHz. Table 2 is the one shot cell detection probability, and since the burst set periodicity is 5 ms, one shot cooresponds to 5 ms.

From Table 1, it can be seen that as SINR increases, the cell detection delay decreases. For AWGN channel and TDL-D channel, 1 shot is enough to meet the 90% detection probability requirement for all cases and SINR conditions. For EPA5, ETU30, TDL-A, TDL-B, and TDL-C channels, the detection delay becomes larger under low SINRs, however, 4 shots are enough for all channels with the lowest SINR.

## SCS 30 kHz

Table 3: 90%-tile cell detection delay (number of SS burst periodicities), 30 kHz

| Eor/Ioc (dB) Channel | Case 1 |  |  | Case 2 |  |  | Case 3 |  |  | Case 4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | -8 | -7 | -6 | -8 | -7 | -6 | -8 | -7 | -6 | -8 | -7 | -6 |
| AWGN | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| EPA5 | 3 | 2 | 1 | 2 | 1 | 1 | 3 | 2 | 1 | 3 | 2 | 1 |
| ETU30 | 2 | 2 | 1 | 2 | 2 | 1 | 2 | 2 | 1 | 2 | 2 | 1 |
| TDL-A | 4 | 3 | 2 | 4 | 3 | 1 | 3 | 2 | 1 | 3 | 2 | 1 |
| TDL-B | 5 | 3 | 1 | 5 | 3 | 1 | 4 | 2 | 1 | 4 | 2 | 1 |
| TDL-C | 3 | 2 | 1 | 3 | 2 | 1 | 3 | 2 | 1 | 4 | 2 | 1 |
| TDL-D | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

Table 4: One shot detection probability, 30 kHz

| Eor/Ioc (dB) Channel | Case 1 |  |  | Case 2 |  |  | Case 3 |  |  | Case 4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | -8 | -7 | -6 | -8 | -7 | -6 | -8 | -7 | -6 | -8 | -7 | -6 |
| AWGN | 100% | 100% | 100% | 100% | 100% | 100% | 98.2% | 99.8% | 100% | 98.5% | 99.8% | 100% |
| EPA5 | 85.5% | 89.5% | 93.8% | 86% | 90.6% | 94% | 84% | 89.6% | 93.4% | 83.6% | 88.9% | 92.9% |
| ETU30 | 74% | 83.8% | 90.6% | 75.1% | 84.4% | 90.8% | 72.3% | 81.7% | 90% | 73% | 84.1% | 91% |
| TDL-A | 77.6% | 84.8% | 89.3% | 78.1% | 83.4% | 90.1% | 81.4% | 86.7% | 90.7% | 79.4% | 85.9% | 90.1% |
| TDL-B | 75.1% | 84% | 90.2% | 76.2% | 84.9% | 91.3% | 76.4% | 85.1% | 90.2% | 76.7% | 84.4% | 90.7% |
| TDL-C | 82.6% | 89.1% | 93% | 82.7% | 89.2% | 93.9% | 77.6% | 85.4% | 91.4% | 79.3% | 86.7% | 91.4% |
| TDL-D | 99.3% | 99.7% | 99.9% | 93% | 97.8% | 99.4% | 94% | 98.2% | 99.3% | 94.8% | 97.5% | 98.8% |

Table 3 and Table 4 are the detection delay and one shot probability for 30 kHz SCS, respectively. In general, the results are similar as for 15 kHz SCS, with a slight difference of 1 or 2 shots for some scenarios. To achieve 90% cell detection delay for all channels, cases, and SINRs conditions, 5 shots are enough.

## SCS 120 kHz

To be updated..

## SCS 240 kHz

To be updated..

# Conclusion

In this paper, we updated the link level simulation results for PSS/SSS detection. It is seen that a maximum of 5 shots are enough to achieve the 90% cell detection probability for all simulated scenarios with SCS 15 kHz and 30 kHz.

# Reference

- R4-1706903, Link level simulation of PSS/SSS detection in NR, Huawei, HiSilicon

- R4-1706904, Link level simulation assumptions for SS block measurements, Ericsson

- R4-1706902, System level simulation in NR RRM, Huawei, HiSilicon, Nokia, Qualcomm

- R4-1708698, Updated link level simulation assumption of cell detection in NR, Huawei, HiSilicon

- R1-1711888, WF on m0 and m1 for NR-SSS, NTT Docomo, ITL, Intel, Samsung, InterDigital, Motorola Mobility, Lenovo

# Annex A: Simulation Assumptions

Table A.1: Cell-specific Parameters for PSS/SSS Detection

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| RF Channel number | - | Channel 1 | Channel 1 |
| Carrier frequency |  | 4GHz; 30GHz | The same frequency as for Cell 1 is used |
| Subcarrier spacing | KHz | 15 kHz; 30 kHz for 4GHz; 120 kHz; 240 kHz for 30GHz | The same values as for Cell 1 are used |
| RB Utilization | % | 100 | 100 |
| Power boosting for PSS and SSS | dB | 0 | 0 |
| Power boosting for Data and Control | dB | 0 | 0 |
| Data Modulation | - | QPSK | QPSK |
| CP Length | - | Normal | Normal |
| Number of transmitted SS block within a SS burst set period (K) | - | 1 (baseline) FFS: 4 | 1(baseline) FFS: 4 |
| SS burst set periodicity | ms | 5 | 5 |
| Relative Delay of 1st Path (synchronous) | μs | 0 | CP/2 |
| Relative Delay of 1st Path (asynchronous) | ms | 0 | 1.5 |
| Frequency Offset relative to UE frequency reference | Hz | 0 | 0 |
| Eor/Ioc for K=1 | dB | 6.4 | -8; FSS: -7; -6 (SINR: -6dB; FFS: -8dB, -7dB) |
| Eor/Ioc for K=4 | dB | 2, 6,4, -2 | 2, 1, 1, 1 (SINR: -2.12, -5.97, -4.45, -1.12); -1, 5, -3, 2 dB (SINR: -5.12, -1.97, -8.45, -0.12) |
| Number of Tx antennas | - | 1 | 1 |
| PSS Sequence ID | - | To be stated by companies together with the results (examples shows in Table 3, 4) | To be stated by companies together with the results (examples shows in Table 3, 4) |
| SSS Sequence ID | - | To be stated by companies together with the results (examples shows in Table 3, 4) | To be stated by companies together with the results (examples shows in Table 3, 4) |
| Propagation Condition | - | AWGN, EPA5, ETU30 TDL-A/B/C/D with 300ns delay spread Note1 CDL-A/B/C/D with 300ns delay spread Note1 (secondary priority) |  |
| Ioc Model Note3 | - | AWGN noise |  |
| NOTE 1: The channel models of CDL and TDL are used for simplified link level evaluations, which are defined in TR 38.900. NOTE 2: Ioc value doesn’t include the three simulated BS signals’ power. |  |  |  |

Table A.2: UE-specific Parameters for Cell Detection

| Simulation parameters | Comments/values |
| --- | --- |
| Prior knowledge of Cell 1 and Cell 2 by the UE | Yes |
| False detect threshold | Required as in a real UE implementation |
| UE having apriority knowledge of system being synchronous or asynchronous | No |
| Performance criterion for comparison | 90%-tile acquisition time for “correct” cell detection of both PSS and SSS sequence IDs. |
| CP length detection | Not simulated |
| Receive antennas | 2, 4 (uncorrelated. Option 1: With receiver beamforming Option 2: Without receiver beamforming) |

Table A.3: Example Cell ID Combinations that can be simulated

| Case # | Cell 1 (Interferer Cell) |  | Cell 2 (Desired Cell) |  | Scenario |
| --- | --- | --- | --- | --- | --- |
| 1 | psc1 | ssc1 | psc2 | ssc2 | Synchronous |
| 2 | psc1 | ssc1 | psc1 | ssc3 | Synchronous |
| 3 | psc1 | ssc1 | psc2 | ssc2 | Asynchronous |
| 4 | psc1 | ssc1 | psc1 | ssc3 | Asynchronous |

Table A.4: Example PSS, SSS indices that can be simulated simulations

| PSS Indices |  | SSS Indices |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Label | Code index | Label | Code index (m0, m1) | Cell ID | (,) |
| psc1 | 43 | ssc1 | (20, 0) | 337 | (112, 1) |
| psc2 | 86 | ssc2 | (40, 0) | 674 | (224, 2) |
|  |  | ssc3 | (35, 0) | 673 | (224, 1) |