3GPP TSG-RAN WG4 NR Adhoc Meeting #3	R4-1709414

Nagoya, Japan, 18 – 21 September, 2017

Source:	SoftBank Corp.

Title:	SCS of sync channel for n41

Agenda item:	3.2.1.1

Document for:	Approval

1.  Introduction

In the last meeting in Berlin, two issues were left unsettled in n41: whether raster scheme for new NR bands is to be applied or not [1] and the selection of SCS for sync channel [2, section 9.3.1.1]. This paper is to express SoftBank’s views on SCS of sync channel for n41.  (Note that raster scheme is discussed in a companion paper [3].)

2.  Consideration

In the last meeting, it was proposed to use 30kHz SCS alone for n41 in its early stage of discussion. Although the author could not attend the session, it seems this proposal might come from 10MHz minimum CBW and/or relatively large total bandwidth.

In OFDM, when SCS is doubled, symbol length will be in half and in principle, CP (cyclic prefix) length has nothing to do with the symbol length or SCS. In current NR physical layer design, however, doubling SCS would also make CP length in half and this will reduce resilience against multipath or inter-symbol interference. While this design seems rational from the standpoint of clean time domain alignment in mixed-numerology cases, the design would also imply/impose trade-off between SCS (or symbol length) and multipath resilience.

Especially when B41 spectrum is re-farmed to NR, it is a natural idea to preserve cell arrangement and serving area as per LTE to the maximum extent. This would cause multipath effects similar to LTE in the long run. For such cases, it is desirable to keep an option offering the same performance as in LTE, i.e. same SCS, to minimize problems occurred. Thus,

[Proposal-1] For sync channel for n41, 15kHz SCS should be supported.

Note that this proposal is not intended to preclude the support of the other SCS, i.e. 30kHz.

The same discussion could go for any LTE re-farming bands considering that there has been LTE eNodeBs /cell design to be reused. Thus, it looks quite rational to support 15kHz SCS for sync channel for these bands in NR, depending on how the cells are deployed for the bands.

[Observation-1] It looks quite rational to support 15kHz SCS for sync channel for LTE re-farming bands in NR.

3.  Conclusion

This paper is to give proposal for an outstanding issue on n41 as:

[Proposal-1] For sync channel for n41, 15kHz SCS should be supported.

4.  Reference

[1]      R4-1708850   WF on Channel raster, Qualcomm,

[2]      R4-170xxxx   (Draft) RAN4#84 Meeting report, RAN4 chair (samsung)

[3]      R4-1709415   Channel raster in n41, SoftBank

form change history:

2013-12-06 v1.14.1 modified §11 to read: <FamilyName>, <GivenName>, (If the person is new to 3GPP work, give full contact coordinates, in particular, email address.)

2013-10-03 v1.14.0 removal of embedded help text

v1.13.2: adds tdoc header

v1.13.1: minor changes resulting from discussions at CT#41 & SA#41

v1.13.0: mods to enforce linkage amongst stages 1, 2, 3

draft mods Scarrone-Meredith 2008-07 ff

v1.12.1: removes revision marks following approval at SP-29
v1.12.0: includes provision for Study Items (SP-29)

v1.11.0: includes those changes from v1.8.0 agreed at SP-25.

v1.10.0: full circle

v1.9.0: a clean sheet

v1.8.0: includes comments from SA#24

v1.7.0: includes comments from RAN, CN and T #24; also includes “early implementation” data

v1.6.0: includes comments made during review period prior to TSGs#24

v1.5.0: includes comments made at TSGs#23 (Phoenix)

v1.4.0: offered to SA#23 for approval

v1.3.0: offered to CN#23, RAN#23 and T#23 for comments

DRAFT4 v1.3.0: 2004-03-09: Incorporation of comments from Leaders list

DRAFT3 v1.3.0: 2004-02-19: Incorporation of comments from MCC members

DRAFT2 v1.3.0: 2004-01-29: Complete redraft:

v1.2.0: 2002-07-04: "USIM" box changed to "UICC apps"

2003-05-28: spelling of “rapporteur” corrected

2002-07-04: "USIM" box changed to "UICC apps"