3GPP TSG-RAN4 Meeting NR Adhoc#3 	R4-1709425

# Nagoya, Japan, 18th – 21th September, 2017

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | xxxx | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR PSCell Addition and Release Delay for E-UTRA-NR Dual Connectivity |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2017-08-27 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Since the NSA operation is supported in TS36.133, therefore, the requirements of NR PSCell addition and release delay with E-UTRA-NR dual connectivity should be specified in TS36.133. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the requirement of NR PSCell addition and release delay with E-UTRA-NR dual connectivity in TS36.133 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirement of NR PSCell addition and release delay with E-UTRA-NR dual connectivity will be missed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section 7.32 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 36.521-3 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

7.32	NR PSCell Addition and Release Delay for E-UTRA – NR Dual Connectivity

7.32.1	Introduction

This section defines requirements for the delay within which the UE shall be able to configure an NR PSCell in E-UTRA – NR dual connectivity. The requirements are applicable to an E-UTRA – NR dual connectivity capable UE. The requirements shall apply for E-UTRA – NR dual connectivity.

7.14.2	PSCell Addition Delay Requirement

The requirements in this section shall apply for the UE configured with only E-UTRA PCell.

Upon receiving NR PSCell addition in subframe n, the UE shall be capable to transmit PRACH preamble towards NR PSCell no later than in subframe n+ Tconfig_ PSCell, EN-DC:

Where:

Tconfig_ PSCell, EN-DC = [TBD]ms + Tactivation_time, EN-DC + [TBD] ms + TPCell_ DU, EN-DC + TPSCell_ DU, EN-DC

Tactivation_time, EN-DC is the NR PSCell activation delay. If the NR PSCell is known, then Tactivation_time, EN-DC is [20]ms. If the NR PSCell is unknown, then Tactivation_time, EN-DC is [30]ms provided the NR PSCell can be successfully detected on the first attempt.

TPCell_ DU, EN-DC is the delay uncertainty due to NR PCell PRACH preamble transmission. TPCell_ DU, EN-DC is up to [20] ms if NR PSCell activation is interrupted by a NR PCell PRACH preamble transmission, otherwise it is 0.

TPSCell_ DU, EN-DC is the delay uncertainty in acquiring the first available PRACH occasion in the NR PSCell. TPSCell_ DU, EN-DC is up to [30]ms.

NR PSCell is known if it has been meeting the following conditions:

During the last [5] seconds before the reception of the NR PSCell configuration command:

-	the UE has sent a valid measurement report for the NR PSCell being configured and

-	the NR PSCell being configured remains detectable according to the cell identification conditions specified in section 8.8,

-	NR PSCell being configured also remains detectable during the NR PSCell configuration delay according to the cell identification conditions specified in section 8.8.

otherwise it is unknown.The PCell interruption specified in section 7.12 is allowed only during the RRC reconfiguration procedure [2].

The NR PSCell addition delay specified in this section can be extended if SRS carrier based switching occurs during the NR PSCell addition procedure.

7.14.3	NR PSCell Release Delay Requirement

The requirements in this section shall apply for a UE configured with E-UTRA PCell and one NR PSCell.

Upon receiving NR PSCell release in subframe n, the UE shall accomplish the release actions specified in [2] no later than in subframe n+20.

The PCell interruption specified in section 7.12 is allowed only during the RRC reconfiguration procedure [2].

The NR PSCell release delay specified in this section can be extended if SRS carrier based switching occurs during the NR PSCell release procedure.