3GPP TSG-RAN WG4 Meeting AH#03	                                   R4-1709469

Nogoya, Japan, 18-21 Sep, 2017

Agenda Item:   3.3.4.5.1

Source:        Samsung

Title:          Potential jammer system in 28GHz OOB range for Region 3

Document for:  Information

Introduction

In RAN4#84 it is agreed in [1] that:

Proposal 1: Companies are encouraged to provide further information on blocking (not preclude other systems than ASDE) especially in Region 3 to define appropriate out-of-band blocking requirements above 24 GHz.

In this contribution information is shared according this agreement.

Information

As summarized in [2] the potential jammer system and system to be protection for 28GHz in ASIA-PACIFIC area could be found in Annex in both frequency range of 24.25-27.5GHz and 31.8-33.4GHz, which also are demonstrated in table 1 and 2 below.

Table 1: characteristics of the existing services/applications necessary for consideration in sharing/compatibility studies in frequency range 24.25-27.5GHz

| Country | Sub Ranges (GHz) | Applications | System Characteristics |
| --- | --- | --- | --- |
| Japan | 24.25-27.5 |  |  |
| Islamic Rep. of Iran | 24.25-27.5 | Fixed (PTP for Backhaul) | ITU-R F.748 R.R Appendices 2&3 |
| Singapore | None | None | None |
| Korea | 24.25-26.65 | SRD for Automotive Radar | Transmitter Power (EIRP) : -41.3dBm/MHz (Average) -24.44 dBm/3MHz (Peak) Occupied Bandwidth : 2.4 GHz or less |
|  | 24-26.6 | SRD | Transmitter Power (power transferred to the antenna) : 100mW and 6dBm/MHz (Average) Antenna Gain : 16dBi or less Occupied Bandwidth : 2.5 GHz or less |
| Socialist Republic of Viet Nam | 24.25-27.5 |  | # Circular 13/2013/TT-BTTTT of Ministry of Information and Communications on frequency arrangements for fixed and mobile service in the frequency range (30-30000) MHz. Decision 860/2002/QĐ-TCBĐ of Ministry of Information and Communications on frequency arrangements for fixed service in the frequency range (30-60) GHz ITU-R Recommendation ITU-R F. 748 |
| Thailand | 24.25-27.5 |  | Thailand is in the process of creating a regulation for spectrum sharing between FSS and FS, and between FSS and MS. This question is therefore to be answered when the regulation is adopted. |
| Australia | 24.25-24.45 | SRDs | Body scanner details available from ACMA website Other SRDs operate on a no interference and no protection basis and must comply with the LIPD Class Licence |
|  | 24.45-24.65 | SRDs | See response for 24.25-24.45 GHz frequency range |
|  | 24.65-24.75 | SRDs | See response for 24.25-24.45 GHz frequency range |
|  | 24.75-25.25 | SRDs | See response for 24.25-24.45 GHz frequency range |
|  | 25.25-25.5 | SRDs | See response for 24.25-24.45 GHz frequency range |
|  | 25.5-27 | SRDs | See response for 24.25-24.45 GHz frequency range |
|  |  | Space research Earth station receivers | - Interference criteria: -156 dB(W/MHz), p(%) = 0.05% - Minimum earth station elevation angles is 5 - Earth station antenna pattern in RR Appendix 7, Annex 3, Section 3 See RALI MS 43 for details |
|  | 27-27.5 | SRDs | See response for 24.25-24.45 GHz frequency range |
|  |  | Satellite Receivers and Gateway Earth stations | It is recommended that AWG consider any contributions from ITU-R WP4A to TG5/1 regarding system characteristics for FSS. |
| Malaysia | 24.25-27.5 GHz | Automotive Radar | Max. mean EIRP: -41.3 dBm/MHz |

Table 2: characteristics of the existing services/applications necessary for consideration in sharing/compatibility studies in frequency range 31.8-33.4 GHz

| Country | Sub Ranges (GHz) | Applications | System Characteristics |
| --- | --- | --- | --- |
| Japan | 31.8-33.4 |  |  |
| Islamic Rep. of Iran | 31.8-33.4 | None | None |
| Singapore | 31.8-33.4 | NA | NA |
| Korea | 31.8-33.4 |  |  |
| Socialist Republic of Viet Nam | 31.8-33.4 |  | # Circular 13/2013/TT-BTTTT of Ministry of Information and Communications on frequency arrangements for fixed and mobile service in the frequency range (30-30000) MHz. Decision 860/2002/QĐ-TCBĐ of Ministry of Information and Communications on frequency arrangements for fixed service in the frequency range (30-60) GHz ITU-R Recommendation ITU-R F. 1520 |
| Thailand | 31.8-33.4 |  | Thailand is in the process of creating a regulation for spectrum sharing between FSS and FS, and between FSS and MS. This question is therefore to be answered when the regulation is adopted. |
| Australia | 31.8-32 | Space research Earth station receivers | Protection criteria used is same as defined in ITU Radio Regulations Appendix 7, Annex 7 Table 8d Minimum earth station elevation angles is 10 Earth station antenna pattern in RR Appendix 7, Annex 3, Section 3 See RALI MS 43 for details |
|  | 32-32.3 | Space research Earth station receivers | See response for 31.8-32 GHz frequency range |
| Indonesia | 31.8 – 33.4 GHz | Point-to-point telecommunication | ITU-R F. 1520 Regulation No. 33 year 2015 of Ministry Communication and Informatics of the Republic of Indonesia on Usage Plan of Radio Frequency Band Point to Point Microwave Link. |

Reference

[1] R4-1707510, Out-of-band blocking around 28 GHz

[2] AWG-21/TMP-51: Working document towards draft new APT Report on Reponses to the Questionnaire on usage and future plan of frequency bands in relation to studies on WRC-19 agenda item 1.13 in Asia-Pacific Region