3GPP TSG-WG RAN4 Meeting #NR3	R4-1709472

# Nagoya, Japan, Date 18th  – 21st Sept, 2017

Source: 	Nokia, Nokia Shanghai Bell

Title: 						NR Range 1 MPR simulations

Agenda Item: 			3.3.3.3.1

Document for:	Approval

# 1	Introduction

In this contribution we present NR UE MPR simulation results based on agreed WF [1].

# 2	Discussion

## 2.1	Simulation assumptions

Simulation assumtios we as stated in [1] with two exceptions, firstly that RB alignment of higher scs were as in Table 1. Reason for this is that the agreement in [1] was not optimum in terms of guadbands and would have caused unnecessary high MPR values. And secondly in-band emissions averaged across 10 subframes due to reason explained in [2].

Table 1: RB alignment of higher scs

| CHBW (MHz) | SCS (kHz) | NRB | RB#0 aligns with ref (RB) | TBC center frequency offset from channel center frequency (kHz) | Lower guard (kHz) | Upper guard (kHz) | Min guard (kHz) | Min guard (%) |  | Total guard (kHz) | TBC BW (kHz) | RB0_SC0 offset (kHz) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 15 | 25 | 0 | -7,5 | 242,5 | 257,5 | 242,5 | 4,85 |  | 500 | 4500 | -2250 |
|  | 30 | 11 | 2 | 75 | 595 | 445 | 445 | 8,9 |  | 1040 | 3960 | -1890 |
| 10 | 15 | 52 | 0 | -7,5 | 312,5 | 327,5 | 312,5 | 3,125 |  | 640 | 9360 | -4680 |
|  | 30 | 24 | 2 | -15 | 665 | 695 | 665 | 6,65 |  | 1360 | 8640 | -4320 |
|  | 60 | 11 | 4 | -30 | 1010 | 1070 | 1010 | 10,1 |  | 2080 | 7920 | -3960 |
| 15 | 15 | 79 | 0 | -7,5 | 382,5 | 397,5 | 382,5 | 2,55 |  | 780 | 14220 | -7110 |
|  | 30 | 38 | 2 | 75 | 735 | 585 | 585 | 3,9 |  | 1320 | 13680 | -6750 |
|  | 60 | 18 | 4 | 60 | 1080 | 960 | 960 | 6,4 |  | 2040 | 12960 | -6390 |
| 20 | 15 | 106 | 0 | -7,5 | 452,5 | 467,5 | 452,5 | 2,2625 |  | 920 | 19080 | -9540 |
|  | 30 | 51 | 2 | -15 | 805 | 835 | 805 | 4,025 |  | 1640 | 18360 | -9180 |
|  | 60 | 24 | 6 | 150 | 1510 | 1210 | 1210 | 6,05 |  | 2720 | 17280 | -8460 |
| 25 | 15 | 133 | 0 | -7,5 | 522,5 | 537,5 | 522,5 | 2,09 |  | 1060 | 23940 | -11970 |
|  | 30 | 65 | 2 | 75 | 875 | 725 | 725 | 2,9 |  | 1600 | 23400 | -11610 |
|  | 60 | 31 | 4 | -120 | 1220 | 1460 | 1220 | 4,88 |  | 2680 | 22320 | -11250 |
| 40 | 15 | 216 | 0 | -7,5 | 552,5 | 567,5 | 552,5 | 1,38125 |  | 1120 | 38880 | -19440 |
|  | 30 | 106 | 2 | -15 | 905 | 935 | 905 | 2,2625 |  | 1840 | 38160 | -19080 |
|  | 60 | 51 | 6 | -30 | 1610 | 1670 | 1610 | 4,025 |  | 3280 | 36720 | -18360 |
| 50 | 15 | 270 | 0 | -7,5 | 692,5 | 707,5 | 692,5 | 1,385 |  | 1400 | 48600 | -24300 |
|  | 30 | 133 | 2 | -15 | 1045 | 1075 | 1045 | 2,09 |  | 2120 | 47880 | -23940 |
|  | 60 | 65 | 6 | 150 | 1750 | 1450 | 1450 | 2,9 |  | 3200 | 46800 | -23220 |
| 60 | 30 | 162 | 0 | -15 | 825 | 855 | 825 | 1,375 |  | 1680 | 58320 | -29160 |
|  | 60 | 79 | 2 | -30 | 1530 | 1590 | 1530 | 2,55 |  | 3120 | 56880 | -28440 |
| 80 | 30 | 217 | 0 | -15 | 925 | 955 | 925 | 1,15625 |  | 1880 | 78120 | -39060 |
|  | 60 | 107 | 2 | 150 | 1630 | 1330 | 1330 | 1,6625 |  | 2960 | 77040 | -38340 |
| 100 | 30 | 273 | 0 | -15 | 845 | 875 | 845 | 0,845 |  | 1720 | 98280 | -49140 |
|  | 60 | 135 | 2 | 150 | 1550 | 1250 | 1250 | 1,25 |  | 2800 | 97200 | -48420 |

## 2.2	Comparison of PA calibration points

Two PA calibration points were simulated to assees the benefit of having better PA linearity that was is assumed for LTE MPR simulations.

Calibration configuration 1 (Nokia’s proposal):

- 20 MHz, 15 kHz

- 100RB at lower channel edge

- QPSK, DFT-S-OFDMA

- MPR = 0 dB

Calibration configuration 2 (LTE PA calibration):

- 20 MHz, 15 kHz

- 100RB at lower channel edge

- QPSK, DFT-S-OFDMA

- MPR = 1 dB

Left-hand side Figures are with Calibration configuration 1 and right-hand side Figures with Calibration configuration 2 (LTE PA calibration).

### 2.2.1	CP-OFDM: QPSK, 16-QAM and 64-QAM (15 kHz scs)

### 2.2.2	CP-OFDM: QPSK (30, 60 kHz scs)

### 2.2.3	DFT-S-OFDM: pi/2 PBSK, QPSK, 16-QAM and 64-QAM (15 kHz scs)

### 2.2.4	DFT-S-OFDM: QPSK (30, 60 kHz scs)

## 2.3	MPR for other channel bandwidth for Calibration configuration 1

In this paragraph we provide simulation results for 5 and 10 MMHz channel bandwidths using only PA calibration configuration 1. Left-hand side figures are CP-OFDM and right-hand side figures are DFT-S-OFDM.

### 2.3.1	5 MHz: pi/2 PBSK, QPSK, 16-QAM and 64-QAM (15 kHz scs)

### 2.3.1	10 MHz: pi/2 PBSK, QPSK, 16-QAM and 64-QAM (15 kHz scs)

## 2.4 	Collection of MPR results

In Table 2 we provide a Collection simulated MPR results for various signals with calibration configuration 1.

Table 2: Collection simulated MPR results for various signals with calibration configuration 1.

| BW [MHz] | SCS [kHz] | M. access | Modulation | RBstart | L_CRB | MPR [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| 5 | 15 | DFT-S-OFDMA | QPSK | 0 | 25 | -0,2 |
| 5 | 15 | CP-OFDMA | QPSK | 0 | 25 | 2 |
| 5 | 30 | DFT-S-OFDMA | QPSK | 0 | 10 | -0,5 |
| 5 | 30 | CP-OFDMA | QPSK | 0 | 11 | 1,8 |
| 20 | 15 | DFT-S-OFDMA | QPSK | 0 | 1 | -1,4 |
| 20 | 15 | CP-OFDMA | QPSK | 0 | 1 | -1,1 |
| 20 | 15 | DFT-S-OFDMA | pi/2-BPSK | 0 | 100 | -1 |
| 20 | 15 | DFT-S-OFDMA | QPSK | 0 | 100 | -0,1 |
| 20 | 15 | DFT-S-OFDMA | 16QAM | 0 | 100 | 0,4 |
| 20 | 15 | DFT-S-OFDMA | 64QAM | 0 | 100 | 0,6 |
| 20 | 15 | CP-OFDMA | QPSK | 0 | 106 | 2,2 |
| 20 | 15 | CP-OFDMA | 16QAM | 0 | 106 | 2,2 |
| 20 | 15 | CP-OFDMA | 64QAM | 0 | 106 | 2,1 |
| 20 | 15 | DFT-S-OFDMA | QPSK | 0 | 18 | -0,6 |
| 20 | 15 | CP-OFDMA | QPSK | 0 | 18 | 1,4 |
| 20 | 15 | DFT-S-OFDMA | pi/2-BPSK | 18 | 18 | -2,2 |
| 20 | 15 | DFT-S-OFDMA | QPSK | 18 | 18 | -1,9 |
| 20 | 15 | DFT-S-OFDMA | 16QAM | 18 | 18 | -0,7 |
| 20 | 15 | DFT-S-OFDMA | 64QAM | 18 | 18 | 0,3 |
| 20 | 15 | CP-OFDMA | QPSK | 18 | 18 | -0,2 |
| 20 | 15 | CP-OFDMA | 16QAM | 18 | 18 | 0,8 |
| 20 | 15 | CP-OFDMA | 64QAM | 18 | 18 | 2 |
| 20 | 30 | DFT-S-OFDMA | QPSK | 0 | 50 | -0,1 |
| 20 | 30 | CP-OFDMA | QPSK | 0 | 51 | 2,1 |
| 20 | 60 | DFT-S-OFDMA | QPSK | 0 | 1 | -2,2 |
| 20 | 60 | CP-OFDMA | QPSK | 0 | 1 | -1,1 |
| 20 | 60 | DFT-S-OFDMA | QPSK | 0 | 4 | -1,2 |
| 20 | 60 | CP-OFDMA | QPSK | 0 | 4 | 0 |
| 20 | 60 | DFT-S-OFDMA | QPSK | 4 | 4 | -2,1 |
| 20 | 60 | CP-OFDMA | QPSK | 4 | 4 | -0,5 |
| 20 | 60 | DFT-S-OFDMA | pi/2-BPSK | 0 | 24 | -1,2 |
| 20 | 60 | DFT-S-OFDMA | QPSK | 0 | 24 | -0,1 |
| 20 | 60 | DFT-S-OFDMA | 64QAM | 0 | 24 | 0,6 |
| 20 | 60 | CP-OFDMA | QPSK | 0 | 24 | 2 |
| 20 | 60 | CP-OFDMA | 64QAM | 0 | 24 | 2,1 |
| 100 | 30 | DFT-S-OFDMA | QPSK | 0 | 270 | 0 |
| 100 | 30 | DFT-S-OFDMA | QPSK | 0 | 20 | -0,4 |
| 100 | 30 | CP-OFDMA | QPSK | 0 | 273 | 2,3 |
| 100 | 30 | CP-OFDMA | QPSK | 0 | 20 | 1,6 |
| 100 | 60 | DFT-S-OFDMA | QPSK | 0 | 135 | 0,1 |
| 100 | 60 | DFT-S-OFDMA | QPSK | 0 | 10 | -0,7 |
| 100 | 60 | CP-OFDMA | QPSK | 0 | 135 | 2,3 |
| 100 | 60 | CP-OFDMA | QPSK | 0 | 10 | 1,2 |

# 3	Conclusion

In this contribution we present our NR range 1 MPR simulation results. In paragraph 2.2 we compare the two PA calibration configuration listed in [1]. In paragraphs 2.3 and 2.4 MPR results are provided only for PA calibration configuration 1.

As can be seen when comparing the MPR results presented in paragraph 2.2 PA calibration configuration 1 provides quite consistently 1 dB lower MPR results than PA calibration configuration 2. With PA calibration configuration 1 there are plenty of QPSK signals that do not require MPR where as with PA calibration configuration 2 some MPR is always needed. PA calibration configuration 1 benefits also DFT-s-OFDM operation with lower MPR.

We think that it is very desireable from system prespective to have CP-OFDM signals that can be specified without allowed MPR thus we propose to use PA calibration configuration 1 as a basis for NR range1 UE MPR.

Proposal: PA Calibration configuration for NR range1 UE MPR:

- 20 MHz, 15 kHz

- 100RB at lower channel edge

- QPSK, DFT-S-OFDMA

- MPR = 0 dB

# 4	References

[1] R4-1708912, NR MPR simulation assumptions for sub-6 GHz operation, Nokia, RAN4#84

[2] R4-1709481, TP for TR General aspects for UE RF for NR: Range 1 in-band emission requirement, Nokia, Nokia Shanghai Bell, Skyworks Solutions, Inc., RAN4# NR AH3

# 5	Annex

For interested readers we provide in this annex all the simulation results we produced for the NR AH3.

## 5.1	Results for 20 MHz Carrier with both calibration configurations

Calibration configuration 1

Calibration configuration 2

## 5.2	Results for 15 MHz Carrier with calibration configuration 1

## 5.3	Results for 10 MHz Carrier with calibration configuration 1

## 5.4	Results for 5 MHz Carrier with calibration configuration 1