3GPP TSG-WG RAN4 Meeting #NR3	R4-1709479

# Nagoya, Japan, Date 18th  – 21st Sept, 2017

Source: 	Nokia, Nokia Shanghai Bell, Skyworks Solutions, Inc.

Title: 					TP for TR General aspects for UE RF for NR: Range 1 UE power class

Agenda Item: 			3.3.3.2

Document for:	Approval

# 1	Introduction

This contribution discussed the NR range 1 power classes and have associated TP.

# 2	Discussion

Following agreements have been made in RAN4 concearning NR range 1 UE power classes.

[1] 23 dBm +2/-3 dB for Band n77 and n78 of PC3

[2] It is proposed to specify both PC2 and PC3 UE for 3.5GHz NR frequency range in Rel-15 to accommodate different operator demands

Open issues are the power class and tolerance for LTE refarming bands, tolerange for power class 2 operation and  power class and tolerance for SUL bands.

In this contribution we make following proposals.

Proposal 1: E-UTRA PC 3 power class and associated tolerances are used for NR bands which are re-farmed from E-UTRA

Proposal 2: E-UTRA PC2 tolerance is used for NR PC2

If above proposals are acceptable we propose to capture Table 1 into the TR.

Table 1: NR UE power classes

| NR Operating band | Class 2 | Class 3 | Comments |
| --- | --- | --- | --- |
| n1 |  | 23 dBm ± 2 dB | LTE refarming band |
| n2 |  | 23 dBm ± 2 dB | LTE refarming band |
| n3 |  | 23 dBm ± 2 dB | LTE refarming band |
| n5 |  | 23 dBm ± 2 dB | LTE refarming band |
| n7 |  | 23 dBm ± 2 dB | LTE refarming band |
| n8 |  | 23 dBm ± 2 dB | LTE refarming band |
| n20 |  | 23 dBm ± 2 dB | LTE refarming band |
| n28 |  | 23 dBm +2/-2.5 dB | LTE refarming band |
| n38 |  | 23 dBm ± 2 dB | LTE refarming band |
| n41 |  | 23 dBm ± 2 dB | LTE refarming band |
| n50 |  | 23 dBm ± 2 dB | LTE refarming band |
| n51 |  | 23 dBm ± 2 dB | LTE refarming band |
| n66 |  | 23 dBm ± 2 dB | LTE refarming band |
| n70 |  | 23 dBm ± 2 dB | LTE refarming band |
| n71 |  | 23 dBm +2/-2.5 dB | LTE refarming band |
| n74 |  | 23 dBm ± 2 dB | LTE refarming band |
| n77 | 26 dBm +2/-3 dB | 23 dBm +2/-3 dB | 3.3 - 4.2 GHz |
| n78 | 26 dBm +2/-3 dB | 23 dBm +2/-3 dB | 3.3 - 3.8 GHz |
| n79 |  | TBD | 4.4 - 5 GHz |
| n80 |  | TBD | SUL 1710 - 1785 MHz |
| n81 |  | TBD | SUL 880 - 915 MHz |
| n82 |  | TBD | SUL 832 - 862 MHz |
| n83 |  | TBD | SUL 703 - 748 MHz |
| NOTE: If the uplink/downlink configuration is 0 or 6, the requirements for power class 2 are not applicable, and the corresponding requirements for a power class 3 UE shall apply. |  |  |  |

Bands refleted in the Table 1 are taken from the document that rapporteur circulated in the email approval after the RAN4#84. If new bands are added into the WI then above table is updated. Once further agreements are reached for items marked as TBD in above table it will be updated.

# 3	Conclusion

This contribution we have made following proposals for  NR range 1 power classes and have associated TP.

Proposal 1: E-UTRA PC 3 power class and associated tolerances are used for NR bands which are re-farmed from E-UTRA

Proposal 2: E-UTRA PC2 tolerance is used for NR PC2

# 4	References

[1] R4-1709133, WF on UE RF requirements for 3.3-3.8 GHz, 3.3-4.2 GHz and 4.4-5 GHz, NTT DOCOMO, INC. RAN4#84

[2] R4-1706068, WF on high power UE for 3.5GHz, RAN4#83

******************************* Start of the TP *******************************************

## 5.2	Transmit power

### 5.2.1		Power Class

Detailed structure of the subclause is TBD.

The following NR UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth for non CA configuration unless otherwise stated.

Table X.X.X-1: NR UE power classes

| NR Operating band | Class 2 | Class 3 | Comments |
| --- | --- | --- | --- |
| n1 |  | 23 dBm ± 2 dB | LTE refarming band |
| n2 |  | 23 dBm ± 2 dB | LTE refarming band |
| n3 |  | 23 dBm ± 2 dB | LTE refarming band |
| n5 |  | 23 dBm ± 2 dB | LTE refarming band |
| n7 |  | 23 dBm ± 2 dB | LTE refarming band |
| n8 |  | 23 dBm ± 2 dB | LTE refarming band |
| n20 |  | 23 dBm ± 2 dB | LTE refarming band |
| n28 |  | 23 dBm +2/-2.5 dB | LTE refarming band |
| n38 |  | 23 dBm ± 2 dB | LTE refarming band |
| n41 |  | 23 dBm ± 2 dB | LTE refarming band |
| n50 |  | 23 dBm ± 2 dB | LTE refarming band |
| n51 |  | 23 dBm ± 2 dB | LTE refarming band |
| n66 |  | 23 dBm ± 2 dB | LTE refarming band |
| n70 |  | 23 dBm ± 2 dB | LTE refarming band |
| n71 |  | 23 dBm +2/-2.5 dB | LTE refarming band |
| n74 |  | 23 dBm ± 2 dB | LTE refarming band |
| n77 | 26 dBm +2/-3 dB | 23 dBm +2/-3 dB | 3.3 - 4.2 GHz |
| n78 | 26 dBm +2/-3 dB | 23 dBm +2/-3 dB | 3.3 - 3.8 GHz |
| n79 |  | TBD | 4.4 - 5 GHz |
| n80 |  | TBD | SUL 1710 - 1785 MHz |
| n81 |  | TBD | SUL 880 - 915 MHz |
| n82 |  | TBD | SUL 832 - 862 MHz |
| n83 |  | TBD | SUL 703 - 748 MHz |
| NOTE: If the uplink/downlink configuration is 0 or 6, the requirements for power class 2 are not applicable, and the corresponding requirements for a power class 3 UE shall apply. |  |  |  |

******************************* End of the TP *******************************************