3GPP TSG-WG RAN4 Meeting #NR3	R4-1709485

# Nagoya, Japan, Date 18th  – 21st Sept, 2017

Source: 	Nokia, Nokia Shanghai Bell, Skyworks Solutions, Inc.

Title: 					TP for TR General aspects for UE RF for NR: Range 1 UE EVM

Agenda Item: 			3.3.3.7.2

Document for:	Approval

# 1	Introduction

In this contribution we discuss NR range 1 UE EVM requirement.

# 2	Discussion

Preivously RAN4 has agreed to use LTE EVM requirement as a basis for NR range 1 MPR simulations [1], see Table 1.

Table 1: EVM target per modulation for NR MPR simulations

| Modulation | EVM |
| --- | --- |
| Pi/2 BPSK | FFS% |
| QPSK | 17.5% |
| 16QAM | 12.5% |
| 64QAM | 8% |
| 256QAM | 3.5% |

We propose that values presented in Table 1 are also specified for NR as a minimum requirement. EVM requirement in the table is FFS and RAN4 needs to make a decision on that.

For LTE PBSK EVM requirement is the same as QPSK i.e. 17.5 % however for NR it could be specified to be more relaxed. In previous RAN4 meeting it was agreed to use 35% EVM target in MPR evaluations for NR Range 2.

In theory EVM requirement for rotated pi/2 BPSK could be 3 dB higher compared to pi/4 QPSK if all relevant transmitter assumptios are same. See Figure 1 where BPSK and QPSK nose margins are compared.

Figure 1: QPSK vs. BPSK EVM difference

Hence 25% EVM could be used for PI/2 PBSK however with an understanding that spectral shaping discussion may cause a need to re-asses the value.

# 3	Conclusion

We are proposing to adopt the EVM values in Table 2 as a minimum requirement for NR range 1 UE. PI/2 PBSK value subject to change due to specrum shaing discussion.

Table 2: EVM  per modulation for NR range 1 UE

| Modulation | EVM |
| --- | --- |
| Pi/2 BPSK | [25%] |
| QPSK | 17.5% |
| 16QAM | 12.5% |
| 64QAM | 8% |
| 256QAM | 3.5% |

# 4	References

[1] R4-1706612, Proposed EVM for NR UE Bellow 6GHz, Skyworks Solutions, Inc., Nokia, Alcatel-Lucent Shanghai Bell, RAN4#NR AH2

[2] R4-1708814, Way Forward on MPR evaluation for mmW, Qualcomm, Skyworks, RAN4#84

******************************* Start of the TP ******************************************

## 5.4	Transmit signal quality

Detailed structure of the subclause is TBD.

### 5.4.X	Error vector magnitude for range 1

Error vector magnitude presented in Table 5.4.X-1 is specified as as a minimum requirement for NR range 1 UE. PI/2 PBSK value subject to change due to specrum shaing discussion.

Table 5.4.X-1: EVM  per modulation for NR range 1 UE

| Modulation | EVM |
| --- | --- |
| Pi/2 BPSK | [25%] |
| QPSK | 17.5% |
| 16QAM | 12.5% |
| 64QAM | 8% |
| 256QAM | 3.5% |

******************************* End of the TP ******************************************