3GPP TSG-RAN WG4 Meeting NR #3	            R4-1709502

Nagoya, Japan, 18-21 September 2017

Agenda item:	3.3.4.4.2

Source: 	Intel Corporation

Title: 	NR UE blocking for sub-6GHz

Document for:	Discussion

# Introduction

NR UE blocking was discussed briefly in the last meetings. This document provides our views on the NR UE in-band blocking.

# Discussion

It was agreed in the previous meetings that the wanted signal bandwidth and the interferer signal bandwidth are the same for ACS. In the following two tables, we propose the settings for in-band blocking.

Table 1: In-band blocking parameter

| Rx parameter | Units | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 50 MHz | 100 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |  |
|  |  | 6 | [6] | [6] | [6] | [6] | [6] |
| BWInterferer | MHz | 5 | 10 | 15 | 20 | 50 | 100 |
| FIoffset, case 1 SCS of 15kHz | MHz | 7.5+0.0125 | 15+0.0025 | 22.5+0.0075 | 30+0.0125 | 75+0.0125 | 150+0.0025 |
| FIoffset, case 1 SCS of 30kHz | MHz | 7.5+0.005 | 15+0.025 | 22.5+0.015 | 30+0.005 | 75+0.005 | 150+0.025 |
| FIoffset, case 1 SCS of 60kHz | MHz | 7.5+0.05 | 15+0.01 | 22.5+0.03 | 30+0.05 | 75+0.05 | 150+0.01 |
| FIoffset, case 2 SCS of 15kHz | MHz | 12.5+0.0075 | 25+0.0075 | 37.5+0.0075 | 50+0.0075 | 125+0.0075 | 250+0.0075 |
| FIoffset, case 2 SCS of 30kHz | MHz | 12.5+0.015 | 25+0.015 | 37.5+0.015 | 50+0.015 | 125+0.015 | 250+0.015 |
| FIoffset, case 2 SCS of 60kHz | MHz | 12.5+0.03 | 25+0.03 | 37.5+0.03 | 50+0.03 | 125+0.03 | 250+0.03 |

Table 2: In-band blocking

| E-UTRA band | Parameter | Unit | Case 1 | Case 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -56 | -44 |  |  |  |
|  | FInterferer (offset) | MHz | =-BW/2 – FIoffset,case 1 & =+BW/2 + FIoffset,case 1 | ≤-BW/2 – FIoffset,case 2 & ≥+BW/2 + FIoffset,case 2 |  |  |  |
| NR sub-6GHz bands | FInterferer | MHz | (NOTE 2) | FDL_low – 15 to FDL_high + 15 |  |  |  |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band NOTE 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency -BW/2 - FIoffset, case 1 and b. the carrier frequency +BW/2 + FIoffset, case 1 NOTE 3: FInterferer range values for unwanted modulated interfering signal are interferer center frequencies |  |  |  |  |  |  |  |

Proposal:  RAN4 should agree on the values in the above tables.

# Conclusion

Proposal:  RAN4 should agree on the values in the above tables.

# References

- R4-1706525: “UE ACS for sub-6 GHz”, Intel Corporation