3GPP TSG-RAN WG4 Meeting NR #3	R4-1709544

Nagoya, Japan, 18 – 21 September, 2017

Source:		Ericsson

Title:			Discussion on combined limit for EMC radiated emission and RF spurious emission for  NR BS 1-R

Agenda item:		3.4.1.4

Document for:	Discussion

# Introduction

For eAAS, it has been agreed in TR 37.843 [1] to specify a combined OTA spurious emissions limit in the conformance specification taking into account both the RF spurious emission and the EMC radiated emission requirements. To enable this combination it has been agreed to sum the individual limits in terms of TRP. This approach is discussed in this contribution in terms of stringency when compared with legacy requirements and concludes a proposal for NR 1-R.

# Discussion

Figure 1 shows some of the different emissions limits to consider when specifying a combined OTA spurious emissions limit for AAS type NR base stations (1-R). The values entered for spurious emission have been taken from the approved draft CR for 38.104 from RAN4-#84 [2], where the legacy RF spurious emission limits have been scaled with fixed scaling factors for UMTS and E-UTRA. The EMC radiated emission limit is the current EMC radiated emission limit from TS 37.113 converted to TRP by adding the dipole gain, in accordance with the approach agreed in TR 37.843 [1]. Note: for some bands the upper frequency limit for spurious emissions is extended to include the 5th harmonic. This is not taken into account in this figure.

-34 dBm

-28 dBm

-30 dBm

-24 dBm

-21 dBm

-13 dBm

-7 dBm

-4 dBm

Frequency

Figure 1. Spurious emission limits and EMC radiated emission limit as expressed in TRP for frequencies 30 MHz to 12.75 GHz.

If the agreed approach of TR 37.843 [1] is applied the total OTA spurious emission limit is defined as the linear sum of the spurious emission limit in TRP and the EMC radiated emission limit expressed in TRP (by applying correction factor of 2.15 dBi). The resulting total limits are shown in Figure 2 below.

Frequency

-28.5 dBm

-26 dBm

-24 dBm

-21 dBm

-13 dBm

-7 dBm

-4 dBm

Figure 2. Spurious emission limits combined through linear sum with EMC radiated emission limit expressed in TRP for frequencies 30 MHz to 12.75 GHz.

The maximum difference when comparing the different sets of limits in Figures 1 and Figure 2 is approximately 1.5 dB (yellow line, CAT B, n=4). This shows that assuming fixed scaling persists for CAT B limits, the addition of the EMC radiated emission limit will only make a minor difference to the stringency of the requirement.

Note: As regulations in Europe are not done in this area and will not be done before Rel-15, it is not possible to find the answer to whether or not the fixed scaling will be accepted or not. There are, however, valid arguments to keep use some form of scaling for the emissions based on what has been previously specified in RAN4 for multiple transmitter products.

Observation 1: Figure 2 shows that the linear sum of the EMC radiated emission limit and the RF spurious emission limits does not significantly impact the total OTA spurious emission limit.

As this discussion has been ongoing for the last few meetings, it is proposed to settle the approach to be able to continue specifying requirements. Therefore it is proposed to agree on the following:

Proposal 1: Apply solely the RF spurious emission OTA TRP limits as the total OTA spurious emission limits for NR BS 1-R, i.e. do not specify the limit as a sum of RF spurious + EMC radiated emission as previously agreed in AAS TR.

Note: Agreeing on proposal 1 will impact the approach also in eAAS as the approaches need to be aligned.

# Conclusion

Based on the discussion in Section 2, the following is proposed:

Proposal 1: Apply solely the RF spurious emission OTA TRP limits as the total OTA spurious emission limits for NR BS 1-R, i.e. do not specify the limit as a sum of RF spurious + EMC radiated emission as previously agreed in AAS TR.

# References

3GPP AAS TR 37.843 Radio Frequency (RF) requirement background for Active Antenna System (AAS) Base Station (BS) radiated requirements, v0.4.0.

R4-1709177, DRAFT CR to TS 37.105 - Section 9.7.7, 10.7 Tx/Rx spurious emissions – mandatory, Huawei, RAN4-#84.