3GPP TSG-RAN WG4 NR#3	R4-1709607

Nagoya, Japan, 18-21 September 2017

Source: 	Ericsson, Nokia

Title:  	TP on dynamic range requirements for Range 2 NR BS in Rel-15 TR 38.xxx: General aspects for RF, RRM and demodulation for NR (Release 15)

Agenda Item:	3.4.4.1

Document for:	Approval

# Introduction

A new TR is being proposed for documenting discussions on different aspects of NR.

In this contribution, we propose a text for this TR which includes texts related to the dynamic range requirement for mmWave NR BS.

# Text proposal

The following text proposal is intended for TR38.xxx

<<<<< start of TP <<<<<<

# 7.3 Dynamic range

### 7.3.1	Dynamic range requirements for Range 2 NR BS

In the table below, we summarize investigations from different companies related to mmWave receiver noise floor rise requirements.

| R4-1706752: BS receiver dynamic range for mmWave bands Source: Huawei, HiSilicon | Proposal: The dynamic range simulation result for mmWave bands is small and no need to specify the requirement. |
| --- | --- |
| - R4-1705097, Simulation results for receiver dynamic range of NR BS receiver with different deployment scenarios - Source: Ericsson | Observation-1: Noise floor rise for dynamic range of NR receiver is ~0dB for all scenarios at 50 percentile UL interference. Observation-2: Noise floor rise for dynamic range of NR receiver is ~0dB for UMa and UMi scenarios at 90 percentile UL interference. Observation-3: Noise floor rise for dynamic range of NR receiver for InH scenario at 99 percentile UL interference is not substantial. Proposal: There is no need to specify dynamic range requirement for NR BS. |
| - R4-1705430: Proposal on mmWave NR BS Receiver Dynamic Range - Source: Nokia, Alcatel-Lucent Shanghai Bell | Proposal: To specify the mmWave NR BS receiver dynamic range requirement with the interference level of 15dB over the receiver thermal noise floor, and the wanted signal level calculated as the interference signal level plus the required UL 16QAM SNR and implementation margin. Here the SNR can be obtained at 95% relative throughput from link level simulations, and the implementation margin can be defined as 2.5dB. |
| R4-1706813: Simulation results for receiver dynamic range of NR BS receiver without array gain for different deployment scenarios, Source: Ericsson | Observation: Noise floor rise for dynamic range of NR receiver without including the array gain is ~0dB for all scenarios at 50 and 99 percentile UL interference. Proposal: There is no need to specify dynamic range requirement for NR BS. |
| R4-1706774: Discussion on receiver dynamic range and ICS requirement of range2 NR BS Source: ZTE Corporation | Proposal 1: not to specify the dynamic range requirement for rang2 NR BS. |
| R4-1704930: Discussion on NR BS dynamic range, Source: CATT | Observation 1: Due to the considerable path loss in mmW frequency range, the interference signals reach to the BS side will be much lower in NR system than in LTE system. Observation 2: The UE specific BF will bring on significant rejections to the interference signals in the directions other than the direction of wanted signal. Observation 3: The large channel bandwidth of NR will result in a higher noise floor. Observation 4: Considering that there has been the REFSENS requirement which is aims to measure the receiver capability to receive the wanted signal in the level of noise floor, it seems unnecessary to define a similar requirement, say dynamic range requirement. Observation 5: For sub-6GHz NR BS dynamic requirement, it’s acceptable to reuse the same requirement of LTE except that the reference measurement channel may need to be further investigated for the new numerologies, channel bandwidth and coding schemes. |

As it is seen from the above summary, most of the investigations points to a common conclusion that there is no need to define NR BS receiver dynamic range requirement for mmWave base station, considering the noise floor rise for NR BS receiver dynamic range in the simulated scenarios.

In addition to the above investigations, it also been discussed whether other requirements implicitly demonstrate compliance to a small (e.g. 5dB) dynamic range.

For LTE, the demodulation requirements are set well above the noise floor. It is reasonable to assume that a similar approach may be adopted for NR demodulation, then the demodulation requirements will be set well above the receiver noise floor.

In case of LTE, with 5MHz channel bandwidth, the noise floor is -103dBm with a 5dB Noise Figure. So, the 5dB noise floor would result in -98dBm in AWGN. As seen in Table 8.2.1.4.2-1 of TS36.141, the AWGN power level at the BS input is specified as -80.4dBm/18MHz for 20MHz channel bandwidth. For receiver dynamic range test, 16-QAM is used, the SINR in that case is around 10.3dB, which results in absolute signal level around 75dBm. This is well above 5dB from the noise floor.

In NR, the AWGN noise level will be higher compared to LTE levels, due to channel bandwidth and noise figure levels. However, if we follow same approach in defining the demodulation requirements as it is done for LTE, the absolute levels for demodulation requirements will be well above the AWGN noise floor. Thus, we do not see any need for testing dynamic range requirements in the order of 5dB.

Based on the above discussions and based on proposals from other companies, the following is agreed:

Agreement: There is no need to specify dynamic range requirement for range2 NR BS.

The absolute levels for demodulation requirements shall be well above (e.g. at least ~5dB) the AWGN noise floor, thus the small dynamic range shall be handled via demodulation requirements.

<<<<< end of TP <<<<<<

# Conclusion

We propose to adopt the above mentioned text proposal for TR 38.xxx.

# References