3GPP TSG-RAN WG4 Meeting #NR-AH3	R4-1709683

Nagoya, Japan, 18 - 21 Sep, 2017

Agenda item:	3.3.2.1

Source:	Dish Network

Title:	Considerations on switched single UL/dual UL transmission in LTE-NR

Document for:	Approval

# 1	Introduction

We provide our considerations on single/dual TX transmission in LTE-NR.

# 2	Discussion

Switched single UL/dual UL transmission in LTE-NR was discussed quite intensively in RAN4#84. Basically the heated topic is to which extent switched single UL/dual UL transmission should be supported in LTE-NR and how to indicate related capability.

At first we clarify the terminology used in this contribution. Switched single UL transmission means that either LTE or NR UL is transmitting at a given time instance but not both. Dual UL transmission means LTE and NR UL’s are transmitting concurrently.

Some of the related RAN1 agreements are copied below for convenience.

R1-1709586:

Agreements:

- For NR NSA for a UE, NR supports the case that when the UE is configured with multiple UL carriers on different frequencies (where there is at least one LTE carrier and at least one NR carrier of a different carrier frequency), the UE operates on only one of the carriers at a given time among a pair of LTE and NR carriers

  - FFS whether or not there is specification impact

    - If there is RAN1 specification impact, aim to minimize the specification impact for NR

    - Note: this feature by itself is not intended to have any LTE RAN1 specification impact

  - Note: the other case of allowing simultaneous operation on two or more UL carriers is already agreed to be supported

R1-1711878:

Agreements:

- Support the following solution to single UL transmission where NW synchronization between eNodeB and gNodeB is assumed (where there is at least one LTE carrier and at least one NR carrier of a different carrier frequency)

  - When UE is activated with multiple UL carriers on different frequencies, time-switching of LTE UL carrier and NR UL carrier is used

    - UL transmission timing pattern of LTE carrier and NR carrier is semi-statically shared between eNodeB and gNodeB

    - FFS: Signaling to UE of UL transmission timing pattern

  - UE simultaneously receives signals/channels from both NR DL carrier and LTE DL carrier

    - For scheduling/HARQ timing of LTE FDD carrier, the following timing can be considered, e.g., for LTE:

      - DL-reference UL/DL configuration for TDD

      - DL-reference UL/DL configuration defined for FDD-SCell in TDD-FDD CA with TDD-PCell

      - Up to NW implementation (i.e., no RAN1 spec. impact)

    - For scheduling/HARQ timing of NR carrier, no special handling would be necessary

  - Other solutions are not precluded

Two essentially different scenarios have been in discussions and should be clearly differentiated; LTE-NR within single CC and LTE-NR within different bands. When LTE and NR UL’s are located within same component carrier, intermodulation issues, if any, are no different to LTE single CC multi-cluster transmissions. When LTE and NR UL’s are on different frequency bands, then the likelihood of IMD issues increases a lot. Harmonic relations i.e. higher band DL CC = x times lower band UL CC occur in both cases.

Observation 1: LTE-NR within single CC and LTE-NR within different bands should be clearly differentiated because intermodulation is really of an issue only when UL’s are on different frequency bands

Observation 2: Harmonic issues are equally possible for both LTE-NR within single CC and LTE-NR within different bands

The remaining part of the contribution considers NR-LTE within different bands only.

LTE 2UL/2DL and 2UL/xDL MSD requirements have been defined for a big number of band combinations. After a long discussion in RAN4 it was finally decided that only the so called worst case is specified (and verified). Hence even some of the MSD numbers are 20-30dB it still does not mean that the whole band with all frequency configurations would suffer that high MSD. These requirements are derived using baseline RF component performance without any 2UL specific optimizations.

Observation 3: MSD numbers in LTE 2UL/2DL and 2UL/xDL represent the worst case single frequency combination spot.

While reading RAN1 agreements we noticed that at least one of the agreements for switched single UL was made under assumption of NW synchronization between eNB and gNB (LTE and NR) What if the networks are not synchronized? At least in LTE DC there were two cases, synchronized and non-synchronized mode. We suggest that the case when NW’s are not synchronized is clarified.

Observation 4: Applicability of switched single UL transmission in NR-LTE seems to be valid for synchronized case only. Case for unsynchronized NW should be clarified as well.

The baseline for LTE-NR should dual UL transmission. In other words, dual UL transmission should be mandatory for all NR-LTE combinations. On RF side, there is virtually no HW impact; in any case one needs to support LTE band in standalone mode and because the NR band is at different frequency, it needs of RF HW as well. In theory, one could build UE with one TX path only but switching quickly between to entirely different frequencies would likely nullify the potential benefits. The only RF FE component that might be shared and hence cost savings would be achieved is the PA, but at least for the current NR-LTE combos the frequencies for LTE and NR are so different that in any case dedicated PA’s are needed.

Proposal 1: Dual TX transmission is the baseline for LTE-NR meaning all NR-LTE combinations shall support dual UL transmission.

Observation 5: HW impact is negligible in NR-LTE within different frequency bands.

Last topic is capability signalling. Before making any decisions whether capability signalling is defined the need for it must be well understood. As an example, assume UE signals that it supports also switched single UL transmission. NW can schedule according to that capability. The open question is what needs to be changed in UE to meet these switched single UL requirements? Changes, if any, could for instance be more stringent time related requirements due to more stringent scheduling patterns. If the UE design would not change, is capability signalling needed at all? Specification could have requirements for all 2UL combinations, pretty much like in LTE. By following those requirements, the NW can make it’s assessment whether to configure switched single UL or dual UL transmissions.

On the contrary, If there are UE design changes needed to support switched single UL in NR-LTE, then capability signalling is more justified. In this case, working groups should study if there are benefits to support both switched single UL and dual UL in IMD scenario and non-IMD scenario.

Proposal 2: The need and additional value for capability signalling must be well understood before making decisions.

# 3	Conclusions

Single/dual TX transmission in LTE-NR was considered with the following observations.

Observation 1: LTE-NR within single CC and LTE-NR within different bands should be clearly differentiated because intermodulation is really of an issue only when UL’s are on different frequency bands

Observation 2: Harmonic issues are equally possible for both LTE-NR within single CC and LTE-NR within different bands

Observation 3: MSD numbers in LTE 2UL/2DL and 2UL/xDL represent the worst case single frequency spot.

Observation 4: Applicability of switched single UL transmission seems to be valid for synchronized case only. Case for unsynchronized NW should be clarified as well.

Proposal 1: Dual TX transmission is the baseline for LTE-NR

Observation 5: HW impact is negligible in NR-LTE within different frequency bands.

Proposal 2: The need and additional value for capability signalling must be well understood before making decisions.

# 4	References