3GPP TSG-RAN WG4 Meeting #NR-AH3	R4-1709684

Nagoya, Japan, 18 - 21 Sep, 2017

Agenda item:	3.3.4.3.1

Source:	Dish Network

Title:	Considerations on mmWave UE maximum input level

Document for:	Approval

# 1	Introduction

We provide our views on mmWave UE maximum input level.

# 2	Discussion

Maximum input level for mmWave UE was discussed in RAN4#84 [1]. Maximum input level may become an issue for indoor deployments, where the distances can be small between UE’s and BS. In addition, in practice the UE locations are fairly static underlining potential issues.

Our concern is that too many UE’s might run into compression in indoor scenario. RAN4 should seek options to define maximum input level in a way that this concern is minimized. So far -35dBm has been proposed in a BW agnostic manner [1]. We are ok to discuss that value, depending on the applicable BW’s. We do recognize that it is neither possible nor meaningful to define maximum input level according to absolute worst case with respect to distance etc.

An important thing to consider is how to define requirements for single CC in NR.

First aspect is that UE can implement wide bandwidths either by single CC or by intra-band CA. RAN4 has not yet agreed the details related to this, for instance if the requirements to be met are from wide single CC requirements or from intra-band CA requirements.

Another aspect is that unlike in LTE, there is no maximum single CC BW defined for mmWave. Hence there is no automatic boundary between single CC and CA requirements.

Observation1: There is no clear boundary between single CC and CA requirements in NR, at least not yet

We suggest RAN4 should define single CC maximum input level up to certain BW, for instance 200MHz. This requirement shall be met with UE in single CC configuration or in CA configuration. For frequencies above 200MHz, scaling of the max input power according to total BW applies.

We are concerned that the lower max level requirement is chosen, the higher amount of UE’s may have input overload in indoor scenario. RAN4 should consider if -32dBm can be chosen as max input level up to [200]MHz. With -32dBm, only about 10% of UE’s would perceive input level above that in indoor scenario [1].

If the requirement for up to [200]MHz is -32dBm, then the requirement for [200+200] aggregated BW would be -29dBm and so forth.

Proposal1: RAN4 should consider if -32dBm can be chosen as max input level up to [200]MHz. This limit shall be met either in single CC configuration or in CA configuration

Proposal2: For BW’s above [200]MHz, power scaling applies

The reasoning behind proposal 1 is that when UE’s have different max CC BW capabilities, the requirement would look strange.

To underline the complexity of selecting appropriate method to define the requirement, we take a couple of examples.

- Assuming the requirement is made according to proposals 1 and 2. UE max UE BW is 400MHz. Hence the requirement for 400MHz would be -29dBm.

- Assuming the requirement is made according to proposals 1 and 2. UE max UE BW is 50MHz or 200MHz. Hence the requirement for 8x50 or 2x200=400MHz would be -29dBm.

- Assuming the requirement for single CC in bandwidth agnostic manner is say -35dBm [1] and the max UE BW is 400MHz. Hence the requirement for 400MHz would be -35dBm.

- Assuming the requirement for single CC in bandwidth agnostic manner is say -35dBm [1] and the max UE BW is 50MHz or 200MHz. Hence the requirement for 8x50MHz or 2x200MHz=400MHz would be FFS or -32dBm, respectively. If power scaling is used, requirement for 8x50MHz would be -26dBm

After all, the effective maximum input level requirement should be the same no matter how 400MHz is implemented in UE. If not, then the point of allowing UE to support different channel BW’s with single CC or intra-band CA configuration is questionable.

# 3	Conclusions

mmWave UE max input level was discussed with the following observations and proposals.

Observation1: There is no clear boundary between single CC and CA requirements in NR, at least not yet

Proposal1: RAN4 should consider if -32dBm can be chosen as max input level up to [200]MHz

Proposal2: For BW’s above [200]MHz, power scaling applies

# 4	References

[1] R4-1707829, “mmWave maximum input level and ACS signal level”, Huawei, Hisilicon, RAN4#84, Berlin, Germany