3GPP TSG-RAN WG4 Meeting #NR-AH3	R4-1709686

Nagoya, Japan, 18 - 21 Sep, 2017

Agenda item:	3.3.4.1.2

Source:	Dish Network

Title:	Considerations on Sub-6GHz UE REFSENS

Document for:	Approval

# 1	Introduction

We provide our views on Sub-6GHz UE REFSENS in this contribution.

# 2	Discussion

The WF agreed in RAN4#84 [1] is as follows.

- REFSENS are calculated for SA case per band and channel bandwidths

- Separately extra losses related to NSA handling are discussed (common/separate antennas, diplexing, solutions to harmonics/IMD)

- This is not reflecting how it will be captured in the spec but the method to share results

- REFSENS = -174dBm + NF + 10*log(RXBW) - Diversity gain + SNR + IM

- Diversity gain is 3dB for 2 RX antenna case

- Antenna correlation issues may be embedded in IM taking into account the frequency range, note that for test the 3dB apply as it is a conducted test.

- RXBW is transmission bandwidth per channel BW and SCS. See values in next page

- NF is maximum [9-10dB] for C-Bands (n77, n78)

- NF is maximum [9-11dB] for C-Bands (n79)

- IM is [0.5-2.5dB] for C-Bands (n77, n78, n79)

- For bellow 3GHz NF and IM may be different especially taking into account critical FDD cases and could be evaluated per band. Values are FSS

- SNR is FSS depending on reference signal and receiver demodulator performance. Assumption is QPSK with coding rate TBD.

We start with generic observations on UE REFSENS. We address RF architecture, receiver NF, IM, delta between 2RX and 4RX, and CA impact to REFSENS.

RF architecture:

In WCDMA and LTE, the assumed RF architecture consisted of RFIC including LNA and RF Front End (RF FE) that was often build from discrete components. During the past 10 years, implementation technology has taken huge steps forward. Modern transceiver comprises RF FE modules for main and diversity paths. Another essential development is LNA’s are inside the RF FE modules.

Receiver NF:

During the past 10 years, IC technology development and RF architecture evolution have made it possible to reach lower and lower NF for the active RF chain. i.e. from LNA onwards. Similarly, development in RF FE components has enable to design better passive RF switches and filters. These developments together have gradually pushed down the receiver total NF over the years.

Our view is that the “RFIC part” of the noise figure i.e the noise figure onwards from LNA input is maximum ~2.5-4.5dB depending on the frequency band and the width of the band. Assuming 1dB extra margin to compensate for instance non-perfect matching results as 3.5-5.5dB NF for “RFIC part”.

The contributors to RF FE loss are RF band filter, antenna switch, TX/RX switch (for TDD bands) and another loss such as coupler loss and trace loss. We note that trace losses are significantly reduced when LNA is very close to antenna.

For TDD bands, the RF FE loss is max around 2dB + band filter loss. Band filter loss varies a lot depending on the band; for say easy bands the loss is around 2dB or even less whilst for say difficult bands the loss can be even around 4-5dB or so. We note that band filter having 5dB loss is very seldom feasible.

In LTE, 9dB NF was assumed for the first LTE band, B1. Actually, 9dB might have even been adopted from WCDMA times. After B1, the applicable NF has been 9dB or more depending on for instance band filter IL and other passive noise contributors. We well recognize that the receiver NF and IM together define the REFSENS, hence on might argue whether bands having worse REFSENS than B1 have higher receiver NF and/or higher IM. Anyhow, it simplifies things to assume IM is fixed.

Summing “RFIC part” of receiver NF and RF FE loss together, the receiver NF is between 7.5-12.5dB.

Observation 1: Receiver noise figure is between 7.5-12.5dB for NR depending on the band assuming feasible implementation technology.

Implementation Margin (IM):

In addition to NF above, some implementation margin is allowed in LTE REFSENS requirements. While is it not explicitly said anywhere, we assume that the IM is 2.5dB for all LTE bands; in that case the NF for LTE bands varies band by band depending on the said difficulty of the band.

The implementation margin accounts for all kind of other losses not mentioned above, including MRC combining losses for both TDD and FDD bands and TX noise for FDD bands. In general, the developments in implementations have allowed practically needed IM to decrease. One major difference to LTE Rel8 times when principles for REFSENS analysis were established is significantly improved isolation in FDD filters. The assumption in Rel8 often was 45dB and today the isolation is 50dB or more in FDD filters. Improved isolation significantly reduces the impact of TX noise in FDD bands.

2.5dB IM is hefty, especially TDD bands because there is no TX noise present. We believe that from feasibility perspective two different IM’s, one for FDD bands and one for TDD bands would be justified. However, IM is used at least in ACS requirements in band agnostic manner. Hence, at the end of the day a single IM would work better unless some compromises in ACS are done an only one IM is used.

Observation 2: 2.5dB IM is very pessimistic, especially for TDD bands where TX noise is not present in REFSENS. It would be justified to have two IM’s, one for TDD bands and one for FDD bands. From ACS perspective, a single IM used for ACS for all bands would work better

Delta between 2RX and 4RX REFSENS:

Theoretically, the difference between 2RX and 4RX would be 3dB. In practice, the difference is a bit smaller because of for instance increased implementation complexity and relaxed implementation losses. Hence, deltas used in LTE which are 2.2dB for difficult bands and 2.7dB for less difficult bands are justified because they somehow resemble the practical difference.

We see chances to modify these deltas for NR depending on the 2RX REFSENS requirements. A simple way would be to define one single delta value applicable for all bands, that could be for instance 2dB or 3dB depending on the general outcome.

Observation 3: Delta between 2RX and 4RX is a bit smaller than theoretical 3dB. The simplest way would be to choose a single delta value between 2RX and 4RX that applies for all bands.

CA impact to REFSENS:

Inter-band CA causes impacts to achievable UE REFSENS because additional components are often needed, especially in single (main) antenna solutions. In LTE, the relaxations vary a lot band by band and in general are not aligned with each other. In perfect case, the relaxation would equal to net impact. This would best show the practical difference. However, this has never been the case because during R10/R11 it was generally accepted that existing REFSENS margin shall eat out at least some of the CA impact.

We think that it would not make much sense to simply copy the LTE RIB relaxations to NR because they are not at all aligned with each other. Basically, we see two alternatives; either to get rid of RIB or to find a way to define RIB in fair and consistent manner for all LTE-NR combos and for NR-NR in future. One option that should be considered is to have a fixed RIB for all bands that are supported in CA configuration. From our perspective, this would be viable if the development of the technology is appropriately accounted in NF and IM.

As a generic note, almost all the bands will be supported in some sort of CA configurations from day one. Thus, it would make sense to use this fact when deriving requirements. We have two alternatives for this one.

Observation 4: CA impacts REFSENS to some extent. Copying LTE RIB relaxations to NR is very questionable because they are not consistent between different bands.

Based on these observations we propose the following. Proposal 7b is not our preference but it is included as an alternative for 7a.

Proposal 1: Baseline NF for standalone NR shall be 8dB for bands below 3GHz whose filter IL is max 2dB in   ETC conditions

Proposal 2: Baseline NF for standalone NR shall be 8.5dB for bands between 3GHz and 4.2GHz whose filter IL is max 2dB in ETC conditions

Proposal 3: Baseline NF for standalone NR shall be 9.5dB for bands above 4.2GHz whose filter IL is max 2dB in ETC conditions

Proposal 4: NF shall be increased proportionally for bands whose filter IL in ETC conditions is above 2dB

Proposal 5: IM for FDD bands shall be 2.5dB and IM for TDD bands shall be 2dB

Proposal 6: Delta between 2RX and 4RX shall be 2dB for all bands

Proposal 7a: RIB of 0dB shall be used for all NR bands that are supported in any CA configuration

[Proposal 7b: RIB of 0.5dB shall be used for all NR bands that are supported in any CA configuration if baseline NF’s in proposals 1, 2, and 3 are improved to 7.5dB, 8.5dB, and 9dB, respectively]

Proposal 8: Proposals 1-7 are valid for both LTE re-farming bands and new NR bands

Proposal 9: Proposals 1-8 are agreed as a package

# 3	Conclusions

NR UE REFSENS was considered with the following observations and proposals.

Observation 1: Receiver noise figure is between 7.5-11.5dB for NR depending on the band assuming feasible implementation technology.

Observation 2: 2.5dB IM is very pessimistic, especially for TDD bands where TX noise is not present in REFSENS. It would be justified to have two IM’s, one for TDD bands and one for FDD bands. From ACS perspective, a single IM used for ACS for all bands would work better.

Observation 3: Delta between 2RX and 4RX is a bit smaller than theoretical 3dB. The simplest way would be to choose a single delta value between 2RX and 4RX that applies for all bands.

Observation 4: CA impacts REFSENS to some extent. Copying LTE RIB relaxations to NR is very questionable because they are not consistent between different bands.

Proposal 1: Baseline NF for standalone NR shall be 7.5dB for bands below 3GHz whose filter IL is max 2dB in   ETC conditions

Proposal 2: Baseline NF for standalone NR shall be 8dB for bands between 3GHz and 4.2GHz whose filter IL is max 2dB in ETC conditions

Proposal 3: Baseline NF for standalone NR shall be 9dB for bands above 4.2GHz whose filter IL is max 2dB in ETC conditions

Proposal 4: NF shall be increased proportionally for bands whose filter IL in ETC conditions is above 2dB

Proposal 5: IM for FDD bands shall be 2.5dB and IM for TDD bands shall be 2dB

Proposal 6: Delta between 2RX and 4RX shall be 2dB for all bands

Proposal 7a: RIB of 0dB shall be used for all NR bands that are supported in any CA configuration

[Proposal 7b: RIB of 0.5dB shall be used for all NR bands that are supported in any CA configuration if baseline NF’s in proposals 1, 2, and 3 are improved to 7.5dB, 8.5dB, and 9dB, respectively]

Proposal 8: Proposals 1-7 are valid for both LTE re-farming bands and new NR bands

Proposal 9: Proposals 1-8 are agreed as a package

# 4	References

[1] R4-1709149, “WF on SA and NSA UE REFSENS for sub 6GHz”, Skyworks Inc, Nokia, Qualcomm, Ericsson, Sony, RAN4#84