3GPP TSG-RAN WG4 Meeting NR ad-hoc #3	R4-1709703

Nagoya, Japan, 18 - 21 September, 2017

Agenda Item:	3.2.4

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Dense Urban Simulation Results at 45GHz for 55dBm EIRP Transportable Stations

Document for:	Discussion

# 1.	Introduction

Requirement for 55dBm EIRP transportable stations was discussed in RAN4#84, and the way forward was agreed in [1]. Further discussions have been held via RAN4 reflector, and some simulation parameters had been revised accordingly [2].

This contribution provides the dense urban simulation results at 45GHz for 55dBm EIRP transportable stations, according to the agreed simulation parameters in [1] as well as those revised by email discussions [2].

# 2.	Discussion

The dense urban simulation results at 45GHz for 55dBm EIRP (35dBm conducted transmit power) transportable stations are provided in Figures 1 to 5 below. The dense urban simulation results at 45GHz for 23dBm conducted transmit power UE are also provided for purpose of comparison. Here the currently agreed 16dB UE ACLR and 23dB BS ACS are used [3].

(a) 23dBm									(b) 35dBm

Figure 1: CDF of UE conducted transmit power

(a) 23dBm									(b) 35dBm

Figure 2: CDF of UL SINR of victim UE

It can be seen from the results in Figures 1 and 2 that around 60% of the 23dBm UE are transmitting with maximum power and cannot meet the UL SINR of 15dB, while the percentage is reduced to around 20% for the 35dBm transportable stations. Therefore, 55dBm EIRP transportable stations can be used effectively to extend the coverage of dense urban network at 45GHz.

(a) 23dBm									(b) 35dBm

Figure 3: CDF of UL interference-over-thermal-noise of victim BS

It can be seen from the results in Figure 3 that the 35dBm transportable stations decrease the interference-over-thermal-noise for around 5dB, because of the higher antenna height which reduces the required transmit power of the transportable stations in general. Note that it has already been agreed that there is no need to specify the mmWave NR BS receiver dynamic range requirement in the RAN4 specifications.

(a) 23dBm									(b) 35dBm

Figure 4: CDF of received blocking signal power of victim BS

It can be seen from the results in Figure 4 that again the 35dBm transportable stations decrease the received blocking signal power for around 5dB, because of the higher antenna height which reduces the required transmit power of the transportable stations in general.

(a) 23dBm									(b) 35dBm

Figure 5: CDF of wanted to blocking signal power ratio of victim BS

It can be seen from the results in Figure 5 that the 35dBm transportable stations decrease the wanted to blocking signal power ratio for around 10dB, because of the higher antenna height which reduces the required transmit power of the transportable stations in general.

The results in Figures 3 to 5 have shown that the 35dBm transportable stations (with the assumed UL power control parameter) would potentially relax the required BS dynamic range and in-band blocking performances.

On the other hand, the simulation results on average and 5%-tile throughput loss using different UE ACLR and BS ACS are summarized in Table 1 below. The results with 23dBm UE using the currently agreed UE ACLR and BS ACS are also provided for purpose of comparison.

Table 1: DL simulation results with 50MHz channel bandwidth for indoor

|  | 23dBm | 35dBm |
| --- | --- | --- |
| NR UE ACLR (dBc) | 16 | 16 |
| NR BS ACS (dBc) | 23 | 23 |
| Average throughput loss (%) | 0.22 | 0.09 |
| 5%-tile throughput loss (%) | NA | 0.17 |

It can be seen from the results in Table 1 that the 35dBm transportable stations have lower average throughput loss using the currently agreed UE ACLR and BS ACS comparing to the 23dBm UE, while 5%-tile throughput can only be achieved with the 35dBm transportable stations, because more than 5% of the 23dBm UE are out-of-coverage in the simulated scenario. Moreover, the 5%-tile throughput loss is sufficiently low using the currently agreed UE ACLR and BS ACS. Therefore, applying the currently agreed UE ACLR and BS ACS will provide sufficient UL coexistence performance for dense urban network at 45GHz with the 35dBm transportable stations.

# 3.	Conclusion and proposals

This contribution has provided the dense urban simulation results at 45GHz for 55dBm EIRP transportable stations, according to the agreed simulation parameters in the way forward as well as those revised by email discussions.

The simulation results have shown that 55dBm EIRP transportable stations can be used effectively to extend the coverage of dense urban network at 45GHz, with potential relaxations on the required BS dynamic range and in-band blocking performances, while applying the currently agreed UE ACLR and BS ACS will provide sufficient UL coexistence performance.

# References

[1]		R4-1708853, “WF on Requirement for 55dBm EIRP Transportable Stations”, Verizon, Ericsson, Qualcomm, Qorvo, Huawei, ZTE, Nokia.

[2]	R4-170xxxx, “WF on Requirement for 55dBm EIRP Transportable Stations”, Verizon.

[3]	R4-1706063, “WF on mmW ACLR and ACS”, Qualcomm Incorporated, Huawei.