3GPP TSG-RAN WG4 Meeting NR#3                                                       	R4-1709750

Nagoya, Japan, 18 - 21 September 2017

Source:	Sony

Title:	UE Power class for mmWave 28GHz

Agenda item:	3.3.3.2.2

Document for:	Approval

# Background

In RAN4 #84 in Berlin power class and was discussed [1], [2], [3], [4], [5], [6], [7], [8], [9] and [10]. A way forward on UE power class was approved in [11]:

- To send an LS to ask if RAN1 assumes independent power control for beam and power changes due to beam changes

- To agree Power class definition in next meeting considering below candidates according to the possible RAN1 answer

  - Peak EIRP over the sphere with minus tolerance

  - EIRP at CDF percentiles with minus tolerance

    - [5-20] %

    - [50] %

    - [80-90] %

  - Maximum allowed EIRP such as 43, 55 dBm (FCC limits)

- Maximum allowed TRP will be discussed in the next meeting

Interested companies shall provide input on feasible values for the above mentioned parameters to next meeting

This contribution is basically a resubmission of [9] discussing the EIRP values for mmWave 28 GHz band in accordance with the WF [11]. The result is based on simulations presented in detail in [9], [12] and [13].

In [11] it was agreed to normalize assumptions from different companies on Total Radiated Power (TRP). By doing so implementation specific questions on how to combine power amplifiers, feeder losses etc. are avoided. We should remember that this TRP is only used for normalizing beam forming (BF) assumptions and will not be specified.

# Antenna assumptions

Two different antenna systems have been studied, shown in Figure 1 and corresponding total scan pattern is shown in Figure 2:

- Two 2x2 patch matrixes. One on front of smart phone and one on rear side. Total 8 elements.

- Three 4x1 linear arrays. One on top and one each side of the side of the smartphone. Antenna elements are edge mounted single polarized dipole antennas. Total 12 elements.

Each array is assumed to have 5 discrete possible beam settings (lookup table phase shifter). Thus there are 10 possible beam directions for UE1 and 15 for UE2. Perfect polarization match between base station and UE antennas is assumed, i.e. the base station is assumed to have dual polarized receive antennas and thus possible to receive any polarization. Switched diversity is used between arrays. The studied antennas systems are summarized in Table 1.

Figure 1. Two patch antenna arrays (UE1) and three linear dipole arrays (UE2)

Figure 2. Total scan pattern for UE1 and UE2

| Parameter | Unit | UE1 | UE2 |
| --- | --- | --- | --- |
| Number of antenna elements in an array |  | 4 | 4 |
| Number of antenna arrays in the UE |  | 2 | 3 |
| Single element gain | dBi | 5 | 3 |
| Gain variation with frequency | dB | 2 | 1 |
| Array implementation loss | dB | 2 | 2 |
| Effective BF gain | dB | 6 | 6 |
| Max total power (TRP) | dBm | 21 | 21 |
| Pout EIRP (bore sight) | dBm | 30 | 29 |

Table 1. Summary of the studied antenna systems in UE1 and UE2

# Result

## EIRP

The simulated cumulative distribution function (CDF) for the two antenna configurations is shown in Figure 3, using +21dBm TRP. Note that the simulation is performed assuming perfect phase shifters without implementation loss and is done for a single frequency and thus no gain variation over the band is included. Antenna efficiency is, however, included (ohmic loss in the antenna).

Figure 3. EIRP for patch arrays (UE1) and dipole arrays (UE2). Simulated performance single array, switched diversity.

Corresponding values according to the format decided in [11] is listed in Table 2. Note that values are compensated for gain variation over frequency with 2dB for UE1 and 1dB for UE2 and phase shifter implementation loss with 2dB for both UE1 and UE2.

| EIRP |  | UE1 | UE2 |
| --- | --- | --- | --- |
| 5 %CDF | dBm | 17 | 15 |
| 10 %CDF | dBm | 18 | 19 |
| 20 %CDF | dBm | 19 | 23 |
| 50 %CDF | dBm | 22 | 25 |
| 80 %CDF | dBm | 26 | 26 |
| 90 %CDF | dBm | 27 | 26 |
| Peak EIRP over the sphere | dBm | 30 | 29 |

Table 2. EIRP CDF values according to the WF [11]. Note that values are compensated for gain variation over frequency and phase shifter implementation loss according to Table 1.

## CDF percentile points for EIRP specification

It could be observed that the value for the 90% percentile in the CDF aligns well with the calculated value in Table 1 when no “Gain variation with frequency” is taken into account. Note that the simulation is done with just a single frequency. Frequency response for different antennas was presented in [14]. The 90% percentile is therefore suggested as the “bore sight” checkpoint.

Proposal 1: EIRP shall at least be specified for the 90% percentile point.

A second CDF percentile point (10% - 20%) may be needed in order to secure the spatial coverage. The 20% percentile should be chosen over the 10% percentile point since 10% point will imply very high design challenge in a real implementation and will require longer test time which is studied further in [15]. Considering the challenge to test the lower percentile (20% or 10%) the need for specifying such a point shall be clearly motivated.

Proposal 2: If a second EIRP percentile shall be specified it shall be the 20% percentile point.

For a wise choice of 90% percentile and a possible 20% percentile there is no need for specifying additional points. This is due to the relatively small delta between the 90% and the 20% percentile points compared with the extra test effort.

Proposal 3: EIRP shall not be specified for more than two percentile points.

## Proposed EIRP values

Proposed EIRP values are listed in Table 2. It is recommended to account for different antenna implementations and therefore the most conservative value from the simulation (shown in Figure 2) is used. Note that it is compensated for “Gain variation with frequency” according to Table 1

| EIRP (90%) | 26 dBm |
| --- | --- |
| EIRP (20%) if needed | 19 dBm |

Table 3. Proposal for EIRP specification for 28GHz band.

# Conclusion

EIRP have been studied for two different antenna configurations, patch arrays and linear dipole arrays, for the 28GHz band. Numbers of EIRP has been proposed.

| EIRP (90%) | 26 dBm |
| --- | --- |
| EIRP (20%) if needed | 19 dBm |

In addition the following proposals are made:

Proposal 1: EIRP shall at least be specified for the 90% percentile point.

Proposal 2: If a second EIRP percentile shall be specified it shall be the 20% percentile point.

Proposal 3: EIRP shall not be specified for more than two percentile points.

# References

- R4-1707823	“mmWave power class definition”, Huawei, HiSilicon

- R4-1708618	“UE Power Class for mm-wave”, Intel Corporation

- R4-1707700	“NR Power Class Definition for cmW and mmW range”, InterDigital, Inc.

- R4-1708810	“NR UE power class for mmWave UE”,  LG Electronics France

- R4-1707818	“mmW UE power class”,	 MediaTek (Chengdu) Inc.

- R4-1707567	“EIRP assumption for mmWave”,	 NTT DOCOMO INC.

- R4-1707512	“CDF requirements for mmWave UEs”, NTT DOCOMO, INC.

- R4-1708609	“mmW Output Power”, Qualcomm Incorporated

- R4-1707331	“UE Power class for mmWave 28GHz”, Sony

- R4-1708143	“Power class definition of mmWave UE”, Sumitomo Elec. Industries, Ltd

- R4-1709146	“WF on power class and CDF for mmWave UEs”, NTT DOCOMO, INC.

- R4-1706562	“UE EIRP for mmWave 28GHz”, Sony, Ericsson

- R4-1707330	“On UE mmWave Antennas”, Sony

- R4-1704866	“Frequency Response of UE mmWave antennas for the 28GHz band”, Sony, Ericsson

- R4-1707315	“On UE EiRP measurements grid at > 6 GHz”, Sony Mobile