3GPP TSG-RAN WG4 Meeting NR#3                                                       	R4-1709752

Nagoya, Japan, 18 - 21 September 2017

Source:	Sony

Title:	UE reference sensitivity for mmWave 28GHz

Agenda item:	3.3.4.1.1

Document for:	Approval

# Background

This contribution is a resubmission of [2] formatted according to WF [1]

REFSENS for mmWave 28 GHz band will be studied using the antenna configurations presented in [1] as a basis. In this document smart phone UE is studied (eMBB use case) i.e. (almost) full sphere coverage is assumed.

# EIS calculation

In order to present the simulated antenna performance in a CDF format expressed in EIS with [dBm] scale the receiver performance need to be estimated. In [19] the below equation was proposed to determine PEIS:

PEIS(dBm) = Nt + 10log10(BW) + NF + SNR + Other losses+ margin - array gain + beam pointing loss – diversity gain

(Note: detailed explanation of respective term is found in [19])

In this document we will use simulated antenna data and “array gain”, “beam pointing loss” and “diversity gain” are included in the simulated “antenna gain”. Especially “diversity gain” is included since the simulation is based on maximum ratio combining (MRC). “Beam pointing loss” is included in the simulation since we use a finite set of possible beams and the simulation is set up with very dense sampling grid. Note that for a real conformance test measurement the number of sampling points need to be further studied in order not to introduce unacceptable measurement errors. “Other losses” accounts for radome losses (plastic losses). Radome losses are in the order of [1 – 2] dB. We have used 2dB radome losses which are added to the result.

Assuming a Nf = 10dB, channel BW = 100 MHz and a minimum SNR = -3dB (QPSK single RX) gives a “conducted” PRX

PRX= -174dBm+80dB+10dB-3dB+2dB= -85dBm

Note 1: In order to show expected nominal performance no implementation margin is added. This is otherwise suggested to be 3dB.

Note 2: PRX for other BW than 100 MHz is obtained by scaling.

## Maximum ratio combining (MRC)

According to WF [18] REFSENS shall be based on maximum ratio combining (MRC). For UE2 switched diversity is assumed where the two best beams from top/left pair or bottom/right pair dipole arrays are maximum ratio combined. For UE1 switched diversity where the best beam from front or backside patch panel is assumed. A fix diversity gain of 2.2 dB is added.

## Implementation margin

Example values for REFSENS should include some implementation margin. In the below calculation we use 3dB which accounts for phase shifter implementation loss and channel estimation implementation loss. Ohmic losses e.g. due to radome is included in “other losses” as described above. Gain variation over the frequency band was studied in [11] and the estimation is 2dB for the patch array and 1dB for the dipole array in the 28GHz band.

## Summary of hand calculations

Summary of hand calculations are found in Table 1

| Parameter | Unit | UE1 | UE2 |
| --- | --- | --- | --- |
| Transmission BW | MHz | 100 | 100 |
| NF | dB | 10 | 10 |
| Other losses | dB | 2 | 2 |
| Required SNR (QPSK single RX) | dB | -3 | -3 |
| “Conducted” sensitivity (single RX) | dBm | -85 | -85 |
| Diversity gain | dB | 2.2 | 1.5 |
| Number of antenna elements in an array |  | 4 | 4 |
| Effective BF gain | dB | 6 | 6 |
| Number of antenna arrays in the UE |  | 2 | 4 |
| Single element gain | dBi | 5 | 3 |
| Gain variation with frequency | dB | 2 | 1 |
| Implementation loss | dB | 3 | 3 |
| Sensitivity EIS (bore sight) | dBm | -93 | -91 |

Table 1. Summary of hand calculations 100MHz transmission BW

# Simulation setup

The same two hybrid beamforming cases as used in [9] are simulated, each having total 16 antenna ports and two digital ports. Beamforming use 4 element/ports for each polarization. The antennas investigated are examples of potential configurations:

- Two 2x2 dual polarized patch matrixes. One on front of smart phone and one on rear side. Total 16 ports.

- Four 4x1 linear arrays. One on each side of the side of the smartphone. Antenna elements are edge mounted single polarized dipole antennas. Total 16 ports

Each array is assumed to have 5 discrete possible beam settings (lookup table phase shifter). The simulation is done in free space. Perfect polarization match is assumed between base station and UE antennas (i.e. base station is assumed to have dual polarized antennas).

# Result

## Scan pattern

Maximum ratio combining scan pattern for patch arrays (UE1) is shown in Figure 1 and for dipole arrays (UE2) in Figure 2. For the maximum ratio combining two beams, one in each polarization, are received and combined in the baseband. This is similar to rank1 diversity in legacy LTE. For UE1 single polarization is simulated and a polarization gain of 2.2dB is added.

Figure 1. Maximum ratio combining pattern for the patch antenna arrays, 5 beams per array (UE 1).

Figure 2. Maximum ratio combining pattern for the linear dipole arrays, 5 beams per array (UE 2).

## EIS

The cumulative distribution function (CDF) for the two antenna configurations is shown in Figure 3, using the assumption made in chapter 2 regarding conducted sensitivity and 100MHz BW. Again it could be seen both antenna configurations have its pros and cons. Many more antenna configurations, including combinations of patches and dipoles, could be possible but in this document we want to show the general principles with text book antennas. Note that the CDF in Figure 3 shows simulated performance without any implementation margin or gain variation over frequency added.

## CDF percentile values for REFSENS specification

Firstly the spread over the CDF curve is considered. The value for the 10% percentile in the CDF (refers to almost boresight direction) would be -97dBm for UE1 and -95dBm for UE2. The values for the 80% percentile (refers to almost full sphere) are -89dBm for UE1 and -92dBm for UE2. Considering to small delta between the 10% and the 80% percentiles there is no need to define requirement for more than two percentiles.

Observation 1: There is no need to specify REFSENS for more than two percentile points.

The 10% (or 20%) percentile in the CDF refers to almost boresight direction and is essential for link budget estimations and need to be specified. In this sense we think 10% percentile is better to specify. There will probably also be an advantage in verification/test methods by specifying 10% percentile over specifying the 20% percentile.

The 80% (or 90%) percentile values will show performance close to omnidirectional coverage. Considering the challenge to test the lower percentile as discussed in [20] the 80% percentile should to be chosen over the 90% percentile.

Proposal 1: REFSENS shall at least be specified for the 10% percentile.

Proposal 2. If a second CDF percentile point is to be specified this shall be the 80% percentile.

Figure 3. EIS for patch arrays (UE1) and dipole arrays (UE2). Simulated performance maximum ratio combining configurations.

Example values for REFSENS should include some implementation margin. It is also recommended to account for different antenna implementations. Applying this to the simulation result above, where the most conservative number between the two antenna configurations has been used is shown in Table 2. A 3dB implementation margin and gain variation with frequency as discussed in section 2.2 has been added.

| Transmission BW | 100MHz |
| --- | --- |
| REFSENS (10%) | -91dBm |
| REFSENS (80%) if needed | -84dBm |

Table 2. Example of REFSENS specification for 28GHz band.

## HW implementation aspects

The above simulation is done for ideal conditions. In a real implementation the number of antennas and the freedom to place antennas in a UE could be limited. Implementation margins has to be further discussed.

# Conclusion

REFSENS have been studied for two different antenna configurations, patch arrays and linear dipole arrays, for the 28GHz band. Example numbers of REFSENS has been shown. The following observation and proposals are made:

Observation 1: There is no need to specify REFSENS for more than two percentile points.

Proposal 1: REFSENS shall at least be specified for the 10% percentile.

Proposal 2. If a second CDF percentile point is to be specified this shall be the 80% percentile.

# References

- R4-1709140	“WF on referense sensitivity for mmWave”, LG Electronics, Huawei, Sumitomo, Sony, Ericsson, Skyworks, Intel Corporation, KT, Media Tek

- R4-1707332	“UE reference sensitivity for mmWave 28GHz”, Sony

- R4-1708175	“NR UE REFSENS requirments”, LGE

- R4-1707828	“Preliminary smartphone EIS performance evaluation”, Huawei

- R4-1708542	“Reference sensitivity for mmW UE”, Qualcomm

- R4-1707511	“MSD for combinations including 3.5GHz, 4.5GHz, 28GHz”, NTT DoCoMo

- R4-1706506	“ mmWave UE REFSENS considerations & proposals”, Intel

- R4-1706563	“UE reference sensitivity for mmWave 28GHz”, Sony, Ericsson

- R4-1704864, 	“UE antenna array configuration for mmWave 28GHz”, Sony, Ericsson

- R4-1704868,	“Spatial multiplexing for mmWave 28GHz”, Sony, Ericsson

- R4-1704866,	“Frequency Response of UE mmWave antennas for the 28GHz band”, Sony, Ericsson

- R4-1704684	“On NR mmWave UE antenna reference architecture”, Intel Corporation

- R4-1704957	“Discussion on antenna reference architecture for mmWave”, LG Electronics Inc.

- R4-1705205	“mmW reference architecture”, Qualcomm Incorporated

- R4-1705202	“Reference sensitivity requirements for mmW UE”, Qualcomm Incorporated

- R4-1704937	“Receiver RF requirements for NR UE”, LG Electronics Inc.

- R4-1705223	“Further discussion on mmWave EIRP/EIS requirement following CDF approach”, Huawei, HiSilicon

- R4-1706067 	“WF on UE Reference Architecture”, Sony, Skyworks Solutions Inc., LG Electronics

- R4-168318	“On UE EIS Determination”, Qualcomm Incorporated

- R4-1707315	“On UE EiRP measurements grid at > 6 GHz”, Sony Mobile