3GPP TSG RAN WG4 NR#3		           			             		R4-1709756
18 - 21 September 2017, Nagoya, Japan

Source:		Rohde & Schwarz

Title:	Measurement uncertainties for non-traditional OTA measurements

Agenda Item:		3.6.2.2

Document for:	Discussion

# Introduction

MUs for OTA measurements have been discussed for traditional OTA measurements in the past. In [1] it has been discussed that the MU budgets for new OTA measurements need to be investigated and developed as well.

This paper takes a look at the UE requirements for NR in 38.803 [2], the currently defined TCs for LTE conducted testing in 36.521-1 [3] and the respective test system uncertainties, to evaluate if further MU elements might need to be considered for NR range 2 OTA testing.

# Discussion

## UE Tx Requirements

Based on the experience from LTE TCs and uncertainties defined in 36.521-1 [3] the list below is compiled to summarize possible additional MUs for Tx TCs, other than the traditional MUs for OTA tests that have previously been discussed in several papers.

| RF requirement | Additional MU to be considered | Comments |
| --- | --- | --- |
| TX Max power | No additional MUs |  |
| MPR | No additional MUs |  |
| A-MPR | No additional MUs |  |
| Pcmax | No additional MUs |  |
| Min. Power | No additional MUs |  |
| TX OFF Power | No additional MUs |  |
| ON/OFF Mask | No additional MUs |  |
| Power Control | Depending on the requirement definition, uncertainty of DL signal strength needs to be considered |  |
| Frequency error | MUs for frequency accuracy of test system need to be considered |  |
| EVM | MUs for EVM uncertainties/inherent test system EVM needs to be considered | For testing specific test points UL power level MU might be required |
| Carrier leakage | No additional MUs |  |
| In-band emissions | No additional MUs |  |
| Occupied BW | MUs for frequency accuracy/resolution of test system need to be considered |  |
| SEM | No additional MUs |  |
| ACLR | No additional MUs |  |
| General SE | No additional MUs |  |
| Additional SE | No additional MUs |  |
| UE to UE coexistence | No additional MUs |  |
| TX Intermodulation | Additional uncertainties for interferer power level required |  |
| Beam Correspondence | TC not existent in LTE | Uncertainties need to be investigated when requirements are being defined |

Table 1 MU elements for Tx test scenarios

## UE Rx Requirements

Based on the experience from LTE TCs and uncertainties defined in 36.521-1 [3] the list below is compiled to summarize possible additional MUs for Rx TCs, other than the traditional MUs for OTA tests that have previously been discussed in several papers.

| RF requirement | Additional MU to be considered | Comments |
| --- | --- | --- |
| REFSENS | No additional MUs |  |
| Maximum input level | No additional MUs |  |
| ACS | - Additional uncertainties for interferer power level required - ACLR impact of interferer on DL signal needs to be considered |  |
| Blocking requirements | - Additional uncertainties for interferer power level required - ACLR/Noise impact of interferer on DL signal needs to be considered |  |
| Spurious response | - Additional uncertainties for interferer power level required - ACLR/Noise impact of interferer on DL signal needs to be considered |  |
| RX intermodulation | - Additional uncertainties for interferer power level required |  |
| RX spurious emission | No additional MUs |  |
| Receiver image | Not tested in LTE |  |
| In-channel selectivity | TC not existent in LTE | Uncertainties need to be investigated when requirements are being defined |

Table 2 MU elements for Rx test scenarios

## General Observations

As can be seen from the tables above, several TCs require the consideration of additional MUs compared to the traditional MUs for OTA tests.

For example for the TCs that require additional interfering signals the uncertainties of this interfering signal need to be included in the uncertainty calculation for that particular TC. It is likely that for those scenarios the MU elements for the interferer uncertainty will be very similar to the MU elements for e.g. EIRP or EIS measurements. However, the exact requirements, e.g. spatial position of the interfering signal, for the TCs will be important to determine the MU elements.

Also it can be noted that some Tx TCs will likely require different approach to calculate the MUs for those TCs, since for example EVM or frequency error TCs will not necessarily depend on an exact power level of the UE, but other factors from the test system like frequency accuracy/stability or inherent EVM will become important.

# Summary

In this contribution we have taken a look at the currently defined test scenarios for Tx and Rx tests in NR and evaluated based on the experiences from LTE TCs which additional MU elements need to be taken into account for calculating the MUs for the individual TCs.

To complete the MU analysis for all the Tx and Rx tests in mm-wave NR (range 2) OTA testing, several new MU elements must be considered to be able to accurately test all the requirements that will be defined in the specification.

This includes new MUs for additional interferers, which might be similar to the existing MU elements for current transmit/receive tests, as well as new MU elements that are needed for tests like EVM or frequency error, which might be independent of the UE output power.

# References

[1]	R4-1707756: WF on NR MU and test tolerance

[3]	3GPP TR 38.803 Version 14.1.0

[3]	3GPP TS 36.521-1 Version 14.3.0