3GPP TSG RAN WG4 NR#3		           			             		R4-1709772
18 - 21 September 2017, Nagoya, Japan

Source:		Rohde & Schwarz

Title:	Combining uncertainty contributions in the derivation of test system uncertainty

Agenda Item:		3.6.2.2

Document for:	Approval

# Introduction

During the last RAN4 meeting, a question was raised how to combine MU elements, specifically whether combination of MU values, mathematically calculated with the root sum square (RSS) method, should take into account the dB or linear values [1, 2]. As the combined MU can yield significant differences, it is important to harmonize on the approach on how to combine MU elements [3, 4].

Traditionally, OTA test plans [5, 6, 7] have used the dB values for the RSS calculations. This contribution aims to analyze the impact of the applied methods on the total uncertainty and to propose which method shall be applied for the calculation of measurement uncertainties. Specifically, this paper investigates the combination of uncertainties expressed in dB, linear power units, and linear voltage units.

# Discussion

2.1 Background

In TS 36.521-1, Annex F.1.2, root sum square with weighting factors is applied on dB values. Canonical examples are the test system uncertainties of TC 6.7 Transmit intermodulation (see Figure 1) and of TC 7.5 Adjacent Channel Selectivity (see Figure 2).

Figure 1: Excerpt of the uncertainty table in TS 36.521-1

Figure 2: Excerpt of the uncertainty table in TS 36.521-1

Another more general reference is [8] (specifically Section 5.2 entitled Combining individual standard uncertainties in different units), where the case of combining uncertainties for cascaded RF measurements is considered. The document states:

“To use formula 5.1 (note: formula 5.1 corresponds to the typical RSS equation) for standard uncertainties of individual contributions which combine by multiplication, logarithmic terms only i.e. dB should be used as they can then be combined by addition. This is essential for the RSS combination to be valid where uncertainty multiplication occurs. This is the case where gains and/or losses (i.e. attenuators, amplifiers, antennas, etc.) are involved as well as under mismatch conditions where modules (i.e. attenuators, cables, RF measuring instruments, etc.) are interconnected in RF measurements. If all parameters and their associated standard uncertainties in a measurement are in the same unit and combine by addition, the RSS method can be applied directly.”

Observation 1: For uncorrelated quantities RSS with appropriate constant weighting factors is applied on individual contributions which are expressed in dB.

Observation 2: Uncertainties due to systematic effects are added arithmetically to the RSS value of uncorrelated contributions.

Proposal 1: The method to combine uncertainties is root square sum of values expressed in dB for quantities which are represented by a product of individual quantities expressed in linear units.

# Summary

The combining of uncertainty contributions in the derivation of test system uncertainty has been discussed.

Observation 1: For uncorrelated quantities RSS with appropriate constant weighting factors is applied on individual contributions which are expressed in dB.

Observation 2: Uncertainties due to systematic effects are added arithmetically to the RSS value of uncorrelated contributions.

Proposal 1: The method to combine uncertainties is root square sum of values expressed in dB for quantities which are represented by a product of individual quantities expressed in linear units.

# References

[1]	R4-1707222, Measurement uncertainty values of EIRP for NR OTA, Anritsu, 3GPP TSG-RAN4 Meeting #84, Berlin, Germany, August 2017

[2]	R4-1707756, WF on NR MU and test tolerance, CATR, Intel Corporation, Rohde & Schwarz, Anritsu, Fraunhofer HHI, 3GPP TSG-RAN WG4 Meeting #84, Berlin, Germany, August, 2017

[2]	R4-1708551, MU analysis for RF baseline, Keysight Technologies, 3GPP TSG-RAN WG4 Meeting #84, Berlin, Germany, August 2017

[3]	R4-1707220, Views on MU contribution “Offset of DUT phase centre from axis of rotation” for NR OTA, Anritsu, 3GPP TSG-RAN4 Meeting #84, August 2017

[5]	CTIA, “Test Plan for Wireless Device Over-the-Air Performance” Version 3.7, June 2017

[6]	3GPP TS 34.114, “User Equipment (UE) / Mobile Station (MS) Over The Air (OTA) antenna performance; Conformance testing”

[7] 	3GPP TR 37.977 v14.4.0 (2017-06) Verification of radiated multi-antenna reception performance of User Equipment (UE)

[8]	ETSI TR 102 273-1-1 V1.2.1 (2001-12)