3GPP TSG-RAN WG4-NR#3	R4- 1709381
Nagoya, Japan, 18 – 21 September 2017

Agenda Item:		3.6.1

Title:			Measurements in Non-Anechoic Environments

Source:		Fraunhofer HHI, Fraunhofer IIS

Document for:	Discussion

# Introduction

In the last Testability Ad-hoc Meeting, an agreement was made to “adopt [the] black box approach” in preference to the “white box approach” [1]. 5G NR user equipment (UE) manufacturers are therefore not obliged to provide information that defines the centre of radiation reference points (CORRP [2]) associated with the antennas and array antennas used in their devices. In the same meeting, certain aspects of over-the air (OTA) measurement uncertainty (MU) associated with the black box approach were noted [3][4][5] and a way forward was agreed for their further investigation [6].

It was proposed in [7] to determine the range length for the far-field baseline according to the maximum dimension of the DUT. Consequently, the physical dimensions of the far-field anechoic measurement chambers needed to accurately test and measure “black box” devices might impose challenges on companies whose building space is limited. Similarly, economic factors should also be considered.

In view of the above concerns, this contribution introduces techniques that allow OTA measurements to be made in non-anechoic environments. This not only offers an alternative solution to the far-field anechoic chamber demands created by the black box approach, but also provides the means to create virtual electromagnetic environments for additional test and measurement purposes.

Observation 1: Practical and implementational issues associated with the black box approach adoption are expected. Therefore, alternative testing methods/setups are proposed to solve the black-box approach complications, e.g., the far-field determination criterion.

# Rationale

The requirement for an anechoic chamber can be removed by using electronic methods to reduce the effects of multipath propagation that would otherwise distort the signals being measured [8] [9]. Furthermore, since such electronic hardware is configurable through software control, it can also be used to provide a virtual electromagnetic environment (VEE) that can emulate a wide variety of propagation landscapes [9] [10]. A variety of VEEs are conceivable, ranging from the simple single user, interference-free, line-of-sight (LOS) channel to complex interference scenarios in which several dimensions – such as frequency, time, direction (space), and polarization – are required to operate dynamically and in a multiuser configuration.

In Figs. 1A, 1B and 1C, three radio communication propagation environments are shown. Common to all environments is the existence of multiple base stations and mobile terminals, some of which will provide “wanted” communication signals, whereas others can be considered as sources of interference that create “unwanted” signals. Typically, it is necessary to determine the performance of the radio communications system in scenarios such as those illustrated in the figures, and practical, reliable and economic measurement systems and methods are therefore sought [11]. As suggested by the coloured circle in Figs. 1A, 1B and 1C, a system comprised of an arrangement of multiple antenna probes provides a potential solution to this challenge.

Observation 2: A virtual electromagnetic environment (VEE) can emulate a wide variety of propagation landscapes.

- (b)

(c)

Figure 1 Simplified pictorial representations of multipath propagation environments for a) pedestrian, b) vehicular and c) high-speed train scenarios.

In order to create an OTA-in-VEE system, a multiprobe configuration is used in which the number of probes and their arrangement is based on factors that include: the dimensions of the device under test (DUT); the required spatial sampling resolution and; the frequency of operation. Such a system is enclosed within a non-anechoic chamber as shown in Figs. 2A and 2B in which the radiation absorbing material has been removed and its function has been replaced by signal processing electronics. The latter can be realized in the form of a signal conditioning unit (SCU) for each probe that comprises the multiprobe system. These can be arranged for reciprocal reception and transmission denoted by uplink and downlink in Figs. 2A and Fig. 2B, respectively. Also shown is the use of an optional shielded room which offers improved electromagnetic compatibility and susceptibility to and from other equipment. In non-anechoic environments, cost savings are offered through the use of signal processing techniques where the restrictions of test-setup dimensions are practically avoided.

- Observation 3: A non-anechoic OTA test and measurement approach is potentially less costly than state-of-the-art methods.

Figure 2: A multiprobe antenna measurement system enclosed inside a shielded room operating (left) a) from DUT to measurement equipment (uplink) and (right) b) from measurement equipment to DUT (downlink).

With reference to the propagation environments presented in Figs. 1A, 1B and 1C, the means necessary to create a VEE comprised of both wanted and interfering signals is illustrated in Fig. 3. When the measurement probes are arranged in two dimensions – such as a circle, semicircle, arc or line – rotation of the UE about its axis may be helpful to perform measurements over a full sphere or hemisphere of space as also shown in Fig. 3.

Figure 3: A multiprobe OTA measurement system in a non-anechoic environment comprised of signal conditioning units for both uplink and downlink measurements.

An alternative arrangement, for which no mechanical rotation of the UE is needed and thus offers a considerable improvement in measurement speed, is shown in Fig. 4. Here the measurement probes are positioned at the (sixty) vertices of a truncated icosahedron (“Buckyball”) which offers the geometrical property of equidistant probe spacing over the surface of an imaginary sphere [8]. In practice, an SCU would be associated with either the full set or a subset of the sixty probes.

Figure 4: The truncated icosahedron offers one form of geometric realization for an antenna measurement system comprised of signal conditioning units at each vertex end (node) (after [8]).

The aforementioned combination of signal processing techniques, including the beneficial use of equalization, removes the need for an anechoic chamber. This allows flexible, adaptable and dynamic test and measurement systems to be constructed that offer both improved measurement speed and measurement throughput. To guard against changes of the measurement environment, the auxiliary observation system of Fig. 5 can be used to predict and notify the measurement system of perturbations that may otherwise affect its accuracy.

Figure 5 Non-anechoic measurement setup where the auxiliary predicts, notifies, and potentially corrects for multipath changes right after a pre-calibration routine was executed.

An example of an OTA measurement procedure, conducted in a non-anechoic environment, is presented in the flowchart of Fig. 6. The placement of the auxiliary array in such a setting plays an important role, which may allow multipath channel components to be corrected or compensated.

Figure 6 OTA measurement procedure utilized by the proposed method to cope with non-anechoic (multipath) environment changes.

# Conclusion

The aforementioned combination of signal processing techniques, including the beneficial use of equalization, removes the need for an anechoic chamber. This allows flexible, adaptable and dynamic test and measurement systems to be constructed that offer both improved measurement speed and measurement throughput. Through software control, such systems can be configured to create virtual electromagnetic environments for additional test and measurement purposes.

Observation 1: Practical and implementational issues associated with the black box approach adoption are expected. Therefore, alternative testing methods/setups are proposed to solve the black-box approach complications, e.g., the far-field determination criterion.

Observation 2: A virtual electromagnetic environment (VEE) can emulate a wide variety of propagation landscapes.

- Observation 3: A non-anechoic OTA test and measurement approach is potentially less costly than state-of-the-art methods.

- Proposal 1: Consideration of non-anechoic OTA measurement techniques as an alternative solution to measuring DUTs with limited manufacturer declared information.

# References

- R4-1708991, “Testability Ad-hoc Meeting Notes”, Chaired by Intel, 3GPP RAN4 #84, Berlin, Germany, August 2017.

- R4-1706617, “Center of Radiation Reference Point – Reference Definition for OTA Measurements of Phased Array Beamforming Patterns”, Fraunhofer HHI and Keysight, 3GPP RAN4 NR AH#2, Qingdao, China, June 2017.

- R4-1706561, “Measurement Uncertainty Contributions for mmWave OTA”, Anritsu, 3GPP RAN4 NR AH#2, Qingdao, China, June 2017.

- R4-1707220, “Views on MU contribution “Offset of DUT phase centre from axis of rotation” for NR OTA”, Anritsu, 3GPP RAN4 #84, Berlin, Germany, August 2017.

- R4-1707222, “Measurement uncertainty values of EIRP for NR OTA”, Anritsu, 3GPP RAN4 #84, Berlin, Germany, August 2017.

- R4-1707756, “WF on NR MU and test tolerance”, CATR, Intel Corporation, Rohde & Schwarz, Anritsu, Fraunhofer HHI, 3GPP TSG-RAN WG4 Meeting #84, Berlin, August 2017.

- R4-1708553, “Far field definition and proposal for alternate RF baseline with deterministic antenna array positioning”, Keysight Technologies, 3GPP TSG-RAN WG4 Meeting #84, Berlin, August 2017.

- P. S. H. Leather, J. D. Parsons, “Equalization for antenna pattern measurements: established technique — new application”, IEEE Antennas Propag. Mag., 2003, 45, (2), pp. 154–161.

- J. D. Parsons, P. S. H. Leather, “Correlation processing and its use to mitigate the effects of multipath and other interfering signals in antenna ranges”, IET Microwaves, Antennas and Propagation, 2017, 11, (9), pp. 1267-1273.

- R. K. Sharma, W. Kotterman, M. H. Landman et al., “Over-the-Air Testing of Cognitive Radio Nodes in a Virtual Electromagnetic Environment,” International Journal of Antennas and Propagation, vol. vol. 2013, no. Article ID 945283, pp. 1-16, 2013.

- C. Schirmer, M. Lorenz, W. A. T. Kotterman, R. Perthold, M. H. Landmann and G. Del Galdo, "MIMO over-the-air testing for electrically large objects in non-anechoic environments," 2016 10th European Conference on Antennas and Propagation (EuCAP), Davos, 2016, pp. 1-6.

- 3GPP TS 37.320, “Radio Measurement collection for minimization of drive test (MDT)”.