3GPP TSG-RAN WG4 NR#3	R4-1709817

# Nagoya, Japan, 18 – 21 September, 2017

Source:	Nokia, Nokia Shanghai Bell

Title:	TP to TS 38.133 v 0.2.0: Number of supported NR carriers

Agenda Item:	3.5.2

Document for:	Approval

# Introduction

A text proposal to specify the number of supported NR carriers to TS 38.133 version 0.0.1. It is proposed to support monitoring of 12 NR inter-frequency carriers depending on UE capability.

# TP to TS 38.133 v 0.2.0

Section 9 in TS 38.133 is updated as follows:

<<<Start of Changes>>>

##### 9.1.x	Monitoring of multiple layers using gaps

When monitoring of multiple inter-frequency NR layers using gaps (or without using gaps provided the UE supports such capability) is configured, the UE shall be capable of performing one measurement of the configured measurement type (SS-RSRP, SS-RSRQ and SS-SINR etc.) of detected cells on all the layers

The effective number of frequencies excluding the frequencies of the PSCell or SCell being monitored is Nfreq, which is defined as:

Nfreq = Nfreq_NR

where

Nfreq_NR is the number of NR carriers being monitored

9.1.x.1	Maximum allowed layers for multiple monitoring

The UE shall be capable of monitoring at least per RAT group:

-	Depending on UE capability, 12 NR inter-frequency carriers

<<<End of Changes>>>