3GPP TSG-WG RAN4 Meeting NR ad-hoc #3	R4-1709840

# Nagoya, Japan, 18nd – 21st September, 2017

Source:	Nokia, Nokia Shanghai Bell

Title:	On BS channel bandwidth set

Agenda item:		3.2.1.3

Document for:		Approval

# 1	Introduction

During the RAN4#84 meeting, the way forward on BS channel bandwidth set was agreed in [1]. It was agreed to analyse the impact and complexity of specifying additional channel bandwidths as well as identify which BS requirements could be bandwidth independent. As NR BS requirements are not finalized yet, this document provides information on relevant LTE BS requirements and analyse which requirements have potential to be bandwidth independent (or scalable) for NR.

# 2	Discussion

Transmitter RF requirements

Table 1 (first column) shows LTE Tx requirements and test aspects (36.104/141) which are channel bandwidth dependent. In the second column NR requirement expectations as well as additional effort to define requirement for all channel bandwidths (in case of channel bandwidth dependent requirement) have been provided.

Table 1. Channel bandwidth dependent Tx requirements

| LTE requirement | NR comments |
| --- | --- |
| Regional requirement for rated output power | No major effort expected, depends on regulatory requirements |
| Total power dynamic range | No major effort expected for additional channel BWs |
| ACLR (channel BW dependent but scalable) | No major effort expected for additional channel BWs as requirement will be scalable as for LTE |
| Operating band unwanted emissions (different requirement for 1.4 and 3MHz channel bandwidths for Wide Area BS) | Channel bandwidth independent requirement |
| Regional operating band unwanted emissions | No major effort expected, depends on regulatory requirements |
| Transmitter intermodulation (channel BW dependent but scalable) | No major effort expected for additional channel BWs as it is expected requirement will be scalable as for LTE |
| Test models | Major effort to define several test models for each channel bandwidth |

Observation 1: No major effort is foreseen (from Tx RF requirements point of view) in order to introduce additional channel bandwidths. However, we see major effort to define several test models for each channel bandwidth.

Receiver RF requirements

Table 2 (first column) shows LTE Rx requirements (36.104) which are channel bandwidth dependent. In the second column NR requirement expectations as well as additional effort to define requirement for all channel bandwidths (in case of channel bandwidth dependent requirement) have been provided.

Table 1. Channel bandwidth dependent Tx requirements

| LTE requirement | NR comments |
| --- | --- |
| Reference sensitivity | Major effort expected, derive reference sensitivity level for each channel BW (LL simulation results) |
| Dynamic range | Major effort expected, derive reference sensitivity level for each channel BW (LL simulation results) |
| In-channel selectivity | Major effort expected, derive reference sensitivity level for each channel BW (LL simulation results) |
| Blocking and narrowband blocking | Major effort expected, allowed desensitization to be agreed for each channel bandwidth |
| ACS | Major effort expected, allowed desensitization to be agreed for each channel bandwidth |
| Blocking | Major effort expected, allowed desensitization to be agreed for each channel bandwidth |
| Rx intermodulation and narrowband intermodulation | Major effort expected, allowed desensitization to be agreed for each channel bandwidth |

Observation 2: Major effort is foreseen (from Rx RF requirements point of view) in order to introduce additional channel bandwidths.

Demodulation requirements

BS demodulation work for NR has not started yet. However, we expect significant workload with respect to define relevant performance requirements for each channel bandwidth. As a reference, there is around 170 performance requirements defined in 36.104 for 10 MHz channel bandwidth (each requirement is defined on the basis of LL simulation results).

Observation 3: Major effort is foreseen (from BS demodulation requirements point of view) in order to introduce additional channel bandwidths.

# 3	Conclusion

Taking into account observations above, it is proposed to agree on proposal 1 below:

Proposal 1: It is proposed to consider limited set of channel bandwidths (essential for initial NR deployments only) to be defined in first version of BS specification.

# 4	References

- R4-1708847	Way forward on BS channel Bandwidth (CBW) Set; Vodafone, Huawei, Ericsson