3GPP TSG-RAN WG4 Meeting NR AH-3	    R4-1709877

Nagoya, Japan, 18 – 21 Sep., 2017

Agenda Item:	3.5.6.2.1

Source: 	Samsung

Title: 	Cell detection evaluation results

Document for:	Discussion

# Introduction

In RAN4 #NR Ad Hoc-2 meeting, both link level and system level simulations for RRM were discussed, and simulation assumptions were agreed in [1]-[3]. In RAN4#84 meeting, companies provided some initial simulation results for PSS/SSS detection. As observed, there existed some differences between the submitted simulation results. In addition, the simulation assumptions were updated in [4].

In this contribution, we provide the updated simulation results of 90%-tile cell search delay and one shot detection probability based on PSS and SSS according to the agreed assumptions in [4].

# Simulation Assumptions

The detailed simulation assumptions are listed in Table A.1~4 in Annex A [4]. Since PBCH occupies 288 subcarriers, the FFT size is chosen to be 512. For PSS detection, time-domain correlation is performed, with false alarm probability set to be 0.01%. For SSS detection, correlation is done in the frequency domain. Besides, correlation is done with received signals from 2 receive antennas seperately, then the maximum correlation value among the two is selected.

# Simulation results

## SCS 15 kHz

Table 3.1-1: 90%-tile cell detection delay (number of SS burst periodicities), 15 kHz

| SINR (dB) Channel | Case 1 |  |  | Case 2 |  |  | Case 3 |  |  | Case 4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | -8 | -7 | -6 | -8 | -7 | -6 | -8 | -7 | -6 | -8 | -7 | -6 |
| AWGN | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| EPA5 | 1 | 1 | 1 | 2 | 1 | 1 | 2 | 1 | 1 | 2 | 1 | 1 |
| ETU30 | 2 | 1 | 1 | 1 | 1 | 1 | 2 | 1 | 1 | 2 | 1 | 1 |
| TDL-A | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 1 | 1 | 2 | 1 | 1 |
| TDL-B | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| TDL-C | 1 | 1 | 1 | 2 | 1 | 1 | 2 | 1 | 1 | 2 | 1 | 1 |
| TDL-D | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

Table 3.1-2: One shot detection probability, 15 kHz

| SINR (dB) Channel | Case 1 |  |  | Case 2 |  |  | Case 3 |  |  | Case 4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | -8 | -7 | -6 | -8 | -7 | -6 | -8 | -7 | -6 | -8 | -7 | -6 |
| AWGN | 100% | 100% | 100% | 100% | 100% | 100% | 99.5% | 100% | 100% | 99.6% | 100% | 100% |
| EPA5 | 90.1% | 93.3% | 96% | 89.6% | 93.4% | 95.8% | 89.6% | 93.2% | 95.8% | 89.5% | 93.4% | 96% |
| ETU30 | 89.3% | 94.1% | 97.5% | 92.2% | 95.8% | 98% | 88.1% | 93.6% | 96.9% | 87.6% | 93.3% | 96.5% |
| TDL-A | 91.2% | 94.7% | 97% | 90.8% | 94.2% | 96.7% | 89.5% | 93.9% | 96.3% | 89.5% | 93.4% | 96.3% |
| TDL-B | 91.8% | 95% | 97.2% | 91.1% | 94.6% | 96.7% | 90.1% | 94% | 96.5% | 90.3% | 93.8% | 96.3% |
| TDL-C | 90% | 93.8% | 96.3% | 89.5% | 93.3% | 96% | 89.5% | 93.7% | 96.1% | 89.6% | 93.4% | 95.9% |
| TDL-D | 98.9% | 99.8% | 100% | 100% | 100% | 100% | 97.9% | 99.3% | 99.9% | 97.8% | 99.3% | 99.9% |

Table 3.1-1 shows the simulation results of 90%-tile cell detection delay under AWGN, EPA5, ETU30, TDL-A, TDL-B, TDL-C, and TDL-D channels with subcarrier spacing 15 kHz. Table 3.1-2 is the one shot cell detection probability, and since the burst set periodicity is 5 ms, one shot cooresponds to 5 ms.

From Table 3.1-1, it can be seen that as SINR increases, the cell detection delay decreases. For AWGN channel and TDL-D channel, since there is strong LOS path, 1 shot is enough to meet the 90% detection probability requirement for all cases and SINR conditions. For EPA5, ETU30, TDL-A, TDL-B, and TDL-C channels, the detection delay becomes larger under low SINRs, but, in general, 2 shots are enough for all channels even with the lowest SINR.

## SCS 30 kHz

Table 3.2-1: 90%-tile cell detection delay (number of SS burst periodicities), 30 kHz

| SINR (dB) Channel | Case 1 |  |  | Case 2 |  |  | Case 3 |  |  | Case 4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | -8 | -7 | -6 | -8 | -7 | -6 | -8 | -7 | -6 | -8 | -7 | -6 |
| AWGN | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| EPA5 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 1 | 1 | 2 | 1 | 1 |
| ETU30 | 2 | 1 | 1 | 1 | 1 | 1 | 2 | 1 | 1 | 2 | 1 | 1 |
| TDL-A | 1 | 1 | 1 | 2 | 1 | 1 | 2 | 1 | 1 | 2 | 1 | 1 |
| TDL-B | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| TDL-C | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 1 | 1 | 2 | 1 | 1 |
| TDL-D | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

Table 3.2-2: One shot detection probability, 30 kHz

| SINR (dB) Channel | Case 1 |  |  | Case 2 |  |  | Case 3 |  |  | Case 4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | -8 | -7 | -6 | -8 | -7 | -6 | -8 | -7 | -6 | -8 | -7 | -6 |
| AWGN | 100% | 100% | 100% | 100% | 100% | 100% | 99.7% | 100% | 100% | 99.6% | 100% | 100% |
| EPA5 | 90% | 94.2% | 96.5% | 90.1% | 93.7% | 96% | 89.4% | 93.4% | 95.7% | 89.6% | 93.4% | 95.8% |
| ETU30 | 86.9% | 93.4% | 97.3% | 91.5% | 95.7% | 98.2% | 86.6% | 93.5% | 97.3% | 86.8% | 93.4% | 97.2% |
| TDL-A | 90.3% | 94.4% | 96.9% | 89.9% | 94% | 96.5% | 89.4% | 93.8% | 96.5% | 89.6% | 93.9% | 96.6% |
| TDL-B | 91% | 95.3% | 97.6% | 90.9% | 94.9% | 97.6% | 90.3% | 94.4% | 97.1% | 90.7% | 94.7% | 97.4% |
| TDL-C | 90.5% | 94.2% | 96.5% | 90.2% | 93.9% | 96.6% | 89.8% | 93.7% | 96.2% | 89.7% | 93.9% | 96.7% |
| TDL-D | 99.3% | 100% | 100% | 100% | 100% | 100% | 98% | 99.4% | 99.9% | 97.7% | 99.3% | 99.8% |

Table 3.2-1 and Table 3.2-2 are the detection delay and one shot probability for 30 kHz SCS, respectively. Generally, the results are quite similar as those of 15 kHz SCS, with only some slight difference for the one shot detection probability. To achieve 90% cell detection delay for all channels, cases, and SINRs conditions, 2 shots are enough.

## SCS 120 kHz

Table 3.3-1: 90%-tile cell detection delay (number of SS burst periodicities), 120 kHz

| SINR (dB) Channel | Case 1 |  |  | Case 2 |  |  | Case 3 |  |  | Case 4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | -8 | -7 | -6 | -8 | -7 | -6 | -8 | -7 | -6 | -8 | -7 | -6 |
| AWGN | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| EPA5 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| ETU30 | 3 | 2 | 1 | 2 | 1 | 1 | 3 | 2 | 1 | 3 | 2 | 1 |
| TDL-A | 2 | 1 | 1 | 2 | 1 | 1 | 2 | 1 | 1 | 2 | 1 | 1 |
| TDL-B | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 1 | 1 |
| TDL-C | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 1 | 1 | 2 | 1 | 1 |
| TDL-D | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

Table 3.3-2: One shot detection probability, 120 kHz

| SINR (dB) Channel | Case 1 |  |  | Case 2 |  |  | Case 3 |  |  | Case 4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | -8 | -7 | -6 | -8 | -7 | -6 | -8 | -7 | -6 | -8 | -7 | -6 |
| AWGN | 100% | 100% | 100% | 100% | 100% | 100% | 99.6% | 100% | 100% | 99.6% | 100% | 100% |
| EPA5 | 92.8% | 96.7% | 98.5% | 92.4% | 96.6% | 98.6% | 91% | 95.8% | 98.2% | 90.9% | 95.7% | 98% |
| ETU30 | 77% | 89.4% | 96.5% | 88.4% | 94.4% | 98% | 74.3% | 87.2% | 94.8% | 73.9% | 87.1% | 94.9% |
| TDL-A | 89.4% | 93.8% | 96.6% | 88.5% | 93.3% | 96.5% | 89.4% | 93.5% | 96.3% | 89.5% | 93.8% | 96.4% |
| TDL-B | 91.2% | 95.1% | 97.4% | 90.6% | 94.9% | 97.5% | 90.2% | 94.3% | 97% | 89.9% | 94.4% | 97.1% |
| TDL-C | 90 % | 94.1% | 96.8% | 90.2% | 94% | 96.6% | 89.3% | 93.8% | 96.4% | 89.7% | 93.5% | 96.2% |
| TDL-D | 99.2% | 99.9% | 100% | 100% | 100% | 100% | 98.2% | 99.5% | 99.9% | 98.3% | 99.6% | 100% |

Table 3.3-1 and Table 3.3-2 are the detection delay and one-shot probability for 120 kHz SCS, respectively. As observed, for most simulation scenarios, 1 shot is enough to meet the 90% detection probability requirement. For ETU30 channel, the detection performance is worse than for other channels. The reason is due to the fact that ETU30 has longer path delay (maximum 5000 ns) than the normal CP length (576 ns), hence, inter-symbol interference arises and reduces the performance. To meet the requirements for all scenarios, a maximum of 3 shots are necessary.

## SCS 240 kHz

Table 3.4-1: 90%-tile cell detection delay (number of SS burst periodicities), 240 kHz

| SINR (dB) Channel | Case 1 |  |  | Case 2 |  |  | Case 3 |  |  | Case 4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | -8 | -7 | -6 | -8 | -7 | -6 | -8 | -7 | -6 | -8 | -7 | -6 |
| AWGN | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| EPA5 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 1 | 1 | 2 | 1 | 1 |
| ETU30 | 3 | 2 | 1 | 2 | 1 | 1 | 3 | 2 | 1 | 3 | 2 | 1 |
| TDL-A | 2 | 1 | 1 | 2 | 1 | 1 | 2 | 1 | 1 | 2 | 1 | 1 |
| TDL-B | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 1 | 1 | 2 | 1 | 1 |
| TDL-C | 2 | 1 | 1 | 1 | 1 | 1 | 2 | 1 | 1 | 2 | 1 | 1 |
| TDL-D | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

Table 3.4-2: One shot detection probability, 240 kHz

| SINR (dB) Channel | Case 1 |  |  | Case 2 |  |  | Case 3 |  |  | Case 4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | -8 | -7 | -6 | -8 | -7 | -6 | -8 | -7 | -6 | -8 | -7 | -6 |
| AWGN | 100% | 100% | 100% | 100% | 100% | 100% | 99.8% | 100% | 100% | 99.8% | 100% | 100% |
| EPA5 | 90.3% | 95.9% | 98.5 | 91.1% | 96.2% | 98.6% | 87.9% | 94.4% | 98% | 88.5% | 94.7% | 98% |
| ETU30 | 71.4% | 85.8% | 94.7% | 83.1% | 91.9% | 97% | 67.4% | 83.3% | 93.5% | 68.1% | 83.4% | 93.4% |
| TDL-A | 87.9% | 93.2% | 96.4% | 89.8% | 94% | 96.9% | 87.4% | 92.8% | 96.1% | 87.8% | 92.6% | 96.1% |
| TDL-B | 90% | 94.5% | 97.4% | 90.4% | 94.7% | 97.3% | 88.5% | 93.8% | 96.9% | 88.5% | 94.1% | 97.3% |
| TDL-C | 89.9% | 93.7% | 96.5% | 91% | 94.7% | 97.2% | 89.4% | 94% | 96.9% | 89.9% | 94.3% | 97% |
| TDL-D | 99.3% | 99.9% | 100% | 100% | 100% | 100% | 98.4% | 99.6% | 100% | 98.1% | 99.5% | 100% |

Table 3.3-1 and Table 3.3-2 are the detection delay and one-shot probability for 120 kHz SCS, respectively. Again, the simulation results are similar as those for SCS 15 kHz, 30 kHz, and 120 kHz except for ETU30 channel due to inter-symbol interference. A maximum of 3 shots are necessary to achieve 90% detection probability for all simulated scenarios.

From Section 3.1-2.4, we have the following observations:

Observation 1: SCS has limited impact on the PSS/SSS detection performance.

Observation 2: A maximum of 3 shots are enough to achieve the 90% detection probability for all simulated scenarios.

# Conclusion

In this paper, we updated the link level simulation results for PSS/SSS detection. By comparing the results for all scenarions, we have the following observations:

Observation 1: SCS has limited impact on the PSS/SSS detection performance.

Observation 2: A maximum of 3 shots are enough to achieve the 90% detection probability for all simulated scenarios.

# Reference

- R4-1706903, Link level simulation of PSS/SSS detection in NR, Huawei, HiSilicon

- R4-1706904, Link level simulation assumptions for SS block measurements, Ericsson

- R4-1706902, System level simulation in NR RRM, Huawei, HiSilicon, Nokia, Qualcomm

- R4-1708698, Updated link level simulation assumption of cell detection in NR, Huawei, HiSilicon

- R1-1711888, WF on m0 and m1 for NR-SSS, NTT Docomo, ITL, Intel, Samsung, InterDigital, Motorola Mobility, Lenovo

# Annex A: Simulation Assumptions

Table 1: Cell-specific Parameters for PSS/SSS Detection

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| RF Channel number | - | Channel 1 | Channel 1 |
| Carrier frequency |  | 4GHz; 30GHz | The same frequency as for Cell 1 is used |
| Subcarrier spacing | KHz | 15 kHz; 30 kHz for 4GHz; 120 kHz; 240 kHz for 30GHz | The same values as for Cell 1 are used |
| RB Utilization | % | 100 | 100 |
| Power boosting for PSS and SSS | dB | 0 | 0 |
| Power boosting for Data and Control | dB | 0 | 0 |
| Data Modulation | - | QPSK | QPSK |
| CP Length | - | Normal | Normal |
| Number of transmitted SS block within a SS burst set period (K) | - | 1 (baseline) FFS: 4 | 1(baseline) FFS: 4 |
| SS burst set periodicity | ms | 5 | 5 |
| Relative Delay of 1st Path (synchronous) | μs | 0 | CP/2 |
| Relative Delay of 1st Path (asynchronous) | ms | 0 | 1.5 |
| Frequency Offset relative to UE frequency reference | Hz | 0 | 0 |
| Es/Noc for K=1 | dB | 6.4 | 1.25; FSS: 0.25; -0.75 (SINR: -6dB; FFS: -8dB, -7dB) |
| Es/Noc for K=4 | dB | 2, 6,4, -2 | 2, 1, 1, 1 (SINR: -2.12, -5.97, -4.45, -1.12); -1, 5, -3, 2 dB (SINR: -5.12, -1.97, -8.45, -0.12) |
| Number of Tx antennas | - | 1 | 1 |
| PSS Sequence ID | - | To be stated by companies together with the results (examples shows in Table 3, 4) | To be stated by companies together with the results (examples shows in Table 3, 4) |
| SSS Sequence ID | - | To be stated by companies together with the results (examples shows in Table 3, 4) | To be stated by companies together with the results (examples shows in Table 3, 4) |
| Propagation Condition | - | AWGN, EPA5, ETU30 TDL-A/B/C/D with 100ns for 4 GHz and 30ns for 30 GHz Note1 CDL-A/B/C/D with 100ns for 4 GHz and 30ns for 30 GHz Note1 (secondary priority) |  |
| Noc Model | - | AWGN noise |  |
| NOTE 1: The channel models of CDL and TDL are used for simplified link level evaluations, which are defined in TR 38.901. |  |  |  |

Table 2: UE-specific Parameters for Cell Detection

| Simulation parameters | Comments/values |
| --- | --- |
| Prior knowledge of Cell 1 by the UE | Yes |
| False detect threshold | Required as in a real UE implementation |
| UE having apriority knowledge of system being synchronous or asynchronous | No |
| Performance criterion for comparison | 90th percentile acquisition time for “correct” cell detection of both PSS and SSS sequence IDs. |
| CP length detection | Not simulated |
| Receive antennas | 2, 4 (uncorrelated (except for CDL model) Option 1: With receiver beamforming Option 2: Without receiver beamforming) |

Table 3: Example Cell ID Combinations that can be simulated

| Case # | Cell 1 (Interferer Cell) |  | Cell 2 (Desired Cell) |  | Scenario |
| --- | --- | --- | --- | --- | --- |
| 1 | psc1 | ssc1 | psc2 | ssc2 | Synchronous |
| 2 | psc1 | ssc1 | psc1 | ssc3 | Synchronous |
| 3 | psc1 | ssc1 | psc2 | ssc2 | Asynchronous |
| 4 | psc1 | ssc1 | psc1 | ssc3 | Asynchronous |

Table 4: Example PSS, SSS indices that can be simulated simulations

| PSS Indices |  | SSS Indices |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Label | Code index | Label | Code index (m0, m1) | Cell ID | (,) |
| psc1 | 43 | ssc1 | (20,0) | 337 | (112, 1) |
| psc2 | 86 | ssc2 | (40, 0) | 674 | (224, 2) |
|  |  | ssc3 | (35,0) | 673 | (224, 1) |