3GPP TSG-WG RAN4 Meeting #NR3	R4-1709950

# Nagoya, Japan, Date 18th  – 21st Sept, 2017

Source: 	Nokia, Nokia Shanghai Bell, Skyworks Solutions, Inc.

Title: 					TP for TR General aspects for UE RF for NR: Range 1 ACLR

Agenda Item: 			3.3.3.8.2

Document for:	Approval

# 1	Introduction

This TP introduces agreements on NR UE ACLR for Range 1 into the TR.

# 2	Discussion

In RAN4#83 [1] was approved and it stated as follows

- NR-Sub6ACLR1 = 30dB for power class 3 for wanted channel bandwidth up to 20MHz.

  - Other power class is FFS.

  - The adjacent channel bandwidth the same as the wanted signal.

  - NR-Sub6ACLR1 for other NR wanted channel bandwidth are FFS.

  - E-UTRAACLR1 for different channel bandwidth is FFS.

- For bands defined also for UTRA, adopt UTRAACLR1 = 33dB and UTRAACLR2 =36dB for power class 3.

- Adopt the E-UTRA UE (-50dBm) absolute limit, whichever is less stringent compared to the ACLR1 or ACLR2.

Further agreements for MPR evaluations [2] from NR AH2

- ACLR

  - Sub6

    - PC3(23dBm) [R4-1706582]

      - 30 dBc for NR vs NR with the same channel bandwidths  up to 100MHz

      - If EUTRA CHBW = NR CHBW only the NR ACLR of 30 dBc shall be measured

      - UTRA1=33 dBc, UTRA2=36 dBc

    - PC2(26dBm) [R4-1706556]

      - 31 dBc for NR vs LTE and NR vs NR with the same channel bandwidths

Further agreements for MPR evaluations [3] from RAN4#84

- ACLR is the ratio of power of wanted signal to the power falling into Adjacent Channel. ACLR measurement bandwidth for both the wanted and adjacent channels is the maximum transmission bandwidth among the different SCSs of CP-OFDM SU for a channel BW with addition of one SCS to account for half SCS shift due to SCS alignment to DC, this measurement bandwidth is centered within the channels.

- Offset for the adjacent measurement BW centre is +/- Channel BW from wanted channel centre (in the middle of the Channel).

- Table below provides these measurement BW in the last row.

# 3	References

[1] R4-1706316, WF on sub-6GHz ACLR and ACS, Nokia, Alcatel-Lucent Shanghai Bell, RAN4#83

[2] R4-1706980, WF on MPR evaluation assumption, NTT Docomo, RAN4#NR2

[3] R4-1708912, NR MPR simulation assumptions for sub-6 GHz operation, Nokia, RAN4#84

**************************************** Start of TP ********************************************

### 5.5.3		Adjacent Channel Leakage ratio

Detailed structure of the subclause is TBD.

Concearning NR Range 1 UE ACLR requirement following agreements have been made

#### 5.5.3.1	NR ACLR

- ACLR is the ratio of power of wanted signal to the power falling into Adjacent Channel. ACLR measurement bandwidth for both the wanted and adjacent channels is the maximum transmission bandwidth among the different SCSs of CP-OFDM SU for a channel BW with addition of one SCS to account for half SCS shift due to SCS alignment to DC, this measurement bandwidth is centered within the channels.

- Offset for the adjacent measurement BW centre is +/- Channel BW from wanted channel centre (in the middle of the Channel).

- For PC3 requirement is NRACLR = 30 dBc for channel bandwidths up to 100 MHz

- For PC2 requirement is NRACLR = 31 dBc for channel bandwidths up to 100 MHz

- Table below provides measurement BWs in the last row

| Sub-6GHz | SCS [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| SU_CP-OFDM [#RB] | 15 | 25 | 52 | 79 | 106 | 133 | 216 | 270 | N.A | N.A | N.A |
|  | 30 | 11 | 24 | 38 | 51 | 65 | 106 | 133 | 162 | 217 | 273 |
|  | 60 | N.A | 11 | 18 | 24 | 31 | 51 | 65 | 79 | 107 | 135 |
| RX&TXBW CP [MHz] | 15 | 4.5 | 9.36 | 14.22 | 19.08 | 23.94 | 38.88 | 48.6 | NA | NA | NA |
|  | 30 | 3.96 | 8.64 | 13.68 | 18.36 | 23.4 | 38.16 | 47.88 | 58.32 | 78.12 | 98.28 |
|  | 60 | NA | 7.92 | 12.96 | 17.28 | 22.32 | 36.72 | 46.8 | 56.88 | 77.04 | 97.2 |
| TXBWsym [MHz] | 15 | 4.515 | 9.375 | 14.235 | 19.095 | 23.955 | 38.895 | 48.615 | NA | NA | NA |
|  | 30 | 3.99 | 8.67 | 13.71 | 18.39 | 23.43 | 38.19 | 47.91 | 58.35 | 78.15 | 98.31 |
|  | 60 | NA | 7.98 | 13.02 | 17.34 | 22.38 | 36.78 | 46.86 | 56.94 | 77.1 | 97.26 |
| maxTXBWsym [MHz] | lowest | 4.515 | 9.375 | 14.235 | 19.095 | 23.955 | 38.895 | 48.615 | 58.35 | 78.15 | 98.31 |

#### 5.5.3.2	E-UTRA ACLR

- If EUTRA CHBW = NR CHBW only the NRACLR of 30 dBc shall be measured

#### 5.5.3.3	UTRA ACLR

- For bands defined also for UTRA, adopt UTRAACLR1 = 33dB and UTRAACLR2 =36dB for power class 3.

**************************************** End of TP ********************************************