3GPP TSG-RAN WG4 (Radio) Meeting NR#3  	R4-1709987

Nagoya, Japan, 18-21September, 2017

Agenda item:	3.2.4

Source: 	Ericsson, Nokia

Title: 		Simulation assumptions of Co-existence study for 55dBm CPE in mmWave spectrum

Document for:	Approval

# Introduction

In RAN4#84, a WF has been agreed on conducting simulations related to 55dBm CPE in mmWave scenario.

The contribution lists down the simulation assumptions of coexistence study for 55dBm CPE in mmWave spectrum. We propose to agree on these simulation assumptions so that companies can provide simulation results in the coming meeting which can be compared with each other.

# Network layout model

## Urban macro

### Single operator layout

| Parameters |  | Values | Remark |
| --- | --- | --- | --- |
| Network layout |  | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |  |
| Inter-site distance |  | 300m | Note 1 |
| BS antenna height |  | 25 m |  |
| CPE location | Outdoor/indoor | Outdoor and indoor |  |
|  | Indoor CPE ratio | 20% | Note 1 |
|  | Low/high Penetration loss ratio | 50% low loss, 50% high loss |  |
|  | LOS/NLOS | LOS and NLOS | Specified in TR38.803 |
|  | CPE antenna height | Same as 3D-UMa in TR 36.873 | Elevation height is 4.5m for both indoor and outdoor scenario |
| CPE distribution (horizontal) |  | Uniform |  |
| Minimum BS - CPE distance (2D) |  | 35 m | Note 1 |
| Channel model |  | UMa | Specified in TR38.803 |
| Shadowing correlation |  | Between cells: 1.0 Between sites: 0.5 |  |
| Note: If we find any issue, then we can revisit parameters. |  |  |  |

### Multi operators layout

| Parameters | Values | Remark |
| --- | --- | --- |
| Multi operators layout | coordinated operation (0% Grid Shift) |  |
|  | uncoordinated operation (100% Grid Shift) |  |

| Coordinated Operation: each network with co-location of sites |  |
| --- | --- |
| Non-coordinated operation | 100% grid shift |

## Dense urban

## Companies can provide simulations results for dense urban micro scenario in both 30GHz and 45GHz. The deployment parameters are same as dense urban scenario as described in TR 38.803 except CPR antenna elevation height is 4.5m.-

# Propagation model

## Path loss

Path loss models are based on TR 38.803.

## LOS probability

LOS probability models are based on TR 38.803.

## O-to-I penetration loss

The pathloss incorporating O-to-I building penetration loss is modelled as in 38.803..

# Transmission power control model

For uplink, TPC model specified in Section 9.1 TR 36.942 is applied with following parameters.

- CL-xile = 100+ 10*log10(200/X)

  - X: UL transmission BW

- Gamma = 1

Note: we need discuss KPI based on simulation results.

# Received signal power model

The following model is applied.

RX_PWR = TX_PWR – pathloss + G_TX + G_RX

where:

RX_PWR is the received signal power

TX_PWR is the transmitted signal power

G_TX is the transmitter antenna gain (directional array gain)

G_RX is the receiver antenna gain (directional array gain).

# Other simulation parameters

| Parameters | Urban macro | Dense Urban micro |
| --- | --- | --- |
| Channel bandwidth | 200MHz | 200MHz |
| Scheduled channel bandwidth per CPE (UL) | 200MHz | 200MHz |
| The number of active CPE (UL) | Same as the number of BS beam | Same as the number of BS beam |
| Traffic model | Full buffer | Full buffer |
| UL power control | YES | YES |
| CPE max TX power in dBm | 35dBm | 35dBm |
| CPE min TX power in dBm | -40dBm | -40dBm |
| BS Noise figure in dB | 10 | 10 for 30GHz 12 for 45GHz |
| Handover margin | 3dB | 3dB |

# Simulation description

Adopt following simulation steps. If companies find issues in the following simulation steps, we can revisit in RAN4#80bis. Note: detailed simulation description is captured in Section 5.1.5 TR25.942.

- Aggressor and victim network are generated.

- CPE associations: CPE are associated to base station based on coupling loss.

  - Associations are made assuming a single element at both CPE and BS.

- Once association is done, round robin scheduling is used. BF weights are adjusted to point to the LOS direction between BS-CPE. This done for both victim and aggressor networks.

- SINR Throughput are measured in the victim systems without considering ACI, i.e. ThputNO ACIbpshz=fSINRICI=fSN+IICI, where IICI is the inter-cell interference.

- SINR and throughput are computed considering ACI: ThputACIbpshz=fSINRICI+ACI=fSN+IICI+IACI, where IACI is the adjacent channel interference.

- RF parameters are determined based degradation cause by ACI: LossACI=1-ThputACIThputSINGLE.

# Evaluation metric

Assume scaled Shannon's formula specified in TR36.942 with following parameters.

| Parameter |  | UL | Notes |
| --- | --- | --- | --- |
| α, attenuation |  | 0.4 | Represents implementation losses |
| SINRMIN, dB |  | -10 | Based on QPSK, 1/5 rate (UL) |
| SINRMAX, dB |  | 22 | Based on 64QAM 0.93 (UL) |

# ACIR range to be evaluated

ACIR range: 5 ~ 45dB, with 5dB as step. Note that: we do not have simulation per 1dB step.

# Antenna configuration

## Base station

Assumptions captured in page 8 - 11 of [R4-168794] are assumed as BS antenna configurations. For UMa and UMi  base stations, we keep this same antenna model.

## CPE

For CPE, we consider the following:

- Baseline: (NV,NH) = (4, 8).

- (dV,dH) = (0.5, 0.5)λ.

- An additional 3dB gain is added to the total beamforming gain to account for the two polarization directions.

- 5dBi element gain, 90 degree HPBW in Azimuth and zenith, Am=25dB, SLAv=25dB

# Co-existence scenarios

Evaluate following scenarios for 55dBm CPE:

| No. | Aggressor | Victim | Simulation freqCPEncy | Direction | Usage scenario | Deployment Scenario |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | NR, 200MHz | NR, 200MHz | 30 GHz | UL to UL | eMBB | Urban macro |
| 2 | NR, 200MHz | NR, 200MHz | 30 GHz | UL to UL | eMBB | Dense Urban |
| 3 | NR, 200MHz | NR, 200MHz | 45 GHz | UL to UL | eMBB | Dense Urban |