3GPP TSG-RAN WG4 Meeting NR#3	R4-1710040

Nagoya, Japan, 18th – 21st September 2017

Source:	Nokia, Nokia Shanghai Bell, Ericsson

Title:	Way forward on alignment the boundary of FR1 NR spectrum emission mask with AAS BS for E-UTRA operating bands ≥ 100 MHz

Agenda item:	3.4.3.2.2

Document for:	Approval

# 1	Introduction

During the RAN4 ad-hoc NR#3 meeting, contributions [1][2] proposed to align the agreement made for NR BS unwanted emission mask with OTA AAS BS for downlink operating bands with widths that are ≥ 100 MHz. There are, however, open issues.

This WF attempts to capture any agreements following online and offline discussion on the open issues.

# 2	Background

ΔfUEM  = 40  MHz

ΔfUEM  = 40  MHz

ΔfUEM = 40  MHz

ΔfUEM = 40  MHz

NR Operating bands

NR Operating bands

Upper frequency ranges of spurious emission

Upper frequency ranges of spurious emission

Lower frequency ranges of spurious emissions

Lower frequency ranges of spurious emissions

Frequency

Frequency

≥ 100 MHz

≥ 100 MHz

Figure 1: Frequency ranges for spurious emissions and UEM

From [3], the boundary of the spurious emissions and UEM is extended to 40 MHz for NR downlink operating bands that are ≥ 100 MHz for both Category A and Category B as shown in Figure 1. [4] and [5] provide explanation that the motivation of the extension is the impact of the transition zone on filter size dependent on the width of the band and that the extension is in line with European regulation and does not impact category A unwanted emissions.

The fundamental justification and regulation impact is the same for E-UTRA as NR. References [1] and [2] propose to apply ΔfUEM = 40 MHz to the E-UTRA downlink operating bands that are ≥ 100 MHz as shown in Table 1.

| Operating Band number | Bandwidth [MHz] | Duplex Mode |
| --- | --- | --- |
| 40 | 100 | TDD |
| 41 | 194 | TDD |
| 42 | 200 | TDD |
| 43 | 200 | TDD |
| 44 | 100 | TDD |
| 48 | 150 | TDD |

Table 1: E-UTRA operating bands

# 3	Way forward

Based on the online and offline discussion, the following options are for further studies (other options are not precluded):

Option 1: apply ΔfUEM = 40 MHz to AAS BS Tx requirements (TS 37.105, 37.145-1, 37.145-2) for NR and E-UTRA operation. Consider if ΔfUEM = 40 MHz can be applied to non-AAS BS Tx requirements (TS 36.104, 36.141, 37.104, 37.141)

Option 2: apply ΔfUEM = 40 MHz to AAS BS Tx requirements (TS 37.105, 37.145-1, 37.145-2) for NR and E-UTRA operation and consider to revisit the ΔfUEM = 40 agreement for FR1 E-UTRA refarming bands for NR BS

Companies are encouraged to provided their views at the RAN4 #84bis meeting in which the final decision will be made.

# References

- R4-1709520, On NR transition zone for bands larger than 100 MHz ad relation to MSR/AAS, Ericsson

- R4-1709821, Discussing Range 1 NR and AAS BS spectrum emission mask requirements for bands equal to or wider than 100 MHz, Nokia, Nokia Shanghai Bell

- R4-1706843, On NR BS boundary between UEM and spurious emission below 6 GHz, Nokia, Alcatel-Lucent Shanghai Bell

- R4-1704616, “NR BS boundary between out-of-band and spurious domain below 6 GHz”, Ericsson

- R4-1703094, “NR BS boundary between out-of-band and spurious domain below 6 GHz”, Ericsson