3GPP TSG-RAN WG5 Meeting #43 	R5-092610

# Vilnius, Lithuania, 20-24 April 2009

| CR-Form-v9.6 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | CRNum | rev | - | Current version: | 8.1.0 |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Text proposal for TDD part of CQI Reporting under Fading conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Japan |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE-UEConTest_RF |  |  |  |  |  |  | Date: |  | 20/04/2009 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-8 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release10) |  |
| Reason for change: |  | TDD portion of the CQI reporting test cases have been introduced in the RAN4 spec R4-091045. These test cases are introduced to align with the RAN4 spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added the TDD portion of the CQI Reporting under Fading conditions – PUSCH 3-0 clause 9.3.1 - Added the TDD portion of the CQI Reporting under Fading conditions – PUCCH 1-0 clause 9.3.2 - Changed FDD to TDD in test applicability, Aligned Initial conditions |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 36.521-1 spec will not be in alignment with RAN4 spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1, 9.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Approved RAN5#42Bis CR R5-091917 is the baseline for this CR and aligned to the TS36101 V8.5.1. 1st Revision of R5-092456 R5-092610 |  |  |  |  |  |  |  |  |

## 9.3	CQI Reporting under fading conditions

### 9.3.1	Frequency-selective scheduling mode

The accuracy of sub-band channel quality indicator (CQI) reporting under frequency selective fading conditions is determined by a double-sided percentile of the reported differential CQI offset level 0 per sub-band, and [the relative increase of the throughput] obtained when transmitting on any one of the sub-bands with the highest reported differential CQI offset level the corresponding transport format compared to the case for which a fixed format is transmitted on any sub-band in set S [36.213].

#### 9.3.1.1	Frequency-selective scheduling mode - PUSCH 3-0

##### 9.3.1.1.1		FDD Frequency-selective scheduling mode – PUSCH 3-0

Editor’s note: The following aspects are either missing or not yet determined:

- Measurement channel used is undefined

- Physical channels used are undefined

- The Initial Conditions including UE setup are undefined

- The Test procedure and test requirements are undefined

- The Message contents are undefined

- The Test system uncertainties applicable to this test are undefined

- Test tolerances for SNR have not yet been applied

###### 9.3.1.1.1.1		Test purpose

To verify that preferred sub-bands can be used for frequently-selective scheduling.

###### 9.3.1.1.1.2		Test applicability

This test applies to all types of E-UTRA FDD UE release 8 and forward.

###### 9.3.1.1.1.3		Minimum conformance requirements

For the parameters specified in Table 9.3.1.1.1.3-1, [and using the downlink physical channels specified in Annex C,] the minimum requirements are specified in Table 9.3.1.1.1.3-2 and by the following

a)	a sub-band differential CQI offset level of 0 shall be reported at least  % of the time but less than % for each sub-band;

b)	[the ratio of the throughput obtained when transmitting on any one of the sub-bands with the highest differential CQI offset level the corresponding TBS and that obtained when transmitting the TBS indicated by the reported wideband CQI median on a randomly selected sub-band in set S shall be ≥ ;]

c)	[BLER requirement]

[Editors note: details of additional requirements (b) and (c) TBD]

The requirements only apply for sub-bands of full size and the random scheduling across the sub-bands is done by selecting a new sub-band in each TTI [for FDD, each available downlink transmission instance for TDD]. The transport block size TBS(wideband CQI median) is that resulting from the code rate which is closest to that indicated by the wideband CQI median and theentry in Table 7.1.7.2.1-1 [TS 36.213] that corresponds to the sub-band size.

Table 9.3.1.1.1.3-1 Sub-band test for single antenna transmission (FDD)

| Parameter | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- |
| Bandwidth | MHz | 10 MHz |  |
| Transmission mode |  | 1 (port 0) |  |
| SNR | dB | [9] | [14] |
|  | dB[mW/15kHz] | [-98] | [-98] |
|  | dB[mW/15kHz] | [-89] | [-84] |
| Propagation channel |  | [Clause B.2.4] |  |
| Correlation |  | [Full] |  |
| Reporting interval | ms | [5] ms |  |
| Reporting mode |  | PUSCH 3-0 |  |
| Max number of HARQ transmissions |  | [1] |  |

Table 9.3.1.1.1.3-2  Minimum requirement (FDD)

|  | Test 1 | Test 2 |  |
| --- | --- | --- | --- |
|  [%] | TBD | TBD |  |
|  [%] | TBD | TBD |  |
|  | TBD | TBD |  |

###### 9.3.1.1.1.4		Test description

9.3.1.1.1.4.1	Initial conditions

Test Environment: Normal as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Mid Range as defined in TS 36.508 [7] clause 4.3.1.1.

Channel Bandwidths to be tested: 10MHz, as defined in TS 36.508 [7] clause 4.3.1.1

1.	Connect the SS to the UE antenna connector (s) as shown in TS 36.508 [7] Annex A, Figure [FFS].

2.	The parameter settings for the cell are set up according to Table 9.3.1.1.1.3-1.

3.	Downlink signals are initially set up according to Annex C.1 and Annex C.3.2, and uplink signals according to Annex H.1 and H.3.2.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in State 4 according to TS 36.508 [7] clause 4.5.4 and receiving payload data from the SS. Message contents are defined in clause 9.3.1.1.1.4.3.

9.3.1.1.1.4.2	Test procedure

[FFS]

9.3.1.1.1.4.3	Message contents

Message contents are according to [clause FFS in reference FFS].

###### 9.3.1.1.1.5		Test requirement

[FFS]

#### 9.3.1.1.2		TDD Frequency-selective scheduling mode – PUSCH 3-0

###### 9.3.1.1.2.1		Test purpose

To verify that preferred sub-bands can be used for frequently-selective scheduling.

###### 9.3.1.1.2.2		Test applicability

This test applies to all types of E-UTRA TDD UE release 8 and forward.

###### 9.3.1.1.2.3		Minimum conformance requirements

For the parameters specified in Table 9.3.1.1.2.3-1, [and using the downlink physical channels specified in Annex C,] the minimum requirements are specified in Table 9.3.1.1.2.3-2 and by the following

- a sub-band differential CQI offset level of 0 shall be reported at least  % of the time but less than % for each sub-band;

- [the ratio of the throughput obtained when transmitting on any one of the sub-bands with the highest differential CQI offset level the corresponding TBS and that obtained when transmitting the TBS indicated by the reported wideband CQI median on a randomly selected sub-band in set S shall be ≥ ;]

- [BLER requirement]

[Editors note: details of additional requirements (b) and (c) TBD]

The requirements only apply for sub-bands of full size and the random scheduling across the sub-bands is done by selecting a new sub-band in each TTI [for FDD, each available downlink transmission instance for TDD]. The transport block size TBS(wideband CQI median) is that resulting from the code rate which is closest to that indicated by the wideband CQI median and theentry in Table 7.1.7.2.1-1 [TS 36.213] that corresponds to the sub-band size.

Table 9.3.1.1.2.3-1 Sub-band test for single antenna transmission (TDD)

| Parameter | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- |
| Bandwidth | MHz | 10 MHz |  |
| Transmission mode |  | 1 (port 0) |  |
| Uplink downlink configuration |  | 1 |  |
| Special subframe configuration |  | 4 |  |
| SNR | dB | [9] | [14] |
|  | dB[mW/15kHz] | [-98] | [-98] |
|  | dB[mW/15kHz] | [-89] | [-84] |
| Propagation channel |  | [Clause B.2.4] |  |
| Correlation |  | [Full] |  |
| Reporting interval | ms | [5] ms |  |
| Reporting mode |  | PUSCH 3-0 |  |
| Max number of HARQ transmissions |  | [1] |  |

Table 9.3.1.1.2.3-2  Minimum requirement (TDD)

|  | Test 1 | Test 2 |  |
| --- | --- | --- | --- |
|  [%] | TBD | TBD |  |
|  [%] | TBD | TBD |  |
|  | TBD | TBD |  |

###### 9.3.1.1.2.4		Test description

9.3.1.1.2.4.1	Initial conditions

Test Environment: Normal as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Mid Range as defined in TS 36.508 [7] clause 4.3.1.2.

Channel Bandwidths to be tested: 10MHz, as defined in TS 36.508 [7] clause 4.3.1.2

1.	Connect the SS to the UE antenna connector (s) as shown in TS 36.508 [7] Annex A, Figure [FFS].

2.	The parameter settings for the cell are set up according to Table 9.3.1.1.2.3-1.

3.	Downlink signals are initially set up according to Annex C.1 and Annex C.3.2, and uplink signals according to Annex H.1 and H.3.2.

4.	Propagation conditions are set according to Annex B clause B.0

5.	Ensure the UE is in State 4 according to TS 36.508 [7] clause 4.5.4 and receiving payload data from the SS. Message contents are defined in clause 9.3.1.1.2.4.3.

9.3.1.1.2.4.2	Test procedure

[FFS]

9.3.1.1.2.4.3	Message contents

Message contents are according to [clause FFS in FFS].

###### 9.3.1.1.2.5		Test requirement

[FFS]

### 9.3.2	Frequency non-selective scheduling mode

The reporting accuracy of the channel quality indicator (CQI) under frequency non-selective fading conditions is determined by a single-sided percentile of the reported CQI, and the relative increase of the throughput obtained when the transport format transmitted is that indicated by the reported CQI compared to the case for which a fixed transport format configured according to the reported median CQI is transmitted.

#### 9.3.2.1	Frequency non-selective scheduling mode – PUCCH 1-0

##### 9.3.2.1.1		FDD Frequency non-selective scheduling mode – PUCCH 1-0

Editor’s note: The following aspects are either missing or not yet determined:

- Measurement channel used is undefined

- Physical channels used are undefined

- The Initial Conditions including UE setup are undefined

- The Test procedure and test requirements are undefined

- The Message contents are undefined

- The Test system uncertainties applicable to this test are undefined

- Test tolerances for SNR have not yet been applied

###### 9.3.2.1.1.1		Test purpose

To verify that the UE is tracking the channel variations and selecting the transport format according to the prevailing channel state for frequently non-selective scheduling

###### 9.3.2.1.1.2		Test applicability

This test applies to all types of E-UTRA FDD UE release 8 and forward.

###### 9.3.2.1.1.3		Minimum conformance requirements

For the parameters specified in Table 9.3.2.1.1.3-1, [and using the downlink physical channels specified in Annex C,] the minimum requirements are specified in Table 9.3.2.1.1.3-2 and by the following

a)		CQI index below (wideband CQI median – [1]) shall be reported at least  % of the time;

b)		[the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index and that obtained when transmitting a fixed transport format configured according to the wideband CQI median shall be ≥ ;]

c)		[BLER requirement]

[Editors note: details of additional requirements (b) and (c) TBD]

The transport block size TBS(wideband CQI median) is that resulting from the code rate which is closest to that indicated by the wideband CQI median and theentry in Table 7.1.7.2.1-1 [36.213] that corresponds to the maximum transmission configuration (Table 5.6-1).

Table 9.3.2.1.1.3-1 Fading test for single antenna (FDD)

| Parameter | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- |
| Bandwidth | MHz | 10 MHz |  |
| Transmission mode |  | 1 (port 0) |  |
| SNR | dB | [6] | [12] |
|  | dB[mW/15kHz] | [-98] | [-98] |
|  | dB[mW/15kHz] | [-92] | [-86] |
| Propagation channel |  | EPA5 |  |
| Correlation |  | High |  |
| Reporting mode |  | PUCCH 1-0 |  |
| Reporting periodicity | ms | [NP = 2] |  |
| PUCCH Format |  | [Format 2] |  |
| PUCCH Report Type |  | 4 |  |
| cqi-pmi-ConfigurationIndex |  | 1 |  |
| Max number of HARQ transmissions |  | [1] |  |

Table 9.3.2.1.1.3-2 Minimum requirement (FDD)

|  | Test 1 | Test 2 |  |
| --- | --- | --- | --- |
|  [%] | TBD | TBD |  |
|  | TBD | TBD |  |

###### 9.3.2.1.1.4		Test description

9.3.2.1.1.4.1	Initial conditions

Test Environment: Normal as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Mid Range as defined in TS 36.508 [7] clause 4.3.1.1.

Channel Bandwidths to be tested: 10MHz, as defined in TS 36.508 [7] clause 4.3.1.1

1.	Connect the SS to the UE antenna connector (s) as shown in TS 36.508 [7] Annex A, Figure [FFS].

2.	The parameter settings for the cell are set up according to Table 9.3.2.1.1.3-1.

3.	Downlink signals are initially set up according to Annex C.1 and Annex C.3.2, and uplink signals according to Annex H.3.2.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in State 4 according to TS 36.508 [7] clause 4.5.4 and receiving payload data from the SS. Message contents are defined in clause 9.3.2.1.1.4.3.

9.3.2.1.1.4.2	Test procedure

[FFS]

9.3.2.1.1.4.3	Message contents

Message contents are according to [clause FFS in reference FFS].

###### 9.3.2.1.1.5		Test requirement

[FFS]

##### 9.3.2.1.2		TDD Frequency non-selective scheduling mode – PUCCH 1-0

###### 9.3.2.1.2.1		Test purpose

To verify that the UE is tracking the channel variations and selecting the transport format according to the prevailing channel state for frequently non-selective scheduling

###### 9.3.2.1.2.2		Test applicability

This test applies to all types of E-UTRA TDD UE release 8 and forward.

###### 9.3.2.1.2.3		Minimum conformance requirements

For the parameters specified in Table 9.3.2.1.2.3-1, [and using the downlink physical channels specified in Annex C,] the minimum requirements are specified in Table 9.3.2.1.2.3-2 and by the following

- a CQI index below (wideband CQI median – [1]) shall be reported at least  % of the time;

- [the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index and that obtained when transmitting a fixed transport format configured according to the wideband CQI median shall be ≥ ;]

- [BLER requirement]

[Editors note: details of additional requirements (b) and (c) TBD]

The transport block size TBS(wideband CQI median) is that resulting from the code rate which is closest to that indicated by the wideband CQI median and theentry in Table 7.1.7.2.1-1 [36.213] that corresponds to the maximum transmission configuration (Table 5.6-1).

Table 9.3.2.1.2.3-1 Fading test for single antenna (TDD)

| Parameter | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- |
| Bandwidth | MHz | 10 MHz |  |
| Transmission mode |  | 1 (port 0) |  |
| Uplink downlink configuration |  | 1 |  |
| Special subframe configuration |  | 4 |  |
| SNR | dB | [6] | [12] |
|  | dB[mW/15kHz] | [-98] | [-98] |
|  | dB[mW/15kHz] | [-92] | [-86] |
| Propagation channel |  | EPA5 |  |
| Correlation |  | High |  |
| Reporting mode |  | PUCCH 1-0 |  |
| Reporting periodicity | ms | [NP = 1] |  |
| PUCCH Format |  | [Format 2] |  |
| PUCCH Report Type |  | 4 |  |
| cqi-pmi-ConfigurationIndex |  | 0 |  |
| Max number of HARQ transmissions |  | [1] |  |

Table 9.3.2.1.2.3-2 Minimum requirement (TDD)

|  | Test 1 | Test 2 |  |
| --- | --- | --- | --- |
|  [%] | TBD | TBD |  |
|  | TBD | TBD |  |

###### 9.3.2.1.2.4		Test description

9.3.2.1.2.4.1	Initial conditions

Test Environment: Normal as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Mid Range as defined in TS 36.508 [7] clause 4.3.1.2.

Channel Bandwidths to be tested: 10MHz, as defined in TS 36.508 [7] clause 4.3.1.2

1.	Connect the SS to the UE antenna connector (s) as shown in TS 36.508 [7] Annex A, Figure [FFS].

2.	The parameter settings for the cell are set up according to Table 9.3.2.1.2.3-1.

3.	Downlink signals are initially set up according to Annex C.1 and Annex C.3.2, and uplink signals according to Annex H.3.2.

4.	Propagation conditions are set according to Annex B clause B.0.

5.	Ensure the UE is in State 4 according to TS 36.508 [7] clause 4.5.4 and receiving payload data from the SS. Message contents are defined in clause 9.3.2.1.2.4.3.

9.3.2.1.2.4.2	Test procedure

[FFS]

9.3.2.1.2.4.3	Message contents

Message contents are according to [clause FFS in reference FFS].

###### 9.3.2.1.2.5		Test requirement

[FFS]