3GPP TSG-RAN5 Meeting #46 	R5-100158

# San Francisco, USA, 22-26 February 2010

| CR-Form-v9.6 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.121-1 | CR | CRNum | rev | - | Current version: | 8.9.0 |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case TC 8.6.7.2 (Combined Interfrequency and E-UTRAN measurements) to TS 34.121-1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | QUALCOMM Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE-UEConTest_RF |  |  |  |  |  |  | Date: |  | 02/12/2010 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-8 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) |  |
| Reason for change: |  | Core specification TS 25.133 has added a new test case on Combined inter frequency and E-UTRAN measurements. Test specification 34.121-1 needs to be updated to accomodate the new test case procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New test case added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The 34.121-1 spec won’t be inline with the core spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.6.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | This CR applies to Rel-8 and later UEs that support UTRA FDD and EUTRA TDD |  |  |  |  |  |  |  |  |

### 8.6.7	Combined Interfrequency and E-UTRAN measurements

#### 8.6.7.2 	Correct reporting of E-UTRA TDD neighbours in Fading propagation condition

##### 8.6.7.2.1	Definition and applicability

In the event triggered reporting period the measurement reporting delay is defined as the time between any event that will trigger a measurement report until the UE starts to transmit the measurement report over the Uu interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH . The delay uncertainty is twice the TTI of the uplink DCCH.

The requirements in this section apply only to UE supporting UTRA FDD and E-UTRA TDD for Release 8 and later releases.

##### 8.6.7.2.2	Minimum requirement

When transmission gaps are scheduled for FDD inter frequency measurements the UE shall be able to identify a new detectable cell belonging to the monitored set within

ms

The UE physical layer shall when transmission gaps are scheduled for FDD inter frequency measurements also be capable of reporting measurements to higher layers with measurement accuracy as specified in sub-clause 9.1.1 and 9.1.2 of 25.133 [2] with measurement period given by

If the UE does not need compressed mode to perform inter-frequency measurements, the measurement period for inter frequency measurements is 480 ms.

TMeasurement_Period Inter = 480 ms. The period used for calculating the measurement period Tmeasurement_inter for inter frequency CPICH measurements.

Tbasic_identify_FDD,inter = 300 ms. This is the time period used in the inter frequency equation where the maximum allowed time for the UE to identify a new FDD cell is defined.

TInter = 28 ms. This is the minimum time that is available for inter frequency measurements, during the period TMeasurement_Period inter with an arbitrarily chosen timing. The minimum time per transmission gap is calculated by using 6 gaps of length 7 slots each.

NFreq: Number of FDD frequencies indicated in the inter frequency measurement control information.

Thus the delay for interfrequency cell detection is equal to 5143 ms.

In CELL_DCH state when a transmission gap pattern sequence is provided by the UTRAN the UE shall continuously measure previously detected E-UTRAN cells and search for new E-UTRAN cells.

When compressed mode gaps are used for E-UTRAN measurements, the UE shall be able to identify a new detectable E-UTRAN cell within

;

If the UE does not need compressed mode to perform E-UTRAN measurements, the UE shall be able to identify a new detectable E-UTRAN cell within  ms.

Where:

TE-UTRAN: 28 ms  This is the minimum time that is available for E-UTRAN measurements during a 480ms period with an arbitrarily chosen timing. . The minimum time per transmission gap is calculated by using 6 gaps of length 7 slots each.

Nfreq,E-UTRAN :This is the number of E-UTRAN carriers being monitored

TBasic_Identify_E-UTRAN = 480ms

Thus the delay for interRAT cell detection is equal to 8229 ms.

The normative reference for these requirements is TS 25.133 [2] clauses 8.1.2.3, 8.1.2.6 and A.8.7.2.

##### 8.6.7.2.3	Test purpose

To verify that the UE makes correct reporting of an event when doing combined inter frequency and EUTRAN TDD measurements.

##### 8.6.7.2.4	Method of test

8.6.7.2.4.1	Initial conditions

Test environment: normal; see clauses G.2.1 and G.2.2.

Frequencies to be tested: mid range; see clause G.2.4.

The test parameters are given in tables 8.6.7.2.4.1, 8.6.7.2.4.2 and 8.6.7.2.4.3. In the measurement control information it is indicated to the UE that event-triggered reporting with Event 2B and 3A shall be used. The test consists of two successive time periods, with time duration T1 and T2.

Table 8.6.7.2.4.1: General test parameters for Correct reporting of neighbours in Fading propagation condition

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| DCH parameters |  | DL Reference Measurement Channel 12.2 kbps | As specified in TS 25.101 section A.3.1 |
| Power Control |  | On |  |
| Compressed mode Interfrequency measurements E-UTRAN measurement |  | DL compressed mode reference pattern 2, set 5 DL compressed mode reference pattern 2, set 5 | Only applicable for UE requiring compressed mode patterns. As specified in table A.22 TS 25.101 section A.5 As specified in table A.22 TS 25.101 section A.5 |
| Active cell |  | Cell 1 |  |
| Inter-RAT measurement quantity |  | E-UTRAN RSRP |  |
| Absolute Threshold (Ec/N0) used frequency | dB | -9 | Ec/N0 threshold for Event 2b and 3a |
| Absolute Threshold (Ec/N0) used for a not used frequency | dB | -18 | Ec/IN0 threshold for Event 2b |
| Threshold other system | dBm | -96 | Absolute E-UTRA RSRP threshold for event 3a. |
| Time to Trigger | ms | 0 |  |
| Filter coefficient |  | 0 |  |
| Monitored cell list size |  | Total 24 FDD neighbours 8 on frequency Channel 2 | Measurement control information is sent before the compressed mode pattern starts. |
| T1 | s | 1 |  |
| T2 | s | 10 |  |

Table 8.6.7.2.4.2: Cell Specific parameters for Correct reporting of neighbours in Fading propagation condition

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| UTRA RF Channel Number |  | Channel 1 |  | Channel 2 |  |
| CPICH_Ec/Ior | dB | -10 |  | -10 |  |
| PCCPCH_Ec/Ior | dB | -12 |  | -12 |  |
| SCH_Ec/Ior | dB | -12 |  | -12 |  |
| PICH_Ec/Ior | dB | -15 |  | -15 |  |
| DPCH_Ec/Ior | dB | Note 1 |  | N/A |  |
| OCNS |  | Note 2 |  | -0.941 |  |
|  | dB | 0 |  | -Infinity | -1.8 |
|  | dBm/3,84 MHz | -70 |  | -70 |  |
| CPICH_Ec/Io | dB | -13 |  | -Infinity | -14 |
| Propagation Condition |  | AWGN |  | Case 5 as specified in Annex B of TS25.101 |  |
| Note 1: The DPCH level is controlled by the power control loop Note 2: The power of the OCNS channel that is added shall make the total power from the cell to be equal to Ior |  |  |  |  |  |

# Table 8.6.7.2.4.3: Cell specific test parameters for Correct reporting of E-UTRAN TDD neighbours in fading propagation condition (cell 3)

| Parameter | Unit | Cell 3 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| OCNG Pattern defined in A.3.2.2.2 in 3GPP TS 36.133 (OP.2 TDD) |  | OP.2 TDD |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
| Note 2 | dBm/15 kHz | -98 |  |
|  | dB | -Infinity | 8 |
|  | dB | -Infinity | 8 |
| RSRP Note 3 | dBm/15 kHz | -Infinity | -90 |
| SCH_RP | dBm/15 kHz | -Infinity | -90 |
| Propagation Condition |  | ETU70 (Note 4) |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. OCNG patterns are specified in annex A 3.2 of 3GPP TS 36.133. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: ETU70 propagation conditions are specified in Annex B.2 of 3GPP TS 36.101. |  |  |  |

8.6.7.2.4.2	Procedure

- The RF parameters are set up according to T1 in Table 8.6.7.2.4.4 and 8.6.7.2.4.5.

- The UE is switched on.

- A call is set up according to the test procedure specified in TS 34.108 [3] sub clause 7.3.2.

- If Compressed mode is required, SS shall transmit PHYSICAL CHANNEL RECONFIGURATION message (compressed gaps). Otherwise, Go to Step6.

- UE shall transmit PHYSICAL CHANNEL RECONFIGURATION COMPLETE message (compressed gaps).

- SS shall transmit a MEASUREMENT CONTROL message (inter frequency)

- SS shall transmit a MEASUREMENT CONTROL message (event triggered inter RAT, activation of TGPSI 1-2)

- 1 second after step 7 has completed, the SS shall switch the power settings from T1 to T2 according to the parameters defined in tables 8.6.7.2.4.4 and 8.6.7.2.4.5.

- UE shall transmit a MEASUREMENT REPORT message (inter frequency) triggered by event 2B for cell 2. The measurement reporting delay from the beginning of T2 shall be less than 5.14 seconds. If the UE fails to report the event within the required delay, then the counter event2B_failure is increased by one. If the reporting delay for this event is within the required limit, the counter event2B_success is increased by one.

- UE shall transmit a MEASUREMENT REPORT message (inter RAT) triggered by event 3A for cell 3. The measurement reporting delay from the beginning of T2 shall be less than 8.229 s. If the UE fails to report the event within the required delay, then the counter event3A_failure is increased by one. If the reporting delay for this event is within the required limit, the counter event3A _success is increased by one.

- After 10 seconds after the beginning of T2, the UE is switched off.

- Repeat steps 1-11 according to Annex F.6.2 Table F.6.2.8. If one counter reaches the pass criterion, this counter is stopped and the remaining counters are continued. For the overall outcome of the test to be considered a pass, the counters for 2B events, 3A events shall independently indicate a pass result. The test is stopped immeadiately and the test is considered to be a fail, if any counter reaches an early fail criterion.

Specific Message Contents

All messages indicated above shall use the same content as described in the default message content in clause 9 of 34.108 [3], with the following exceptions:

MEASUREMENT CONTROL message (inter frequency)

| Information Element/Group name | Value/Remark |
| --- | --- |
| Message Type (10.2.17) |  |
| UE information elements |  |
| -RRC transaction identifier | 0 |
| -Integrity check info |  |
| -message authentication code | SS calculates the value of MAC-I for this message and writes to this IE. The first/ leftmost bit of the bit string contains the most significant bit of the MAC-I. |
| -RRC message sequence number | SS provides the value of this IE, from its internal counter. |
| Measurement Information elements |  |
| -Measurement Identity | 1 |
| -Measurement Command (10.3.7.46) | Setup |
| -Measurement Reporting Mode (10.3.7.49) |  |
| -Measurement Report Transfer Mode | AM RLC |
| -Periodical Reporting / Event Trigger Reporting Mode | Event trigger |
| -Additional measurements list (10.3.7.1) | Not Present |
| -CHOICE Measurement type | Inter-frequency measurement |
| -Inter-frequency measurement (10.3.7.16) |  |
| -Inter-frequency measurement objects list (10.3.7.13) |  |
| - CHOICE Inter-frequency cell removal | Not Present |
| - New Inter frequency cells |  |
| - Inter frequency cell id | Refer to table K.1 |
| - Frequency info |  |
| - CHOICE mode | FDD |
| - UARFCN uplink(Nu) | Not Present |
| - UARFCN downlink(Nd) | Same frequency as "Channel2" in Table 8.6.5.1.4.2 |
| - Cell info |  |
| - Cell individual offset | Not Present |
| - Reference time difference to cell | Not Present |
| - Read SFN indicator | FALSE |
| - CHOICE mode | FDD |
| - Primary CPICH info |  |
| - Primary scrambling code | Set to Primary scrambling code of Cell2 |
| - Primary CPICH Tx Power | Not present |
| - Tx Diversity Indicator | FALSE |
| - Cell for measurement | Not Present |
| -Inter-frequency measurement quantity (10.3.7.18) |  |
| -CHOICE reporting criteria | Inter-frequency reporting criteria |
| -Inter-frequency reporting criteria |  |
| -Filter coefficient | 0 |
| -CHOICE mode | FDD |
| -Measurement quantity for frequency quality estimate | CPICH Ec/N0 |
| -Inter-frequency reporting quantity (10.3.7.21) |  |
| -UTRA Carrier RSSI | FALSE |
| -Frequency quality estimate | FALSE |
| -Non frequency related cell reporting quantities (10.3.7.5) |  |
| -Cell synchronisation information reporting indicator | FALSE |
| -Cell Identity reporting indicator | FALSE |
| -CHOICE mode | FDD |
| -CPICH Ec/N0 reporting indicator | TRUE |
| -CPICH RSCP reporting indicator | FALSE |
| -Pathloss reporting indicator | FALSE |
| -Reporting cell status (10.3.7.61) | Not Present |
| -Measurement validity (10.3.7.51) | Not Present |
| -Inter-frequency set update (10.3.7.22) |  |
| -UE autonomous update mode | On with no reporting |
| -CHOICE report criteria | Inter-frequency measurement reporting criteria |
| -Inter-frequency measurement reporting criteria (10.3.7.19) |  |
| - Parameters required for each events |  |
| - Inter-frequency event identity | 2b |
| - Threshold used frequency | -15 dB |
| - W used frequency | 0 |
| - Hysteresis | 0 dB |
| - Time to trigger | 0 ms |
| - Reporting cell status | Within active set or within virtual active set or of the other RAT |
| - Maximum number of reporting cells | 1 |
| - Non used frequency parameter list |  |
| - Threshold non used frequency | -15 dB |
| - W non-used frequency | 0 |
| Physical channel information elements |  |
| -DPCH compressed mode status info (10.3.6.34) | Not present |

MEASUREMENT CONTROL message (event triggered inter RAT, activation of TGPSI 1-2)

| Information Element/Group name | Value/Remark |
| --- | --- |
| Message Type (10.2.17) |  |
| UE information elements |  |
| -RRC transaction identifier | 0 |
| -Integrity check info |  |
| -message authentication code | SS calculates the value of MAC-I for this message and writes to this IE. The first/ leftmost bit of the bit string contains the most significant bit of the MAC-I. |
| -RRC message sequence number | SS provides the value of this IE, from its internal counter. |
| Measurement Information elements |  |
| -Measurement Identity | 2 |
| -Measurement Command (10.3.7.46) | Setup |
| -Measurement Reporting Mode (10.3.7.49) |  |
| -Measurement Report Transfer Mode | AM RLC |
| -Periodical Reporting / Event Trigger Reporting Mode | Event trigger |
| -Additional measurements list (10.3.7.1) | Not Present |
| -CHOICE Measurement type | Inter-RAT measurement |
| -Inter-RAT measurement (10.3.7.27) |  |
| -Inter-RAT measurement objects list (10.3.7.23) | Not Present |
| -Inter-RAT measurement quantity (10.3.7.29) |  |
| -Measurement quantity for UTRAN quality estimate (10.3.7.38) |  |
| -Filter coefficient | 0 |
| -CHOICE mode | FDD |
| -Measurement quantity | CPICH EcNo |
| -CHOICE system | E-UTRA |
| -Measurement quantity | RSRP |
| -Filter coefficient | 0 |
| -Inter-RAT reporting quantity (10.3.7.32) |  |
| - UTRAN estimated quality | FALSE |
| - CHOICE system | E-UTRA |
| - Reporting quantity | Measurement quantity |
| -Reporting cell status (10.3.7.61) |  |
| -CHOICE report criteria | Inter-RAT measurement reporting criteria |
| -Inter-RAT measurement reporting criteria (10.3.7.30) |  |
| -Parameters required for each event | 1 |
| -Inter-RAT event identity (10.3.7.24 | Event 3A |
| -Threshold own system | -9 |
| -W | 0 |
| -Threshold other system | -96 |
| -Hysteresis | 0 dB |
| -Time to trigger | 0 ms |
| -Reporting cell status (10.3.7.61) |  |
| -CHOICE reported cell | Report cells within active set or within virtual active set or of the other RAT |
| -Maximum number of reported cells | 2 |
| Physical channel information elements |  |
| -DPCH compressed mode status info (10.3.6.34) | If Compressed mode is required, Active . Otherwise , Not present. |
| - TGPS reconfiguration CFN | (Current CFN + (230 – TTI/10msec))mod 256 |
| -Transmission gap pattern sequence (1 to <MaxTGPS>) |  |
| - TGPSI | 1 |
| - TGPS Status Flag | activate |
| - TGCFN | (TGPS reconfiguration CFN +6)mod 256 |
| - TGPSI | 2 |
| - TGPS Status Flag | activate |
| - TGCFN | (TGPS reconfiguration CFN +10)mod 256 |

PHYSICAL CHANNEL RECONFIGURATION (compressed gaps):

| Information Element | Value/Remark | Version |
| --- | --- | --- |
| Message Type (10.2.22) |  |  |
| UE Information Elements |  |  |
| -RRC transaction identifier | 0 |  |
| -Integrity check info |  |  |
| -message authentication code | SS calculates the value of MAC-I for this message and writes to this IE. The first/ leftmost bit of the bit string contains the most significant bit of the MAC-I. |  |
| -RRC message sequence number | SS provides the value of this IE, from its internal counter. |  |
| -Integrity protection mode info | Not Present |  |
| -Ciphering mode info | Not Present |  |
| -Activation time | Not Present |  |
| -New U-RNTI | Not Present |  |
| -New C-RNTI | Not Present |  |
| -RRC State Indicator | CELL_DCH |  |
| -UTRAN DRX cycle length coefficient | Not Present |  |
| CN Information Elements |  |  |
| -CN Information info | Not Present |  |
| UTRAN mobility information elements |  |  |
| -URA identity | Not Present |  |
| RB information elements |  |  |
| -Downlink counter synchronisation info | Not Present |  |
| PhyCH information elements |  |  |
| -Frequency info (10.3.6.36) | Not Present |  |
| Uplink radio resources |  |  |
| -Maximum allowed UL TX power | 33 dBm |  |
| -CHOICE channel requirement | Not Present |  |
| Downlink radio resources |  |  |
| -CHOICE mode | FDD |  |
| -Downlink information common for all radio links (10.3.6.24) |  |  |
| -Downlink DPCH info common for all RL (10.3.6.18) | Not Present |  |
| -CHOICE mode | FDD |  |
| -DPCH compressed mode info (10.3.6.33) |  |  |
| - Transmission gap pattern sequence | (1/2 TGPS) |  |
| - TGPSI | 1 |  |
| - TGPS Status Flag | Deactivate |  |
| - Transmission gap pattern sequence configuration parameters |  |  |
| -TGMP | FDD measurement |  |
| -TGPRC | Infinity |  |
| -TGSN | 10 |  |
| -TGL1 | 10 |  |
| -TGL2 | Not Present |  |
| -TGD | UNDEFINED |  |
| -TGPL1 | 8 |  |
| -RPP | mode 0 |  |
| -ITP | mode 0 |  |
| -CHOICE UL/DL mode | UL and DL |  |
| -Downlink compressed mode method | SF/2 |  |
| -Uplink compressed mode method | SF/2 |  |
| -Downlink frame type | B |  |
| -DeltaSIR1 | 3.0 |  |
| -DeltaSIRafter1 | 3.0 |  |
| -DeltaSIR2 | Not Present |  |
| -DeltaSIRafter2 | Not Present |  |
| -N Identify abort | Not Present |  |
| -T Reconfirm abort | Not Present |  |
| - Transmission gap pattern sequence | (2/2 TGPS) |  |
| - TGPSI | 2 |  |
| - TGPS Status Flag | Deactivate |  |
| - Transmission gap pattern sequence configuration parameters |  |  |
| -TGMP | E-UTRA Measurement |  |
| -TGPRC | Infinity |  |
| -TGSN | 10 |  |
| -TGL1 | 10 |  |
| -TGL2 | Not Present |  |
| -TGD | UNDEFINED |  |
| -TGPL1 | 8 |  |
| -RPP | mode 0 |  |
| -ITP | mode 0 |  |
| -CHOICE UL/DL mode | UL and DL |  |
| -Downlink compressed mode method | SF/2 |  |
| -Uplink compressed mode method | SF/2 |  |
| -Downlink frame type | B |  |
| -DeltaSIR1 | 3.0 |  |
| -DeltaSIRafter1 | 3.0 |  |
| -DeltaSIR2 | Not Present |  |
| -DeltaSIRafter2 | Not Present |  |
| -N Identify abort | Not Present |  |
| -T Reconfirm abort | Not Present |  |
| -TX Diversity mode (10.3.6.86) | None |  |
| -Default DPCH Offset Value (10.3.6.16) | Not Present |  |
| -Downlink information per radio link list | 1 |  |
| -Downlink information for each radio link (10.3.6.27) |  |  |
| -CHOICE mode | FDD |  |
| -Primary CPICH info (10.3.6.60) |  |  |
| -Primary scrambling code | Set to Primary scrambling code of Cell1 |  |
| -Downlink DPCH info for each RL (10.3.6.21) |  |  |
| -CHOICE mode | FDD |  |
| -Primary CPICH usage for channel estimation | Primary CPICH may be used |  |
| -DPCH frame offset | Set to value Default DPCH Offset Value ( as currently stored in SS) mod 38400 |  |
| -Secondary CPICH info | Not Present |  |
| -DL channelisation code |  |  |
| -Secondary scrambling code | Not Present |  |
| -Spreading factor | 128 |  |
| -Code number | 96 |  |
| -Scrambling code change | No change |  |
| -TPC combination index | 0 |  |
| - Closed loop timing adjustment mode | Not Present |  |
| - SCCPCH information for FACH (10.3.6.70) | Not Present |  |

MEASUREMENT REPORT message for inter-frequency

This message shall use the same content as described in Annex I “Contents of MEASUREMENT REPORT message for Inter frequency test cases” with the following exceptions:

| Information Element | Value/remark |
| --- | --- |
| Message Type |  |
| Integrity check info | The presence of this IE is dependent on IXIT statements in TS 34.123-2. If integrity protection is indicated to be active, this IE shall be present with the values of the sub IEs as stated below. Else, this IE and the sub-IEs shall be absent. |
| - Message authentication code | This IE is checked to see if it is present. The value is compared against the XMAC-I value computed by SS. |
| - RRC Message sequence number | This IE is checked to see if it is present. The value is used by SS to compute the XMAC-I value. |
| Measurement identity | 1 |

MEASUREMENT REPORT message for inter-RAT, Event triggered

This message shall use the same content as described in Annex I “Contents of MEASUREMENT REPORT message for inter – RAT test cases” with the following exceptions:

| Information Element | Value/remark |
| --- | --- |
| Message Type |  |
| Integrity check info | The presence of this IE is dependent on IXIT statements in TS 34.123-2. If integrity protection is indicated to be active, this IE shall be present with the values of the sub IEs as stated below. Else, this IE and the sub-IEs shall be absent. |
| - Message authentication code | This IE is checked to see if it is present. The value is compared against the XMAC-I value computed by SS. |
| - RRC Message sequence number | This IE is checked to see if it is present. The value is used by SS to compute the XMAC-I value. |
| Measurement identity | 2 |

##### 8.6.7.2.5	Test requirements

Table 8.6.7.2.4.4: Cell Specific parameters for Correct reporting of neighbours in Fading propagation condition

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| UTRA RF Channel Number |  | Channel 1 |  | Channel 2 |  |
| CPICH_Ec/Ior | dB | -10 + TT |  | -10 + TT |  |
| PCCPCH_Ec/Ior | dB | -12 + TT |  | -12 + TT |  |
| SCH_Ec/Ior | dB | -12 + TT |  | -12 +TT |  |
| PICH_Ec/Ior | dB | -15 + TT |  | -15 + TT |  |
| DPCH_Ec/Ior | dB | Note 1 |  | N/A |  |
| OCNS |  | Note 2 |  | -0.941 |  |
|  | dB | 0 |  | -Infinity | -1.8 |
|  | dBm/3,84 MHz | -70 |  | -70 |  |
| CPICH_Ec/Io | dB | -13 + TT |  | -Infinity | -14 + TT |
| Propagation Condition |  | AWGN |  | Case 5 as specified in Annex B of TS25.101 |  |
| Note 1: The DPCH level is controlled by the power control loop Note 2: The power of the OCNS channel that is added shall make the total power from the cell to be equal to Ior |  |  |  |  |  |

# Table 8.6.7.2.4.5: Cell specific test parameters for Correct reporting of E-UTRAN TDD neighbours in fading propagation condition (cell 3)

| Parameter | Unit | Cell 3 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| OCNG Pattern defined in A.3.2.2.2 in 3GPP TS 36.133 (OP.2 TDD) |  | OP.2 TDD |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
| Note 2 | dBm/15 kHz | -98 |  |
|  | dB | -Infinity | 8 + TT |
|  | dB | -Infinity | 8 +TT |
| RSRP Note 3 | dBm/15 kHz | -Infinity | -90 + TT |
| SCH_RP | dBm/15 kHz | -Infinity | -90 + TT |
| Propagation Condition |  | ETU70 (Note 4) |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. OCNG patterns are specified in annex A 3.2 of 3GPP TS 36.133. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: ETU70 propagation conditions are specified in Annex B.2 of 3GPP TS 36.101. |  |  |  |

For the test to pass, the total number of successful tests shall be at least 90% of the cases, with a confidence level of 95%. For the overall outcome of the test to be considered a pass, the counters for 2B events, 3A events and shall independently indicate a pass result.

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause F.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in clause F.4.