3GPP TSG-RAN-WG5 Meeting #50	R5-110298

# Taipei, China, 21-25 February 2011

| CR-Form-v9.7 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.121-1 | CR | CRNum | rev | - | Current version: | 9.3.0 |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to test case 8.3.4d |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE-UEConTest_RF |  |  |  |  |  |  | Date: |  | 02/08/2011 |
| Category: |  |  |  |  |  |  |  | Release: |  | Rel.-9 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change: |  | To Update the RRC procedure delay and Hard Handover Delay |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating section 8.3.4d.2 and table 8.3.4d.3 in compliance to core requirements. Corrected Total Interruption time |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case would not be completed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.4d.2 and table 8.3.4d.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  |  |  |  |
| affected: |  |  |  | Test specifications |  |  |  |  |  |  |
|  |  |  |  | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | This CR applies from Rel-8 and later releases |  |  |  |  |  |  |  |  |

[Many Sections are skipped here]

### 8.3.4d	Inter-system Handover from UTRAN FDD to E-UTRAN TDD; Unknown Target Cell

Editor’s note: This section is incomplete. The following aspects are either missing or not yet determined:

- The Test system uncertainties applicable to this test are undefined

- The Test tolerances applicable to this test are undefined

- Statistical testing of handover delay performance requirements is undefined

#### 8.3.4d.1	Definition and applicability

The UTRAN FDD to E-UTRAN TDD cell handover delay is defined as the time from the end of the last TTI containing an RRC message implying hard handover to the transmission on the channel of the new RAT.

The requirements and this test apply to the combined UTRAN FDD and E-UTRAN TDD UE.

#### 8.3.4d.2	Minimum requirement

The UE shall begin to send PRACH to the target cell less than 165 ms from the beginning of time period T2.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The test requirement in this case is expressed as:

- The total HO delay is the sum of the RRC procedure delay and interruption time.

- The RRC procedure delay is 50 ms.

- The interruption time is defined as follows:

Tinterrupt = Tsearch + TIU +20 ms

Tsearch:	is the time required to search the target cell when the target cell is not already known when the handover command is received by the UE. Tsearch is 80 ms

TIU:	is the interruption uncertainty in acquiring the first available UpPTS or PRACH occasion in the new cell. TIU can be up to 30 ms. It is chosen 15 ms since the PRACH configuration used in the target cell (E-UTRAN) is assumed to be 53 for TDD respectively as specified in table 5.7.1-2 and 5.7.1-4 in TS 36.211.

The total interruption time is 115 ms.

Thus the total HO delay is 165 ms.

The UE shall start to transmit the PRACH to Cell 2 with a handover delay less than 165ms.

The normative reference for this requirement is TS 25.133 [2] clauses 5.4b.2 and A.5.4d.

#### 8.3.4d.3	Test purpose

To verify that the UE meets the minimum requirement.

#### 8.3.4d.4	Method of test

##### 8.3.4d.4.1	Initial conditions

Test environment: normal; see clauses G.2.1 and G.2.2.

Frequencies to be tested: mid range; see clause G.2.4.

The test parameters are given in Table 8.3.4d.1, 8.3.4d.2 and 8.3.4d.3 below. The test consists of two successive time periods, with time durations of T1 and T2 respectively. At the start of time duration T1, the UE does not have any timing information of Cell 2. No compressed mode patterns are configured in the test case.

The UTRAN shall send a Handover from UTRAN command with activation time "now" with a new active cell, cell 2. In the E-UTRAN Handover command contained in that message, the IE starting time shall not be included. The RRC HANDOVER FROM UTRAN COMMAND message shall be sent to the UE towards the end of the time period T1. The start of T2 is defined as the end of last TTI containing the handover command.

Table 8.3.4d.1: General test parameters for UTRAN FDD to E-UTRAN TDD handover test case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| DCH parameters |  | DL Reference Measurement Channel 12.2 kbps | As specified in TS 25.101 section A.3.1 |
| Power Control |  | On |  |
| Target quality value on DTCH | BLER | 0.001 |  |
| Initial condition | Active cell | Cell 1 | UTRAN FDD |
|  | Neighbouring cell | Cell 2 | E-UTRAN TDD |
| Final condition | Active cell | Cell 2 | E-UTRAN TDD |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 TDD | As specified in 3GPP TS 36.521-1 section A.3.1.1.2 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 TDD | As specified in 3GPP TS 36.521-1 section A.3.1.2.2 |
| PRACH configuration |  | s | As specified in table 5.7.1-2 in 3GPP TS 36.211 |
| Access Barring Information | - | Not sent | No additional delays in random access procedure |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in 3GPP TS 36.211 |
| Uplink-downlink configuration |  | 1 | As specified in table 4.2-2 in 3GPP TS 36.211 |
| PRACH configuration |  | 53 | As specified in table 5.7.1-3 in 3GPP TS 36.211 |
| T1 | s | 5 |  |
| T2 | s | 1 |  |

Table 8.3.4d.2: Cell specific test parameters for UTRAN FDD to E-UTRAN TDD handover test case (cell 1)

| Parameter | Unit |  | Cell 1 (UTRA FDD) |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1, T2 |  |
| CPICH_Ec/Ior | dB | -10 |  |
| PCCPCH_Ec/Ior | dB | -12 |  |
| SCH_Ec/Ior | dB | -12 |  |
| PICH_Ec/Ior | dB | -15 |  |
| DCH_Ec/Ior | dB | Note 1 |  |
| OCNS_Ec/Ior | dB | Note 2 |  |
|  | dB | 0 + TT |  |
|  | dBm/3,84 MHz | -70 |  |
| CPICH_Ec/Io | dB | -13 + TT |  |
| Propagation Condition |  | AWGN |  |
| Note 1: The DPCH level is controlled by the power control loop Note 2: The power of the OCNS channel that is added shall make the total power from the cell to be equal to Ior. |  |  |  |

Table 8.3.4d.3: Cell specific test parameters for UTRAN FDD to E-UTRAN TDD handover test case (cell 2)

| Parameter | Unit | Cell 2 (E-UTRA TDD) |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| OCNG Patterns defined in 3GPP 36.521-1 Section A.3.2.2.1 (OP.1 TDD) and in A.3.2.2.2 (OP.2 TDD) |  | OP.2 TDD | OP.1 TDD |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
|  | dB | -infinity | 0 + TT |
| Note 2 | dBm/15 kHz | -98 |  |
|  | dB | -infinity | 0 + TT |
| RSRP Note 3 | dBm/15 KHz | -infinity | -98 + TT |
| Propagation Condition |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |

##### 8.3.4d.4.2	Procedure

- The RF parameters for cell 1 are set up according to T1 in Table 8.3.4d.2.

- The UE is switched on.

- A call is set up according to the test procedure specified in TS 34.108 [3] subclause 7.3.7. Data shall be sent on the DTCH on the UTRA cell throughout the call.

- The RF parameters for cell 2 are set up according to T1 in Table 8.3.4d.3.

- The start of T1 is TTI aligned.

- The SS shall transmit a HANDOVER FROM UTRAN COMMAND message with activation time "now" with a new active cell, cell 2. The IE starting time shall not be included in the E-UTRAN handover command. The start of T2 is defined as the end of the last TTI, containing the HANDOVER command.

- The UE shall transmit a PRACH to cell 2 implying that it has switched to the E-UTRAN TDD cell. If the UE transmits the PRACH to cell2 less than 165 ms from the beginning of time period T2, then the number of successful tests is increased by one.

- At the end of T2 SS shall end the call and UE is switched off. Any timing information of cell 2 is deleted in the UE.

- Repeat step 1-7 until the confidence level according to annex F.6.2 is achieved.

Specific Message Contents

All messages indicated above shall use the same content as described in the default message content in clause 9 of 34.108 [3], with the following exceptions:

HANDOVER FROM UTRAN COMMAND message (step 6):

| Information Element | Value/remark |
| --- | --- |
| Message Type (10.2.15 in TS25.331) |  |
| UE information elements |  |
| -RRC transaction identifier | 0 |
| -Integrity check info |  |
| -message authentication code | SS calculates the value of MAC-I for this message and writes to this IE. The first/ leftmost bit of the bit string contains the most significant bit of the MAC-I. |
| -RRC message sequence number | SS provides the value of this IE, from its internal counter. |
| -Activation time | Now |
| RB information elements |  |
| -RAB information list | 1 |
| -RAB Info |  |
| - RAB identity | 0000 0001B The first/ leftmost bit of the bit string contains the most significant bit of the RAB identity. |
| - CN domain identity | CS domain |
| - NAS Synchronization Indicator | Not present |
| - Re-establishment timer | Use T315 |
| Other information elements |  |
| -CHOICE System type | E-UTRA |
| -E-UTRA message | Formatted and coded according to E-UTRA specifications. The first/leftmost/most significant bit of the octet string contains bit 8 of the first octet of the E-UTRA message. |

HANDOVER COMMAND

| Derivation Path: TS 36.508 [33] clause 4.6.1, Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |  |
| nas-SecurityParamToEUTRA |  | OCTET STRING (SIZE(6)) | This field is used to activate NAS security after inter-RAT handover to E-UTRA. The content is defined in TS 24.301. | I-RATHO |
| mobilityControlInfo |  | mobilityControlInfo |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |  |
| targetPhysCellId |  | PhysCellId |  |  |
| carrierFreq |  | CarrierFreqEUTRA |  |  |
| carrierBandwidth |  | CarrierBandwidthEUTRA |  |  |
| additionalSpectrumEmission |  | AdditionalSpectrumEmission |  |  |
| ... |  |  |  |  |
| } |  |  |  |  |

#### 8.3.4d.5	Test requirements

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95 %.

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause F.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in clause F.4.

[END OF CHANGE]