3GPP TSG-RAN WG5 Meeting #57 	R5-125237

# Baltimore, USA, November 5th-9th, 2012

| CR-Form-v10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.108 | CR | CRNum | rev | - | Current version: | 11.3.0 |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to default RADIO BEARER SETUP message (FDD) condition A28a for support of MIMO/Non-MIMO Co-existence Tests |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI7_Test |  |  |  |  |  |  | Date: |  | 05/11/2012 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-11 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change: |  | Test case 8.2.2.71 and 8.2.2.72 uses RBsetup message based on A28a which has user RB-id configured on DCH/ HS-DSCH. In the Expected Sequence and specific message contents SRBs are on E-DCH/HS-DSCH. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modify uplink configuration from DCH to E-DCH by redfining condition A28a. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect uplink configuration will remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS/TR ... CR ... 34.123-1 |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### 9.1.1	Default RRC Message Contents (FDD)

<skipped>

###### Contents of RADIO BEARER SETUP message: AM or UM

| Information Element |  |  | Condition |  | Value/remark | Version |  | Index |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type |  |  | A1, A2, A3, A4, A5, A6, A7, A8, A11 |  |  |  |  | RBS-003 |  |
|  |  |  | , A9, A10 |  |  | Rel-5 |  | RBS-004 |  |
|  |  |  | , A12, A13, A14, A15, A16 |  |  | Rel-6 |  | RBS-005 |  |
|  |  |  | , A17, A17a, A17b, A17c, A17d, A17e, A18, A19, A19a, A20, A21, A22 |  |  | Rel-7 |  | RBS-006 |  |
|  |  |  | , A23, A24,A28a |  |  | Rel-7 Rel-8 |  | RBS-007 |  |
|  |  |  | , A25, A25a, A25b, A26, A27, A28, A29, A30 |  |  | Rel-8 |  | RBS-008 |  |
|  |  |  | , A25c, A31, A32, A33, A34 |  |  | Rel-9 Rel-10 |  | RBS-009 |  |
| RRC transaction identifier |  |  |  |  | Arbitrarily selects an integer between 0 and 3 |  |  | RBS-010 |  |
| Integrity check info |  |  |  |  |  |  |  | RBS-011 |  |
| - message authentication code |  |  |  |  | SS calculates the value of MAC-I for this message and writes to this IE. The first/ leftmost bit of the bit string contains the most significant bit of the MAC-I. |  |  | RBS-012 |  |
| - RRC message sequence number |  |  |  |  | SS provides the value of this IE, from its internal counter. |  |  | RBS-013 |  |
| Integrity protection mode info |  |  |  |  | Not Present |  |  | RBS-014 |  |
| Ciphering mode info |  |  |  |  | Not Present |  |  | RBS-015 |  |
| Sr-vcc-Info |  |  |  |  | Not Present |  |  |  |  |
| Activation time |  |  | A1, A2, A3, A11 |  | (256+CFN-(CFN MOD 8 + 8))MOD 256 |  |  | RBS-016 |  |
|  |  |  | , A9 |  |  | Rel-5 |  | RBS-017 |  |
|  |  |  | , A12, A13, A14, A15, A16 |  |  | Rel-6 |  | RBS-018 |  |
|  |  |  | , A17, A17a, A17b, A17d, A17e, A18, A19, A19a, A20, A21, A22 |  |  | Rel-7 |  | RBS-019 |  |
|  |  |  | , A23, A28a |  |  | Rel-7 Rel-8 |  | RBS-020 |  |
|  |  |  | , A25, A25a, A25b, A26, A27, A27a, A28, A30 |  |  | Rel-8 |  | RBS-021 |  |
| Activation time |  |  | A4, A5, A6, A7, A8 |  | Not Present |  |  | RBS-022 |  |
|  |  |  | A10, A24, |  |  | Rel-5 |  | RBS-023 |  |
|  |  |  | A29 |  |  | Rel-8 |  | RBS-024 |  |
|  |  |  | , A25c, A31, A32, A33, A34 |  |  | Rel-9 Rel-10 |  | RBS-025 |  |
| New U-RNTI |  |  | A1, A2, A3, A4, A5, A6, A7, A8, A11 |  | Not Present |  |  | RBS-026 |  |
|  |  |  | , A9, A10 |  |  | Rel-5 |  | RBS-027 |  |
|  |  |  | , A12, A13, A14, A15, A16 |  |  | Rel-6 |  | RBS-028 |  |
|  |  |  | , A17, A17a, A17b, A17c, A17d, A17e, A18, A19, A19a, A20, A21, A22, A24 |  |  | Rel-7 |  | RBS-029 |  |
|  |  |  | , A23, A28a |  |  | Rel-7 Rel-8 |  | RBS-030 |  |
|  |  |  | , A25, A25a, A25b, A26, A27, A27a, A28, A29, A30 |  |  | Rel-8 |  | RBS-031 |  |
|  |  |  | , A25c, A31, A32, A33, A34 |  |  | Rel-9 Rel-10 |  | RBS-032 |  |
| New C-RNTI |  |  | A1, A2, A3, A4, A7, A8, A11 |  | Not Present |  |  | RBS-033 |  |
|  |  |  | , A9, A10 |  |  | Rel-5 |  | RBS-034 |  |
|  |  |  | , A12, A13, A14, A15, A16 |  |  | Rel-6 |  | RBS-035 |  |
|  |  |  | , A17, A17a, A17b, A17c, A17d, A17e, A18, A19, A19a, A20, A21, A22, A24 |  |  | Rel-7 |  | RBS-036 |  |
|  |  |  | , A23, A28a |  |  | Rel-7 Rel-8 |  | RBS-037 |  |
|  |  |  | , A25, A25a, A25b, A26, A27, A27a, A28, A29, A30 |  |  | Rel-8 |  | RBS-038 |  |
|  |  |  | , A25c, A31, A32, A33, A34 |  |  | Rel-9 Rel-10 |  | RBS-039 |  |
| New C-RNTI |  |  | A5, A6 |  | '1010 1010 1010 1010' |  |  | RBS-040 |  |
| New DSCH-RNTI |  |  | A1, A2, A3, A4, A5, A6, A7, A8, A11 |  | Not Present | R99 and Rel-4 only |  | RBS-041 |  |
| New H-RNTI |  |  | A1, A2, A3, A4, A5, A6, A7, A8, A11 |  | Not Present | Rel-5 |  | RBS-042 |  |
|  |  |  | , A29, A30 |  |  |  |  | RBS-043 |  |
|  |  |  | , A31, A32, A33, A34 |  |  | Rel-9 Rel-10 |  | RBS-044 |  |
| New H-RNTI |  |  | A9, A10 |  | '1010 1010 1010 1010' | Rel-5 |  | RBS-045 |  |
|  |  |  | A12, A13, A14, A15, A16 |  |  | Rel-6 |  | RBS-046 |  |
|  |  |  | , A17, A17a, A17b, A17c, A17d, A17e, A18, A19, A19a, A20, A21, A22, A24 |  |  | Rel-7 |  | RBS-047 |  |
|  |  |  | , A23, A28a |  |  | Rel-7 Rel-8 |  | RBS-048 |  |
|  |  |  | , A25, A25a, A25b, A26, A27, A27a, A28 |  |  | Rel-8 |  | RBS-049 |  |
|  |  |  | , A25c, |  |  | Rel-9 |  | RBS-049b |  |
| New Primary E-RNTI |  |  | A1, A2, A3, A4, A5, A6, A7, A8, A9, A10, A11 |  | Not Present | Rel-6 |  | RBS-050 |  |
|  |  |  | , A17, A17a, A18, A24, A28a |  |  | Rel-7 |  | RBS-051 |  |
|  |  |  | , A25, A25a, A25b, A28, A29, A30 |  |  | Rel-8 |  | RBS-052 |  |
|  |  |  | , A31, A32, A33, A34 |  |  | Rel-9 Rel-10 |  | RBS-053 |  |
| New Primary E-RNTI |  |  | A12, A13, A14, A15, A16, A17b, A17c, A17d, A17e |  | '1010 1010 1010 1010' | Rel-6 |  | RBS-054 |  |
|  |  |  | A19, A19a, A20, A21, A22 |  |  | Rel-7 |  | RBS-055 |  |
|  |  |  | , A23 |  |  | Rel-7 Rel-8 |  | RBS-056 |  |
|  |  |  | A26, A27, A27a |  |  | Rel-8 |  | RBS-057 |  |
|  |  |  | A25c |  |  | Rel-9 |  | RBS-057b |  |
| New Secondary E-RNTI |  |  | A1, A2, A3, A4, A5, A6, A7, A8, A9, A10, A11, A12, A13, A14, A15, A16 |  | Not Present | Rel-6 |  | RBS-058 |  |
|  |  |  | , A17, A17a, A17b, A17c, A17d, A17e, A18, A19, A19a, A20, A21, A22, A24 |  |  | Rel-7 |  | RBS-059 |  |
|  |  |  | , A23, A28a |  |  | Rel-7 Rel-8 |  | RBS-060 |  |
|  |  |  | , A25, A25a, A25b, A26, A27, A27a, A28, A29, A30 |  |  | Rel-8 |  | RBS-061 |  |
|  |  |  | , A31, A32, A33, A34 |  |  | Rel-9 Rel-10 |  | RBS-062 |  |
| RRC State indicator |  |  | A1, A2, A3, A4, A7, A8, A11 |  | CELL_DCH |  |  | RBS-063 |  |
|  |  |  | , A9, A10 |  |  | Rel-5 |  | RBS-064 |  |
|  |  |  | , A12, A13, A14, A15, A16 |  |  | Rel-6 |  | RBS-065 |  |
|  |  |  | , A17, A17a, A17b, A17c, A17d, A17e, A18, A19, A19a, A20, A21, A22 |  |  | Rel-7 |  | RBS-066 |  |
|  |  |  | , A23, A28a |  |  | Rel-7 Rel-8 |  | RBS-067 |  |
|  |  |  | , A25, A25a, A25b, A26, A27, A27a, A28, A30 |  |  | Rel-8 |  | RBS-068 |  |
|  |  |  | , A25c, A31, A32, A33, A34 |  |  | Rel-9 Rel-10 |  | RBS-069 |  |
| RRC State indicator |  |  | A5, A6, |  | CELL_FACH |  |  | RBS-070 |  |
|  |  |  | A24 |  |  | Rel-7 |  | RBS-071 |  |
|  |  |  | A29 |  |  |  |  | RBS-072 |  |
| UTRAN DRX cycle length coefficient |  |  | A1, A2, A3, A4, A5, A6, A7, A8, A11 |  | Not Present |  |  | RBS-073 |  |
|  |  |  | , A9, A10 |  |  | Rel-5 |  | RBS-074 |  |
|  |  |  | , A12, A13, A14, A15, A16 |  |  | Rel-6 |  | RBS-075 |  |
|  |  |  | , A17, A17a, A17b, A17c, A17d, A17e, A18, A19, A19a, A20, A21, A22, A24 |  |  | Rel-7 |  | RBS-076 |  |
|  |  |  | , A23, A28a |  |  | Rel-7 Rel-8 |  | RBS-077 |  |
|  |  |  | , A25, A25a, A25b, A26, A27, A27a, A28, A29, A30 |  |  | Rel-8 |  | RBS-078 |  |
|  |  |  | , A25c, A31, A32, A33, A34 |  |  | Rel-9 Rel-10 |  | RBS-079 |  |
| CN information info |  |  |  |  | Not Present |  |  | RBS-080 |  |
| URA identity |  |  |  |  | Not Present |  |  | RBS-081 |  |
| RNC support for change of UE capability |  |  |  |  | Not Present | Rel-7 |  | RBS-082 |  |
| CHOICE Specification mode |  |  |  |  | Complete specification | Rel-6 |  | RBS-083 |  |
| - Signalling RB information to setup |  |  |  |  | Not Present |  |  | RBS-084 |  |
| - RAB information for setup |  |  | A1, A7 |  |  |  |  | RBS-085 |  |
| - RAB info |  |  |  |  |  |  |  | RBS-086 |  |
| - RAB identity |  |  |  |  | 0000 0001B The first/ leftmost bit of the bit string contains the most significant bit of the RAB identity. |  |  | RBS-087 |  |
| - CN domain identity |  |  |  |  | CS domain |  |  | RBS-088 |  |
| - NAS Synchronization Indicator |  |  |  |  | Not Present |  |  | RBS-089 |  |
| - Re-establishment timer |  |  |  |  | useT314 |  |  | RBS-090 |  |
| - RB information to setup |  |  |  |  |  |  |  | RBS-091 |  |
| - RB identity |  |  |  |  | 10 |  |  | RBS-092 |  |
| - PDCP info |  |  |  |  | Not Present |  |  | RBS-093 |  |
| - CHOICE RLC info type |  |  |  |  | RLC info |  |  | RBS-094 |  |
| - CHOICE Uplink RLC mode |  |  |  |  | TM RLC |  |  | RBS-095 |  |
| - Transmission RLC discard |  |  |  |  | Not Present |  |  | RBS-096 |  |
| - Segmentation indication |  |  |  |  | FALSE |  |  | RBS-097 |  |
| - CHOICE Downlink RLC mode |  |  |  |  | TM RLC |  |  | RBS-098 |  |
| - Segmentation indication |  |  |  |  | FALSE |  |  | RBS-099 |  |
| - RB mapping info |  |  |  |  |  |  |  | RBS-100 |  |
| - Information for each multiplexing option |  |  |  |  |  |  |  | RBS-101 |  |
| - RLC logical channel mapping indicator |  |  |  |  | Not Present |  |  | RBS-102 |  |
| - Number of uplink RLC logical channels |  |  |  |  | 1 |  |  | RBS-103 |  |
| - Uplink transport channel type |  |  |  |  | DCH |  |  | RBS-104 |  |
| - UL Transport channel identity |  |  |  |  | 1 |  |  | RBS-105 |  |
| - Logical channel identity |  |  |  |  | Not Present |  |  | RBS-106 |  |
| - CHOICE RLC size list |  |  |  |  | Configured |  |  | RBS-107 |  |
| - MAC logical channel priority |  |  |  |  | 7 |  |  | RBS-108 |  |
| - Downlink RLC logical channel info |  |  |  |  |  |  |  | RBS-109 |  |
| - Number of downlink RLC logical channels |  |  |  |  | 1 |  |  | RBS-110 |  |
| - Downlink transport channel type |  |  |  |  | DCH |  |  | RBS-111 |  |
| - DL DCH Transport channel identity |  |  |  |  | 6 |  |  | RBS-112 |  |
| - DL DSCH Transport channel identity |  |  |  |  | Not Present |  |  | RBS-113 |  |
| - Logical channel identity |  |  |  |  | Not Present |  |  | RBS-114 |  |
| - RAB information for setup |  |  | A2, A8 |  |  |  |  | RBS-115 |  |
| - RAB info |  |  |  |  |  |  |  | RBS-116 |  |
| - RAB identity |  |  |  |  | 0000 0001B The first/ leftmost bit of the bit string contains the most significant bit of the RAB identity. |  |  | RBS-117 |  |
| - CN domain identity |  |  |  |  | CS domain |  |  | RBS-118 |  |
| - NAS Synchronization Indicator |  |  |  |  | Not Present |  |  | RBS-119 |  |
| - Re-establishment timer |  |  |  |  | useT314 |  |  | RBS-120 |  |
| - RB information to setup |  |  |  |  |  |  |  | RBS-121 |  |
| - RB identity |  |  |  |  | 10 |  |  | RBS-122 |  |
| - PDCP info |  |  |  |  | Not Present |  |  | RBS-123 |  |
| - CHOICE RLC info type |  |  |  |  | RLC info |  |  | RBS-124 |  |
| - CHOICE Uplink RLC mode |  |  |  |  | TM RLC |  |  | RBS-125 |  |
| - Transmission RLC discard |  |  |  |  | Not Present |  |  | RBS-126 |  |
| - Segmentation indication |  |  |  |  | FALSE |  |  | RBS-127 |  |
| - CHOICE Downlink RLC mode |  |  |  |  | TM RLC |  |  | RBS-128 |  |
| - Segmentation indication |  |  |  |  | FALSE |  |  | RBS-129 |  |
| - RB mapping info |  |  |  |  |  |  |  | RBS-130 |  |
| - Information for each multiplexing option |  |  |  |  |  |  |  | RBS-131 |  |
| - RLC logical channel mapping indicator |  |  |  |  | Not Present |  |  | RBS-132 |  |
| - Number of uplink RLC logical channels |  |  |  |  | 1 |  |  | RBS-133 |  |
| - Uplink transport channel type |  |  |  |  | DCH |  |  | RBS-134 |  |
| - UL Transport channel identity |  |  |  |  | 1 |  |  | RBS-135 |  |
| - Logical channel identity |  |  |  |  | Not Present |  |  | RBS-136 |  |
| - CHOICE RLC size list |  |  |  |  | Configured |  |  | RBS-137 |  |
| - MAC logical channel priority |  |  |  |  | 6 |  |  | RBS-138 |  |
| - Downlink RLC logical channel info |  |  |  |  |  |  |  | RBS-139 |  |
| - Number of downlink RLC logical channels |  |  |  |  | 1 |  |  | RBS-140 |  |
| - Downlink transport channel type |  |  |  |  | DCH |  |  | RBS-141 |  |
| - DL DCH Transport channel identity |  |  |  |  | 6 |  |  | RBS-142 |  |
| - DL DSCH Transport channel identity |  |  |  |  | Not Present |  |  | RBS-143 |  |
| - Logical channel identity |  |  |  |  | Not Present |  |  | RBS-144 |  |
| - RB identity |  |  |  |  | 11 |  |  | RBS-145 |  |
| - PDCP info |  |  |  |  | Not Present |  |  | RBS-146 |  |
| - CHOICE RLC info type |  |  |  |  | RLC info |  |  | RBS-147 |  |
| - CHOICE Uplink RLC mode |  |  |  |  | TM RLC |  |  | RBS-148 |  |
| - Transmission RLC discard |  |  |  |  | Not Present |  |  | RBS-149 |  |
| - Segmentation indication |  |  |  |  | FALSE |  |  | RBS-150 |  |
| - CHOICE Downlink RLC mode |  |  |  |  | TM RLC |  |  | RBS-151 |  |
| - Segmentation indication |  |  |  |  | FALSE |  |  | RBS-152 |  |
| - RB mapping info |  |  |  |  |  |  |  | RBS-153 |  |
| - Information for each multiplexing option |  |  |  |  |  |  |  | RBS-154 |  |
| - RLC logical channel mapping indicator |  |  |  |  | Not Present |  |  | RBS-155 |  |
| - Number of uplink RLC logical channels |  |  |  |  | 1 |  |  | RBS-156 |  |
| - Uplink transport channel type |  |  |  |  | DCH |  |  | RBS-157 |  |
| - UL Transport channel identity |  |  |  |  | 2 |  |  | RBS-158 |  |
| - Logical channel identity |  |  |  |  | Not Present |  |  | RBS-159 |  |
| - CHOICE RLC size list |  |  |  |  | Configured |  |  | RBS-160 |  |
| - MAC logical channel priority |  |  |  |  | 6 |  |  | RBS-161 |  |
| - Downlink RLC logical channel info |  |  |  |  |  |  |  | RBS-162 |  |
| - Number of downlink RLC logical channels |  |  |  |  | 1 |  |  | RBS-163 |  |
| - Downlink transport channel type |  |  |  |  | DCH |  |  | RBS-164 |  |
| - DL DCH Transport channel identity |  |  |  |  | 7 |  |  | RBS-165 |  |
| - DL DSCH Transport channel identity |  |  |  |  | Not Present |  |  | RBS-166 |  |
| - Logical channel identity |  |  |  |  | Not Present |  |  | RBS-167 |  |
| - RB identity |  |  |  |  | 12 |  |  | RBS-168 |  |
| - PDCP info |  |  |  |  | Not Present |  |  | RBS-169 |  |
| - CHOICE RLC info type |  |  |  |  | RLC info |  |  | RBS-170 |  |
| - CHOICE Uplink RLC mode |  |  |  |  | TM RLC |  |  | RBS-171 |  |
| - Transmission RLC discard |  |  |  |  | Not Present |  |  | RBS-172 |  |
| - Segmentation indication |  |  |  |  | FALSE |  |  | RBS-173 |  |
| - CHOICE Downlink RLC mode |  |  |  |  | TM RLC |  |  | RBS-174 |  |
| - Segmentation indication |  |  |  |  | FALSE |  |  | RBS-175 |  |
| - RB mapping info |  |  |  |  |  |  |  | RBS-176 |  |
| - Information for each multiplexing option |  |  |  |  |  |  |  | RBS-177 |  |
| - RLC logical channel mapping indicator |  |  |  |  | Not Present |  |  | RBS-178 |  |
| - Number of uplink RLC logical channels |  |  |  |  | 1 |  |  | RBS-179 |  |
| - Uplink transport channel type |  |  |  |  | DCH |  |  | RBS-180 |  |
| - UL Transport channel identity |  |  |  |  | 3 |  |  | RBS-181 |  |
| - Logical channel identity |  |  |  |  | Not Present |  |  | RBS-182 |  |
| - CHOICE RLC size list |  |  |  |  | Configured |  |  | RBS-183 |  |
| - MAC logical channel priority |  |  |  |  | 6 |  |  | RBS-184 |  |
| - Downlink RLC logical channel info |  |  |  |  |  |  |  | RBS-185 |  |
| - Number of downlink RLC logical channels |  |  |  |  | 1 |  |  | RBS-186 |  |
| - Downlink transport channel type |  |  |  |  | DCH |  |  | RBS-187 |  |
| - DL DCH Transport channel identity |  |  |  |  | 8 |  |  | RBS-188 |  |
| - DL DSCH Transport channel identity |  |  |  |  | Not Present |  |  | RBS-189 |  |
| - Logical channel identity |  |  |  |  | Not Present |  |  | RBS-190 |  |
| - RAB information for setup |  |  | A3, A4, A5, A6 |  |  |  |  | RBS-191 |  |
| - RAB info |  |  |  |  | (AM DTCH for PS domain) |  |  | RBS-192 |  |
| - RAB identity |  |  |  |  | 0000 0101B The first/ leftmost bit of the bit string contains the most significant bit of the RAB identity. |  |  | RBS-193 |  |
| - CN domain identity |  |  |  |  | PS domain |  |  | RBS-194 |  |
| - NAS Synchronization Indicator |  |  |  |  | Not Present |  |  | RBS-195 |  |
| - Re-establishment timer |  |  |  |  | useT315 |  |  | RBS-196 |  |
| - RB information to setup |  |  |  |  |  |  |  | RBS-197 |  |
| - RB identity |  |  |  |  | 20 |  |  | RBS-198 |  |
| - PDCP info |  |  |  |  |  |  |  | RBS-199 |  |
| - Support for lossless SRNS relocation |  |  |  |  | FALSE |  |  | RBS-200 |  |
| - Max PDCP SN window size |  |  |  |  | Not present |  |  | RBS-201 |  |
| - PDCP PDU header |  |  |  |  | Absent |  |  | RBS-202 |  |
| - Header compression information |  |  |  |  | Not present |  |  | RBS-203 |  |
| - CHOICE RLC info type |  |  |  |  | RLC info |  |  | RBS-204 |  |
| - CHOICE Uplink RLC mode |  |  |  |  | AM RLC |  |  | RBS-205 |  |
| - Transmission RLC discard |  |  |  |  |  |  |  | RBS-206 |  |
| - CHOICE SDU discard mode |  |  |  |  | No Discard |  |  | RBS-207 |  |
| - MAX_DAT |  |  |  |  | 15 |  |  | RBS-208 |  |
| - Transmission window size |  |  |  |  | 128 |  |  | RBS-209 |  |
| - Timer_RST |  |  |  |  | 500 |  |  | RBS-210 |  |
| - Max_RST |  |  |  |  | 4 |  |  | RBS-211 |  |
| - Polling info |  |  |  |  |  |  |  | RBS-212 |  |
| - Timer_poll_prohibit |  |  |  |  | 200 |  |  | RBS-213 |  |
| - Timer_poll |  |  |  |  | 200 |  |  | RBS-214 |  |
| - Poll_PDU |  |  |  |  | Not Present |  |  | RBS-215 |  |
| - Poll_SDU |  |  |  |  | 1 |  |  | RBS-216 |  |
| - Last transmission PDU poll |  |  |  |  | TRUE |  |  | RBS-217 |  |
| - Last retransmission PDU poll |  |  |  |  | TRUE |  |  | RBS-218 |  |
| - Poll_Windows |  |  |  |  | 99 |  |  | RBS-219 |  |
| - Timer_poll_periodic |  |  |  |  | Not Present |  |  | RBS-220 |  |
| - CHOICE Downlink RLC mode |  |  |  |  | AM RLC |  |  | RBS-221 |  |
| - CHOICE Downlink RLC PDU Size |  |  |  |  | Reference to clause 6 Parameter Set | Rel-5 |  | RBS-222 |  |
| - In-sequence delivery |  |  |  |  | TRUE |  |  | RBS-223 |  |
| - Receiving window size |  |  |  |  | 128 |  |  | RBS-224 |  |
| - Downlink RLC status info |  |  |  |  |  |  |  | RBS-225 |  |
| - Timer_status_prohibit |  |  |  |  | 200 |  |  | RBS-226 |  |
| - Timer_EPC |  |  |  |  | Not Present |  |  | RBS-227 |  |
| - Missing PDU indicator |  |  |  |  | TRUE |  |  | RBS-228 |  |
| - Timer_STATUS_periodic |  |  |  |  | Not Present |  |  | RBS-229 |  |
| - RB mapping info |  |  |  |  |  |  |  | RBS-230 |  |
| - Information for each multiplexing option |  |  |  |  | 2 RBMuxOptions |  |  | RBS-231 |  |
| - RLC logical channel mapping indicator |  |  |  |  | Not Present |  |  | RBS-232 |  |
| - Number of uplink RLC logical channels |  |  |  |  | 1 |  |  | RBS-233 |  |
| - Uplink transport channel type |  |  |  |  | DCH |  |  | RBS-234 |  |
| - UL Transport channel identity |  |  |  |  | 1 |  |  | RBS-235 |  |
| - Logical channel identity |  |  |  |  | Not Present |  |  | RBS-236 |  |
| - CHOICE RLC size list |  |  |  |  | Configured |  |  | RBS-237 |  |
| - MAC logical channel priority |  |  |  |  | 8 |  |  | RBS-238 |  |
| - Downlink RLC logical channel info |  |  |  |  |  |  |  | RBS-239 |  |
| - Number of downlink RLC logical channels |  |  |  |  | 1 |  |  | RBS-240 |  |
| - Downlink transport channel type |  |  |  |  | DCH |  |  | RBS-241 |  |
| - DL DCH Transport channel identity |  |  |  |  | 6 |  |  | RBS-242 |  |
| - DL DSCH Transport channel identity |  |  |  |  | Not Present |  |  | RBS-243 |  |
| - Logical channel identity |  |  |  |  | Not Present |  |  | RBS-244 |  |
| - RLC logical channel mapping indicator |  |  |  |  | Not Present |  |  | RBS-245 |  |
| - Number of uplink RLC logical channels |  |  |  |  | 1 |  |  | RBS-246 |  |
| - Uplink transport channel type |  |  |  |  | RACH |  |  | RBS-247 |  |
| - UL Transport channel identity |  |  |  |  | Not Present |  |  | RBS-248 |  |
| - Logical channel identity |  |  |  |  | 7 |  |  | RBS-249 |  |
| - CHOICE RLC size list |  |  |  |  | Explicit list |  |  | RBS-250 |  |
| - RLC size index |  |  |  |  | Reference to clause 6 Parameter Set |  |  | RBS-251 |  |
| - MAC logical channel priority |  |  |  |  | 8 |  |  | RBS-252 |  |
| - Downlink RLC logical channel info |  |  |  |  |  |  |  | RBS-253 |  |
| - Number of downlink RLC logical channels |  |  |  |  | 1 |  |  | RBS-254 |  |
| - Downlink transport channel type |  |  |  |  | FACH |  |  | RBS-255 |  |
| - DL DCH Transport channel identity |  |  |  |  | Not Present |  |  | RBS-256 |  |
| - DL DSCH Transport channel identity |  |  |  |  | Not Present |  |  | RBS-257 |  |
| - Logical channel identity |  |  |  |  | 7 |  |  | RBS-258 |  |
| - RAB information for setup |  |  | A9 |  |  | Rel-5 |  | RBS-259 |  |
| - RAB info |  |  |  |  | (high-speed AM DTCH for PS domain) |  |  | RBS-260 |  |
| - RAB identity |  |  |  |  | 0000 0101B The first/ leftmost bit of the bit string contains the most significant bit of the RAB identity. |  |  | RBS-261 |  |
| - CN domain identity |  |  |  |  | PS domain |  |  | RBS-262 |  |
| - NAS Synchronization Indicator |  |  |  |  | Not Present |  |  | RBS-263 |  |
| - Re-establishment timer |  |  |  |  | useT315 |  |  | RBS-264 |  |
| - RB information to setup |  |  |  |  |  |  |  | RBS-265 |  |
| - RB identity |  |  |  |  | 25 |  |  | RBS-266 |  |
| - PDCP info |  |  |  |  |  |  |  | RBS-267 |  |
| - Support for lossless SRNS relocation |  |  |  |  | FALSE |  |  | RBS-268 |  |
| - Max PDCP SN window size |  |  |  |  | Not present |  |  | RBS-269 |  |
| - PDCP PDU header |  |  |  |  | Absent |  |  | RBS-270 |  |
| - Header compression information |  |  |  |  | Not present |  |  | RBS-271 |  |
| - CHOICE RLC info type |  |  |  |  | RLC info |  |  | RBS-272 |  |
| - CHOICE Uplink RLC mode |  |  |  |  | AM RLC |  |  | RBS-273 |  |
| - Transmission RLC discard |  |  |  |  |  |  |  | RBS-274 |  |
| - CHOICE SDU discard mode |  |  |  |  | No Discard |  |  | RBS-275 |  |
| - MAX_DAT |  |  |  |  | 15 |  |  | RBS-276 |  |
| - Transmission window size |  |  |  |  | 128 |  |  | RBS-277 |  |
| - Timer_RST |  |  |  |  | 500 |  |  | RBS-278 |  |
| - Max_RST |  |  |  |  | 4 |  |  | RBS-279 |  |
| - Polling info |  |  |  |  |  |  |  | RBS-280 |  |
| - Timer_poll_prohibit |  |  |  |  | 100 |  |  | RBS-281 |  |
| - Timer_poll |  |  |  |  | 100 |  |  | RBS-282 |  |
| - Poll_PDU |  |  |  |  | Not Present |  |  | RBS-283 |  |
| - Poll_SDU |  |  |  |  | 1 |  |  | RBS-284 |  |
| - Last transmission PDU poll |  |  |  |  | TRUE |  |  | RBS-285 |  |
| - Last retransmission PDU poll |  |  |  |  | TRUE |  |  | RBS-286 |  |
| - Poll_Windows |  |  |  |  | 99 |  |  | RBS-287 |  |
| - Timer_poll_periodic |  |  |  |  | Not Present |  |  | RBS-288 |  |
| - CHOICE Downlink RLC mode |  |  |  |  | AM RLC |  |  | RBS-289 |  |
| - CHOICE Downlink RLC PDU Size |  |  |  |  | Reference to clause 6 Parameter Set |  |  | RBS-290 |  |
| - In-sequence delivery |  |  |  |  | TRUE |  |  | RBS-291 |  |
| - Receiving window size |  |  |  |  | 768 |  |  | RBS-292 |  |
| - Downlink RLC status info |  |  |  |  |  |  |  | RBS-293 |  |
| - Timer_status_prohibit |  |  |  |  | 100 |  |  | RBS-294 |  |
| - Timer_EPC |  |  |  |  | Not Present |  |  | RBS-295 |  |
| - Missing PDU indicator |  |  |  |  | TRUE |  |  | RBS-296 |  |
| - Timer_STATUS_periodic |  |  |  |  | Not Present |  |  | RBS-297 |  |
| - One sided RLC re-establishment |  |  |  |  | FALSE |  |  | RBS-298 |  |
| - RB mapping info |  |  |  |  |  |  |  | RBS-299 |  |
| - Information for each multiplexing option |  |  |  |  | 3 RBMuxOptions |  |  | RBS-300 |  |
| - RLC logical channel mapping indicator |  |  |  |  | Not Present |  |  | RBS-301 |  |
| - Number of uplink RLC logical channels |  |  |  |  | 1 |  |  | RBS-302 |  |
| - Uplink transport channel type |  |  |  |  | DCH |  |  | RBS-303 |  |
| - UL Transport channel identity |  |  |  |  | 1 |  |  | RBS-304 |  |
| - Logical channel identity |  |  |  |  | Not Present |  |  | RBS-305 |  |
| - CHOICE RLC size list |  |  |  |  | Configured |  |  | RBS-306 |  |
| - MAC logical channel priority |  |  |  |  | 8 |  |  | RBS-307 |  |
| - Downlink RLC logical channel info |  |  |  |  |  |  |  | RBS-308 |  |
| - Number of downlink RLC logical channels |  |  |  |  | 1 |  |  | RBS-309 |  |
| - Downlink transport channel type |  |  |  |  | DCH |  |  | RBS-310 |  |
| - DL DCH Transport channel identity |  |  |  |  | 6 |  |  | RBS-311 |  |
| - DL DSCH Transport channel identity |  |  |  |  | Not Present |  |  | RBS-312 |  |
| - DL HS-DSCH MAC-d flow identity |  |  |  |  | Not Present |  |  | RBS-313 |  |
| - Logical channel identity |  |  |  |  | Not Present |  |  | RBS-314 |  |
| - RLC logical channel mapping indicator |  |  |  |  | Not Present |  |  | RBS-315 |  |
| - Number of uplink RLC logical channels |  |  |  |  | 1 |  |  | RBS-316 |  |
| - Uplink transport channel type |  |  |  |  | DCH |  |  | RBS-317 |  |
| - UL Transport channel identity |  |  |  |  | 1 |  |  | RBS-318 |  |
| - Logical channel identity |  |  |  |  | Not Present |  |  | RBS-319 |  |
| - CHOICE RLC size list |  |  |  |  | Configured |  |  | RBS-320 |  |
| - MAC logical channel priority |  |  |  |  | 8 |  |  | RBS-321 |  |
| - Downlink RLC logical channel info |  |  |  |  |  |  |  | RBS-322 |  |
| - Number of downlink RLC logical channels |  |  |  |  | 1 |  |  | RBS-323 |  |
| - Downlink transport channel type |  |  |  |  | HS-DSCH |  |  | RBS-324 |  |
| - DL DCH Transport channel identity |  |  |  |  | Not Present |  |  | RBS-325 |  |
| - DL DSCH Transport channel identity |  |  |  |  | Not Present |  |  | RBS-326 |  |
| - DL HS-DSCH MAC-d flow identity |  |  |  |  | 0 |  |  | RBS-327 |  |
| - Logical channel identity |  |  |  |  | Not Present |  |  | RBS-328 |  |
| - RLC logical channel mapping indicator |  |  |  |  | Not Present |  |  | RBS-329 |  |
| - Number of uplink RLC logical channels |  |  |  |  | 1 |  |  | RBS-330 |  |
| - Uplink transport channel type |  |  |  |  | RACH |  |  | RBS-331 |  |
| - UL Transport channel identity |  |  |  |  | Not Present |  |  | RBS-332 |  |
| - Logical channel identity |  |  |  |  | 7 |  |  | RBS-333 |  |
| - CHOICE RLC size list |  |  |  |  | Explicit list |  |  | RBS-334 |  |
| - RLC size index |  |  |  |  | Reference to clause 6 Parameter Set |  |  | RBS-335 |  |
| - MAC logical channel priority |  |  |  |  | 8 |  |  | RBS-336 |  |
| - Downlink RLC logical channel info |  |  |  |  |  |  |  | RBS-337 |  |
| - Number of downlink RLC logical channels |  |  |  |  | 1 |  |  | RBS-338 |  |
| - Downlink transport channel type |  |  |  |  | FACH |  |  | RBS-339 |  |
| - DL DCH Transport channel identity |  |  |  |  | Not Present |  |  | RBS-340 |  |
| - DL DSCH Transport channel identity |  |  |  |  | Not Present |  |  | RBS-341 |  |
| - Logical channel identity |  |  |  |  | 7 |  |  | RBS-342 |  |
| - RAB information for setup |  |  | A10 |  |  | Rel-5 |  | RBS-343 |  |
| - RAB info |  |  |  |  | (high-speed AM DTCH for PS domain) |  |  | RBS-344 |  |
| - RAB identity |  |  |  |  | 0000 0101B The first/ leftmost bit of the bit string contains the most significant bit of the RAB identity. |  |  | RBS-345 |  |
| - CN domain identity |  |  |  |  | PS domain |  |  | RBS-346 |  |
| - NAS Synchronization Indicator |  |  |  |  | Not Present |  |  | RBS-347 |  |
| - Re-establishment timer |  |  |  |  | useT315 |  |  | RBS-348 |  |
| - RB information to setup |  |  |  |  |  |  |  | RBS-349 |  |
| - RB identity |  |  |  |  | 25 |  |  | RBS-350 |  |
| - PDCP info |  |  |  |  |  |  |  | RBS-351 |  |
| - Support for lossless SRNS relocation |  |  |  |  | FALSE |  |  | RBS-352 |  |
| - Max PDCP SN window size |  |  |  |  | Not present |  |  | RBS-353 |  |
| - PDCP PDU header |  |  |  |  | Absent |  |  | RBS-354 |  |
| - Header compression information |  |  |  |  | Not present |  |  | RBS-355 |  |
| - CHOICE RLC info type |  |  |  |  | RLC info |  |  | RBS-356 |  |
| - CHOICE Uplink RLC mode |  |  |  |  | AM RLC |  |  | RBS-357 |  |
| - Transmission RLC discard |  |  |  |  |  |  |  | RBS-358 |  |
| - CHOICE SDU discard mode |  |  |  |  | No Discard |  |  | RBS-359 |  |
| - MAX_DAT |  |  |  |  | 15 |  |  | RBS-360 |  |
| - Transmission window size |  |  |  |  | 128 |  |  | RBS-361 |  |
| - Timer_RST |  |  |  |  | 500 |  |  | RBS-362 |  |
| - Max_RST |  |  |  |  | 4 |  |  | RBS-363 |  |
| - Polling info |  |  |  |  |  |  |  | RBS-364 |  |
| - Timer_poll_prohibit |  |  |  |  | 100 |  |  | RBS-365 |  |
| - Timer_poll |  |  |  |  | 100 |  |  | RBS-366 |  |
| - Poll_PDU |  |  |  |  | Not Present |  |  | RBS-367 |  |
| - Poll_SDU |  |  |  |  | 1 |  |  | RBS-368 |  |
| - Last transmission PDU poll |  |  |  |  | TRUE |  |  | RBS-369 |  |
| - Last retransmission PDU poll |  |  |  |  | TRUE |  |  | RBS-370 |  |
| - Poll_Windows |  |  |  |  | 99 |  |  | RBS-371 |  |
| - Timer_poll_periodic |  |  |  |  | Not Present |  |  | RBS-372 |  |
| - CHOICE Downlink RLC mode |  |  |  |  | AM RLC |  |  | RBS-373 |  |
| - CHOICE Downlink RLC PDU Size |  |  |  |  | Reference to clause 6 Parameter Set |  |  | RBS-374 |  |
| - In-sequence delivery |  |  |  |  | TRUE |  |  | RBS-375 |  |
| - Receiving window size |  |  |  |  | 768 |  |  | RBS-376 |  |
| - Downlink RLC status info |  |  |  |  |  |  |  | RBS-377 |  |
| - Timer_status_prohibit |  |  |  |  | 100 |  |  | RBS-378 |  |
| - Timer_EPC |  |  |  |  | Not Present |  |  | RBS-379 |  |
| - Missing PDU indicator |  |  |  |  | TRUE |  |  | RBS-380 |  |
| - Timer_STATUS_periodic |  |  |  |  | Not Present |  |  | RBS-381 |  |
| - One sided RLC re-establishment |  |  |  |  | FALSE |  |  | RBS-382 |  |
| - RB mapping info |  |  |  |  |  |  |  | RBS-383 |  |
| - Information for each multiplexing option |  |  |  |  | 1 RBMuxOption |  |  | RBS-384 |  |
| - RLC logical channel mapping indicator |  |  |  |  | Not present |  |  | RBS-385 |  |
| - Number of uplink RLC logical channels |  |  |  |  | 1 |  |  | RBS-386 |  |
| - Uplink transport channel type |  |  |  |  | DCH |  |  | RBS-387 |  |
| - UL Transport channel identity |  |  |  |  | 1 |  |  | RBS-388 |  |
| - Logical channel identity |  |  |  |  | Not Present |  |  | RBS-389 |  |
| - CHOICE RLC size list |  |  |  |  | Configured |  |  | RBS-390 |  |
| - MAC logical channel priority |  |  |  |  | 8 |  |  | RBS-391 |  |
| - Downlink RLC logical channel info |  |  |  |  |  |  |  | RBS-392 |  |
| - Number of downlink RLC logical channels |  |  |  |  | 1 |  |  | RBS-393 |  |
| - Downlink transport channel type |  |  |  |  | HS-DSCH |  |  | RBS-394 |  |
| - DL DCH Transport channel identity |  |  |  |  | Not present |  |  | RBS-395 |  |
| - DL DSCH Transport channel identity |  |  |  |  | Not present |  |  | RBS-396 |  |
| - DL HS-DSCH MAC-d flow identity |  |  |  |  | 0 |  |  | RBS-397 |  |
| - Logical channel identity |  |  |  |  | Not Present |  |  | RBS-398 |  |
| - RAB information for setup |  |  | A11 |  |  |  |  | RBS-399 |  |
| - RAB info |  |  |  |  | (AM DTCH for PS domain) |  |  | RBS-400 |  |
| - RAB identity |  |  |  |  | 0000 0101B The first/ leftmost bit of the bit string contains the most significant bit of the RAB identity. |  |  | RBS-401 |  |
| - CN domain identity |  |  |  |  | PS domain |  |  | RBS-402 |  |
| - NAS Synchronization Indicator |  |  |  |  | Not Present |  |  | RBS-403 |  |
| - Re-establishment timer |  |  |  |  | useT315 |  |  | RBS-404 |  |
| - RB information to setup |  |  |  |  |  |  |  | RBS-405 |  |
| - RB identity |  |  |  |  | 20 |  |  | RBS-406 |  |
| - PDCP info |  |  |  |  |  |  |  | RBS-407 |  |
| - Support for lossless SRNS relocation |  |  |  |  | FALSE |  |  | RBS-408 |  |
| - Max PDCP SN window size |  |  |  |  | Not present |  |  | RBS-409 |  |
| - PDCP PDU header |  |  |  |  | Absent |  |  | RBS-410 |  |
| - Header compression information |  |  |  |  | Not present |  |  | RBS-411 |  |
| - CHOICE RLC info type |  |  |  |  | RLC info |  |  | RBS-412 |  |
| - CHOICE Uplink RLC mode |  |  |  |  | AM RLC |  |  | RBS-413 |  |
| - Transmission RLC discard |  |  |  |  |  |  |  | RBS-414 |  |
| - CHOICE SDU discard mode |  |  |  |  | No Discard |  |  | RBS-415 |  |
| - MAX_DAT |  |  |  |  | 15 |  |  | RBS-416 |  |
| - Transmission window size |  |  |  |  | 128 |  |  | RBS-417 |  |
| - Timer_RST |  |  |  |  | 500 |  |  | RBS-418 |  |
| - Max_RST |  |  |  |  | 4 |  |  | RBS-419 |  |
| - Polling info |  |  |  |  |  |  |  | RBS-420 |  |
| - Timer_poll_prohibit |  |  |  |  | 200 |  |  | RBS-421 |  |
| - Timer_poll |  |  |  |  | 200 |  |  | RBS-422 |  |
| - Poll_PDU |  |  |  |  | Not Present |  |  | RBS-423 |  |
| - Poll_SDU |  |  |  |  | 1 |  |  | RBS-424 |  |
| - Last transmission PDU poll |  |  |  |  | TRUE |  |  | RBS-425 |  |
| - Last retransmission PDU poll |  |  |  |  | TRUE |  |  | RBS-426 |  |
| - Poll_Windows |  |  |  |  | 99 |  |  | RBS-427 |  |
| - Timer_poll_periodic |  |  |  |  | Not Present |  |  | RBS-428 |  |
| - CHOICE Downlink RLC mode |  |  |  |  | AM RLC |  |  | RBS-429 |  |
| - CHOICE Downlink RLC PDU Size |  |  |  |  | Reference to clause 6 Parameter Set | Rel-5 |  | RBS-430 |  |
| - In-sequence delivery |  |  |  |  | TRUE |  |  | RBS-431 |  |
| - Receiving window size |  |  |  |  | 128 |  |  | RBS-432 |  |
| - Downlink RLC status info |  |  |  |  |  |  |  | RBS-433 |  |
| - Timer_status_prohibit |  |  |  |  | 200 |  |  | RBS-434 |  |
| - Timer_EPC |  |  |  |  | Not Present |  |  | RBS-435 |  |
| - Missing PDU indicator |  |  |  |  | TRUE |  |  | RBS-436 |  |
| - Timer_STATUS_periodic |  |  |  |  | Not Present |  |  | RBS-437 |  |
| - RB mapping info |  |  |  |  |  |  |  | RBS-438 |  |
| - Information for each multiplexing option |  |  |  |  | 2 RBMuxOptions |  |  | RBS-439 |  |
| - RLC logical channel mapping indicator |  |  |  |  | Not Present |  |  | RBS-440 |  |
| - Number of uplink RLC logical channels |  |  |  |  | 1 |  |  | RBS-441 |  |
| - Uplink transport channel type |  |  |  |  | DCH |  |  | RBS-442 |  |
| - UL Transport channel identity |  |  |  |  | 4 |  |  | RBS-443 |  |
| - Logical channel identity |  |  |  |  | Not Present |  |  | RBS-444 |  |
| - CHOICE RLC size list |  |  |  |  | Configured |  |  | RBS-445 |  |
| - MAC logical channel priority |  |  |  |  | 8 |  |  | RBS-446 |  |
| - Downlink RLC logical channel info |  |  |  |  |  |  |  | RBS-447 |  |
| - Number of downlink RLC logical channels |  |  |  |  | 1 |  |  | RBS-448 |  |
| - Downlink transport channel type |  |  |  |  | DCH |  |  | RBS-449 |  |
| - DL DCH Transport channel identity |  |  |  |  | 9 |  |  | RBS-450 |  |
| - DL DSCH Transport channel identity |  |  |  |  | Not Present |  |  | RBS-451 |  |
| - Logical channel identity |  |  |  |  | Not Present |  |  | RBS-452 |  |
| - RLC logical channel mapping indicator |  |  |  |  | Not Present |  |  | RBS-453 |  |
| - Number of uplink RLC logical channels |  |  |  |  | 1 |  |  | RBS-454 |  |
| - Uplink transport channel type |  |  |  |  | RACH |  |  | RBS-455 |  |
| - UL Transport channel identity |  |  |  |  | Not Present |  |  | RBS-456 |  |
| - Logical channel identity |  |  |  |  | 7 |  |  | RBS-457 |  |
| - CHOICE RLC size list |  |  |  |  | Explicit list |  |  | RBS-458 |  |
| - RLC size index |  |  |  |  | Reference to clause 6 Parameter Set |  |  | RBS-459 |  |
| - MAC logical channel priority |  |  |  |  | 8 |  |  | RBS-460 |  |
| - Downlink RLC logical channel info |  |  |  |  |  |  |  | RBS-461 |  |
| - Number of downlink RLC logical channels |  |  |  |  | 1 |  |  | RBS-462 |  |
| - Downlink transport channel type |  |  |  |  | FACH |  |  | RBS-463 |  |
| - DL DCH Transport channel identity |  |  |  |  | Not Present |  |  | RBS-464 |  |
| - DL DSCH Transport channel identity |  |  |  |  | Not Present |  |  | RBS-465 |  |
| - Logical channel identity |  |  |  |  | 7 |  |  | RBS-466 |  |
| - RAB information for setup |  |  | A12 |  |  | Rel-6 |  | RBS-467 |  |
|  |  |  | A19 |  |  | Rel-7 |  | RBS-468 |  |
| - RAB info |  |  |  |  | (high-speed AM DTCH for PS domain) |  |  | RBS-469 |  |
| - RAB identity |  |  |  |  | 0000 0101B The first/ leftmost bit of the bit string contains the most significant bit of the RAB identity. |  |  | RBS-470 |  |
| - CN domain identity |  |  |  |  | PS domain |  |  | RBS-471 |  |
| - NAS Synchronization Indicator |  |  |  |  | Not Present |  |  | RBS-472 |  |
| - Re-establishment timer |  |  |  |  | useT315 |  |  | RBS-473 |  |
| - RB information to setup |  |  |  |  |  |  |  | RBS-474 |  |
| - RB identity |  |  |  |  | 25 |  |  | RBS-475 |  |
| - PDCP info |  |  |  |  |  |  |  | RBS-476 |  |
| - Support for lossless SRNS relocation |  |  |  |  | FALSE |  |  | RBS-477 |  |
| - Max PDCP SN window size |  |  |  |  | Not present |  |  | RBS-478 |  |
| - PDCP PDU header |  |  |  |  | Absent |  |  | RBS-479 |  |
| - Header compression information |  |  |  |  | Not present |  |  | RBS-480 |  |
| - CHOICE RLC info type |  |  |  |  | RLC info |  |  | RBS-481 |  |
| - CHOICE Uplink RLC mode |  |  |  |  | AM RLC |  |  | RBS-482 |  |
| - Transmission RLC discard |  |  |  |  |  |  |  | RBS-483 |  |
| - CHOICE SDU discard mode |  |  |  |  | No Discard |  |  | RBS-484 |  |
| - MAX_DAT |  |  |  |  | 15 |  |  | RBS-485 |  |
| - Transmission window size |  |  |  |  | 256 |  |  | RBS-486 |  |
| - Timer_RST |  |  |  |  | 500 |  |  | RBS-487 |  |
| - Max_RST |  |  |  |  | 4 |  |  | RBS-488 |  |
| - Polling info |  |  |  |  |  |  |  | RBS-489 |  |
| - Timer_poll_prohibit |  |  |  |  | 100 |  |  | RBS-490 |  |
| - Timer_poll |  |  |  |  | 100 |  |  | RBS-491 |  |
| - Poll_PDU |  |  |  |  | Not Present |  |  | RBS-492 |  |
| - Poll_SDU |  |  |  |  | 1 |  |  | RBS-493 |  |
| - Last transmission PDU poll |  |  |  |  | TRUE |  |  | RBS-494 |  |
| - Last retransmission PDU poll |  |  |  |  | TRUE |  |  | RBS-495 |  |
| - Poll_Windows |  |  |  |  | 99 |  |  | RBS-496 |  |
| - Timer_poll_periodic |  |  |  |  | Not Present |  |  | RBS-497 |  |
| - CHOICE Downlink RLC mode |  |  |  |  | AM RLC |  |  | RBS-498 |  |
| - CHOICE Downlink RLC PDU Size |  |  |  |  | Reference to clause 6 Parameter Set |  |  | RBS-499 |  |
| - In-sequence delivery |  |  |  |  | TRUE |  |  | RBS-500 |  |
| - Receiving window size |  |  |  |  | 768 |  |  | RBS-501 |  |
| - Downlink RLC status info |  |  |  |  |  |  |  | RBS-502 |  |
| - Timer_status_prohibit |  |  |  |  | 100 |  |  | RBS-503 |  |
| - Timer_EPC |  |  |  |  | Not Present |  |  | RBS-504 |  |
| - Missing PDU indicator |  |  |  |  | TRUE |  |  | RBS-505 |  |
| - Timer_STATUS_periodic |  |  |  |  | Not Present |  |  | RBS-506 |  |
| - One sided RLC re-establishment |  |  |  |  | FALSE |  |  | RBS-507 |  |
| - RB mapping info |  |  |  |  |  |  |  | RBS-508 |  |
| - Information for each multiplexing option |  |  |  |  | 3 RBMuxOptions |  |  | RBS-509 |  |
| - RLC logical channel mapping indicator |  |  |  |  | Not Present |  |  | RBS-510 |  |
| - Number of uplink RLC logical channels |  |  |  |  | 1 |  |  | RBS-511 |  |
| - Uplink transport channel type |  |  |  |  | DCH |  |  | RBS-512 |  |
| - UL Transport channel identity |  |  |  |  | 1 |  |  | RBS-513 |  |
| - Logical channel identity |  |  |  |  | Not Present |  |  | RBS-514 |  |
| - CHOICE RLC size list |  |  |  |  | Configured |  |  | RBS-515 |  |
| - MAC logical channel priority |  |  |  |  | 8 |  |  | RBS-516 |  |
| - Downlink RLC logical channel info |  |  |  |  |  |  |  | RBS-517 |  |
| - Number of downlink RLC logical channels |  |  |  |  | 1 |  |  | RBS-518 |  |
| - Downlink transport channel type |  |  |  |  | DCH |  |  | RBS-519 |  |
| - DL DCH Transport channel identity |  |  |  |  | 6 |  |  | RBS-520 |  |
| - DL DSCH Transport channel identity |  |  |  |  | Not Present |  |  | RBS-521 |  |
| - DL HS-DSCH MAC-d flow identity |  |  |  |  | Not Present |  |  | RBS-522 |  |
| - Logical channel identity |  |  |  |  | Not Present |  |  | RBS-523 |  |
| - RLC logical channel mapping indicator |  |  |  |  | Not Present |  |  | RBS-524 |  |
| - Number of uplink RLC logical channels |  |  |  |  | 1 |  |  | RBS-525 |  |
| - Uplink transport channel type |  |  |  |  | E-DCH |  |  | RBS-526 |  |
| - Logical channel identity |  |  |  |  | 7 |  |  | RBS-527 |  |
| - E-DCH MAC-d flow identity |  |  |  |  | 2 |  |  | RBS-528 |  |
| - CHOICE RLC PDU size |  |  | MAC-I-FIXED |  | Fixed size | Rel-8 |  | RBS-529 |  |
| - DDI |  |  |  |  | 5 |  |  | RBS-530 |  |
| - RLC PDU size list |  |  |  |  | 1 RLC PDU size |  |  | RBS-531 |  |
| - RLC PDU size |  |  |  |  | 336 bits |  |  | RBS-532 |  |
| - CHOICE RLC PDU size |  |  | MAC-I-FLEX |  | Flexible size | Rel-8 |  | RBS-533 |  |
| - Length indicator size |  |  |  |  | 15 bit |  |  | RBS-534 |  |
| - Minimum UL RLC PDU size |  |  |  |  | See clause 6.10 |  |  | RBS-535 |  |
| - Largest UL RLC PDU size |  |  |  |  | See clause 6.10 |  |  | RBS-536 |  |
| - Include in scheduling info |  |  |  |  | TRUE |  |  | RBS-537 |  |
| - MAC logical channel priority |  |  |  |  | 8 |  |  | RBS-538 |  |
| - Downlink RLC logical channel info |  |  |  |  |  |  |  | RBS-539 |  |
| - Number of downlink RLC logical channels |  |  |  |  | 1 |  |  | RBS-540 |  |
| - Downlink transport channel type |  |  |  |  | HS-DSCH |  |  | RBS-541 |  |
| - DL DCH Transport channel identity |  |  |  |  | Not Present |  |  | RBS-542 |  |
| - DL DSCH Transport channel identity |  |  |  |  | Not Present |  |  | RBS-543 |  |
| - DL HS-DSCH MAC-d flow identity |  |  |  |  | 0 |  |  | RBS-544 |  |
| - Logical channel identity |  |  |  |  | Not Present |  |  | RBS-545 |  |
| - RLC logical channel mapping indicator |  |  |  |  | Not Present |  |  | RBS-546 |  |
| - Number of uplink RLC logical channels |  |  |  |  | 1 |  |  | RBS-547 |  |
| - Uplink transport channel type |  |  |  |  | RACH |  |  | RBS-548 |  |
| - UL Transport channel identity |  |  |  |  | Not Present |  |  | RBS-549 |  |
| - Logical channel identity |  |  |  |  | 7 |  |  | RBS-550 |  |
| - CHOICE RLC size list |  |  |  |  | Explicit list |  |  | RBS-551 |  |
| - RLC size index |  |  |  |  | Reference to clause 6 Parameter Set |  |  | RBS-552 |  |
| - MAC logical channel priority |  |  |  |  | 8 |  |  | RBS-553 |  |
| - Downlink RLC logical channel info |  |  |  |  |  |  |  | RBS-554 |  |
| - Number of downlink RLC logical channels |  |  |  |  | 1 |  |  | RBS-555 |  |
| - Downlink transport channel type |  |  |  |  | FACH |  |  | RBS-556 |  |
| - DL DCH Transport channel identity |  |  |  |  | Not Present |  |  | RBS-557 |  |
| - DL DSCH Transport channel identity |  |  |  |  | Not Present |  |  | RBS-558 |  |
| - RAB information for setup |  |  | A13, A14, A15, A16 |  |  | Rel-6 |  | RBS-559 |  |
|  |  |  | , A19a |  |  | Rel-7 |  | RBS-560 |  |
| - RAB info |  |  |  |  | (high-speed AM DTCH for PS domain) |  |  | RBS-561 |  |
| - RAB identity |  |  |  |  | 0000 0101B The first/ leftmost bit of the bit string contains the most significant bit of the RAB identity. |  |  | RBS-562 |  |
| - CN domain identity |  |  |  |  | PS domain |  |  | RBS-563 |  |
| - NAS Synchronization Indicator |  |  |  |  | Not Present |  |  | RBS-564 |  |
| - Re-establishment timer |  |  |  |  | useT315 |  |  | RBS-565 |  |
| - RB information to setup |  |  |  |  |  |  |  | RBS-566 |  |
| - RB identity |  |  |  |  | 25 |  |  | RBS-567 |  |
| - PDCP info |  |  |  |  |  |  |  | RBS-568 |  |
| - Support for lossless SRNS relocation |  |  |  |  | FALSE |  |  | RBS-569 |  |
| - Max PDCP SN window size |  |  |  |  | Not present |  |  | RBS-570 |  |
| - PDCP PDU header |  |  |  |  | Absent |  |  | RBS-571 |  |
| - Header compression information |  |  |  |  | Not present |  |  | RBS-572 |  |
| - CHOICE RLC info type |  |  |  |  | RLC info |  |  | RBS-573 |  |
| - CHOICE Uplink RLC mode |  |  |  |  | AM RLC |  |  | RBS-574 |  |
| - Transmission RLC discard |  |  |  |  |  |  |  | RBS-575 |  |
| - CHOICE SDU discard mode |  |  |  |  | No Discard |  |  | RBS-576 |  |
| - MAX_DAT |  |  |  |  | 15 |  |  | RBS-577 |  |
| - Transmission window size |  |  |  |  | 256 |  |  | RBS-578 |  |
| - Timer_RST |  |  |  |  | 500 |  |  | RBS-579 |  |
| - Max_RST |  |  |  |  | 4 |  |  | RBS-580 |  |
| - Polling info |  |  |  |  |  |  |  | RBS-581 |  |
| - Timer_poll_prohibit |  |  |  |  | 100 |  |  | RBS-582 |  |
| - Timer_poll |  |  |  |  | 100 |  |  | RBS-583 |  |
| - Poll_PDU |  |  |  |  | Not Present |  |  | RBS-584 |  |
| - Poll_SDU |  |  |  |  | 1 |  |  | RBS-585 |  |
| - Last transmission PDU poll |  |  |  |  | TRUE |  |  | RBS-586 |  |
| - Last retransmission PDU poll |  |  |  |  | TRUE |  |  | RBS-587 |  |
| - Poll_Windows |  |  |  |  | 99 |  |  | RBS-588 |  |
| - Timer_poll_periodic |  |  |  |  | Not Present |  |  | RBS-589 |  |
| - CHOICE Downlink RLC mode |  |  |  |  | AM RLC |  |  | RBS-590 |  |
| - CHOICE Downlink RLC PDU Size |  |  |  |  | Reference to clause 6 Parameter Set |  |  | RBS-591 |  |
| - In-sequence delivery |  |  |  |  | TRUE |  |  | RBS-592 |  |
| - Receiving window size |  |  |  |  | 768 |  |  | RBS-593 |  |
| - Downlink RLC status info |  |  |  |  |  |  |  | RBS-594 |  |
| - Timer_status_prohibit |  |  |  |  | 100 |  |  | RBS-595 |  |
| - Timer_EPC |  |  |  |  | Not Present |  |  | RBS-596 |  |
| - Missing PDU indicator |  |  |  |  | TRUE |  |  | RBS-597 |  |
| - Timer_STATUS_periodic |  |  |  |  | Not Present |  |  | RBS-598 |  |
| - One sided RLC re-establishment |  |  |  |  | FALSE |  |  | RBS-599 |  |
| - RB mapping info |  |  |  |  |  |  |  | RBS-600 |  |
| - Information for each multiplexing option |  |  |  |  | 1 RBMuxOption |  |  | RBS-601 |  |
| - RLC logical channel mapping indicator |  |  |  |  | Not Present |  |  | RBS-602 |  |
| - Number of uplink RLC logical channels |  |  |  |  | 1 |  |  | RBS-603 |  |
| - Uplink transport channel type |  |  |  |  | E-DCH |  |  | RBS-604 |  |
| - Logical channel identity |  |  |  |  | 7 |  |  | RBS-605 |  |
| - E-DCH MAC-d flow identity |  |  |  |  | 2 |  |  | RBS-606 |  |
| - CHOICE RLC PDU size |  |  | MAC-I-FIXED |  | Fixed size | Rel-8 |  | RBS-607 |  |
| - DDI |  |  |  |  | 5 |  |  | RBS-608 |  |
| - RLC PDU size list |  |  |  |  | 1 RLC PDU size |  |  | RBS-609 |  |
| - RLC PDU size |  |  |  |  | 336 bits |  |  | RBS-610 |  |
| - CHOICE RLC PDU size |  |  | MAC-I-FLEX |  | Flexible size | Rel-8 |  | RBS-611 |  |
| - Length indicator size |  |  |  |  | - 15 bit |  |  | RBS-612 |  |
| - Minimum UL RLC PDU size |  |  |  |  | See clause 6.10 |  |  | RBS-613 |  |
| - Largest UL RLC PDU size |  |  |  |  | See clause 6.10 |  |  | RBS-614 |  |
| - Include in scheduling info |  |  |  |  | TRUE |  |  | RBS-615 |  |
| - MAC logical channel priority |  |  |  |  | 8 |  |  | RBS-616 |  |
| - Downlink RLC logical channel info |  |  |  |  |  |  |  | RBS-617 |  |
| - Number of downlink RLC logical channels |  |  |  |  | 1 |  |  | RBS-618 |  |
| - Downlink transport channel type |  |  |  |  | HS-DSCH |  |  | RBS-619 |  |
| - DL DCH Transport channel identity |  |  |  |  | Not present |  |  | RBS-620 |  |
| - DL DSCH Transport channel identity |  |  |  |  | Not present |  |  | RBS-621 |  |
| - DL HS-DSCH MAC-d flow identity |  |  |  |  | 0 |  |  | RBS-622 |  |
| - Logical channel identity |  |  |  |  | Not Present |  |  | RBS-623 |  |
| - RAB information for setup |  |  | A15 |  |  | Rel-6 |  | RBS-624 |  |
| - RAB info |  |  |  |  | (second high-speed AM DTCH for PS domain) |  |  | RBS-625 |  |
| - RAB identity |  |  |  |  | 0000 0110B The first/ leftmost bit of the bit string contains the most significant bit of the RAB identity. |  |  | RBS-626 |  |
| - CN domain identity |  |  |  |  | PS domain |  |  | RBS-627 |  |
| - NAS Synchronization Indicator |  |  |  |  | Not Present |  |  | RBS-628 |  |
| - Re-establishment timer |  |  |  |  | useT315 |  |  | RBS-629 |  |
| - RB information to setup |  |  |  |  |  |  |  | RBS-630 |  |
| - RB identity |  |  |  |  | 17 |  |  | RBS-631 |  |
| - PDCP info |  |  |  |  |  |  |  | RBS-632 |  |
| - Support for lossless SRNS relocation |  |  |  |  | FALSE |  |  | RBS-633 |  |
| - Max PDCP SN window size |  |  |  |  | Not present |  |  | RBS-634 |  |
| - PDCP PDU header |  |  |  |  | Absent |  |  | RBS-635 |  |
| - Header compression information |  |  |  |  | Not present |  |  | RBS-636 |  |
| - CHOICE RLC info type |  |  |  |  | RLC info |  |  | RBS-637 |  |
| - CHOICE Uplink RLC mode |  |  |  |  | AM RLC |  |  | RBS-638 |  |
| - Transmission RLC discard |  |  |  |  |  |  |  | RBS-639 |  |
| - CHOICE SDU discard mode |  |  |  |  | No Discard |  |  | RBS-640 |  |
| - MAX_DAT |  |  |  |  | 15 |  |  | RBS-641 |  |
| - Transmission window size |  |  |  |  | 256 |  |  | RBS-642 |  |
| - Timer_RST |  |  |  |  | 500 |  |  | RBS-643 |  |
| - Max_RST |  |  |  |  | 4 |  |  | RBS-644 |  |
| - Polling info |  |  |  |  |  |  |  | RBS-645 |  |
| - Timer_poll_prohibit |  |  |  |  | 100 |  |  | RBS-646 |  |
| - Timer_poll |  |  |  |  | 100 |  |  | RBS-647 |  |
| - Poll_PDU |  |  |  |  | Not Present |  |  | RBS-648 |  |
| - Poll_SDU |  |  |  |  | 1 |  |  | RBS-649 |  |
| - Last transmission PDU poll |  |  |  |  | TRUE |  |  | RBS-650 |  |
| - Last retransmission PDU poll |  |  |  |  | TRUE |  |  | RBS-651 |  |
| - Poll_Windows |  |  |  |  | 99 |  |  | RBS-652 |  |
| - Timer_poll_periodic |  |  |  |  | Not Present |  |  | RBS-653 |  |
| - CHOICE Downlink RLC mode |  |  |  |  | AM RLC |  |  | RBS-654 |  |
| - CHOICE Downlink RLC PDU Size |  |  |  |  | Reference to clause 6 Parameter Set |  |  | RBS-655 |  |
| - In-sequence delivery |  |  |  |  | TRUE |  |  | RBS-656 |  |
| - Receiving window size |  |  |  |  | 768 |  |  | RBS-657 |  |
| - Downlink RLC status info |  |  |  |  |  |  |  | RBS-658 |  |
| - Timer_status_prohibit |  |  |  |  | 100 |  |  | RBS-659 |  |
| - Timer_EPC |  |  |  |  | Not Present |  |  | RBS-660 |  |
| - Missing PDU indicator |  |  |  |  | TRUE |  |  | RBS-661 |  |
| - Timer_STATUS_periodic |  |  |  |  | Not Present |  |  | RBS-662 |  |
| - One sided RLC re-establishment |  |  |  |  | FALSE |  |  | RBS-663 |  |
| - RB mapping info |  |  |  |  |  |  |  | RBS-664 |  |
| - Information for each multiplexing option |  |  |  |  | 1 RBMuxOption |  |  | RBS-665 |  |
| - RLC logical channel mapping indicator |  |  |  |  | Not Present |  |  | RBS-666 |  |
| - Number of uplink RLC logical channels |  |  |  |  | 1 |  |  | RBS-667 |  |
| - Uplink transport channel type |  |  |  |  | E-DCH |  |  | RBS-668 |  |
| - Logical channel identity |  |  |  |  | 8 |  |  | RBS-669 |  |
| - E-DCH MAC-d flow identity |  |  |  |  | 3 |  |  | RBS-670 |  |
| - CHOICE RLC PDU size |  |  | MAC-I-FIXED |  | Fixed size | Rel-8 |  | RBS-671 |  |
| - DDI |  |  |  |  | 6 |  |  | RBS-672 |  |
| - RLC PDU size list |  |  |  |  | 1 RLC PDU size |  |  | RBS-673 |  |
| - RLC PDU size |  |  |  |  | 336 bits |  |  | RBS-674 |  |
| - CHOICE RLC PDU size |  |  | MAC-I-FLEX |  | Flexible size | Rel-8 |  | RBS-675 |  |
| - Length indicator size |  |  |  |  | - 15 bit |  |  | RBS-676 |  |
| - Minimum UL RLC PDU size |  |  |  |  | See clause 6.10 |  |  | RBS-677 |  |
| - Largest UL RLC PDU size |  |  |  |  | See clause 6.10 |  |  | RBS-678 |  |
| - Include in scheduling info |  |  |  |  | TRUE |  |  | RBS-679 |  |
| - MAC logical channel priority |  |  |  |  | 8 |  |  | RBS-680 |  |
| - Downlink RLC logical channel info |  |  |  |  |  |  |  | RBS-681 |  |
| - Number of downlink RLC logical channels |  |  |  |  | 1 |  |  | RBS-682 |  |
| - Downlink transport channel type |  |  |  |  | HS-DSCH |  |  | RBS-683 |  |
| - DL DCH Transport channel identity |  |  |  |  | Not present |  |  | RBS-684 |  |
| - DL DSCH Transport channel identity |  |  |  |  | Not present |  |  | RBS-685 |  |
| - DL HS-DSCH MAC-d flow identity |  |  |  |  | 2 |  |  | RBS-686 |  |
| - Logical channel identity |  |  |  |  | Not Present |  |  | RBS-687 |  |
| - RAB information for setup |  |  | A16 |  |  | Rel-6 |  | RBS-688 |  |
| - RAB info |  |  |  |  | (Conversational UM DTCH for PS domain) |  |  | RBS-689 |  |
| - RAB identity |  |  |  |  | 0000 0110B The first/ leftmost bit of the bit string contains the most significant bit of the RAB identity. |  |  | RBS-690 |  |
| - CN domain identity |  |  |  |  | PS domain |  |  | RBS-691 |  |
| - NAS Synchronization Indicator |  |  |  |  | Not Present |  |  | RBS-692 |  |
| - Re-establishment timer |  |  |  |  | useT314 |  |  | RBS-693 |  |
| - RB information to setup |  |  |  |  |  |  |  | RBS-694 |  |
| - RB identity |  |  |  |  | 27 |  |  | RBS-695 |  |
| - PDCP info |  |  |  |  |  |  |  | RBS-696 |  |
| - Support for lossless SRNS relocation |  |  |  |  | FALSE |  |  | RBS-697 |  |
| - Max PDCP SN window size |  |  |  |  | Not present |  |  | RBS-698 |  |
| - PDCP PDU header |  |  |  |  | Absent |  |  | RBS-699 |  |
| - Header compression information |  |  |  |  | Not present |  |  | RBS-700 |  |
| - CHOICE RLC info type |  |  |  |  | RLC info |  |  | RBS-701 |  |
| - CHOICE Uplink RLC mode |  |  |  |  | UM RLC |  |  | RBS-702 |  |
| - Transmission RLC discard |  |  |  |  | Not present |  |  | RBS-703 |  |
| - CHOICE Downlink RLC mode |  |  |  |  | UM RLC |  |  | RBS-704 |  |
| - DL UM RLC LI size |  |  |  |  | 7 |  |  | RBS-705 |  |
| - DL Reception Window Size |  |  |  |  | 32 |  |  | RBS-706 |  |
| - One sided RLC re-establishment |  |  |  |  | FALSE |  |  | RBS-707 |  |
| - Alternative E-bit interpretation |  |  |  |  | Not present |  |  | RBS-708 |  |
| - RB mapping info |  |  |  |  |  |  |  | RBS-709 |  |
| - Information for each multiplexing option |  |  |  |  | 1 RBMuxOption |  |  | RBS-710 |  |
| - RLC logical channel mapping indicator |  |  |  |  | Not Present |  |  | RBS-711 |  |
| - Number of uplink RLC logical channels |  |  |  |  | 1 |  |  | RBS-712 |  |
| - Uplink transport channel type |  |  |  |  | E-DCH |  |  | RBS-713 |  |
| - Logical channel identity |  |  |  |  | 9 |  |  | RBS-714 |  |
| - E-DCH MAC-d flow identity |  |  |  |  | 4 |  |  | RBS-715 |  |
| - CHOICE RLC PDU size |  |  | MAC-I-FIXED |  | Fixed size | Rel-8 |  | RBS-716 |  |
| - DDI |  |  |  |  | 7 |  |  | RBS-717 |  |
| - RLC PDU size list |  |  |  |  | 12 RLC PDU sizes |  |  | RBS-718 |  |
| - RLC PDU size |  |  |  |  | 96 bits |  |  | RBS-719 |  |
| - RLC PDU size |  |  |  |  | 112 bits |  |  | RBS-720 |  |
| - RLC PDU size |  |  |  |  | 144 bits |  |  | RBS-721 |  |
| - RLC PDU size |  |  |  |  | 160 bits |  |  | RBS-722 |  |
| - RLC PDU size |  |  |  |  | 176 bits |  |  | RBS-723 |  |
| - RLC PDU size |  |  |  |  | 192 bits |  |  | RBS-724 |  |
| - RLC PDU size |  |  |  |  | 208 bits |  |  | RBS-725 |  |
| - RLC PDU size |  |  |  |  | 224 bits |  |  | RBS-726 |  |
| - RLC PDU size |  |  |  |  | 288 bits |  |  | RBS-727 |  |
| - RLC PDU size |  |  |  |  | 296 bits |  |  | RBS-728 |  |
| - RLC PDU size |  |  |  |  | 312 bits |  |  | RBS-729 |  |
| - RLC PDU size |  |  |  |  | 336 bits |  |  | RBS-730 |  |
| - CHOICE RLC PDU size |  |  | MAC-I-FLEX |  | Flexible size | Rel-8 |  | RBS-731 |  |
| - Length indicator size |  |  |  |  | Not present |  |  | RBS-732 |  |
| - Minimum UL RLC PDU size |  |  |  |  | See clause 6.10 |  |  | RBS-733 |  |
| - Largest UL RLC PDU size |  |  |  |  | See clause 6.10 |  |  | RBS-734 |  |
| - Include in scheduling info |  |  |  |  | TRUE |  |  | RBS-735 |  |
| - MAC logical channel priority |  |  |  |  | 8 |  |  | RBS-736 |  |
| - Downlink RLC logical channel info |  |  |  |  |  |  |  | RBS-737 |  |
| - Number of downlink RLC logical channels |  |  |  |  | 1 |  |  | RBS-738 |  |
| - Downlink transport channel type |  |  |  |  | HS-DSCH |  |  | RBS-739 |  |
| - DL DCH Transport channel identity |  |  |  |  | Not present |  |  | RBS-740 |  |
| - DL DSCH Transport channel identity |  |  |  |  | Not present |  |  | RBS-741 |  |
| - DL HS-DSCH MAC-d flow identity |  |  |  |  | 3 |  |  | RBS-742 |  |
| - Logical channel identity |  |  |  |  | Not Present |  |  | RBS-743 |  |
| - RAB information for setup |  |  | A17, A17a |  |  | Rel-7 |  | RBS-744 |  |
|  |  |  | A25a, A28 |  |  | Rel-8 |  | RBS-745 |  |
| - RAB info |  |  |  |  | (high-speed AM DTCH for PS domain) |  |  | RBS-746 |  |
| - RAB identity |  |  |  |  | 0000 0101B The first/ leftmost bit of the bit string contains the most significant bit of the RAB identity. |  |  | RBS-747 |  |
| - CN domain identity |  |  |  |  | PS domain |  |  | RBS-748 |  |
| - NAS Synchronization Indicator |  |  |  |  | Not Present |  |  | RBS-749 |  |
| - Re-establishment timer |  |  |  |  | useT315 |  |  | RBS-750 |  |
| - RB information to setup |  |  |  |  |  |  |  | RBS-751 |  |
| - RB identity |  |  |  |  | 25 |  |  | RBS-752 |  |
| - PDCP info |  |  |  |  |  |  |  | RBS-753 |  |
| - Support for lossless SRNS relocation |  |  |  |  | FALSE |  |  | RBS-754 |  |
| - Max PDCP SN window size |  |  |  |  | Not present |  |  | RBS-755 |  |
| - PDCP PDU header |  |  |  |  | Absent |  |  | RBS-756 |  |
| - Header compression information |  |  |  |  | Not present |  |  | RBS-757 |  |
| - CHOICE RLC info type |  |  |  |  | RLC info |  |  | RBS-758 |  |
| - CHOICE Uplink RLC mode |  |  |  |  | AM RLC |  |  | RBS-759 |  |
| - Transmission RLC discard |  |  |  |  |  |  |  | RBS-760 |  |
| - CHOICE SDU discard mode |  |  |  |  | No Discard |  |  | RBS-761 |  |
| - MAX_DAT |  |  |  |  | 15 |  |  | RBS-762 |  |
| - Transmission window size |  |  |  |  | 128 |  |  | RBS-763 |  |
| - Timer_RST |  |  |  |  | 500 |  |  | RBS-764 |  |
| - Max_RST |  |  |  |  | 4 |  |  | RBS-765 |  |
| - Polling info |  |  |  |  |  |  |  | RBS-766 |  |
| - Timer_poll_prohibit |  |  |  |  | 100 |  |  | RBS-767 |  |
| - Timer_poll |  |  |  |  | 100 |  |  | RBS-768 |  |
| - Poll_PDU |  |  |  |  | Not Present |  |  | RBS-769 |  |
| - Poll_SDU |  |  |  |  | 1 |  |  | RBS-770 |  |
| - Last transmission PDU poll |  |  |  |  | TRUE |  |  | RBS-771 |  |
| - Last retransmission PDU poll |  |  |  |  | TRUE |  |  | RBS-772 |  |
| - Poll_Windows |  |  |  |  | 99 |  |  | RBS-773 |  |
| - Timer_poll_periodic |  |  |  |  | Not Present |  |  | RBS-774 |  |
| - CHOICE Downlink RLC mode |  |  |  |  | AM RLC |  |  | RBS-775 |  |
| - CHOICE Downlink RLC PDU Size |  |  |  |  | Reference to clause 6 Parameter Set |  |  | RBS-776 |  |
| - Length indicator size |  |  |  |  | This IE is present and set to “7” if Downlink RLC PDU Size is set to “Flexible” |  |  | RBS-777 |  |
| - In-sequence delivery |  |  |  |  | TRUE |  |  | RBS-778 |  |
| - Receiving window size |  |  |  |  | 768 |  |  | RBS-779 |  |
| - Downlink RLC status info |  |  |  |  |  |  |  | RBS-780 |  |
| - Timer_status_prohibit |  |  |  |  | 100 |  |  | RBS-781 |  |
| - Timer_EPC |  |  |  |  | Not Present |  |  | RBS-782 |  |
| - Missing PDU indicator |  |  |  |  | TRUE |  |  | RBS-783 |  |
| - Timer_STATUS_periodic |  |  |  |  | Not Present |  |  | RBS-784 |  |
| - One sided RLC re-establishment |  |  |  |  | FALSE |  |  | RBS-785 |  |
| - Alternative E-bit interpretation |  |  |  |  | Not present |  |  | RBS-786 |  |
| - Use special value of HE field |  |  |  |  | TRUE |  |  | RBS-787 |  |
| - RB mapping info |  |  |  |  |  |  |  | RBS-788 |  |
| - Information for each multiplexing option |  |  |  |  | 1 RBMuxOption |  |  | RBS-789 |  |
| - RLC logical channel mapping indicator |  |  |  |  | Not present |  |  | RBS-790 |  |
| - Number of uplink RLC logical channels |  |  |  |  | 1 |  |  | RBS-791 |  |
| - Uplink transport channel type |  |  |  |  | DCH |  |  | RBS-792 |  |
| - UL Transport channel identity |  |  |  |  | 1 |  |  | RBS-793 |  |
| - Logical channel identity |  |  |  |  | Not Present |  |  | RBS-794 |  |
| - CHOICE RLC size list |  |  |  |  | Configured |  |  | RBS-795 |  |
| - MAC logical channel priority |  |  |  |  | 8 |  |  | RBS-796 |  |
| - Downlink RLC logical channel info |  |  |  |  |  |  |  | RBS-797 |  |
| - Number of downlink RLC logical channels |  |  |  |  | 1 |  |  | RBS-798 |  |
| - Downlink transport channel type |  |  |  |  | HS-DSCH |  |  | RBS-799 |  |
| - DL DCH Transport channel identity |  |  |  |  | Not present |  |  | RBS-800 |  |
| - DL DSCH Transport channel identity |  |  |  |  | Not present |  |  | RBS-801 |  |
| - CHOICE DL MAC header type |  |  |  |  | MAC-ehs |  |  | RBS-802 |  |
| - DL HS-DSCH MAC-ehs Queue Id |  |  |  |  | 0 |  |  | RBS-803 |  |
| - Logical channel identity |  |  |  |  | 7 |  |  | RBS-804 |  |
| - RAB information for setup |  |  | A17b, A17c,A17d, A17e,A28a |  |  | Rel-7 |  | RBS-805 |  |
| - RAB info |  |  |  |  | (high-speed AM DTCH for PS domain) |  |  | RBS-806 |  |
| - RAB identity |  |  |  |  | 0000 0101B The first/ leftmost bit of the bit string contains the most significant bit of the RAB identity. |  |  | RBS-807 |  |
| - CN domain identity |  |  |  |  | PS domain |  |  | RBS-808 |  |
| - NAS Synchronization Indicator |  |  |  |  | Not Present |  |  | RBS-809 |  |
| - Re-establishment timer |  |  |  |  | useT315 |  |  | RBS-810 |  |
| - RB information to setup |  |  |  |  |  |  |  | RBS-811 |  |
| - RB identity |  |  |  |  | 25 |  |  | RBS-812 |  |
| - PDCP info |  |  |  |  |  |  |  | RBS-813 |  |
| - Support for lossless SRNS relocation |  |  |  |  | FALSE |  |  | RBS-814 |  |
| - Max PDCP SN window size |  |  |  |  | Not present |  |  | RBS-815 |  |
| - PDCP PDU header |  |  |  |  | Absent |  |  | RBS-816 |  |
| - Header compression information |  |  |  |  | Not present |  |  | RBS-817 |  |
| - CHOICE RLC info type |  |  |  |  | RLC info |  |  | RBS-818 |  |
| - CHOICE Uplink RLC mode |  |  |  |  | AM RLC |  |  | RBS-819 |  |
| - Transmission RLC discard |  |  |  |  |  |  |  | RBS-820 |  |
| - CHOICE SDU discard mode |  |  |  |  | No Discard |  |  | RBS-821 |  |
| - MAX_DAT |  |  |  |  | 15 |  |  | RBS-822 |  |
| - Transmission window size |  |  |  |  | 256 |  |  | RBS-823 |  |
| - Timer_RST |  |  |  |  | 500 |  |  | RBS-824 |  |
| - Max_RST |  |  |  |  | 4 |  |  | RBS-825 |  |
| - Polling info |  |  |  |  |  |  |  | RBS-826 |  |
| - Timer_poll_prohibit |  |  |  |  | 100 |  |  | RBS-827 |  |
| - Timer_poll |  |  |  |  | 100 |  |  | RBS-828 |  |
| - Poll_PDU |  |  |  |  | Not Present |  |  | RBS-829 |  |
| - Poll_SDU |  |  |  |  | 1 |  |  | RBS-830 |  |
| - Last transmission PDU poll |  |  |  |  | TRUE |  |  | RBS-831 |  |
| - Last retransmission PDU poll |  |  |  |  | TRUE |  |  | RBS-832 |  |
| - Poll_Windows |  |  |  |  | 99 |  |  | RBS-833 |  |
| - Timer_poll_periodic |  |  |  |  | Not Present |  |  | RBS-834 |  |
| - CHOICE Downlink RLC mode |  |  |  |  | AM RLC |  |  | RBS-835 |  |
| - CHOICE Downlink RLC PDU Size |  |  |  |  | Reference to clause 6 Parameter Set |  |  | RBS-836 |  |
| - Length indicator size |  |  |  |  | This IE is present and set to “7” if Downlink RLC PDU Size is set to “Flexible” |  |  | RBS-837 |  |
| - In-sequence delivery |  |  |  |  | TRUE |  |  | RBS-838 |  |
| - Receiving window size |  |  |  |  | 768 |  |  | RBS-839 |  |
| - Downlink RLC status info |  |  |  |  |  |  |  | RBS-840 |  |
| - Timer_status_prohibit |  |  |  |  | 100 |  |  | RBS-841 |  |
| - Timer_EPC |  |  |  |  | Not Present |  |  | RBS-842 |  |
| - Missing PDU indicator |  |  |  |  | TRUE |  |  | RBS-843 |  |
| - Timer_STATUS_periodic |  |  |  |  | Not Present |  |  | RBS-844 |  |
| - One sided RLC re-establishment |  |  |  |  | FALSE |  |  | RBS-845 |  |
| - Alternative E-bit interpretation |  |  |  |  | Not present |  |  | RBS-846 |  |
| - Use special value of HE field |  |  |  |  | TRUE |  |  | RBS-847 |  |
| - RB mapping info |  |  |  |  |  |  |  | RBS-848 |  |
| - Information for each multiplexing option |  |  |  |  | 1 RBMuxOption |  |  | RBS-849 |  |
| - RLC logical channel mapping indicator |  |  |  |  | Not Present |  |  | RBS-850 |  |
| - Number of uplink RLC logical channels |  |  |  |  | 1 |  |  | RBS-851 |  |
| - Uplink transport channel type |  |  |  |  | E-DCH |  |  | RBS-852 |  |
| - Logical channel identity |  |  |  |  | 7 |  |  | RBS-853 |  |
| - E-DCH MAC-d flow identity |  |  |  |  | 2 |  |  | RBS-854 |  |
| - CHOICE RLC PDU size |  |  | MAC-I-FIXED |  | Fixed size | Rel-8 |  | RBS-855 |  |
| - DDI |  |  |  |  | 5 |  |  | RBS-856 |  |
| - RLC PDU size list |  |  |  |  | 1 RLC PDU size |  |  | RBS-857 |  |
| - RLC PDU size |  |  |  |  | 336 bits |  |  | RBS-858 |  |
| - CHOICE RLC PDU size |  |  | MAC-I-FLEX |  | Flexible size | Rel-8 |  | RBS-859 |  |
| - Length indicator size |  |  |  |  | - 15 bit |  |  | RBS-860 |  |
| - Minimum UL RLC PDU size |  |  |  |  | See clause 6.10 |  |  | RBS-861 |  |
| - Largest UL RLC PDU size |  |  |  |  | See clause 6.10 |  |  | RBS-862 |  |
| - Include in scheduling info |  |  |  |  | TRUE |  |  | RBS-863 |  |
| - MAC logical channel priority |  |  |  |  | 8 |  |  | RBS-864 |  |
| - Downlink RLC logical channel info |  |  |  |  |  |  |  | RBS-865 |  |
| - Number of downlink RLC logical channels |  |  |  |  | 1 |  |  | RBS-866 |  |
| - Downlink transport channel type |  |  |  |  | HS-DSCH |  |  | RBS-867 |  |
| - DL DCH Transport channel identity |  |  |  |  | Not present |  |  | RBS-868 |  |
| - DL DSCH Transport channel identity |  |  |  |  | Not present |  |  | RBS-869 |  |
| - CHOICE DL MAC header type |  |  |  |  | MAC-ehs |  |  | RBS-870 |  |
| - DL HS-DSCH MAC-ehs Queue Id |  |  |  |  | 0 |  |  | RBS-871 |  |
| - Logical channel identity |  |  |  |  | 7 |  |  | RBS-872 |  |
| - RAB information for setup |  |  | A18 |  |  | Rel-7 |  | RBS-873 |  |
| - RAB info |  |  |  |  | (high-speed UM DTCH for PS domain) |  |  | RBS-874 |  |
| - RAB identity |  |  |  |  | 0000 0101B The first/ leftmost bit of the bit string contains the most significant bit of the RAB identity. |  |  | RBS-875 |  |
| - CN domain identity |  |  |  |  | PS domain |  |  | RBS-876 |  |
| - NAS Synchronization Indicator |  |  |  |  | Not Present |  |  | RBS-877 |  |
| - Re-establishment timer |  |  |  |  | useT315 |  |  | RBS-878 |  |
| - RB information to setup |  |  |  |  |  |  |  | RBS-879 |  |
| - RB identity |  |  |  |  | 25 |  |  | RBS-880 |  |
| - PDCP info |  |  |  |  |  |  |  | RBS-881 |  |
| - Support for lossless SRNS relocation |  |  |  |  | FALSE |  |  | RBS-882 |  |
| - Max PDCP SN window size |  |  |  |  | Not present |  |  | RBS-883 |  |
| - PDCP PDU header |  |  |  |  | Absent |  |  | RBS-884 |  |
| - Header compression information |  |  |  |  | Not present |  |  | RBS-885 |  |
| - CHOICE RLC info type |  |  |  |  | RLC info |  |  | RBS-886 |  |
| - CHOICE Uplink RLC mode |  |  |  |  | UM RLC |  |  | RBS-887 |  |
| - Transmission RLC discard |  |  |  |  | Not present |  |  | RBS-888 |  |
| - CHOICE Downlink RLC mode |  |  |  |  | UM RLC |  |  | RBS-889 |  |
| - DL UM RLC LI size |  |  |  |  | 7 |  |  | RBS-890 |  |
| - DL Reception Window Size |  |  |  |  | Not present |  |  | RBS-891 |  |
| - One sided RLC re-establishment |  |  |  |  | FALSE |  |  | RBS-892 |  |
| - Alternative E-bit interpretation |  |  |  |  | TRUE |  |  | RBS-893 |  |
| - Use special value of HE field |  |  |  |  | Not present |  |  | RBS-894 |  |
| - RB mapping info |  |  |  |  |  |  |  | RBS-895 |  |
| - Information for each multiplexing option |  |  |  |  | 1 RBMuxOption |  |  | RBS-896 |  |
| - RLC logical channel mapping indicator |  |  |  |  | Not present |  |  | RBS-897 |  |
| - Number of uplink RLC logical channels |  |  |  |  | 1 |  |  | RBS-898 |  |
| - Uplink transport channel type |  |  |  |  | DCH |  |  | RBS-899 |  |
| - UL Transport channel identity |  |  |  |  | 1 |  |  | RBS-900 |  |
| - Logical channel identity |  |  |  |  | Not Present |  |  | RBS-901 |  |
| - CHOICE RLC size list |  |  |  |  | Configured |  |  | RBS-902 |  |
| - MAC logical channel priority |  |  |  |  | 8 |  |  | RBS-903 |  |
| - Downlink RLC logical channel info |  |  |  |  |  |  |  | RBS-904 |  |
| - Number of downlink RLC logical channels |  |  |  |  | 1 |  |  | RBS-905 |  |
| - Downlink transport channel type |  |  |  |  | HS-DSCH |  |  | RBS-906 |  |
| - DL DCH Transport channel identity |  |  |  |  | Not present |  |  | RBS-907 |  |
| - DL DSCH Transport channel identity |  |  |  |  | Not present |  |  | RBS-908 |  |
| - CHOICE DL MAC header type |  |  |  |  | MAC-ehs |  |  | RBS-909 |  |
| - DL HS-DSCH MAC-ehs Queue Id |  |  |  |  | 0 |  |  | RBS-910 |  |
| - Logical channel identity |  |  |  |  | 7 |  |  | RBS-911 |  |
| - RAB information for setup |  |  | ,A20, A21 |  |  | Rel-7 |  | RBS-912 |  |
| - RAB info |  |  |  |  | (high-speed AM DTCH for PS domain) |  |  | RBS-913 |  |
| - RAB identity |  |  |  |  | 0000 0101B The first/ leftmost bit of the bit string contains the most significant bit of the RAB identity. |  |  | RBS-914 |  |
| - CN domain identity |  |  |  |  | PS domain |  |  | RBS-915 |  |
| - NAS Synchronization Indicator |  |  |  |  | Not Present |  |  | RBS-916 |  |
| - Re-establishment timer |  |  |  |  | useT315 |  |  | RBS-917 |  |
| - RB information to setup |  |  |  |  |  |  |  | RBS-918 |  |
| - RB identity |  |  |  |  | 25 |  |  | RBS-919 |  |
| - PDCP info |  |  |  |  |  |  |  | RBS-920 |  |
| - Support for lossless SRNS relocation |  |  |  |  | FALSE |  |  | RBS-921 |  |
| - Max PDCP SN window size |  |  |  |  | Not present |  |  | RBS-922 |  |
| - PDCP PDU header |  |  |  |  | Absent |  |  | RBS-923 |  |
| - Header compression information |  |  |  |  | Not present |  |  | RBS-924 |  |
| - CHOICE RLC info type |  |  |  |  | RLC info |  |  | RBS-925 |  |
| - CHOICE Uplink RLC mode |  |  |  |  | AM RLC |  |  | RBS-926 |  |
| - Transmission RLC discard |  |  |  |  |  |  |  | RBS-927 |  |
| - CHOICE SDU discard mode |  |  |  |  | No Discard |  |  | RBS-928 |  |
| - MAX_DAT |  |  |  |  | 15 |  |  | RBS-929 |  |
| - Transmission window size |  |  |  |  | 256 |  |  | RBS-930 |  |
| - Timer_RST |  |  |  |  | 500 |  |  | RBS-931 |  |
| - Max_RST |  |  |  |  | 4 |  |  | RBS-932 |  |
| - Polling info |  |  |  |  |  |  |  | RBS-933 |  |
| - Timer_poll_prohibit |  |  |  |  | 100 |  |  | RBS-934 |  |
| - Timer_poll |  |  |  |  | 100 |  |  | RBS-935 |  |
| - Poll_PDU |  |  |  |  | Not Present |  |  | RBS-936 |  |
| - Poll_SDU |  |  |  |  | 1 |  |  | RBS-937 |  |
| - Last transmission PDU poll |  |  |  |  | TRUE |  |  | RBS-938 |  |
| - Last retransmission PDU poll |  |  |  |  | TRUE |  |  | RBS-939 |  |
| - Poll_Windows |  |  |  |  | 99 |  |  | RBS-940 |  |
| - Timer_poll_periodic |  |  |  |  | Not Present |  |  | RBS-941 |  |
| - CHOICE Downlink RLC mode |  |  |  |  | AM RLC |  |  | RBS-942 |  |
| - CHOICE Downlink RLC PDU Size |  |  |  |  | Reference to clause 6 Parameter Set |  |  | RBS-943 |  |
| - In-sequence delivery |  |  |  |  | TRUE |  |  | RBS-944 |  |
| - Receiving window size |  |  |  |  | 768 |  |  | RBS-945 |  |
| - Downlink RLC status info |  |  |  |  |  |  |  | RBS-946 |  |
| - Timer_status_prohibit |  |  |  |  | 100 |  |  | RBS-947 |  |
| - Timer_EPC |  |  |  |  | Not Present |  |  | RBS-948 |  |
| - Missing PDU indicator |  |  |  |  | TRUE |  |  | RBS-949 |  |
| - Timer_STATUS_periodic |  |  |  |  | Not Present |  |  | RBS-950 |  |
| - One sided RLC re-establishment |  |  |  |  | FALSE |  |  | RBS-951 |  |
| - RB mapping info |  |  |  |  |  |  |  | RBS-952 |  |
| - Information for each multiplexing option |  |  |  |  | 1 RBMuxOption |  |  | RBS-953 |  |
| - RLC logical channel mapping indicator |  |  |  |  | Not Present |  |  | RBS-954 |  |
| - Number of uplink RLC logical channels |  |  |  |  | 1 |  |  | RBS-955 |  |
| - Uplink transport channel type |  |  |  |  | E-DCH |  |  | RBS-956 |  |
| - Logical channel identity |  |  |  |  | 7 |  |  | RBS-957 |  |
| - E-DCH MAC-d flow identity |  |  |  |  | 2 |  |  | RBS-958 |  |
| - CHOICE RLC PDU size |  |  | MAC-I-FIXED |  | Fixed size | Rel-8 |  | RBS-959 |  |
| - DDI |  |  |  |  | 5 |  |  | RBS-960 |  |
| - RLC PDU size list |  |  |  |  | 1 RLC PDU size |  |  | RBS-961 |  |
| - RLC PDU size |  |  |  |  | 336 bits |  |  | RBS-962 |  |
| - CHOICE RLC PDU size |  |  | MAC-I-FLEX |  | Flexible size | Rel-8 |  | RBS-963 |  |
| - Length indicator size |  |  |  |  | - 15 bit |  |  | RBS-964 |  |
| - Minimum UL RLC PDU size |  |  |  |  | See clause 6.10 |  |  | RBS-965 |  |
| - Largest UL RLC PDU size |  |  |  |  | See clause 6.10 |  |  | RBS-966 |  |
| - Include in scheduling info |  |  |  |  | TRUE |  |  | RBS-967 |  |
| - MAC logical channel priority |  |  |  |  | 8 |  |  | RBS-968 |  |
| - Downlink RLC logical channel info |  |  |  |  |  |  |  | RBS-969 |  |
| - Number of downlink RLC logical channels |  |  |  |  | 1 |  |  | RBS-970 |  |
| - Downlink transport channel type |  |  |  |  | HS-DSCH |  |  | RBS-971 |  |
| - DL DCH Transport channel identity |  |  |  |  | Not present |  |  | RBS-972 |  |
| - DL DSCH Transport channel identity |  |  |  |  | Not present |  |  | RBS-973 |  |
| - CHOICE DL MAC header type |  |  |  |  | MAC-hs |  |  | RBS-974 |  |
| - DL HS-DSCH MAC-d flow identity |  |  |  |  | 0 |  |  | RBS-975 |  |
| - Logical channel identity |  |  |  |  | Not Present |  |  | RBS-976 |  |
| - RAB information for setup |  |  | A21 |  |  | Rel-7 |  | RBS-977 |  |
| - RAB info |  |  |  |  | (Conversational UM DTCH for PS domain) |  |  | RBS-978 |  |
| - RAB identity |  |  |  |  | 0000 0110B The first/ leftmost bit of the bit string contains the most significant bit of the RAB identity. |  |  | RBS-979 |  |
| - CN domain identity |  |  |  |  | PS domain |  |  | RBS-980 |  |
| - NAS Synchronization Indicator |  |  |  |  | Not Present |  |  | RBS-981 |  |
| - Re-establishment timer |  |  |  |  | useT314 |  |  | RBS-982 |  |
| - RB information to setup |  |  |  |  |  |  |  | RBS-983 |  |
| - RB identity |  |  |  |  | 27 |  |  | RBS-984 |  |
| - PDCP info |  |  |  |  |  |  |  | RBS-985 |  |
| - Support for lossless SRNS relocation |  |  |  |  | FALSE |  |  | RBS-986 |  |
| - Max PDCP SN window size |  |  |  |  | Not present |  |  | RBS-987 |  |
| - PDCP PDU header |  |  |  |  | Absent |  |  | RBS-988 |  |
| - Header compression information |  |  |  |  | Not present |  |  | RBS-989 |  |
| - CHOICE RLC info type |  |  |  |  | RLC info |  |  | RBS-990 |  |
| - CHOICE Uplink RLC mode |  |  |  |  | UM RLC |  |  | RBS-991 |  |
| - Transmission RLC discard |  |  |  |  | Not present |  |  | RBS-992 |  |
| - CHOICE Downlink RLC mode |  |  |  |  | UM RLC |  |  | RBS-993 |  |
| - DL UM RLC LI size |  |  |  |  | 7 |  |  | RBS-994 |  |
| - DL Reception Window Size |  |  |  |  | 32 |  |  | RBS-995 |  |
| - One sided RLC re-establishment |  |  |  |  | FALSE |  |  | RBS-996 |  |
| - Alternative E-bit interpretation |  |  |  |  | Not present |  |  | RBS-997 |  |
| - RB mapping info |  |  |  |  |  |  |  | RBS-998 |  |
| - Information for each multiplexing option |  |  |  |  | 1 RBMuxOption |  |  | RBS-999 |  |
| - RLC logical channel mapping indicator |  |  |  |  | Not Present |  |  | RBS-1000 |  |
| - Number of uplink RLC logical channels |  |  |  |  | 1 |  |  | RBS-1001 |  |
| - Uplink transport channel type |  |  |  |  | E-DCH |  |  | RBS-1002 |  |
| - Logical channel identity |  |  |  |  | 9 |  |  | RBS-1003 |  |
| - E-DCH MAC-d flow identity |  |  |  |  | 4 |  |  | RBS-1004 |  |
| - CHOICE RLC PDU size |  |  | MAC-I-FIXED |  | Fixed size | Rel-8 |  | RBS-1005 |  |
| - DDI |  |  |  |  | 7 |  |  | RBS-1006 |  |
| - RLC PDU size list |  |  |  |  | 12 RLC PDU sizes |  |  | RBS-1007 |  |
| - RLC PDU size |  |  |  |  | 96 bits |  |  | RBS-1008 |  |
| - RLC PDU size |  |  |  |  | 112 bits |  |  | RBS-1009 |  |
| - RLC PDU size |  |  |  |  | 144 bits |  |  | RBS-1010 |  |
| - RLC PDU size |  |  |  |  | 160 bits |  |  | RBS-1011 |  |
| - RLC PDU size |  |  |  |  | 176 bits |  |  | RBS-1012 |  |
| - RLC PDU size |  |  |  |  | 192 bits |  |  | RBS-1013 |  |
| - RLC PDU size |  |  |  |  | 208 bits |  |  | RBS-1014 |  |
| - RLC PDU size |  |  |  |  | 224 bits |  |  | RBS-1015 |  |
| - RLC PDU size |  |  |  |  | 288 bits |  |  | RBS-1016 |  |
| - RLC PDU size |  |  |  |  | 296 bits |  |  | RBS-1017 |  |
| - RLC PDU size |  |  |  |  | 312 bits |  |  | RBS-1018 |  |
| - RLC PDU size |  |  |  |  | 336 bits |  |  | RBS-1019 |  |
| - CHOICE RLC PDU size |  |  | MAC-I-FLEX |  | Flexible size | Rel-8 |  | RBS-1020 |  |
| - Length indicator size |  |  |  |  | Not present |  |  | RBS-1021 |  |
| - Minimum UL RLC PDU size |  |  |  |  | See clause 6.10 |  |  | RBS-1022 |  |
| - Largest UL RLC PDU size |  |  |  |  | See clause 6.10 |  |  | RBS-1023 |  |
| - Include in scheduling info |  |  |  |  | TRUE |  |  | RBS-1024 |  |
| - MAC logical channel priority |  |  |  |  | 8 |  |  | RBS-1025 |  |
| - Downlink RLC logical channel info |  |  |  |  |  |  |  | RBS-1026 |  |
| - Number of downlink RLC logical channels |  |  |  |  | 1 |  |  | RBS-1027 |  |
| - Downlink transport channel type |  |  |  |  | HS-DSCH |  |  | RBS-1028 |  |
| - DL DCH Transport channel identity |  |  |  |  | Not present |  |  | RBS-1029 |  |
| - DL DSCH Transport channel identity |  |  |  |  | Not present |  |  | RBS-1030 |  |
| - CHOICE DL MAC header type |  |  |  |  | MAC-hs |  |  | RBS-1031 |  |
| - DL HS-DSCH MAC-d flow identity |  |  |  |  | 3 |  |  | RBS-1032 |  |
| - Logical channel identity |  |  |  |  | Not Present |  |  | RBS-1033 |  |
| - RAB information for setup |  |  | A22 |  |  | Rel-7 |  | RBS-1034 |  |
| - RAB info |  |  |  |  | (second high-speed UM DTCH for PS domain) |  |  | RBS-1035 |  |
| - RAB identity |  |  |  |  | 0000 0110B The first/ leftmost bit of the bit string contains the most significant bit of the RAB identity. |  |  | RBS-1036 |  |
| - CN domain identity |  |  |  |  | PS domain |  |  | RBS-1037 |  |
| - NAS Synchronization Indicator |  |  |  |  | Not Present |  |  | RBS-1038 |  |
| - Re-establishment timer |  |  |  |  | useT315 |  |  | RBS-1039 |  |
| - RB information to setup |  |  |  |  |  |  |  | RBS-1040 |  |
| - RB identity |  |  |  |  | 27 |  |  | RBS-1041 |  |
| - PDCP info |  |  |  |  |  |  |  | RBS-1042 |  |
| - Support for lossless SRNS relocation |  |  |  |  | FALSE |  |  | RBS-1043 |  |
| - Max PDCP SN window size |  |  |  |  | Not present |  |  | RBS-1044 |  |
| - PDCP PDU header |  |  |  |  | Absent |  |  | RBS-1045 |  |
| - Header compression information |  |  |  |  | Not present |  |  | RBS-1046 |  |
| - CHOICE RLC info type |  |  |  |  | RLC info |  |  | RBS-1047 |  |
| - CHOICE Uplink RLC mode |  |  |  |  | UM RLC |  |  | RBS-1048 |  |
| - Transmission RLC discard |  |  |  |  | Not present |  |  | RBS-1049 |  |
| - CHOICE Downlink RLC mode |  |  |  |  | UM RLC |  |  | RBS-1050 |  |
| - DL UM RLC LI size |  |  |  |  | 15 |  |  | RBS-1051 |  |
| - DL Reception Window Size |  |  |  |  | Not present |  |  | RBS-1052 |  |
| - One sided RLC re-establishment |  |  |  |  | FALSE |  |  | RBS-1053 |  |
| - Alternative E-bit interpretation |  |  |  |  | TRUE |  |  | RBS-1054 |  |
| - Use special value of HE field |  |  |  |  | Not present |  |  | RBS-1055 |  |
| - RB mapping info |  |  |  |  |  |  |  | RBS-1056 |  |
| - Information for each multiplexing option |  |  |  |  | 1 RBMuxOption |  |  | RBS-1057 |  |
| - RLC logical channel mapping indicator |  |  |  |  | Not present |  |  | RBS-1058 |  |
| - Number of uplink RLC logical channels |  |  |  |  | 1 |  |  | RBS-1059 |  |
| - Uplink transport channel type |  |  |  |  | E-DCH |  |  | RBS-1060 |  |
| - Logical channel identity |  |  |  |  | 9 |  |  | RBS-1061 |  |
| - E-DCH MAC-d flow identity |  |  |  |  | 4 |  |  | RBS-1062 |  |
| - CHOICE RLC PDU size |  |  | MAC-I-FIXED |  | Fixed size | Rel-8 |  | RBS-1063 |  |
| - DDI |  |  |  |  | 7 |  |  | RBS-1064 |  |
| - RLC PDU size list |  |  |  |  | See clause 6.10 |  |  | RBS-1065 |  |
| - CHOICE RLC PDU size |  |  | MAC-I-FLEX |  | Flexible size | Rel-8 |  | RBS-1066 |  |
| - Length indicator size |  |  |  |  | - 15 bit |  |  | RBS-1067 |  |
| - Minimum UL RLC PDU size |  |  |  |  | See clause 6.10 |  |  | RBS-1068 |  |
| - Largest UL RLC PDU size |  |  |  |  | See clause 6.10 |  |  | RBS-1069 |  |
| - Include in scheduling info |  |  |  |  | TRUE |  |  | RBS-1070 |  |
| - MAC logical channel priority |  |  |  |  | 8 |  |  | RBS-1071 |  |
| - Downlink RLC logical channel info |  |  |  |  |  |  |  | RBS-1072 |  |
| - Number of downlink RLC logical channels |  |  |  |  | 1 |  |  | RBS-1073 |  |
| - Downlink transport channel type |  |  |  |  | HS-DSCH |  |  | RBS-1074 |  |
| - DL DCH Transport channel identity |  |  |  |  | Not present |  |  | RBS-1075 |  |
| - DL DSCH Transport channel identity |  |  |  |  | Not present |  |  | RBS-1076 |  |
| - CHOICE DL MAC header type |  |  |  |  | MAC-ehs |  |  | RBS-1077 |  |
| - DL HS-DSCH MAC-ehs Queue Id |  |  |  |  | 3 |  |  | RBS-1078 |  |
| - Logical channel identity |  |  |  |  | 9 |  |  | RBS-1079 |  |
| - RAB information for setup |  |  | A22 |  |  | Rel-7 |  | RBS-1080 |  |
|  |  |  | , A25, A25b |  |  | Rel-8 |  | RBS-1081 |  |
|  |  |  | A25c |  |  | Rel-9 |  | RBS-1081a |  |
| - RAB info |  |  |  |  | (high-speed AM DTCH for PS domain) |  |  | RBS-1082 |  |
| - RAB identity |  |  |  |  | 0000 0101B The first/ leftmost bit of the bit string contains the most significant bit of the RAB identity. |  |  | RBS-1083 |  |
| - CN domain identity |  |  |  |  | PS domain |  |  | RBS-1084 |  |
| - NAS Synchronization Indicator |  |  |  |  | Not Present |  |  | RBS-1085 |  |
| - Re-establishment timer |  |  |  |  | useT315 |  |  | RBS-1086 |  |
| - RB information to setup |  |  |  |  |  |  |  | RBS-1087 |  |
| - RB identity |  |  |  |  | 25 |  |  | RBS-1088 |  |
| - PDCP info |  |  |  |  |  |  |  | RBS-1089 |  |
| - Support for lossless SRNS relocation |  |  |  |  | FALSE |  |  | RBS-1090 |  |
| - Max PDCP SN window size |  |  |  |  | Not present |  |  | RBS-1091 |  |
| - PDCP PDU header |  |  |  |  | Absent |  |  | RBS-1092 |  |
| - Header compression information |  |  |  |  | Not present |  |  | RBS-1093 |  |
| - CHOICE RLC info type |  |  |  |  | RLC info |  |  | RBS-1094 |  |
| - CHOICE Uplink RLC mode |  |  |  |  | AM RLC |  |  | RBS-1095 |  |
| - Transmission RLC discard |  |  |  |  |  |  |  | RBS-1096 |  |
| - CHOICE SDU discard mode |  |  |  |  | No Discard |  |  | RBS-1097 |  |
| - MAX_DAT |  |  |  |  | 15 |  |  | RBS-1098 |  |
| - Transmission window size |  |  |  |  | 128 |  |  | RBS-1099 |  |
| - Timer_RST |  |  |  |  | 500 |  |  | RBS-1100 |  |
| - Max_RST |  |  |  |  | 4 |  |  | RBS-1101 |  |
| - Polling info |  |  |  |  |  |  |  | RBS-1102 |  |
| - Timer_poll_prohibit |  |  |  |  | 100 |  |  | RBS-1103 |  |
| - Timer_poll |  |  |  |  | 100 |  |  | RBS-1104 |  |
| - Poll_PDU |  |  |  |  | Not Present |  |  | RBS-1105 |  |
| - Poll_SDU |  |  |  |  | 1 |  |  | RBS-1106 |  |
| - Last transmission PDU poll |  |  |  |  | TRUE |  |  | RBS-1107 |  |
| - Last retransmission PDU poll |  |  |  |  | TRUE |  |  | RBS-1108 |  |
| - Poll_Windows |  |  |  |  | 99 |  |  | RBS-1109 |  |
| - Timer_poll_periodic |  |  |  |  | Not Present |  |  | RBS-1110 |  |
| - CHOICE Downlink RLC mode |  |  |  |  | AM RLC |  |  | RBS-1111 |  |
| - CHOICE Downlink RLC PDU Size |  |  |  |  | Reference to clause 6 Parameter Set |  |  | RBS-1112 |  |
| - Length indicator size |  |  |  |  | This IE is present and set to “7” if Downlink RLC PDU Size is set to “Flexible” |  |  | RBS-1113 |  |
| - In-sequence delivery |  |  |  |  | TRUE |  |  | RBS-1114 |  |
| - Receiving window size |  |  |  |  | 768 |  |  | RBS-1115 |  |
| - Downlink RLC status info |  |  |  |  |  |  |  | RBS-1116 |  |
| - Timer_status_prohibit |  |  |  |  | 100 |  |  | RBS-1117 |  |
| - Timer_EPC |  |  |  |  | Not Present |  |  | RBS-1118 |  |
| - Missing PDU indicator |  |  |  |  | TRUE |  |  | RBS-1119 |  |
| - Timer_STATUS_periodic |  |  |  |  | Not Present |  |  | RBS-1120 |  |
| - One sided RLC re-establishment |  |  |  |  | FALSE |  |  | RBS-1121 |  |
| - Alternative E-bit interpretation |  |  |  |  | Not present |  |  | RBS-1122 |  |
| - Use special value of HE field |  |  |  |  | TRUE |  |  | RBS-1123 |  |
| - RB mapping info |  |  |  |  |  |  |  | RBS-1124 |  |
| - Information for each multiplexing option |  |  |  |  | 1 RBMuxOption |  |  | RBS-1125 |  |
| - RLC logical channel mapping indicator |  |  |  |  | Not present |  |  | RBS-1126 |  |
| - Number of uplink RLC logical channels |  |  |  |  | 1 |  |  | RBS-1127 |  |
| - Uplink transport channel type |  |  |  |  | E-DCH |  |  | RBS-1128 |  |
| - Logical channel identity |  |  |  |  | 7 |  |  | RBS-1129 |  |
| - E-DCH MAC-d flow identity |  |  |  |  | 2 |  |  | RBS-1130 |  |
| - CHOICE RLC PDU size |  |  | MAC-I-FIXED |  | Fixed size | Rel-8 |  | RBS-1131 |  |
| - DDI |  |  |  |  | 5 |  |  | RBS-1132 |  |
| - RLC PDU size list |  |  |  |  | 1 RLC PDU size |  |  | RBS-1133 |  |
| - RLC PDU size |  |  |  |  | 336 bits |  |  | RBS-1134 |  |
| - CHOICE RLC PDU size |  |  | MAC-I-FLEX |  | Flexible size | Rel-8 |  | RBS-1135 |  |
| - Length indicator size |  |  |  |  | - 15 bit |  |  | RBS-1136 |  |
| - Minimum UL RLC PDU size |  |  |  |  | See clause 6.10 |  |  | RBS-1137 |  |
| - Largest UL RLC PDU size |  |  |  |  | See clause 6.10 |  |  | RBS-1138 |  |
| - Include in scheduling info |  |  |  |  | TRUE |  |  | RBS-1139 |  |
| - MAC logical channel priority |  |  |  |  | 8 |  |  | RBS-1140 |  |
| - Downlink RLC logical channel info |  |  |  |  |  |  |  | RBS-1141 |  |
| - Number of downlink RLC logical channels |  |  |  |  | 1 |  |  | RBS-1142 |  |
| - Downlink transport channel type |  |  |  |  | HS-DSCH |  |  | RBS-1143 |  |
| - DL DCH Transport channel identity |  |  |  |  | Not present |  |  | RBS-1144 |  |
| - DL DSCH Transport channel identity |  |  |  |  | Not present |  |  | RBS-1145 |  |
| - CHOICE DL MAC header type |  |  |  |  | MAC-ehs |  |  | RBS-1146 |  |
| - DL HS-DSCH MAC-ehs Queue Id |  |  |  |  | 0 |  |  | RBS-1147 |  |
| - Logical channel identity |  |  |  |  | 7 |  |  | RBS-1148 |  |
| - RAB information for setup |  |  | A23 |  |  | Rel-7 Rel-8 |  | RBS-1149 |  |
| - RAB info |  |  |  |  | (high-speed UM DTCH for CS domain) |  |  | RBS-1150 |  |
| - RAB identity |  |  |  |  | 0000 0101B The first/ leftmost bit of the bit string contains the most significant bit of the RAB identity. |  |  | RBS-1151 |  |
| - CN domain identity |  |  |  |  | CS domain |  |  | RBS-1152 |  |
| - NAS Synchronization Indicator |  |  |  |  | ‘1010’ if WB-AMR is tested, otherwise ‘0110’ |  |  | RBS-1153 |  |
| - Re-establishment timer |  |  |  |  | useT314 |  |  | RBS-1154 |  |
| - CS-HSPA information |  |  |  |  |  |  |  | RBS-1155 |  |
| - UL AMR rate |  |  |  |  | Not Present |  |  | RBS-1156 |  |
| - Max CS delay |  |  |  |  | 60 |  |  | RBS-1157 |  |
| - RB information to setup |  |  |  |  |  |  |  | RBS-1158 |  |
| - RB identity |  |  |  |  | 26 |  |  | RBS-1159 |  |
| - PDCP info |  |  |  |  |  |  |  | RBS-1160 |  |
| - Support for lossless SRNS relocation |  |  |  |  | FALSE |  |  | RBS-1161 |  |
| - Max PDCP SN window size |  |  |  |  | Not present |  |  | RBS-1162 |  |
| - PDCP PDU header |  |  |  |  | present |  |  | RBS-1163 |  |
| - Header compression information |  |  |  |  | Not present |  |  | RBS-1164 |  |
| - CHOICE RLC info type |  |  |  |  | RLC info |  |  | RBS-1165 |  |
| - CHOICE Uplink RLC mode |  |  |  |  | UM RLC |  |  | RBS-1166 |  |
| - Transmission RLC discard |  |  |  |  |  |  |  | RBS-1167 |  |
| - CHOICE SDU discard mode |  |  |  |  | Timer based no explicit |  |  | RBS-1168 |  |
| - Timer_discard |  |  |  |  | 50 |  |  | RBS-1169 |  |
| - CHOICE Downlink RLC mode |  |  |  |  | UM RLC |  |  | RBS-1170 |  |
| - DL UM RLC LI size |  |  |  |  | 7 |  |  | RBS-1171 |  |
| - DL Reception Window Size |  |  |  |  | Not present |  |  | RBS-1172 |  |
| - One sided RLC re-establishment |  |  |  |  | FALSE |  |  | RBS-1173 |  |
| - Alternative E-bit interpretation |  |  |  |  | TRUE |  |  | RBS-1174 |  |
| - Use special value of HE field |  |  |  |  | Not present |  |  | RBS-1175 |  |
| - RB mapping info |  |  |  |  |  |  |  | RBS-1176 |  |
| - Information for each multiplexing option |  |  |  |  | 1 RBMuxOption |  |  | RBS-1177 |  |
| - RLC logical channel mapping indicator |  |  |  |  | Not present |  |  | RBS-1178 |  |
| - Number of uplink RLC logical channels |  |  |  |  | 1 |  |  | RBS-1179 |  |
| - Uplink transport channel type |  |  |  |  | E-DCH |  |  | RBS-1180 |  |
| - Logical channel identity |  |  |  |  | 7 |  |  | RBS-1181 |  |
| - E-DCH MAC-d flow identity |  |  |  |  | 2 |  |  | RBS-1182 |  |
| - CHOICE RLC PDU size |  |  | MAC-I-FIXED |  | Fixed size | Rel-8 |  | RBS-1183 |  |
| - DDI |  |  |  |  | 6 |  |  | RBS-1184 |  |
| - RLC PDU size list |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-1185 |  |
| - CHOICE RLC PDU size |  |  | MAC-I-FLEX |  | Flexible size | Rel-8 |  | RBS-1186 |  |
| - Length indicator size |  |  |  |  | Not present |  |  | RBS-1187 |  |
| - Minimum UL RLC PDU size |  |  |  |  | See clause 6.10 |  |  | RBS-1188 |  |
| - Largest UL RLC PDU size |  |  |  |  | See clause 6.10 |  |  | RBS-1189 |  |
| - Include in scheduling info |  |  |  |  | TRUE |  |  | RBS-1190 |  |
| - MAC logical channel priority |  |  |  |  | 8 |  |  | RBS-1191 |  |
| - Downlink RLC logical channel info |  |  |  |  |  |  |  | RBS-1192 |  |
| - Number of downlink RLC logical channels |  |  |  |  | 1 |  |  | RBS-1193 |  |
| - Downlink transport channel type |  |  |  |  | HS-DSCH |  |  | RBS-1194 |  |
| - DL DCH Transport channel identity |  |  |  |  | Not present |  |  | RBS-1195 |  |
| - DL DSCH Transport channel identity |  |  |  |  | Not present |  |  | RBS-1196 |  |
| - CHOICE DL MAC header type |  |  |  |  | MAC-ehs |  |  | RBS-1197 |  |
| - DL HS-DSCH MAC-ehs Queue Id |  |  |  |  | 0 |  |  | RBS-1198 |  |
| - Logical channel identity |  |  |  |  | 7 |  |  | RBS-1199 |  |
| - RAB information for setup |  |  | A24 |  |  |  |  | RBS-1200 |  |
| - RAB info |  |  |  |  | (high-speed AM DTCH for PS domain) |  |  | RBS-1201 |  |
| - RAB identity |  |  |  |  | 0000 0101B The first/ leftmost bit of the bit string contains the most significant bit of the RAB identity. |  |  | RBS-1202 |  |
| - CN domain identity |  |  |  |  | PS domain |  |  | RBS-1203 |  |
| - NAS Synchronization Indicator |  |  |  |  | Not Present |  |  | RBS-1204 |  |
| - Re-establishment timer |  |  |  |  | useT315 |  |  | RBS-1205 |  |
| - RB information to setup |  |  |  |  |  |  |  | RBS-1206 |  |
| - RB identity |  |  |  |  | 25 |  |  | RBS-1207 |  |
| - PDCP info |  |  |  |  |  |  |  | RBS-1208 |  |
| - Support for lossless SRNS relocation |  |  |  |  | FALSE |  |  | RBS-1209 |  |
| - Max PDCP SN window size |  |  |  |  | Not present |  |  | RBS-1210 |  |
| - PDCP PDU header |  |  |  |  | Absent |  |  | RBS-1211 |  |
| - Header compression information |  |  |  |  | Not present |  |  | RBS-1212 |  |
| - CHOICE RLC info type |  |  |  |  | RLC info |  |  | RBS-1213 |  |
| - CHOICE Uplink RLC mode |  |  |  |  | AM RLC |  |  | RBS-1214 |  |
| - Transmission RLC discard |  |  |  |  |  |  |  | RBS-1215 |  |
| - CHOICE SDU discard mode |  |  |  |  | No Discard |  |  | RBS-1216 |  |
| - MAX_DAT |  |  |  |  | 15 |  |  | RBS-1217 |  |
| - Transmission window size |  |  |  |  | 128 |  |  | RBS-1218 |  |
| - Timer_RST |  |  |  |  | 500 |  |  | RBS-1219 |  |
| - Max_RST |  |  |  |  | 4 |  |  | RBS-1220 |  |
| - Polling info |  |  |  |  |  |  |  | RBS-1221 |  |
| - Timer_poll_prohibit |  |  |  |  | 100 |  |  | RBS-1222 |  |
| - Timer_poll |  |  |  |  | 100 |  |  | RBS-1223 |  |
| - Poll_PDU |  |  |  |  | Not Present |  |  | RBS-1224 |  |
| - Poll_SDU |  |  |  |  | 1 |  |  | RBS-1225 |  |
| - Last transmission PDU poll |  |  |  |  | TRUE |  |  | RBS-1226 |  |
| - Last retransmission PDU poll |  |  |  |  | TRUE |  |  | RBS-1227 |  |
| - Poll_Windows |  |  |  |  | 99 |  |  | RBS-1228 |  |
| - Timer_poll_periodic |  |  |  |  | Not Present |  |  | RBS-1229 |  |
| - CHOICE Downlink RLC mode |  |  |  |  | AM RLC |  |  | RBS-1230 |  |
| - CHOICE Downlink RLC PDU Size |  |  |  |  | Reference to clause 6 Parameter Set |  |  | RBS-1231 |  |
| - In-sequence delivery |  |  |  |  | TRUE |  |  | RBS-1232 |  |
| - Receiving window size |  |  |  |  | 768 |  |  | RBS-1233 |  |
| - Downlink RLC status info |  |  |  |  |  |  |  | RBS-1234 |  |
| - Timer_status_prohibit |  |  |  |  | 100 |  |  | RBS-1235 |  |
| - Timer_EPC |  |  |  |  | Not Present |  |  | RBS-1236 |  |
| - Missing PDU indicator |  |  |  |  | TRUE |  |  | RBS-1237 |  |
| - Timer_STATUS_periodic |  |  |  |  | Not Present |  |  | RBS-1238 |  |
| - One sided RLC re-establishment |  |  |  |  | FALSE |  |  | RBS-1239 |  |
| - Alternative E-bit interpretation |  |  |  |  | Not present |  |  | RBS-1240 |  |
| - Use special value of HE field |  |  |  |  | TRUE |  |  | RBS-1241 |  |
| - RB mapping info |  |  |  |  |  |  |  | RBS-1242 |  |
| - Information for each multiplexing option |  |  |  |  | 1 RBMuxOption |  |  | RBS-1243 |  |
| - RLC logical channel mapping indicator |  |  |  |  | Not present |  |  | RBS-1244 |  |
| - Number of uplink RLC logical channels |  |  |  |  | 1 |  |  | RBS-1245 |  |
| - Uplink transport channel type |  |  |  |  | RACH |  |  | RBS-1246 |  |
| - UL Transport channel identity |  |  |  |  | Not Present |  |  | RBS-1247 |  |
| - Logical channel identity |  |  |  |  | 7 |  |  | RBS-1248 |  |
| - CHOICE RLC size list |  |  |  |  | Explicit list |  |  | RBS-1249 |  |
| - RLC size index |  |  |  |  | Reference to clause 6 Parameter Set |  |  | RBS-1250 |  |
| - MAC logical channel priority |  |  |  |  | 8 |  |  | RBS-1251 |  |
| - Downlink RLC logical channel info |  |  |  |  |  |  |  | RBS-1252 |  |
| - Number of downlink RLC logical channels |  |  |  |  | 1 |  |  | RBS-1253 |  |
| - Downlink transport channel type |  |  |  |  | HS-DSCH |  |  | RBS-1254 |  |
| - DL DCH Transport channel identity |  |  |  |  | Not present |  |  | RBS-1255 |  |
| - DL DSCH Transport channel identity |  |  |  |  | Not present |  |  | RBS-1256 |  |
| - CHOICE DL MAC header type |  |  |  |  | MAC-ehs |  |  | RBS-1257 |  |
| - DL HS-DSCH MAC-ehs Queue Id |  |  |  |  | 2 |  |  | RBS-1258 |  |
| - Logical channel identity |  |  |  |  | Not Present |  |  | RBS-1259 |  |
| - RAB information for setup |  |  | A26 |  |  | Rel-8 |  | RBS-1260 |  |
| - RAB info |  |  |  |  | (first UM DTCH for PS domain) |  |  | RBS-1261 |  |
| - RAB identity |  |  |  |  | 0000 0101B The first/ leftmost bit of the bit string contains the most significant bit of the RAB identity. |  |  | RBS-1262 |  |
| - CN domain identity |  |  |  |  | PS domain |  |  | RBS-1263 |  |
| - NAS Synchronization Indicator |  |  |  |  | Not Present |  |  | RBS-1264 |  |
| - Re-establishment timer |  |  |  |  | useT315 |  |  | RBS-1265 |  |
| - RB information to setup |  |  |  |  |  |  |  | RBS-1266 |  |
| - RB identity |  |  |  |  | 26 |  |  | RBS-1267 |  |
| - PDCP info |  |  |  |  |  |  |  | RBS-1268 |  |
| - Support for lossless SRNS relocation |  |  |  |  | FALSE |  |  | RBS-1269 |  |
| - Max PDCP SN window size |  |  |  |  | Not present |  |  | RBS-1270 |  |
| - PDCP PDU header |  |  |  |  | Absent |  |  | RBS-1271 |  |
| - Header compression information |  |  |  |  | Not present |  |  | RBS-1272 |  |
| - CHOICE RLC info type |  |  |  |  | RLC info |  |  | RBS-1273 |  |
| - CHOICE Uplink RLC mode |  |  |  |  | UM RLC |  |  | RBS-1274 |  |
| - Transmission RLC discard |  |  |  |  | Not present |  |  | RBS-1275 |  |
| - CHOICE Downlink RLC mode |  |  |  |  | UM RLC |  |  | RBS-1276 |  |
| - DL UM RLC LI size |  |  |  |  | 7 |  |  | RBS-1277 |  |
| - DL Reception Window Size |  |  |  |  | Not present |  |  | RBS-1278 |  |
| - Alternative E-bit interpretation |  |  |  |  | TRUE |  |  | RBS-1279 |  |
| - One sided RLC re-establishment |  |  |  |  | FALSE |  |  | RBS-1280 |  |
| - RB mapping info |  |  |  |  |  |  |  | RBS-1281 |  |
| - Information for each multiplexing option |  |  |  |  | 1 RBMuxOption |  |  | RBS-1282 |  |
| - RLC logical channel mapping indicator |  |  |  |  | Not Present |  |  | RBS-1283 |  |
| - Number of uplink RLC logical channels |  |  |  |  | 1 |  |  | RBS-1284 |  |
| - Uplink transport channel type |  |  |  |  | E-DCH |  |  | RBS-1285 |  |
| - Logical channel identity |  |  |  |  | 7 |  |  | RBS-1286 |  |
| - E-DCH MAC-d flow identity |  |  |  |  | 2 |  |  | RBS-1287 |  |
| - CHOICE RLC PDU size |  |  |  |  | Flexible size |  |  | RBS-1288 |  |
| - Length indicator size |  |  |  |  | Not present |  |  | RBS-1289 |  |
| - Minimum UL RLC PDU size |  |  |  |  | See clause 6.10 |  |  | RBS-1290 |  |
| - Largest UL RLC PDU size |  |  |  |  | See clause 6.10 |  |  | RBS-1291 |  |
| - Include in scheduling info |  |  |  |  | TRUE |  |  | RBS-1292 |  |
| - MAC logical channel priority |  |  |  |  | 8 |  |  | RBS-1293 |  |
| - Downlink RLC logical channel info |  |  |  |  |  |  |  | RBS-1294 |  |
| - Number of downlink RLC logical channels |  |  |  |  | 1 |  |  | RBS-1295 |  |
| - Downlink transport channel type |  |  |  |  | HS-DSCH |  |  | RBS-1296 |  |
| - DL DCH Transport channel identity |  |  |  |  | Not present |  |  | RBS-1297 |  |
| - DL DSCH Transport channel identity |  |  |  |  | Not present |  |  | RBS-1298 |  |
| - CHOICE DL MAC header type |  |  |  |  | MAC-ehs |  |  | RBS-1299 |  |
| - DL HS-DSCH MAC-ehs Queue Id |  |  |  |  | 2 |  |  | RBS-1300 |  |
| - Logical channel identity |  |  |  |  | 7 |  |  | RBS-1301 |  |
| - RAB information for setup |  |  | A26 |  |  | Rel-8 |  | RBS-1302 |  |
| - RAB info |  |  |  |  | (second high-speed UM DTCH for PS domain) |  |  | RBS-1303 |  |
| - RAB identity |  |  |  |  | 0000 0110B The first/ leftmost bit of the bit string contains the most significant bit of the RAB identity. |  |  | RBS-1304 |  |
| - CN domain identity |  |  |  |  | PS domain |  |  | RBS-1305 |  |
| - NAS Synchronization Indicator |  |  |  |  | Not Present |  |  | RBS-1306 |  |
| - Re-establishment timer |  |  |  |  | useT315 |  |  | RBS-1307 |  |
| - RB information to setup |  |  |  |  |  |  |  | RBS-1308 |  |
| - RB identity |  |  |  |  | 27 |  |  | RBS-1309 |  |
| - PDCP info |  |  |  |  |  |  |  | RBS-1310 |  |
| - Support for lossless SRNS relocation |  |  |  |  | FALSE |  |  | RBS-1311 |  |
| - Max PDCP SN window size |  |  |  |  | Not present |  |  | RBS-1312 |  |
| - PDCP PDU header |  |  |  |  | Absent |  |  | RBS-1313 |  |
| - Header compression information |  |  |  |  | Not present |  |  | RBS-1314 |  |
| - CHOICE RLC info type |  |  |  |  | RLC info |  |  | RBS-1315 |  |
| - CHOICE Uplink RLC mode |  |  |  |  | UM RLC |  |  | RBS-1316 |  |
| - Transmission RLC discard |  |  |  |  | Not present |  |  | RBS-1317 |  |
| - CHOICE Downlink RLC mode |  |  |  |  | UM RLC |  |  | RBS-1318 |  |
| - DL UM RLC LI size |  |  |  |  | 7 |  |  | RBS-1319 |  |
| - DL Reception Window Size |  |  |  |  | Not present |  |  | RBS-1320 |  |
| - Alternative E-bit interpretation |  |  |  |  | TRUE |  |  | RBS-1321 |  |
| - One sided RLC re-establishment |  |  |  |  | FALSE |  |  | RBS-1322 |  |
| - RB mapping info |  |  |  |  |  |  |  | RBS-1323 |  |
| - Information for each multiplexing option |  |  |  |  | 1 RBMuxOption |  |  | RBS-1324 |  |
| - RLC logical channel mapping indicator |  |  |  |  | Not Present |  |  | RBS-1325 |  |
| - Number of uplink RLC logical channels |  |  |  |  | 1 |  |  | RBS-1326 |  |
| - Uplink transport channel type |  |  |  |  | E-DCH |  |  | RBS-1327 |  |
| - Logical channel identity |  |  |  |  | 8 |  |  | RBS-1328 |  |
| - E-DCH MAC-d flow identity |  |  |  |  | 3 |  |  | RBS-1329 |  |
| - CHOICE RLC PDU size |  |  |  |  | Flexible size |  |  | RBS-1330 |  |
| - Length indicator size |  |  |  |  | Not present |  |  | RBS-1331 |  |
| - Minimum UL RLC PDU size |  |  |  |  | See clause 6.10 |  |  | RBS-1332 |  |
| - Largest UL RLC PDU size |  |  |  |  | See clause 6.10 |  |  | RBS-1333 |  |
| - Include in scheduling info |  |  |  |  | TRUE |  |  | RBS-1334 |  |
| - MAC logical channel priority |  |  |  |  | 8 |  |  | RBS-1335 |  |
| - Downlink RLC logical channel info |  |  |  |  |  |  |  | RBS-1336 |  |
| - Number of downlink RLC logical channels |  |  |  |  | 1 |  |  | RBS-1337 |  |
| - Downlink transport channel type |  |  |  |  | HS-DSCH |  |  | RBS-1338 |  |
| - DL DCH Transport channel identity |  |  |  |  | Not present |  |  | RBS-1339 |  |
| - DL DSCH Transport channel identity |  |  |  |  | Not present |  |  | RBS-1340 |  |
| - CHOICE DL MAC header type |  |  |  |  | MAC-ehs |  |  | RBS-1341 |  |
| - DL HS-DSCH MAC-ehs Queue Id |  |  |  |  | 3 |  |  | RBS-1342 |  |
| - Logical channel identity |  |  |  |  | 8 |  |  | RBS-1343 |  |
| - RAB information for setup |  |  | A26 |  |  | Rel-8 |  | RBS-1344 |  |
| - RAB info |  |  |  |  | (third high-speed UM DTCH for PS domain) |  |  | RBS-1345 |  |
| - RAB identity |  |  |  |  | 0000 0111B The first/ leftmost bit of the bit string contains the most significant bit of the RAB identity. |  |  | RBS-1346 |  |
| - CN domain identity |  |  |  |  | PS domain |  |  | RBS-1347 |  |
| - NAS Synchronization Indicator |  |  |  |  | Not Present |  |  | RBS-1348 |  |
| - Re-establishment timer |  |  |  |  | useT315 |  |  | RBS-1349 |  |
| - RB information to setup |  |  |  |  |  |  |  | RBS-1350 |  |
| - RB identity |  |  |  |  | 21 |  |  | RBS-1351 |  |
| - PDCP info |  |  |  |  |  |  |  | RBS-1352 |  |
| - Support for lossless SRNS relocation |  |  |  |  | FALSE |  |  | RBS-1353 |  |
| - Max PDCP SN window size |  |  |  |  | Not present |  |  | RBS-1354 |  |
| - PDCP PDU header |  |  |  |  | Absent |  |  | RBS-1355 |  |
| - Header compression information |  |  |  |  | Not present |  |  | RBS-1356 |  |
| - CHOICE RLC info type |  |  |  |  | RLC info |  |  | RBS-1357 |  |
| - CHOICE Uplink RLC mode |  |  |  |  | UM RLC |  |  | RBS-1358 |  |
| - Transmission RLC discard |  |  |  |  | Not present |  |  | RBS-1359 |  |
| - CHOICE Downlink RLC mode |  |  |  |  | UM RLC |  |  | RBS-1360 |  |
| - DL UM RLC LI size |  |  |  |  | 7 |  |  | RBS-1361 |  |
| - DL Reception Window Size |  |  |  |  | Not present |  |  | RBS-1362 |  |
| - Alternative E-bit interpretation |  |  |  |  | TRUE |  |  | RBS-1363 |  |
| - One sided RLC re-establishment |  |  |  |  | FALSE |  |  | RBS-1364 |  |
| - RB mapping info |  |  |  |  |  |  |  | RBS-1365 |  |
| - Information for each multiplexing option |  |  |  |  | 1 RBMuxOption |  |  | RBS-1366 |  |
| - RLC logical channel mapping indicator |  |  |  |  | Not Present |  |  | RBS-1367 |  |
| - Number of uplink RLC logical channels |  |  |  |  | 1 |  |  | RBS-1368 |  |
| - Uplink transport channel type |  |  |  |  | E-DCH |  |  | RBS-1369 |  |
| - Logical channel identity |  |  |  |  | 9 |  |  | RBS-1370 |  |
| - E-DCH MAC-d flow identity |  |  |  |  | 4 |  |  | RBS-1371 |  |
| - CHOICE RLC PDU size |  |  |  |  | Flexible size |  |  | RBS-1372 |  |
| - Length indicator size |  |  |  |  | Not present |  |  | RBS-1373 |  |
| - Minimum UL RLC PDU size |  |  |  |  | See clause 6.10 |  |  | RBS-1374 |  |
| - Largest UL RLC PDU size |  |  |  |  | See clause 6.10 |  |  | RBS-1375 |  |
| - Include in scheduling info |  |  |  |  | TRUE |  |  | RBS-1376 |  |
| - MAC logical channel priority |  |  |  |  | 8 |  |  | RBS-1377 |  |
| - Downlink RLC logical channel info |  |  |  |  |  |  |  | RBS-1378 |  |
| - Number of downlink RLC logical channels |  |  |  |  | 1 |  |  | RBS-1379 |  |
| - Downlink transport channel type |  |  |  |  | HS-DSCH |  |  | RBS-1380 |  |
| - DL DCH Transport channel identity |  |  |  |  | Not present |  |  | RBS-1381 |  |
| - DL DSCH Transport channel identity |  |  |  |  | Not present |  |  | RBS-1382 |  |
| - CHOICE DL MAC header type |  |  |  |  | MAC-ehs |  |  | RBS-1383 |  |
| - DL HS-DSCH MAC-ehs Queue Id |  |  |  |  | 4 |  |  | RBS-1384 |  |
| - Logical channel identity |  |  |  |  | 9 |  |  | RBS-1385 |  |
| - RAB information for setup |  |  | A27, A27a |  |  | Rel-8 |  | RBS-1386 |  |
| - RAB info |  |  |  |  | (high-speed UM DTCH for PS domain) |  |  | RBS-1387 |  |
| - RAB identity |  |  |  |  | 0000 0101B The first/ leftmost bit of the bit string contains the most significant bit of the RAB identity. |  |  | RBS-1388 |  |
| - CN domain identity |  |  |  |  | PS domain |  |  | RBS-1389 |  |
| - NAS Synchronization Indicator |  |  |  |  | Not Present |  |  | RBS-1390 |  |
| - Re-establishment timer |  |  |  |  | useT315 |  |  | RBS-1391 |  |
| - RB information to setup |  |  |  |  |  |  |  | RBS-1392 |  |
| - RB identity |  |  |  |  | 25 |  |  | RBS-1393 |  |
| - PDCP info |  |  |  |  |  |  |  | RBS-1394 |  |
| - Support for lossless SRNS relocation |  |  |  |  | FALSE |  |  | RBS-1395 |  |
| - Max PDCP SN window size |  |  |  |  | Not present |  |  | RBS-1396 |  |
| - PDCP PDU header |  |  |  |  | Absent |  |  | RBS-1397 |  |
| - Header compression information |  |  |  |  | Not present |  |  | RBS-1398 |  |
| - CHOICE RLC info type |  |  |  |  | RLC info |  |  | RBS-1399 |  |
| - CHOICE Uplink RLC mode |  |  |  |  | UM RLC |  |  | RBS-1400 |  |
| - Transmission RLC discard |  |  |  |  | Not Present |  |  | RBS-1401 |  |
| - CHOICE Downlink RLC mode |  |  |  |  | UM RLC |  |  | RBS-1402 |  |
| - DL UM RLC LI size |  |  |  |  | 15 |  |  | RBS-1403 |  |
| - DL Reception Window Size |  |  |  |  | Not present |  |  | RBS-1404 |  |
| - One sided RLC re-establishment |  |  |  |  | FALSE |  |  | RBS-1405 |  |
| - Alternative E-bit interpretation |  |  |  |  | TRUE |  |  | RBS-1406 |  |
| - Use special value of HE field |  |  |  |  | Not present |  |  | RBS-1407 |  |
| - RB mapping info |  |  |  |  |  |  |  | RBS-1408 |  |
| - Information for each multiplexing option |  |  |  |  | 1 RBMuxOption |  |  | RBS-1409 |  |
| - RLC logical channel mapping indicator |  |  |  |  | Not present |  |  | RBS-1410 |  |
| - Number of uplink RLC logical channels |  |  |  |  | 1 |  |  | RBS-1411 |  |
| - Uplink transport channel type |  |  |  |  | E-DCH |  |  | RBS-1412 |  |
| - Logical channel identity |  |  |  |  | 7 |  |  | RBS-1413 |  |
| - E-DCH MAC-d flow identity |  |  |  |  | 2 |  |  | RBS-1414 |  |
| - CHOICE RLC PDU size |  |  |  |  | Flexible size |  |  | RBS-1415 |  |
| - Length indicator size |  |  |  |  | Not present |  |  | RBS-1416 |  |
| - Minimum UL RLC PDU size |  |  |  |  | See clause 6.10 |  |  | RBS-1417 |  |
| - Largest UL RLC PDU size |  |  |  |  | See clause 6.10 |  |  | RBS-1418 |  |
| - Include in scheduling info |  |  |  |  | TRUE |  |  | RBS-1419 |  |
| - MAC logical channel priority |  |  |  |  | 8 |  |  | RBS-1420 |  |
| - Downlink RLC logical channel info |  |  |  |  |  |  |  | RBS-1421 |  |
| - Number of downlink RLC logical channels |  |  |  |  | 1 |  |  | RBS-1422 |  |
| - Downlink transport channel type |  |  |  |  | HS-DSCH |  |  | RBS-1423 |  |
| - DL DCH Transport channel identity |  |  |  |  | Not present |  |  | RBS-1424 |  |
| - DL DSCH Transport channel identity |  |  |  |  | Not present |  |  | RBS-1425 |  |
| - CHOICE DL MAC header type |  |  |  |  | MAC-ehs |  |  | RBS-1426 |  |
| - DL HS-DSCH MAC-ehs Queue Id |  |  |  |  | 0 |  |  | RBS-1427 |  |
| - Logical channel identity |  |  |  |  | 7 |  |  | RBS-1428 |  |
| - RAB information for setup |  |  | A29 |  |  | Rel-8 |  | RBS-1429 |  |
| - RAB info |  |  |  |  | (high-speed AM DTCH for PS domain) |  |  | RBS-1430 |  |
| - RAB identity |  |  |  |  | 0000 0101B |  |  | RBS-1431 |  |
| The first/ leftmost bit of the bit string contains the most significant bit of the RAB identity. |  |  |  |  |  |  |  | RBS-1432 |  |
| - CN domain identity |  |  |  |  | PS domain |  |  | RBS-1433 |  |
| - NAS Synchronization Indicator |  |  |  |  | Not Present |  |  | RBS-1434 |  |
| - Re-establishment timer |  |  |  |  | useT315 |  |  | RBS-1435 |  |
| - RB information to setup |  |  |  |  |  |  |  | RBS-1436 |  |
| - RB identity |  |  |  |  | 25 |  |  | RBS-1437 |  |
| - PDCP info |  |  |  |  |  |  |  | RBS-1438 |  |
| - Support for lossless SRNS relocation |  |  |  |  | FALSE |  |  | RBS-1439 |  |
| - Max PDCP SN window size |  |  |  |  | Not present |  |  | RBS-1440 |  |
| - PDCP PDU header |  |  |  |  | Absent |  |  | RBS-1441 |  |
| - Header compression information |  |  |  |  | Not present |  |  | RBS-1442 |  |
| - CHOICE RLC info type |  |  |  |  | RLC info |  |  | RBS-1443 |  |
| - CHOICE Uplink RLC mode |  |  |  |  | AM RLC |  |  | RBS-1444 |  |
| - Transmission RLC discard |  |  |  |  |  |  |  | RBS-1445 |  |
| - CHOICE SDU discard mode |  |  |  |  | No Discard |  |  | RBS-1446 |  |
| - MAX_DAT |  |  |  |  | 15 |  |  | RBS-1447 |  |
| - Transmission window size |  |  |  |  | 128 |  |  | RBS-1448 |  |
| - Timer_RST |  |  |  |  | 500 |  |  | RBS-1449 |  |
| - Max_RST |  |  |  |  | 4 |  |  | RBS-1450 |  |
| - Polling info |  |  |  |  |  |  |  | RBS-1451 |  |
| - Timer_poll_prohibit |  |  |  |  | 100 |  |  | RBS-1452 |  |
| - Timer_poll |  |  |  |  | 100 |  |  | RBS-1453 |  |
| - Poll_PDU |  |  |  |  | Not Present |  |  | RBS-1454 |  |
| - Poll_SDU |  |  |  |  | 1 |  |  | RBS-1455 |  |
| - Last transmission PDU poll |  |  |  |  | TRUE |  |  | RBS-1456 |  |
| - Last retransmission PDU poll |  |  |  |  | TRUE |  |  | RBS-1457 |  |
| - Poll_Windows |  |  |  |  | 99 |  |  | RBS-1458 |  |
| - Timer_poll_periodic |  |  |  |  | Not Present |  |  | RBS-1459 |  |
| - CHOICE Downlink RLC mode |  |  |  |  | AM RLC |  |  | RBS-1460 |  |
| - CHOICE Downlink RLC PDU Size |  |  |  |  | Reference to clause 6 Parameter Set |  |  | RBS-1461 |  |
| - In-sequence delivery |  |  |  |  | TRUE |  |  | RBS-1462 |  |
| - Receiving window size |  |  |  |  | 768 |  |  | RBS-1463 |  |
| - Downlink RLC status info |  |  |  |  |  |  |  | RBS-1464 |  |
| - Timer_status_prohibit |  |  |  |  | 100 |  |  | RBS-1465 |  |
| - Timer_EPC |  |  |  |  | Not Present |  |  | RBS-1466 |  |
| - Missing PDU indicator |  |  |  |  | TRUE |  |  | RBS-1467 |  |
| - Timer_STATUS_periodic |  |  |  |  | Not Present |  |  | RBS-1468 |  |
| - One sided RLC re-establishment |  |  |  |  | FALSE |  |  | RBS-1469 |  |
| - Alternative E-bit interpretation |  |  |  |  | Not present |  |  | RBS-1470 |  |
| - Use special value of HE field |  |  |  |  | TRUE |  |  | RBS-1471 |  |
| - RB mapping info |  |  |  |  |  |  |  | RBS-1472 |  |
| - Information for each multiplexing option |  |  |  |  | 1 RBMuxOption |  |  | RBS-1473 |  |
| - RLC logical channel mapping indicator |  |  |  |  | Not present |  |  | RBS-1474 |  |
| - Number of uplink RLC logical channels |  |  |  |  | 1 |  |  | RBS-1475 |  |
| - Uplink transport channel type |  |  |  |  | E-DCH |  |  | RBS-1476 |  |
| - Logical channel identity |  |  |  |  | 7 |  |  | RBS-1477 |  |
| - E-DCH MAC-d flow identity |  |  |  |  | 0 |  |  | RBS-1478 |  |
| - CHOICE RLC PDU size |  |  |  |  | Flexible size |  |  | RBS-1479 |  |
| - Length indicator size |  |  |  |  | 15 bit |  |  | RBS-1480 |  |
| - Minimum UL RLC PDU size |  |  |  |  | See clause 6.10 |  |  | RBS-1481 |  |
| - Largest UL RLC PDU size |  |  |  |  | See clause 6.10 |  |  | RBS-1482 |  |
| - Include in scheduling info |  |  |  |  | TRUE |  |  | RBS-1483 |  |
| - MAC logical channel priority |  |  |  |  | 8 |  |  | RBS-1484 |  |
| - Downlink RLC logical channel info |  |  |  |  |  |  |  | RBS-1485 |  |
| - Number of downlink RLC logical channels |  |  |  |  | 1 |  |  | RBS-1486 |  |
| - Downlink transport channel type |  |  |  |  | HS-DSCH |  |  | RBS-1487 |  |
| - DL DCH Transport channel identity |  |  |  |  | Not present |  |  | RBS-1488 |  |
| - DL DSCH Transport channel identity |  |  |  |  | Not present |  |  | RBS-1489 |  |
| - CHOICE DL MAC header type |  |  |  |  | MAC-ehs |  |  | RBS-1490 |  |
| - DL HS-DSCH MAC-ehs Queue Id |  |  |  |  | 2 |  |  | RBS-1491 |  |
| - Logical channel identity |  |  |  |  | 7 |  |  | RBS-1492 |  |
| - RAB information for setup |  |  | A30 |  |  | Rel-8 |  | RBS-1493 |  |
| - RAB info |  |  |  |  | (high-speed AM DTCH for PS domain) |  |  | RBS-1494 |  |
| - RAB identity |  |  |  |  | 0000 0101B |  |  | RBS-1495 |  |
| The first/ leftmost bit of the bit string contains the most significant bit of the RAB identity. |  |  |  |  |  |  |  | RBS-1496 |  |
| - CN domain identity |  |  |  |  | PS domain |  |  | RBS-1497 |  |
| - NAS Synchronization Indicator |  |  |  |  | Not Present |  |  | RBS-1498 |  |
| - Re-establishment timer |  |  |  |  | useT315 |  |  | RBS-1499 |  |
| - RB information to setup |  |  |  |  |  |  |  | RBS-1500 |  |
| - RB identity |  |  |  |  | 25 |  |  | RBS-1501 |  |
| - PDCP info |  |  |  |  |  |  |  | RBS-1502 |  |
| - Support for lossless SRNS relocation |  |  |  |  | FALSE |  |  | RBS-1503 |  |
| - Max PDCP SN window size |  |  |  |  | Not present |  |  | RBS-1504 |  |
| - PDCP PDU header |  |  |  |  | Absent |  |  | RBS-1505 |  |
| - Header compression information |  |  |  |  | Not present |  |  | RBS-1506 |  |
| - CHOICE RLC info type |  |  |  |  | RLC info |  |  | RBS-1507 |  |
| - CHOICE Uplink RLC mode |  |  |  |  | AM RLC |  |  | RBS-1508 |  |
| - Transmission RLC discard |  |  |  |  |  |  |  | RBS-1509 |  |
| - CHOICE SDU discard mode |  |  |  |  | No Discard |  |  | RBS-1510 |  |
| - MAX_DAT |  |  |  |  | 15 |  |  | RBS-1511 |  |
| - Transmission window size |  |  |  |  | 128 |  |  | RBS-1512 |  |
| - Timer_RST |  |  |  |  | 500 |  |  | RBS-1513 |  |
| - Max_RST |  |  |  |  | 4 |  |  | RBS-1514 |  |
| - Polling info |  |  |  |  |  |  |  | RBS-1515 |  |
| - Timer_poll_prohibit |  |  |  |  | 100 |  |  | RBS-1516 |  |
| - Timer_poll |  |  |  |  | 100 |  |  | RBS-1517 |  |
| - Poll_PDU |  |  |  |  | Not Present |  |  | RBS-1518 |  |
| - Poll_SDU |  |  |  |  | 1 |  |  | RBS-1519 |  |
| - Last transmission PDU poll |  |  |  |  | TRUE |  |  | RBS-1520 |  |
| - Last retransmission PDU poll |  |  |  |  | TRUE |  |  | RBS-1521 |  |
| - Poll_Windows |  |  |  |  | 99 |  |  | RBS-1522 |  |
| - Timer_poll_periodic |  |  |  |  | Not Present |  |  | RBS-1523 |  |
| - CHOICE Downlink RLC mode |  |  |  |  | AM RLC |  |  | RBS-1524 |  |
| - CHOICE Downlink RLC PDU Size |  |  |  |  | Reference to clause 6 Parameter Set |  |  | RBS-1525 |  |
| - In-sequence delivery |  |  |  |  | TRUE |  |  | RBS-1526 |  |
| - Receiving window size |  |  |  |  | 768 |  |  | RBS-1527 |  |
| - Downlink RLC status info |  |  |  |  |  |  |  | RBS-1528 |  |
| - Timer_status_prohibit |  |  |  |  | 100 |  |  | RBS-1529 |  |
| - Timer_EPC |  |  |  |  | Not Present |  |  | RBS-1530 |  |
| - Missing PDU indicator |  |  |  |  | TRUE |  |  | RBS-1531 |  |
| - Timer_STATUS_periodic |  |  |  |  | Not Present |  |  | RBS-1532 |  |
| - One sided RLC re-establishment |  |  |  |  | FALSE |  |  | RBS-1533 |  |
| - Alternative E-bit interpretation |  |  |  |  | Not present |  |  | RBS-1534 |  |
| - Use special value of HE field |  |  |  |  | TRUE |  |  | RBS-1535 |  |
| - RB mapping info |  |  |  |  |  |  |  | RBS-1536 |  |
| - Information for each multiplexing option |  |  |  |  | 1 RBMuxOption |  |  | RBS-1537 |  |
| - RLC logical channel mapping indicator |  |  |  |  | Not present |  |  | RBS-1538 |  |
| - Number of uplink RLC logical channels |  |  |  |  | 1 |  |  | RBS-1539 |  |
| - Uplink transport channel type |  |  |  |  | E-DCH |  |  | RBS-1540 |  |
| - Logical channel identity |  |  |  |  | 7 |  |  | RBS-1541 |  |
| - E-DCH MAC-d flow identity |  |  |  |  | 2 |  |  | RBS-1542 |  |
| - CHOICE RLC PDU size |  |  |  |  | Flexible size |  |  | RBS-1543 |  |
| - Length indicator size |  |  |  |  | 15 bit |  |  | RBS-1544 |  |
| - Minimum UL RLC PDU size |  |  |  |  | See clause 6.10 |  |  | RBS-1545 |  |
| - Largest UL RLC PDU size |  |  |  |  | See clause 6.10 |  |  | RBS-1546 |  |
| - Include in scheduling info |  |  |  |  | TRUE |  |  | RBS-1547 |  |
| - MAC logical channel priority |  |  |  |  | 8 |  |  | RBS-1548 |  |
| - Downlink RLC logical channel info |  |  |  |  |  |  |  | RBS-1549 |  |
| - Number of downlink RLC logical channels |  |  |  |  | 1 |  |  | RBS-1550 |  |
| - Downlink transport channel type |  |  |  |  | HS-DSCH |  |  | RBS-1551 |  |
| - DL DCH Transport channel identity |  |  |  |  | Not present |  |  | RBS-1552 |  |
| - DL DSCH Transport channel identity |  |  |  |  | Not present |  |  | RBS-1553 |  |
| - CHOICE DL MAC header type |  |  |  |  | MAC-ehs |  |  | RBS-1554 |  |
| - DL HS-DSCH MAC-ehs Queue Id |  |  |  |  | 2 |  |  | RBS-1555 |  |
| - Logical channel identity |  |  |  |  | 7 |  |  | RBS-1556 |  |
| - RAB information for setup | A31, A32, A33, A34 |  |  |  |  | Rel-9 Rel-10 |  | RBS-1557 |  |
| - RAB info |  |  |  |  | (high-speed AM DTCH for PS domain) |  |  | RBS-1558 |  |
| - RAB identity |  |  |  |  | 0000 0101B The first/ leftmost bit of the bit string contains the most significant bit of the RAB identity. |  |  | RBS-1559 |  |
| - CN domain identity |  |  |  |  | PS domain |  |  | RBS-1560 |  |
| - NAS Synchronization Indicator |  |  |  |  | Not Present |  |  | RBS-1561 |  |
| - Re-establishment timer |  |  |  |  | useT315 |  |  | RBS-1562 |  |
| - RB information to setup |  |  |  |  |  |  |  | RBS-1563 |  |
| - RB identity |  |  |  |  | 25 |  |  | RBS-1564 |  |
| - PDCP info |  |  |  |  |  |  |  | RBS-1565 |  |
| - Support for lossless SRNS relocation |  |  |  |  | FALSE |  |  | RBS-1566 |  |
| - Max PDCP SN window size |  |  |  |  | Not present |  |  | RBS-1567 |  |
| - PDCP PDU header |  |  |  |  | Absent |  |  | RBS-1568 |  |
| - Header compression information |  |  |  |  | Not present |  |  | RBS-1569 |  |
| - CHOICE RLC info type |  |  |  |  | RLC info |  |  | RBS-1570 |  |
| - CHOICE Uplink RLC mode |  |  |  |  | AM RLC |  |  | RBS-1571 |  |
| - Transmission RLC discard |  |  |  |  |  |  |  | RBS-1572 |  |
| - CHOICE SDU discard mode |  |  |  |  | No Discard |  |  | RBS-1573 |  |
| - MAX_DAT |  |  |  |  | 15 |  |  | RBS-1574 |  |
| - Transmission window size |  |  |  |  | 2047 |  |  | RBS-1575 |  |
| - Timer_RST |  |  |  |  | 500 |  |  | RBS-1576 |  |
| - Max_RST |  |  |  |  | 4 |  |  | RBS-1577 |  |
| - Polling info |  |  |  |  |  |  |  | RBS-1578 |  |
| - Timer_poll_prohibit |  |  |  |  | 100 |  |  | RBS-1579 |  |
| - Timer_poll |  |  |  |  | 100 |  |  | RBS-1580 |  |
| - Poll_PDU |  |  |  |  | Not Present |  |  | RBS-1581 |  |
| - Poll_SDU |  |  |  |  | 1 |  |  | RBS-1582 |  |
| - Last transmission PDU poll |  |  |  |  | TRUE |  |  | RBS-1583 |  |
| - Last retransmission PDU poll |  |  |  |  | TRUE |  |  | RBS-1584 |  |
| - Poll_Windows |  |  |  |  | 50 |  |  | RBS-1585 |  |
| - Timer_poll_periodic |  |  |  |  | Not Present |  |  | RBS-1586 |  |
| - CHOICE Downlink RLC mode |  |  |  |  | AM RLC |  |  | RBS-1587 |  |
| - CHOICE Downlink RLC PDU Size |  |  |  |  | Reference to clause 6 Parameter Set |  |  | RBS-1588 |  |
| - Length indicator size |  |  |  |  | This IE is present and set to “7” if Downlink RLC PDU Size is set to “Flexible” |  |  | RBS-1589 |  |
| - In-sequence delivery |  |  |  |  | TRUE |  |  | RBS-1590 |  |
| - Receiving window size |  |  |  |  | 2047 |  |  | RBS-1591 |  |
| - Downlink RLC status info |  |  |  |  |  |  |  | RBS-1592 |  |
| - Timer_status_prohibit |  |  |  |  | 80 |  |  | RBS-1593 |  |
| - Timer_EPC |  |  |  |  | Not Present |  |  | RBS-1594 |  |
| - Missing PDU indicator |  |  |  |  | TRUE |  |  | RBS-1595 |  |
| - Timer_STATUS_periodic |  |  |  |  | Not Present |  |  | RBS-1596 |  |
| - One sided RLC re-establishment |  |  |  |  | FALSE |  |  | RBS-1597 |  |
| - Alternative E-bit interpretation |  |  |  |  | Not present |  |  | RBS-1598 |  |
| - Use special value of HE field |  |  |  |  | TRUE |  |  | RBS-1599 |  |
| - RB mapping info |  |  |  |  |  |  |  | RBS-1600 |  |
| - Information for each multiplexing option |  |  |  |  | 1 RBMuxOption |  |  | RBS-1601 |  |
| - RLC logical channel mapping indicator |  |  |  |  | Not present |  |  | RBS-1602 |  |
| - Number of uplink RLC logical channels |  |  |  |  | 1 |  |  | RBS-1603 |  |
| - Uplink transport channel type |  |  |  |  | E-DCH |  |  | RBS-1604 |  |
| - Logical channel identity |  |  |  |  | 7 |  |  | RBS-1605 |  |
| - E-DCH MAC-d flow identity |  |  |  |  | 2 |  |  | RBS-1606 |  |
| - CHOICE RLC PDU size |  |  |  |  | Flexible size |  |  | RBS-1607 |  |
| - Length indicator size |  |  |  |  | 15 bit |  |  | RBS-1608 |  |
| - Minimum UL RLC PDU size |  |  |  |  | See clause 6.10 |  |  | RBS-1609 |  |
| - Largest UL RLC PDU size |  |  |  |  | See clause 6.10 |  |  | RBS-1610 |  |
| - Include in scheduling info |  |  |  |  | TRUE |  |  | RBS-1611 |  |
| - MAC logical channel priority |  |  |  |  | 8 |  |  | RBS-1612 |  |
| - Downlink RLC logical channel info |  |  |  |  |  |  |  | RBS-1613 |  |
| - Number of downlink RLC logical channels |  |  |  |  | 1 |  |  | RBS-1614 |  |
| - Downlink transport channel type |  |  |  |  | HS-DSCH |  |  | RBS-1615 |  |
| - DL DCH Transport channel identity |  |  |  |  | Not present |  |  | RBS-1616 |  |
| - DL DSCH Transport channel identity |  |  |  |  | Not present |  |  | RBS-1617 |  |
| - CHOICE DL MAC header type |  |  |  |  | MAC-ehs |  |  | RBS-1618 |  |
| - DL HS-DSCH MAC-ehs Queue Id |  |  |  |  | 0 |  |  | RBS-1619 |  |
| - Logical channel identity |  |  |  |  | 7 |  |  | RBS-1620 |  |
| RB information to reconfigure list |  |  | A1, A2, A3, A4, A5, A6, A7, A8 |  | Not Present |  |  | RBS-1621 |  |
|  |  |  | A9, A10 |  |  | Rel-5 |  | RBS-1622 |  |
|  |  |  | A12, A13, A14, A15, A16 |  |  | Rel-6 |  | RBS-1623 |  |
|  |  |  | , A17, A17a, A18, A19, A19a, A20, A21, A22 |  |  | Rel-7 |  | RBS-1624 |  |
|  |  |  | , A23, A28a |  |  | Rel-7 Rel-8 |  | RBS-1625 |  |
|  |  |  | , A25, A25a, A25b, A26, A27, A27a, A28, A29 |  |  | Rel-8 |  | RBS-1626 |  |
|  |  |  | ,A25c, A31, A32, A33, A34 |  |  | Rel-9 Rel-10 |  | RBS-1627 |  |
| RB information to be affected |  |  | A1, A2, A3, A4, A5, A6, A7, A8, A11 |  | Not Present |  |  | RBS-1628 |  |
|  |  |  | , A9, A10 |  |  | Rel-5 |  | RBS-1629 |  |
|  |  |  | A12 |  |  | Rel-6 |  | RBS-1630 |  |
|  |  |  | , A17, A17a, A18, A19, A20, A21, A24 |  |  | Rel-7 |  | RBS-1631 |  |
|  |  |  | , A23, A28a |  |  | Rel-7 Rel-8 |  | RBS-1632 |  |
|  |  |  | , A25a, A28, A29 |  |  | Rel-8 |  | RBS-1633 |  |
| RB information to be affected |  |  | A13, A15 A19a, A25b |  |  | Rel-6 |  | RBS-1634 |  |
|  |  |  | , A17d, A17e |  |  | Rel-7 |  | RBS-1635 |  |
|  |  |  | , A26 |  |  | Rel-8 |  | RBS-1636 |  |
|  |  |  | ,A25c, A31, A32, A33, A34 |  |  | Rel-9 Rel-10 |  | RBS-1637 |  |
| - RB identity |  |  |  |  | 1 (UM DCCH for RRC) |  |  | RBS-1638 |  |
| - RB mapping info |  |  |  |  |  |  |  | RBS-1639 |  |
| - Information for each multiplexing option |  |  |  |  | 1 RBMuxOption |  |  | RBS-1640 |  |
| - RLC logical channel mapping indicator |  |  |  |  | Not Present |  |  | RBS-1641 |  |
| - Number of uplink RLC logical channels |  |  |  |  | 1 |  |  | RBS-1642 |  |
| - Uplink transport channel type |  |  |  |  | E-DCH |  |  | RBS-1643 |  |
| - Logical channel identity |  |  |  |  | 1 |  |  | RBS-1644 |  |
| - E-DCH MAC-d flow identity |  |  |  |  | 1 |  |  | RBS-1645 |  |
| - CHOICE RLC PDU size |  |  |  |  | Fixed size | Rel-8 |  | RBS-1646 |  |
| - DDI |  |  |  |  | 1 |  |  | RBS-1647 |  |
| - RLC PDU size list |  |  |  |  | 1 RLC PDU size |  |  | RBS-1648 |  |
| - RLC PDU size |  |  |  |  | 144 bits |  |  | RBS-1649 |  |
| - Include in scheduling info |  |  |  |  | FALSE |  |  | RBS-1650 |  |
| - MAC logical channel priority |  |  |  |  | 1 |  |  | RBS-1651 |  |
| - Downlink RLC logical channel info |  |  |  |  |  |  |  | RBS-1652 |  |
| - Number of RLC logical channels |  |  |  |  | 1 |  |  | RBS-1653 |  |
| - Downlink transport channel type |  |  |  |  | DCH |  |  | RBS-1654 |  |
| - DL DCH Transport channel identity |  |  |  |  | 10 |  |  | RBS-1655 |  |
| - DL DSCH Transport channel identity |  |  |  |  | Not Present |  |  | RBS-1656 |  |
| - Logical channel identity |  |  |  |  | 1 |  |  | RBS-1657 |  |
| - RB identity |  |  |  |  | 2 (AM DCCH for RRC) |  |  | RBS-1658 |  |
| - RB mapping info |  |  |  |  |  |  |  | RBS-1659 |  |
| - Information for each multiplexing option |  |  |  |  | 1 RBMuxOption |  |  | RBS-1660 |  |
| - RLC logical channel mapping indicator |  |  |  |  | Not Present |  |  | RBS-1661 |  |
| - Number of uplink RLC logical channels |  |  |  |  | 1 |  |  | RBS-1662 |  |
| - Uplink transport channel type |  |  |  |  | E-DCH |  |  | RBS-1663 |  |
| - Logical channel identity |  |  |  |  | 2 |  |  | RBS-1664 |  |
| - E-DCH MAC-d flow identity |  |  |  |  | 1 |  |  | RBS-1665 |  |
| - CHOICE RLC PDU size |  |  |  |  | Fixed size | Rel-8 |  | RBS-1666 |  |
| - DDI |  |  |  |  | 2 |  |  | RBS-1667 |  |
| - RLC PDU size list |  |  |  |  | 1 RLC PDU size |  |  | RBS-1668 |  |
| - RLC PDU size |  |  |  |  | 144 bits |  |  | RBS-1669 |  |
| - Include in scheduling info |  |  |  |  | FALSE |  |  | RBS-1670 |  |
| - MAC logical channel priority |  |  |  |  | 2 |  |  | RBS-1671 |  |
| - Downlink RLC logical channel info |  |  |  |  |  |  |  | RBS-1672 |  |
| - Number of RLC logical channels |  |  |  |  | 1 |  |  | RBS-1673 |  |
| - Downlink transport channel type |  |  |  |  | DCH |  |  | RBS-1674 |  |
| - DL DCH Transport channel identity |  |  |  |  | 10 |  |  | RBS-1675 |  |
| - DL DSCH Transport channel identity |  |  |  |  | Not Present |  |  | RBS-1676 |  |
| - Logical channel identity |  |  |  |  | 2 |  |  | RBS-1677 |  |
| - RB identity |  |  |  |  | 3 (AM DCCH for NAS High Priority) |  |  | RBS-1678 |  |
| - RB mapping info |  |  |  |  |  |  |  | RBS-1679 |  |
| - Information for each multiplexing option |  |  |  |  | 1 RBMuxOption |  |  | RBS-1680 |  |
| - RLC logical channel mapping indicator |  |  |  |  | Not Present |  |  | RBS-1681 |  |
| - Number of uplink RLC logical channels |  |  |  |  | 1 |  |  | RBS-1682 |  |
| - Uplink transport channel type |  |  |  |  | E-DCH |  |  | RBS-1683 |  |
| - Logical channel identity |  |  |  |  | 3 |  |  | RBS-1684 |  |
| - E-DCH MAC-d flow identity |  |  |  |  | 1 |  |  | RBS-1685 |  |
| - CHOICE RLC PDU size |  |  |  |  | Fixed size | Rel-8 |  | RBS-1686 |  |
| - DDI |  |  |  |  | 3 |  |  | RBS-1687 |  |
| - RLC PDU size list |  |  |  |  | 1 RLC PDU size |  |  | RBS-1688 |  |
| - RLC PDU size |  |  |  |  | 144 bits |  |  | RBS-1689 |  |
| - Include in scheduling info |  |  |  |  | FALSE |  |  | RBS-1690 |  |
| - MAC logical channel priority |  |  |  |  | 3 |  |  | RBS-1691 |  |
| - Downlink RLC logical channel info |  |  |  |  |  |  |  | RBS-1692 |  |
| - Number of RLC logical channels |  |  |  |  | 1 |  |  | RBS-1693 |  |
| - Downlink transport channel type |  |  |  |  | DCH |  |  | RBS-1694 |  |
| - DL DCH Transport channel identity |  |  |  |  | 10 |  |  | RBS-1695 |  |
| - DL DSCH Transport channel identity |  |  |  |  | Not Present |  |  | RBS-1696 |  |
| - Logical channel identity |  |  |  |  | 3 |  |  | RBS-1697 |  |
| - RB identity |  |  |  |  | 4 (AM DCCH for NAS Low Priority) |  |  | RBS-1698 |  |
| - RB mapping info |  |  |  |  |  |  |  | RBS-1699 |  |
| - Information for each multiplexing option |  |  |  |  | 1 RBMuxOption |  |  | RBS-1700 |  |
| - RLC logical channel mapping indicator |  |  |  |  | Not Present |  |  | RBS-1701 |  |
| - Number of uplink RLC logical channels |  |  |  |  | 1 |  |  | RBS-1702 |  |
| - Uplink transport channel type |  |  |  |  | E-DCH |  |  | RBS-1703 |  |
| - Logical channel identity |  |  |  |  | 4 |  |  | RBS-1704 |  |
| - E-DCH MAC-d flow identity |  |  |  |  | 1 |  |  | RBS-1705 |  |
| - CHOICE RLC PDU size |  |  |  |  | Fixed size | Rel-8 |  | RBS-1706 |  |
| - DDI |  |  |  |  | 4 |  |  | RBS-1707 |  |
| - RLC PDU size list |  |  |  |  | 1 RLC PDU size |  |  | RBS-1708 |  |
| - RLC PDU size |  |  |  |  | 144 bits |  |  | RBS-1709 |  |
| - Include in scheduling info |  |  |  |  | FALSE |  |  | RBS-1710 |  |
| - MAC logical channel priority |  |  |  |  | 4 |  |  | RBS-1711 |  |
| - Downlink RLC logical channel info |  |  |  |  |  |  |  | RBS-1712 |  |
| - Number of RLC logical channels |  |  |  |  | 1 |  |  | RBS-1713 |  |
| - Downlink transport channel type |  |  |  |  | DCH |  |  | RBS-1714 |  |
| - DL DCH Transport channel identity |  |  |  |  | 10 |  |  | RBS-1715 |  |
| - DL DSCH Transport channel identity |  |  |  |  | Not Present |  |  | RBS-1716 |  |
| - Logical channel identity |  |  |  |  | 4 |  |  | RBS-1717 |  |
| RB information to be affected |  |  | A14, A16 |  |  | Rel-6 |  | RBS-1718 |  |
| - RB identity |  |  |  |  | 1 (UM DCCH for RRC) |  |  | RBS-1719 |  |
| - RB mapping info |  |  |  |  |  |  |  | RBS-1720 |  |
| - Information for each multiplexing option |  |  |  |  | 1 RBMuxOption |  |  | RBS-1721 |  |
| - RLC logical channel mapping indicator |  |  |  |  | Not Present |  |  | RBS-1722 |  |
| - Number of uplink RLC logical channels |  |  |  |  | 1 |  |  | RBS-1723 |  |
| - Uplink transport channel type |  |  |  |  | E-DCH |  |  | RBS-1724 |  |
| - Logical channel identity |  |  |  |  | 1 |  |  | RBS-1725 |  |
| - E-DCH MAC-d flow identity |  |  |  |  | 1 |  |  | RBS-1726 |  |
| - CHOICE RLC PDU size |  |  |  |  | Fixed size | Rel-8 |  | RBS-1727 |  |
| - DDI |  |  |  |  | 1 |  |  | RBS-1728 |  |
| - RLC PDU size list |  |  |  |  | 1 RLC PDU size |  |  | RBS-1729 |  |
| - RLC PDU size |  |  |  |  | 144 bits |  |  | RBS-1730 |  |
| - Include in scheduling info |  |  |  |  | FALSE |  |  | RBS-1731 |  |
| - MAC logical channel priority |  |  |  |  | 1 |  |  | RBS-1732 |  |
| - Downlink RLC logical channel info |  |  |  |  |  |  |  | RBS-1733 |  |
| - Number of RLC logical channels |  |  |  |  | 1 |  |  | RBS-1734 |  |
| - Downlink transport channel type |  |  |  |  | HS-DSCH |  |  | RBS-1735 |  |
| - DL DCH Transport channel identity |  |  |  |  | Not present |  |  | RBS-1736 |  |
| - DL DSCH Transport channel identity |  |  |  |  | Not present |  |  | RBS-1737 |  |
| - DL HS-DSCH MAC-d flow identity |  |  |  |  | 1 |  |  | RBS-1738 |  |
| - Logical channel identity |  |  |  |  | 1 |  |  | RBS-1739 |  |
| - RB identity |  |  |  |  | 2 (AM DCCH for RRC) |  |  | RBS-1740 |  |
| - RB mapping info |  |  |  |  |  |  |  | RBS-1741 |  |
| - Information for each multiplexing option |  |  |  |  | 1 RBMuxOption |  |  | RBS-1742 |  |
| - RLC logical channel mapping indicator |  |  |  |  | Not Present |  |  | RBS-1743 |  |
| - Number of uplink RLC logical channels |  |  |  |  | 1 |  |  | RBS-1744 |  |
| - Uplink transport channel type |  |  |  |  | E-DCH |  |  | RBS-1745 |  |
| - Logical channel identity |  |  |  |  | 2 |  |  | RBS-1746 |  |
| - E-DCH MAC-d flow identity |  |  |  |  | 1 |  |  | RBS-1747 |  |
| - CHOICE RLC PDU size |  |  |  |  | Fixed size | Rel-8 |  | RBS-1748 |  |
| - DDI |  |  |  |  | 2 |  |  | RBS-1749 |  |
| - RLC PDU size list |  |  |  |  | 1 RLC PDU size |  |  | RBS-1750 |  |
| - RLC PDU size |  |  |  |  | 144 bits |  |  | RBS-1751 |  |
| - Include in scheduling info |  |  |  |  | FALSE |  |  | RBS-1752 |  |
| - MAC logical channel priority |  |  |  |  | 2 |  |  | RBS-1753 |  |
| - Downlink RLC logical channel info |  |  |  |  |  |  |  | RBS-1754 |  |
| - Number of RLC logical channels |  |  |  |  | 1 |  |  | RBS-1755 |  |
| - Downlink transport channel type |  |  |  |  | HS-DSCH |  |  | RBS-1756 |  |
| - DL DCH Transport channel identity |  |  |  |  | Not Present |  |  | RBS-1757 |  |
| - DL DSCH Transport channel identity |  |  |  |  | Not Present |  |  | RBS-1758 |  |
| - DL HS-DSCH MAC-d flow identity |  |  |  |  | 1 |  |  | RBS-1759 |  |
| - Logical channel identity |  |  |  |  | 2 |  |  | RBS-1760 |  |
| - RB identity |  |  |  |  | 3 (AM DCCH for NAS High Priority) |  |  | RBS-1761 |  |
| - RB mapping info |  |  |  |  |  |  |  | RBS-1762 |  |
| - Information for each multiplexing option |  |  |  |  | 1 RBMuxOption |  |  | RBS-1763 |  |
| - RLC logical channel mapping indicator |  |  |  |  | Not Present |  |  | RBS-1764 |  |
| - Number of uplink RLC logical channels |  |  |  |  | 1 |  |  | RBS-1765 |  |
| - Uplink transport channel type |  |  |  |  | E-DCH |  |  | RBS-1766 |  |
| - Logical channel identity |  |  |  |  | 3 |  |  | RBS-1767 |  |
| - E-DCH MAC-d flow identity |  |  |  |  | 1 |  |  | RBS-1768 |  |
| - CHOICE RLC PDU size |  |  |  |  | Fixed size | Rel-8 |  | RBS-1769 |  |
| - DDI |  |  |  |  | 3 |  |  | RBS-1770 |  |
| - RLC PDU size list |  |  |  |  | 1 RLC PDU size |  |  | RBS-1771 |  |
| - RLC PDU size |  |  |  |  | 144 bits |  |  | RBS-1772 |  |
| - Include in scheduling info |  |  |  |  | FALSE |  |  | RBS-1773 |  |
| - MAC logical channel priority |  |  |  |  | 3 |  |  | RBS-1774 |  |
| - Downlink RLC logical channel info |  |  |  |  |  |  |  | RBS-1775 |  |
| - Number of RLC logical channels |  |  |  |  | 1 |  |  | RBS-1776 |  |
| - Downlink transport channel type |  |  |  |  | HS-DSCH |  |  | RBS-1777 |  |
| - DL DCH Transport channel identity |  |  |  |  | Not Present |  |  | RBS-1778 |  |
| - DL DSCH Transport channel identity |  |  |  |  | Not Present |  |  | RBS-1779 |  |
| - DL HS-DSCH MAC-d flow identity |  |  |  |  | 1 |  |  | RBS-1780 |  |
| - Logical channel identity |  |  |  |  | 3 |  |  | RBS-1781 |  |
| - RB identity |  |  |  |  | 4 (AM DCCH for NAS Low Priority) |  |  | RBS-1782 |  |
| - RB mapping info |  |  |  |  |  |  |  | RBS-1783 |  |
| - Information for each multiplexing option |  |  |  |  | 1 RBMuxOption |  |  | RBS-1784 |  |
| - RLC logical channel mapping indicator |  |  |  |  | Not Present |  |  | RBS-1785 |  |
| - Number of uplink RLC logical channels |  |  |  |  | 1 |  |  | RBS-1786 |  |
| - Uplink transport channel type |  |  |  |  | E-DCH |  |  | RBS-1787 |  |
| - Logical channel identity |  |  |  |  | 4 |  |  | RBS-1788 |  |
| - E-DCH MAC-d flow identity |  |  |  |  | 1 |  |  | RBS-1789 |  |
| - CHOICE RLC PDU size |  |  |  |  | Fixed size | Rel-8 |  | RBS-1790 |  |
| - DDI |  |  |  |  | 4 |  |  | RBS-1791 |  |
| - RLC PDU size list |  |  |  |  | 1 RLC PDU size |  |  | RBS-1792 |  |
| - RLC PDU size |  |  |  |  | 144 bits |  |  | RBS-1793 |  |
| - Include in scheduling info |  |  |  |  | FALSE |  |  | RBS-1794 |  |
| - MAC logical channel priority |  |  |  |  | 4 |  |  | RBS-1795 |  |
| - Downlink RLC logical channel info |  |  |  |  |  |  |  | RBS-1796 |  |
| - Number of RLC logical channels |  |  |  |  | 1 |  |  | RBS-1797 |  |
| - Downlink transport channel type |  |  |  |  | HS-DSCH |  |  | RBS-1798 |  |
| - DL DCH Transport channel identity |  |  |  |  | Not Present |  |  | RBS-1799 |  |
| - DL DSCH Transport channel identity |  |  |  |  | Not Present |  |  | RBS-1800 |  |
| - DL HS-DSCH MAC-d flow identity |  |  |  |  | 1 |  |  | RBS-1801 |  |
| - Logical channel identity |  |  |  |  | 4 |  |  | RBS-1802 |  |
| RB information to be affected |  |  | A17b, A17c, A22 |  |  | Rel-7 |  | RBS-1803 |  |
|  |  |  | A25 |  |  | Rel-8 |  | RBS-1804 |  |
| - RB identity |  |  |  |  | 1 (UM DCCH for RRC) |  |  | RBS-1805 |  |
| - RB mapping info |  |  |  |  |  |  |  | RBS-1806 |  |
| - Information for each multiplexing option |  |  |  |  | 1 RBMuxOption |  |  | RBS-1807 |  |
| - RLC logical channel mapping indicator |  |  |  |  | Not Present |  |  | RBS-1808 |  |
| - Number of uplink RLC logical channels |  |  |  |  | 1 |  |  | RBS-1809 |  |
| - Uplink transport channel type |  |  |  |  | E-DCH |  |  | RBS-1810 |  |
| - Logical channel identity |  |  |  |  | 1 |  |  | RBS-1811 |  |
| - E-DCH MAC-d flow identity |  |  |  |  | 1 |  |  | RBS-1812 |  |
| - CHOICE RLC PDU size |  |  |  |  | Fixed size | Rel-8 |  | RBS-1813 |  |
| - DDI |  |  |  |  | 1 |  |  | RBS-1814 |  |
| - RLC PDU size list |  |  |  |  | 1 RLC PDU size |  |  | RBS-1815 |  |
| - RLC PDU size |  |  |  |  | 144 bits |  |  | RBS-1816 |  |
| - Include in scheduling info |  |  |  |  | FALSE |  |  | RBS-1817 |  |
| - MAC logical channel priority |  |  |  |  | 1 |  |  | RBS-1818 |  |
| - Downlink RLC logical channel info |  |  |  |  |  |  |  | RBS-1819 |  |
| - Number of RLC logical channels |  |  |  |  | 1 |  |  | RBS-1820 |  |
| - Downlink transport channel type |  |  |  |  | HS-DSCH |  |  | RBS-1821 |  |
| - DL DCH Transport channel identity |  |  |  |  | Not present |  |  | RBS-1822 |  |
| - DL DSCH Transport channel identity |  |  |  |  | Not present |  |  | RBS-1823 |  |
| - CHOICE DL MAC header type |  |  |  |  | MAC-ehs |  |  | RBS-1824 |  |
| - DL HS-DSCH MAC-ehs Queue Id |  |  |  |  | 1 |  |  | RBS-1825 |  |
| - Logical channel identity |  |  |  |  | 1 |  |  | RBS-1826 |  |
| - RB identity |  |  |  |  | 2 (AM DCCH for RRC) |  |  | RBS-1827 |  |
| - RB mapping info |  |  |  |  |  |  |  | RBS-1828 |  |
| - Information for each multiplexing option |  |  |  |  | 1 RBMuxOption |  |  | RBS-1829 |  |
| - RLC logical channel mapping indicator |  |  |  |  | Not Present |  |  | RBS-1830 |  |
| - Number of uplink RLC logical channels |  |  |  |  | 1 |  |  | RBS-1831 |  |
| - Uplink transport channel type |  |  |  |  | E-DCH |  |  | RBS-1832 |  |
| - Logical channel identity |  |  |  |  | 2 |  |  | RBS-1833 |  |
| - E-DCH MAC-d flow identity |  |  |  |  | 1 |  |  | RBS-1834 |  |
| - CHOICE RLC PDU size |  |  |  |  | Fixed size | Rel-8 |  | RBS-1835 |  |
| - DDI |  |  |  |  | 2 |  |  | RBS-1836 |  |
| - RLC PDU size list |  |  |  |  | 1 RLC PDU size |  |  | RBS-1837 |  |
| - RLC PDU size |  |  |  |  | 144 bits |  |  | RBS-1838 |  |
| - Include in scheduling info |  |  |  |  | FALSE |  |  | RBS-1839 |  |
| - MAC logical channel priority |  |  |  |  | 2 |  |  | RBS-1840 |  |
| - Downlink RLC logical channel info |  |  |  |  |  |  |  | RBS-1841 |  |
| - Number of RLC logical channels |  |  |  |  | 1 |  |  | RBS-1842 |  |
| - Downlink transport channel type |  |  |  |  | HS-DSCH |  |  | RBS-1843 |  |
| - DL DCH Transport channel identity |  |  |  |  | Not Present |  |  | RBS-1844 |  |
| - DL DSCH Transport channel identity |  |  |  |  | Not Present |  |  | RBS-1845 |  |
| - CHOICE DL MAC header type |  |  |  |  | MAC-ehs |  |  | RBS-1846 |  |
| - DL HS-DSCH MAC-ehs Queue Id |  |  |  |  | 1 |  |  | RBS-1847 |  |
| - Logical channel identity |  |  |  |  | 2 |  |  | RBS-1848 |  |
| - RB identity |  |  |  |  | 3 (AM DCCH for NAS High Priority) |  |  | RBS-1849 |  |
| - RB mapping info |  |  |  |  |  |  |  | RBS-1850 |  |
| - Information for each multiplexing option |  |  |  |  | 1 RBMuxOption |  |  | RBS-1851 |  |
| - RLC logical channel mapping indicator |  |  |  |  | Not Present |  |  | RBS-1852 |  |
| - Number of uplink RLC logical channels |  |  |  |  | 1 |  |  | RBS-1853 |  |
| - Uplink transport channel type |  |  |  |  | E-DCH |  |  | RBS-1854 |  |
| - Logical channel identity |  |  |  |  | 3 |  |  | RBS-1855 |  |
| - E-DCH MAC-d flow identity |  |  |  |  | 1 |  |  | RBS-1856 |  |
| - CHOICE RLC PDU size |  |  |  |  | Fixed size | Rel-8 |  | RBS-1857 |  |
| - DDI |  |  |  |  | 3 |  |  | RBS-1858 |  |
| - RLC PDU size list |  |  |  |  | 1 RLC PDU size |  |  | RBS-1859 |  |
| - RLC PDU size |  |  |  |  | 144 bits |  |  | RBS-1860 |  |
| - Include in scheduling info |  |  |  |  | FALSE |  |  | RBS-1861 |  |
| - MAC logical channel priority |  |  |  |  | 3 |  |  | RBS-1862 |  |
| - Downlink RLC logical channel info |  |  |  |  |  |  |  | RBS-1863 |  |
| - Number of RLC logical channels |  |  |  |  | 1 |  |  | RBS-1864 |  |
| - Downlink transport channel type |  |  |  |  | HS-DSCH |  |  | RBS-1865 |  |
| - DL DCH Transport channel identity |  |  |  |  | Not Present |  |  | RBS-1866 |  |
| - DL DSCH Transport channel identity |  |  |  |  | Not Present |  |  | RBS-1867 |  |
| - CHOICE DL MAC header type |  |  |  |  | MAC-ehs |  |  | RBS-1868 |  |
| - DL HS-DSCH MAC-ehs Queue Id |  |  |  |  | 1 |  |  | RBS-1869 |  |
| - Logical channel identity |  |  |  |  | 3 |  |  | RBS-1870 |  |
| - RB identity |  |  |  |  | 4 (AM DCCH for NAS Low Priority) |  |  | RBS-1871 |  |
| - RB mapping info |  |  |  |  |  |  |  | RBS-1872 |  |
| - Information for each multiplexing option |  |  |  |  | 1 RBMuxOption |  |  | RBS-1873 |  |
| - RLC logical channel mapping indicator |  |  |  |  | Not Present |  |  | RBS-1874 |  |
| - Number of uplink RLC logical channels |  |  |  |  | 1 |  |  | RBS-1875 |  |
| - Uplink transport channel type |  |  |  |  | E-DCH |  |  | RBS-1876 |  |
| - Logical channel identity |  |  |  |  | 4 |  |  | RBS-1877 |  |
| - E-DCH MAC-d flow identity |  |  |  |  | 1 |  |  | RBS-1878 |  |
| - CHOICE RLC PDU size |  |  |  |  | Fixed size | Rel-8 |  | RBS-1879 |  |
| - DDI |  |  |  |  | 4 |  |  | RBS-1880 |  |
| - RLC PDU size list |  |  |  |  | 1 RLC PDU size |  |  | RBS-1881 |  |
| - RLC PDU size |  |  |  |  | 144 bits |  |  | RBS-1882 |  |
| - Include in scheduling info |  |  |  |  | FALSE |  |  | RBS-1883 |  |
| - MAC logical channel priority |  |  |  |  | 4 |  |  | RBS-1884 |  |
| - Downlink RLC logical channel info |  |  |  |  |  |  |  | RBS-1885 |  |
| - Number of RLC logical channels |  |  |  |  | 1 |  |  | RBS-1886 |  |
| - Downlink transport channel type |  |  |  |  | HS-DSCH |  |  | RBS-1887 |  |
| - DL DCH Transport channel identity |  |  |  |  | Not Present |  |  | RBS-1888 |  |
| - DL DSCH Transport channel identity |  |  |  |  | Not Present |  |  | RBS-1889 |  |
| - CHOICE DL MAC header type |  |  |  |  | MAC-ehs |  |  | RBS-1890 |  |
| - DL HS-DSCH MAC-ehs Queue Id |  |  |  |  | 1 |  |  | RBS-1891 |  |
| - Logical channel identity |  |  |  |  | 4 |  |  | RBS-1892 |  |
| RB information to be affected |  |  | A27, A27a |  |  | Rel-8 |  | RBS-1893 |  |
| - RB identity |  |  |  |  | 1 (UM DCCH for RRC) |  |  | RBS-1894 |  |
| - RB mapping info |  |  |  |  |  |  |  | RBS-1895 |  |
| - Information for each multiplexing option |  |  |  |  | 1 RBMuxOption |  |  | RBS-1896 |  |
| - RLC logical channel mapping indicator |  |  |  |  | Not Present |  |  | RBS-1897 |  |
| - Number of uplink RLC logical channels |  |  |  |  | 1 |  |  | RBS-1898 |  |
| - Uplink transport channel type |  |  |  |  | E-DCH |  |  | RBS-1899 |  |
| - Logical channel identity |  |  |  |  | 1 |  |  | RBS-1900 |  |
| - E-DCH MAC-d flow identity |  |  |  |  | 1 |  |  | RBS-1901 |  |
| - CHOICE RLC PDU size |  |  |  |  | Fixed size |  |  | RBS-1902 |  |
| - DDI |  |  |  |  | Not Present |  |  | RBS-1903 |  |
| - RLC PDU size list |  |  |  |  | 1 RLC PDU size |  |  | RBS-1904 |  |
| - RLC PDU size |  |  |  |  | 144 bits |  |  | RBS-1905 |  |
| - Include in scheduling info |  |  |  |  | FALSE |  |  | RBS-1906 |  |
| - MAC logical channel priority |  |  |  |  | 1 |  |  | RBS-1907 |  |
| - Downlink RLC logical channel info |  |  |  |  |  |  |  | RBS-1908 |  |
| - Number of RLC logical channels |  |  |  |  | 1 |  |  | RBS-1909 |  |
| - Downlink transport channel type |  |  |  |  | HS-DSCH |  |  | RBS-1910 |  |
| - DL DCH Transport channel identity |  |  |  |  | Not present |  |  | RBS-1911 |  |
| - DL DSCH Transport channel identity |  |  |  |  | Not present |  |  | RBS-1912 |  |
| - CHOICE DL MAC header type |  |  |  |  | MAC-ehs |  |  | RBS-1913 |  |
| - DL HS-DSCH MAC-ehs Queue Id |  |  |  |  | 1 |  |  | RBS-1914 |  |
| - Logical channel identity |  |  |  |  | 1 |  |  | RBS-1915 |  |
| - RB identity |  |  |  |  | 2 (AM DCCH for RRC) |  |  | RBS-1916 |  |
| - RB mapping info |  |  |  |  |  |  |  | RBS-1917 |  |
| - Information for each multiplexing option |  |  |  |  | 1 RBMuxOption |  |  | RBS-1918 |  |
| - RLC logical channel mapping indicator |  |  |  |  | Not Present |  |  | RBS-1919 |  |
| - Number of uplink RLC logical channels |  |  |  |  | 1 |  |  | RBS-1920 |  |
| - Uplink transport channel type |  |  |  |  | E-DCH |  |  | RBS-1921 |  |
| - Logical channel identity |  |  |  |  | 2 |  |  | RBS-1922 |  |
| - E-DCH MAC-d flow identity |  |  |  |  | 1 |  |  | RBS-1923 |  |
| - CHOICE RLC PDU size |  |  |  |  | Fixed size |  |  | RBS-1924 |  |
| - DDI |  |  |  |  | Not Present |  |  | RBS-1925 |  |
| - RLC PDU size list |  |  |  |  | 1 RLC PDU size |  |  | RBS-1926 |  |
| - RLC PDU size |  |  |  |  | 144 bits |  |  | RBS-1927 |  |
| - Include in scheduling info |  |  |  |  | FALSE |  |  | RBS-1928 |  |
| - MAC logical channel priority |  |  |  |  | 2 |  |  | RBS-1929 |  |
| - Downlink RLC logical channel info |  |  |  |  |  |  |  | RBS-1930 |  |
| - Number of RLC logical channels |  |  |  |  | 1 |  |  | RBS-1931 |  |
| - Downlink transport channel type |  |  |  |  | HS-DSCH |  |  | RBS-1932 |  |
| - DL DCH Transport channel identity |  |  |  |  | Not Present |  |  | RBS-1933 |  |
| - DL DSCH Transport channel identity |  |  |  |  | Not Present |  |  | RBS-1934 |  |
| - CHOICE DL MAC header type |  |  |  |  | MAC-ehs |  |  | RBS-1935 |  |
| - DL HS-DSCH MAC-ehs Queue Id |  |  |  |  | 1 |  |  | RBS-1936 |  |
| - Logical channel identity |  |  |  |  | 2 |  |  | RBS-1937 |  |
| - RB identity |  |  |  |  | 3 (AM DCCH for NAS High Priority) |  |  | RBS-1938 |  |
| - RB mapping info |  |  |  |  |  |  |  | RBS-1939 |  |
| - Information for each multiplexing option |  |  |  |  | 1 RBMuxOption |  |  | RBS-1940 |  |
| - RLC logical channel mapping indicator |  |  |  |  | Not Present |  |  | RBS-1941 |  |
| - Number of uplink RLC logical channels |  |  |  |  | 1 |  |  | RBS-1942 |  |
| - Uplink transport channel type |  |  |  |  | E-DCH |  |  | RBS-1943 |  |
| - Logical channel identity |  |  |  |  | 3 |  |  | RBS-1944 |  |
| - E-DCH MAC-d flow identity |  |  |  |  | 1 |  |  | RBS-1945 |  |
| - CHOICE RLC PDU size |  |  |  |  | Fixed size |  |  | RBS-1946 |  |
| - DDI |  |  |  |  | Not Present |  |  | RBS-1947 |  |
| - RLC PDU size list |  |  |  |  | 1 RLC PDU size |  |  | RBS-1948 |  |
| - RLC PDU size |  |  |  |  | 144 bits |  |  | RBS-1949 |  |
| - Include in scheduling info |  |  |  |  | FALSE |  |  | RBS-1950 |  |
| - MAC logical channel priority |  |  |  |  | 3 |  |  | RBS-1951 |  |
| - Downlink RLC logical channel info |  |  |  |  |  |  |  | RBS-1952 |  |
| - Number of RLC logical channels |  |  |  |  | 1 |  |  | RBS-1953 |  |
| - Downlink transport channel type |  |  |  |  | HS-DSCH |  |  | RBS-1954 |  |
| - DL DCH Transport channel identity |  |  |  |  | Not Present |  |  | RBS-1955 |  |
| - DL DSCH Transport channel identity |  |  |  |  | Not Present |  |  | RBS-1956 |  |
| - CHOICE DL MAC header type |  |  |  |  | MAC-ehs |  |  | RBS-1957 |  |
| - DL HS-DSCH MAC-ehs Queue Id |  |  |  |  | 1 |  |  | RBS-1958 |  |
| - Logical channel identity |  |  |  |  | 3 |  |  | RBS-1959 |  |
| - RB identity |  |  |  |  | 4 (AM DCCH for NAS Low Priority) |  |  | RBS-1960 |  |
| - RB mapping info |  |  |  |  |  |  |  | RBS-1961 |  |
| - Information for each multiplexing option |  |  |  |  | 1 RBMuxOption |  |  | RBS-1962 |  |
| - RLC logical channel mapping indicator |  |  |  |  | Not Present |  |  | RBS-1963 |  |
| - Number of uplink RLC logical channels |  |  |  |  | 1 |  |  | RBS-1964 |  |
| - Uplink transport channel type |  |  |  |  | E-DCH |  |  | RBS-1965 |  |
| - Logical channel identity |  |  |  |  | 4 |  |  | RBS-1966 |  |
| - E-DCH MAC-d flow identity |  |  |  |  | 1 |  |  | RBS-1967 |  |
| - CHOICE RLC PDU size |  |  |  |  | Fixed size |  |  | RBS-1968 |  |
| - DDI |  |  |  |  | Not Present |  |  | RBS-1969 |  |
| - RLC PDU size list |  |  |  |  | 1 RLC PDU size |  |  | RBS-1970 |  |
| - RLC PDU size |  |  |  |  | 144 bits |  |  | RBS-1971 |  |
| - Include in scheduling info |  |  |  |  | FALSE |  |  | RBS-1972 |  |
| - MAC logical channel priority |  |  |  |  | 4 |  |  | RBS-1973 |  |
| - Downlink RLC logical channel info |  |  |  |  |  |  |  | RBS-1974 |  |
| - Number of RLC logical channels |  |  |  |  | 1 |  |  | RBS-1975 |  |
| - Downlink transport channel type |  |  |  |  | HS-DSCH |  |  | RBS-1976 |  |
| - DL DCH Transport channel identity |  |  |  |  | Not Present |  |  | RBS-1977 |  |
| - DL DSCH Transport channel identity |  |  |  |  | Not Present |  |  | RBS-1978 |  |
| - CHOICE DL MAC header type |  |  |  |  | MAC-ehs |  |  | RBS-1979 |  |
| - DL HS-DSCH MAC-ehs Queue Id |  |  |  |  | 1 |  |  | RBS-1980 |  |
| - Logical channel identity |  |  |  |  | 4 |  |  | RBS-1981 |  |
| RB information to be affected |  |  | A30 |  |  | Rel-8 |  | RBS-1982 |  |
| - RB identity |  |  |  |  | 1 (UM DCCH for RRC) |  |  | RBS-1983 |  |
| - RB mapping info |  |  |  |  |  |  |  | RBS-1984 |  |
| - Information for each multiplexing option |  |  |  |  | 1 RBMuxOption |  |  | RBS-1985 |  |
| - RLC logical channel mapping indicator |  |  |  |  | Not Present |  |  | RBS-1986 |  |
| - Number of uplink RLC logical channels |  |  |  |  | 1 |  |  | RBS-1987 |  |
| - Uplink transport channel type |  |  |  |  | E-DCH |  |  | RBS-1988 |  |
| - Logical channel identity |  |  |  |  | 1 |  |  | RBS-1989 |  |
| - E-DCH MAC-d flow identity |  |  |  |  | 3 |  |  | RBS-1990 |  |
| - CHOICE RLC PDU size |  |  |  |  | Fixed size |  |  | RBS-1991 |  |
| - DDI |  |  |  |  | 0 (Not applicable for MAC-i/is) |  |  | RBS-1992 |  |
| - RLC PDU size list |  |  |  |  | 1 RLC PDU size |  |  | RBS-1993 |  |
| - RLC PDU size |  |  |  |  | 144 bits |  |  | RBS-1994 |  |
| - Include in scheduling info |  |  |  |  | FALSE |  |  | RBS-1995 |  |
| - MAC logical channel priority |  |  |  |  | 1 |  |  | RBS-1996 |  |
| - Downlink RLC logical channel info |  |  |  |  |  |  |  | RBS-1997 |  |
| - Number of RLC logical channels |  |  |  |  | 1 |  |  | RBS-1998 |  |
| - Downlink transport channel type |  |  |  |  | HS-DSCH |  |  | RBS-1999 |  |
| - DL DCH Transport channel identity |  |  |  |  | Not present |  |  | RBS-2000 |  |
| - DL DSCH Transport channel identity |  |  |  |  | Not present |  |  | RBS-2001 |  |
| - CHOICE DL MAC header type |  |  |  |  | MAC-ehs |  |  | RBS-2002 |  |
| - DL HS-DSCH MAC-ehs Queue Id |  |  |  |  | 3 |  |  | RBS-2003 |  |
| - Logical channel identity |  |  |  |  | 1 |  |  | RBS-2004 |  |
| - RB identity |  |  |  |  | 2 (AM DCCH for RRC) |  |  | RBS-2005 |  |
| - RB mapping info |  |  |  |  |  |  |  | RBS-2006 |  |
| - Information for each multiplexing option |  |  |  |  | 1 RBMuxOption |  |  | RBS-2007 |  |
| - RLC logical channel mapping indicator |  |  |  |  | Not Present |  |  | RBS-2008 |  |
| - Number of uplink RLC logical channels |  |  |  |  | 1 |  |  | RBS-2009 |  |
| - Uplink transport channel type |  |  |  |  | E-DCH |  |  | RBS-2010 |  |
| - Logical channel identity |  |  |  |  | 2 |  |  | RBS-2011 |  |
| - E-DCH MAC-d flow identity |  |  |  |  | 3 |  |  | RBS-2012 |  |
| - CHOICE RLC PDU size |  |  |  |  | Fixed size |  |  | RBS-2013 |  |
| - DDI |  |  |  |  | 0 (Not applicable for MAC-i/is) |  |  | RBS-2014 |  |
| - RLC PDU size list |  |  |  |  | 1 RLC PDU size |  |  | RBS-2015 |  |
| - RLC PDU size |  |  |  |  | 144 bits |  |  | RBS-2016 |  |
| - Include in scheduling info |  |  |  |  | FALSE |  |  | RBS-2017 |  |
| - MAC logical channel priority |  |  |  |  | 2 |  |  | RBS-2018 |  |
| - Downlink RLC logical channel info |  |  |  |  |  |  |  | RBS-2019 |  |
| - Number of RLC logical channels |  |  |  |  | 1 |  |  | RBS-2020 |  |
| - Downlink transport channel type |  |  |  |  | HS-DSCH |  |  | RBS-2021 |  |
| - DL DCH Transport channel identity |  |  |  |  | Not Present |  |  | RBS-2022 |  |
| - DL DSCH Transport channel identity |  |  |  |  | Not Present |  |  | RBS-2023 |  |
| - CHOICE DL MAC header type |  |  |  |  | MAC-ehs |  |  | RBS-2024 |  |
| - DL HS-DSCH MAC-ehs Queue Id |  |  |  |  | 3 |  |  | RBS-2025 |  |
| - Logical channel identity |  |  |  |  | 2 |  |  | RBS-2026 |  |
| - RB identity |  |  |  |  | 3 (AM DCCH for NAS High Priority) |  |  | RBS-2027 |  |
| - RB mapping info |  |  |  |  |  |  |  | RBS-2028 |  |
| - Information for each multiplexing option |  |  |  |  | 1 RBMuxOption |  |  | RBS-2029 |  |
| - RLC logical channel mapping indicator |  |  |  |  | Not Present |  |  | RBS-2030 |  |
| - Number of uplink RLC logical channels |  |  |  |  | 1 |  |  | RBS-2031 |  |
| - Uplink transport channel type |  |  |  |  | E-DCH |  |  | RBS-2032 |  |
| - Logical channel identity |  |  |  |  | 3 |  |  | RBS-2033 |  |
| - E-DCH MAC-d flow identity |  |  |  |  | 3 |  |  | RBS-2034 |  |
| - CHOICE RLC PDU size |  |  |  |  | Fixed size | Rel-8 |  | RBS-2035 |  |
| - DDI |  |  |  |  | 0 (Not applicable for MAC-i/is) |  |  | RBS-2036 |  |
| - RLC PDU size list |  |  |  |  | 1 RLC PDU size |  |  | RBS-2037 |  |
| - RLC PDU size |  |  |  |  | 144 bits |  |  | RBS-2038 |  |
| - Include in scheduling info |  |  |  |  | FALSE |  |  | RBS-2039 |  |
| - MAC logical channel priority |  |  |  |  | 3 |  |  | RBS-2040 |  |
| - Downlink RLC logical channel info |  |  |  |  |  |  |  | RBS-2041 |  |
| - Number of RLC logical channels |  |  |  |  | 1 |  |  | RBS-2042 |  |
| - Downlink transport channel type |  |  |  |  | HS-DSCH |  |  | RBS-2043 |  |
| - DL DCH Transport channel identity |  |  |  |  | Not Present |  |  | RBS-2044 |  |
| - DL DSCH Transport channel identity |  |  |  |  | Not Present |  |  | RBS-2045 |  |
| - CHOICE DL MAC header type |  |  |  |  | MAC-ehs |  |  | RBS-2046 |  |
| - DL HS-DSCH MAC-ehs Queue Id |  |  |  |  | 3 |  |  | RBS-2047 |  |
| - Logical channel identity |  |  |  |  | 3 |  |  | RBS-2048 |  |
| - RB identity |  |  |  |  | 4 (AM DCCH for NAS Low Priority) |  |  | RBS-2049 |  |
| - RB mapping info |  |  |  |  |  |  |  | RBS-2050 |  |
| - Information for each multiplexing option |  |  |  |  | 1 RBMuxOption |  |  | RBS-2051 |  |
| - RLC logical channel mapping indicator |  |  |  |  | Not Present |  |  | RBS-2052 |  |
| - Number of uplink RLC logical channels |  |  |  |  | 1 |  |  | RBS-2053 |  |
| - Uplink transport channel type |  |  |  |  | E-DCH |  |  | RBS-2054 |  |
| - Logical channel identity |  |  |  |  | 4 |  |  | RBS-2055 |  |
| - E-DCH MAC-d flow identity |  |  |  |  | 3 |  |  | RBS-2056 |  |
| - CHOICE RLC PDU size |  |  |  |  | Fixed size | Rel-8 |  | RBS-2057 |  |
| - DDI |  |  |  |  | 4 |  |  | RBS-2058 |  |
| - RLC PDU size list |  |  |  |  | 1 RLC PDU size |  |  | RBS-2059 |  |
| - RLC PDU size |  |  |  |  | 144 bits |  |  | RBS-2060 |  |
| - Include in scheduling info |  |  |  |  | FALSE |  |  | RBS-2061 |  |
| - MAC logical channel priority |  |  |  |  | 4 |  |  | RBS-2062 |  |
| - Downlink RLC logical channel info |  |  |  |  |  |  |  | RBS-2063 |  |
| - Number of RLC logical channels |  |  |  |  | 1 |  |  | RBS-2064 |  |
| - Downlink transport channel type |  |  |  |  | HS-DSCH |  |  | RBS-2065 |  |
| - DL DCH Transport channel identity |  |  |  |  | Not Present |  |  | RBS-2066 |  |
| - DL DSCH Transport channel identity |  |  |  |  | Not Present |  |  | RBS-2067 |  |
| - CHOICE DL MAC header type |  |  |  |  | MAC-ehs |  |  | RBS-2068 |  |
| - DL HS-DSCH MAC-ehs Queue Id |  |  |  |  | 3 |  |  | RBS-2069 |  |
| - Logical channel identity |  |  |  |  | 4 |  |  | RBS-2070 |  |
| Downlink counter synchronization info |  |  | A1, A2, A3, A4, A5, A6, A7, A8, A11 |  | Not Present |  |  | RBS-2071 |  |
|  |  |  | , A9, A10 |  |  | Rel-5 |  | RBS-2072 |  |
|  |  |  | , A12, A13, A14, A15, A16 |  |  | Rel-6 |  | RBS-2073 |  |
|  |  |  | , A17, A17a, A17b, A17c, A17d, A17e, A18, A19, A19a, A20, A21, A22, A24 |  |  | Rel-7 |  | RBS-2074 |  |
|  |  |  | , A23, A28a |  |  | Rel-7 Rel-8 |  | RBS-2075 |  |
|  |  |  | , A25, A25a, A25b, A26, A27, A27a, A28, A29, A30 |  |  | Rel-8 |  | RBS-2076 |  |
|  |  |  | , A31, A32, A33, A34 |  |  | Rel-9 Rel-10 |  | RBS-2077 |  |
|  |  |  | , A25c |  |  | Rel-9 |  | RBS-2077a |  |
| PDCP ROHC target mode |  |  | A9, A10 |  | Not Present | Rel-5 |  | RBS-2078 |  |
|  |  |  | , A12, A13, A14, A15, A16 |  |  | Rel-6 |  | RBS-2079 |  |
|  |  |  | , A17, A17a, A17b, A17c, A17d, A17e, A18, A19, A19a, A20, A21, A22, A24 |  |  | Rel-7 |  | RBS-2080 |  |
|  |  |  | , A23, A28a |  |  | Rel-7 Rel-8 |  | RBS-2081 |  |
|  |  |  | , A25, A25a, A25b, A26, A27, A27a, A28, A29, A30 |  |  | Rel-8 |  | RBS-2082 |  |
|  |  |  | , A31, A32 A33, A34 |  |  | Rel-9 Rel-10 |  | RBS-2083 |  |
|  |  |  | , A25c |  |  | Rel-9 |  | RBS-2083a |  |
| UL Transport channel information for all transport channels |  |  | A1, A2, A3, A4, A5, A6, A7, A8, A11 |  |  |  |  | RBS-2084 |  |
|  |  |  | , A9, A10 |  |  | Rel-5 |  | RBS-2085 |  |
|  |  |  | , A17, A17a, A18, A28a |  |  | Rel-7 |  | RBS-2086 |  |
|  |  |  | , A25a, A28 |  |  | Rel-8 |  | RBS-2087 |  |
|  |  |  | , A31, A32 A33, A34 |  |  | Rel-9 Rel-10 |  | RBS-2088 |  |
| - PRACH TFCS |  |  |  |  | Not Present |  |  | RBS-2089 |  |
| - CHOICE mode |  |  |  |  | FDD |  |  | RBS-2090 |  |
| - TFC subset |  |  |  |  | Not Present |  |  | RBS-2091 |  |
| - UL DCH TFCS |  |  |  |  |  |  |  | RBS-2092 |  |
| - CHOICE TFCI signalling |  |  |  |  | Normal |  |  | RBS-2093 |  |
| - TFCI Field 1 information |  |  |  |  |  |  |  | RBS-2094 |  |
| - CHOICE TFCS representation |  |  |  |  | Complete reconfiguration |  |  | RBS-2095 |  |
| - TFCS complete reconfigure information |  |  |  |  |  |  |  | RBS-2096 |  |
| - CHOICE CTFC Size |  |  |  |  | Number of bits used must be enough to cover all combinations of CTFC from clause 6.10.2.4 Parameter Set. |  |  | RBS-2097 |  |
| - CTFC information |  |  |  |  | This IE is repeated for TFC numbers and reference to clause 6.10.2.4 Parameter Set |  |  | RBS-2098 |  |
| - CTFC |  |  |  |  | Reference to clause 6.10.2.4 Parameter Set |  |  | RBS-2099 |  |
| - Power offset information |  |  |  |  |  |  |  | RBS-2100 |  |
| - CHOICE Gain Factors |  |  |  |  | Computed Gain Factors(The last TFC is set to Signalled Gain Factors) |  |  | RBS-2101 |  |
| - Gain factor βc |  |  |  |  | 11 (below 64 kbps) 9 (equal or higher than 64 kbps) when HSDPA is not configured 9 (equal or higher than 64 kbps and below 384 kbps) when HSDPA is also configured 6 (equal or higher than 384 kbps) when HSDPA is also configured (Not Present if the CHOICE Gain Factors is set to Computed Gain Factors) |  |  | RBS-2102 |  |
| - Gain factor βd |  |  |  |  | 15 (Not Present if the CHOICE Gain Factors is set to Computed Gain Factors) |  |  | RBS-2103 |  |
| - Reference TFC ID |  |  |  |  | 0 |  |  | RBS-2104 |  |
| - CHOICE mode |  |  |  |  | FDD |  |  | RBS-2105 |  |
| - Power offset P p-m |  |  |  |  | Not Present |  |  | RBS-2106 |  |
| UL Transport channel information for all transport channels |  |  | A12 |  |  | Rel-6 |  | RBS-2107 |  |
|  |  |  | A19 |  |  | Rel-7 |  | RBS-2108 |  |
| - PRACH TFCS |  |  |  |  | Not Present |  |  | RBS-2109 |  |
| - CHOICE mode |  |  |  |  | FDD |  |  | RBS-2110 |  |
| - TFC subset |  |  |  |  | Not Present |  |  | RBS-2111 |  |
| - UL DCH TFCS |  |  |  |  |  |  |  | RBS-2112 |  |
| - CHOICE TFCI signalling |  |  |  |  | Normal |  |  | RBS-2113 |  |
| - TFCI Field 1 information |  |  |  |  |  |  |  | RBS-2114 |  |
| - CHOICE TFCS representation |  |  |  |  | Complete reconfiguration |  |  | RBS-2115 |  |
| - TFCS complete reconfigure information |  |  |  |  |  |  |  | RBS-2116 |  |
| - CHOICE CTFC Size |  |  |  |  | ctfc2bit |  |  | RBS-2117 |  |
| - CTFC information |  |  |  |  |  |  |  | RBS-2118 |  |
| - CTFC |  |  |  |  | 0 ((UL DCH RAB, DCCH)=(TF0, TF0)) |  |  | RBS-2119 |  |
| - Power offset information |  |  |  |  |  |  |  | RBS-2120 |  |
| - CHOICE Gain Factors |  |  |  |  | Computed Gain Factors |  |  | RBS-2121 |  |
| - CTFC |  |  |  |  | 1 ((UL DCH RAB, DCCH)=(TF0, TF1)) |  |  | RBS-2122 |  |
| - Power offset information |  |  |  |  |  |  |  | RBS-2123 |  |
| - CHOICE Gain Factors |  |  |  |  | Signalled Gain Factors |  |  | RBS-2124 |  |
| - Gain factor βc |  |  |  |  | 11 (below 64 kbps) 9 (equal or higher than 64 kbps) when HSDPA is not configured 9 (equal or higher than 64 kbps and below 384 kbps) when HSDPA is also configured 6 (equal or higher than 384 kbps) when HSDPA is also configured (Not Present if the CHOICE Gain Factors is set to Computed Gain Factors) |  |  | RBS-2125 |  |
| - Gain factor βd |  |  |  |  | 15 |  |  | RBS-2126 |  |
| - Reference TFC ID |  |  |  |  | 0 |  |  | RBS-2127 |  |
| - CHOICE mode |  |  |  |  | FDD |  |  | RBS-2128 |  |
| - Power offset P p-m |  |  |  |  | Not Present |  |  | RBS-2129 |  |
| UL Transport channel information for all transport channels |  |  | A13, A14, A15, A16, |  | Not Present | Rel-6 |  | RBS-2130 |  |
|  |  |  | A17b, A17c, A17d, A17e, A19a, A20, A21, A22, A24 |  |  | Rel-7 |  | RBS-2131 |  |
|  |  |  | , A23 |  |  | Rel-7 Rel-8 |  | RBS-2132 |  |
|  |  |  | , A25, A25b, A26, A27, A27a, A29, A30 |  |  | Rel-8 |  | RBS-2133 |  |
|  |  |  | A25c |  |  | Rel-9 |  | RBS-2133a |  |
| Deleted UL TrCH information |  |  | A1, A2, A3, A4, A5, A6, A7, A8, A11 |  | Not Present |  |  | RBS-2134 |  |
|  |  |  | , A9, A10 |  |  | Rel-5 |  | RBS-2135 |  |
|  |  |  | , A12 |  |  | Rel-6 |  | RBS-2136 |  |
|  |  |  | , A17, A17a, A18, A19, A20, A21, A24 |  |  | Rel-7 |  | RBS-2137 |  |
|  |  |  | , A23, A28a |  |  | Rel-7 Rel-8 |  | RBS-2138 |  |
|  |  |  | , A28, A29, A30 |  |  | Rel-8 |  | RBS-2139 |  |
|  |  |  | , A31, A32 A33, A34 |  |  | Rel-9 Rel-10 |  | RBS-2140 |  |
| Deleted UL TrCH information |  |  | A13, A14, A15, A16 |  |  | Rel-6 |  | RBS-2141 |  |
|  |  |  | A17b, A17c, A17d, A17e, A19a, A22, A26, A27, A27a |  |  | Rel-7 |  | RBS-2142 |  |
|  |  |  | A25, A25b |  |  | Rel-8 |  | RBS-2143 |  |
|  |  |  | , A25c |  |  | Rel-9 |  | RBS-2143a |  |
| - Uplink transport channel type |  |  | DCH |  |  |  |  | RBS-2144 |  |
| - UL transport channel identity |  |  | 5 |  |  |  |  | RBS-2145 |  |
| Added or Reconfigured UL TrCH information |  |  | A1, A3 A4, A5, A6, A7 |  | 1 DCH added, 1 DCH reconfigured (if from cell_DCH) OR 2 DCHs added (if from cell_FACH) |  |  | RBS-2146 |  |
|  |  |  | , A9, A10 |  |  | Rel-5 |  | RBS-2147 |  |
|  |  |  | , A17, A17a, A18, A28a |  |  | Rel-7 |  | RBS-2148 |  |
|  |  |  | , A28 |  |  | Rel-8 |  | RBS-2149 |  |
| - Uplink transport channel type |  |  |  |  | DCH |  |  | RBS-2150 |  |
| - UL Transport channel identity |  |  |  |  | 1 |  |  | RBS-2151 |  |
| - TFS |  |  |  |  |  |  |  | RBS-2152 |  |
| - CHOICE Transport channel type |  |  |  |  | Dedicated transport channels |  |  | RBS-2153 |  |
| - Dynamic Transport format information |  |  |  |  |  |  |  | RBS-2154 |  |
| - RLC Size |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2155 |  |
| - Number of TBs and TTI List |  |  |  |  | (This IE is repeated for TFI number.) |  |  | RBS-2156 |  |
| - Transmission Time Interval |  |  |  |  | Not Present |  |  | RBS-2157 |  |
| - Number of Transport blocks |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2158 |  |
| - CHOICE Logical channel list |  |  |  |  | All |  |  | RBS-2159 |  |
| - Semi-static Transport Format information |  |  |  |  |  |  |  | RBS-2160 |  |
| - Transmission time interval |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2161 |  |
| - Type of channel coding |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2162 |  |
| - Coding Rate |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2163 |  |
| - Rate matching attribute |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2164 |  |
| - CRC size |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2165 |  |
| - Uplink transport channel type |  |  |  |  | DCH |  |  | RBS-2166 |  |
| - UL Transport channel identity |  |  |  |  | 5 |  |  | RBS-2167 |  |
| - TFS |  |  |  |  |  |  |  | RBS-2168 |  |
| - CHOICE Transport channel type |  |  |  |  | Dedicated transport channels |  |  | RBS-2169 |  |
| - Dynamic Transport format information |  |  |  |  |  |  |  | RBS-2170 |  |
| - RLC Size |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2171 |  |
| - Number of TBs and TTI List |  |  |  |  | (This IE is repeated for TFI number.) |  |  | RBS-2172 |  |
| - Transmission Time Interval |  |  |  |  | Not Present |  |  | RBS-2173 |  |
| - Number of Transport blocks |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2174 |  |
| - CHOICE Logical channel list |  |  |  |  | All |  |  | RBS-2175 |  |
| - Semi-static Transport Format information |  |  |  |  |  |  |  | RBS-2176 |  |
| - Transmission time interval |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2177 |  |
| - Type of channel coding |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2178 |  |
| - Coding Rate |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2179 |  |
| - Rate matching attribute |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2180 |  |
| - CRC size |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2181 |  |
| Added or Reconfigured UL TrCH information |  |  | A11 |  | 1 DCH added for DTCH |  |  | RBS-2182 |  |
| - Uplink transport channel type |  |  |  |  | DCH |  |  | RBS-2183 |  |
| - UL Transport channel identity |  |  |  |  | 4 |  |  | RBS-2184 |  |
| - TFS |  |  |  |  |  |  |  | RBS-2185 |  |
| - CHOICE Transport channel type |  |  |  |  | Dedicated transport channels |  |  | RBS-2186 |  |
| - Dynamic Transport format information |  |  |  |  |  |  |  | RBS-2187 |  |
| - RLC Size |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2188 |  |
| - Number of TBs and TTI List |  |  |  |  | (This IE is repeated for TFI number.) |  |  | RBS-2189 |  |
| - Transmission Time Interval |  |  |  |  | Not Present |  |  | RBS-2190 |  |
| - Number of Transport blocks |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2191 |  |
| - CHOICE Logical channel list |  |  |  |  | All |  |  | RBS-2192 |  |
| - Semi-static Transport Format information |  |  |  |  |  |  |  | RBS-2193 |  |
| - Transmission time interval |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2194 |  |
| - Type of channel coding |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2195 |  |
| - Coding Rate |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2196 |  |
| - Rate matching attribute |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2197 |  |
| - CRC size |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2198 |  |
| Added or Reconfigured UL TrCH information |  |  | A2, A8 |  | 4 TrCHs(DCH for DCCH and 3DCHs for DTCH) |  |  | RBS-2199 |  |
| - Uplink transport channel type |  |  |  |  | DCH |  |  | RBS-2200 |  |
| - UL Transport channel identity |  |  |  |  | 5 |  |  | RBS-2201 |  |
| - TFS |  |  |  |  |  |  |  | RBS-2202 |  |
| - CHOICE Transport channel type |  |  |  |  | Dedicated transport channels |  |  | RBS-2203 |  |
| - Dynamic Transport format information |  |  |  |  |  |  |  | RBS-2204 |  |
| - RLC Size |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2205 |  |
| - Number of TBs and TTI List |  |  |  |  | (This IE is repeated for TFI number.) |  |  | RBS-2206 |  |
| - Transmission Time Interval |  |  |  |  | Not Present |  |  | RBS-2207 |  |
| - Number of Transport blocks |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2208 |  |
| - CHOICE Logical channel list |  |  |  |  | All |  |  | RBS-2209 |  |
| - Semi-static Transport Format information |  |  |  |  |  |  |  | RBS-2210 |  |
| - Transmission time interval |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2211 |  |
| - Type of channel coding |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2212 |  |
| - Coding Rate |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2213 |  |
| - Rate matching attribute |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2214 |  |
| - CRC size |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2215 |  |
| - Uplink transport channel type |  |  |  |  | DCH |  |  | RBS-2216 |  |
| - UL Transport channel identity |  |  |  |  | 1 |  |  | RBS-2217 |  |
| - TFS |  |  |  |  |  |  |  | RBS-2218 |  |
| - CHOICE Transport channel type |  |  |  |  | Dedicated transport channels |  |  | RBS-2219 |  |
| - Dynamic Transport format information |  |  |  |  |  |  |  | RBS-2220 |  |
| - RLC Size |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2221 |  |
| - Number of TBs and TTI List |  |  |  |  | (This IE is repeated for TFI number.) |  |  | RBS-2222 |  |
| - Transmission Time Interval |  |  |  |  | Not Present |  |  | RBS-2223 |  |
| - Number of Transport blocks |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2224 |  |
| - CHOICE Logical channel list |  |  |  |  | All |  |  | RBS-2225 |  |
| - Semi-static Transport Format information |  |  |  |  |  |  |  | RBS-2226 |  |
| - Transmission time interval |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2227 |  |
| - Type of channel coding |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2228 |  |
| - Coding Rate |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2229 |  |
| - Rate matching attribute |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2230 |  |
| - CRC size |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2231 |  |
| - Uplink transport channel type |  |  |  |  | DCH |  |  | RBS-2232 |  |
| - UL Transport channel identity |  |  |  |  | 2 |  |  | RBS-2233 |  |
| - TFS |  |  |  |  |  |  |  | RBS-2234 |  |
| - CHOICE Transport channel type |  |  |  |  | Dedicated transport channels |  |  | RBS-2235 |  |
| - Dynamic Transport format information |  |  |  |  |  |  |  | RBS-2236 |  |
| - RLC Size |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2237 |  |
| - Number of TBs and TTI List |  |  |  |  | (This IE is repeated for TFI number.) |  |  | RBS-2238 |  |
| - Transmission Time Interval |  |  |  |  | Not Present |  |  | RBS-2239 |  |
| - Number of Transport blocks |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2240 |  |
| - CHOICE Logical channel list |  |  |  |  | All |  |  | RBS-2241 |  |
| - Semi-static Transport Format information |  |  |  |  |  |  |  | RBS-2242 |  |
| - Transmission time interval |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2243 |  |
| - Type of channel coding |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2244 |  |
| - Coding Rate |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2245 |  |
| - Rate matching attribute |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2246 |  |
| - CRC size |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2247 |  |
| - Uplink transport channel type |  |  |  |  | DCH |  |  | RBS-2248 |  |
| - UL Transport channel identity |  |  |  |  | 3 |  |  | RBS-2249 |  |
| - TFS |  |  |  |  |  |  |  | RBS-2250 |  |
| - CHOICE Transport channel type |  |  |  |  | Dedicated transport channels |  |  | RBS-2251 |  |
| - Dynamic Transport format information |  |  |  |  |  |  |  | RBS-2252 |  |
| - RLC Size |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2253 |  |
| - Number of TBs and TTI List |  |  |  |  | (This IE is repeated for TFI number.) |  |  | RBS-2254 |  |
| - Transmission Time Interval |  |  |  |  | Not Present |  |  | RBS-2255 |  |
| - Number of Transport blocks |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2256 |  |
| - CHOICE Logical channel list |  |  |  |  | All |  |  | RBS-2257 |  |
| - Semi-static Transport Format information |  |  |  |  |  |  |  | RBS-2258 |  |
| - Transmission time interval |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2259 |  |
| - Type of channel coding |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2260 |  |
| - Coding Rate |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2261 |  |
| - Rate matching attribute |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2262 |  |
| - CRC size |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2263 |  |
| Added or Reconfigured UL TrCH information |  |  | A12 |  | 1 E-DCH added, 1 DCH added, 1 DCH reconfigured | Rel-6 |  | RBS-2264 |  |
|  |  |  | A19 |  |  | Rel-7 |  | RBS-2265 |  |
| - Uplink transport channel type |  |  |  |  | E-DCH |  |  | RBS-2266 |  |
| - CHOICE UL parameters |  |  |  |  | E-DCH |  |  | RBS-2267 |  |
| - UL MAC header type |  |  |  |  | Not present | Rel-8 |  | RBS-2268 |  |
| - UL MAC header type |  |  | MAC-I-FIXED, MAC-I-FLEX |  | MAC-i/is | Rel-8 |  | RBS-2269 |  |
| - E-DCH Transmission Time Interval |  |  |  |  | set to 2ms if supported by the UE E-DCH category, or 10ms if the UE E-DCH category does not support 2ms TTI |  |  | RBS-2270 |  |
| - HARQ info for E-DCH |  |  |  |  |  |  |  | RBS-2271 |  |
| - HARQ RV Configuration |  |  |  |  | rvtable |  |  | RBS-2272 |  |
| - Added or reconfigured E-DCH MAC-d flow |  |  |  |  |  |  |  | RBS-2273 |  |
| - E-DCH MAC-d flow identity |  |  |  |  | 2 |  |  | RBS-2274 |  |
| - E-DCH MAC-d flow power offset |  |  |  |  | 0 |  |  | RBS-2275 |  |
| - E-DCH MAC-d flow maximum number of retransmissions |  |  |  |  | 7 |  |  | RBS-2276 |  |
| - E-DCH MAC-d flow multiplexing list |  |  |  |  | Not Present |  |  | RBS-2277 |  |
| - CHOICE transmission grant type |  |  |  |  | Scheduled grant info |  |  | RBS-2278 |  |
| - Uplink transport channel type |  |  |  |  | DCH |  |  | RBS-2279 |  |
| - UL Transport channel identity |  |  |  |  | 1 |  |  | RBS-2280 |  |
| - TFS |  |  |  |  |  |  |  | RBS-2281 |  |
| - CHOICE Transport channel type |  |  |  |  | Dedicated transport channels |  |  | RBS-2282 |  |
| - Dynamic Transport format information |  |  |  |  |  |  |  | RBS-2283 |  |
| - RLC Size |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2284 |  |
| - Number of TBs and TTI List |  |  |  |  | (This IE is repeated for TFI number.) |  |  | RBS-2285 |  |
| - Transmission Time Interval |  |  |  |  | Not Present |  |  | RBS-2286 |  |
| - Number of Transport blocks |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2287 |  |
| - CHOICE Logical channel list |  |  |  |  | All |  |  | RBS-2288 |  |
| - Semi-static Transport Format information |  |  |  |  |  |  |  | RBS-2289 |  |
| - Transmission time interval |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2290 |  |
| - Type of channel coding |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2291 |  |
| - Coding Rate |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2292 |  |
| - Rate matching attribute |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2293 |  |
| - CRC size |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2294 |  |
| - Uplink transport channel type |  |  |  |  | DCH |  |  | RBS-2295 |  |
| - UL Transport channel identity |  |  |  |  | 5 |  |  | RBS-2296 |  |
| - TFS |  |  |  |  |  |  |  | RBS-2297 |  |
| - CHOICE Transport channel type |  |  |  |  | Dedicated transport channels |  |  | RBS-2298 |  |
| - Dynamic Transport format information |  |  |  |  |  |  |  | RBS-2299 |  |
| - RLC Size |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2300 |  |
| - Number of TBs and TTI List |  |  |  |  | (This IE is repeated for TFI number.) |  |  | RBS-2301 |  |
| - Transmission Time Interval |  |  |  |  | Not Present |  |  | RBS-2302 |  |
| - Number of Transport blocks |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2303 |  |
| - CHOICE Logical channel list |  |  |  |  | All |  |  | RBS-2304 |  |
| - Semi-static Transport Format information |  |  |  |  |  |  |  | RBS-2305 |  |
| - Transmission time interval |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2306 |  |
| - Type of channel coding |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2307 |  |
| - Coding Rate |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2308 |  |
| - Rate matching attribute |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2309 |  |
| - CRC size |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2310 |  |
| Added or Reconfigured UL TrCH information |  |  | A13, A14, |  | 1 E-DCH added with one DCCH MAC-d flow and one DTCH MAC-d flow | Rel-6 |  | RBS-2311 |  |
|  |  |  | A17b, A17c, A17d, A17e, A19a, A20 |  |  | Rel-7 |  | RBS-2312 |  |
|  |  |  | , A25, A25b, A27, A27a |  |  | Rel-8 |  | RBS-2313 |  |
|  |  |  | , A25c, A31, A32 A33, A34 |  |  | Rel-9 Rel-10 |  | RBS-2314 |  |
| - Uplink transport channel type |  |  |  |  | E-DCH |  |  | RBS-2315 |  |
| - CHOICE UL parameters |  |  |  |  | E-DCH |  |  | RBS-2316 |  |
| - UL MAC header type |  |  |  |  | Not present | Rel-8 |  | RBS-2317 |  |
| - UL MAC header type |  |  | MAC-I-FIXED, MAC-I-FLEX |  | MAC-i/is | Rel-8 |  | RBS-2318 |  |
| - E-DCH Transmission Time Interval |  |  |  |  | set to 2ms if supported by the UE E-DCH category, or 10ms if the UE E-DCH category does not support 2ms TTI |  |  | RBS-2319 |  |
| - HARQ info for E-DCH |  |  |  |  |  |  |  | RBS-2320 |  |
| - HARQ RV Configuration |  |  |  |  | rvtable |  |  | RBS-2321 |  |
| - Added or reconfigured E-DCH MAC-d flow |  |  |  |  | (for DCCH) |  |  | RBS-2322 |  |
| - E-DCH MAC-d flow identity |  |  |  |  | 1 |  |  | RBS-2323 |  |
| - E-DCH MAC-d flow power offset |  |  |  |  | 0 |  |  | RBS-2324 |  |
| - E-DCH MAC-d flow maximum number of retransmissions |  |  |  |  | 7 |  |  | RBS-2325 |  |
| - E-DCH MAC-d flow multiplexing list |  |  |  |  | Not Present |  |  | RBS-2326 |  |
| - CHOICE transmission grant type |  |  |  |  | Non-scheduled grant info |  |  | RBS-2327 |  |
| - Max MAC-e PDU contents size |  |  | MAC-I-FIXED, MAC-I-FLEX |  | 168 bits | Rel-8 |  | RBS-2328 |  |
| - Max MAC-e PDU contents size |  |  |  |  | 162 bits |  |  | RBS-2329 |  |
| - 2 ms non-scheduled transmission grant HARQ process allocation |  |  |  |  | Not Present |  |  | RBS-2330 |  |
| - Added or reconfigured E-DCH MAC-d flow |  |  |  |  | (for DTCH) |  |  | RBS-2331 |  |
| - E-DCH MAC-d flow identity |  |  |  |  | 2 |  |  | RBS-2332 |  |
| - E-DCH MAC-d flow power offset |  |  |  |  | 0 |  |  | RBS-2333 |  |
| - E-DCH MAC-d flow maximum number of retransmissions |  |  |  |  | 7 |  |  | RBS-2334 |  |
| - E-DCH MAC-d flow multiplexing list |  |  |  |  | Not Present |  |  | RBS-2335 |  |
| - CHOICE transmission grant type |  |  |  |  | Scheduled grant info |  |  | RBS-2336 |  |
| Added or Reconfigured UL TrCH information |  |  | A15 |  | 1 E-DCH added with one DCCH MAC-d flow and two DTCH MAC-d flows | Rel-6 |  | RBS-2337 |  |
| - Uplink transport channel type |  |  |  |  | E-DCH |  |  | RBS-2338 |  |
| - CHOICE UL parameters |  |  |  |  | E-DCH |  |  | RBS-2339 |  |
| - UL MAC header type |  |  |  |  | Not present | Rel-8 |  | RBS-2340 |  |
| - UL MAC header type |  |  | MAC-I-FIXED, MAC-I-FLEX |  | MAC-i/is | Rel-8 |  | RBS-2341 |  |
| - E-DCH Transmission Time Interval |  |  |  |  | set to 2ms if supported by the UE E-DCH category, or 10ms if the UE E-DCH category does not support 2ms TTI |  |  | RBS-2342 |  |
| - HARQ info for E-DCH |  |  |  |  |  |  |  | RBS-2343 |  |
| - HARQ RV Configuration |  |  |  |  | rvtable |  |  | RBS-2344 |  |
| - Added or reconfigured E-DCH MAC-d flow |  |  |  |  | (for DCCH) |  |  | RBS-2345 |  |
| - E-DCH MAC-d flow identity |  |  |  |  | 1 |  |  | RBS-2346 |  |
| - E-DCH MAC-d flow power offset |  |  |  |  | 0 |  |  | RBS-2347 |  |
| - E-DCH MAC-d flow maximum number of retransmissions |  |  |  |  | 7 |  |  | RBS-2348 |  |
| - E-DCH MAC-d flow multiplexing list |  |  |  |  | Not Present |  |  | RBS-2349 |  |
| - CHOICE transmission grant type |  |  |  |  | Non-scheduled grant info |  |  | RBS-2350 |  |
| - Max MAC-e PDU contents size |  |  |  |  | 162 bits |  |  | RBS-2351 |  |
| - Max MAC-e PDU contents size |  |  | MAC-I-FIXED, MAC-I-FLEX |  | 168 bits | Rel-8 |  | RBS-2352 |  |
| - 2 ms non-scheduled transmission grant HARQ process allocation |  |  |  |  | Not Present |  |  | RBS-2353 |  |
| - Added or reconfigured E-DCH MAC-d flow |  |  |  |  | (for first DTCH) |  |  | RBS-2354 |  |
| - E-DCH MAC-d flow identity |  |  |  |  | 2 |  |  | RBS-2355 |  |
| - E-DCH MAC-d flow power offset |  |  |  |  | 0 |  |  | RBS-2356 |  |
| - E-DCH MAC-d flow maximum number of retransmissions |  |  |  |  | 7 |  |  | RBS-2357 |  |
| - E-DCH MAC-d flow multiplexing list |  |  |  |  | Not Present |  |  | RBS-2358 |  |
| - CHOICE transmission grant type |  |  |  |  | Scheduled grant info |  |  | RBS-2359 |  |
| - Added or reconfigured E-DCH MAC-d flow |  |  |  |  | (for second DTCH) |  |  | RBS-2360 |  |
| - E-DCH MAC-d flow identity |  |  |  |  | 3 |  |  | RBS-2361 |  |
| - E-DCH MAC-d flow power offset |  |  |  |  | 0 |  |  | RBS-2362 |  |
| - E-DCH MAC-d flow maximum number of retransmissions |  |  |  |  | 7 |  |  | RBS-2363 |  |
| - E-DCH MAC-d flow multiplexing list |  |  |  |  | Not Present |  |  | RBS-2364 |  |
| - CHOICE transmission grant type |  |  |  |  | Scheduled grant info |  |  | RBS-2365 |  |
| Added or Reconfigured UL TrCH information |  |  | A16 |  | 1 E-DCH added with one DCCH MAC-d flow and two DTCH MAC-d flows | Rel-6 |  | RBS-2366 |  |
|  |  |  | , A21, A22 |  |  | Rel-7 |  | RBS-2367 |  |
| - Uplink transport channel type |  |  |  |  | E-DCH |  |  | RBS-2368 |  |
| - CHOICE UL parameters |  |  |  |  | E-DCH |  |  | RBS-2369 |  |
| - UL MAC header type |  |  |  |  | Not present | Rel-8 |  | RBS-2370 |  |
| - UL MAC header type |  |  | MAC-I-FIXED, MAC-I-FLEX |  | MAC-i/is | Rel-8 |  | RBS-2371 |  |
| - CHOICE mode |  |  |  |  | FDD | Rel-7 |  | RBS-2372 |  |
| - E-DCH Transmission Time Interval |  |  |  |  | set to 2ms if supported by the UE E-DCH category, or 10ms if the UE E-DCH category does not support 2ms TTI |  |  | RBS-2373 |  |
| - HARQ info for E-DCH |  |  |  |  |  |  |  | RBS-2374 |  |
| - HARQ RV Configuration |  |  |  |  | rvtable |  |  | RBS-2375 |  |
| - Added or reconfigured E-DCH MAC-d flow |  |  |  |  | (for DCCH) |  |  | RBS-2376 |  |
| - E-DCH MAC-d flow identity |  |  |  |  | 1 |  |  | RBS-2377 |  |
| - E-DCH MAC-d flow power offset |  |  |  |  | 0 |  |  | RBS-2378 |  |
| - E-DCH MAC-d flow maximum number of retransmissions |  |  |  |  | 7 |  |  | RBS-2379 |  |
| - E-DCH MAC-d flow multiplexing list |  |  |  |  | Not Present |  |  | RBS-2380 |  |
| - CHOICE transmission grant type |  |  |  |  | Non-scheduled grant info |  |  | RBS-2381 |  |
| - Max MAC-e PDU contents size |  |  |  |  | 162 bits |  |  | RBS-2382 |  |
| - Max MAC-e PDU contents size |  |  | MAC-I-FIXED, MAC-I-FLEX |  | 168 bits | Rel-8 |  | RBS-2383 |  |
| - 2 ms non-scheduled transmission grant HARQ process allocation |  |  |  |  | Not Present |  |  | RBS-2384 |  |
| - Added or reconfigured E-DCH MAC-d flow |  |  |  |  | (for first DTCH) |  |  | RBS-2385 |  |
| - E-DCH MAC-d flow identity |  |  |  |  | 2 |  |  | RBS-2386 |  |
| - E-DCH MAC-d flow power offset |  |  |  |  | 0 |  |  | RBS-2387 |  |
| - E-DCH MAC-d flow maximum number of retransmissions |  |  |  |  | 7 |  |  | RBS-2388 |  |
| - E-DCH MAC-d flow multiplexing list |  |  |  |  | Not Present |  |  | RBS-2389 |  |
| - CHOICE transmission grant type |  |  |  |  | Scheduled grant info |  |  | RBS-2390 |  |
| - Added or reconfigured E-DCH MAC-d flow |  |  |  |  | (for second DTCH) |  |  | RBS-2391 |  |
| - E-DCH MAC-d flow identity |  |  |  |  | 4 |  |  | RBS-2392 |  |
| - E-DCH MAC-d flow power offset |  |  |  |  | 0 |  |  | RBS-2393 |  |
| - E-DCH MAC-d flow maximum number of retransmissions |  |  |  |  | 7 |  |  | RBS-2394 |  |
| - E-DCH MAC-d flow multiplexing list |  |  |  |  | Not Present |  |  | RBS-2395 |  |
| - CHOICE transmission grant type |  |  |  |  | Scheduled grant info |  |  | RBS-2396 |  |
| Added or Reconfigured UL TrCH information |  |  | A23 |  | 1 E-DCH added with one DCCH MAC-d flow and one DTCH MAC-d flow | Rel-7 Rel-8 |  | RBS-2397 |  |
| - Uplink transport channel type |  |  |  |  | E-DCH |  |  | RBS-2398 |  |
| - CHOICE UL parameters |  |  |  |  | E-DCH |  |  | RBS-2399 |  |
| - UL MAC header type |  |  |  |  | Not present | Rel-8 |  | RBS-2400 |  |
| - UL MAC header type |  |  | MAC-I-FIXED, MAC-I-FLEX |  | MAC-i/is | Rel-8 |  | RBS-2401 |  |
| - E-DCH Transmission Time Interval |  |  |  |  | set to 2ms if supported by the UE E-DCH category, or 10ms if the UE E-DCH category does not support 2ms TTI |  |  | RBS-2402 |  |
| - HARQ info for E-DCH |  |  |  |  |  |  |  | RBS-2403 |  |
| - HARQ RV Configuration |  |  |  |  | rvtable |  |  | RBS-2404 |  |
| - Added or reconfigured E-DCH MAC-d flow |  |  |  |  | (for DCCH) |  |  | RBS-2405 |  |
| - E-DCH MAC-d flow identity |  |  |  |  | 1 |  |  | RBS-2406 |  |
| - E-DCH MAC-d flow power offset |  |  |  |  | 0 |  |  | RBS-2407 |  |
| - E-DCH MAC-d flow maximum number of retransmissions |  |  |  |  | 7 |  |  | RBS-2408 |  |
| - E-DCH MAC-d flow multiplexing list |  |  |  |  | Not Present |  |  | RBS-2409 |  |
| - CHOICE transmission grant type |  |  |  |  | Non-scheduled grant info |  |  | RBS-2410 |  |
| - Max MAC-e PDU contents size |  |  |  |  | 162 bits | Rel-6 |  | RBS-2411 |  |
| - Max MAC-e PDU contents size |  |  | MAC-I-FIXED, MAC-I-FLEX |  | 168 bits | Rel-8 |  | RBS-2412 |  |
| - 2 ms non-scheduled transmission grant HARQ process allocation |  |  |  |  | ‘01000000’B if 2ms TTI configured otherwise Not Present |  |  | RBS-2413 |  |
| - Added or reconfigured E-DCH MAC-d flow |  |  |  |  | (for DTCH) |  |  | RBS-2414 |  |
| - E-DCH MAC-d flow identity |  |  |  |  | 2 |  |  | RBS-2415 |  |
| - E-DCH MAC-d flow power offset |  |  |  |  | 0 |  |  | RBS-2416 |  |
| - E-DCH MAC-d flow maximum number of retransmissions |  |  |  |  | 3 if 2ms TTI configured, otherwise 1 |  |  | RBS-2417 |  |
| - E-DCH MAC-d flow multiplexing list |  |  |  |  | Not Present |  |  | RBS-2418 |  |
| - CHOICE transmission grant type |  |  |  |  | Non-scheduled grant info |  |  | RBS-2419 |  |
| - Max MAC-e PDU contents size |  |  |  |  | 546 bits | Rel-6 |  | RBS-2420 |  |
| - Max MAC-e PDU contents size |  |  | MAC-I-FIXED, MAC-I-FLEX |  | 552 bits | Rel-8 |  | RBS-2421 |  |
| Added or Reconfigured UL TrCH information |  |  | A26 |  | 1 E-DCH added with one DCCH MAC-d flow and three DTCH MAC-d flows | Rel-8 |  | RBS-2422 |  |
| - Uplink transport channel type |  |  |  |  | E-DCH |  |  | RBS-2423 |  |
| - CHOICE UL parameters |  |  |  |  | E-DCH |  |  | RBS-2424 |  |
| - UL MAC header type |  |  |  |  | Not present |  |  | RBS-2425 |  |
| - UL MAC header type |  |  |  |  | MAC-i/is |  |  | RBS-2426 |  |
| - CHOICE mode |  |  |  |  | FDD | Rel-7 |  | RBS-2427 |  |
| - E-DCH Transmission Time Interval |  |  |  |  | set to 2ms if supported by the UE E-DCH category, or 10ms if the UE E-DCH category does not support 2ms TTI |  |  | RBS-2428 |  |
| - HARQ info for E-DCH |  |  |  |  |  |  |  | RBS-2429 |  |
| - HARQ RV Configuration |  |  |  |  | rvtable |  |  | RBS-2430 |  |
| - Added or reconfigured E-DCH MAC-d flow |  |  |  |  | (for DCCH) |  |  | RBS-2431 |  |
| - E-DCH MAC-d flow identity |  |  |  |  | 1 |  |  | RBS-2432 |  |
| - E-DCH MAC-d flow power offset |  |  |  |  | 0 |  |  | RBS-2433 |  |
| - E-DCH MAC-d flow maximum number of retransmissions |  |  |  |  | 7 |  |  | RBS-2434 |  |
| - E-DCH MAC-d flow multiplexing list |  |  |  |  | Not Present |  |  | RBS-2435 |  |
| - CHOICE transmission grant type |  |  |  |  | Non-scheduled grant info |  |  | RBS-2436 |  |
| - Max MAC-e PDU contents size |  |  |  |  | 168 bits |  |  | RBS-2437 |  |
| - 2 ms non-scheduled transmission grant HARQ process allocation |  |  |  |  | Not Present |  |  | RBS-2438 |  |
| - Added or reconfigured E-DCH MAC-d flow |  |  |  |  | (for first DTCH) |  |  | RBS-2439 |  |
| - E-DCH MAC-d flow identity |  |  |  |  | 2 |  |  | RBS-2440 |  |
| - E-DCH MAC-d flow power offset |  |  |  |  | 0 |  |  | RBS-2441 |  |
| - E-DCH MAC-d flow maximum number of retransmissions |  |  |  |  | 7 |  |  | RBS-2442 |  |
| - E-DCH MAC-d flow multiplexing list |  |  |  |  | Not Present |  |  | RBS-2443 |  |
| - CHOICE transmission grant type |  |  |  |  | Scheduled grant info |  |  | RBS-2444 |  |
| - Added or reconfigured E-DCH MAC-d flow |  |  |  |  | (for second DTCH) |  |  | RBS-2445 |  |
| - E-DCH MAC-d flow identity |  |  |  |  | 3 |  |  | RBS-2446 |  |
| - E-DCH MAC-d flow power offset |  |  |  |  | 0 |  |  | RBS-2447 |  |
| - E-DCH MAC-d flow maximum number of retransmissions |  |  |  |  | 7 |  |  | RBS-2448 |  |
| - E-DCH MAC-d flow multiplexing list |  |  |  |  | Not Present |  |  | RBS-2449 |  |
| - CHOICE transmission grant type |  |  |  |  | Scheduled grant info |  |  | RBS-2450 |  |
| - Added or reconfigured E-DCH MAC-d flow |  |  |  |  | (for third DTCH) |  |  | RBS-2451 |  |
| - E-DCH MAC-d flow identity |  |  |  |  | 4 |  |  | RBS-2452 |  |
| - E-DCH MAC-d flow power offset |  |  |  |  | 0 |  |  | RBS-2453 |  |
| - E-DCH MAC-d flow maximum number of retransmissions |  |  |  |  | 7 |  |  | RBS-2454 |  |
| - E-DCH MAC-d flow multiplexing list |  |  |  |  | Not Present |  |  | RBS-2455 |  |
| - CHOICE transmission grant type |  |  |  |  | Scheduled grant info |  |  | RBS-2456 |  |
| Added or Reconfigured UL TrCH information |  |  | A29 |  | one DTCH MAC-d flow | Rel-8 |  | RBS-2457 |  |
| - Uplink transport channel type |  |  |  |  | E-DCH |  |  | RBS-2458 |  |
| - CHOICE UL parameters |  |  |  |  | E-DCH |  |  | RBS-2459 |  |
| - UL MAC header type |  |  | MAC-I-FIXED, MAC-I-FLEX |  | MAC-i/is | Rel-8 |  | RBS-2460 |  |
| - E-DCH Transmission Time Interval |  |  |  |  | set to 2ms if supported by the UE E-DCH category, or 10ms if the UE E-DCH category does not support 2ms TTI |  |  | RBS-2461 |  |
| - HARQ info for E-DCH |  |  |  |  |  |  |  | RBS-2462 |  |
| - HARQ RV Configuration |  |  |  |  | rvtable |  |  | RBS-2463 |  |
| - Added or reconfigured E-DCH MAC-d flow |  |  |  |  |  |  |  | RBS-2464 |  |
| - E-DCH MAC-d flow identity |  |  |  |  | 0 |  |  | RBS-2465 |  |
| - E-DCH MAC-d flow power offset |  |  |  |  | 0 |  |  | RBS-2466 |  |
| - E-DCH MAC-d flow maximum number of retransmissions |  |  |  |  | 7 |  |  | RBS-2467 |  |
| - E-DCH MAC-d flow multiplexing list |  |  |  |  | Not Present |  |  | RBS-2468 |  |
| - CHOICE transmission grant type |  |  |  |  | Scheduled grant info |  |  | RBS-2469 |  |
| Added or Reconfigured UL TrCH information |  |  | A30 |  | 1 E-DCH added with one DCCH MAC-d flow and one DTCH MAC-d flow | Rel-8 |  | RBS-2470 |  |
| - Uplink transport channel type |  |  |  |  | E-DCH |  |  | RBS-2471 |  |
| - CHOICE UL parameters |  |  |  |  | E-DCH |  |  | RBS-2472 |  |
| - UL MAC header type |  |  | MAC-I-FIXED, MAC-I-FLEX |  | MAC-i/is |  |  | RBS-2473 |  |
| - E-DCH Transmission Time Interval |  |  |  |  | set to 2ms if supported by the UE E-DCH category, or 10ms if the UE E-DCH category does not support 2ms TTI |  |  | RBS-2474 |  |
| - HARQ info for E-DCH |  |  |  |  |  |  |  | RBS-2475 |  |
| - HARQ RV Configuration |  |  |  |  | rvtable |  |  | RBS-2476 |  |
| - Added or reconfigured E-DCH MAC-d flow |  |  |  |  | (for DCCH) |  |  | RBS-2477 |  |
| - E-DCH MAC-d flow identity |  |  |  |  | 2 |  |  | RBS-2478 |  |
| - E-DCH MAC-d flow power offset |  |  |  |  | 0 |  |  | RBS-2479 |  |
| - E-DCH MAC-d flow maximum number of retransmissions |  |  |  |  | 7 |  |  | RBS-2480 |  |
| - E-DCH MAC-d flow multiplexing list |  |  |  |  | Not Present |  |  | RBS-2481 |  |
| - CHOICE transmission grant type |  |  |  |  | Non-scheduled grant info |  |  | RBS-2482 |  |
| - Max MAC-e PDU contents size |  |  |  |  | 168 bits |  |  | RBS-2483 |  |
| - 2 ms non-scheduled transmission grant HARQ process allocation |  |  |  |  | Not Present |  |  | RBS-2484 |  |
| - Added or reconfigured E-DCH MAC-d flow |  |  |  |  | (for DTCH) |  |  | RBS-2485 |  |
| - E-DCH MAC-d flow identity |  |  |  |  | 3 |  |  | RBS-2486 |  |
| - E-DCH MAC-d flow power offset |  |  |  |  | 0 |  |  | RBS-2487 |  |
| - E-DCH MAC-d flow maximum number of retransmissions |  |  |  |  | 7 |  |  | RBS-2488 |  |
| - E-DCH MAC-d flow multiplexing list |  |  |  |  | Not Present |  |  | RBS-2489 |  |
| - CHOICE transmission grant type |  |  |  |  | Scheduled grant info |  |  | RBS-2490 |  |
| DL Transport channel information common for all transport channel |  |  | A1, A2, A7, A8 |  |  |  |  | RBS-2491 |  |
| - SCCPCH TFCS |  |  |  |  | Not Present |  |  | RBS-2492 |  |
| - CHOICE mode |  |  |  |  | FDD |  |  | RBS-2493 |  |
| - CHOICE DL parameters |  |  |  |  | SameasUL |  |  | RBS-2494 |  |
| DL Transport channel information common for all transport channel |  |  | A3, A4, A5, A6, A11 |  |  |  |  | RBS-2495 |  |
|  |  |  | A10 |  |  | Rel-5 |  | RBS-2496 |  |
|  |  |  | A12, A13, A15 |  |  | Rel-6 |  | RBS-2497 |  |
|  |  |  | , A17, A18, A17a, A17d, A17e, A19, A19a, A28a |  |  | Rel-7 |  | RBS-2498 |  |
|  |  |  | A25a, A25b, A26, A28 |  |  | Rel-8 |  | RBS-2499 |  |
|  |  |  | , A25c, A31, A32 A33, A34 |  |  | Rel-9 Rel-10 |  | RBS-2500 |  |
| - SCCPCH TFCS |  |  |  |  | Not Present |  |  | RBS-2501 |  |
| - CHOICE mode |  |  |  |  | FDD |  |  | RBS-2502 |  |
| - CHOICE DL parameters |  |  |  |  | Explicit |  |  | RBS-2503 |  |
| - DL DCH TFCS |  |  |  |  |  |  |  | RBS-2504 |  |
| - CHOICE TFCI Signalling |  |  |  |  | Normal |  |  | RBS-2505 |  |
| - TFCI Field 1 Information |  |  |  |  |  |  |  | RBS-2506 |  |
| - CHOICE TFCS representation |  |  |  |  | Complete reconfiguration |  |  | RBS-2507 |  |
| - TFCS complete reconfigure |  |  |  |  |  |  |  | RBS-2508 |  |
| - CHOICE CTFC Size |  |  |  |  | Number of bits used must be enough to cover all combinations of CTFC from clause 6.10.2.4 Parameter Set. |  |  | RBS-2509 |  |
| - CTFC information |  |  |  |  | This IE is repeated for TFC numbers and reference to clause 6.10.2.4 |  |  | RBS-2510 |  |
| - CTFC |  |  |  |  | Reference to clause 6.10.2.4 Parameter Set |  |  | RBS-2511 |  |
| - Power offset information |  |  |  |  | Not Present |  |  | RBS-2512 |  |
| DL Transport channel information common for all transport channel |  |  | A9 |  |  | Rel-5 |  | RBS-2513 |  |
| - SCCPCH TFCS |  |  |  |  | Not Present |  |  | RBS-2514 |  |
| - CHOICE mode |  |  |  |  | FDD |  |  | RBS-2515 |  |
| - CHOICE DL parameters |  |  |  |  | Explicit |  |  | RBS-2516 |  |
| - DL DCH TFCS |  |  |  |  |  |  |  | RBS-2517 |  |
| - CHOICE TFCI Signalling |  |  |  |  | Normal |  |  | RBS-2518 |  |
| - TFCI Field 1 Information |  |  |  |  |  |  |  | RBS-2519 |  |
| - CHOICE TFCS representation |  |  |  |  | Complete reconfiguration |  |  | RBS-2520 |  |
| - TFCS complete reconfigure |  |  |  |  |  |  |  | RBS-2521 |  |
| - CHOICE CTFC Size |  |  |  |  | ctfc2bit |  |  | RBS-2522 |  |
| - CTFC information |  |  |  |  |  |  |  | RBS-2523 |  |
| - CTFC |  |  |  |  | 0 ((DL DCH RAB, DCCH)=(TF0, TF0)) |  |  | RBS-2524 |  |
| - Power offset information |  |  |  |  | Not Present |  |  | RBS-2525 |  |
| - CTFC |  |  |  |  | 1 ((DL DCH RAB, DCCH)=(TF0, TF1)) |  |  | RBS-2526 |  |
| - Power offset information |  |  |  |  | Not Present |  |  | RBS-2527 |  |
| DL Transport channel information common for all transport channel |  |  | A14, A16 |  | Not Present | Rel-6 |  | RBS-2528 |  |
|  |  |  | A17b, A17c, A20, A21, A22, A24 |  |  | Rel-7 |  | RBS-2529 |  |
|  |  |  | , A23 |  |  | Rel-7 Rel-8 |  | RBS-2530 |  |
|  |  |  | A25, A27, A27a, A29, A30 |  |  | Rel-8 |  | RBS-2531 |  |
| Deleted DL TrCH information |  |  | A1, A2, A3, A4, A5, A6, A7, A8 |  | Not Present |  |  | RBS-2532 |  |
|  |  |  | , A9, A10 |  |  | Rel-5 |  | RBS-2533 |  |
|  |  |  | , A12, A13 |  |  | Rel-6 |  | RBS-2534 |  |
|  |  |  | , A17, A18, A17a, A17d, A17e, A19, A19a, A20, A21, A24, A28a |  |  | Rel-7 |  | RBS-2535 |  |
|  |  |  | , A25a, A25b, A26, A28, A29, A30 |  |  | Rel-8 |  | RBS-2536 |  |
|  |  |  | , A25c, A31, A32 A33, A34 |  |  | Rel-9 Rel-10 |  | RBS-2537 |  |
| Deleted DL TrCH information |  |  | A14, A16 |  |  | Rel-6 |  | RBS-2538 |  |
|  |  |  | A17b, A17c, A22 |  |  | Rel-7 |  | RBS-2539 |  |
|  |  |  | A25, A27, A27a |  |  | Rel-8 |  | RBS-2540 |  |
| - Downlink transport channel type |  |  |  |  | DCH |  |  | RBS-2541 |  |
| - DL Transport channel identity |  |  |  |  | 10 |  |  | RBS-2542 |  |
| Added or Reconfigured DL TrCH information |  |  | A1 |  | 1 DCH added, 1 DCH reconfigured |  |  | RBS-2543 |  |
| - Downlink transport channel type |  |  |  |  | DCH |  |  | RBS-2544 |  |
| - DL Transport channel identity |  |  |  |  | 6 |  |  | RBS-2545 |  |
| - CHOICE DL parameters |  |  |  |  | Same as UL |  |  | RBS-2546 |  |
| - Uplink transport channel type |  |  |  |  | DCH |  |  | RBS-2547 |  |
| - UL TrCH identity |  |  |  |  | 1 |  |  | RBS-2548 |  |
| - DCH quality target |  |  |  |  |  |  |  | RBS-2549 |  |
| - BLER Quality value |  |  |  |  | -20 (-2.0) |  |  | RBS-2550 |  |
| - Downlink transport channel type |  |  |  |  | DCH |  |  | RBS-2551 |  |
| - DL Transport channel identity |  |  |  |  | 10 |  |  | RBS-2552 |  |
| - CHOICE DL parameters |  |  |  |  | Same as UL |  |  | RBS-2553 |  |
| - Uplink transport channel type |  |  |  |  | DCH |  |  | RBS-2554 |  |
| - UL TrCH identity |  |  |  |  | 5 |  |  | RBS-2555 |  |
| - DCH quality target |  |  |  |  |  |  |  | RBS-2556 |  |
| - BLER Quality value |  |  |  |  | -20 (-2.0) |  |  | RBS-2557 |  |
| Added or Reconfigured DL TrCH information |  |  | A3, A4, A5, A6, A7 |  | 2 TrCHs(DCH for DCCH and DCH for DTCH) |  |  | RBS-2558 |  |
| - Downlink transport channel type |  |  |  |  | DCH |  |  | RBS-2559 |  |
| - DL Transport channel identity |  |  |  |  | 10 |  |  | RBS-2560 |  |
| - CHOICE DL parameters |  |  |  |  | Same as UL |  |  | RBS-2561 |  |
| - Uplink transport channel type |  |  |  |  | DCH |  |  | RBS-2562 |  |
| - UL TrCH identity |  |  |  |  | 5 |  |  | RBS-2563 |  |
| - DCH quality target |  |  |  |  |  |  |  | RBS-2564 |  |
| - BLER Quality value |  |  |  |  | -20 (-2.0) |  |  | RBS-2565 |  |
| - Downlink transport channel type |  |  |  |  | DCH |  |  | RBS-2566 |  |
| - DL Transport channel identity |  |  |  |  | 6 |  |  | RBS-2567 |  |
| - CHOICE DL parameters |  |  |  |  | Explicit Except for RAB with the symmetric DL and UL rate: Same as UL |  |  | RBS-2568 |  |
| - TFS |  |  |  |  |  |  |  | RBS-2569 |  |
| - CHOICE Transport channel type |  |  |  |  | Dedicated transport channel |  |  | RBS-2570 |  |
| - Dynamic transport format information |  |  |  |  |  |  |  | RBS-2571 |  |
| - RLC Size |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2572 |  |
| - Number of TBs and TTI List |  |  |  |  | (This IE is repeated for TFI number.) |  |  | RBS-2573 |  |
| - Transmission Time Interval |  |  |  |  | Not Present |  |  | RBS-2574 |  |
| - Number of Transport blocks |  |  |  |  | Reference to clause 6.10 Parameter Set only including TF0 |  |  | RBS-2575 |  |
| - CHOICE Logical channel list |  |  |  |  | All |  |  | RBS-2576 |  |
| - Semi-static Transport Format information |  |  |  |  |  |  |  | RBS-2577 |  |
| - Transmission time interval |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2578 |  |
| - Type of channel coding |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2579 |  |
| - Coding Rate |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2580 |  |
| - Rate matching attribute |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2581 |  |
| - CRC size |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2582 |  |
| - DCH quality target |  |  |  |  |  |  |  | RBS-2583 |  |
| - BLER Quality value |  |  |  |  | -20 (-2.0) |  |  | RBS-2584 |  |
| Added or Reconfigured DL TrCH information |  |  | A2, A8 |  | 4 TrCHs(DCH for DCCH and 3DCHs for DTCH) |  |  | RBS-2585 |  |
| - Downlink transport channel type |  |  |  |  | DCH |  |  | RBS-2586 |  |
| - DL Transport channel identity |  |  |  |  | 10 |  |  | RBS-2587 |  |
| - CHOICE DL parameters |  |  |  |  | Same as UL |  |  | RBS-2588 |  |
| - Uplink transport channel type |  |  |  |  | DCH |  |  | RBS-2589 |  |
| - UL TrCH identity |  |  |  |  | 5 |  |  | RBS-2590 |  |
| - DCH quality target |  |  |  |  |  |  |  | RBS-2591 |  |
| - BLER Quality value |  |  |  |  | -20 (-2.0) |  |  | RBS-2592 |  |
| - Downlink transport channel type |  |  |  |  | DCH |  |  | RBS-2593 |  |
| - DL Transport channel identity |  |  |  |  | 6 |  |  | RBS-2594 |  |
| - CHOICE DL parameters |  |  |  |  | Explicit |  |  | RBS-2595 |  |
| - TFS |  |  |  |  |  |  |  | RBS-2596 |  |
| - CHOICE Transport channel type |  |  |  |  | Dedicated transport channel |  |  | RBS-2597 |  |
| - Dynamic transport format information |  |  |  |  |  |  |  | RBS-2598 |  |
| - RLC Size |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2599 |  |
| - Number of TBs and TTI List |  |  |  |  | (This IE is repeated for TFI number.) |  |  | RBS-2600 |  |
| - Dynamic transport format information |  |  |  |  |  |  |  | RBS-2601 |  |
| - Transmission Time Interval |  |  |  |  | Not Present |  |  | RBS-2602 |  |
| - Number of Transport blocks |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2603 |  |
| - CHOICE Logical channel list |  |  |  |  | All |  |  | RBS-2604 |  |
| - Semi-static Transport Format information |  |  |  |  |  |  |  | RBS-2605 |  |
| - Transmission time interval |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2606 |  |
| - Type of channel coding |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2607 |  |
| - Coding Rate |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2608 |  |
| - Rate matching attribute |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2609 |  |
| - CRC size |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2610 |  |
| - DCH quality target |  |  |  |  |  |  |  | RBS-2611 |  |
| - BLER Quality value |  |  |  |  | Not Present |  |  | RBS-2612 |  |
| - Downlink transport channel type |  |  |  |  | DCH |  |  | RBS-2613 |  |
| - DL Transport channel identity |  |  |  |  | 7 |  |  | RBS-2614 |  |
| - CHOICE DL parameters |  |  |  |  | Explicit |  |  | RBS-2615 |  |
| - TFS |  |  |  |  |  |  |  | RBS-2616 |  |
| - CHOICE Transport channel type |  |  |  |  | Dedicated transport channel |  |  | RBS-2617 |  |
| - Dynamic transport format information |  |  |  |  |  |  |  | RBS-2618 |  |
| - RLC Size |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2619 |  |
| - Number of TBs and TTI List |  |  |  |  | (This IE is repeated for TFI number.) |  |  | RBS-2620 |  |
| - Dynamic transport format information |  |  |  |  |  |  |  | RBS-2621 |  |
| - Transmission Time Interval |  |  |  |  | Not Present |  |  | RBS-2622 |  |
| - Number of Transport blocks |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2623 |  |
| - CHOICE Logical channel list |  |  |  |  | All |  |  | RBS-2624 |  |
| - Semi-static Transport Format information |  |  |  |  |  |  |  | RBS-2625 |  |
| - Transmission time interval |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2626 |  |
| - Type of channel coding |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2627 |  |
| - Coding Rate |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2628 |  |
| - Rate matching attribute |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2629 |  |
| - CRC size |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2630 |  |
| - DCH quality target |  |  |  |  |  |  |  | RBS-2631 |  |
| - BLER Quality value |  |  |  |  | Not Present |  |  | RBS-2632 |  |
| - Downlink transport channel type |  |  |  |  | DCH |  |  | RBS-2633 |  |
| - DL Transport channel identity |  |  |  |  | 8 |  |  | RBS-2634 |  |
| - CHOICE DL parameters |  |  |  |  | Explicit |  |  | RBS-2635 |  |
| - TFS |  |  |  |  |  |  |  | RBS-2636 |  |
| - CHOICE Transport channel type |  |  |  |  | Dedicated transport channel |  |  | RBS-2637 |  |
| - Dynamic transport format information |  |  |  |  |  |  |  | RBS-2638 |  |
| - RLC Size |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2639 |  |
| - Number of TBs and TTI List |  |  |  |  | (This IE is repeated for TFI number.) |  |  | RBS-2640 |  |
| - Dynamic transport format information |  |  |  |  |  |  |  | RBS-2641 |  |
| - Transmission Time Interval |  |  |  |  | Not Present |  |  | RBS-2642 |  |
| - Number of Transport blocks |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2643 |  |
| - CHOICE Logical channel list |  |  |  |  | All |  |  | RBS-2644 |  |
| - Semi-static Transport Format information |  |  |  |  |  |  |  | RBS-2645 |  |
| - Transmission time interval |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2646 |  |
| - Type of channel coding |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2647 |  |
| - Coding Rate |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2648 |  |
| - Rate matching attribute |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2649 |  |
| - CRC size |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2650 |  |
| - DCH quality target |  |  |  |  |  |  |  | RBS-2651 |  |
| - BLER Quality value |  |  |  |  | Not Present |  |  | RBS-2652 |  |
| Added or Reconfigured DL TrCH information |  |  | A9 |  | 3 TrCHs (DCH for DCCH and DCH plus HS-DSCH for DTCH) | Rel-5 |  | RBS-2653 |  |
|  |  |  | A12 |  |  | Rel-6 |  | RBS-2654 |  |
|  |  |  | A19, A19a |  |  | Rel-7 |  | RBS-2655 |  |
| - Downlink transport channel type |  |  |  |  | DCH |  |  | RBS-2656 |  |
| - DL Transport channel identity |  |  |  |  | 10 |  |  | RBS-2657 |  |
| - CHOICE DL parameters |  |  |  |  | Same as UL |  |  | RBS-2658 |  |
| - Uplink transport channel type |  |  |  |  | DCH |  |  | RBS-2659 |  |
| - UL TrCH identity |  |  |  |  | 5 |  |  | RBS-2660 |  |
| - DCH quality target |  |  |  |  |  |  |  | RBS-2661 |  |
| - BLER Quality value |  |  |  |  | -20 (-2.0) |  |  | RBS-2662 |  |
| - Downlink transport channel type |  |  |  |  | DCH |  |  | RBS-2663 |  |
| - DL Transport channel identity |  |  |  |  | 6 |  |  | RBS-2664 |  |
| - CHOICE DL parameters |  |  |  |  | Explicit |  |  | RBS-2665 |  |
| - TFS |  |  |  |  |  |  |  | RBS-2666 |  |
| - CHOICE Transport channel type |  |  |  |  | Dedicated transport channel |  |  | RBS-2667 |  |
| - Dynamic transport format information |  |  |  |  |  |  |  | RBS-2668 |  |
| - RLC Size |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2669 |  |
| - Number of TBs and TTI List |  |  |  |  | (This IE is repeated for TFI number.) |  |  | RBS-2670 |  |
| - Dynamic transport format information |  |  |  |  |  |  |  | RBS-2671 |  |
| - Transmission Time Interval |  |  |  |  | Not Present |  |  | RBS-2672 |  |
| - Number of Transport blocks |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2673 |  |
| - CHOICE Logical channel list |  |  |  |  | All |  |  | RBS-2674 |  |
| - Semi-static Transport Format information |  |  |  |  |  |  |  | RBS-2675 |  |
| - Transmission time interval |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2676 |  |
| - Type of channel coding |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2677 |  |
| - Coding Rate |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2678 |  |
| - Rate matching attribute |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2679 |  |
| - CRC size |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2680 |  |
| - DCH quality target |  |  |  |  |  |  |  | RBS-2681 |  |
| - BLER Quality value |  |  |  |  | -20 (-2.0) |  |  | RBS-2682 |  |
| - Downlink transport channel type |  |  |  |  | HS-DSCH |  |  | RBS-2683 |  |
| - DL Transport channel identity |  |  |  |  | Not Present |  |  | RBS-2684 |  |
| - CHOICE DL parameters |  |  |  |  | HS-DSCH |  |  | RBS-2685 |  |
| - HARQ Info |  |  |  |  |  |  |  | RBS-2686 |  |
| - Number of Processes |  |  |  |  | Reference to clause 6.10.2.4.5 Parameter Set |  |  | RBS-2687 |  |
| - CHOICE Memory Partitioning |  |  |  |  | Implicit |  |  | RBS-2688 |  |
| - Added or reconfigured MAC-d flow |  |  |  |  |  |  |  | RBS-2689 |  |
| - MAC-hs queue to add or reconfigure list |  |  |  |  | (one queue) |  |  | RBS-2690 |  |
| - MAC-hs queue Id |  |  |  |  | 0 |  |  | RBS-2691 |  |
| - MAC-d Flow Identity |  |  |  |  | 0 |  |  | RBS-2692 |  |
| - T1 |  |  |  |  | 50 |  |  | RBS-2693 |  |
| - MAC-hs window size |  |  |  |  | 16 |  |  | RBS-2694 |  |
| - MAC-d PDU size Info |  |  |  |  |  |  |  | RBS-2695 |  |
| - MAC-d PDU size |  |  |  |  | 336 |  |  | RBS-2696 |  |
| - MAC-d PDU size index |  |  |  |  | 0 |  |  | RBS-2697 |  |
| - MAC-hs queue to delete list |  |  |  |  | Not present |  |  | RBS-2698 |  |
| - DCH quality target |  |  |  |  | Not present |  |  | RBS-2699 |  |
| Added or Reconfigured DL TrCH information |  |  | A10 |  | 2 TrCHs (DCH for DCCH and HS-DSCH for DTCH) | Rel-5 |  | RBS-2700 |  |
| - Downlink transport channel type |  |  |  |  | DCH |  |  | RBS-2701 |  |
| - DL Transport channel identity |  |  |  |  | 10 |  |  | RBS-2702 |  |
| - CHOICE DL parameters |  |  |  |  | Same as UL |  |  | RBS-2703 |  |
| - Uplink transport channel type |  |  |  |  | DCH |  |  | RBS-2704 |  |
| - UL TrCH identity |  |  |  |  | 5 |  |  | RBS-2705 |  |
| - DCH quality target |  |  |  |  |  |  |  | RBS-2706 |  |
| - BLER Quality value |  |  |  |  | -20 (-2.0) |  |  | RBS-2707 |  |
| - Downlink transport channel type |  |  |  |  | HS-DSCH |  |  | RBS-2708 |  |
| - DL Transport channel identity |  |  |  |  | Not Present |  |  | RBS-2709 |  |
| - CHOICE DL parameters |  |  |  |  | HS-DSCH |  |  | RBS-2710 |  |
| - HARQ Info |  |  |  |  |  |  |  | RBS-2711 |  |
| - Number of Processes |  |  |  |  | Reference to clause 6.10.2.4.5 Parameter Set |  |  | RBS-2712 |  |
| - CHOICE Memory Partitioning |  |  |  |  | Implicit |  |  | RBS-2713 |  |
| - Added or reconfigured MAC-d flow |  |  |  |  |  |  |  | RBS-2714 |  |
| - MAC-hs queue to add or reconfigure list |  |  |  |  | (one queue) |  |  | RBS-2715 |  |
| - MAC-hs queue Id |  |  |  |  | 0 |  |  | RBS-2716 |  |
| - MAC-d Flow Identity |  |  |  |  | 0 |  |  | RBS-2717 |  |
| - T1 |  |  |  |  | 50 |  |  | RBS-2718 |  |
| - MAC-hs window size |  |  |  |  | 16 |  |  | RBS-2719 |  |
| - MAC-d PDU size Info |  |  |  |  |  |  |  | RBS-2720 |  |
| - MAC-d PDU size |  |  |  |  | 336 |  |  | RBS-2721 |  |
| - MAC-d PDU size index |  |  |  |  | 0 |  |  | RBS-2722 |  |
| - MAC-hs queue to delete list |  |  |  |  | Not present |  |  | RBS-2723 |  |
| - DCH quality target |  |  |  |  | Not present |  |  | RBS-2724 |  |
| Added or Reconfigured DL TrCH information |  |  | A11 |  | 1 DCH for DTCH |  |  | RBS-2725 |  |
| - Downlink transport channel type |  |  |  |  | DCH |  |  | RBS-2726 |  |
| - DL Transport channel identity |  |  |  |  | 9 |  |  | RBS-2727 |  |
| - CHOICE DL parameters |  |  |  |  | Explicit |  |  | RBS-2728 |  |
| - TFS |  |  |  |  |  |  |  | RBS-2729 |  |
| - CHOICE Transport channel type |  |  |  |  | Dedicated transport channel |  |  | RBS-2730 |  |
| - Dynamic transport format information |  |  |  |  |  |  |  | RBS-2731 |  |
| - RLC Size |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2732 |  |
| - Number of TBs and TTI List |  |  |  |  | (This IE is repeated for TFI number.) |  |  | RBS-2733 |  |
| - Dynamic transport format information |  |  |  |  |  |  |  | RBS-2734 |  |
| - Transmission Time Interval |  |  |  |  | Not Present |  |  | RBS-2735 |  |
| - Number of Transport blocks |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2736 |  |
| - CHOICE Logical channel list |  |  |  |  | All |  |  | RBS-2737 |  |
| - Semi-static Transport Format information |  |  |  |  |  |  |  | RBS-2738 |  |
| - Transmission time interval |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2739 |  |
| - Type of channel coding |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2740 |  |
| - Coding Rate |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2741 |  |
| - Rate matching attribute |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2742 |  |
| - CRC size |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2743 |  |
| - DCH quality target |  |  |  |  |  |  |  | RBS-2744 |  |
| - BLER Quality value |  |  |  |  | -20 (-2.0) |  |  | RBS-2745 |  |
| Added or Reconfigured DL TrCH information |  |  | A13 |  | 2 TrCHs (DCH for DCCH and HS-DSCH for DTCH) | Rel-6 |  | RBS-2746 |  |
| - Downlink transport channel type |  |  |  |  | DCH |  |  | RBS-2747 |  |
| - DL Transport channel identity |  |  |  |  | 10 |  |  | RBS-2748 |  |
| - CHOICE DL parameters |  |  |  |  | Explicit |  |  | RBS-2749 |  |
| - TFS |  |  |  |  |  |  |  | RBS-2750 |  |
| - CHOICE Transport channel type |  |  |  |  | Dedicated transport channels |  |  | RBS-2751 |  |
| - Dynamic Transport format information |  |  |  |  |  |  |  | RBS-2752 |  |
| - RLC Size |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2753 |  |
| - Number of TBs and TTI List |  |  |  |  | (This IE is repeated for TFI number.) |  |  | RBS-2754 |  |
| - Transmission Time Interval |  |  |  |  | Not Present |  |  | RBS-2755 |  |
| - Number of Transport blocks |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2756 |  |
| - CHOICE Logical channel list |  |  |  |  | All |  |  | RBS-2757 |  |
| - Semi-static Transport Format information |  |  |  |  |  |  |  | RBS-2758 |  |
| - Transmission time interval |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2759 |  |
| - Type of channel coding |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2760 |  |
| - Coding Rate |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2761 |  |
| - Rate matching attribute |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2762 |  |
| - CRC size |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2763 |  |
| - DCH quality target |  |  |  |  |  |  |  | RBS-2764 |  |
| - BLER Quality value |  |  |  |  | -20 (-2.0) |  |  | RBS-2765 |  |
| - Downlink transport channel type |  |  |  |  | HS-DSCH |  |  | RBS-2766 |  |
| - DL Transport channel identity |  |  |  |  | Not Present |  |  | RBS-2767 |  |
| - CHOICE DL parameters |  |  |  |  | HS-DSCH |  |  | RBS-2768 |  |
| - HARQ Info |  |  |  |  |  |  |  | RBS-2769 |  |
| - Number of Processes |  |  |  |  | Reference to clause 6.10.2.4.5 Parameter Set |  |  | RBS-2770 |  |
| - CHOICE Memory Partitioning |  |  |  |  | Implicit |  |  | RBS-2771 |  |
| - Added or reconfigured MAC-d flow |  |  |  |  |  |  |  | RBS-2772 |  |
| - MAC-hs queue to add or reconfigure list |  |  |  |  | (one queue) |  |  | RBS-2773 |  |
| - MAC-hs queue Id |  |  |  |  | 0 |  |  | RBS-2774 |  |
| - MAC-d Flow Identity |  |  |  |  | 0 |  |  | RBS-2775 |  |
| - T1 |  |  |  |  | 50 |  |  | RBS-2776 |  |
| - MAC-hs window size |  |  |  |  | 16 |  |  | RBS-2777 |  |
| - MAC-d PDU size Info |  |  |  |  |  |  |  | RBS-2778 |  |
| - MAC-d PDU size |  |  |  |  | 336 |  |  | RBS-2779 |  |
| - MAC-d PDU size index |  |  |  |  | 0 |  |  | RBS-2780 |  |
| - MAC-hs queue to delete list |  |  |  |  | Not present |  |  | RBS-2781 |  |
| - DCH quality target |  |  |  |  | Not present |  |  | RBS-2782 |  |
| Added or Reconfigured DL TrCH information |  |  | A14 |  | 1 TrCH (HS-DSCH for DTCH and DCCH) | Rel-6 |  | RBS-2783 |  |
|  |  |  | , A20 |  |  | Rel-7 |  | RBS-2784 |  |
| - Downlink transport channel type |  |  |  |  | HS-DSCH |  |  | RBS-2785 |  |
| - DL Transport channel identity |  |  |  |  | Not Present |  |  | RBS-2786 |  |
| - CHOICE DL parameters |  |  |  |  | HS-DSCH |  |  | RBS-2787 |  |
| - HARQ Info |  |  |  |  |  |  |  | RBS-2788 |  |
| - Number of Processes |  |  |  |  | Reference to clause 6.10.2.4.5 Parameter Set |  |  | RBS-2789 |  |
| - CHOICE Memory Partitioning |  |  |  |  | Implicit |  |  | RBS-2790 |  |
| - Added or reconfigured MAC-d flow |  |  |  |  |  |  |  | RBS-2791 |  |
| - MAC-hs queue to add or reconfigure list |  |  |  |  | (two queues) |  |  | RBS-2792 |  |
| - MAC-hs queue Id |  |  |  |  | 0 (for DTCH) |  |  | RBS-2793 |  |
| - MAC-d Flow Identity |  |  |  |  | 0 |  |  | RBS-2794 |  |
| - T1 |  |  |  |  | 50 |  |  | RBS-2795 |  |
| - MAC-hs window size |  |  |  |  | 16 |  |  | RBS-2796 |  |
| - MAC-d PDU size Info |  |  |  |  |  |  |  | RBS-2797 |  |
| - MAC-d PDU size |  |  |  |  | 336 |  |  | RBS-2798 |  |
| - MAC-d PDU size index |  |  |  |  | 0 |  |  | RBS-2799 |  |
| - MAC-hs queue Id |  |  |  |  | 1 (for DCCH) |  |  | RBS-2800 |  |
| - MAC-d Flow Identity |  |  |  |  | 1 |  |  | RBS-2801 |  |
| - T1 |  |  |  |  | 50 |  |  | RBS-2802 |  |
| - MAC-hs window size |  |  |  |  | 16 |  |  | RBS-2803 |  |
| - MAC-d PDU size Info |  |  |  |  |  |  |  | RBS-2804 |  |
| - MAC-d PDU size |  |  |  |  | 148 |  |  | RBS-2805 |  |
| - MAC-d PDU size index |  |  |  |  | 0 |  |  | RBS-2806 |  |
| - MAC-hs queue to delete list |  |  |  |  | Not present |  |  | RBS-2807 |  |
| - DCH quality target |  |  |  |  | Not present |  |  | RBS-2808 |  |
| Added or Reconfigured DL TrCH information |  |  | A15 |  | 2 TrCHs (DCH for DCCH and HS-DSCH for DTCH) | Rel-6 |  | RBS-2809 |  |
| - Downlink transport channel type |  |  |  |  | DCH |  |  | RBS-2810 |  |
| - DL Transport channel identity |  |  |  |  | 10 |  |  | RBS-2811 |  |
| - CHOICE DL parameters |  |  |  |  | Explicit |  |  | RBS-2812 |  |
| - TFS |  |  |  |  |  |  |  | RBS-2813 |  |
| - CHOICE Transport channel type |  |  |  |  | Dedicated transport channels |  |  | RBS-2814 |  |
| - Dynamic Transport format information |  |  |  |  |  |  |  | RBS-2815 |  |
| - RLC Size |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2816 |  |
| - Number of TBs and TTI List |  |  |  |  | (This IE is repeated for TFI number.) |  |  | RBS-2817 |  |
| - Transmission Time Interval |  |  |  |  | Not Present |  |  | RBS-2818 |  |
| - Number of Transport blocks |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2819 |  |
| - CHOICE Logical channel list |  |  |  |  | All |  |  | RBS-2820 |  |
| - Semi-static Transport Format information |  |  |  |  |  |  |  | RBS-2821 |  |
| - Transmission time interval |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2822 |  |
| - Type of channel coding |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2823 |  |
| - Coding Rate |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2824 |  |
| - Rate matching attribute |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2825 |  |
| - CRC size |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2826 |  |
| - DCH quality target |  |  |  |  |  |  |  | RBS-2827 |  |
| - BLER Quality value |  |  |  |  | -20 (-2.0) |  |  | RBS-2828 |  |
| - Downlink transport channel type |  |  |  |  | HS-DSCH |  |  | RBS-2829 |  |
| - DL Transport channel identity |  |  |  |  | Not Present |  |  | RBS-2830 |  |
| - CHOICE DL parameters |  |  |  |  | HS-DSCH |  |  | RBS-2831 |  |
| - HARQ Info |  |  |  |  |  |  |  | RBS-2832 |  |
| - Number of Processes |  |  |  |  | Reference to clause 6.10.2.4.5 Parameter Set |  |  | RBS-2833 |  |
| - CHOICE Memory Partitioning |  |  |  |  | Implicit |  |  | RBS-2834 |  |
| - Added or reconfigured MAC-d flow |  |  |  |  |  |  |  | RBS-2835 |  |
| - MAC-hs queue to add or reconfigure list |  |  |  |  | (two queues) |  |  | RBS-2836 |  |
| - MAC-hs queue Id |  |  |  |  | 0 (for first DTCH) |  |  | RBS-2837 |  |
| - MAC-d Flow Identity |  |  |  |  | 0 |  |  | RBS-2838 |  |
| - T1 |  |  |  |  | 50 |  |  | RBS-2839 |  |
| - MAC-hs window size |  |  |  |  | 16 |  |  | RBS-2840 |  |
| - MAC-d PDU size Info |  |  |  |  |  |  |  | RBS-2841 |  |
| - MAC-d PDU size |  |  |  |  | 336 |  |  | RBS-2842 |  |
| - MAC-d PDU size index |  |  |  |  | 0 |  |  | RBS-2843 |  |
| - MAC-hs queue Id |  |  |  |  | 2 (for second DTCH) |  |  | RBS-2844 |  |
| - MAC-d Flow Identity |  |  |  |  | 2 |  |  | RBS-2845 |  |
| - T1 |  |  |  |  | 50 |  |  | RBS-2846 |  |
| - MAC-hs window size |  |  |  |  | 16 |  |  | RBS-2847 |  |
| - MAC-d PDU size Info |  |  |  |  |  |  |  | RBS-2848 |  |
| - MAC-d PDU size |  |  |  |  | 336 |  |  | RBS-2849 |  |
| - MAC-d PDU size index |  |  |  |  | 0 |  |  | RBS-2850 |  |
| - MAC-hs queue to delete list |  |  |  |  | Not present |  |  | RBS-2851 |  |
| - DCH quality target |  |  |  |  | Not present |  |  | RBS-2852 |  |
| Added or Reconfigured DL TrCH information |  |  | A16 |  | 1 TrCH (HS-DSCH for 2 DTCHs and DCCH) | Rel-6 |  | RBS-2853 |  |
|  |  |  | , A21 |  |  | Rel-7 |  | RBS-2854 |  |
| - Downlink transport channel type |  |  |  |  | HS-DSCH |  |  | RBS-2855 |  |
| - DL Transport channel identity |  |  |  |  | Not Present |  |  | RBS-2856 |  |
| - CHOICE DL parameters |  |  |  |  | HS-DSCH |  |  | RBS-2857 |  |
| - HARQ Info |  |  |  |  |  |  |  | RBS-2858 |  |
| - Number of Processes |  |  |  |  | Reference to clause 6.10.2.4.5 Parameter Set |  |  | RBS-2859 |  |
| - CHOICE Memory Partitioning |  |  |  |  | Implicit |  |  | RBS-2860 |  |
| - Added or reconfigured MAC-d flow |  |  |  |  |  |  |  | RBS-2861 |  |
| - MAC-hs queue to add or reconfigure list |  |  |  |  | (three queues) |  |  | RBS-2862 |  |
| - MAC-hs queue Id |  |  |  |  | 0 (for first DTCH) |  |  | RBS-2863 |  |
| - MAC-d Flow Identity |  |  |  |  | 0 |  |  | RBS-2864 |  |
| - T1 |  |  |  |  | 50 |  |  | RBS-2865 |  |
| - MAC-hs window size |  |  |  |  | 16 |  |  | RBS-2866 |  |
| - MAC-d PDU size Info |  |  |  |  |  |  |  | RBS-2867 |  |
| - MAC-d PDU size |  |  |  |  | 336 |  |  | RBS-2868 |  |
| - MAC-d PDU size index |  |  |  |  | 0 |  |  | RBS-2869 |  |
| - MAC-hs queue Id |  |  |  |  | 1 (for DCCH) |  |  | RBS-2870 |  |
| - MAC-d Flow Identity |  |  |  |  | 1 |  |  | RBS-2871 |  |
| - T1 |  |  |  |  | 50 |  |  | RBS-2872 |  |
| - MAC-hs window size |  |  |  |  | 16 |  |  | RBS-2873 |  |
| - MAC-d PDU size Info |  |  |  |  |  |  |  | RBS-2874 |  |
| - MAC-d PDU size |  |  |  |  | 148 |  |  | RBS-2875 |  |
| - MAC-d PDU size index |  |  |  |  | 0 |  |  | RBS-2876 |  |
| - MAC-hs queue Id |  |  |  |  | 3 (for second DTCH) |  |  | RBS-2877 |  |
| - MAC-d Flow Identity |  |  |  |  | 3 |  |  | RBS-2878 |  |
| - T1 |  |  |  |  | 50 |  |  | RBS-2879 |  |
| - MAC-hs window size |  |  |  |  | 16 |  |  | RBS-2880 |  |
| - MAC-d PDU size Info |  |  |  |  |  |  |  | RBS-2881 |  |
| - MAC-d PDU size |  |  |  |  | 112 |  |  | RBS-2882 |  |
| - MAC-d PDU size index |  |  |  |  | 0 |  |  | RBS-2883 |  |
| - MAC-d PDU size |  |  |  |  | 144 |  |  | RBS-2884 |  |
| - MAC-d PDU size index |  |  |  |  | 1 |  |  | RBS-2885 |  |
| - MAC-d PDU size |  |  |  |  | 160 |  |  | RBS-2886 |  |
| - MAC-d PDU size index |  |  |  |  | 2 |  |  | RBS-2887 |  |
| - MAC-d PDU size |  |  |  |  | 176 |  |  | RBS-2888 |  |
| - MAC-d PDU size index |  |  |  |  | 3 |  |  | RBS-2889 |  |
| - MAC-d PDU size |  |  |  |  | 192 |  |  | RBS-2890 |  |
| - MAC-d PDU size index |  |  |  |  | 4 |  |  | RBS-2891 |  |
| - MAC-d PDU size |  |  |  |  | 224 |  |  | RBS-2892 |  |
| - MAC-d PDU size index |  |  |  |  | 5 |  |  | RBS-2893 |  |
| - MAC-d PDU size |  |  |  |  | 296 |  |  | RBS-2894 |  |
| - MAC-d PDU size index |  |  |  |  | 6 |  |  | RBS-2895 |  |
| - MAC-d PDU size |  |  |  |  | 344 |  |  | RBS-2896 |  |
| - MAC-d PDU size index |  |  |  |  | 7 |  |  | RBS-2897 |  |
| - MAC-hs queue to delete list |  |  |  |  | Not present |  |  | RBS-2898 |  |
| - DCH quality target |  |  |  |  | Not present |  |  | RBS-2899 |  |
| Added or Reconfigured DL TrCH information |  |  | A17, A17a, A17d, A17e, A18 |  | 2 TrCHs (DCH for DCCH and HS-DSCH for DTCH) | Rel-7 |  | RBS-2900 |  |
|  |  |  | A25a |  |  | Rel-8 |  | RBS-2901 |  |
| - Downlink transport channel type |  |  |  |  | DCH |  |  | RBS-2902 |  |
| - DL Transport channel identity |  |  |  |  | 10 |  |  | RBS-2903 |  |
| - CHOICE DL parameters |  |  |  |  | Same as UL |  |  | RBS-2904 |  |
| - Uplink transport channel type |  |  |  |  | DCH |  |  | RBS-2905 |  |
| - UL TrCH identity |  |  |  |  | 5 |  |  | RBS-2906 |  |
| - DCH quality target |  |  |  |  |  |  |  | RBS-2907 |  |
| - BLER Quality value |  |  |  |  | -20 (-2.0) |  |  | RBS-2908 |  |
| - Downlink transport channel type |  |  |  |  | HS-DSCH |  |  | RBS-2909 |  |
| - DL Transport channel identity |  |  |  |  | Not Present |  |  | RBS-2910 |  |
| - CHOICE DL parameters |  |  |  |  | HS-DSCH |  |  | RBS-2911 |  |
| - HARQ Info |  |  |  |  |  |  |  | RBS-2912 |  |
| - Number of Processes |  |  |  |  | Reference to clause 6.10.2.4.5 Parameter Set |  |  | RBS-2913 |  |
| - CHOICE Memory Partitioning |  |  |  |  | Implicit |  |  | RBS-2914 |  |
| - CHOICE DL MAC header type |  |  |  |  | MAC-ehs |  |  | RBS-2915 |  |
| - Added or reconfigured MAC-ehs reordering queue |  |  |  |  |  |  |  | RBS-2916 |  |
| - MAC-ehs queue to add or reconfigure list |  |  |  |  | (one queue) |  |  | RBS-2917 |  |
| - MAC-ehs queue Id |  |  |  |  | 0 |  |  | RBS-2918 |  |
| - T1 |  |  |  |  | 50 |  |  | RBS-2919 |  |
| - MAC-ehs window size |  |  |  |  | 16 |  |  | RBS-2920 |  |
| - MAC-ehs queue to delete list |  |  |  |  | Not present |  |  | RBS-2921 |  |
| - DCH quality target |  |  |  |  | Not present |  |  | RBS-2922 |  |
| Added or Reconfigured DL TrCH information |  |  | A17b, A17c |  | 1 TrCH (HS-DSCH for DTCH and DCCH) |  |  | RBS-2923 |  |
| - Downlink transport channel type |  |  |  |  | HS-DSCH |  |  | RBS-2924 |  |
| - DL Transport channel identity |  |  |  |  | Not Present |  |  | RBS-2925 |  |
| - CHOICE DL parameters |  |  |  |  | HS-DSCH |  |  | RBS-2926 |  |
| - HARQ Info |  |  |  |  |  |  |  | RBS-2927 |  |
| - Number of Processes |  |  |  |  | Reference to clause 6.10.2.4.5 Parameter Set |  |  | RBS-2928 |  |
| - CHOICE Memory Partitioning |  |  |  |  | Implicit |  |  | RBS-2929 |  |
| - CHOICE DL MAC header type |  |  |  |  | MAC-ehs |  |  | RBS-2930 |  |
| - Added or reconfigured MAC-ehs reordering queue |  |  |  |  |  |  |  | RBS-2931 |  |
| - MAC-hs queue to add or reconfigure list |  |  |  |  | (two queues) |  |  | RBS-2932 |  |
| - MAC-ehs queue Id |  |  |  |  | 0 (for DTCH) |  |  | RBS-2933 |  |
| - T1 |  |  |  |  | 50 |  |  | RBS-2934 |  |
| - MAC-ehs window size |  |  |  |  | 16 |  |  | RBS-2935 |  |
| - MAC-ehs queue Id |  |  |  |  | 1 (for DCCH) |  |  | RBS-2936 |  |
| - T1 |  |  |  |  | 50 |  |  | RBS-2937 |  |
| - MAC-hs window size |  |  |  |  | 16 |  |  | RBS-2938 |  |
| - MAC-ehs queue to delete list |  |  |  |  | Not present |  |  | RBS-2939 |  |
| - DCH quality target |  |  |  |  | Not present |  |  | RBS-2940 |  |
| Added or Reconfigured DL TrCH information |  |  | A22 |  | 1 TrCH (HS-DSCH for 2 DTCHs and DCCH) | Rel-7 |  | RBS-2941 |  |
| - Downlink transport channel type |  |  |  |  | HS-DSCH |  |  | RBS-2942 |  |
| - DL Transport channel identity |  |  |  |  | Not Present |  |  | RBS-2943 |  |
| - CHOICE DL parameters |  |  |  |  | HS-DSCH |  |  | RBS-2944 |  |
| - HARQ Info |  |  |  |  |  |  |  | RBS-2945 |  |
| - Number of Processes |  |  |  |  | Reference to clause 6.10.2.4.5 Parameter Set |  |  | RBS-2946 |  |
| - CHOICE Memory Partitioning |  |  |  |  | Implicit |  |  | RBS-2947 |  |
| - CHOICE DL MAC header type |  |  |  |  | MAC-ehs |  |  | RBS-2948 |  |
| - Added or reconfigured MAC-ehs reordering queue |  |  |  |  |  |  |  | RBS-2949 |  |
| - MAC-ehs queue to add or reconfigure list |  |  |  |  | (three queues) |  |  | RBS-2950 |  |
| - MAC-ehs queue Id |  |  |  |  | 0 (for first DTCH) |  |  | RBS-2951 |  |
| - T1 |  |  |  |  | 50 |  |  | RBS-2952 |  |
| - MAC-ehs window size |  |  |  |  | 16 |  |  | RBS-2953 |  |
| - MAC-ehs queue Id |  |  |  |  | 1 (for DCCH) |  |  | RBS-2954 |  |
| - T1 |  |  |  |  | 50 |  |  | RBS-2955 |  |
| - MAC-ehs window size |  |  |  |  | 16 |  |  | RBS-2956 |  |
| - MAC-ehs queue Id |  |  |  |  | 3 (for second DTCH) |  |  | RBS-2957 |  |
| - T1 |  |  |  |  | 50 |  |  | RBS-2958 |  |
| - MAC-ehs window size |  |  |  |  | 16 |  |  | RBS-2959 |  |
| - MAC-ehs queue to delete list |  |  |  |  | Not present |  |  | RBS-2960 |  |
| - DCH quality target |  |  |  |  | Not present |  |  | RBS-2961 |  |
| Added or Reconfigured DL TrCH information |  |  | A23 |  |  | Rel-7 Rel-8 |  | RBS-2962 |  |
| - Downlink transport channel type |  |  |  |  | HS-DSCH |  |  | RBS-2963 |  |
| - DL Transport channel identity |  |  |  |  | Not Present |  |  | RBS-2964 |  |
| - CHOICE DL parameters |  |  |  |  | HS-DSCH |  |  | RBS-2965 |  |
| - HARQ Info |  |  |  |  |  |  |  | RBS-2966 |  |
| - Number of Processes |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-2967 |  |
| - CHOICE Memory Partitioning |  |  |  |  | Implicit |  |  | RBS-2968 |  |
| - CHOICE DL MAC header type |  |  |  |  | MAC-ehs |  |  | RBS-2969 |  |
| - Added or reconfigured MAC-ehs reordering queue |  |  |  |  |  |  |  | RBS-2970 |  |
| - MAC-ehs queue to add or reconfigure list |  |  |  |  | (two queues) |  |  | RBS-2971 |  |
| - MAC-ehs queue Id |  |  |  |  | 0 (for first DTCH) |  |  | RBS-2972 |  |
| - T1 |  |  |  |  | 50 |  |  | RBS-2973 |  |
| - MAC-ehs window size |  |  |  |  | 16 |  |  | RBS-2974 |  |
| - MAC-ehs queue Id |  |  |  |  | 1 (for second DTCH) |  |  | RBS-2975 |  |
| - T1 |  |  |  |  | 50 |  |  | RBS-2976 |  |
| - MAC-ehs window size |  |  |  |  | 16 |  |  | RBS-2977 |  |
| - MAC-ehs queue to delete list |  |  |  |  | Not present |  |  | RBS-2978 |  |
| - DCH quality target |  |  |  |  | Not present |  |  | RBS-2979 |  |
| Added or Reconfigured DL TrCH information |  |  | A25 |  | 1 TrCH (HS-DSCH for DTCH and DCCH) | Rel-8 |  | RBS-2980 |  |
| - Downlink transport channel type |  |  |  |  | HS-DSCH |  |  | RBS-2981 |  |
| - DL Transport channel identity |  |  |  |  | Not Present |  |  | RBS-2982 |  |
| - CHOICE DL parameters |  |  |  |  | HS-DSCH |  |  | RBS-2983 |  |
| - HARQ Info |  |  |  |  |  |  |  | RBS-2984 |  |
| - Number of Processes |  |  |  |  | Reference to clause 6.10.2.4.5 Parameter Set |  |  | RBS-2985 |  |
| - CHOICE Memory Partitioning |  |  |  |  | Implicit |  |  | RBS-2986 |  |
| - CHOICE DL MAC header type |  |  |  |  | MAC-ehs |  |  | RBS-2987 |  |
| - Added or reconfigured MAC-ehs reordering queue |  |  |  |  |  |  |  | RBS-2988 |  |
| - MAC-ehs queue to add or reconfigure list |  |  |  |  | (two queues) |  |  | RBS-2989 |  |
| - MAC-ehs queue Id |  |  |  |  | 0 (for DTCH) |  |  | RBS-2990 |  |
| - T1 |  |  |  |  | 50 |  |  | RBS-2991 |  |
| - MAC-ehs window size |  |  |  |  | 16 |  |  | RBS-2992 |  |
| - MAC-ehs queue Id |  |  |  |  | 1 (for DCCH) |  |  | RBS-2993 |  |
| - T1 |  |  |  |  | 50 |  |  | RBS-2994 |  |
| - MAC-ehs window size |  |  |  |  | 16 |  |  | RBS-2995 |  |
| - MAC-ehs queue to delete list |  |  |  |  | Not present |  |  | RBS-2996 |  |
| - DCH quality target |  |  |  |  | Not present |  |  | RBS-2997 |  |
| Added or Reconfigured DL TrCH information |  |  | A28 |  | 2 TrCHs (DCH for DCCH and HS-DSCH for DTCH) | Rel-8 |  | RBS-2998 |  |
| - Downlink transport channel type |  |  |  |  | DCH |  |  | RBS-2999 |  |
| - DL Transport channel identity |  |  |  |  | 10 |  |  | RBS-3000 |  |
| - CHOICE DL parameters |  |  |  |  | Same as UL |  |  | RBS-3001 |  |
| - Uplink transport channel type |  |  |  |  | DCH |  |  | RBS-3002 |  |
| - UL TrCH identity |  |  |  |  | 5 |  |  | RBS-3003 |  |
| - DCH quality target |  |  |  |  |  |  |  | RBS-3004 |  |
| - BLER Quality value |  |  |  |  | -20 (-2.0) |  |  | RBS-3005 |  |
| - Downlink transport channel type |  |  |  |  | HS-DSCH |  |  | RBS-3006 |  |
| - DL Transport channel identity |  |  |  |  | Not Present |  |  | RBS-3007 |  |
| - CHOICE DL parameters |  |  |  |  | HS-DSCH |  |  | RBS-3008 |  |
| - HARQ Info |  |  |  |  |  |  |  | RBS-3009 |  |
| - Number of Processes |  |  |  |  | Reference to clause 6.10.2.4.5 Parameter Set |  |  | RBS-3010 |  |
| - CHOICE Memory Partitioning |  |  |  |  | Implicit |  |  | RBS-3011 |  |
| - CHOICE DL MAC header type |  |  |  |  | MAC-ehs |  |  | RBS-3012 |  |
| - Added or reconfigured MAC-ehs reordering queue |  |  |  |  |  |  |  | RBS-3013 |  |
| - MAC-ehs queue to add or reconfigure list |  |  |  |  | (one queue) |  |  | RBS-3014 |  |
| - MAC-ehs queue Id |  |  |  |  | 0 |  |  | RBS-3015 |  |
| - T1 |  |  |  |  | 50 |  |  | RBS-3016 |  |
| - MAC-ehs window size |  |  |  |  | 32 |  |  | RBS-3017 |  |
| - MAC-ehs queue to delete list |  |  |  |  | Not present |  |  | RBS-3018 |  |
| - DCH quality target |  |  |  |  | Not present |  |  | RBS-3019 |  |
| Added or Reconfigured DL TrCH information list |  |  | A24, A29 |  | 1 TrCH (HS-DSCH for DTCH) |  |  | RBS-3020 |  |
| - Downlink transport channel type |  |  |  |  | HS-DSCH |  |  | RBS-3021 |  |
| - DL Transport channel identity |  |  |  |  | Not Present |  |  | RBS-3022 |  |
| - CHOICE DL parameters |  |  |  |  | HS-DSCH |  |  | RBS-3023 |  |
| - HARQ Info |  |  |  |  |  |  |  | RBS-3024 |  |
| - Number of Processes |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-3025 |  |
| - CHOICE Memory Partitioning |  |  |  |  | Implicit |  |  | RBS-3026 |  |
| - CHOICE DL MAC header type |  |  |  |  | MAC-ehs |  |  | RBS-3027 |  |
| - Added or reconfigured MAC-ehs reordering queue |  |  |  |  |  |  |  | RBS-3028 |  |
| - MAC-ehs queue to add or reconfigure list |  |  |  |  | (one queue) |  |  | RBS-3029 |  |
| - MAC-ehs queue Id |  |  |  |  | 2 (for DTCH) |  |  | RBS-3030 |  |
| - T1 |  |  |  |  | 50 |  |  | RBS-3031 |  |
| - MAC-ehs window size |  |  |  |  | 16 |  |  | RBS-3032 |  |
| - DCH quality target |  |  |  |  | Not present |  |  | RBS-3033 |  |
| Added or Reconfigured DL TrCH information |  |  | A26 |  | DCH for DCCH and HS-DSCH for 3 DTCHs | Rel-8 |  | RBS-3034 |  |
| - Downlink transport channel type |  |  |  |  | DCH |  |  | RBS-3035 |  |
| - DL Transport channel identity |  |  |  |  | 10 |  |  | RBS-3036 |  |
| - CHOICE DL parameters |  |  |  |  | Explicit |  |  | RBS-3037 |  |
| - TFS |  |  |  |  |  |  |  | RBS-3038 |  |
| - CHOICE Transport channel type |  |  |  |  | Dedicated transport channels |  |  | RBS-3039 |  |
| - Dynamic Transport format information |  |  |  |  |  |  |  | RBS-3040 |  |
| - RLC Size |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-3041 |  |
| - Number of TBs and TTI List |  |  |  |  | (This IE is repeated for TFI number.) |  |  | RBS-3042 |  |
| - Transmission Time Interval |  |  |  |  | Not Present |  |  | RBS-3043 |  |
| - Number of Transport blocks |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-3044 |  |
| - CHOICE Logical channel list |  |  |  |  | All |  |  | RBS-3045 |  |
| - Semi-static Transport Format information |  |  |  |  |  |  |  | RBS-3046 |  |
| - Transmission time interval |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-3047 |  |
| - Type of channel coding |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-3048 |  |
| - Coding Rate |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-3049 |  |
| - Rate matching attribute |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-3050 |  |
| - CRC size |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-3051 |  |
| - DCH quality target |  |  |  |  |  |  |  | RBS-3052 |  |
| - BLER Quality value |  |  |  |  | -20 (-2.0) |  |  | RBS-3053 |  |
| - Downlink transport channel type |  |  |  |  | HS-DSCH |  |  | RBS-3054 |  |
| - DL Transport channel identity |  |  |  |  | Not Present |  |  | RBS-3055 |  |
| - CHOICE DL parameters |  |  |  |  | HS-DSCH |  |  | RBS-3056 |  |
| - HARQ Info |  |  |  |  |  |  |  | RBS-3057 |  |
| - Number of Processes |  |  |  |  | Reference to clause 6.10.2.4.5 Parameter Set |  |  | RBS-3058 |  |
| - CHOICE Memory Partitioning |  |  |  |  | Implicit |  |  | RBS-3059 |  |
| - CHOICE DL MAC header type |  |  |  |  | MAC-ehs |  |  | RBS-3060 |  |
| - Added or reconfigured MAC-ehs reordering queue |  |  |  |  |  |  |  | RBS-3061 |  |
| - MAC-ehs queue to add or reconfigure list |  |  |  |  | (three queues) |  |  | RBS-3062 |  |
|  |  |  |  |  |  |  |  | RBS-3063 |  |
| - MAC-ehs queue Id |  |  |  |  | 2 (for first DTCH) |  |  | RBS-3064 |  |
| - T1 |  |  |  |  | 50 |  |  | RBS-3065 |  |
| - MAC-ehs window size |  |  |  |  | 16 |  |  | RBS-3066 |  |
| - MAC-ehs queue Id |  |  |  |  | 3 (for second DTCH) |  |  | RBS-3067 |  |
| - T1 |  |  |  |  | 50 |  |  | RBS-3068 |  |
| - MAC-ehs window size |  |  |  |  | 16 |  |  | RBS-3069 |  |
| - MAC-ehs queue Id |  |  |  |  | 4 (for third DTCH) |  |  | RBS-3070 |  |
| - T1 |  |  |  |  | 50 |  |  | RBS-3071 |  |
| - MAC-ehs window size |  |  |  |  | 16 |  |  | RBS-3072 |  |
| - DCH quality target |  |  |  |  | Not present |  |  | RBS-3073 |  |
| Added or Reconfigured DL TrCH information |  |  | A27, A27a |  | HS-DSCH for 2 DTCHs and DCCH | Rel-8 |  | RBS-3074 |  |
| - Downlink transport channel type |  |  |  |  | HS-DSCH |  |  | RBS-3075 |  |
| - DL Transport channel identity |  |  |  |  | Not Present |  |  | RBS-3076 |  |
| - CHOICE DL parameters |  |  |  |  | HS-DSCH |  |  | RBS-3077 |  |
| - HARQ Info |  |  |  |  |  |  |  | RBS-3078 |  |
| - Number of Processes |  |  |  |  | Reference to clause 6.10.2.4.5 Parameter Set |  |  | RBS-3079 |  |
| - CHOICE Memory Partitioning |  |  |  |  | Implicit |  |  | RBS-3080 |  |
| - CHOICE DL MAC header type |  |  |  |  | MAC-ehs |  |  | RBS-3081 |  |
| - Added or reconfigured MAC-ehs reordering queue |  |  |  |  |  |  |  | RBS-3082 |  |
| - MAC-ehs queue to add or reconfigure list |  |  |  |  | (two queues) |  |  | RBS-3083 |  |
| - MAC-ehs queue Id |  |  |  |  | 0 (for first DTCH) |  |  | RBS-3084 |  |
| - T1 |  |  |  |  | 50 |  |  | RBS-3085 |  |
| - MAC-ehs window size |  |  |  |  | 16 |  |  | RBS-3086 |  |
| - MAC-ehs queue Id |  |  |  |  | 1 (for DCCH) |  |  | RBS-3087 |  |
| - T1 |  |  |  |  | 50 |  |  | RBS-3088 |  |
| - MAC-ehs window size |  |  |  |  | 16 |  |  | RBS-3089 |  |
| - DCH quality target |  |  |  |  | Not present |  |  | RBS-3090 |  |
| Added or Reconfigured DL TrCH information |  |  | A30 |  | 1 TrCH (HS-DSCH for DTCH and DCCH) | Rel-8 |  | RBS-3091 |  |
| - Downlink transport channel type |  |  |  |  | HS-DSCH |  |  | RBS-3092 |  |
| - DL Transport channel identity |  |  |  |  | Not Present |  |  | RBS-3093 |  |
| - CHOICE DL parameters |  |  |  |  | HS-DSCH |  |  | RBS-3094 |  |
| - HARQ Info |  |  |  |  |  |  |  | RBS-3095 |  |
| - Number of Processes |  |  |  |  | Reference to clause 6.10.2.4.5 Parameter Set |  |  | RBS-3096 |  |
| - CHOICE Memory Partitioning |  |  |  |  | Implicit |  |  | RBS-3097 |  |
| - CHOICE DL MAC header type |  |  |  |  | MAC-ehs |  |  | RBS-3098 |  |
| - Added or reconfigured MAC-ehs reordering queue |  |  |  |  |  |  |  | RBS-3099 |  |
| - MAC-hs queue to add or reconfigure list |  |  |  |  | (two queues) |  |  | RBS-3100 |  |
| - MAC-ehs queue Id |  |  |  |  | 2 (for DTCH) |  |  | RBS-3101 |  |
| - T1 |  |  |  |  | 50 |  |  | RBS-3102 |  |
| - MAC-ehs window size |  |  |  |  | 16 |  |  | RBS-3103 |  |
| - MAC-ehs queue Id |  |  |  |  | 3 (for DCCH) |  |  | RBS-3104 |  |
| - T1 |  |  |  |  | 50 |  |  | RBS-3105 |  |
| - MAC-hs window size |  |  |  |  | 16 |  |  | RBS-3106 |  |
| - MAC-ehs queue to delete list |  |  |  |  | Not present |  |  | RBS-3107 |  |
| - DCH quality target |  |  |  |  | Not present |  |  | RBS-3108 |  |
| Added or Reconfigured DL TrCH information list | A31, A32 A33, A34 |  |  |  | 2 TrCh (DCH for DCCH and HS-DSCH for DTCH) | Rel-9 Rel-10 |  | RBS-3109 |  |
| - Downlink transport channel type |  |  |  |  | DCH |  |  | RBS-3110 |  |
| - DL Transport channel identity |  |  |  |  | 10 |  |  | RBS-3111 |  |
| - CHOICE DL parameters |  |  |  |  | Same as UL |  |  | RBS-3112 |  |
| - Uplink transport channel type |  |  |  |  | DCH |  |  | RBS-3113 |  |
| - UL TrCH identity |  |  |  |  | 5 |  |  | RBS-3114 |  |
| - DCH quality target |  |  |  |  |  |  |  | RBS-3115 |  |
| - Downlink transport channel type |  |  |  |  | HS-DSCH |  |  | RBS-3116 |  |
| - DL Transport channel identity |  |  |  |  | Not Present |  |  | RBS-3117 |  |
| - CHOICE DL parameters |  |  |  |  | HS-DSCH |  |  | RBS-3118 |  |
| - HARQ Info |  |  |  |  |  |  |  | RBS-3119 |  |
| - Number of Processes |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-3120 |  |
| - CHOICE Memory Partitioning |  |  |  |  | Implicit |  |  | RBS-3121 |  |
| - CHOICE DL MAC header type |  |  |  |  | MAC-ehs |  |  | RBS-3122 |  |
| - Added or reconfigured MAC-ehs reordering queue |  |  |  |  |  |  |  | RBS-3123 |  |
| - MAC-ehs queue to add or reconfigure list |  |  |  |  | (one queue) |  |  | RBS-3124 |  |
| - MAC-ehs queue Id |  |  |  |  | 2 (for DTCH) |  |  | RBS-3125 |  |
| - T1 |  |  |  |  | 50 |  |  | RBS-3126 |  |
| - MAC-ehs window size |  |  |  |  | 64 |  |  | RBS-3127 |  |
| - DCH quality target |  |  |  |  | Not present |  |  | RBS-3128 |  |
| Frequency info |  |  | A1, A2, A3, A4, A5, A7, A8, 11 |  |  |  |  | RBS-3129 |  |
|  |  |  | , A9, A10 |  |  | Rel-5 |  | RBS-3130 |  |
|  |  |  | , A12, A13, A14, A15, A16 |  |  | Rel-6 |  | RBS-3131 |  |
|  |  |  | , A17, A17a, A17b, A17c, A17d, A17e, A18, A19, A19a, A20, A21, A22, A24 |  |  | Rel-7 |  | RBS-3132 |  |
|  |  |  | , A23, A28a |  |  | Rel-7 |  | RBS-3133 |  |
|  |  |  | , A25, A25a, A25b, A26, A27, A27a, A28, A30 |  |  | Rel-8 |  | RBS-3134 |  |
|  |  |  | , A25c, |  |  | Rel-9 |  | RBS-3134a |  |
| - UARFCN uplink (Nu) |  |  |  |  | Reference to clause 5.1 Test frequencies. This IE should be present, if the default duplex distance defined for the operating frequency band is not used and frequency is different from the current frequency, otherwise set to Not Present. |  |  | RBS-3135 |  |
| - UARFCN downlink (Nd) |  |  |  |  | Reference to clause 5.1 Test frequencies if frequency is different from the current frequency otherwise set to Not Present. |  |  | RBS-3136 |  |
| Frequency info |  |  | A6 |  | Not Present |  |  | RBS-3137 |  |
|  |  |  | , A29 |  |  | Rel-8 |  | RBS-3138 |  |
|  |  |  | , A31, A32 A33, A34 |  |  | Rel-9 Rel-10 |  | RBS-3139 |  |
| DTX-DRX timing information |  |  | A20, A21 |  |  | Rel-7 |  | RBS-3140 |  |
|  |  |  | , A23 |  |  | Rel-7 Rel-8 |  | RBS-3141 |  |
| CHOICE timing |  |  |  |  |  |  |  | RBS-3142 |  |
| - New timing |  |  |  |  |  |  |  | RBS-3143 |  |
| - Enabling Delay |  |  |  |  | 0 |  |  | RBS-3144 |  |
| - UE DTX DRX Offset |  |  |  |  | 1 if 2ms TTI selected, otherwise 0 |  |  | RBS-3145 |  |
| DTX-DRX Information |  |  |  |  |  |  |  | RBS-3146 |  |
| - CHOICE E-DCH TTI length |  |  |  |  | Unless stated otherwise, this should be set to 2ms if the UE supports 2ms TTI, or 10ms if the UE does not support 2ms TTI. |  |  | RBS-3147 |  |
| - UE DTX cycle 1 |  |  |  |  | 8 if 2ms TTI selected, otherwise 10 |  |  | RBS-3148 |  |
| - UE DTX cycle 2 |  |  |  |  | 16 if 2ms TTI selected, otherwise 20 |  |  | RBS-3149 |  |
| - MAC DTX cycle |  |  |  |  | 8 if 2ms TTI selected, otherwise 10 |  |  | RBS-3150 |  |
| - Inactivity Threshold for UE DTX cycle 2 |  |  |  |  | 32 if 2ms TTI selected, otherwise 8 |  |  | RBS-3151 |  |
| - UE DTX long preamble length |  |  |  |  | 4 |  |  | RBS-3152 |  |
| - MAC Inactivity Threshold |  |  |  |  | 1 if 2ms TTI selected, otherwise 8 |  |  | RBS-3153 |  |
| - CQI DTX Timer |  |  |  |  | 32 |  |  | RBS-3154 |  |
| - UE DPCCH burst_1 |  |  |  |  | 1 |  |  | RBS-3155 |  |
| - UE DPCCH burst_2 |  |  |  |  | 1 |  |  | RBS-3156 |  |
| DRX Information |  |  |  |  |  |  |  | RBS-3157 |  |
| - UE DRX cycle |  |  |  |  | 8 if 2ms TTI selected, otherwise 10 |  |  | RBS-3158 |  |
| - Inactivity Threshold for UE DRX cycle |  |  |  |  | 32 |  |  | RBS-3159 |  |
| - Inactivity Threshold for UE Grant Monitoring |  |  |  |  | 32 if 2ms TTI selected, otherwise 8 |  |  | RBS-3160 |  |
| - UE DRX Grant Monitoring |  |  |  |  | TRUE |  |  | RBS-3161 |  |
| Uplink DPCCH slot format information |  |  |  |  | 1 |  |  | RBS-3162 |  |
| HS-SCCH less information |  |  |  |  | Not Present |  |  | RBS-3163 |  |
| MIMO parameters |  |  | A28a A28 |  |  | Rel-7 Rel-8 |  | RBS-3164 |  |
| - MIMO operation |  |  |  |  | start |  |  | RBS-3165 |  |
| - CHOICE mode |  |  |  |  | FDD |  |  | RBS-3166 |  |
| - MIMO N_cqi_typeA/M_cqi ratio |  |  |  |  | 1/1 |  |  | RBS-3167 |  |
| - MIMO pilot configuration |  |  |  |  |  |  |  | RBS-3168 |  |
| - CHOICE Second CPICH pattern |  |  |  |  | Antenna1 S-CPICH |  |  | RBS-3169 |  |
| - Channelisation code |  |  |  |  | 12 |  |  | RBS-3170 |  |
| MIMO parameters | A31, A32 A33, A34 |  |  |  |  | Rel-9 Rel-10 |  | RBS-3171 |  |
| - MIMO operation |  |  |  |  | start |  |  | RBS-3172 |  |
| - CHOICE mode |  |  |  |  | FDD |  |  | RBS-3173 |  |
| - MIMO N_cqi_typeA/M_cqi ratio |  |  |  |  | 1/1 |  |  | RBS-3174 |  |
| - MIMO pilot configuration |  |  |  |  |  |  |  | RBS-3175 |  |
| -CHOICE Second CPICH pattern |  |  |  |  | Antenna1 S-CPICH |  |  | RBS-3176 |  |
| - Channelisation code |  |  |  |  | 13 |  |  | RBS-3177 |  |
| - Power Offset for S-CPICH for MIMO |  |  |  |  | 0 |  |  | RBS-3178 |  |
| - Precoding weight set restriction |  |  |  |  | True |  |  | RBS-3179 |  |
| Maximum allowed UL TX power |  |  | A1, A2, A3, A4, A7, A8, A11 |  | 33dBm |  |  | RBS-3180 |  |
|  |  |  | , A9, A10 |  |  | Rel-5 |  | RBS-3181 |  |
|  |  |  | , A12, A13, A14, A15, A16 |  |  | Rel-6 |  | RBS-3182 |  |
|  |  |  | , A17, A17a, A17b, A17c, A17d, A17e, A18, A19, A19a, A20, A21, A22, A24 |  |  | Rel-7 |  | RBS-3183 |  |
|  |  |  | , A23, A28a |  |  | Rel-7 Rel-8 |  | RBS-3184 |  |
|  |  |  | , A25, A25a, A25b, A26, A27, A27a, A28, A29, A30 |  |  | Rel-8 |  | RBS-3185 |  |
|  |  |  | , A25c |  |  | Rel-9 |  | RBS-3185a |  |
| Maximum allowed UL TX power |  |  | A5, A6 |  | Not Present |  |  | RBS-3186 |  |
| CHOICE channel requirement |  |  | A1, A2, A3, A4, A7, A8, A11 |  | Uplink DPCH info | Rel-5 and earlier |  | RBS-3187 |  |
| - Uplink DPCH power control info |  |  |  |  |  |  |  | RBS-3188 |  |
| - DPCCH power offset |  |  |  |  | -40 (-80dB) |  |  | RBS-3189 |  |
| - PC Preamble |  |  |  |  | 1 frame |  |  | RBS-3190 |  |
| - SRB delay |  |  |  |  | 7 frames |  |  | RBS-3191 |  |
| - Power Control Algorithm |  |  |  |  | Algorithm1 |  |  | RBS-3192 |  |
| - TPC step size |  |  |  |  | 0 (1dB) |  |  | RBS-3193 |  |
| - NACK |  |  |  |  | Not Present | Rel-5 |  | RBS-3194 |  |
| - NACK |  |  |  |  | Not Present | Rel-5 |  | RBS-3195 |  |
| - Ack-Nack repetition factor |  |  |  |  | Not Present | Rel-5 |  | RBS-3196 |  |
| - Scrambling code type |  |  |  |  | Long |  |  | RBS-3197 |  |
| - Scrambling code number |  |  |  |  | 0 (0 to 16777215) |  |  | RBS-3198 |  |
| - Number of DPDCH |  |  |  |  | Not Present(1) |  |  | RBS-3199 |  |
| - spreading factor |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-3200 |  |
| - TFCI existence |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-3201 |  |
| - Number of FBI bit |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-3202 |  |
| - Puncturing Limit |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-3203 |  |
| - Number of TPC bits |  |  |  |  | Not Present | Rel-7 |  | RBS-3204 |  |
| CHOICE channel requirement |  |  | A9, A10 |  | Uplink DPCH info | Rel-5 |  | RBS-3205 |  |
|  |  |  | , A17, A17a, A18, A28a |  |  | Rel-7 |  | RBS-3206 |  |
|  |  |  | A25a, A28 |  |  | Rel-8 |  | RBS-3207 |  |
| - Uplink DPCH power control info |  |  |  |  |  |  |  | RBS-3208 |  |
| - DPCCH power offset |  |  |  |  | -40 (-80dB) |  |  | RBS-3209 |  |
| - PC Preamble |  |  |  |  | 1 frame |  |  | RBS-3210 |  |
| - SRB delay |  |  |  |  | 7 frames |  |  | RBS-3211 |  |
| - Power Control Algorithm |  |  |  |  | Algorithm1 |  |  | RBS-3212 |  |
| - TPC step size |  |  |  |  | 0 (1dB) |  |  | RBS-3213 |  |
| - ACK |  |  |  |  | 3 |  |  | RBS-3214 |  |
| - NACK |  |  |  |  | 3 |  |  | RBS-3215 |  |
| - Ack-Nack repetition factor |  |  |  |  | 1 |  |  | RBS-3216 |  |
| - HARQ_preamble_mode |  |  |  |  | 0 | Rel-6 |  | RBS-3217 |  |
| - Scrambling code type |  |  |  |  | Long |  |  | RBS-3218 |  |
| - Scrambling code number |  |  |  |  | 0 (0 to 16777215) |  |  | RBS-3219 |  |
| - Number of DPDCH |  |  |  |  | Not Present(1) |  |  | RBS-3220 |  |
| - spreading factor |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-3221 |  |
| - TFCI existence |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-3222 |  |
| - Number of FBI bit |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-3223 |  |
| - Puncturing Limit |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-3224 |  |
| CHOICE channel requirement |  |  | A5,A6 |  | Not Present | Rel-5 and earlier |  | RBS-3225 |  |
| Uplink DPCH info |  |  | A12 |  |  | Rel-6 |  | RBS-3226 |  |
|  |  |  | A19 |  |  | Rel-7 |  | RBS-3227 |  |
| - Uplink DPCH power control info |  |  |  |  |  |  |  | RBS-3228 |  |
| - DPCCH power offset |  |  |  |  | -40 (-80dB) |  |  | RBS-3229 |  |
| - PC Preamble |  |  |  |  | 1 frame |  |  | RBS-3230 |  |
| - SRB delay |  |  |  |  | 7 frames |  |  | RBS-3231 |  |
| - Power Control Algorithm |  |  |  |  | Algorithm1 |  |  | RBS-3232 |  |
| - TPC step size |  |  |  |  | 0 (1dB) |  |  | RBS-3233 |  |
| - ACK |  |  |  |  | 3 |  |  | RBS-3234 |  |
| - NACK |  |  |  |  | 3 |  |  | RBS-3235 |  |
| - Ack-Nack repetition factor |  |  |  |  | 1 |  |  | RBS-3236 |  |
| - HARQ_preamble_mode |  |  |  |  | 0 |  |  | RBS-3237 |  |
| - Scrambling code type |  |  |  |  | Long |  |  | RBS-3238 |  |
| - Scrambling code number |  |  |  |  | 0 (0 to 16777215) |  |  | RBS-3239 |  |
| - Number of DPDCH |  |  |  |  | Not Present(1) |  |  | RBS-3240 |  |
| - spreading factor |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-3241 |  |
| - TFCI existence |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-3242 |  |
| - Number of FBI bit |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-3243 |  |
| - Puncturing Limit |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-3244 |  |
| - Number of TPC bits |  |  |  |  | Not Present | Rel-7 |  | RBS-3245 |  |
| Uplink DPCH info |  |  | A13, A14, A15, A16 |  |  | Rel-6 |  | RBS-3246 |  |
|  |  |  | A17b, A17c, A17d, A17e, A19a, A20, A21, A22 |  |  | Rel-7 |  | RBS-3247 |  |
|  |  |  | , A23 |  |  | Rel-7 Rel-8 |  | RBS-3248 |  |
|  |  |  | , A25, A25b, A26, A27, A27a, A30 |  |  | Rel-8 |  | RBS-3249 |  |
|  |  |  | , A25c |  |  | Rel-9 |  | RBS-3249a |  |
| - Uplink DPCH power control info |  |  |  |  |  |  |  | RBS-3250 |  |
| - DPCCH power offset |  |  |  |  | -40 (-80dB) |  |  | RBS-3251 |  |
| - PC Preamble |  |  |  |  | 1 frame |  |  | RBS-3252 |  |
| - SRB delay |  |  |  |  | 7 frames |  |  | RBS-3253 |  |
| - Power Control Algorithm |  |  |  |  | Algorithm1 |  |  | RBS-3254 |  |
| - TPC step size |  |  |  |  | 0 (1dB) |  |  | RBS-3255 |  |
| - ACK |  |  |  |  | 3 |  |  | RBS-3256 |  |
| - NACK |  |  |  |  | 3 |  |  | RBS-3257 |  |
| - Ack-Nack repetition factor |  |  |  |  | 1 |  |  | RBS-3258 |  |
| - HARQ_preamble_mode |  |  |  |  | 0 |  |  | RBS-3259 |  |
| - Scrambling code type |  |  |  |  | Long |  |  | RBS-3260 |  |
| - Scrambling code number |  |  |  |  | 0 (0 to 16777215) |  |  | RBS-3261 |  |
| - Number of DPDCH |  |  |  |  | 0 |  |  | RBS-3262 |  |
| - spreading factor |  |  |  |  | Not Present |  |  | RBS-3263 |  |
| - TFCI existence |  |  |  |  | FALSE |  |  | RBS-3264 |  |
| - Number of FBI bit |  |  |  |  | Not Present |  |  | RBS-3265 |  |
| - Puncturing Limit |  |  |  |  | Not Present |  |  | RBS-3266 |  |
| - Number of TPC bits |  |  |  |  | Not Present | Rel-7 |  | RBS-3267 |  |
| Uplink DPCH info |  |  | A24, A29 |  | Not Present | Rel-7 |  | RBS-3268 |  |
| Uplink DPCH info | A31, A32 |  |  |  |  | Rel-9 Rel-10 |  | RBS-3269 |  |
| - Uplink DPCH power control info |  |  |  |  |  |  |  | RBS-3270 |  |
| - DPCCH power offset |  |  |  |  | -40 (-80dB) |  |  | RBS-3271 |  |
| - PC Preamble |  |  |  |  | 1 frame |  |  | RBS-3272 |  |
| - SRB delay |  |  |  |  | 7 frames |  |  | RBS-3273 |  |
| - Power Control Algorithm |  |  |  |  | Algorithm1 |  |  | RBS-3274 |  |
| - TPC step size |  |  |  |  | 0 (1dB) |  |  | RBS-3275 |  |
| - ∆ACK |  |  |  |  | 3 |  |  | RBS-3276 |  |
| - ∆NACK |  |  |  |  | 3 |  |  | RBS-3277 |  |
| - Ack-Nack repetition factor |  |  |  |  | 1 |  |  | RBS-3278 |  |
| - HARQ_preamble_mode |  |  |  |  | 0 |  |  | RBS-3279 |  |
| - Scrambling code type |  |  |  |  | Short |  |  | RBS-3280 |  |
| - Scrambling code number |  |  |  |  | 0 (0 to 16777215) |  |  | RBS-3281 |  |
| - Number of DPDCH |  |  |  |  | Not Present(1) |  |  | RBS-3282 |  |
| - spreading factor |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-3283 |  |
| - TFCI existence |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-3284 |  |
| - Number of FBI bit |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-3285 |  |
| - Puncturing Limit |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-3286 |  |
| - Number of TPC bits |  |  |  |  | Not Present |  |  | RBS-3287 |  |
| E-DCH info |  |  | A12, A13, A14, A15, A16 |  |  | Rel-6 |  | RBS-3288 |  |
|  |  |  | A17b, A17c, A17d, A17e, A20, A21, A22 |  |  | Rel-7 |  | RBS-3289 |  |
|  |  |  | , A23 |  |  | Rel-7 Rel-8 |  | RBS-3290 |  |
|  |  |  | , A25, A25b, A26, A27, A30 |  |  | Rel-8 |  | RBS-3291 |  |
|  |  |  | , A25c |  |  | Rel-9 |  | RBS-3291a |  |
| - MAC-es/e reset indicator |  |  |  |  | TRUE |  |  | RBS-3292 |  |
| - E-DPCCH info |  |  |  |  |  |  |  | RBS-3293 |  |
| - E-DPCCH/DPCCH power offset |  |  |  |  | 0 |  |  | RBS-3294 |  |
| - Happy bit delay condition |  |  |  |  | 100 ms |  |  | RBS-3295 |  |
| - E-TFC Boost Info |  |  |  |  | Not Present | Rel-7 |  | RBS-3296 |  |
| - E-DPDCH power interpolation |  |  |  |  | Not Present | Rel-7 |  | RBS-3297 |  |
| - E-DPDCH info |  |  |  |  |  |  |  | RBS-3298 |  |
| - E-TFCI table index |  |  |  |  | 0 |  |  | RBS-3299 |  |
| - E-DCH minimum set E-TFCI |  |  |  |  | 9 |  |  | RBS-3300 |  |
| - Reference E-TFCIs |  |  |  |  | 2 E-TFCIs |  |  | RBS-3301 |  |
| - Reference E-TFCI |  |  |  |  | 11 |  |  | RBS-3302 |  |
| - Reference E-TFCI PO |  |  |  |  | 4 |  |  | RBS-3303 |  |
| - Reference E-TFCI |  |  |  |  | 83 |  |  | RBS-3304 |  |
| - Reference E-TFCI PO |  |  |  |  | 16 |  |  | RBS-3305 |  |
| - Maximum channelisation codes |  |  |  |  | 2sf4 |  |  | RBS-3306 |  |
| - PLnon-max |  |  |  |  | 0.84 |  |  | RBS-3307 |  |
| - Scheduling Information Configuration |  |  |  |  |  |  |  | RBS-3308 |  |
| - Periodicity for Scheduling Info – no grant |  |  |  |  | Not present |  |  | RBS-3309 |  |
| - Periodicity for Scheduling Info – grant |  |  |  |  | Not present |  |  | RBS-3310 |  |
| - Power Offset for Scheduling Info |  |  |  |  | 0 |  |  | RBS-3311 |  |
| - 3-Index-Step Threshold |  |  |  |  | Not present |  |  | RBS-3312 |  |
| - 2-Index-Step Threshold |  |  |  |  | Not present |  |  | RBS-3313 |  |
| - Scheduled Transmission configuration |  |  |  |  |  |  |  | RBS-3314 |  |
| - 2ms scheduled transmission grant HARQ process allocation |  |  |  |  | Not present |  |  | RBS-3315 |  |
| - Serving Grant |  |  |  |  | Not present |  |  | RBS-3316 |  |
| -UL 16QAM settings |  |  |  |  | Not Present | Rel-7 |  | RBS-3317 |  |
| E-DCH info |  |  | A19, A19a |  |  | Rel-7 |  | RBS-3318 |  |
|  |  |  | A27a |  |  | Rel-8 |  | RBS-3319 |  |
| - MAC-es/e reset indicator |  |  |  |  | TRUE |  |  | RBS-3320 |  |
| - E-DPCCH info |  |  |  |  |  |  |  | RBS-3321 |  |
| - E-DPCCH/DPCCH power offset |  |  |  |  | 0 |  |  | RBS-3322 |  |
| - Happy bit delay condition |  |  |  |  | 100 ms |  |  | RBS-3323 |  |
| - E-TFC Boost Info |  |  |  |  | Not Present | Rel-7 |  | RBS-3324 |  |
| - E-DPDCH power interpolation |  |  |  |  | Not Present | Rel-7 |  | RBS-3325 |  |
| - E-DPDCH info |  |  |  |  |  |  |  | RBS-3326 |  |
| - E-TFCI table index |  |  |  |  | 0 |  |  | RBS-3327 |  |
| - E-DCH minimum set E-TFCI |  |  |  |  | 9 |  |  | RBS-3328 |  |
| - Reference E-TFCIs |  |  |  |  | 2 E-TFCIs |  |  | RBS-3329 |  |
| - Reference E-TFCI |  |  |  |  | 11 |  |  | RBS-3330 |  |
| - Reference E-TFCI PO |  |  |  |  | 4 |  |  | RBS-3331 |  |
| - Reference E-TFCI |  |  |  |  | 83 |  |  | RBS-3332 |  |
| - Reference E-TFCI PO |  |  |  |  | 16 |  |  | RBS-3333 |  |
| - Maximum channelisation codes |  |  |  |  | 2sf2and2sf4 |  |  | RBS-3334 |  |
| - PLnon-max |  |  |  |  | 0.84 |  |  | RBS-3335 |  |
| - Scheduling Information Configuration |  |  |  |  |  |  |  | RBS-3336 |  |
| - Periodicity for Scheduling Info – no grant |  |  |  |  | Not present |  |  | RBS-3337 |  |
| - Periodicity for Scheduling Info – grant |  |  |  |  | Not present |  |  | RBS-3338 |  |
| - Power Offset for Scheduling Info |  |  |  |  | 0 |  |  | RBS-3339 |  |
| - 3-Index-Step Threshold |  |  |  |  | Not present |  |  | RBS-3340 |  |
| - 2-Index-Step Threshold |  |  |  |  | Not present |  |  | RBS-3341 |  |
| - Scheduled Transmission configuration |  |  |  |  |  |  |  | RBS-3342 |  |
| - 2ms scheduled transmission grant HARQ process allocation |  |  |  |  | Not present |  |  | RBS-3343 |  |
| - Serving Grant |  |  |  |  | Not present |  |  | RBS-3344 |  |
| -UL 16QAM settings |  |  |  |  |  | Rel-7 |  | RBS-3345 |  |
| -BetaEd gain E-AGCH table selection |  |  |  |  | 1 |  |  | RBS-3346 |  |
| E-DCH info |  |  | A24 |  | Not Present | Rel-7 |  | RBS-3347 |  |
|  |  |  | A29 |  |  | Rel-8 |  | RBS-3348 |  |
| Uplink secondary cell info FDD |  |  | A31, A32 A33, A34 |  | Not Present | Rel-9 Rel-10 |  | RBS-3349 |  |
| Uplink secondary cell info FDD |  | A25c |  |  |  |  | Rel-9 |  | RBS-3350 |
| - Secondary serving E-DCH cell info |  |  |  |  |  |  |  |  | RBS-3351 |
| - Primary E-RNTI |  |  |  | '1010 1010 1010 1010' |  |  |  |  | RBS-3352 |
| - Secondary E-RNTI |  |  |  | Not Present |  |  |  |  | RBS-3353 |
| - Secondary E-DCH info common |  |  |  |  |  |  |  |  | RBS-3354 |
| - Frequency info |  |  |  |  |  |  |  |  | RBS-3355 |
| - UARFCN uplink (Nu) |  |  |  | Reference to clause 5.1 Test frequencies |  |  |  |  | RBS-3356 |
| - UARFCN downlink (Nd) |  |  |  | Reference to clause 5.1 Test frequencies |  |  |  |  | RBS-3357 |
| - Scrambling code type |  |  |  | Short |  |  |  |  | RBS-3358 |
| - Scrambling code number |  |  |  | 0 |  |  |  |  | RBS-3359 |
| - 2ms scheduled transmission grant HARQ process allocation |  |  |  | Not Present |  |  |  |  | RBS-3360 |
| - Serving Grant |  |  |  |  |  |  |  |  | RBS-3361 |
| - Primary/Secondary Grant Selector |  |  |  | Primary |  |  |  |  | RBS-3362 |
| - Minimum reduced E-DPDCH gain factor. |  |  |  | 21/15 |  |  |  |  | RBS-3363 |
| - E-DCH minimum set E-TFCI |  |  |  | 1 |  |  |  |  | RBS-3364 |
| - DPCCH Power offset for secondary UL frequency |  |  |  | 3 dB |  |  |  |  | RBS-3365 |
| - PC Preamble |  |  |  | 0 frame |  |  |  |  | RBS-3366 |
| - Downlink information per radio link list on secondary UL frequency |  |  |  |  |  |  |  |  | RBS-3367 |
| - Downlink information for each radio link on secondary UL frequency |  |  |  | 1 |  |  |  |  | RBS-3368 |
| - Primary CPICH info |  |  |  |  |  |  |  |  | RBS-3369 |
| - Primary scrambling code |  |  |  | Ref. to the Default setting in clause 6.1 (FDD) |  |  |  |  | RBS-3370 |
| - Cell ID |  |  |  | Not Present |  |  |  |  | RBS-3371 |
| - Downlink F-DPCH info for each RL on secondary UL frequency |  |  |  |  |  |  |  |  | RBS-3372 |
| - Downlink F-DPCH info for each RL |  |  |  |  |  |  |  |  | RBS-3373 |
| - Primary CPICH usage for channel estimate |  |  |  |  |  |  |  |  | RBS-3374 |
| - F-DPCH frame offset |  |  |  | Set to value Default DPCH Offset Value (as currently stored in SS) mod 38 400 |  |  |  |  | RBS-3375 |
| - F-DPCH slot format |  |  |  | 3 if UE supports enhanced F-DPCH, otherwise Not Present |  |  |  |  | RBS-3376 |
| - Secondary CPICH info |  |  |  | Not Present |  |  |  |  | RBS-3377 |
| - Secondary scrambling code |  |  |  | Not Present |  |  |  |  | RBS-3378 |
| - Code number |  |  |  | 12 |  |  |  |  | RBS-3379 |
| - TPC combination index |  |  |  | 11 |  |  |  |  | RBS-3380 |
| - STTD |  |  |  | TRUE |  |  |  |  | RBS-3381 |
| - E-AGCH Info |  |  |  |  |  |  |  |  | RBS-3382 |
| - E-AGCH Channelisation Code |  |  |  | 10 |  |  |  |  | RBS-3383 |
| - E-HICH Info |  |  |  |  |  |  |  |  | RBS-3384 |
| - Channelisation Code |  |  |  | 4 |  |  |  |  | RBS-3385 |
| - Signature Sequence |  |  |  | 1 |  |  |  |  | RBS-3386 |
| - E-RGCH Info |  |  |  |  |  |  |  |  | RBS-3387 |
| - Signature Sequence |  |  |  | 0 |  |  |  |  | RBS-3388 |
| - RG combination index |  |  |  | 0 |  |  |  |  | RBS-3389 |
| CHOICE Mode |  |  | A1, A2, A3, A4, A5, A6, A7, A8, A11 |  | FDD | R99 and Rel-4 only |  | RBS-3390 |  |
| - Downlink PDSCH information |  |  |  |  | Not Present |  |  | RBS-3391 |  |
| Downlink HS-PDSCH Information |  |  | A1, A2, A3, A4, A5, A6, A7, A8, A11 |  | Not Present | Rel-5 |  | RBS-3392 |  |
| Downlink HS-PDSCH Information |  |  | A9, A10 |  |  | Rel-5 |  | RBS-3393 |  |
|  |  |  | A12, A13, A14, A15, A16 |  |  | Rel-6 |  | RBS-3394 |  |
|  |  |  | A17, A17d, A18, A19, A19a, A20, A21, A22, A24 |  |  | Rel-7 |  | RBS-3395 |  |
|  |  |  | , A25, A25b, A29 |  |  | Rel-8 |  | RBS-3396 |  |
|  |  |  | , A25c |  |  | Rel-9 |  | RBS-3397 |  |
| - HS-SCCH Info |  |  |  |  |  |  |  | RBS-3398 |  |
| - CHOICE mode |  |  |  |  | FDD |  |  | RBS-3399 |  |
| - DL Scrambling Code |  |  |  |  | Not present |  |  | RBS-3400 |  |
| - HS-SCCH Channelisation Code Information |  |  |  |  |  |  |  | RBS-3401 |  |
| - HS-SCCH Channelisation Code |  |  |  |  | 7 |  |  | RBS-3402 |  |
| - Measurement Feedback Info |  |  |  |  |  |  |  | RBS-3403 |  |
| - CHOICE mode |  |  |  |  | FDD |  |  | RBS-3404 |  |
| - POhsdsch |  |  |  |  | 6 dB |  |  | RBS-3405 |  |
| - CQI Feedback cycle, k |  |  |  |  | 4 ms |  |  | RBS-3406 |  |
| - CQI repetition factor |  |  |  |  | 1 |  |  | RBS-3407 |  |
| - CQI |  |  |  |  | 5 (corresponds to 0dB in relative power offset) |  |  | RBS-3408 |  |
| - CHOICE mode |  |  |  |  | FDD (no data) |  |  | RBS-3409 |  |
| - Downlink 64QAM configured |  |  |  |  | Not present | Rel-7 |  | RBS-3410 |  |
| - HS-DSCH TB size table |  |  |  |  | Not present | Rel-7 |  | RBS-3411 |  |
| Downlink HS-PDSCH Information |  |  | A25a |  |  | Rel-8 |  | RBS-3412 |  |
| - HS-SCCH Info |  |  |  |  |  |  |  | RBS-3413 |  |
| - CHOICE mode |  |  |  |  | FDD |  |  | RBS-3414 |  |
| - DL Scrambling Code |  |  |  |  | Not present |  |  | RBS-3415 |  |
| - HS-SCCH Channelisation Code Information |  |  |  |  |  |  |  | RBS-3416 |  |
| - HS-SCCH Channelisation Code |  |  |  |  | 7 |  |  | RBS-3417 |  |
| - Measurement Feedback Info |  |  |  |  |  |  |  | RBS-3418 |  |
| - CHOICE mode |  |  |  |  | FDD |  |  | RBS-3419 |  |
| - POhsdsch |  |  |  |  | 6 dB |  |  | RBS-3420 |  |
| - CQI Feedback cycle, k |  |  |  |  | 4 ms |  |  | RBS-3421 |  |
| - CQI repetition factor |  |  |  |  | 1 |  |  | RBS-3422 |  |
| - CQI |  |  |  |  | 5 (corresponds to 0dB in relative power offset) |  |  | RBS-3423 |  |
| - CHOICE mode |  |  |  |  | FDD (no data) |  |  | RBS-3424 |  |
| - Downlink 64QAM configured |  |  |  |  | Not present | Rel-7 |  | RBS-3425 |  |
| - HS-DSCH TB size table |  |  |  |  | Octet Aligned | Rel-7 |  | RBS-3426 |  |
| Downlink HS-PDSCH Information |  |  | A17a |  |  | Rel-7 |  | RBS-3427 |  |
|  |  |  | , A28 |  |  | Rel-8 |  | RBS-3428 |  |
| - HS-SCCH Info |  |  |  |  |  |  |  | RBS-3429 |  |
| - CHOICE mode |  |  |  |  | FDD |  |  | RBS-3430 |  |
| - DL Scrambling Code |  |  |  |  | Not Present |  |  | RBS-3431 |  |
| - HS-SCCH Channelisation Code Information |  |  |  |  |  |  |  | RBS-3432 |  |
| - HS-SCCH Channelisation Code |  |  |  |  | 4 |  |  | RBS-3433 |  |
| - HS-SCCH Channelisation Code |  |  |  |  | 5 |  |  | RBS-3434 |  |
| - Measurement Feedback Info |  |  |  |  |  |  |  | RBS-3435 |  |
| - CHOICE mode |  |  |  |  | FDD |  |  | RBS-3436 |  |
| - POhsdsch |  |  |  |  | 6 dB |  |  | RBS-3437 |  |
| - CQI Feedback cycle, k |  |  |  |  | 4 ms |  |  | RBS-3438 |  |
| - CQI repetition factor |  |  |  |  | 1 |  |  | RBS-3439 |  |
| - CQI |  |  |  |  | 5 (corresponds to 0dB in relative power offset) |  |  | RBS-3440 |  |
| - CHOICE mode |  |  |  |  | FDD |  |  | RBS-3441 |  |
| - Downlink 64QAM configured |  |  |  |  | TRUE | Rel-7 |  | RBS-3442 |  |
| - HS-DSCH TB size table |  |  |  |  | Not Present | Rel-7 |  | RBS-3443 |  |
| Downlink HS-PDSCH Information |  |  | A17b |  |  | Rel-7 |  | RBS-3444 |  |
|  |  |  | A23 |  |  | Rel-7 Rel-8 |  | RBS-3445 |  |
|  |  |  | A26, A27, A27a, A30 |  |  | Rel-8 |  | RBS-3446 |  |
| - HS-SCCH Info |  |  |  |  |  |  |  | RBS-3447 |  |
| - CHOICE mode |  |  |  |  | FDD |  |  | RBS-3448 |  |
| - DL Scrambling Code |  |  |  |  | Not present |  |  | RBS-3449 |  |
| - HS-SCCH Channelisation Code Information |  |  |  |  |  |  |  | RBS-3450 |  |
| - HS-SCCH Channelisation Code |  |  |  |  | 7 |  |  | RBS-3451 |  |
| - Measurement Feedback Info |  |  |  |  |  |  |  | RBS-3452 |  |
| - CHOICE mode |  |  |  |  | FDD |  |  | RBS-3453 |  |
| - POhsdsch |  |  |  |  | 6 dB |  |  | RBS-3454 |  |
| - CQI Feedback cycle, k |  |  |  |  | 4 ms |  |  | RBS-3455 |  |
| - CQI repetition factor |  |  |  |  | 1 |  |  | RBS-3456 |  |
| - CQI |  |  |  |  | 5 (corresponds to 0dB in relative power offset) |  |  | RBS-3457 |  |
| - CHOICE mode |  |  |  |  | FDD (no data) |  |  | RBS-3458 |  |
| - Downlink 64QAM configured |  |  |  |  | Not present | Rel-7 |  | RBS-3459 |  |
| - HS-DSCH TB size table |  |  |  |  | Octet Aligned | Rel-7 |  | RBS-3460 |  |
| Downlink HS-PDSCH Information |  |  | A17c, A17e |  |  | Rel-7 |  | RBS-3461 |  |
| - HS-SCCH Info |  |  |  |  |  |  |  | RBS-3462 |  |
| - CHOICE mode |  |  |  |  | FDD |  |  | RBS-3463 |  |
| - DL Scrambling Code |  |  |  |  | Not Present |  |  | RBS-3464 |  |
| - HS-SCCH Channelisation Code Information |  |  |  |  |  |  |  | RBS-3465 |  |
| - HS-SCCH Channelisation Code |  |  |  |  | 6 |  |  | RBS-3466 |  |
| - HS-SCCH Channelisation Code |  |  |  |  | 7 |  |  | RBS-3467 |  |
| - Measurement Feedback Info |  |  |  |  |  |  |  | RBS-3468 |  |
| - CHOICE mode |  |  |  |  | FDD |  |  | RBS-3469 |  |
| - Pohsdsch |  |  |  |  | 6 dB |  |  | RBS-3470 |  |
| - CQI Feedback cycle, k |  |  |  |  | 4 ms |  |  | RBS-3471 |  |
| - CQI repetition factor |  |  |  |  | 1 |  |  | RBS-3472 |  |
| - CQI |  |  |  |  | 5 (corresponds to 0dB in relative power offset) |  |  | RBS-3473 |  |
| - CHOICE mode |  |  |  |  | FDD |  |  | RBS-3474 |  |
| - Downlink 64QAM configured |  |  |  |  | TRUE |  |  | RBS-3475 |  |
| - HS-DSCH TB size table |  |  |  |  | Not Present |  |  | RBS-3476 |  |
| Downlink HS-PDSCH Information | A28a |  |  |  |  | Rel-7 |  | RBS-3477 |  |
| - HS-SCCH Info |  |  |  |  |  |  |  | RBS-3478 |  |
| - CHOICE mode |  |  |  |  | FDD |  |  | RBS-3479 |  |
| - DL Scrambling Code |  |  |  |  | Not Present |  |  | RBS-3480 |  |
| - HS-SCCH Channelisation Code Information |  |  |  |  |  |  |  | RBS-3481 |  |
| - HS-SCCH Channelisation Code |  |  |  |  | 4 |  |  | RBS-3482 |  |
| - HS-SCCH Channelisation Code |  |  |  |  | 5 |  |  | RBS-3483 |  |
| - Measurement Feedback Info |  |  |  |  |  |  |  | RBS-3484 |  |
| - CHOICE mode |  |  |  |  | FDD |  |  | RBS-3485 |  |
| - Pohsdsch |  |  |  |  | 6 dB |  |  | RBS-3486 |  |
| - CQI Feedback cycle, k |  |  |  |  | 4 ms |  |  | RBS-3487 |  |
| - CQI repetition factor |  |  |  |  | 1 |  |  | RBS-3488 |  |
| - CQI |  |  |  |  | 5 (corresponds to 0dB in relative power offset) |  |  | RBS-3489 |  |
| - CHOICE mode |  |  |  |  | FDD |  |  | RBS-3490 |  |
| - Downlink 64QAM configured |  |  |  |  | Not Present |  |  | RBS-3491 |  |
| - HS-DSCH TB size table |  |  |  |  | Not Present |  |  | RBS-3492 |  |
| Downlink HS-PDSCH Information | A31 A34 |  |  |  |  | Rel-9 Rel-10 |  | RBS-3493 |  |
| - HS-SCCH Info |  |  |  |  |  |  |  | RBS-3494 |  |
| - CHOICE mode |  |  |  |  | FDD |  |  | RBS-3495 |  |
| - DL Scrambling Code |  |  |  |  | Not Present |  |  | RBS-3496 |  |
| - HS-SCCH Channelisation Code Information |  |  |  |  |  |  |  | RBS-3497 |  |
| - HS-SCCH Channelisation Code |  |  |  |  | 6 |  |  | RBS-3498 |  |
| - HS-SCCH Channelisation Code |  |  |  |  | 7 |  |  | RBS-3499 |  |
| - Measurement Feedback Info |  |  |  |  |  |  |  | RBS-3500 |  |
| - CHOICE mode |  |  |  |  | FDD |  |  | RBS-3501 |  |
| - Pohsdsch |  |  |  |  | 8 dB |  |  | RBS-3502 |  |
| - CQI Feedback cycle, k |  |  |  |  | 8 ms |  |  | RBS-3503 |  |
| - CQI repetition factor |  |  |  |  | 1 |  |  | RBS-3504 |  |
| - CQI |  |  |  |  | 4 (corresponds to 0dB in relative power offset) |  |  | RBS-3505 |  |
| - CHOICE mode |  |  |  |  | FDD |  |  | RBS-3506 |  |
| - Downlink 64QAM configured |  |  |  |  | Not Present |  |  | RBS-3507 |  |
| - HS-DSCH TB size table |  |  |  |  | Not Present |  |  | RBS-3508 |  |
| Downlink HS-PDSCH Information | A32 A33 |  |  |  |  | Rel-9 Rel-10 |  | RBS-3509 |  |
| - HS-SCCH Info |  |  |  |  |  |  |  | RBS-3510 |  |
| - CHOICE mode |  |  |  |  | FDD |  |  | RBS-3511 |  |
| - DL Scrambling Code |  |  |  |  | Not Present |  |  | RBS-3512 |  |
| - HS-SCCH Channelisation Code Information |  |  |  |  |  |  |  | RBS-3513 |  |
| - HS-SCCH Channelisation Code |  |  |  |  | 6 |  |  | RBS-3514 |  |
| - HS-SCCH Channelisation Code |  |  |  |  | 7 |  |  | RBS-3515 |  |
| - Measurement Feedback Info |  |  |  |  |  |  |  | RBS-3516 |  |
| - CHOICE mode |  |  |  |  | FDD |  |  | RBS-3517 |  |
| - Pohsdsch |  |  |  |  | 8 dB |  |  | RBS-3518 |  |
| - CQI Feedback cycle, k |  |  |  |  | 8 ms |  |  | RBS-3519 |  |
| - CQI repetition factor |  |  |  |  | 1 |  |  | RBS-3520 |  |
| - CQI |  |  |  |  | 4 (corresponds to 0dB in relative power offset) |  |  | RBS-3521 |  |
| - CHOICE mode |  |  |  |  | FDD |  |  | RBS-3522 |  |
| - Downlink 64QAM configured |  |  |  |  | TRUE |  |  | RBS-3523 |  |
| - HS-DSCH TB size table |  |  |  |  | Not Present |  |  | RBS-3524 |  |
| Downlink information common for all radio links |  |  | A1, A2, A3, A11 |  |  |  |  | RBS-3525 |  |
| - Downlink DPCH info common for all RL |  |  |  |  |  |  |  | RBS-3526 |  |
| - Timing indicator |  |  |  |  | Maintain |  |  | RBS-3527 |  |
| - CFN-targetSFN frame offset |  |  |  |  | Not Present |  |  | RBS-3528 |  |
| - Downlink DPCH power control information |  |  |  |  |  |  |  | RBS-3529 |  |
| - DPC mode |  |  |  |  | 0 (single) |  |  | RBS-3530 |  |
| - CHOICE mode |  |  |  |  | FDD |  |  | RBS-3531 |  |
| - Power offset PPilot-DPDCH |  |  |  |  | 0 |  |  | RBS-3532 |  |
| - DL rate matching restriction information |  |  |  |  | Not Present |  |  | RBS-3533 |  |
| - Spreading factor |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-3534 |  |
| - Fixed or Flexible Position |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-3535 |  |
| - TFCI existence |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-3536 |  |
| - CHOICE SF |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-3537 |  |
| - CHOICE mode |  |  |  |  | FDD |  |  | RBS-3538 |  |
| - DPCH compressed mode info |  |  |  |  | Not Present |  |  | RBS-3539 |  |
| - TX Diversity mode |  |  |  |  | None |  |  | RBS-3540 |  |
| - SSDT information |  |  |  |  | Not Present | R99 and Rel-4 only |  | RBS-3541 |  |
| - Default DPCH Offset Value |  |  |  |  | Not Present |  |  | RBS-3542 |  |
| Downlink information common for all radio links |  |  | A9 |  |  | Rel-5 |  | RBS-3543 |  |
|  |  |  | A12, A13, A15 |  |  | Rel-6 |  | RBS-3544 |  |
|  |  |  | A17, A17a, A17d, A17e, A18. A19, A19a, A28a |  |  | Rel-7 |  | RBS-3545 |  |
|  |  |  | A25a, A25b, A26, A28 |  |  | Rel-8 |  | RBS-3546 |  |
|  |  |  | , A25c |  |  | Rel-9 |  | RBS-3547 |  |
| - Downlink DPCH info common for all RL |  |  |  |  |  |  |  | RBS-3548 |  |
| - Timing indicator |  |  |  |  | Maintain |  |  | RBS-3549 |  |
| - CFN-targetSFN frame offset |  |  |  |  | Not Present |  |  | RBS-3550 |  |
| - Downlink DPCH power control information |  |  |  |  |  |  |  | RBS-3551 |  |
| - DPC mode |  |  |  |  | 0 (single) |  |  | RBS-3552 |  |
| - CHOICE mode |  |  |  |  | FDD |  |  | RBS-3553 |  |
| - Power offset PPilot-DPDCH |  |  |  |  | 0 |  |  | RBS-3554 |  |
| - DL rate matching restriction information |  |  |  |  | Not Present |  |  | RBS-3555 |  |
| - Spreading factor |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-3556 |  |
| - Fixed or Flexible Position |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-3557 |  |
| - TFCI existence |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-3558 |  |
| - CHOICE SF |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-3559 |  |
| - CHOICE mode |  |  |  |  | FDD |  |  | RBS-3560 |  |
| - DPCH compressed mode info |  |  |  |  | Not Present |  |  | RBS-3561 |  |
| - TX Diversity mode |  |  |  |  | None |  |  | RBS-3562 |  |
| - Default DPCH Offset Value |  |  |  |  | Not Present |  |  | RBS-3563 |  |
| - MAC-hs reset indicator |  |  |  |  | Not Present |  |  | RBS-3564 |  |
| Downlink information common for all radio links |  |  | A4,A7,A8 |  |  |  |  | RBS-3565 |  |
| - Downlink DPCH info common for all RL |  |  |  |  |  |  |  | RBS-3566 |  |
| - Timing indicator |  |  |  |  | Initialize |  |  | RBS-3567 |  |
| - CFN-targetSFN frame offset |  |  |  |  | Not Present |  |  | RBS-3568 |  |
| - Downlink DPCH power control information |  |  |  |  |  |  |  | RBS-3569 |  |
| - DPC mode |  |  |  |  | 0 (single) |  |  | RBS-3570 |  |
| - CHOICE mode |  |  |  |  | FDD |  |  | RBS-3571 |  |
| - Power offset PPilot-DPDCH |  |  |  |  | 0 |  |  | RBS-3572 |  |
| - DL rate matching restriction information |  |  |  |  | Not Present |  |  | RBS-3573 |  |
| - Spreading factor |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-3574 |  |
| - Fixed or Flexible Position |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-3575 |  |
| - TFCI existence |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-3576 |  |
| - CHOICE SF |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-3577 |  |
| - CHOICE mode |  |  |  |  | FDD |  |  | RBS-3578 |  |
| - DPCH compressed mode info |  |  |  |  | Not Present |  |  | RBS-3579 |  |
| - TX Diversity mode |  |  |  |  | None |  |  | RBS-3580 |  |
| - SSDT information |  |  |  |  | Not Present | R99 and Rel-4 only |  | RBS-3581 |  |
| - Default DPCH Offset Value |  |  |  |  | Arbitrary set to value 0..306688 by step of 512 |  |  | RBS-3582 |  |
| Downlink information common for all radio links |  |  | A10 |  |  | Rel-5 |  | RBS-3583 |  |
| - Downlink DPCH info common for all RL |  |  |  |  |  |  |  | RBS-3584 |  |
| - Timing indicator |  |  |  |  | Initialize |  |  | RBS-3585 |  |
| - CFN-targetSFN frame offset |  |  |  |  | Not Present |  |  | RBS-3586 |  |
| - Downlink DPCH power control information |  |  |  |  |  |  |  | RBS-3587 |  |
| - DPC mode |  |  |  |  | 0 (single) |  |  | RBS-3588 |  |
| - CHOICE mode |  |  |  |  | FDD |  |  | RBS-3589 |  |
| - Power offset PPilot-DPDCH |  |  |  |  | 0 |  |  | RBS-3590 |  |
| - DL rate matching restriction information |  |  |  |  | Not Present |  |  | RBS-3591 |  |
| - Spreading factor |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-3592 |  |
| - Fixed or Flexible Position |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-3593 |  |
| - TFCI existence |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-3594 |  |
| - CHOICE SF |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-3595 |  |
| - CHOICE mode |  |  |  |  | FDD |  |  | RBS-3596 |  |
| - DPCH compressed mode info |  |  |  |  | Not Present |  |  | RBS-3597 |  |
| - TX Diversity mode |  |  |  |  | None |  |  | RBS-3598 |  |
| - Default DPCH Offset Value |  |  |  |  | Arbitrary set to value 0..306688 by step of 512 |  |  | RBS-3599 |  |
| - MAC-hs reset indicator |  |  |  |  | Not Present |  |  | RBS-3600 |  |
| Downlink information common for all radio links |  |  | A14, A16 |  |  | Rel-6 |  | RBS-3601 |  |
|  |  |  | A17b, A17c, A20, A21, A22 |  |  | Rel-7 |  | RBS-3602 |  |
|  |  |  | , A23 |  |  | Rel-7 Rel-8 |  | RBS-3603 |  |
|  |  |  | , A25, A27, A27a, A30 |  |  | Rel-8 |  | RBS-3604 |  |
| - Downlink F-DPCH info common for all RL |  |  |  |  |  |  |  | RBS-3605 |  |
| - Timing Indication |  |  |  |  | Maintain |  |  | RBS-3606 |  |
| - Timing maintained Synchronization indicator |  |  |  |  | FALSE |  |  | RBS-3607 |  |
| - Downlink F-DPCH power control information |  |  |  |  |  |  |  | RBS-3608 |  |
| - DPC mode |  |  |  |  | 0 (single) |  |  | RBS-3609 |  |
| - TPC command error rate target |  |  |  |  | 0.04 |  |  | RBS-3610 |  |
| - CHOICE mode |  |  |  |  | FDD |  |  | RBS-3611 |  |
| - DPCH compressed mode info |  |  |  |  | Not Present |  |  | RBS-3612 |  |
| - TX Diversity mode |  |  |  |  | None |  |  | RBS-3613 |  |
| - Default DPCH Offset Value |  |  |  |  | Not Present |  |  | RBS-3614 |  |
| - MAC-hs reset indicator |  |  |  |  | Not Present |  |  | RBS-3615 |  |
| Downlink information common for all radio links |  |  | A5,A6 |  | Not Present |  |  | RBS-3616 |  |
|  |  |  | A24 |  | Not Present | Rel-7 |  | RBS-3617 |  |
|  |  |  | A29 |  |  |  |  | RBS-3618 |  |
| Downlink information for each radio link list |  |  | A1, A2, A3, A4, A7, A8, A11 |  |  |  |  | RBS-3619 |  |
| - Downlink information for each radio link |  |  |  |  |  |  |  | RBS-3620 |  |
| - Choice mode |  |  |  |  | FDD |  |  | RBS-3621 |  |
| - Primary CPICH info |  |  |  |  |  |  |  | RBS-3622 |  |
| - Primary scrambling code |  |  |  |  | Ref. to the Default setting in clause 6.1 (FDD) |  |  | RBS-3623 |  |
| - PDSCH with SHO DCH info |  |  |  |  | Not Present | R99 and Rel-4 only |  | RBS-3624 |  |
| - PDSCH code mapping |  |  |  |  | Not Present | R99 and Rel-4 only |  | RBS-3625 |  |
| - Serving HS-DSCH radio link indicator |  |  |  |  | FALSE | Rel-5 |  | RBS-3626 |  |
| - Downlink DPCH info for each RL |  |  |  |  |  |  |  | RBS-3627 |  |
| - Primary CPICH usage for channel estimation |  |  |  |  | Primary CPICH may be used |  |  | RBS-3628 |  |
| - DPCH frame offset |  |  |  |  | Set to value Default DPCH Offset Value (as currently stored in SS) mod 38 400 |  |  | RBS-3629 |  |
| - Secondary CPICH info |  |  |  |  | Not Present |  |  | RBS-3630 |  |
| - DL channelisation code |  |  |  |  |  |  |  | RBS-3631 |  |
| - Secondary scrambling code |  |  |  |  | 1 |  |  | RBS-3632 |  |
| - Spreading factor |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-3633 |  |
| - Code number |  |  |  |  | 0 |  |  | RBS-3634 |  |
| - Scrambling code change |  |  |  |  | Set to value Default1: No code change (if the UE has a compressed mode pattern sequence configured in variable TGPS_IDENTITY or included in the message including IE "Downlink DPCH info for each RL", which is using compressed mode method "SF/2") Set to value Default2: OMIT (otherwise) |  |  | RBS-3635 |  |
| - TPC combination index |  |  |  |  | 0 |  |  | RBS-3636 |  |
| - SSDT Cell Identity |  |  |  |  | Not Present | R99 and Rel-4 only |  | RBS-3637 |  |
| - Closed loop timing adjustment mode |  |  |  |  | Not Present |  |  | RBS-3638 |  |
| - SCCPCH information for FACH |  |  |  |  | Not Present | R99 and Rel-4 only |  | RBS-3639 |  |
| Downlink information for each radio link list |  |  | A5 |  |  |  |  | RBS-3640 |  |
| - Downlink information for each radio link |  |  |  |  |  |  |  | RBS-3641 |  |
| - Choice mode |  |  |  |  | FDD |  |  | RBS-3642 |  |
| - Primary CPICH info |  |  |  |  |  |  |  | RBS-3643 |  |
| - Primary scrambling code |  |  |  |  | Ref. to the Default setting in clause 6.1 (FDD) |  |  | RBS-3644 |  |
| - PDSCH with SHO DCH info |  |  |  |  | Not Present | R99 and Rel-4 only |  | RBS-3645 |  |
| - PDSCH code mapping |  |  |  |  | Not Present | R99 and Rel-4 only |  | RBS-3646 |  |
| - Serving HS-DSCH radio link indicator |  |  |  |  | FALSE | Rel-5 |  | RBS-3647 |  |
| - Downlink DPCH info for each RL |  |  |  |  | Not present |  |  | RBS-3648 |  |
| - SCCPCH information for FACH |  |  |  |  | Not Present | R99 and Rel-4 only |  | RBS-3649 |  |
| Downlink information for each radio link list |  |  | A9, A10 |  |  | Rel-5 |  | RBS-3650 |  |
|  |  |  | , A17, A18 |  |  | Rel-7 |  | RBS-3651 |  |
| - Downlink information for each radio link |  |  |  |  |  |  |  | RBS-3652 |  |
| - Choice mode |  |  |  |  | FDD |  |  | RBS-3653 |  |
| - Primary CPICH info |  |  |  |  |  |  |  | RBS-3654 |  |
| - Primary scrambling code |  |  |  |  | Ref. to the Default setting in clause 6.1 (FDD) |  |  | RBS-3655 |  |
| - PDSCH with SHO DCH info |  |  |  |  | Not Present | R99 and Rel-4 only |  | RBS-3656 |  |
| - PDSCH code mapping |  |  |  |  | Not Present | R99 and Rel-4 only |  | RBS-3657 |  |
| - Serving HS-DSCH radio link indicator |  |  |  |  | TRUE |  |  | RBS-3658 |  |
| - Serving E-DCH radio link indicator |  |  |  |  | FALSE | Rel-6 |  | RBS-3659 |  |
| - Downlink DPCH info for each RL |  |  |  |  |  |  |  | RBS-3660 |  |
| - Primary CPICH usage for channel estimation |  |  |  |  | Primary CPICH may be used |  |  | RBS-3661 |  |
| - DPCH frame offset |  |  |  |  | Set to value Default DPCH Offset Value (as currently stored in SS) mod 38 400 |  |  | RBS-3662 |  |
| - Secondary CPICH info |  |  |  |  | Not Present |  |  | RBS-3663 |  |
| - DL channelisation code |  |  |  |  |  |  |  | RBS-3664 |  |
| - Secondary scrambling code |  |  |  |  | 1 |  |  | RBS-3665 |  |
| - Spreading factor |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-3666 |  |
| - Code number |  |  |  |  | 0 |  |  | RBS-3667 |  |
| - Scrambling code change |  |  |  |  | Set to value Default1: No code change (if the UE has a compressed mode pattern sequence configured in variable TGPS_IDENTITY or included in the message including IE "Downlink DPCH info for each RL", which is using compressed mode method "SF/2") Set to value Default2: OMIT (otherwise) |  |  | RBS-3668 |  |
| - TPC combination index |  |  |  |  | 0 |  |  | RBS-3669 |  |
| - SSDT Cell Identity |  |  |  |  | Not Present | R99 and Rel-4 only |  | RBS-3670 |  |
| - Closed loop timing adjustment mode |  |  |  |  | Not Present |  |  | RBS-3671 |  |
| - E-AGCH Info |  |  |  |  | Not Present | Rel-6 |  | RBS-3672 |  |
| - CHOICE E-HICH Information |  |  |  |  | Not Present | Rel-6 |  | RBS-3673 |  |
| - CHOICE E-RGCH Information |  |  |  |  | Not Present | Rel-6 |  | RBS-3674 |  |
| - SCCPCH information for FACH |  |  |  |  | Not Present | R99 and Rel-4 only |  | RBS-3675 |  |
| Downlink information for each radio link list |  |  | A17a, A17d, A17e, A28a |  |  | Rel-7 |  | RBS-3676 |  |
|  |  |  | A25a, A28 |  |  | Rel-8 |  | RBS-3677 |  |
| - Downlink information for each radio link |  |  |  |  |  |  |  | RBS-3678 |  |
| - Choice mode |  |  |  |  | FDD |  |  | RBS-3679 |  |
| - Primary CPICH info |  |  |  |  |  |  |  | RBS-3680 |  |
| - Primary scrambling code |  |  |  |  | Ref. to the Default setting in clause 6.1 (FDD) |  |  | RBS-3681 |  |
| - Serving HS-DSCH radio link indicator |  |  |  |  | TRUE |  |  | RBS-3682 |  |
| - Serving E-DCH radio link indicator |  |  |  |  | FALSE | Rel-6 |  | RBS-3683 |  |
| - Downlink DPCH info for each RL |  |  |  |  |  |  |  | RBS-3684 |  |
| - Primary CPICH usage for channel estimation |  |  |  |  | Primary CPICH may be used |  |  | RBS-3685 |  |
| - DPCH frame offset |  |  |  |  | Set to value Default DPCH Offset Value (as currently stored in SS) mod 38 400 |  |  | RBS-3686 |  |
| - Secondary CPICH info |  |  |  |  | Not Present |  |  | RBS-3687 |  |
| - DL channelisation code |  |  |  |  |  |  |  | RBS-3688 |  |
| - Secondary scrambling code |  |  |  |  | Not Present |  |  | RBS-3689 |  |
| - Spreading factor |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-3690 |  |
| - Code number |  |  |  |  | 13 |  |  | RBS-3691 |  |
| - Scrambling code change |  |  |  |  | Not Present |  |  | RBS-3692 |  |
| - TPC combination index |  |  |  |  | 0 |  |  | RBS-3693 |  |
| - Closed loop timing adjustment mode |  |  |  |  | Not Present |  |  | RBS-3694 |  |
| - E-AGCH Info |  |  |  |  | Not Present | Rel-6 |  | RBS-3695 |  |
| - CHOICE E-HICH Information |  |  |  |  | Not Present | Rel-6 |  | RBS-3696 |  |
| - CHOICE E-RGCH Information |  |  |  |  | Not Present | Rel-6 |  | RBS-3697 |  |
| Downlink information for each radio link list |  |  | A12, A13, A15 |  |  | Rel-6 |  | RBS-3698 |  |
|  |  |  | A19, A19a |  |  | Rel-7 |  | RBS-3699 |  |
|  |  |  | A26 |  |  | Rel-8 |  | RBS-3700 |  |
| - Downlink information for each radio link |  |  |  |  |  |  |  | RBS-3701 |  |
| - Choice mode |  |  |  |  | FDD |  |  | RBS-3702 |  |
| - Primary CPICH info |  |  |  |  |  |  |  | RBS-3703 |  |
| - Primary scrambling code |  |  |  |  | Ref. to the Default setting in clause 6.1 (FDD) |  |  | RBS-3704 |  |
| - Serving HS-DSCH radio link indicator |  |  |  |  | TRUE |  |  | RBS-3705 |  |
| - Serving E-DCH radio link indicator |  |  |  |  | TRUE |  |  | RBS-3706 |  |
| - Downlink DPCH info for each RL |  |  |  |  |  |  |  | RBS-3707 |  |
| - Primary CPICH usage for channel estimation |  |  |  |  | Primary CPICH may be used |  |  | RBS-3708 |  |
| - DPCH frame offset |  |  |  |  | Set to value Default DPCH Offset Value (as currently stored in SS) mod 38 400 |  |  | RBS-3709 |  |
| - Secondary CPICH info |  |  |  |  | Not Present |  |  | RBS-3710 |  |
| - DL channelisation code |  |  |  |  |  |  |  | RBS-3711 |  |
| - Secondary scrambling code |  |  |  |  | 1 |  |  | RBS-3712 |  |
| - Spreading factor |  |  |  |  | Reference to clause 6.10 Parameter Set |  |  | RBS-3713 |  |
| - Code number |  |  |  |  | 0 |  |  | RBS-3714 |  |
| - Scrambling code change |  |  |  |  | Set to value: No code change (if the UE has a compressed mode pattern sequence configured in variable TGPS_IDENTITY or included in the message including IE "Downlink DPCH info for each RL", which is using compressed mode method "SF/2") Set to value: OMIT (otherwise) | Default1 Default2 |  | RBS-3715 |  |
| - TPC combination index |  |  |  |  | 0 |  |  | RBS-3716 |  |
| - Closed loop timing adjustment mode |  |  |  |  | Not Present |  |  | RBS-3717 |  |
| - E-AGCH Info |  |  |  |  |  |  |  | RBS-3718 |  |
| - E-AGCH Channelisation Code |  |  |  |  | 10 |  |  | RBS-3719 |  |
| - CHOICE E-HICH Information |  |  |  |  |  |  |  | RBS-3720 |  |
| - E-HICH Information |  |  |  |  |  |  |  | RBS-3721 |  |
| - Channelisation code |  |  |  |  | 4 |  |  | RBS-3722 |  |
| - Signature sequence |  |  |  |  | 1 |  |  | RBS-3723 |  |
| - CHOICE E-RGCH Information |  |  |  |  |  |  |  | RBS-3724 |  |
| - E-RGCH Information |  |  |  |  |  |  |  | RBS-3725 |  |
| - Signature Sequence |  |  |  |  | 0 |  |  | RBS-3726 |  |
| - RG combination index |  |  |  |  | 0 |  |  | RBS-3727 |  |
| Downlink information for each radio link list |  |  | A14, A16 |  |  | Rel-6 |  | RBS-3728 |  |
| - Downlink information for each radio link |  |  |  |  |  |  |  | RBS-3729 |  |
| - Choice mode |  |  |  |  | FDD |  |  | RBS-3730 |  |
| - Primary CPICH info |  |  |  |  |  |  |  | RBS-3731 |  |
| - Primary scrambling code |  |  |  |  | Ref. to the Default setting in clause 6.1 (FDD) |  |  | RBS-3732 |  |
| - Serving HS-DSCH radio link indicator |  |  |  |  | TRUE |  |  | RBS-3733 |  |
| - Serving E-DCH radio link indicator |  |  |  |  | TRUE |  |  | RBS-3734 |  |
| - Downlink DPCH info for each RL |  |  |  |  | Not Present |  |  | RBS-3735 |  |
| - Downlink F-DPCH info for each RL |  |  |  |  |  |  |  | RBS-3736 |  |
| - Primary CPICH usage for channel estimation |  |  |  |  | Primary CPICH may be used |  |  | RBS-3737 |  |
| - F-DPCH frame offset |  |  |  |  | Set to value Default DPCH Offset Value (as currently stored in SS) mod 38 400 |  |  | RBS-3738 |  |
| - F-DPCH slot format |  |  |  |  | 3 if UE supports enhanced F-DPCH, otherwise Not Present | Rel-7 |  | RBS-3739 |  |
| - Secondary CPICH info |  |  |  |  | Not Present |  |  | RBS-3740 |  |
| - Secondary scrambling code |  |  |  |  | Not Present |  |  | RBS-3741 |  |
| - Code number |  |  |  |  | 12 |  |  | RBS-3742 |  |
| - TPC combination index |  |  |  |  | 0 |  |  | RBS-3743 |  |
| - E-AGCH Info |  |  |  |  |  |  |  | RBS-3744 |  |
| - E-AGCH Channelisation Code |  |  |  |  | 10 |  |  | RBS-3745 |  |
| - CHOICE E-HICH Information |  |  |  |  |  |  |  | RBS-3746 |  |
| - E-HICH Information |  |  |  |  |  |  |  | RBS-3747 |  |
| - Channelisation code |  |  |  |  | 4 |  |  | RBS-3748 |  |
| - Signature sequence |  |  |  |  | 1 |  |  | RBS-3749 |  |
| - CHOICE E-RGCH Information |  |  |  |  | Not Present |  |  | RBS-3750 |  |
| Downlink information for each radio link list |  |  | A17b, A17c |  |  | Rel-7 |  | RBS-3751 |  |
| - Downlink information for each radio link |  |  |  |  |  |  |  | RBS-3752 |  |
| - Choice mode |  |  |  |  | FDD |  |  | RBS-3753 |  |
| - Primary CPICH info |  |  |  |  |  |  |  | RBS-3754 |  |
| - Primary scrambling code |  |  |  |  | Ref. to the Default setting in clause 6.1 (FDD) |  |  | RBS-3755 |  |
| - Serving HS-DSCH radio link indicator |  |  |  |  | TRUE |  |  | RBS-3756 |  |
| - Serving E-DCH radio link indicator |  |  |  |  | TRUE |  |  | RBS-3757 |  |
| - Downlink DPCH info for each RL |  |  |  |  | Not Present |  |  | RBS-3758 |  |
| - Downlink F-DPCH info for each RL |  |  |  |  |  |  |  | RBS-3759 |  |
| - Primary CPICH usage for channel estimation |  |  |  |  | Primary CPICH may be used |  |  | RBS-3760 |  |
| - F-DPCH frame offset |  |  |  |  | Set to value Default DPCH Offset Value (as currently stored in SS) mod 38 400 |  |  | RBS-3761 |  |
| - F-DPCH slot format |  |  |  |  | 3 if UE supports enhanced F-DPCH, otherwise Not Present |  |  | RBS-3762 |  |
| - Secondary CPICH info |  |  |  |  | Not Present |  |  | RBS-3763 |  |
| - Secondary scrambling code |  |  |  |  | Not Present |  |  | RBS-3764 |  |
| - Code number |  |  |  |  | 11 |  |  | RBS-3765 |  |
| - TPC combination index |  |  |  |  | 0 |  |  | RBS-3766 |  |
| - E-AGCH Info |  |  |  |  |  |  |  | RBS-3767 |  |
| - E-AGCH Channelisation Code |  |  |  |  | 10 |  |  | RBS-3768 |  |
| - CHOICE E-HICH Information |  |  |  |  |  |  |  | RBS-3769 |  |
| - E-HICH Information |  |  |  |  |  |  |  | RBS-3770 |  |
| - Channelisation code |  |  |  |  | 4 |  |  | RBS-3771 |  |
| - Signature sequence |  |  |  |  | 1 |  |  | RBS-3772 |  |
| - CHOICE E-RGCH Information |  |  |  |  | Not Present |  |  | RBS-3773 |  |
|  |  |  |  |  |  |  |  | RBS-3774 |  |
| Downlink information for each radio link list |  |  | A30 |  |  | Rel-8 |  | RBS-3775 |  |
| - Downlink information for each radio link |  |  |  |  |  |  |  | RBS-3776 |  |
| - Choice mode |  |  |  |  | FDD |  |  | RBS-3777 |  |
| - Primary CPICH info |  |  |  |  |  |  |  | RBS-3778 |  |
| - Primary scrambling code |  |  |  |  | Ref. to the Default setting in clause 6.1 (FDD) |  |  | RBS-3779 |  |
| - Serving HS-DSCH radio link indicator |  |  |  |  | TRUE |  |  | RBS-3780 |  |
| - Serving E-DCH radio link indicator |  |  |  |  | TRUE |  |  | RBS-3781 |  |
| - Downlink DPCH info for each RL |  |  |  |  | Not Present |  |  | RBS-3782 |  |
| - Downlink F-DPCH info for each RL |  |  |  |  |  |  |  | RBS-3783 |  |
| - Primary CPICH usage for channel estimation |  |  |  |  | Primary CPICH may be used |  |  | RBS-3784 |  |
| - F-DPCH frame offset |  |  |  |  | Set to value Default DPCH Offset Value (as currently stored in SS) mod 38 400 |  |  | RBS-3785 |  |
| - F-DPCH slot format |  |  |  |  | 3 if UE supports enhanced F-DPCH, otherwise Not Present |  |  | RBS-3786 |  |
| - Secondary CPICH info |  |  |  |  | Not Present |  |  | RBS-3787 |  |
| - Secondary scrambling code |  |  |  |  | Not Present |  |  | RBS-3788 |  |
| - Code number |  |  |  |  | 12 |  |  | RBS-3789 |  |
| - TPC combination index |  |  |  |  | 0 |  |  | RBS-3790 |  |
| - E-AGCH Info |  |  |  |  |  |  |  | RBS-3791 |  |
| - E-AGCH Channelisation Code |  |  |  |  | 11 |  |  | RBS-3792 |  |
| - CHOICE E-HICH Information |  |  |  |  |  |  |  | RBS-3793 |  |
| - E-HICH Information |  |  |  |  |  |  |  | RBS-3794 |  |
| - Channelisation code |  |  |  |  | 4 |  |  | RBS-3795 |  |
| - Signature sequence |  |  |  |  | 10 |  |  | RBS-3796 |  |
| - CHOICE E-RGCH Information |  |  |  |  | Not Present |  |  | RBS-3797 |  |
| Downlink information for each radio link list |  |  | A20, A21, |  |  | Rel-7 |  | RBS-3798 |  |
|  |  |  | , A23 |  |  | Rel-7 Rel-8 |  | RBS-3799 |  |
|  |  |  | , A25 |  |  | Rel-8 |  | RBS-3800 |  |
|  |  |  | , A27, A27a |  |  | Rel-8 |  | RBS-3801 |  |
|  |  |  | , A31, A32 A33, A34 |  |  | Rel-9 Rel-10 |  | RBS-3802 |  |
| - Downlink information for each radio link |  |  |  |  |  |  |  | RBS-3803 |  |
| - Choice mode |  |  |  |  | FDD |  |  | RBS-3804 |  |
| - Primary CPICH info |  |  |  |  |  |  |  | RBS-3805 |  |
| - Primary scrambling code |  |  |  |  | Ref. to the Default setting in clause 6.1 (FDD) |  |  | RBS-3806 |  |
| - Serving HS-DSCH radio link indicator |  |  |  |  | TRUE |  |  | RBS-3807 |  |
| - Serving E-DCH radio link indicator |  |  |  |  | TRUE |  |  | RBS-3808 |  |
| - Downlink DPCH info for each RL |  |  |  |  | Not Present |  |  | RBS-3809 |  |
| - Downlink F-DPCH info for each RL |  |  |  |  |  |  |  | RBS-3810 |  |
| - Primary CPICH usage for channel estimation |  |  |  |  | Primary CPICH may be used |  |  | RBS-3811 |  |
| - F-DPCH frame offset |  |  |  |  | Set to value Default DPCH Offset Value (as currently stored in SS) mod 38 400 |  |  | RBS-3812 |  |
| - F-DPCH slot format |  |  |  |  | 3 if UE supports enhanced F-DPCH, otherwise Not Present | Rel-7 |  | RBS-3813 |  |
| - Secondary CPICH info |  |  |  |  | Not Present |  |  | RBS-3814 |  |
| - Secondary scrambling code |  |  |  |  | Not Present |  |  | RBS-3815 |  |
| - Code number |  |  |  |  | 12 |  |  | RBS-3816 |  |
| - TPC combination index |  |  |  |  | 0 |  |  | RBS-3817 |  |
| - E-AGCH Info |  |  |  |  |  |  |  | RBS-3818 |  |
| - E-AGCH Channelisation Code |  |  |  |  | 10 |  |  | RBS-3819 |  |
| - CHOICE E-HICH Information |  |  |  |  |  |  |  | RBS-3820 |  |
| - E-HICH Information |  |  |  |  |  |  |  | RBS-3821 |  |
| - Channelisation code |  |  |  |  | 4 |  |  | RBS-3822 |  |
| - Signature sequence |  |  |  |  | 1 |  |  | RBS-3823 |  |
| - CHOICE E-RGCH Information |  |  |  |  |  |  |  | RBS-3824 |  |
| - E-RGCH Information |  |  |  |  |  |  |  | RBS-3825 |  |
| - Signature Sequence |  |  |  |  | 0 |  |  | RBS-3826 |  |
| - RG combination index |  |  |  |  | 0 |  |  | RBS-3827 |  |
| Downlink information for each radio link list |  |  | A22 |  |  | Rel-7 |  | RBS-3828 |  |
| - Downlink information for each radio link |  |  |  |  |  |  |  | RBS-3829 |  |
| - Choice mode |  |  |  |  | FDD |  |  | RBS-3830 |  |
| - Primary CPICH info |  |  |  |  |  |  |  | RBS-3831 |  |
| - Primary scrambling code |  |  |  |  | Ref. to the Default setting in clause 6.1 (FDD) |  |  | RBS-3832 |  |
| - Serving HS-DSCH radio link indicator |  |  |  |  | TRUE |  |  | RBS-3833 |  |
| - Serving E-DCH radio link indicator |  |  |  |  | TRUE |  |  | RBS-3834 |  |
| - Downlink DPCH info for each RL |  |  |  |  | Not Present |  |  | RBS-3835 |  |
| - Downlink F-DPCH info for each RL |  |  |  |  |  |  |  | RBS-3836 |  |
| - Primary CPICH usage for channel estimation |  |  |  |  | Primary CPICH may be used |  |  | RBS-3837 |  |
| - F-DPCH frame offset |  |  |  |  | Set to value Default DPCH Offset Value (as currently stored in SS) mod 38 400 |  |  | RBS-3838 |  |
| - F-DPCH slot format |  |  |  |  | 3 if UE supports enhanced F-DPCH, otherwise Not Present | Rel-7 |  | RBS-3839 |  |
| - Secondary CPICH info |  |  |  |  | Not Present |  |  | RBS-3840 |  |
| - Secondary scrambling code |  |  |  |  | Not Present |  |  | RBS-3841 |  |
| - Code number |  |  |  |  | 12 |  |  | RBS-3842 |  |
| - TPC combination index |  |  |  |  | 0 |  |  | RBS-3843 |  |
| - E-AGCH Info |  |  |  |  |  |  |  | RBS-3844 |  |
| - E-AGCH Channelisation Code |  |  |  |  | 10 |  |  | RBS-3845 |  |
| - CHOICE E-HICH Information |  |  |  |  |  |  |  | RBS-3846 |  |
| - E-HICH Information |  |  |  |  |  |  |  | RBS-3847 |  |
| - Channelisation code |  |  |  |  | 4 |  |  | RBS-3848 |  |
| - Signature sequence |  |  |  |  | 1 |  |  | RBS-3849 |  |
| - CHOICE E-RGCH Information |  |  |  |  |  |  |  | RBS-3850 |  |
| - E-RGCH Information |  |  |  |  | Not present |  |  | RBS-3851 |  |
| Downlink information for each radio link list |  |  | A6, A24 |  | Not Present |  |  | RBS-3852 |  |
|  |  |  | A29 |  |  |  |  | RBS-3853 |  |
| Downlink secondary cell info FDD |  |  | A25a, |  |  | Rel-8 |  | RBS-3854 |  |
| - CHOICE Configuration info |  |  |  |  | New configuration |  |  | RBS-3855 |  |
| - New H-RNTI |  |  |  |  | '1010 1010 1010 1010' |  |  | RBS-3856 |  |
| - Downlink 64QAM configured |  |  |  |  | Not Present |  |  | RBS-3857 |  |
| - HS-DSCH TB size table |  |  |  |  | Octet Aligned |  |  | RBS-3858 |  |
| - Primary CPICH info |  |  |  |  |  |  |  | RBS-3859 |  |
| - Primary scrambling code |  |  |  |  | Ref. to the Default setting in clause 6.1 (FDD) |  |  | RBS-3860 |  |
| - DL Scrambling Code |  |  |  |  | Not Present |  |  | RBS-3861 |  |
| - HS-SCCH Channelisation Code Information |  |  |  |  |  |  |  | RBS-3862 |  |
| - HS-SCCH Channelisation Code |  |  |  |  | 7 |  |  | RBS-3863 |  |
| - Measurement Power Offset |  |  |  |  | 6 dB |  |  | RBS-3864 |  |
| - UARFCN downlink (Nd) |  |  |  |  | Reference to clause 5.1 Test frequencies |  |  | RBS-3865 |  |
| -Secondary cell MIMO parameters |  |  |  |  | Not Present | Rel-9 |  | RBS-3866 |  |
| Downlink secondary cell info FDD | A25, A25b, A25c, A31 A34 |  |  |  |  | Rel-8 Rel-9 Rel-10 |  | RBS-3867 |  |
|  |  |  |  |  |  |  |  | RBS-3868 |  |
| - CHOICE Configuration info |  |  |  |  | New configuration |  |  | RBS-3869 |  |
| - New H-RNTI |  |  |  |  | '1010 1010 1010 1010' |  |  | RBS-3870 |  |
| - Downlink 64QAM configured |  |  |  |  | Not Present |  |  | RBS-3871 |  |
| - HS-DSCH TB size table |  |  |  |  | Not Present |  |  | RBS-3872 |  |
| - Primary CPICH info |  |  |  |  |  |  |  | RBS-3873 |  |
| - Primary scrambling code |  |  |  |  | Ref. to the Default setting in clause 6.1 (FDD) |  |  | RBS-3874 |  |
| - DL Scrambling Code |  |  |  |  | Not Present |  |  | RBS-3875 |  |
| - HS-SCCH Channelisation Code Information |  |  |  |  |  |  |  | RBS-3876 |  |
| - HS-SCCH Channelisation Code |  |  |  |  | 7 |  |  | RBS-3877 |  |
| - Measurement Power Offset |  |  |  |  | 6 dB |  |  | RBS-3878 |  |
| - UARFCN downlink (Nd) |  |  |  |  | Reference to clause 5.1 Test frequencies |  |  | RBS-3879 |  |
| -Secondary cell MIMO parameters |  |  |  |  |  | Rel-9 |  | RBS-3880 |  |
| - CHOICE Configuration info |  |  |  |  |  |  |  | RBS-3881 |  |
| - Continue |  |  |  |  |  |  |  | RBS-3882 |  |
| - New configuration |  |  |  |  |  |  |  | RBS-3883 |  |
| - MIMO N_cqi_typeA/M_cqi ratio |  |  |  |  | 1/1 |  |  | RBS-3884 |  |
| - MIMO pilot configuration |  |  |  |  |  |  |  | RBS-3885 |  |
| -CHOICE Second CPICH pattern |  |  |  |  |  |  |  | RBS-3886 |  |
| -Antenna2 P-CPICH |  |  |  |  | No data |  |  | RBS-3887 |  |
| -Antenna1 S-CPICH |  |  |  |  |  |  |  | RBS-3888 |  |
| -Channelisation code |  |  |  |  | 15 |  |  | RBS-3889 |  |
| -Power Offset for S-CPICH for MIMO |  |  |  |  | 0 |  |  | RBS-3890 |  |
| Downlink secondary cell info FDD | A32 A33 |  |  |  |  | Rel-9 Rel-10 |  | RBS-3891 |  |
| - CHOICE Configuration info |  |  |  |  | New configuration |  |  | RBS-3892 |  |
| - New H-RNTI |  |  |  |  | '1010 1010 1010 1010' |  |  | RBS-3893 |  |
| - Downlink 64QAM configured |  |  |  |  | TRUE |  |  | RBS-3894 |  |
| - HS-DSCH TB size table |  |  |  |  | Not Present |  |  | RBS-3895 |  |
| - Primary CPICH info |  |  |  |  |  |  |  | RBS-3896 |  |
| - Primary scrambling code |  |  |  |  | Ref. to the Default setting in clause 6.1 (FDD) |  |  | RBS-3897 |  |
| - DL Scrambling Code |  |  |  |  | Not Present |  |  | RBS-3898 |  |
| - HS-SCCH Channelisation Code Information |  |  |  |  |  |  |  | RBS-3899 |  |
| - HS-SCCH Channelisation Code |  |  |  |  | 7 |  |  | RBS-3900 |  |
| - Measurement Power Offset |  |  |  |  | 6 dB |  |  | RBS-3901 |  |
| - UARFCN downlink (Nd) |  |  |  |  | Reference to clause 5.1 Test frequencies |  |  | RBS-3902 |  |
| - Secondary cell MIMO parameters |  |  |  |  |  | Rel-9 |  | RBS-3903 |  |
| - CHOICE Configuration info |  |  |  |  |  |  |  | RBS-3904 |  |
| - Continue |  |  |  |  |  |  |  | RBS-3905 |  |
| - New configuration |  |  |  |  |  |  |  | RBS-3906 |  |
| - MIMO N_cqi_typeA/M_cqi ratio |  |  |  |  | 1/1 |  |  | RBS-3907 |  |
| - MIMO pilot configuration |  |  |  |  |  |  |  | RBS-3908 |  |
| -CHOICE Second CPICH pattern |  |  |  |  |  |  |  | RBS-3909 |  |
| -Antenna2 P-CPICH |  |  |  |  | No data |  |  | RBS-3910 |  |
| -Antenna1 S-CPICH |  |  |  |  |  |  |  | RBS-3911 |  |
| -Channelisation code |  |  |  |  | 15 |  |  | RBS-3912 |  |
| -Power Offset for S-CPICH for MIMO - precodingWeightSetRestriction | A33, A34 |  |  |  | 0 | Rel-10 |  | RBS-3913 |  |
| MBMS PL Service Restriction Information |  |  | A1, A2, A3, A4, A5, A6, A7, A8 |  | Not Present |  |  | RBS-3914 |  |
|  |  |  | A9, A10 |  |  | Rel-5 |  | RBS-3915 |  |
|  |  |  | A12, A13, A14, A15, A16 |  |  | Rel-6 |  | RBS-3916 |  |
|  |  |  | A17, A17a, A17b, A17c, A18, A19, A19a, A20, A21, A22 |  |  | Rel-7 |  | RBS-3917 |  |
|  |  |  | , A23, A24, A28a |  |  | Rel-7 Rel-8 |  | RBS-3918 |  |
|  |  |  | , A25, A25a, A26, A27, A27a, A28, A29, A30 |  |  | Rel-8 |  | RBS-3919 |  |
|  |  |  | , A31, A32 A33, A34 |  |  | Rel-9 Rel-10 |  | RBS-3920 |  |

| Condition | Explanation | Version |
| --- | --- | --- |
| A1 | This IE is needed for "Non speech to CELL_DCH from CELL_DCH in CS" |  |
| A2 | This IE is needed for "Speech to CELL_DCH from CELL_DCH in CS" |  |
| A3 | This IE is needed for "Packet to CELL_DCH from CELL_DCH in PS" |  |
| A4 | This IE is needed for "Packet to CELL_DCH from CELL_FACH in PS" |  |
| A5 | This IE is needed for "Packet to CELL_FACH from CELL_DCH in PS" |  |
| A6 | This IE is needed for "Packet to CELL_FACH from CELL_FACH in PS" |  |
| A7 | This IE is needed for "Non speech to CELL_DCH from CELL_FACH in CS" |  |
| A8 | This IE is needed for "Speech to CELL_DCH from CELL_FACH in CS" |  |
| A9 | This IE is needed for "Packet to CELL_DCH / HS-DSCH using three multiplexing options", or when not stated otherwise, for "Packet to CELL_DCH / HS-DSCH from CELL_DCH in PS" | Rel-5 |
| A10 | This IE is needed for "Packet to CELL_DCH / HS-DSCH using one multiplexing option", or when not stated otherwise, for "Packet to CELL_DCH / HS-DSCH from CELL_FACH in PS" | Rel-5 |
| A11 | This IE is needed for " Packet RAB Setup after Speech RAB Setup in CELL_DCH" |  |
| A12 | This IE is needed for "Packet to CELL_DCH / E-DCH / HS-DSCH using three multiplexing options (3/3) and SRBs mapped on DCH/DCH" | Rel-6 |
| A13 | This IE is needed for "Packet to CELL_DCH / E-DCH / HS-DSCH using one multiplexing option (1/1) and SRBs mapped on E-DCH/DCH" | Rel-6 |
| A14 | This IE is needed for "Packet to CELL_DCH / E-DCH / HS-DSCH using one multiplexing option (1/1) and SRBs mapped on E-DCH/HS-DSCH" | Rel-6 |
| A15 | This IE is needed for "Packet to CELL_DCH / E-DCH / HS-DSCH with multiple RABs (two streaming/interactive/background) using one multiplexing option (1/1) and SRBs mapped on E-DCH/DCH” | Rel-6 |
| A16 | This IE is needed for "Packet to CELL_DCH / E-DCH / HS-DSCH with multiple RABs (one conversational and one streaming/interactive/background) using one multiplexing option (1/1) and SRBs mapped on E-DCH/HS-DSCH” | Rel-6 |
| A17 | This IE is needed for "Packet to CELL_DCH / HS-DSCH with enhanced data rate and RLC AM" | Rel-7 |
| A17a | This IE is needed for "Packet to CELL_DCH / HS-DSCH [DL : 64QAM] with enhanced data rate and RLC AM" | Rel-7 |
| A17b | This IE is needed for "Packet to CELL_DCH / HS-DSCH with enhanced data rate and RLC AM using one multiplexing option (1/1) and SRBs mapped on E-DCH/HS-DSCH (MAC-ehs)" | Rel-7 |
| A17c | This IE is needed for "Packet to CELL_DCH / HS-DSCH [DL : 64QAM] with enhanced data rate and RLC AM using one multiplexing option (1/1) and SRBs mapped on E-DCH/HS-DSCH (MAC-ehs)" | Rel-7 |
| A17d | This IE is needed for "Packet to CELL_DCH / HS-DSCH with enhanced data rate and RLC AM using one multiplexing option (1/1) and SRBs mapped on E-DCH/DCH (MAC-ehs)" | Rel-7 |
| A17e | This IE is needed for "Packet to CELL_DCH / HS-DSCH [DL : 64QAM] with enhanced data rate and RLC AM using one multiplexing option (1/1) and SRBs mapped on E-DCH/DCH (MAC-ehs)" | Rel-7 |
| A18 | This IE is needed for "Packet to CELL_DCH / HS-DSCH with enhanced data rate and RLC UM" | Rel-7 |
| A19 | This IE is needed for "Packet to CELL_DCH / E-DCH[UL : 16QAM] / HS-DSCH using three multiplexing options (3/3) and SRBs mapped on DCH/DCH" | Rel-7 |
| A19a | This IE is needed for "Packet to CELL_DCH / E-DCH[UL : 16QAM] / HS-DSCH using one multiplexing option (1/1) and SRBs mapped on E-DCH/DCH" | Rel-7 |
| A20 | This IE is needed for "Packet to CELL_DCH / E-DCH / HS-DSCH with DTX/DRX using one multiplexing option (1/1) and SRBs mapped on E-DCH/HS-DSCH" | Rel-7 |
| A21 | This IE is needed for "Packet to CELL_DCH / E-DCH / HS-DSCH with DTX/DRX and multiple RABs (one conversational and one streaming/interactive/background) using one multiplexing option (1/1) and SRBs mapped on E-DCH/HS-DSCH” | Rel-7 |
| A22 | This IE is needed for "Packet to CELL_DCH / E-DCH / HS-DSCH with multiple RABs (one conversational and one streaming/interactive/background) with enhanced data rate using one multiplexing option (1/1) and SRBs mapped on E-DCH/HS-DSCH” | Rel-7 |
| A23 | This IE is needed for “Speech to CELL_DCH / E-DCH / HS-DSCH CS RAB with DTX/DRX and enhanced data rate using one multiplexing option (1/1) and SRBs mapped on E-DCH/HS-DSCH” . | Rel-7 Rel-8 (Note 1) |
| A24 | This IE is needed for “Packet to CELL_FACH from CELL_FACH using one multiplexing option (1/1) and SRBs mapped on RACH/HS-DSCH” | Rel-7 |
| A25 | This IE is needed for "Packet to CELL_DCH / E-DCH / HS-DSCH [Dual Carrier Adjacent Channels] with enhanced data rate using one multiplexing option (1/1) and SRBs mapped on E-DCH( MACe/es ) /HS-DSCH”" | Rel-8 |
| A25a | This IE is needed for “Packet to CELL_DCH / HS-DSCH [Dual Carrier Adjacent Channels] with enhanced data rate and RLC AM and SRBs mapped on DCH/DCH” | Rel-8 |
| A25b | This IE is needed for "Packet to CELL_DCH / E-DCH ( MACe/es ) / HS-DSCH [Dual Carrier Adjacent Channels] with enhanced data rate using one multiplexing option (1/1) and SRBs mapped on E-DCH/DCH” | Rel-8 |
| A25c | This IE is needed for "Packet to CELL_DCH / E-DCH ( MACi/is ) [Dual-Cell] / HS-DSCH [Dual Carrier Adjacent Channels] with enhanced data rate using one multiplexing option (1/1) and SRBs mapped on E-DCH/DCH” | Rel-9 |
| A26 | This IE is needed for "UM Packet to CELL_DCH / E-DCH (MAC-i/is) / HS-DSCH (MAC-ehs) with multiple RABs (three streaming/interactive/background) using one multiplexing option (1/1) and SRBs mapped on E-DCH (MAC-i/is)/DCH” | Rel-8 |
| A27 | This IE is needed for "UM Packet to CELL_DCH / E-DCH (MAC-i/is) / HS-DSCH (MAC-ehs) using one multiplexing option (1/1) and SRBs mapped on E-DCH (MAC-i/is)/HS-DSCH (MAC-ehs)" | Rel-8 |
| A27a | This IE is needed for "UM Packet to CELL_DCH / E-DCH [UL : 16QAM] (MAC-i/is) / HS-DSCH (MAC-ehs) using one multiplexing option (1/1) and SRBs mapped on E-DCH (MAC-i/is)/HS-DSCH” | Rel-8 |
| A28 | This IE is needed for "Packet to CELL_DCH / HS-DSCH [DL : 64QAM+MIMO] with enhanced data rate and RLC AM" | Rel-8 |
| A28a | This IE is needed for "Packet to CELL_DCH / HS-DSCH [DL : 16QAM+MIMO] with enhanced data rate and RLC AM" | Rel-7 |
| A29 | This IE is needed for "AM Packet to Enhanced CELL_FACH from Enhanced CELL_FACH in PS with SRBs mapped on common E-DCH/HS-DSCH" | Rel-8 |
| A30 | This IE is needed for “AM Packet to CELL_DCH from Enhanced CELL_FACH in PS with SRBs mapped on E-DCH (MAC-i/is)/HS-DSCH(MAC-ehs)” | Rel-8 |
| A31 | This IE is needed for "Packet to CELL_DCH / HS-DSCH [UL: E-DCH DL: DC-HSDPA and MIMO] with enhanced data rate and RLC AM" | Rel-9 |
| A32 | This IE is needed for "Packet to CELL_DCH / HS-DSCH [UL: E-DCH DL: 64QAM, DC-HSDPA and MIMO] with enhanced data rate and RLC AM" | Rel-9 |
| A33 | This IE is needed for "Packet to CELL_DCH / HS-DSCH [UL: E-DCH DL: 64QAM, 4C-HSDPA and MIMO] with enhanced data rate and RLC AM" | Rel-10 |
| A34 | This IE is needed for "Packet to CELL_DCH / HS-DSCH [UL: E-DCH DL: 16QAM, 4C-HSDPA and MIMO] with enhanced data rate and RLC AM" | Rel-10 |

NOTE 1: Support depends on the UE capability: Support for CS voice over HSPA. This is supported in Rel-8 and may be supported in Rel-7.

| Condition | Explanation | Version |
| --- | --- | --- |
| MAC-I-FIXED | Used with other condition when MAC-i/is with Fixed RLC PDU size is configured | Rel-8 |
| MAC-I-FLEX | Used with other condition when MAC-i/is with Flexible RLC PDU size is configured | Rel-8 |