3GPP TSG-RAN WG5 Meeting #62 	R5-140225

# Prague, Czech Republic, 10 - 14 February 2014

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | CRNum | rev | - | Current version: | 12.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to TC 7.3A.2 for CA_3C |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom，CATR |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_Rel12-UEConTest |  |  |  |  |  |  | Date: |  | 2014-01-21 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | A new WI proposal of Further REL-12 Configurations for LTE Advanced Carrier Aggregation was agreed in RAN#61 meeting and updates for new CA combinations should be available to technical specifications.Currently, test case of Reference sensitivity level for CA_3C is still missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Initial conditions for 20MHz +5MHz were added to test configuration table 7.3A.2.4.1-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TC 7.3A.2 will not be available to CA_3C. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

7.3A.2	Reference sensitivity level for CA (intra-band contiguous DL CA without UL CA)

7.3A.2.1	Test purpose

Same as in clause 7.3A.1.1.

7.3A.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 10 and forward that support intra-band contiguous DL CA and but no UL CA.

7.3A.2.3	Minimum conformance requirements

Same as in clause 7.3A.1.3.

7.3A.2.4	Test description

7.3A.2.4.1	Initial conditions

Same as in clause 7.3A.1.4.1 with the following exceptions:

-	Instead of Table 7.3A.1.4.1-1 use Table 7.3A.2.4.1-1.

Table 7.3A.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  | C: Low range, High range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest NRB_agg, Highest NRB_agg |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation |  |  | UL Allocation |  |  |  |  |  |  |
| PCC NRB | SCCs NRB | CC MOD |  | PCC & SCC RB allocation | CC MOD | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |  |
| 75 | 75 | QPSK |  | 75+75 | QPSK | 75 | P_75@0 |  | - | - | - |
| 75 | 75 | QPSK |  | 75+75 | QPSK | 50 | P_50@0 |  | - | - | - |
| 75 | 75 | QPSK |  | 75+75 | QPSK | 25 | P_25@0 |  | - | - | - |
| 75 | 75 | QPSK |  | 75+75 | QPSK | 20 | P_20@0 |  | - | - | - |
| 100 | 25 | QPSK |  | 100+25 | QPSK | 50 | P_50@0 |  | - | - | - |
| 100 | 25 | QPSK |  | 100+25 | QPSK | 25 | P_25@0 |  | - | - | - |
| 100 | 25 | QPSK |  | 100+25 | QPSK | 20 | P_20@0 |  | - | - | - |
| 100 | 50 | QPSK |  | 100+50 | QPSK | 100 | P_100@0 |  | - | - | - |
| 100 | 50 | QPSK |  | 100+50 | QPSK | 75 | P_75@0 |  | - | - | - |
| 100 | 50 | QPSK |  | 100+50 | QPSK | 50 | P_50@0 |  | - | - | - |
| 100 | 50 | QPSK |  | 100+50 | QPSK | 25 | P_25@0 |  | - | - | - |
| 100 | 50 | QPSK |  | 100+50 | QPSK | 20 | P_20@0 |  | - | - | - |
| 100 | 100 | QPSK |  | 100+100 | QPSK | 100 | P_100@0 |  | - | - | - |
| 100 | 100 | QPSK |  | 100+100 | QPSK | 75 | P_75@0 |  | - | - | - |
| 100 | 100 | QPSK |  | 100+100 | QPSK | 50 | P_50@0 |  | - | - | - |
| 100 | 100 | QPSK |  | 100+100 | QPSK | 25 | P_25@0 |  | - | - | - |
| 100 | 100 | QPSK |  | 100+100 | QPSK | 20 | P_20@0 |  | - | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1 and Table 5.4.2A.1-2. Note 2: Depending on CA configurations, only the appropriate Uplink RB allocation value according to table 7.3.3-2 for UE supporting one uplink carrier is tested per Test CA configuration. |  |  |  |  |  |  |  |  |  |  |  |

7.3A.2.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.1 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 7.3A.1.4.3

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS transmits PDSCH via PDCCH DCI format 1A for C_RNTI to transmit the DL RMC according to Tables 7.3A.2.4.1-1 on both PCC and SCC. The SS sends downlink MAC padding bits on the DL RMC.

5.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 7.3A.2.4.1-1 on PCC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

6.	Set the Downlink signal level for PCC and SCC to the appropriate REFSENS value defined in Table 7.3A.1.5-1. Send continuously uplink power control "up" commands in every uplink scheduling information to the UE to ensure the UE transmits PUMAX level for at least the duration of the Throughput measurement.

7.	Measure the average throughput for each component carrier for a duration sufficient to achieve statistical significance according to Annex G.2A.

7.3A.2.4.3	Message contents

Same as in clause 7.3A.1.4.3.

7.3A.2.5	Test requirement

Same as in clause 7.3A.1.5.

<End of modified section>