3GPP TSG-RAN WG5 Meeting #65	R5-145188

# Venice, Italy, 10th – 14th November 2014

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.123-1 | CR | CRNum | rev | - | Current version: | 11.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Multiflow Test Case 8.2.1.45.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm, TF 160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | HSDPA_MFTX-UEConTest |  |  |  |  |  |  | Date: |  | 2014-10-15 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-11 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Clarification of Initial conditions and Test Procedure is needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updated Initial conditions for System Simulator and UE. - Clarification of Test Procedure 3. Editorial Corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test Case will be specified incorrectly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.45.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

##### 8.2.1.45.3	Radio Bearer Establishment  for transition between CELL_DCH and CELL_DCH with SRBs mapped on DCH: Successful Activation of 3 cells/two frequencies Multiflow HSDPA for Dual Bands, intra-NodeB

8.2.1.45.3.1	Definition and applicability

UE categories (Cat 29, or Cat 30) which support combination of Multiflow HSDPA on Dual Frequency/3 Cell on Dual Band..

UE supports FDD.

8.2.1.45.3.2	Conformance requirement

Same as Subclause 8.2.1.45.1.2

8.2.1.45.3.3	Test purpose

To confirm that the UE establishes radio bearer mapped to HS-DSCH, transits from CELL_DCH to CELL_DCH state, and starts simultaneous 16QAM reception over multiflow HSDPA operation (Dual Frequency/3 Cell Multiflow  HSDPA for Dual band)  according to the received RADIO BEARER SETUP message.

8.2.1.45.3.4	Method of test

Initial Condition

System Simulator:

SS configures Cells 2 and 3 as per Table 5.0aC of TS 25.101 [21] and TS 34.108 [9]], clause 5.1.1..

3 cells –Cell 1/2/3 with Cell 1 (f1) (Anchor Carrier), and Cell 2 (f1) (Assisting Carrier) and Cell 3(f2) (Assisting Secondary Carrier) – One Anchor Carrier and One Assisting Carriers on Band A, and One Assisting Secondary Carrier on Band B. (Dual Band DF-3C). Assisting Carrier and Assisting Secondary Carrier have the same radio frame timing. Tcell of Cell 2 is set to 3840 chips (1.5 slots).

UE: CELL_DCH (state 6-7)  as specified in clause 7.4 of TS 34.108 [9], depending on the CN domain(s) supported by the UE. The procedure P9a in TS 34.108 [9], clause 7.4.2.4a is executed to add Cell 2 in the active set.

Related ICS/IXIT statement(s)

-	UE supports FDD

-	UE category supports Multiflow HSDPA operation on 3 cells on dual frequency (UE Category 29-30)

Test Procedure

The UE is in CELL_DCH state. Before step 1, only signalling radio bearers have been established. The SS transmits a RADIO BEARER SETUP message to the UE. This message includes RAB information to configure PS domain RB on E-DCH/HSDPA (with DF-3C-HSDPA) and 16 QAM simultaneously configured for L1). SRBs are  mapped to DCH/DCH. After the UE receives this message, it configures RAB accordingly and transmits a RADIO BEARER SETUP COMPLETE message using AM RLC.

SS sends UE CAPABILITY ENQUIRY on Anchor Carrier and Assisting Carrier Cells using SRB2 mapped onto DCH. The UE shall respond to the message with UE CAPABILITY INFORMATION message sent to Anchor Carrier Cell 1. In that RRC message, it shall include the IE “HS-DSCH physical layer category extension4”. The SS completes the procedure by transmitting a UE CAPABILITY INFORMATION CONFIRM message.

To check that the UE has activated Multiflow HSDPA operation, SS checks that UE sends CQI reports  for Cell 1, Cell 2 and Cell 3.

SS calls for generic procedure C.3 to check that UE is in CELL_DCH state.

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
|  |  |  |  | SS configures three cells. |
| 1 |  |  | RADIO BEARER SETUP | Configuration of PS RB on E-DCH/HSDPA , using Multiflow HSDPA, and 16QAM configuration |
| 2 |  |  | RADIO BEARER SETUP COMPLETE |  |
| 2a |  |  |  | SS sends an HS-SCCH order to activate the assisting secondary serving cell 3. |
| 3 |  |  | UE CAPABILITY ENQUIRY | Use default message. SRB2 on DCH from Anchor Cell 1 and Cell 2.. |
| 4 |  |  | UE CAPABILITY INFORMATION |  |
| 5 |  |  | UE CAPABILITY INFORMATION CONFIRM |  |
| 6 |  |  |  | SS checks for 2 CQI reports at 4 ms interval with: - CQI Report for Anchor Carrier (Cell 1) - concatenated CQI Report for Assisting Carrier (Cell 2) - CQI Report for Assisting Secondary Carrier (Cell 3) |
| 7 |  |  |  | SS calls for generic procedure C.3 to check that UE is in CELL_DCH |

Specific Message Contents

RADIO BEARER SETUP (Step 1)

PS_DCCH+DTCH E_DCH/HS_DSCH (state 6-18) under condition A42, as specified in clause 7.4 of TS 34.108 [9] using Dual band Dual Frequency combination with 3 cells.

8.2.1.45.3.5	Test requirements

Same as Subclause 8.2.1.45.2.5