3GPP TSG-RAN WG5 Meeting #67 	R5-151356

# Fukuoka, Japan, 25-29 May 2015

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 2947 | rev | - | Current version: | 12.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add pre-test steps to check the band capability of UE in MFBI protocol test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATR |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI9_Test |  |  |  |  |  |  | Date: |  | 2015-05-12 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | For the MFBI protocol test cases, it is important to check the UE radio capability before test.If the px_OverlappingNotSupportedFrequencyBandMFBI band is included in the UE radio capability, the test procedures of these test cases, in the test specification now ,can not verify the behavor of UE to deal with the IE “multiBandInfoList”, though the running result of test case is “PASS”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For the pre-test condition of UE, add the step to check the UE radio capability in the message sent from UE during the register procedure,such as “UECapabilityInformation”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MFBI test cases can not verify the behavor of UE to deal with the IE “multiBandInfoList” correctly in some cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.19, 6.1.2.20, 6.1.2.21, 6.2.3.34, 6.2.3.35,8.2.4.22, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

6.1.2.19	Intra-frequency cell reselection / MFBI

6.1.2.19.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state }

ensure that {

when { an equal priority E-UTRAN Intra-frequency neighbouring cell which has been included in the multiBandInfoList provided by the serving

cell becomes available, and, is better ranked than the serving cell during a time interval TreselectionRAT, and, more than 1 second has elapsed since the UE camped on the current serving cell }

then { the UE reselects the new cell }

}

6.1.2.19.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304, clause 5.2.4.2 and 5.2.4.6, TS 36.331, clause 5.2.2.7 and 6.3.1. Unless otherwise stated these are Rel-8 requirements.

[TS 36.304, clause 5.2.4.2]

When evaluating for reselection purposes cell selection criterion or SnonServingCell, x, the UE shall use parameters provided by the serving cell.

Following rules are used by the UE to limit needed measurements:

-	If Sintrasearch is sent in the serving cell and SServingCell > Sintrasearch, UE may choose to not perform intra-frequency measurements.

-	If SServingCell <= Sintrasearch, or Sintrasearch is not sent in the serving cell UE shall perform intra-frequency measurements.

-	The UE shall apply the following rules for E-UTRAN inter-frequencies and inter-RAT frequencies which are indicated in system information and for which the UE has priority provided as defined in 5.2.4.1:

-	For an E-UTRAN inter-frequency or inter-RAT frequency with a reselection priority higher than the reselection priority of the current E-UTRA frequency the UE shall perform measurements of higher priority E-UTRAN inter-frequency inter-RAT frequencies according to [10].

-	For an E-UTRAN inter-frequency with an equal or lower reselection priority than the reselection priority of the current E-UTRA frequency and for inter-RAT frequency with lower reselection priority than the reselection priority of the current E-UTRAN frequency:

-	If Snonintrasearch is sent in the serving cell and SServingCell > Snonintrasearch UE may choose not to perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority.

-	If SServingCell <= Snonintrasearch or Snonintrasearch is not sent in the serving cell  the UE shall perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority according to [10].

Where SServingCell is the Srxlev-value of the serving cell.

[TS 36.304, clause 5.2.4.6]

The cell-ranking criterion Rs for serving cell and Rn for neighbouring cells is defined by:

where:

| Qmeas | RSRP measurement quantity used in cell reselections. |
| --- | --- |
| Qoffset | For intra-frequency: Equals to Qoffsets,n, if Qoffsets,n is valid, otherwise this equals to zero. For inter-frequency: Equals to Qoffsets,n plus Qoffsetfrequency, if Qoffsets,n is valid, otherwise this equals to Qoffsetfrequency. |

The UE shall perform ranking of all cells that fulfil the cell selection criterion S, which is defined in 5.2.3.2, but may exclude all CSG cells that are known by the UE to be not allowed.

The cells shall be ranked according to the R criteria specified above, deriving Qmeas,n and Qmeas,s and calculating the R values using averaged RSRP results.

If a cell is ranked as the best cell the UE shall perform cell re-selection to that cell. If this cell is found to be not-suitable, the UE shall behave according to subclause 5.2.4.4.

In all cases, the UE shall reselect the new cell, only if the following conditions are met:

-	the new cell is better ranked than the serving cell during a time interval TreselectionRAT

-	more than 1 second has elapsed since the UE camped on the current serving cell.

[TS 36.331, clause 5.2.2.7]

Upon receiving the SystemInformationBlockType1 either via broadcast or via dedicated signalling, the UE shall:

1>	if in RRC_CONNECTED while T311 is not running, and the UE supports multi-band cells as defined by bit 31 in featureGroupIndicators:

2>	disregard the freqBandIndicator and multiBandInfoList, if received, while in RRC_CONNECTED;

2>	forward the cellIdentity to upper layers;

2>	forward the trackingAreaCode to upper layers;

1>	else:

2>	if the frequency band indicated in the freqBandIndicator is part of the frequency bands supported by the UE; or

2>	if the UE supports multiBandInfoList, and if one or more of the frequency bands indicated in the multiBandInfoList are part of the frequency bands supported by the UE:

3>	forward the cellIdentity to upper layers;

3>	forward the trackingAreaCode to upper layers;

2>	else:

3>	consider the cell as barred in accordance with TS 36.304 [4]; and

3>	perform barring as if intraFreqReselection is set to notAllowed, and as if the csg-Indication is set to FALSE;

[TS 36.331, clause 6.3.1]

-	SystemInformationBlockType2

The IE SystemInformationBlockType2 contains radio resource configuration information that is common for all UEs.

NOTE:	UE timers and constants related to functionality for which parameters are provided in another SIB are included in the corresponding SIB.

…

SystemInformationBlockType2 field descriptions

multiBandInfoList A list of additionalSpectrumEmission i.e. one for each additional frequency band included in multiBandInfoList in SystemInformationBlockType1, listed in the same order.

6.1.2.19.3	Test description

6.1.2.19.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 2 have different tracking areas according to table 6.0.1-2 and are MFBI capable cells.

-	Cell 1 belongs to the absolute centre frequency which overlaps between bands controlled by IXITs px_MFBI_FrequencyBand and px_OverlappingNotSupportedFrequencyBandMFBI.

-	Cell 2 belongs to another absolute centre frequency which again overlaps between bands controlled by IXITs px_MFBI_FrequencyBand and px_OverlappingNotSupportedFrequencyBandMFBI.

-	System information combination 1 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) on Cell 1 (serving cell) according to [18], and it has been checked that the px_OverlappingNotSupportedFrequencyBandMFBI band is not included in UE radio capability during register procedure.

6.1.2.19.3.2	Test procedure sequence

Table 6.1.2.19.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The configuration marked "T1" is applied at the point indicated in the Main behaviour description in Table 6.1.2.19.3.2-2.

Table 6.1.2.19.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | The power level values are set so that RCell 1 < RCell 2. |

Table 6.1.2.19.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | Wait 1 second. (to ensure than 1 second has elapsed since the UE camped on the current serving cell) | - | - | - | - |
| 1 | The SS changes the cells power level setting according to the row "T1" in table 6.1.2.19.3.2-1. | - | - | - | - |
| 2 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 2? NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | 1 | - |

6.1.2.19.3.3	Specific message contents

Table 6.1.2.19.3.3-1: SystemInformationBlockType1 for Cell 1 and Cell 2 (preamble and all steps, Table 6.1.2.19.3.2-2)

| Derivation Path: 36.508 table 4.4.3.2-3 |  |  |
| --- | --- | --- |
| Information Element |  | Value/remark | Comment |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| freqBandIndicator |  | An overlapping not supported frequency band MFBI under test (px_OverlappingNotSupportedFrequencyBandMFBI). | Cell 1 Cell 2 |
| lateNonCriticalExtension |  |  |  |
| SystemInformationBlockType1-v8h0-IEs ::= SEQUENCE { |  |  |  |
| multiBandInfoList SEQUENCE { |  |  |  |
| freqBandIndicator |  | An overlapping Band under test (px_MFBI_FrequencyBand). | Cell 1 Cell 2 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.19.3.3-2: SystemInformationBlockType2 for Cell 1 and Cell 2 (preamble and all steps, Table 6.1.2.19.3.2-2)

| Derivation Path: 36.508 table 4.4.3.3-1 |  |  |
| --- | --- | --- |
| Information Element |  | Value/remark | Comment |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| lateNonCriticalExtension |  |  |  |
| SystemInformationBlockType2-v8h0-IEs ::= SEQUENCE { |  |  |  |
| multiBandInfoList SEQUENCE { |  |  |  |
| AdditionalSpectrumEmission |  | 1 (NS_01) | A-MPR doesn’t apply by default. See TS 36.101 table 6.2.4-1. |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.1.2.20	Inter-frequency cell reselection / MFBI

6.1.2.20.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state }

ensure that {

when { an equal priority E-UTRAN inter-frequency neighbouring cell which has been included in the multiBandInfoList provided by the serving cell becomes available, and, is better ranked than the serving cell during a time interval TreselectionRAT, and, more than 1 second has elapsed since the UE camped on the current serving }

then { the UE reselects the new cell }

}

6.1.2.20.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304, clause 5.2.4.6 and TS 36.331, clause 6.3.1. Unless otherwise stated these are Rel-8 requirements.

[TS 36.304, clause 5.2.4.6]

The cell-ranking criterion Rs for serving cell and Rn for neighbouring cells is defined by:

where:

| Qmeas | RSRP measurement quantity used in cell reselections. |
| --- | --- |
| Qoffset | For intra-frequency: Equals to Qoffsets,n, if Qoffsets,n is valid, otherwise this equals to zero. For inter-frequency: Equals to Qoffsets,n plus Qoffsetfrequency, if Qoffsets,n is valid, otherwise this equals to Qoffsetfrequency. |

The UE shall perform ranking of all cells that fulfil the cell selection criterion S, which is defined in 5.2.3.2, but may exclude all CSG cells that are known by the UE to be not allowed.

The cells shall be ranked according to the R criteria specified above, deriving Qmeas,n and Qmeas,s and calculating the R values using averaged RSRP results.

If a cell is ranked as the best cell the UE shall perform cell reselection to that cell. If this cell is found to be not-suitable, the UE shall behave according to subclause 5.2.4.4.

In all cases, the UE shall reselect the new cell, only if the following conditions are met:

-	the new cell is better ranked than the serving cell during a time interval TreselectionRAT;

-	more than 1 second has elapsed since the UE camped on the current serving cell.

[TS 36.331, clause 6.3.1]

-	SystemInformationBlockType5

The IE SystemInformationBlockType5 contains information relevant only for inter-frequency cell re-selection i.e. information about other E-UTRA frequencies and inter-frequency neighbouring cells relevant for cell re-selection. The IE includes cell re-selection parameters common for a frequency as well as cell specific re-selection parameters.

…

SystemInformationBlockType5 field descriptions

multiBandInfoList Indicates the list of frequency bands in addition to the band represented by dl-CarrierFreq for which cell reselection parameters are common. E-UTRAN indicates at most maxMultiBands frequency bands (i.e. the total number of entries across both multiBandInfoList and multiBandInfoList-v9e0 is below this limit).

6.1.2.20.3	Test description

6.1.2.20.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 3 have different tracking areas according to table 6.0.1-2 and are MFBI capable cells.

-	Cell 1 belongs to the absolute centre frequency which overlaps between bands controlled by IXITs px_MFBI_FrequencyBand and px_OverlappingNotSupportedFrequencyBandMFBI.

-	Cell 3 belongs to another absolute centre frequency which again overlaps between bands controlled by IXITs px_MFBI_FrequencyBand and px_OverlappingNotSupportedFrequencyBandMFBI.

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) on Cell 1 (serving cell) according to [18], and it has been checked that the px_OverlappingNotSupportedFrequencyBandMFBI band is not included in UE radio capability during register procedure.

6.1.2.20.3.2	Test procedure sequence

Table 6.1.2.20.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The configuration marked "T1" is applied at the point indicated in the Main behaviour description in Table 6.1.2.20.3.2-2.

Table 6.1.2.20.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -73 | The power level values are set so that RCell 1 < RCell 2. |

Table 6.1.2.20.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | Wait 1 second (to ensure than 1 second has elapsed since the UE camped on the current serving cell) | - | - | - | - |
| 1 | The SS changes the cells power level setting according to the row "T1" in table 6.1.2.20.3.2-1. | - | - | - | - |
| 2 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 3? NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | 1 | - |

6.1.2.20.3.3	Specific message contents

Table 6.1.2.20.3.3-1: SystemInformationBlockType1 for cell 1 and cell 3 (preamble)

| Derivation Path: 36.508 table 4.4.3.2-3 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Information Element |  |  | Value/remark |  | Comment |  |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |  |  |  |
| freqBandIndicator |  |  | An overlapping not supported frequency band MFBI under test (px_OverlappingNotSupportedFrequencyBandMFBI). |  | Cell 1 Cell 3 |  |
| lateNonCriticalExtension |  |  |  |  |  |  |
| SystemInformationBlockType1-v8h0-IEs ::= SEQUENCE { |  |  |  |  |  |  |
| multiBandInfoList SEQUENCE { |  |  |  |  |  |  |
| freqBandIndicator |  |  | An Overlapping Band under test (px_MFBI_FrequencyBand). |  | Cell 1 Cell 3 |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |

Table 6.1.2.20.3.3-1A: SystemInformationBlockType2 for Cell 6 and Cell3 (preamble and all steps, Table 6.1.2.20.3.2-2)

| Derivation Path: 36.508 table 4.4.3.3-1 |  |  |
| --- | --- | --- |
| Information Element |  | Value/remark | Comment |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| lateNonCriticalExtension |  |  |  |
| SystemInformationBlockType2-v8h0-IEs ::= SEQUENCE { |  |  |  |
| multiBandInfoList SEQUENCE { |  |  |  |
| AdditionalSpectrumEmission |  | 1 (NS_01) | A-MPR doesn’t apply by default. See TS 36.101 table 6.2.4-1. |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.20.3.3-2: SystemInformationBlockType5 for cell 1 and cell 3 (preamble)

| Derivation Path: 36.508 table 4.4.3.3-4 |  |  |
| --- | --- | --- |
| Information Element |  | Value/remark | Comment |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| lateNonCriticalExtension |  |  |  |
| SystemInformationBlockType5-v8h0-IEs ::= SEQUENCE { |  |  |  |
| InterFreqCarrierFreqList-v8h0 SEQUENCE { |  |  |  |
| InterFreqCarrierFreqInfo-v8h0 SEQUENCE { |  |  |  |
| multiBandInfoList SEQUENCE { |  |  |  |
| freqBandIndicator |  | An Overlapping Bandunder test (px_MFBI_FrequencyBand). | Cell 1 Cell 3 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.1.2.21	Inter-band cell reselection / MFBI

6.1.2.21.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state }

ensure that {

when { an equal priority E-UTRAN another band neighbouring cell which has been included in the multiBandInfoList provided by the serving cell becomes available, and, is better ranked than the serving cell during a time interval TreselectionRAT, and, more than 1 second has elapsed since the UE camped on the current serving cell }

then { the UE reselects the new cell }

}

6.1.2.21.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304, clause 5.2.4.5 and 5.2.4.6; TS 36.331, clause 6.3.1. Unless otherwise stated these are Rel-8 requirements.

[TS 36.304, clause 5.2.4.5]

Cell reselection to a cell on an equal priority E-UTRAN frequency shall be based on ranking for Intra-frequency cell reselection as defined in sub-clause 5.2.4.6.

[TS 36.304, clause 5.2.4.6]

The cell-ranking criterion Rs for serving cell and Rn for neighbouring cells is defined by:

where:

| Qmeas | RSRP measurement quantity used in cell reselections. |
| --- | --- |
| Qoffset | For intra-frequency: Equals to Qoffsets,n, if Qoffsets,n is valid, otherwise this equals to zero. For inter-frequency: Equals to Qoffsets,n plus Qoffsetfrequency, if Qoffsets,n is valid, otherwise this equals to Qoffsetfrequency. |

The UE shall perform ranking of all cells that fulfil the cell selection criterion S, which is defined in 5.2.3.2, but may exclude all CSG cells that are known by the UE to be not allowed.

The cells shall be ranked according to the R criteria specified above, deriving Qmeas,n and Qmeas,s and calculating the R values using averaged RSRP results.

If a cell is ranked as the best cell the UE shall perform cell reselection to that cell. If this cell is found to be not-suitable, the UE shall behave according to subclause 5.2.4.4.

In all cases, the UE shall reselect the new cell, only if the following conditions are met:

-	the new cell is better ranked than the serving cell during a time interval TreselectionRAT;

-	more than 1 second has elapsed since the UE camped on the current serving cell.

[TS 36.331, clause 6.3.1]

-	SystemInformationBlockType5

The IE SystemInformationBlockType5 contains information relevant only for inter-frequency cell re-selection i.e. information about other E-UTRA frequencies and inter-frequency neighbouring cells relevant for cell re-selection. The IE includes cell re-selection parameters common for a frequency as well as cell specific re-selection parameters.

…

SystemInformationBlockType5 field descriptions

multiBandInfoList Indicates the list of frequency bands in addition to the band represented by dl-CarrierFreq for which cell reselection parameters are common. E-UTRAN indicates at most maxMultiBands frequency bands (i.e. the total number of entries across both multiBandInfoList and multiBandInfoList-v9e0 is below this limit).

6.1.2.21.3	Test description

6.1.2.21.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 10 have different tracking areas according to table 6.0.1-2.

-	Cell 10 is MFBI capable cell.

-	The E-UTRA Primary frequency band under test (px_ePrimaryFrequencyBand) is supported by the UE.

-	Cell 10 belongs to the absolute centre frequency which overlaps between bands controlled by IXITs px_MFBI_FrequencyBand and px_OverlappingNotSupportedFrequencyBandMFBI.

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) on Cell 1 (serving cell) according to [18], and it has been checked that the px_OverlappingNotSupportedFrequencyBandMFBI band is not included in UE radio capability during register procedure.

6.1.2.21.3.2	Test procedure sequence

Table 6.1.2.21.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The configuration marked "T1" is applied at the point indicated in the Main behaviour description in Table 6.1.2.21.3.2-2.

Table 6.1.2.21.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 10 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -73 | Cell 10 become stronger than Threshx, high. |

Table 6.1.2.21.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | Wait 1 second. (to ensure than 1 second has elapsed since the UE camped on the current serving cell) | - | - | - | - |
| 1 | The SS changes the cells power level setting according to the row "T1" in table 6.1.2.21.3.2-1. | - | - | - | - |
| 2 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 10 belonging to the MFBI band (px_MFBI_FrequencyBand)? NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | 1 | - |

6.1.2.21.3.3	Specific message contents

Table 6.1.2.21.3.3-0: Conditions for specific message contents
in Tables 6.1.2.21.3.3-3 and 6.1.2.21.3.3-4

| Condition | Explanation |
| --- | --- |
| Cell 1 | This condition applies to system information transmitted on Cell 1. |
| Cell 10 | This condition applies to system information transmitted on Cell 10. |

6.2.3.34	Inter-RAT cell reselection from E-UTRA to UTRA / MFBI

6.2.3.34.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state }

ensure that {

when { an higher priority UTRAN neighbouring cell which has been included in the multiBandInfoList provided by the serving cell becomes available, and, is better ranked than the serving cell during a time interval TreselectionRAT, and, more than 1 second has elapsed since the UE camped on the current serving cell }

then { UE reselects the UTRA cell }

}

6.2.3.34.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304, clause 5.2.4.5.

[TS 36.304, clause 5.2.4.5]

Criteria 1: the SnonServingCell,x of a cell on evaluated frequency is greater than Threshx, high during a time interval TreselectionRAT;

Cell reselection to a cell on a higher priority E-UTRAN frequency or inter-RAT frequency than serving frequency shall be performed if:

-	A cell of a higher priority E-UTRAN frequency or inter-RAT frequency fulfils criteria 1; and

- more than 1 second has elapsed since the UE camped on the current serving cell.

…

For GERAN, UTRAN, and E-UTRAN, SnonServingCell,x is the Srxlev-value of an evaluated cell. ...

[TS 36.331, clause 6.3.1]

-	SystemInformationBlockType6

The IE SystemInformationBlockType6 contains information relevant only for inter-RAT cell re-selection i.e. information about UTRA frequencies and UTRA neighbouring cells relevant for cell re-selection. The IE includes cell re-selection parameters common for a frequency.

…

SystemInformationBlockType6 field descriptions

multiBandInfoList Indicates the list of frequency bands in addition to the band represented by carrierFreq in the CarrierFreqUTRA-FDD for which UTRA cell reselection parameters are common.

6.2.3.34.3	Test description

6.2.3.34.3.1	Pre-test conditions

System Simulator:

-	2 cells, one E-UTRA and one UTRA cell :

-	Cell 1 is a E-UTRA cell and is an MFBI capable cell

-	Cell 5 is a UTRA cell and is an MFBI capable cell

-	Cell 1 belongs to the absolute centre frequency which overlaps between bands controlled by IXITs px_MFBI_FrequencyBand and px_OverlappingNotSupportedFrequencyBandMFBI.

-	Cell 5 belongs to absolute centre frequency which overlaps between bands controlled by IXITs px_MFBI_UTRAN_FrequencyBand and px_UTRAN_OverlappingNotSupportedFrequencyBandMFBI.

-	System information combination 4 as defined in TS 36.508 [18] clause 4.4.3.1 is used in the E-UTRA cell.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) on Cell 1 (serving cell) according to [18], and it has been checked that both px_OverlappingNotSupportedFrequencyBandMFBI and px_UTRAN_OverlappingNotSupportedFrequencyBandMFBI bands are not included in UE radio capability during register procedure..

6.2.3.34.3.2	Test procedure sequence

Table 6.2.3.34.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Subsequent configuration marked "T1" is applied at the points indicated in the Main behaviour description in Table 6.2.3.34.3.2-2.

Table 6.2.3.34.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 5 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | - |  |
|  | CPICH_Ec | dBm/3.84 MHz | - | -65 | S nonServingCell, Cell5 > Thresh Cell 5,high |
|  | P-CCPCH | dBm/1.28 MHz |  | -67 |  |

Table 6.2.3.34.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | Wait for 6 s for UE to receive system information. | - | - | - | - |
| 1 | The SS changes Cell 1 and Cell 5 levels according to the row "T1" in table 6.2.3.34.3.2-1. | - | - | - | - |
| 2 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.8 indicate that the UE is camped on UTRAN Cell 5? NOTE: The UE performs an RAU procedure and the RRC connection is released. | - | - | 1 | - |

6.2.3.34.3.3	Specific message contents

Table 6.2.3.34.3.3-1: SystemInformationBlockType1 for cell 1 (preamble)

| Derivation Path: 36.508 table 4.4.3.2-3 |  |  |
| --- | --- | --- |
| Information Element |  | Value/remark | Comment |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| freqBandIndicator |  | An overlapping not supported frequency band MFBI under test (px_OverlappingNotSupportedFrequencyBandMFBI). |  |
| lateNonCriticalExtension |  |  |  |
| SystemInformationBlockType1-v8h0-IEs ::= SEQUENCE { |  |  |  |
| multiBandInfoList SEQUENCE { |  |  |  |
| freqBandIndicator |  | Overlapping Bands for Cell 1 under test (px_MFBI_FrequencyBand). | Cell 1 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.34.3.3-2: SystemInformationBlockType6 for Cell 1 (preamble and all steps, Table 6.2.3.34.3.2-2)

| Derivation Path: 36.508 table 4.4.3.3-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType6 ::= SEQUENCE { |  |  |  |
| carrierFreqListUTRA-FDD SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF SEQUENCE { |  |  | UTRA-FDD |
| carrierFreq[n] | Same downlink UARFCN as used for cell 5 |  |  |
| cellReselectionPriority[n] | 5 | Higher priority than E-UTRA |  |
| } |  |  |  |
| carrierFreqListUTRA-TDD SEQUENCE (SIZE (1..maxUTRA-TDD-Carrier)) OF SEQUENCE { |  |  | UTRA-TDD |
| carrierFreq[n] | Same downlink UARFCN as used for cell 5 |  |  |
| cellReselectionPriority[n] | 5 | Higher priority than E-UTRA |  |
| } |  |  |  |
| } |  |  |  |
| SystemInformationBlockType6-v8h0-IEs ::= SEQUENCE { |  |  |  |
| CarrierFreqListUTRA-FDD-v8h0 SEQUENCE { |  |  |  |
| CarrierFreqInfoUTRA-FDD-v8h0 SEQUENCE { |  |  |  |
| multiBandInfoList SEQUENCE { |  |  |  |
| FreqBandIndicator-UTRA-FDD | Overlapping Bands for Cell 5 under test (px_MFBI_FrequencyBand). | Cell 5 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

Table 6.2.3.34.3.3-3: System Information Block type 19 for Cell 5 (preamble and all steps, Table 6.2.3.34.3.2-2)

| Derivation Path: 36.508 table 4.4.4.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| utra-PriorityInfoList ::= SEQUENCE { |  |  |  |
| utra-ServingCell ::= SEQUENCE { |  |  |  |
| Priority | 5 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.34.3.3-4: System Information Block type 5 for Cell 5 (preamble and all steps, Table 6.2.3.34.3.2-2)

| Derivation Path: 25.331 clause 10.2.48.8.8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Frequency Band Indicator | Not Present |  | A1 |
|  | px_UTRAN_OverlappingNotSupportedFrequencyBandMFBI |  | A2 |
|  | Extension indicator |  | A3 or A4 |
| Frequency Band Indicator 2 | Not Present |  | A1 |
|  | Not Present |  | A2 |
|  | px_UTRAN_OverlappingNotSupportedFrequencyBandMFBI |  | A3 |
|  | Extension indicator |  | A4 |
| Frequency Band Indicator 3 | Not Present |  | A1 |
|  | Not Present |  | A2 |
|  | Not Present |  | A3 |
|  | px_UTRAN_OverlappingNotSupportedFrequencyBandMFBI |  | A4 |
| Multiple Frequency Band indicator list | px_MFBI_UTRAN_FrequencyBand |  |  |

| Condition | Explanation |
| --- | --- |
| A1 | Band I, Band II, Band III |
| A2 | Band V, Band VI, Band VII |
| A3 | Band VIII & bands beyond Band X |
| A4 | Beyond BandXXII |

Table 6.2.3.34.3.3-5: System Information Block type 5bis for Cell 5 (preamble and all steps, Table 6.2.3.34.3.2-2)

| Derivation Path: 25.331 clause 10.2.48.8.8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Frequency Band Indicator | px_UTRAN_OverlappingNotSupportedFrequencyBandMFBI |  | A1 |
|  | Extension indicator |  | A2 |
| Frequency Band Indicator 2 | Not Present |  | A1 |
|  | px_UTRAN_OverlappingNotSupportedFrequencyBandMFBI |  | A2 |
| Multiple Frequency Band indicator list | px_MFBI_UTRAN_FrequencyBand |  |  |

| Condition | Explanation |
| --- | --- |
| A1 | Band IV |
| A2 | Band IX and Band X |

6.2.3.35	Inter-RAT cell reselection from UTRA to E-UTRA / MFBI

6.2.3.35.1	Test Purpose (TP)

(1)

with { UE in UTRA RRC_IDLE state }

ensure that {

when { the cell re-selection criteria is met for the E-UTRA cell included in the Multiple E-UTRA frequency info list }

then { the UE performs cell reselection to the E-UTRA cell }

}

6.2.3.35.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 25.331, clause 8.6.7.3c. Unless otherwise stated these are Rel-10 requirements.

[TS 25.331, clause 8.6.7.3c]

If the IE "E-UTRA frequency and priority info list" is received in System Information Block Type 19, the UE shall:

1>	for each occurrence of the IE "E-UTRA frequency and priority":

2>	create a new entry in the IE "Frequency Info List" in the variable EUTRA_FREQUENCY_INFO_LIST, and in that new entry:

3>	store the IEs "EARFCN" and "Measurement bandwidth";

3>	if the IE "Blacklisted cells per freq list" is present in the IE "E-UTRA frequency and priority":

4>	store the blacklisted cells information.

2>	if the UE supports multi-band signalling and the UE does not support the E-UTRA Frequency Band associated with the EARFCN in the IE "EARFCN", but does understand the EARFCN downlink (Nd):

3>	if the IE "Multiple E-UTRA frequency info list" is present for this occurrence of "E-UTRA frequency and priority":

4>	for the first entry in the IE "Multiple E-UTRA frequency band indicator list" which the UE supports, the UE shall:

5>	extract the supported band and compute the corresponding EARFCN downlink (Nd) value;

5>	overwrite the previously stored EARFCN value with the computed EARFCN downlink (Nd) value.

6.2.3.35.3	Test description

6.2.3.35.3.1	Pre-test conditions

System Simulator:

-	Cell 1 is a E-UTRA cell and is an MFBI capable cell.

-	Cell 5

-	System information combination 4 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA Cell 1.

-	Cell 1 belongs to the absolute centre frequency which overlaps between bands controlled by IXITs px_MFBI_FrequencyBand and px_OverlappingNotSupportedFrequencyBandMFBI.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode (state 3 or 7) on Cell 5 according to [5], and it has been checked that the px_OverlappingNotSupportedFrequencyBandMFBI band is not included in UE radio capability during register procedure.

6.2.3.35.3.2	Test procedure sequence

Table 6.2.3.35.3.2-1 and  Table 6.2.3.35.3.2-2 illustrate the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions, while row marked "T1"  is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 6.2.3.35.3.2-1: Time instances of cell power level and parameter changes for E-UTRA cell

|  | Parameter | Unit | Cell 1 | Remark |
| --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | “Off” | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1. |
|  | Qrxlevmin | dBm | -106 |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 |  |
|  | Srxlev | dB | 21 | Srxlev = Cell-specific RS EPRE - qRxLevMinEUTRA |

Table 6.2.3.35.3.2-2: Time instances of cell power level and parameter changes for UTRA cell

|  | Parameter | Unit | Cell 5 | Remark |
| --- | --- | --- | --- | --- |
| T0 | CPICH_Ec | dBm/3.84 MHz | -60 |  |
|  | P-CCPCH | dBm/1.28 MHz | -62 |  |
|  | Qrxlevmin (FDD) | dBm | -79 | Default value |
|  | Qrxlevmin(TDD) | dBm | -81 | Default value |
|  | Srxlev | dB | 19 |  |
| T1 | CPICH_Ec | dBm/3.84 MHz | -90 |  |
|  | P-CCPCH | dBm/1.28 MHz | -92 |  |
|  | Srxlev | dB | -11 |  |

Table 6.2.3.35.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS change power levels according to the row "T1" in tables 6.2.3.35.3.2-1 and 6.2.3.35.3.2-2. | - | - | - | - |
| 2 | Check: Does the test result of the generic test procedure in TS 36.508 Table 6.4.2.7A-1 indicate that the UE is camped on E-UTRAN Cell 1? NOTE: The UE performs a TAU procedure and the RRC Connection is released. | - | - | 1 | - |

6.2.3.35.3.3	Specific message contents

Table 6.2.3.35.3.3-1: System Information Block type 19 for Cell 5 (Pre-test conditions and all steps, Table 6.2.3.35.3.2-3

| Derivation Path: 36.508 clause 4.4.4.1, Table 4.4.4.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| eutra-FrequencyAndPriorityInfoList SEQUENCE { |  |  |  |
| earfcn | Associated with an overlapping not supported frequency band MFBI under test (px_OverlappingNotSupportedFrequencyBandMFBI). |  |  |
| } |  |  |  |
| v920NonCriticalExtensions SEQUENCE { |  |  |  |
| sysInfoType19-v920ext SEQUENCE {} |  |  |  |
| va80NonCriticalExtensions SEQUENCE { |  |  |  |
| sysInfoType19-va80ext SEQUENCE { |  |  |  |
| MultipleEutraFrequencyInfoList SEQUENCE { |  |  |  |
| MultipleEUTRAFrequencyBandInfo SEQUENCE { |  |  |  |
| MultipleEUTRAFrequencyBandIndicatorList SEQUENCE { |  |  |  |
| RadioFrequencyBandEUTRA | An overlapping Band under test (px_MFBI_FrequencyBand). |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| vb30NonCriticalExtensions SEQUENCE {} |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.21.3.3-1: SystemInformationBlockType1 for Cell 10 (preamble and all steps, Table 6.1.2.21.3.2-2)

| Derivation Path: 36.508 table 4.4.3.2-3 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |  |  |
| freqBandIndicator |  | An overlapping not supported frequency band MFBI under test (px_OverlappingNotSupportedFrequencyBandMFBI). |  | Cell 10 |  |
| lateNonCriticalExtension |  |  |  |  |  |
| SystemInformationBlockType1-v8h0-IEs ::= SEQUENCE { |  |  |  |  |  |
| multiBandInfoList SEQUENCE { |  |  |  |  |  |
| freqBandIndicator |  | An overlapping Band under test (px_MFBI_FrequencyBand). |  | Cell 10 |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |  |

Table 6.1.2.21.3.3-2: SystemInformationBlockType2 for Cell 10 (preamble and all steps, Table 6.1.2.21.3.2-2)

| Derivation Path: 36.508 table 4.4.3.3-1 |  |  |
| --- | --- | --- |
| Information Element |  | Value/remark | Comment |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| lateNonCriticalExtension |  |  |  |
| SystemInformationBlockType2-v8h0-IEs ::= SEQUENCE { |  |  |  |
| multiBandInfoList SEQUENCE { |  |  |  |
| AdditionalSpectrumEmission |  | 1 (NS_01) | A-MPR doesn’t apply by default. See TS 36.101 table 6.2.4-1. |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.21.3.3-3: SystemInformationBlockType3 for Cell 1 and Cell 10 (preamble and all steps, Table 6.1.2.21.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| cellReselectionPriority | 1 |  | Cell 1 |
|  | 5 |  | Cell 10 |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.21.3.3-4: SystemInformationBlockType5 for Cell 1 and Cell 10 (preamble and all steps, Table 6.1.2.21.3.2-2)

| Derivation Path: 36.508 table 4.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |  |
| dl-CarrierFreq |  | Same downlink EARFCN as used for Cell 10 |  | Cell 1 |
|  |  | Same downlink EARFCN as used for Cell 1 |  | Cell 10 |
| t-ReselectionEUTRA |  | 7 |  |  |
| threshX-High |  | 10 | 20dB |  |
| cellReselectionPriority |  | 5 |  | Cell 1 |
|  |  | 1 |  | Cell 10 |
| } |  |  |  |  |
| lateNonCriticalExtension |  |  |  | Cell 1 |
| SystemInformationBlockType5-v8h0-IEs ::= SEQUENCE { |  |  |  | Cell 1 |
| InterFreqCarrierFreqList-v8h0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  | Cell 1 |
| InterFreqCarrierFreqInfo-v8h0 [n] SEQUENCE { |  |  |  | Cell 1 |
| multiBandInfoList SEQUENCE { |  |  |  | Cell 1 |
| freqBandIndicator |  | An overlapping Band for Cell 10 under test (px_MFBI_FrequencyBand). |  | Cell 1 |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

8.2.4.22	RRC connection reconfiguration / Handover / MFBI / Target cell broadcasting information disregarded by the UE

8.2.4.22.1	Test Purpose (TP)

(1)

with {UE in RRC_CONNECTED and T311 is not running, and the UE supports multi-band cells as defined by bit 31 in featureGroupIndicators}

ensure that {
  when { UE receives freqBandIndicator and multiBandInfoList from SystemInformationBlockType1 and additionalSpectrumEmission and ul-CarrierFreq from SystemInformationBlockType2 }

then { the UE disregard the related system information fields and continues connected mode operations}

}

8.2.4.22.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clauses 5.2.2.7, 5.2.2.9. Unless otherwise stated these are Rel-8 requirements.

[TS 36.331, clause 5.2.2.7]

Upon receiving the SystemInformationBlockType1 either via broadcast or via dedicated signalling, the UE shall:

1>	if in RRC_CONNECTED while T311 is not running, and the UE supports multi-band cells as defined by bit 31 in featureGroupIndicators:

2>	disregard the freqBandIndicator and multiBandInfoList, if received, while in RRC_CONNECTED;

2>	forward the cellIdentity to upper layers;

2>	forward the trackingAreaCode to upper layers;

1>	else:

2>	if the frequency band indicated in the freqBandIndicator is part of the frequency bands supported by the UE; or

2>	if the UE supports multiBandInfoList, and if one or more of the frequency bands indicated in the multiBandInfoList are part of the frequency bands supported by the UE:

3>	forward the cellIdentity to upper layers;

3>	forward the trackingAreaCode to upper layers;

2>	else:

3>	consider the cell as barred in accordance with TS 36.304 [4]; and

3>	perform barring as if intraFreqReselection is set to notAllowed, and as if the csg-Indication is set to FALSE;

[TS 36.331, clause 5.2.2.9]

Upon receiving SystemInformationBlockType2, the UE shall:

1>	apply the configuration included in the radioResourceConfigCommon;

1>	if upper layers indicate that a (UE specific) paging cycle is configured:

2>	apply the shortest of the (UE specific) paging cycle and the defaultPagingCycle included in the radioResourceConfigCommon;

1>	if the mbsfn-SubframeConfigList is included:

2>	consider that DL assignments may occur in the MBSFN subframes indicated in the mbsfn-SubframeConfigList under the conditions specified in [23, 7.1];

1>	apply the specified PCCH configuration defined in 9.1.1.3;

1>	not apply the timeAlignmentTimerCommon;

1>	if in RRC_CONNECTED and UE is configured with RLF timers and constants values received within rlf-TimersAndConstants:

2>	not update its values of the timers and constants in ue-TimersAndConstants except for the value of timer T300;

1>	if in RRC_CONNECTED while T311 is not running; and the UE supports multi-band cells as defined by bit 31 in featureGroupIndicators:

2>	disregard the additionalSpectrumEmission and ul-CarrierFreq, if received, while in RRC_CONNECTED;

8.2.4.22.3	Test description

8.2.4.22.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 2 are MFBI capable cells.

-	Cell 1 belongs to the absolute centre frequency which overlaps between bands controlled by IXITs px_MFBI_FrequencyBand and px_OverlappingNotSupportedFrequencyBandMFBI.

-	Cell 2 belongs to another absolute centre frequency which again overlaps between bands controlled by IXITs px_MFBI_FrequencyBand and px_OverlappingNotSupportedFrequencyBandMFBI.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18], and it has been checked that the px_OverlappingNotSupportedFrequencyBandMFBI band is not included in UE radio capability during register procedure.

8.2.4.22.3.2	Test procedure sequence

Table 8.2.4.22.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.2.4.22.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 | The power level values are such that measurement results for Cell 1 (M1) and Cell 2 (M2) satisfy exit condition for event A3 (M2 < M1). |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -79 | The power level values are such that measurement results for Cell 1 (M1) and Cell 2 (M2) satisfy entry condition for event A3 (M2 > M1). |

Table 8.2.4.22.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to setup intra frequency measurement. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS changes Cell 1 and Cell 2 parameters according to the row "T1" in table 8.2.4.22.3.2-1. | - | - | - | - |
| 4 | The UE transmits a MeasurementReport message on Cell 1 to report event A3 with the measured RSRP, RSRQ value for Cell 2. | --> | MeasurementReport | - | - |
| 5 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to order the UE to perform intra frequency handover to Cell 2. | <-- | RRCConnectionReconfiguration | - | - |
| 6 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message on Cell 2 using common preamble to confirm the successful completion of the intra frequency handover? | --> | RRCConnectionReconfigurationComplete | - | - |
| 7 | Wait 10 secs to ensure that the UE receives system information from the Cell 2 and ignores freqBandIndicator and multiBandInfoList from SystemInformationBlockType1 and additionalSpectrumEmission and ul-CarrierFreq from SystemInformationBlockType2 as the UE is in connected mode. | - | - | - | - |
| 8 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state on Cell 2? | - | - | 1 | P |

8.2.4.22.3.3	Specific message contents

Table 8.2.4.22.3.3-0A: SystemInformationBlockType1 for Cell 1 and Cell 2 (preamble and all steps, Table 8.2.4.22.3.2-2)

| Derivation Path: 36.508 table 4.4.3.2-3 |  |  |
| --- | --- | --- |
| Information Element |  | Value/remark | Comment |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| freqBandIndicator |  | An overlapping not supported frequency band MFBI under test (px_OverlappingNotSupportedFrequencyBandMFBI). | Cell 1 Cell 2 |
| lateNonCriticalExtension |  |  |  |
| SystemInformationBlockType1-v8h0-IEs ::= SEQUENCE { |  |  |  |
| multiBandInfoList SEQUENCE { |  |  |  |
| freqBandIndicator |  | An overlapping Band under test (px_MFBI_FrequencyBand). | Cell 1 Cell 2 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.22.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.2.4.22.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 8.2.4.22.3.3-2: MeasConfig (Table 8.2.4.22.3.3-1)

| Derivation Path: 36.508, Table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A3 |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.22.3.3-2A: MeasObjectEUTRA-GENERIC (Table 8.2.4.22.3.3-2)

| Derivation Path: 36.508 clause 4.6.6 table 4.6.6-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectEUTRA-GENERIC ::= SEQUENCE { |  |  |  |
| carrierFreq SEQUENCE {} | Corresponding Downlink EARFCN of the Band in px_MFBI_FrequencyBand |  |  |
| } |  |  |  |

Table 8.2.4.22.3.3-3: MeasurementReport (step 4, Table 8.2.4.22.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 2 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.4.22.3.3-4: RRCConnectionReconfiguration (step 5, Table 8.2.4.22.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition HO

Table 8.2.4.22.3.3-5: MobilityControlInfo (Table 8.2.4.22.3.3-4)

| Derivation Path: 36.508 clause 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 2 |  |  |
| carrierFreq | Not present |  |  |
| AdditionalSpectrumEmission | 1 (NS_01) | Correct value for Cell 2 is transmitted. |  |
| } |  |  |  |

Table 8.2.4.22.3.3-6: SystemInformationBlockType2 for Cell 2 (step 7, Table 8.2.4.22.3.2-2)

| Derivation Path: 36.508 table 4.4.3.3-1 |  |  |
| --- | --- | --- |
| Information Element |  | Value/remark | Comment |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| freqInfo SEQUENCE { |  |  |  |
| AdditionalSpectrumEmission |  | 4 (NS_04) | When in connected mode, UE shall ignore the AdditionalSpectrumEmission value if transmitted. To check this a incorrect value is transmitted. |
| } |  |  |  |
| lateNonCriticalExtension |  |  |  |
| SystemInformationBlockType2-v8h0-IEs ::= SEQUENCE { |  |  |  |
| multiBandInfoList SEQUENCE { |  |  |  |
| AdditionalSpectrumEmission |  | 5 (NS_05) | When in connected mode, UE shall ignore the AdditionalSpectrumEmission value if transmitted. To check this a incorrect value is transmitted. |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

< End of changes >