3GPP TSG RAN WG5 Meeting #71	R5-162529

# Nanjing, China, 23 - 27, May 2016

| Title: | Discussion for new A-MPR for NS_05 |
| --- | --- |
| Source: | KDDI |
| Agenda Item: | 5.4.1.11 |
| Document for: | Discussion and endorsement |

1	Introduction

In RAN4#78 meeting, new A-MPR for NS_05 was endorsed [1]. In this paper, we discuss the test case modifications for new A-MPR.

2	Discussion

Firstly the new A-MPR for NS_05 is explained. In [1], new A-MPR is defined as follows. The modified parts are changed to yellow.

Table 1: Additional Maximum Power Reduction (A-MPR) (Table 6.2.4-1 in [1])

| Network Signalling value | Requirements (subclause) | E-UTRA Band | Channel bandwidth (MHz) | Resources Blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| … |  |  |  |  |  |
| NS_04 | 6.6.2.2.2, 6.6.3.3.19 | 41 | 5, 10, 15, 20 | Table 6.2.4-4 |  |
| NS_05 | 6.6.3.3.1 | 1 | 10,15,20 | ≥ 50 | ≤ 1 (NOTE1) |
|  |  |  | 15, 20 | Table 6.2.4-18 (NOTE2) |  |
| NS_06 | 6.6.2.2.3 | 12, 13, 14, 17 | 1.4, 3, 5, 10 | Table 5.6-1 | N/A |
| … |  |  |  |  |  |
| NOTE 1 Applicable when the lower edge of the assigned E-UTRA UL channel bandwidth frequency is larger than or equal to the upper edge of PHS band (1915.7 MHz) + 4 MHz + the channel BW assigned, where channel BW is as defined in subclause 5.6. For 10MHz channel bandwidth whose carrier frequency is larger than or equal to 1945 MHz or 15 MHz channel bandwidth whose carrier frequency is larger than or equal to 1947.5 MHz, no A-MPR applies. NOTE 2 Applicable when carrier frequency is 1932.5 MHz for 15MHz channel bandwidth or 1930 MHz for 20MHz channel bandwidth case. … |  |  |  |  |  |

Table 2: A-MPR for "NS_05" (Table 6.2.4-18 in [1])

| Channel Bandwidth [MHz] | Parameters |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | Fc [MHz] | 1932.5 |  |  |  |  |  |  |  |  |  |  |
|  | RBstart | 0-7 | 8 – 66 |  |  |  |  |  | 67-74 |  |  |  |
|  | LCRB [RBs] | ≥1 | ≤30 |  | 31 – 54 |  | > 54 |  | ≤6 |  | >6 |  |
|  | A-MPR [dB] | ≤11 | 0 |  | ≤3 |  | ≤5 |  | ≤5 |  | ≤1 |  |
| 20 | Fc [MHz] | 1930 |  |  |  |  |  |  |  |  |  |  |
|  | RBstart | 0-23 | 24-75 |  |  |  |  |  |  | 76-99 |  |  |
|  | LCRB [RBs] | ≥1 | ≤24 | 25 – 40 |  | 41 – 50 |  | > 50 |  | ≤6 |  | >6 |
|  | A-MPR [dB] | ≤11 | 0 | ≤3 |  | ≤5 |  | ≤10 |  | ≤5 |  | ≤1 |

The relationship among Release, modifiedMPRbehavior and support of new A-MPR is summarized in Table 3. It is apparent that the support of new A-MPR depends on whether the index field 1 in modifiedMPRbehavior, which is an IE of the UE Radio Access Capability, is set to 1 or not.

Table 3: Support of new A-MPR for NS_05

| Release | Index field 1 in modifiedMPRbehavior | Support of new A-MPR |
| --- | --- | --- |
| Rel-9 and earlier | - | No |
| Rel-10 and 11 | 0 | No |
|  | 1 | Yes |
| Rel-12 and later | 1 (UE supporting A-MPR associated to NS_05 for Band 1 shall set to 1.) | Yes |

It is straightforward to change the test cases described in the clause 6.2.4 and 6.6.3.3 in [2] depending on the index field 1 in modifiedMPRbehavior. The current spec has already supported to change the A-MPR test cases depending on the Release that UE supports, such as that for NS_12.  Then, we propose to modify the current description of specification in [2] to the followings in order to change the test cases depending on the modifiedMPRbehavior.

6.2.4	Additional Maximum Power Reduction (A-MPR)

<< Unchanged parts omitted >>

6.2.4.3	Minimum conformance requirements

<< Unchanged parts omitted >>

Table 6.2.4.3-1: Additional Maximum Power Reduction (A-MPR) / Spectrum Emission requirements

| Network Signalling value | Requirements (sub-clause) | E-UTRA Band | Channel bandwidth (MHz) | Resources Blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 | 6.6.2.1.1 | Table 5.2-1 | 1.4,3,5,10,15,20 | Table 5.4.2-1 | N/A |
| NS_03 | 6.6.2.2.3.1 | 2,4,10, 23, 25,35,36, 66 | 3 | >5 | ≤ 1 |
|  |  |  | 5 | >6 | ≤ 1 |
|  |  |  | 10 | >6 | ≤ 1 |
|  |  |  | 15 | >8 | ≤ 1 |
|  |  |  | 20 | >10 | ≤ 1 |
| NS_04 | 6.6.2.2.3.2 | 41 | 5 | >6 | ≤1 |
|  |  |  | 10, 15, 20 | Table 6.2.4.3-4 |  |
| NS_05 | 6.6.3.3.3.1 | 1 | 10,15,20 | ≥ 50 | ≤ 1 (NOTE1) |
|  |  |  | 15, 20 | Table 6.2.4-18 (NOTE2) |  |
| NS_06 | 6.6.2.2.3.3 | 12, 13, 14, 17 | 1.4, 3, 5, 10 | Table 5.4.2-1 | N/A |
| NS_07 | 6.6.2.2.3.3 6.6.3.3.3.2 | 13 | 10 | Table 6.2.4.3-2 | Table 6.2.4.3-2 |
| NS_08 | 6.6.3.3.3.3 | 19 | 10, 15 | > 44 | ≤ 3 |
| NS_09 | 6.6.3.3.3.4 | 21 | 10, 15 | > 40 | ≤ 1 |
|  |  |  |  | > 55 | ≤ 2 |
| NS_10 |  | 20 | 15, 20 | Table 6.2.4.3-3 | Table 6.2.4.3-3 |
| NS_11 | 6.6.2.2.1 6.6.3.3.13 | 231 | 1.4, 3, 5, 10,15,20 | Table 6.2.4.3-5 | Table 6.2.4.3-5 |
| NS_12 | 6.6.3.3.5 | 26 | 1.4, 3, 5, 10, 15 | Table 6.2.4.3-6, Table 6.2.4.3-6a | Table 6.2.4.3-6, Table 6.2.4.3-6a |
| NS_13 | 6.6.3.3.6 | 26 | 5 | Table 6.2.4.3-7 | Table 6.2.4.3-7 |
| NS_14 | 6.6.3.3.7 | 26 | 10, 15 | Table 6.2.4.3-8 | Table 6.2.4.3-8 |
| NS_15 | 6.6.3.3.8 | 26 | 1.4, 3, 5, 10, 15 | Table 6.2.4.3-9 Table 6.2.4.3-10 | Table 6.2.4.3-9, Table 6.2.4.3-10 |
| NS_16 | 6.6.3.3.9 | 27 | 3, 5, 10 | Table 6.2.4.3-11, Table 6.2.4.3-12, Table 6.2.4.3-13 |  |
| NS_17 | 6.6.3.3.10 | 28 | 5, 10 | Table 5.4.2-1 | N/A |
| NS_18 | 6.6.3.3.11 | 28 | 5 | ≥ 2 | ≤ 1 |
|  |  |  | 10, 15, 20 | ≥ 1 | ≤ 4 |
| NS_19 | 6.6.3.3.12 | 44 | 10, 15, 20 | Table 6.2.4.3-15 | Table 6.2.4.3-15 |
| NS_20 | 6.2.2 6.6.2.2.1 6.6.3.3.14 | 23 | 5, 10, 15, 20 | Table 6.2.4.3-14 | Table 6.2.4.3-14 |
| NS_21 | 6.6.2.2.3.1 6.6.3.3.3.15 | 30 | 5, 10 | Table 6.2.4.3-16 | Table 6.2.4.3-16 |
| ... |  |  |  |  |  |
| NS_32 | - | - | - | - | - |
| NOTE 1 Applicable when the lower edge of the assigned E-UTRA UL channel bandwidth frequency is larger than or equal to the upper edge of PHS band (1915.7 MHz) + 4 MHz + the channel BW assigned, where channel BW is as defined in subclause 5.6. A-MPR for operations below this frequency is not covered in this version of specifications except for the channel assignments in NOTE 2 as the emissions requirement in 6.6.3.3.1 may not be met. For 10MHz channel bandwidth whose carrier frequency is larger than or equal to 1945 MHz or 15 MHz channel bandwidth whose carrier frequency is larger than or equal to 1947.5 MHz, no A-MPR applies. NOTE2 Applicable when carrier frequency is 1932.5 MHz for 15MHz channel bandwidth or 1930 MHz for 20MHz channel bandwidth case. |  |  |  |  |  |

<< Unchanged parts omitted >>

Table 2: A-MPR for "NS_05" (Index field 1 in modifiedMPRbehavior is set to 1)

| Channel Bandwidth [MHz] | Parameters |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | Fc [MHz] | 1932.5 |  |  |  |  |  |  |  |  |  |  |
|  | RBstart | 0-7 | 8 – 66 |  |  |  |  |  | 67-74 |  |  |  |
|  | LCRB [RBs] | ≥1 | ≤30 |  | 31 – 54 |  | > 54 |  | ≤6 |  | >6 |  |
|  | A-MPR [dB] | ≤11 | 0 |  | ≤3 |  | ≤5 |  | ≤5 |  | ≤1 |  |
| 20 | Fc [MHz] | 1930 |  |  |  |  |  |  |  |  |  |  |
|  | RBstart | 0-23 | 24-75 |  |  |  |  |  |  | 76-99 |  |  |
|  | LCRB [RBs] | ≥1 | ≤24 | 25 – 40 |  | 41 – 50 |  | > 50 |  | ≤6 |  | >6 |
|  | A-MPR [dB] | ≤11 | 0 | ≤3 |  | ≤5 |  | ≤10 |  | ≤5 |  | ≤1 |

6.2.4.4	Test description

<< Unchanged parts omitted >>

Table 6.2.4.4.1-3: Test Configuration Table (network signalled value "NS_05")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | Normal |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Low range, Mid range In case of Low range: - For 5MHz Channel Bandwidth: 1927.2MHz (NUL = 18072) - For 10 MHz Channel Bandwidth: 1934.7 MHz (NUL = 18147) - For 15 MHz Channel Bandwidth: 1932.5 MHz (NUL = 18125) - For 20 MHz Channel Bandwidth: 1930 MHz (NUL = 18100) |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | 5MHz, 10MHz, 15MHz, 20MHz |  |
| Test Parameters for NS_05 A-MPR |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |
| Configuration ID | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD |
| 1 | 5MHz | N/A for A-MPR testing |  | QPSK | 1 (Note 1) |
| 2 | 5MHz |  |  | QPSK | 25 |
| 3 | 10MHz |  |  | QPSK | 1 (Note 1) |
| 4 | 10MHz |  |  | QPSK | 12 (Note 2) |
| 5 | 10MHz |  |  | QPSK | 48 (Note 2) |
| 6 | 10MHz |  |  | QPSK | 50 |
| 7 | 10MHz |  |  | 16QAM (Note 7) | 50 (Note 4) |
| 8 (Note 8) | 15MHz |  |  | QPSK | 1 (Note 1) |
| 9 (Note 8) | 15MHz |  |  | QPSK | 16 (Note 2) |
| 10 (Note 8) | 15MHz |  |  | QPSK | 30 (Note 2, 5) |
| 11 (Note 8) | 15MHz |  |  | QPSK | 48 (Note 2, 6) |
| 12 (Note 8) | 15MHz |  |  | QPSK | 75 (Note 6) |
| 13 (Note 8) | 15MHz |  |  | 16QAM (Note 7) | 75 (Note 4, 6) |
| 14 (Note 8) | 20MHz |  |  | QPSK | 1 (Note 1) |
| 15 (Note 8) | 20MHz |  |  | QPSK | 18 (Note 2) |
| 16 (Note 8) | 20MHz |  |  | QPSK | 24 (Note 2, 5) |
| 17 (Note 8) | 20MHz |  |  | QPSK | 48 (Note 2, 6) |
| 18 (Note 8) | 20MHz |  |  | QPSK | 100 (Note 6) |
| 19 (Note 8) | 20MHz |  |  | 16QAM (Note 7) | 100 (Note 4, 6) |
| 20 (Note 9) | 15MHz |  |  | QPSK | 1 (Note 10) |
| 21 (Note 9) | 15MHz |  |  | QPSK | 1 (Note 11) |
| 22 (Note 9) | 15MHz |  |  | QPSK | 1 (Note 12) |
| 23 (Note 9) | 15MHz |  |  | QPSK | 31 (Note 11) |
| 24 (Note 9) | 15MHz |  |  | QPSK | 55 (Note 11) |
| 25 (Note 9) | 15MHz |  |  | QPSK | 7 (Note 13) |
| 26 (Note 9) | 15MHz |  |  | QPSK | 75 |
| 27 (Note 9) | 15MHz |  |  | 16QAM (Note 7) | 75 (Note 4) |
| 28 (Note 9) | 20MHz |  |  | QPSK | 1 (Note 11) |
| 29 (Note 9) | 20MHz |  |  | QPSK | 1 (Note 14) |
| 30 (Note 9) | 20MHz |  |  | QPSK | 1 (Note 15) |
| 31 (Note 9) | 20MHz |  |  | QPSK | 25 (Note 14) |
| 32 (Note 9) | 20MHz |  |  | QPSK | 41 (Note 14) |
| 33 (Note 9) | 20MHz |  |  | QPSK | 51 (Note 14) |
| 34 (Note 9) | 20MHz |  |  | QPSK | 7 (Note 16) |
| 35 (Note 9) | 20MHz |  |  | QPSK | 100 |
| 36 (Note 9) | 20MHz |  |  | 16QAM (Note 7) | 100 (Note 4) |
| Note 1: The 1 RB allocation shall be tested at both RB #0 and RB #max except for 15MHz and 20MHz of Low Range. For 15MHz of Low Range, the 1 RB allocation shall be tested at both RB#8 and RB#66. For 20MHz of Low Range, the 1 RB allocation shall be tested at both RB#24 and RB#75. Note 2: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth except for 15MHz and 20MHz of Low Range. For 15MHz of Low Range, the RBstart shall be RB#8 and RB# (67 - RB allocation). For 20MHz of Low Range, the RBstart shall be RB#24 and RB# (76 - RB allocation).. Note 3: The Configuration ID will be used to map the applicable Test Configuration to the corresponding Test Requirement in subclause 6.2.4.5 as not all combinations are necessarily required based on the applicability of the UE. Note 4: Applies only for UE-Categories ≥2. Note 5: Required for Low Range only. Note 6: Not available for Low Range. Note 7: The 16QAM configurations shall not apply to UEs which support UL 64QAM. Note 8: Except for the condition that UEs which the index of field 1 in modifiedMPRbehavior is set to 1 and test frequency is Low Range. Note 9: Only for the condition that UEs which the index of field 1 in modifiedMPRbehavior is set to 1 and test frequency is Low Range. Note 10: The RBstart shall be RB# 0. Note 11: The RBstart shall be RB# 8. Note 12: The RBstart shall be RB# 74. Note 13: The RBstart shall be RB# 68. Note 14: The RBstart shall be RB# 24. Note 15: The RBstart shall be RB# 99. Note 16: The RBstart shall be RB# 93. |  |  |  |  |  |

<< Unchanged parts omitted >>

6.2.4.5	Test requirements

<< Unchanged parts omitted >>

Table 6.2.4.5-4: UE Power Class test requirements (network signalled value "NS_05")

| Configuration ID | EUTRA band | Class 1 (dBm) | Tol. (dB) | Class 2 (dBm) | Tol. (dB) | Class 3 (dBm) | Tol. (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 |  |  |  |  | 23 | +2.7 /-2.7 |
| 2 | 1 |  |  |  |  | 23 | +2.7 / -3.7 |
| 3 | 1 |  |  |  |  | 23 | +2.7 / -2.7 |
| 4 | 1 |  |  |  |  | 23 | +2.7 / -2.7 |
| 5 | 1 |  |  |  |  | 23 | +2.7 / -3.7 |
| 6 | 1 |  |  |  |  | 23 | +2.7 / -4.7 |
| 7 | 1 |  |  |  |  | 23 | +2.7 / -6.2 |
| 8 | 1 |  |  |  |  | 23 | +2.7 / -2.7 |
| 9 | 1 |  |  |  |  | 23 | +2.7 / -2.7 |
| 10 | 1 |  |  |  |  | 23 | +2.7/ -3.7 |
| 11 | 1 |  |  |  |  | 23 | +2.7 / -3.7 |
| 12 | 1 |  |  |  |  | 23 | +2.7 / -4.7 |
| 13 | 1 |  |  |  |  | 23 | +2.7 / -6.2 |
| 14 | 1 |  |  |  |  | 23 | +2.7 / -2.7 |
| 15 | 1 |  |  |  |  | 23 | +2.7 / -2.7 |
| 16 | 1 |  |  |  |  | 23 | +2.7/ -3.7 |
| 17 | 1 |  |  |  |  | 23 | +2.7 / -3.7 |
| 18 | 1 |  |  |  |  | 23 | +2.7 / -4.7 |
| 19 | 1 |  |  |  |  | 23 | +2.7 / -6.2 |
| 20 | 1 |  |  |  |  | 23 | +2.7 / -13.7 |
| 21 | 1 |  |  |  |  | 23 | +2.7 / -2.7 |
| 22 | 1 |  |  |  |  | 23 | +2.7 / -7.7 |
| 23 | 1 |  |  |  |  | 23 | +2.7 / -5.7 |
| 24 | 1 |  |  |  |  | 23 | +2.7 / -3.7 |
| 25 | 1 |  |  |  |  | 23 | +2.7 / -8.7 |
| 26 | 1 |  |  |  |  | 23 | +2.7 / -14.7 |
| 27 | 1 |  |  |  |  | 23 | +2.7 / -15.7 |
| 28 | 1 |  |  |  |  | 23 | +2.7 / -13.7 |
| 29 | 1 |  |  |  |  | 23 | +2.7 / -2.7 |
| 30 | 1 |  |  |  |  | 23 | +2.7 / -7.7 |
| 31 | 1 |  |  |  |  | 23 | +2.7 / -6.7 |
| 32 | 1 |  |  |  |  | 23 | +2.7 / -8.7 |
| 33 | 1 |  |  |  |  | 23 | +2.7 / -13.7 |
| 34 | 1 |  |  |  |  | 23 | +2.7 / -4.7 |
| 35 | 1 |  |  |  |  | 23 | +2.7 / -14.7 |
| 36 | 1 |  |  |  |  | 23 | +2.7 / -15.7 |

6.6.3.3	Additional spurious emissions

<< Unchanged parts omitted >>

6.6.3.3.3	Minimum conformance requirements

<< Unchanged parts omitted >>

6.6.3.3.3.1	Minimum conformance requirements (network signalled value "NS_05")

When "NS_05" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3.3.3.1-1. This requirement also applies for the frequency ranges that are less than ΔfOOB (MHz) in Table 6.6.3.1.3-1 from the edge of the channel bandwidth.

Table 6.6.3.3.3.1-1: Additional requirements (PHS)

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) |  |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- | --- |
|  | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |
| 1884.5 f 1915.7*1 | -41 | -41 | -41 | -41 | 300 KHz |
| Note 1: Applicable when the lower edge of the assigned E-UTRA UL channel bandwidth frequency is larger than or equal to the upper edge of PHS band (1915.7 MHz) + 4 MHz + the Channel BW assigned, where Channel BW is as defined in sub-clause 5.4.2. Additional restrictions apply for operations below this point. |  |  |  |  |  |

The requirements in Table 6.6.3.3.3.1-1 apply with the additional restrictions specified in Table 6.6.3.3.3.1-2 when the lower edge of the assigned E-UTRA UL channel bandwidth frequency is less than the upper edge of PHS band (1915.7 MHz) + 4 MHz + the channel BW assigned and the index field 1 in modifiedMPRbehavior is not set to 1.

Table 6.6.3.3.3.1-2: RB restrictions for additional requirement (PHS) (Index field 1 in modifiedMPRbehavior is not set to 1)

| 15 MHz channel bandwidth with fc = 1932.5 MHz |  |  |  |
| --- | --- | --- | --- |
| RBstart | 0-7 | 8-66 | 67-74 |
| LCRB | N/A | ≤ MIN(30, 67 – RBstart) | N/A |
| 20 MHz channel bandwidth with fc = 1930 MHz |  |  |  |
| RBstart | 0-23 | 24-75 | 76-99 |
| LCRB | N/A | ≤ MIN(24, 76 – RBstart) | N/A |

<< Unchanged parts omitted >>

6.6.3.3.4	Test description

6.6.3.3.4.1	Initial conditions

<< Unchanged parts omitted >>

Table 6.6.3.3.4.1-1: Test Configuration Table (network signalled value "NS_05")

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range In case of Low range: - For 5MHz Channel Bandwidth: 1927.2MHz (NUL = 18072) - For 10MHz Channel Bandwidth: 1934.7MHz (NUL = 18147) - For 15MHz Channel Bandwidth: 1932.5MHz (NUL = 18125) - For 20MHz Channel Bandwidth: 1930MHz (NUL = 18100) |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | 5MHz, 10MHz, 15MHz, 20MHz |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |  |
| Ch BW | Mod'n | RB allocation |  | Mod'n | RB allocation |  |
|  |  | FDD | TDD |  | FDD | TDD |
| 5MHz | N/A for Additional Spurious Emissions testing |  |  | QPSK | 1 (Note 1) | N/A |
| 5MHz |  |  |  | QPSK | 25 |  |
| 10MHz |  |  |  | QPSK | 1 (Note 1) |  |
| 10MHz |  |  |  | QPSK | 12 (Note 2) |  |
| 10MHz |  |  |  | QPSK | 48 (Note 2) |  |
| 10MHz |  |  |  | QPSK | 50 |  |
| 10MHz |  |  |  | 16QAM (Note 6) | 50 (Note 3) |  |
| 15MHz (Note 7) |  |  |  | QPSK | 1 (Note 1) |  |
| 15MHz (Note 7) |  |  |  | QPSK | 16 (Note 2) |  |
| 15MHz (Note 7) |  |  |  | QPSK | 30 (Note 2, 4) |  |
| 15MHz (Note 7) |  |  |  | QPSK | 48 (Note 2, 5) |  |
| 15MHz (Note 7) |  |  |  | QPSK | 75 (Note 5) |  |
| 15MHz (Note 7) |  |  |  | 16QAM (Note 6) | 75 (Note 3, 5) |  |
| 20MHz (Note 7) |  |  |  | QPSK | 1 (Note 1) |  |
| 20MHz (Note 7) |  |  |  | QPSK | 18 (Note 2) |  |
| 20MHz (Note 7) |  |  |  | QPSK | 24 (Note 2, 4) |  |
| 20MHz (Note 7) |  |  |  | QPSK | 48 (Note 2, 5) |  |
| 20MHz (Note 7) |  |  |  | QPSK | 100 (Note 5) |  |
| 20MHz (Note 7) |  |  |  | 16QAM (Note 6) | 100 (Note 3, 5) |  |
| 15MHz (Note 8) |  |  |  | QPSK | 1 (Note 9) |  |
| 15MHz (Note 8) |  |  |  | QPSK | 1 (Note 10) |  |
| 15MHz (Note 8) |  |  |  | QPSK | 1 (Note 11) |  |
| 15MHz (Note 8) |  |  |  | QPSK | 31 (Note 10) |  |
| 15MHz (Note 8) |  |  |  | QPSK | 55 (Note 10) |  |
| 15MHz (Note 8) |  |  |  | QPSK | 7 (Note 12) |  |
| 15MHz (Note 8) |  |  |  | QPSK | 75 |  |
| 15MHz (Note 8) |  |  |  | 16QAM (Note 6) | 75 (Note 4) |  |
| 20MHz (Note 8) |  |  |  | QPSK | 1 (Note 10) |  |
| 20MHz (Note 8) |  |  |  | QPSK | 1 (Note 13) |  |
| 20MHz (Note 8) |  |  |  | QPSK | 1 (Note 14) |  |
| 20MHz (Note 8) |  |  |  | QPSK | 25 (Note 13) |  |
| 20MHz (Note 8) |  |  |  | QPSK | 41 (Note 13) |  |
| 20MHz (Note 8) |  |  |  | QPSK | 51 (Note 13) |  |
| 20MHz (Note 8) |  |  |  | QPSK | 7 (Note 15) |  |
| 20MHz (Note 8) |  |  |  | QPSK | 100 |  |
| 20MHz (Note 8) |  |  |  | 16QAM (Note 6) | 100 (Note 4) |  |
| Note 1: The 1 RB allocation shall be tested at both RB #0 and RB #max except for 15MHz and 20MHz of Low Range. For 15MHz of Low Range, the 1 RB allocation shall be tested at both RB#8 and RB#66. For 20MHz of Low Range, the 1 RB allocation shall be tested at both RB#24 and RB#75. Note 2: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth except for 15MHz and 20MHz of Low Range. For 15MHz of Low Range, the RBstart shall be RB#8 and RB# (67 - RB allocation). For 20MHz of Low Range, the RBstart shall be RB#24 and RB# (76 - RB allocation). Note 3: Applies only for UE-Categories ≥2. Note 4: Required for Low Range only. Note 5: Not available for Low Range. Note 6: The 16QAM configurations shall not apply to UEs which support UL 64QAM. Note 7: Except for the condition that UEs which the index of field 1 in modifiedMPRbehavior is set to 1 and test frequency is Low Range. Note 8: Only for the condition that UEs which the index of field 1 in modifiedMPRbehavior is set to 1 and test frequency is Low Range. Note 9: The RBstart shall be RB# 0. Note 10: The RBstart shall be RB# 8. Note 11: The RBstart shall be RB# 74. Note 12: The RBstart shall be RB# 68. Note 13: The RBstart shall be RB# 24. Note 14: The RBstart shall be RB# 99. Note 15: The RBstart shall be RB# 93. |  |  |  |  |  |  |

6.6.3.3.5	Test requirement

<< Unchanged parts omitted >>

6.6.3.3.5.1	Test requirement (network signalled value "NS_05")

When "NS_05" is indicated in the cell:

-	the measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2.4.5-4 as appropriate,

and

the measured average power of spurious emission, derived in step 2, shall not exceed the described value in Table 6.6.3.3.5.1-1. This requirement also applies for the frequency ranges that are less than ΔfOOB (MHz) in Table 6.6.3.1.3-1 from the edge of the channel bandwidth.

Table 6.6.3.3.5.1-1: Additional requirements (PHS) test requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) |  |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- | --- |
|  | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |
| 1884.5 f 1915.7*1 | -41 | -41 | -41 | -41 | 300 KHz |
| Note 1: Applicable when the lower edge of the assigned E-UTRA UL channel bandwidth frequency is larger than or equal to the upper edge of PHS band (1915.7 MHz) + 4 MHz + the Channel BW assigned, where Channel BW is as defined in sub-clause 5.4.2. Additional restrictions apply for operations below this point. |  |  |  |  |  |

The requirements in Table 6.6.3.3.5.1-1 apply with the additional restrictions specified in Table 6.6.3.3.5.1-2 when the lower edge of the assigned E-UTRA UL channel bandwidth frequency is less than the upper edge of PHS band (1915.7 MHz) + 4 MHz + the channel BW assigned and the index field 1 in modifiedMPRbehavior is not set to 1.

Table 6.6.3.3.5.1-2: RB restrictions for additional requirement (PHS) (Index field 1 in modifiedMPRbehavior is not set to 1)

| 15 MHz channel bandwidth with fc = 1932.5 MHz |  |  |  |
| --- | --- | --- | --- |
| RBstart | 0-7 | 8-66 | 67-74 |
| LCRB | N/A | ≤ MIN(30, 67 – RBstart) | N/A |
| 20 MHz channel bandwidth with fc = 1930 MHz |  |  |  |
| RBstart | 0-23 | 24-75 | 76-99 |
| LCRB | N/A | ≤ MIN(24, 76 – RBstart) | N/A |

<< Unchanged parts omitted >>

In addition, we propose the following modification in [3] since the test cases need to be changed depending on the modifiedMPRbehavior.

4	Recommended test case applicability

<< Unchanged parts omitted >>

4.1	RF conformance test cases

| Clause | Title | Release | Applicability |  | Tested Bands / CA-Configurations Selection | Additional Information |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comments |  |  |
|  | Transmitter Characteristics |  |  |  |  |  |
| … |  |  |  |  |  |  |
| 6.6.3.3 | Additional spurious emissions | Rel-8 | C113 | UE supporting E-UTRA | D01 | FDD (Note 3) |
|  |  |  |  |  |  | TDD |
| … |  |  |  |  |  |  |
| Note 1: Due to UE capability signalling for UL 64QAM is introduced from Rel-12, this test case can optionally be executed with a Rel-12 UE. Note 2: For a transition period until RAN5#72, this condition in version 13.0.0 of 36.521-2 shall be used. This is to ensure no test coverage is lost before the UL 64QAM test case becomes available. Note 3: The test cases for NS_05 change depending on the index field 1 in modifiedMPRbehavior. |  |  |  |  |  |  |

Then, the proporsal in this paper is the following.

Proporsal: The above modifications are applied to the specification to change the test cases depending on the modifiedMPRbehavior.

3	Proposal

In this document, the following is proposed.

- Proporsal: The above modification is applied to the specification to change the test cases depending on the modifiedMPRbehavior.

4	References

[1]	TS 36.101 V12.11.0

[2] TS36.521-1V13.0.1

[3] TS36.521-2 V13.1.0