3GPP TSG-RAN5 Meeting #73	R5-168035

# Reno, Nevada, United States, 14th Nov 2016 - 18th Nov 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 2990 | rev | - | Current version: | 14.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates of 5.2A operating bands for CA_1A-3A-41A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_R14-UEConTest |  |  |  |  |  | Date: |  |  | 2016-10-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Operating bands for new R14 CA combination CA_1A-3A-41A is currently missing in 36.521-1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Impacted Rel-14 CA configurations: CA_1A-3A-41A Added operating bands for CA_1A-3A-41A Editorial corrections in Table 5.2A-2a |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The operating bands for 3DL CA configuration will remain incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### <<Unchanged sections omitted>>

## 5.2A	Operating bands for CA

E-UTRA carrier aggregation is designed to operate in the operating bands defined in Tables 5.2A-1 and 5.2A-2.

Table 5.2A-1: Intra-band contiguous CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_1 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
| CA_2 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
| CA_3 | 3 | 1710MHz | – | 1785MHz | 1805MHz | – | 1880MHz | FDD |
| CA_5 | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
| CA_7 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
| CA_8 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
| CA_12 | 12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
| CA_23 | 23 | 2000 MHz | – | 2020 MHz | 2180 MHz | – | 2200 MHz | FDD |
| CA_27 | 27 | 807 MHz | – | 824 MHz | 852 MHz | – | 869 MHz | FDD |
| CA_38 | 38 | 2570 MHz | – | 2620 MHz | 2570MHz | – | 2620 MHz | TDD |
| CA_39 | 39 | 1880 MHz | – | 1920 MHz | 1880 MHz | – | 1920 MHz | TDD |
| CA_40 | 40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
| CA_41 | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
| CA_42 | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
| CA_66 | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

Table 5.2A-2: Inter-band CA operating bands (two bands)

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_1-3 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz |  |
| CA_1-3-3 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz |  |
| CA_1-5 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz |  |
| CA_1-7 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz |  |
| CA_1-8 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz |  |
| CA_1-11 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz |  |
| CA_1-18 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz |  |
| CA_1-19 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz |  |
| CA_1-20 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz |  |
| CA_1-21 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz |  |
| CA_1-26 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 26 | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz |  |
| CA_1-28 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz |  |
| CA_1-40 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
| CA_1-41 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
| CA_1-42 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
| CA_1-46 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 46 | 5150 MHz | – | 5925 MHz | 5150 MHz | – | 5925 MHz | TDD |
| CA_2-2-5 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz |  |
| CA_2-2-12 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz |  |
| CA_2-2-13 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz |  |
| CA_2-4 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 4 | 1710 MHz | – | 1755 MHz | 2110 MHz | – | 2155 MHz |  |
| CA_2-4-4 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 4 | 1710 MHz | – | 1755 MHz | 2110 MHz | – | 2155 MHz |  |
| CA_2-5 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz |  |
| CA_2-7 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz |  |
| CA_2-12 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz |  |
| CA_2-13 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz |  |
| CA_2-17 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 17 | 704 MHz | – | 716 MHz | 734 MHz | – | 746 MHz |  |
| CA_2-28 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz |  |
| CA_2-29 | 2 | 1850 MHz | - | 1910 MHz | 1930 MHz | - | 1990 MHz | FDD |
|  | 29 | N/A |  |  | 717 MHz | - | 728 MHz |  |
| CA_2-30 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 30 | 2305 MHz | – | 2315 MHz | 2350 MHz | – | 2360 MHz |  |
| CA_2-46 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 46 | 5150 MHz | – | 5925 MHz | 5150 MHz | – | 5925 MHz | TDD |
| CA_3-5 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz |  |
| CA_3-3-5 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz |  |
| CA_3-7 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz |  |
| CA_3-8 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz |  |
| CA_3-3-8 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz |  |
| CA_3-19 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz |  |
| CA_3-20 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz |  |
| CA_3-26 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 26 | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz |  |
| CA_3-27 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 27 | 807 MHz | – | 824 MHz | 852 MHz | – | 869 MHz |  |
| CA_3-28 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz |  |
| CA_3-31 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 31 | 452.5 MHz | – | 457.5 MHz | 462.5 MHz | – | 467.5 MHz |  |
| CA_3-38 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
| CA_3-40 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
| CA_3-41 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
| CA_3-42 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
| CA_3-46 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 46 | 5150 MHz | – | 5925 MHz | 5150 MHz | – | 5925 MHz | TDD |
| CA_4-4-5 | 4 | 1710 MHz | – | 1755 MHz | 2110 MHz | – | 2155 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz |  |
| CA_4-4-7 | 4 | 1710 MHz | – | 1755 MHz | 2110 MHz | – | 2155 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz |  |
| CA_4-4-12 | 4 | 1710 MHz | – | 1755 MHz | 2110 MHz | – | 2155 MHz | FDD |
|  | 12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz |  |
| CA_4-4-13 | 4 | 1710 MHz | – | 1755 MHz | 2110 MHz | – | 2155 MHz | FDD |
|  | 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz |  |
| CA_4-5 | 4 | 1710 MHz | – | 1755 MHz | 2110 MHz | – | 2155 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz |  |
| CA_4-7 | 4 | 1710 MHz |  | 1755 MHz | 2110 MHz |  | 2155 MHz | FDD |
|  | 7 | 2500 MHz |  | 2570 MHz | 2620 MHz |  | 2690 MHz |  |
| CA_4-12 | 4 | 1710 MHz | – | 1755 MHz | 2110 MHz | – | 2155 MHz | FDD |
|  | 12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz |  |
| CA_4-13 | 4 | 1710 MHz | – | 1755 MHz | 2110 MHz | – | 2155 MHz | FDD |
|  | 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz |  |
| CA_4-17 | 4 | 1710 MHz | – | 1755 MHz | 2110 MHz | – | 2155 MHz | FDD |
|  | 17 | 704 MHz | – | 716 MHz | 734 MHz | – | 746 MHz |  |
| CA_4-27 | 4 | 1710 MHz | – | 1755 MHz | 2110 MHz | – | 2155 MHz | FDD |
|  | 27 | 807 MHz | – | 824 MHz | 852 MHz | – | 869 MHz |  |
| CA_4-28 | 4 | 1710 MHz | – | 1755 MHz | 2110 MHz | – | 2155 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz |  |
| CA_4-29 | 4 | 1710 MHz | – | 1755 MHz | 2110 MHz | – | 2155 MHz | FDD |
|  | 29 | N/A |  |  | 717 MHz | - | 728 MHz |  |
| CA_4-4-29 | 4 | 1710 MHz | – | 1755 MHz | 2110 MHz | – | 2155 MHz | FDD |
|  | 29 | N/A |  |  | 717 MHz | – | 728 MHz |  |
| CA_4-30 | 4 | 1710 MHz | – | 1755 MHz | 2110 MHz | – | 2155 MHz | FDD |
|  | 30 | 2305 MHz | – | 2315 MHz | 2350 MHz | – | 2360 MHz |  |
| CA_4-4-30 | 4 | 1710 MHz | – | 1755 MHz | 2110 MHz | – | 2155 MHz | FDD |
|  | 30 | 2305 MHz | – | 2315 MHz | 2350 MHz | – | 2360 MHz |  |
| CA_4-46 | 4 | 1710 MHz | – | 1755 MHz | 2110 MHz | – | 2155 MHz | FDD |
|  | 46 | 5150 MHz | – | 5925 MHz | 5150 MHz | – | 5925 MHz | TDD |
| CA_5-7 | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz |  |
| CA_5-12 | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | 12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz |  |
| CA_5-13 | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz |  |
| CA_5-17 | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | 17 | 704 MHz | – | 716 MHz | 734 MHz | – | 746 MHz |  |
| CA_5-25 | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | 25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz |  |
| CA_5-29 | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | 29 | N/A |  |  | 717 MHz | – | 728 MHz |  |
| CA_5-30 | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | 30 | 2305 MHz | – | 2315 MHz | 2350 MHz | – | 2360 MHz |  |
| CA_5-38 | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | 38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
| CA_5-40 | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | 40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
| CA_7-8 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz |  |
| CA_7-12 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz |  |
| CA_7-20 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz |  |
| CA_7-22 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 22 | 3410 MHz | – | 3490 MHz | 3510 MHz | – | 3590 MHz |  |
| CA_7-28 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz |  |
| CA_7-40 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
| CA_7-42 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
| CA_7-42-42 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
| CA_7-46 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 46 | 5150 MHz | – | 5925 MHz | 5150 MHz | – | 5925 MHz | TDD |
| CA_8-11 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz |  |
| CA_8-20 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz |  |
| CA_8-40 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
| CA_8-41 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
| CA_8-42 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
| CA_11-18 | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz |  |
| CA_12-25 | 12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
|  | 25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz |  |
| CA_12-30 | 12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
|  | 30 | 2305 MHz | – | 2315 MHz | 2350 MHz | – | 2360 MHz |  |
| CA_18-28 | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | 28 | 703 MHz | – | 733 MHz | 758 MHz | – | 788 MHz |  |
| CA_19-21 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz |  |
| CA_19-28 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 28 | 718 MHz 1 | – | 748 MHz | 773 MHz 1 | – | 803 MHz |  |
| CA_19-42 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
| CA_20-31 | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | 31 | 452.5 MHz | – | 457.5 MHz | 462.5 MHz | – | 467.5 MHz |  |
| CA_20-32 | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | 32 | N/A |  |  | 1452 MHz | – | 1496 MHz |  |
| CA_20-38 | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | 38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
| CA_20-40 | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | 40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
| CA_20-42 | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
| CA_20-42-42 | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
| CA_20-67 | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | 67 | N/A |  |  | 738 MHz | – | 758 MHz |  |
| CA_21-42 | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
| CA_23-29 | 23 | 2000 MHz | – | 2020 MHz | 2180 MHz | – | 2200 MHz | FDD |
|  | 29 | N/A |  |  | 717 MHz | – | 728 MHz |  |
| CA_25-26 | 25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | 26 | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz |  |
| CA_25-41 | 25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
| CA_26-41 | 26 | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
| CA_28-40 | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | 40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
| CA_28-41 | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
| CA_28-42 | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
| CA_29-30 | 29 | N/A |  |  | 717 MHz | – | 728 MHz | FDD |
|  | 30 | 2305 MHz | – | 2315 MHz | 2350 MHz | – | 2360 MHz |  |
| CA_38-40 | 38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | 40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz |  |
| CA_39-41 | 39 | 1880 MHz | – | 1920 MHz | 1880 MHz | – | 1920 MHz | TDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz |  |
| CA_41-42 | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz |  |
| CA_41-46 | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | 46 | 5150 MHz | – | 5925 MHz | 5150 MHz | – | 5925 MHz |  |
| CA_42-46 | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | 46 | 5150 MHz | – | 5925 MHz | 5150 MHz | – | 5925 MHz |  |
| NOTE 1: The frequency range in band 28 is restricted for this CA band combination. |  |  |  |  |  |  |  |  |

Table 5.2A-2a: Inter-band CA operating bands (three bands)

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  |  |  |  | Downlink (DL) operating band |  |  |  |  |  |  | Duplex Mode |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  |  |  |  | BS transmit / UE receive |  |  |  |  |  |  |  |  |
|  |  | FUL_low – FUL_high |  |  |  |  |  | FDL_low – FDL_high |  |  |  |  |  |  |  |  |
| CA_1-3-5 | 1 | 1920 MHz |  | – |  | 1980 MHz |  | 2110 MHz |  |  | – |  | 2170 MHz |  | FDD |  |
|  | 3 | 1710 MHz |  | – |  | 1785 MHz |  | 1805 MHz |  |  | – |  | 1880 MHz |  |  |  |
|  | 5 | 824 MHz |  | – |  | 849 MHz |  | 869 MHz |  |  | – |  | 894 MHz |  |  |  |
| CA_1-3-7 | 1 | 1920 MHz |  | – |  | 1980 MHz |  | 2110 MHz |  |  | – |  | 2170 MHz |  | FDD |  |
|  | 3 | 1710 MHz |  | – |  | 1785 MHz |  | 1805 MHz |  |  | – |  | 1880 MHz |  |  |  |
|  | 7 | 2500 MHz |  | – |  | 2570 MHz |  | 2620 MHz |  |  | – |  | 2690 MHz |  |  |  |
| CA_1-3-8 | 1 | 1920 MHz |  | – |  | 1980 MHz |  | 2110 MHz |  |  | – |  | 2170 MHz |  | FDD |  |
|  | 3 | 1710 MHz |  | – |  | 1785 MHz |  | 1805 MHz |  |  | – |  | 1880 MHz |  |  |  |
|  | 8 | 880 MHz |  | – |  | 915 MHz |  | 925 MHz |  |  | – |  | 960 MHz |  |  |  |
| CA_1-3-19 | 1 | 1920 MHz |  | – |  | 1980 MHz |  | 2110 MHz |  |  | – |  | 2170 MHz |  | FDD |  |
|  | 3 | 1710 MHz |  | – |  | 1785 MHz |  | 1805 MHz |  |  | – |  | 1880 MHz |  |  |  |
|  | 19 | 830 MHz |  | – |  | 845 MHz |  | 875 MHz |  |  | – |  | 890 MHz |  |  |  |
| CA_1-3-20 | 1 | 1920 MHz |  | – |  | 1980 MHz |  | 2110 MHz |  |  | – |  | 2170 MHz |  | FDD |  |
|  | 3 | 1710 MHz |  | – |  | 1785 MHz |  | 1805 MHz |  |  | – |  | 1880 MHz |  |  |  |
|  | 20 | 832 MHz |  | – |  | 862 MHz |  | 791 MHz |  |  | – |  | 821 MHz |  |  |  |
| CA_1-3-26 | 1 | 1920 MHz |  | – |  | 1980 MHz |  | 2110 MHz |  |  | – |  | 2170 MHz |  | FDD |  |
|  | 3 | 1710 MHz |  | – |  | 1785 MHz |  | 1805 MHz |  |  | – |  | 1880 MHz |  |  |  |
|  | 26 | 814 MHz |  | – |  | 849 MHz |  | 859 MHz |  |  | – |  | 894 MHz |  |  |  |
| CA_1-3-28 | 1 |  | 1920 MHz |  | – |  | 1980 MHz |  | 2110 MHz |  |  | – |  | 2170 MHz |  | FDD |
|  | 3 |  | 1710 MHz |  | – |  | 1785 MHz |  | 1805 MHz |  |  | – |  | 1880 MHz |  |  |
|  | 28 |  | 703 MHz |  | – |  | 748 MHz |  | 758 MHz |  |  | – |  | 803 MHz |  |  |
| CA_1-3-40 | 1 |  | 1920 MHz |  | – |  | 1980 MHz |  | 2110 MHz |  |  | – |  | 2170 MHz |  | FDD |
|  | 3 |  | 1710 MHz |  | – |  | 1785 MHz |  | 1805 MHz |  |  | – |  | 1880 MHz |  |  |
|  | 40 |  | 2300 MHz |  | – |  | 2400 MHz |  | 2300 MHz |  |  | – |  | 2400 MHz |  | TDD |
| CA_1-3-41 | 1 | 1920 MHz |  | – |  | 1980 MHz |  | 2110 MHz |  |  | – |  | 2170 MHz |  | FDD |  |
|  | 3 | 1710 MHz |  | – |  | 1785 MHz |  | 1805 MHz |  |  | – |  | 1880 MHz |  |  |  |
|  | 41 | 2496 MHz |  |  |  | 2690 MHz |  | 2496 MHz |  |  |  |  | 2690 MHz |  | TDD |  |
| CA_1-3-42 | 1 | 1920 MHz |  | – |  | 1980 MHz |  | 2110 MHz |  |  | – |  | 2170 MHz |  | FDD |  |
|  | 3 | 1710 MHz |  | – |  | 1785 MHz |  | 1805 MHz |  |  | – |  | 1880 MHz |  |  |  |
|  | 42 | 3400 MHz |  | – |  | 3600 MHz |  | 3400 MHz |  |  | – |  | 3600 MHz |  | TDD |  |
| CA_1-5-7 | 1 | 1920 MHz |  | – |  | 1980 MHz |  | 2110 MHz |  |  | – |  | 2170 MHz |  | FDD |  |
|  | 5 | 824 MHz |  | – |  | 849 MHz |  | 869 MHz |  |  | – |  | 894 MHz |  |  |  |
|  | 7 | 2500 MHz |  | – |  | 2570 MHz |  | 2620 MHz |  |  | – |  | 2690 MHz |  |  |  |
| CA_1-5-40 | 1 | 1920 MHz |  | – |  | 1980 MHz |  | 2110 MHz |  |  | – |  | 2170 MHz |  | FDD |  |
|  | 5 | 824 MHz |  | – |  | 849 MHz |  | 869 MHz |  |  | – |  | 894 MHz |  |  |  |
|  | 40 | 2300 MHz |  | – |  | 2400 MHz |  | 2300 MHz |  |  | – |  | 2400 MHz |  | TDD |  |
| CA_1-7-8 | 1 | 1920 MHz |  | – |  | 1980 MHz |  | 2110 MHz |  |  | – |  | 2170 MHz |  | FDD |  |
|  | 7 | 2500 MHz |  | – |  | 2570 MHz |  | 2620 MHz |  |  | – |  | 2690 MHz |  |  |  |
|  | 8 | 880 MHz |  | – |  | 915 MHz |  | 925 MHz |  |  | – |  | 960 MHz |  |  |  |
| CA_1-7-20 | 1 | 1920 MHz |  | – |  | 1980 MHz |  | 2110 MHz |  |  | – |  | 2170 MHz |  | FDD |  |
|  | 7 | 2500 MHz |  | – |  | 2570 MHz |  | 2620 MHz |  |  | – |  | 2690 MHz |  |  |  |
|  | 20 | 832 MHz |  | – |  | 862 MHz |  | 791 MHz |  |  | – |  | 821 MHz |  |  |  |
| CA_1-7-28 | 1 |  | 1920 MHz |  | – |  | 1980 MHz |  | 2110 MHz |  |  | – |  | 2170 MHz |  | FDD |
|  | 7 |  | 2500 MHz |  | – |  | 2570 MHz |  | 2620 MHz |  |  | – |  | 2690 MHz |  |  |
|  | 28 |  | 703 MHz |  | – |  | 748 MHz |  | 758 MHz |  |  | – |  | 803 MHz |  |  |
| CA_1-8-11 | 1 |  | 1920 MHz |  | – |  | 1980 MHz |  | 2110 MHz |  |  | – |  | 2170 MHz |  | FDD |
|  | 8 |  | 880 MHz |  | – |  | 915 MHz |  | 925 MHz |  |  | – |  | 960 MHz |  |  |
|  | 11 |  | 1427.9 MHz |  | – |  | 1447.9 MHz |  | 1475.9 MHz |  |  | – |  | 1495.9 MHz |  |  |
| CA_1-8-40 | 1 |  | 1920 MHz |  | – |  | 1980 MHz |  | 2110 MHz |  |  | – |  | 2170 MHz |  | FDD |
|  | 8 |  | 880 MHz |  | – |  | 915 MHz |  | 925 MHz |  |  | – |  | 960 MHz |  |  |
|  | 40 |  | 2300 MHz |  | – |  | 2400 MHz |  | 2300 MHz |  |  | – |  | 2400 MHz |  | TDD |
| CA_1-11-18 | 1 |  | 1920 MHz |  | – |  | 1980 MHz |  | 2110 MHz |  |  | – |  | 2170 MHz |  | FDD |
|  | 11 |  | 880 MHz |  | – |  | 915 MHz |  | 925 MHz |  |  | – |  | 960 MHz |  |  |
|  | 18 |  | 1427.9 MHz |  | – |  | 1447.9 MHz |  | 1475.9 MHz |  |  | – |  | 1495.9 MHz |  |  |
| CA_1-18-28 | 1 | 1920 MHz |  | – |  | 1980 MHz |  | 2110 MHz |  |  | – |  | 2170 MHz |  | FDD |  |
|  | 18 | 815 MHz |  | – |  | 830 MHz |  | 860 MHz |  |  | – |  | 875 MHz |  |  |  |
|  | 28 | 703 MHz |  | – |  | 733 MHz |  | 758 MHz |  |  | – |  | 788 MHz |  |  |  |
| CA_1-19-21 | 1 | 1920 MHz |  | – |  | 1980 MHz |  | 2110 MHz |  |  | – |  | 2170 MHz |  | FDD |  |
|  | 19 | 830 MHz |  | – |  | 845 MHz |  | 875 MHz |  |  | – |  | 890 MHz |  |  |  |
|  | 21 | 1447.9 MHz |  | – |  | 1462.9 MHz |  | 1495.9 MHz |  |  | – |  | 1510.9 MHz |  |  |  |
| CA_1-19-28 | 1 | 1920 MHz |  | – |  | 1980 MHz |  | 2110 MHz |  |  | – |  | 2170 MHz |  | FDD |  |
|  | 19 | 830 MHz |  | – |  | 845 MHz |  | 875 MHz |  |  | – |  | 890 MHz |  |  |  |
|  | 28 | 718 MHz 1 |  | – |  | 748 MHz |  | 773 MHz 1 |  |  | – |  | 803 MHz |  |  |  |
| CA_1-19-42 | 1 | 1920 MHz |  | – |  | 1980 MHz |  | 2110 MHz |  |  | – |  | 2170 MHz |  | FDD |  |
|  | 19 | 830 MHz |  | – |  | 845 MHz |  | 875 MHz |  |  | – |  | 890 MHz |  |  |  |
|  | 42 | 3400 MHz |  | – |  | 3600 MHz |  | 3400 MHz |  |  | – |  | 3600 MHz |  | TDD |  |
| CA_1-21-42 | 1 | 1920 MHz |  | – |  | 1980 MHz |  | 2110 MHz |  |  | – |  | 2170 MHz |  | FDD |  |
|  | 21 | 1447.9 MHz |  | – |  | 1462.9 MHz |  | 1495.9 MHz |  |  | – |  | 1510.9 MHz |  |  |  |
|  | 42 | 3400 MHz |  | – |  | 3600 MHz |  | 3400 MHz |  |  | – |  | 3600 MHz |  | TDD |  |
| CA_2-2-4-5 | 2 |  | 1850 MHz |  | – |  | 1910 MHz |  | 1930 MHz |  |  | – |  | 1990 MHz |  | FDD |
|  | 4 |  | 1710 MHz |  | – |  | 1755 MHz |  | 2110 MHz |  |  | – |  | 2155 MHz |  |  |
|  | 5 |  | 824 MHz |  | – |  | 849 MHz |  | 869 MHz |  |  | – |  | 894 MHz |  |  |
| CA_2-2-5-12 | 2 |  | 1850 MHz |  | – |  | 1910 MHz |  | 1930 MHz |  |  | – |  | 1990 MHz |  | FDD |
|  | 5 |  | 824 MHz |  | – |  | 849 MHz |  | 869 MHz |  |  | – |  | 894 MHz |  |  |
|  | 12 |  | 699 MHz |  | – |  | 716 MHz |  | 729 MHz |  |  | – |  | 746 MHz |  |  |
| CA_2-4-4-5 | 2 |  | 1850 MHz |  | – |  | 1910 MHz |  | 1930 MHz |  |  | – |  | 1990 MHz |  | FDD |
|  | 4 |  | 1710 MHz |  | – |  | 1755 MHz |  | 2110 MHz |  |  | – |  | 2155 MHz |  |  |
|  | 5 |  | 824 MHz |  | – |  | 849 MHz |  | 869 MHz |  |  | – |  | 894 MHz |  |  |
| CA_2-4-5 | 2 | 1850 MHz |  | – |  | 1910 MHz |  | 1930 MHz |  |  | – |  | 1990 MHz |  | FDD |  |
|  | 4 | 1710 MHz |  | – |  | 1755 MHz |  | 2110 MHz |  |  | – |  | 2155 MHz |  |  |  |
|  | 5 | 824 MHz |  | – |  | 849 MHz |  | 869 MHz |  |  | – |  | 894 MHz |  |  |  |
| CA_2-4-7 | 2 |  | 1850 MHz |  | – |  | 1910 MHz |  | 1930 MHz |  |  | – |  | 1990 MHz |  | FDD |
|  | 4 |  | 1710 MHz |  | – |  | 1755 MHz |  | 2110 MHz |  |  | – |  | 2155 MHz |  |  |
|  | 7 |  | 2500 MHz |  | – |  | 2570 MHz |  | 2620 MHz |  |  | – |  | 2690 MHz |  |  |
| CA_2-4-12 | 2 | 1850 MHz |  | – |  | 1910 MHz |  | 1930 MHz |  |  | – |  | 1990 MHz |  | FDD |  |
|  | 4 | 1710 MHz |  | – |  | 1755 MHz |  | 2110 MHz |  |  | – |  | 2155 MHz |  |  |  |
|  | 12 | 699 MHz |  | – |  | 716 MHz |  | 729 MHz |  |  | – |  | 746 MHz |  |  |  |
| CA_2-4-13 | 2 | 1850 MHz |  | – |  | 1910 MHz |  | 1930 MHz |  |  | – |  | 1990 MHz |  | FDD |  |
|  | 4 | 1710 MHz |  | – |  | 1755 MHz |  | 2110 MHz |  |  | – |  | 2155 MHz |  |  |  |
|  | 13 | 777 MHz |  | – |  | 787 MHz |  | 746 MHz |  |  | – |  | 756 MHz |  |  |  |
| CA_2-4-30 | 2 | 1850 MHz |  | – |  | 1910 MHz |  | 1930 MHz |  |  | – |  | 1990 MHz |  | FDD |  |
|  | 4 | 1710 MHz |  | – |  | 1755 MHz |  | 2110 MHz |  |  | – |  | 2155 MHz |  |  |  |
|  | 30 | 2305 MHz |  | – |  | 2315 MHz |  | 2350 MHz |  |  | – |  | 2360 MHz |  |  |  |
| CA_2-4-29 | 2 | 1850 MHz |  | – |  | 1910 MHz |  | 1930 MHz |  |  | – |  | 1990 MHz |  | FDD |  |
|  | 4 | 1710 MHz |  | – |  | 1755 MHz |  | 2110 MHz |  |  | – |  | 2155 MHz |  |  |  |
|  | 29 | N/A |  |  |  |  |  | 717 MHz |  |  | – |  | 728 MHz |  |  |  |
| CA_2-5-12 | 2 | 1850 MHz |  | – |  | 1910 MHz |  | 1930 MHz |  |  | – |  | 1990 MHz |  | FDD |  |
|  | 5 | 824 MHz |  | – |  | 849 MHz |  | 869 MHz |  |  | – |  | 894 MHz |  |  |  |
|  | 12 | 699 MHz |  | – |  | 716 MHz |  | 729 MHz |  |  | – |  | 746 MHz |  |  |  |
| CA_2-5-13 | 2 | 1850 MHz |  | – |  | 1910 MHz |  | 1930 MHz |  |  | – |  | 1990 MHz |  | FDD |  |
|  | 5 | 824 MHz |  | – |  | 849 MHz |  | 869 MHz |  |  | – |  | 894 MHz |  |  |  |
|  | 13 | 777 MHz |  | – |  | 787 MHz |  | 746 MHz |  |  | – |  | 756 MHz |  |  |  |
| CA_2-5-29 | 2 |  | 1850 MHz |  | – |  | 1910 MHz |  | 1930 MHz |  |  | – |  | 1990 MHz |  | FDD |
|  | 5 |  | 824 MHz |  | – |  | 849 MHz |  | 869 MHz |  |  | – |  | 894MHz |  |  |
|  | 29 |  | N/A |  | – |  | N/A |  | 717 MHz |  |  | – |  | 728 MHz |  |  |
| CA_2-5-30 | 2 | 1850 MHz |  | – |  | 1910 MHz |  | 1930 MHz |  |  | – |  | 1990 MHz |  | FDD |  |
|  | 5 | 824 MHz |  | – |  | 849 MHz |  | 869 MHz |  |  | – |  | 894 MHz |  |  |  |
|  | 30 | 2305 MHz |  | – |  | 2315 MHz |  | 2350 MHz |  |  | – |  | 2360 MHz |  |  |  |
| CA_2-7-12 | 2 |  | 1850 MHz |  | – |  | 1910 MHz |  | 1930 MHz |  |  | – |  | 1990 MHz |  | FDD |
|  | 7 |  | 2500 MHz |  | – |  | 2570 MHz |  | 2620 MHz |  |  | – |  | 2690 MHz |  |  |
|  | 12 |  | 699 MHz |  | – |  | 716 MHz |  | 729 MHz |  |  | – |  | 746 MHz |  |  |
| CA_2-12-30 | 2 | 1850 MHz |  | – |  | 1910 MHz |  | 1930 MHz |  |  | – |  | 1990 MHz |  | FDD |  |
|  | 12 | 699 MHz |  | – |  | 716 MHz |  | 729 MHz |  |  | – |  | 746 MHz |  |  |  |
|  | 30 | 2305 MHz |  | – |  | 2315 MHz |  | 2350 MHz |  |  | – |  | 2360 MHz |  |  |  |
| CA_3-5-40 | 3 | 1710 MHz |  | – |  | 1785 MHz |  | 1805 MHz |  |  | – |  | 1880 MHz |  | FDD |  |
|  | 5 | 824 MHz |  | – |  | 849 MHz |  | 869 MHz |  |  | – |  | 894 MHz |  |  |  |
|  | 40 | 2300 MHz |  | – |  | 2400 MHz |  | 2300 MHz |  |  | – |  | 2400 MHz |  | TDD |  |
| CA_2-29-30 | 2 | 1850 MHz |  | – |  | 1910 MHz |  | 1930 MHz |  |  | – |  | 1990 MHz |  | FDD |  |
|  | 29 | N/A |  |  |  |  |  | 717 MHz |  |  | – |  | 728 MHz |  |  |  |
|  | 30 | 2305 MHz |  | – |  | 2315 MHz |  | 2350 MHz |  |  | – |  | 2360 MHz |  |  |  |
| CA_3-7-8 | 3 | 1710 MHz |  | – |  | 1785 MHz |  | 1805 MHz |  |  | – |  | 1880 MHz |  | FDD |  |
|  | 7 | 2500 MHz |  | – |  | 2570 MHz |  | 2620 MHz |  |  | – |  | 2690 MHz |  |  |  |
|  | 8 | 880 MHz |  | – |  | 915 MHz |  | 925 MHz |  |  | – |  | 960 MHz |  |  |  |
| CA_3-7-20 | 3 | 1710 MHz |  | – |  | 1785 MHz |  | 1805 MHz |  |  | – |  | 1880 MHz |  | FDD |  |
|  | 7 | 2500 MHz |  | – |  | 2570 MHz |  | 2620 MHz |  |  | – |  | 2690 MHz |  |  |  |
|  | 20 | 832 MHz |  | – |  | 862 MHz |  | 791 MHz |  |  | – |  | 821 MHz |  |  |  |
| CA_3-7-28 | 3 |  | 1710 MHz |  | – |  | 1785 MHz |  | 1805 MHz |  |  | – |  | 1880 MHz |  | FDD |
|  | 7 |  | 2500 MHz |  | – |  | 2570 MHz |  | 2620 MHz |  |  | – |  | 2690 MHz |  |  |
|  | 28 |  | 703 MHz |  | – |  | 748 MHz |  | 758 MHz |  |  | – |  | 803 MHz |  |  |
| CA_3-7-38 | 3 |  | 1710 MHz |  | – |  | 1785 MHz |  | 1805 MHz |  |  | – |  | 1880 MHz |  | FDD |
|  | 7 |  | N/A |  | – |  | N/A |  | 2620 MHz |  |  | – |  | 2690 MHz |  |  |
|  | 38 |  | N/A |  | – |  | N/A |  | 2570 MHz |  |  | – |  | 2620 MHz |  | TDD |
| CA_3-8-40 | 3 |  | 1710 MHz |  | – |  | 1785 MHz |  | 1805 MHz |  |  | – |  | 1880 MHz |  | FDD |
|  | 8 |  | 880 MHz |  | – |  | 915 MHz |  | 925 MHz |  |  | – |  | 960 MHz |  |  |
|  | 40 |  | 2300 MHz |  | – |  | 2400 MHz |  | 2300 MHz |  |  | – |  | 2400 MHz |  | TDD |
| CA_3-28-40 | 3 |  | 1710 MHz |  | – |  | 1785 MHz |  | 1805 MHz |  |  | – |  | 1880 MHz |  | FDD |
|  | 28 |  | 703 MHz |  | – |  | 748 MHz |  | 758 MHz |  |  | – |  | 803 MHz |  |  |
|  | 40 |  | 2300 MHz |  | – |  | 2400 MHz |  | 2300 MHz |  |  | – |  | 2400 MHz |  | TDD |
| CA_3-19-42 | 3 | 1710 MHz |  | – |  | 1785 MHz |  | 1805 MHz |  |  | – |  | 1880 MHz |  | FDD |  |
|  | 19 | 830 MHz |  | – |  | 845 MHz |  | 875 MHz |  |  | – |  | 890 MHz |  |  |  |
|  | 42 | 3400 MHz |  | – |  | 3600 MHz |  | 3400 MHz |  |  | – |  | 3600 MHz |  | TDD |  |
| CA_3-41-42 | 3 |  | 1710 MHz |  | – |  | 1785 MHz |  | 1805 MHz |  |  | – |  | 1880 MHz |  | FDD |
|  | 41 |  | 2496 MHz |  | – |  | 2690 MHz |  | 2496 MHz |  |  | – |  | 2690 MHz |  | TDD |
|  | 42 |  | 3400 MHz |  | – |  | 3600 MHz |  | 3400 MHz |  |  | – |  | 3600 MHz |  |  |
| CA_4-4-5-12 | 4 | 1710 MHz |  | – |  | 1755 MHz |  | 2110 MHz |  |  | – |  | 2155 MHz |  | FDD |  |
|  | 5 | 824 MHz |  | – |  | 849 MHz |  | 869 MHz |  |  | – |  | 894 MHz |  |  |  |
|  | 12 | 699 MHz |  | – |  | 716 MHz |  | 729 MHz |  |  | – |  | 746 MHz |  |  |  |
| CA_4-4-5-30 | 4 | 1710 MHz |  | – |  | 1755 MHz |  | 2110 MHz |  |  | – |  | 2155 MHz |  | FDD |  |
|  | 5 | 824 MHz |  | – |  | 849 MHz |  | 869 MHz |  |  | – |  | 894 MHz |  |  |  |
|  | 30 | 2305 MHz |  | – |  | 2315 MHz |  | 2350 MHz |  |  | – |  | 2360 MHz |  |  |  |
| CA_4-4-12-30 | 4 | 1710 MHz |  | – |  | 1755 MHz |  | 2110 MHz |  |  | – |  | 2155 MHz |  | FDD |  |
|  | 12 | 699 MHz |  | – |  | 716 MHz |  | 729 MHz |  |  | – |  | 746 MHz |  |  |  |
|  | 30 | 2305 MHz |  | – |  | 2315 MHz |  | 2350 MHz |  |  | – |  | 2360 MHz |  |  |  |
| CA_4-4-29-30 | 4 | 1710 MHz |  | – |  | 1755 MHz |  | 2110 MHz |  |  | – |  | 2155 MHz |  | FDD |  |
|  | 29 | N/A |  |  |  |  |  | 717 MHz |  |  | – |  | 728 MHz |  |  |  |
|  | 30 | 2305 MHz |  | – |  | 2315 MHz |  | 2350 MHz |  |  | – |  | 2360 MHz |  |  |  |
| CA_4-5-12 | 4 | 1710 MHz |  | – |  | 1755 MHz |  | 2110 MHz |  |  | – |  | 2155 MHz |  | FDD |  |
|  | 5 | 824 MHz |  | – |  | 849 MHz |  | 869 MHz |  |  | – |  | 894 MHz |  |  |  |
|  | 12 | 699 MHz |  | – |  | 716 MHz |  | 729 MHz |  |  | – |  | 746 MHz |  |  |  |
| CA_4-5-13 | 4 | 1710 MHz |  | – |  | 1755 MHz |  | 2110 MHz |  |  | – |  | 2155 MHz |  | FDD |  |
|  | 5 | 824 MHz |  | – |  | 849 MHz |  | 869 MHz |  |  | – |  | 894 MHz |  |  |  |
|  | 13 | 777 MHz |  | – |  | 787 MHz |  | 746 MHz |  |  | – |  | 756 MHz |  |  |  |
| CA_4-5-29 | 4 | 1710 MHz |  | – |  | 1755 MHz |  | 2110 MHz |  |  | – |  | 2155 MHz |  | FDD |  |
|  | 5 | 824 MHz |  | – |  | 849 MHz |  | 869 MHz |  |  | – |  | 894 MHz |  |  |  |
|  | 29 | N/A |  |  |  |  |  | 717 MHz |  |  | – |  | 728 MHz |  |  |  |
| CA_4-5-30 | 4 | 1710 MHz |  | – |  | 1755 MHz |  | 2110 MHz |  |  | – |  | 2155 MHz |  | FDD |  |
|  | 5 | 824 MHz |  | – |  | 849 MHz |  | 869 MHz |  |  | – |  | 894 MHz |  |  |  |
|  | 30 | 2305 MHz |  | – |  | 2315 MHz |  | 2350 MHz |  |  | – |  | 2360 MHz |  |  |  |
| CA_4-7-12 | 4 | 1710 MHz |  | – |  | 1755 MHz |  | 2110 MHz |  |  | – |  | 2155 MHz |  | FDD |  |
|  | 7 | 2500 MHz |  | – |  | 2570 MHz |  | 2620 MHz |  |  | – |  | 2690 MHz |  |  |  |
|  | 12 | 699 MHz |  | – |  | 716 MHz |  | 729 MHz |  |  | – |  | 746 MHz |  |  |  |
| CA_4-12-30 | 4 | 1710 MHz |  | – |  | 1755 MHz |  | 2110 MHz |  |  | – |  | 2155 MHz |  | FDD |  |
|  | 12 | 699 MHz |  | – |  | 716 MHz |  | 729 MHz |  |  | – |  | 746 MHz |  |  |  |
|  | 30 | 2305 MHz |  | – |  | 2315 MHz |  | 2350 MHz |  |  | – |  | 2360 MHz |  |  |  |
| CA_4-29-30 | 4 | 1710 MHz |  | – |  | 1755 MHz |  | 2110 MHz |  |  | – |  | 2155 MHz |  | FDD |  |
|  | 29 | N/A |  |  |  |  |  | 717 MHz |  |  | – |  | 728 MHz |  |  |  |
|  | 30 | 2305 MHz |  | – |  | 2315 MHz |  | 2350 MHz |  |  | – |  | 2360 MHz |  |  |  |
| CA_7-8-20 | 7 | 2500 MHz |  | – |  | 2570 MHz |  | 2620 MHz |  |  | – |  | 2690 MHz |  | FDD |  |
|  | 8 | 880 MHz |  | – |  | 915 MHz |  | 925 MHz |  |  | – |  | 960 MHz |  |  |  |
|  | 20 | 832 MHz |  | – |  | 862 MHz |  | 791 MHz |  |  | – |  | 821 MHz |  |  |  |
| CA_7-20-38 | 7 | N/A |  |  | – |  | N/A |  |  | 2620 MHz | – |  | 2690 MHz |  | FDD |  |
|  | 20 | 832 MHz |  |  | – |  | 862 MHz |  |  | 791 MHz | – |  | 821 MHz |  |  |  |
|  | 38 | N/A |  |  | – |  | N/A |  |  | 2570 MHz | – |  | 2620 MHz |  | TDD |  |
| CA_19-21-42 | 19 | 830 MHz |  | – |  | 845 MHz |  | 875 MHz |  |  | – |  | 890 MHz |  | FDD |  |
|  | 21 | 1447.9 MHz |  | – |  | 1462.9 MHz |  | 1495.9 MHz |  |  | – |  | 1510.9 MHz |  |  |  |
|  | 42 | 3400 MHz |  | – |  | 3600 MHz |  | 3400 MHz |  |  | – |  | 3600 MHz |  | TDD |  |
| NOTE 1: The frequency range in band 28 is restricted for this CA band combination. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### <<Unchanged sections omitted>>