3GPP TSG-RAN5 Meeting #73	R5-168038

# Reno, Nevada, United States, 14th Nov 2016 - 18th Nov 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 2993 | rev | - | Current version: | 14.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates of 7.3.3 Delta RIB for CA_1A-3A-41A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_R14-UEConTest |  |  |  |  |  | Date: |  |  | 2016-10-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Delta RIB for new R14 CA combination CA_1A-3A-41A is currently missing in 36.521-1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Impacted Rel-14 CA configurations: CA_1A-3A-41A Updated Delta RIB for CA_1A-3A-41A |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Delta RIB for 3DL CA configuration will remain incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### <<Unchanged sections omitted>>

## 7.3	Reference sensitivity level

Editor's note:	FDD/TDD aspects missing or not yet determined:

-	The Maximum Sensitivity Degradation figures for large transmission configurations are not finalised in the core specification.

Editor's note: This clause is also incomplete for CA_42A-46A and CA_7A-8A-20.

-	ΔRIB,c is in square brackets (awaiting for RAN4 decision).

### 7.3.1	Test purpose

To verify the UE's ability to receive data with a given average throughput for a specified reference measurement channel, under conditions of low signal level, ideal propagation and no added noise.

A UE unable to meet the throughput requirement under these conditions will decrease the effective coverage area of an e-NodeB.

### 7.3.2	Test applicability

This test applies to all types of E-UTRA UE release 8 and forward.

### 7.3.3	Minimum conformance requirements

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.3-1 and Table 7.3.3-2.

Table 7.3.3-1: Reference sensitivity QPSK PREFSENS

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| 1 | - | - | -100 | -97 | -95.2 | -94 | FDD |
| 2 | -102.7 | -99.7 | -98 | -95 | -93.2 | -92 | FDD |
| 3 | -101.7 | -98.7 | -97 | -94 | -92.2 | -91 | FDD |
| 4 | -104.7 | -101.7 | -100 | -97 | -95.2 | -94 | FDD |
| 5 | -103.2 | -100.2 | -98 | -95 |  |  | FDD |
| 6 | - | - | -100 | -97 |  |  | FDD |
| 7 | - | - | -98 | -95 | -93.2 | -92 | FDD |
| 8 | -102.2 | -99.2 | -97 | -94 |  |  | FDD |
| 9 | - | - | -99 | -96 | -94.2 | -93 | FDD |
| 10 | - | - | -100 | -97 | -95.2 | -94 | FDD |
| 11 | - | - | -100 | -97 |  |  | FDD |
| 12 | -101.7 | -98.7 | -97 | -94 |  |  | FDD |
| 13 |  |  | -97 | -94 |  |  | FDD |
| 14 |  | - | -97 | -94 |  |  | FDD |
| ... |  |  |  |  |  |  |  |
| 17 | - | - | -97 | -94 |  |  | FDD |
| 18 | - | - | -1007 | -977 | -95.27 | - | FDD |
| 19 | - | - | -100 | -97 | -95.2 | - | FDD |
| 20 |  |  | -97 | -94 | -91.2 | -90 | FDD |
| 21 |  |  | -100 | -97 | -95.2 |  | FDD |
| 22 |  |  | -97 | -94 | -92.2 | -91 | FDD |
| 23 | -104.7 | -101.7 | -100 | -97 | -95.2 | -94 | FDD |
| 24 |  |  | -100 | -97 |  |  | FDD |
| 25 | -101.2 | -98.2 | -96.5 | -93.5 | -91.7 | -90.5 | FDD |
| 26 | -102.7 | -99.7 | -97.56 | -94.56 | -92.76 |  | FDD |
| 27 | -103.2 | -100.2 | -98 | -95 |  |  | FDD |
| 28 |  | -100.2 | -98.5 | -95.5 | -93.7 | -91 | FDD |
| 30 | - | - | -99 | -96 | - | - | FDD |
| 31 | -99.0 | -95.7 | -93.5 |  |  |  | FDD |
| ... |  |  |  |  |  |  |  |
| 33 | - | - | -100 | -97 | -95.2 | -94 | TDD |
| 34 | - | - | -100 | -97 | -95.2 | - | TDD |
| 35 | -106.2 | -102.2 | -100 | -97 | -95.2 | -94 | TDD |
| 36 | -106.2 | -102.2 | -100 | -97 | -95.2 | -94 | TDD |
| 37 | - | - | -100 | -97 | -95.2 | -94 | TDD |
| 38 | - | - | -100 | -97 | -95.2 | -94 | TDD |
| 39 | - | - | -100 | -97 | -95.2 | -94 | TDD |
| 40 | - | - | -100 | -97 | -95.2 | -94 | TDD |
| 41 | - | - | -98 | -95 | -93.2 | -92 | TDD |
| 42 | - | - | -99 | -96 | -94.2 | -93 | TDD |
| 43 | - | - | -99 | -96 | -94.2 | -93 | TDD |
| 44 |  | [-100.2] | [-98] | [-95] | [-93.2] | [-92] | TDD |
| 45 | - | - | -100 | -97 | -95.2 | -94 | TDD |
| … |  |  |  |  |  |  |  |
| 65 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | FDD |
| 66 | -104.2 | -101.2 | -99.5 | -96.5 | -94.7 | -93.5 | FDD |
| Note 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5 Note 2: The Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 Note 3: The signal power is specified per port Note 4: For the UE which supports both Band 3 and Band 9 the reference sensitivity level is FFS. Note 5: For the UE which supports both Band 11 and Band 21 the reference sensitivity level is FFS. Note 6: 6 indicates that the requirement is modified by -0.5 dB when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. Note 7: For a UE that support both Band 18 and Band 26, the reference sensitivity level for Band 26 applies for the applicable channel bandwidths. |  |  |  |  |  |  |  |

For UE(s) equipped with 4 antenna ports the following additional requirements apply. The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.3-1a and Table 7.3.3-2.

Table 7.3.3-1a: Reference sensitivity QPSK PREFSENS

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| 1 |  |  | -102.7 | -99.7 | -97.9 | -96.7 | FDD |
| 2 | [-105.4] | [-102.4] | [-100.7] | [-97.7] | [-95.9] | [-94.7] | FDD |
| 3 | [-104.4] | [-101.4] | [-99.7] | [-96.7] | [-94.9] | [-93.7] | FDD |
| … |  |  |  |  |  |  |  |
| 7 |  |  | [-100.7] | [-97.7] | [-95.9] | [-94.7] | FDD |
| … |  |  |  |  |  |  |  |
| 20 |  |  | [-99.7] | [-96.7] | [-93.9] | [-92.7] | FDD |
| … |  |  |  |  |  |  |  |
| 39 |  |  | [-102.7] | [-99.7] | [-97.9] | [-96.7] | TDD |
| … |  |  |  |  |  |  |  |
| 41 |  |  | [-100.7] | [-97.7] | [-95.9] | [-94.7] | TDD |
| 42 |  |  | [-101.2] | [-98.2] | [-96.4] | [-95.2] | TDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5 NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: The signal power is specified per port. |  |  |  |  |  |  |  |

The reference receive sensitivity (REFSENS) requirement specified in Table 7.3.3-1 (two antenna ports) and Table 7.3.3-1a (four antenna ports) shall be met for an uplink transmission bandwidth less than or equal to that specified in Table 7.3.3-2.

NOTE:	Table 7.3.3-2 is intended for conformance tests and does not necessarily reflect the operational conditions of the network, where the number of uplink and downlink allocated resource blocks will be practically constrained by other factors. Typical receiver sensitivity performance with HARQ retransmission enabled and using a residual BLER metric relevant for e.g. Speech Services is given in the Annex G of TS 36.101 [2] (informative).

For the UE which supports inter-band carrier aggregation configuration with the uplink in one or two E-UTRA bands, the minimum requirement for reference sensitivity in Table 7.3.3-1 and Table 7.3.3-1a shall be increased by the amount given in ΔRIB,c in Table 7.3.3-1A, Table 7.3.3-1B and Table 7.3.3-1C for the applicable E-UTRA bands.

Table 7.3.3-1A: ΔRIB,c (two bands)

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1A-3A | 1 | 0 |
|  | 3 | 0 |
| CA_1A-3C | 1 | 0 |
|  | 3 | 0 |
| CA_1A-5A | 1 | 0 |
|  | 5 | 0 |
| CA_1A-7A | 1 | 0 |
|  | 7 | 0 |
| CA_1A-7C | 1 | 0 |
|  | 7 | 0 |
| CA_1A-8A | 1 | 0 |
|  | 8 | 0 |
| CA_1A-11A | 1 | 0 |
|  | 11 | 0 |
| CA_1A-18A | 1 | 0 |
|  | 18 | 0 |
| CA_1A-19A | 1 | 0 |
|  | 19 | 0 |
| CA_1A-20A | 1 | 0 |
|  | 20 | 0 |
| CA_1A-21A | 1 | 0 |
|  | 21 | 0 |
| CA_1A-26A | 1 | 0 |
|  | 26 | 0 |
| CA_1A-28A | 1 | 0 |
|  | 28 | 0.2 |
| CA_1A-40A | 1 | 0 |
|  | 40 | 0 |
| CA_1A-41A8 | 1 | 0 |
|  | 41 | 0 |
| CA_1A-41C8 | 1 | 0 |
|  | 41 | 0 |
| CA_1A-42A | 1 | 0 |
|  | 42 | 0.5 |
| CA_1A-42C | 1 | 0 |
|  | 42 | 0.5 |
| CA_2A-4A | 2 | 0.3 |
|  | 4 | 0.3 |
| CA_2A-2A-4A | 2 | 0.3 |
|  | 4 | 0.3 |
| CA_2A-4A-4A | 2 | 0.3 |
|  | 4 | 0.3 |
| CA_2A-2A-4A-4A | 2 | 0.3 |
|  | 4 | 0.3 |
| CA_2A-5A | 2 | 0 |
|  | 5 | 0 |
| CA_2A-2A-5A | 2 | 0 |
|  | 5 | 0 |
| CA_2C-5A | 2 | 0 |
|  | 5 | 0 |
| CA_2A-7A | 2 | 0 |
|  | 7 | 0 |
| CA_2A-12A | 2 | 0 |
|  | 12 | 0 |
| CA_2A-2A-12A | 2 | 0 |
|  | 12 | 0 |
| CA_2A-2A-12B | 2 | 0 |
|  | 12 | 0 |
| CA_2A-12B | 2 | 0 |
|  | 12 | 0 |
| CA_2C-12A | 2 | 0 |
|  | 12 | 0 |
| CA_2A-13A | 2 | 0 |
|  | 13 | 0 |
| CA_2A-2A-13A | 2 | 0 |
|  | 13 | 0 |
| CA_2A-17A | 2 | 0 |
|  | 17 | 0.5 |
| CA_2A_28A | 2 | 0 |
|  | 28 | 0 |
| CA_2A-30A | 2 | 0.4 |
|  | 30 | 0.5 |
| CA_2C-30A | 2 | 0.4 |
|  | 30 | 0.5 |
| CA_3A-5A | 3 | 0 |
|  | 5 | 0 |
| CA_3C-5A | 3 | 0 |
|  | 5 | 0 |
| CA_3A-7A | 3 | 0 |
|  | 7 | 0 |
| CA_3A-7B | 3 | 0 |
|  | 7 | 0 |
| CA_3A-7C | 3 | 0 |
|  | 7 | 0 |
| CA_3C-7A | 3 | 0 |
|  | 7 | 0 |
| CA_3C-7C | 3 | 0 |
|  | 7 | 0 |
| CA_3A-8A | 3 | 0 |
|  | 8 | 0 |
| CA_3A-3A-8A | 3 | 0 |
|  | 8 | 0 |
| CA_3A-19A | 3 | 0 |
|  | 19 | 0 |
| CA_3A-20A | 3 | 0 |
|  | 20 | 0 |
| CA_3A-26A | 3 | 0 |
|  | 26 | 0 |
| CA_3A-27A | 3 | 0 |
|  | 27 | 0 |
| CA_3A-28A | 3 | 0 |
|  | 28 | 0 |
| CA_3C-28A | 3 | 0 |
|  | 28 | 0 |
| CA_3A-31A | 3 | 0 |
|  | 31 | 0.2 |
| CA_3A-38A | 3 | 0 |
|  | 38 | 0 |
| CA_3A-40A | 3 | 0 |
|  | 40 | 0 |
| CA_3A-40C | 3 | 0 |
|  | 40 | 0 |
| CA_3A-41A | 3 | 0 |
|  | 41 | 010 |
|  |  | 0.511 |
| CA_3A-41C | 3 | 0 |
|  | 41 | 010 |
|  |  | 0.511 |
| CA_3A-42A | 3 | 0.2 |
|  | 42 | 0.5 |
| CA_3A-42C | 3 | 0.2 |
|  | 42 | 0.5 |
| CA_4A-5A | 4 | 0 |
|  | 5 | 0 |
| CA_4A-4A-5A | 4 | 0 |
|  | 5 | 0 |
| CA_4A-7A | 4 | 0.5 |
|  | 7 | 0.5 |
| CA_4A-4A-7A | 4 | 0.5 |
|  | 7 | 0.5 |
| CA_4A-12A | 4 | 0 |
|  | 12 | 0.5 |
| CA_4A-12B | 4 | 0 |
|  | 12 | 0.5 |
| CA_4A-4A-12A | 4 | 0 |
|  | 12 | 0.5 |
| CA_4A-4A-13A | 4 | 0 |
|  | 13 | 0 |
| CA_4A-13A | 4 | 0 |
|  | 13 | 0 |
| CA_4A-17A | 4 | 0 |
|  | 17 | 0.5 |
| CA_4A-27A | 4 | 0 |
|  | 27 | 0 |
| CA_4A-28A | 4 | 0 |
|  | 28 | 0.2 |
| CA_4A-30A | 4 | 0.4 |
|  | 30 | 0.5 |
| CA_4A-4A-30A | 4 | 0.4 |
|  | 30 | 0.5 |
| CA_5A-7A | 5 | 0 |
|  | 7 | 0 |
| CA_5A-12A | 5 | 0.5 |
|  | 12 | 0.3 |
| CA_5A-12B | 5 | 0.5 |
|  | 12 | 0.3 |
| CA_5A-13A | 5 | 0 |
|  | 13 | 0 |
| CA_5A-17A | 5 | 0.5 |
|  | 17 | 0.3 |
| CA_5A-25A | 5 | 0 |
|  | 25 | 0 |
| CA_5A-30A | 5 | 0 |
|  | 30 | 0 |
| CA_5A-38A | 5 | 0 |
|  | 38 | 0 |
| CA_5A-40A | 5 | 0 |
|  | 40 | 0 |
| CA_5A-40C | 5 | 0 |
|  | 40 | 0 |
| CA_7A-8A | 7 | 0 |
|  | 8 | 0.2 |
| CA_7A-12A | 7 | 0 |
|  | 12 | 0 |
| CA_7A-20A | 7 | 0 |
|  | 20 | 0 |
| CA_7A-22A | 7 | 0 |
|  | 22 | 0.5 |
| CA_7A-28A | 7 | 0 |
|  | 28 | 0 |
| CA_7B-28A | 7 | 0 |
|  | 28 | 0 |
| CA_7C-28A | 7 | 0 |
|  | 28 | 0 |
| CA_7A-40A | 7 | 0 |
|  | 40 | 0.5 |
| CA_7A-40C | 7 | 0 |
|  | 40 | 0.5 |
| CA_7A-42A | 7 | 0 |
|  | 42 | 0.5 |
| CA_7A-42A-42A | 7 | 0 |
|  | 42 | 0.5 |
| CA_8A-11A | 8 | 0 |
|  | 11 | 0 |
| CA_8A-20A | 8 | 0 |
|  | 20 | 0 |
| CA_8A-40A | 8 | 0 |
|  | 40 | 0 |
| CA_8A-41A | 8 | 0 |
|  | 41 | 0 |
| CA_8A-41C | 8 | 0 |
|  | 41 | 0 |
| CA_8A-42A | 8 | 0.2 |
|  | 42 | 0.5 |
| CA_8A-42C | 8 | 0.2 |
|  | 42 | 0.5 |
| CA_11A-18A | 11 | 0 |
|  | 18 | 0 |
| CA_12A-25A | 12 | 0 |
|  | 25 | 0 |
| CA_12A-30A | 12 | 0 |
|  | 30 | 0 |
| CA_18A-28A9 | 18 | 0 |
|  | 28 | 0 |
| CA_19A-21A | 19 | 0 |
|  | 21 | 0 |
| CA_19A-28A9 | 19 | 0 |
|  | 28 | 0 |
| CA_19A-42A | 19 | 0 |
|  | 42 | 0.5 |
| CA_19A-42C | 19 | 0 |
|  | 42 | 0.5 |
| CA_20A-31A | 20 | 0 |
|  | 31 | 0 |
| CA_20A-38A | 20 | 0 |
|  | 38 | 0 |
| CA_20A-40A | 20 | 0 |
|  | 40 | 0 |
| CA_20A-42A | 20 | 0 |
|  | 42 | 0.5 |
| CA_20A-42A-42A | 20 | 0 |
|  | 42 | 0.5 |
| CA_21A-42A | 21 | 0.2 |
|  | 42 | 0.5 |
| CA_21A-42C | 21 | 0.2 |
|  | 42 | 0.5 |
| CA_25A-26A | 25 | 0 |
|  | 26 | 0 |
| CA_25A-41A8 | 25 | 0 |
|  | 41 | 0 |
| CA_25A-41C8 | 25 | 0 |
|  | 41 | 0 |
| CA_25A-41D8 | 25 | 0 |
|  | 41 | 0 |
| CA_26A-41A | 26 | 0 |
|  | 41 | 0 |
| CA_26A-41C | 26 | 0 |
|  | 41 | 0 |
| CA_28A-40A | 28 | 0 |
|  | 40 | 0 |
| CA_28A-40C | 28 | 0 |
|  | 40 | 0 |
| CA_28A-40D | 28 | 0 |
|  | 40 | 0 |
| CA_28A-41A | 28 | 0 |
|  | 41 | 0 |
| CA_28A-41C | 28 | 0 |
|  | 41 | 0 |
| CA_28A-42A | 28 | 0.2 |
|  | 42 | 0.5 |
| CA_28A-42C | 28 | 0.2 |
|  | 42 | 0.5 |
| CA_38A-40A | 38 | 0.54 |
|  | 40 | 0.54 |
| CA_38A-40A-40A | 38 | 0.54 |
|  | 40 | 0.54 |
| CA_38A-40C | 38 | 0.54 |
|  | 40 | 0.54 |
| CA_39A-41A | 39 | 0.24 |
|  | 41 | 0.24 |
| CA_39A-41A | 39 | 0.27 |
|  | 41 | 0.27 |
| CA_39A-41C | 39 | 0.24 |
|  | 41 | 0.24 |
| CA_39A-41C | 39 | 0.27 |
|  | 41 | 0.27 |
| CA_39A-41D | 39 | 0.24 |
|  | 41 | 0.24 |
| CA_39C-41A | 39 | 0.24 |
|  | 41 | 0.24 |
| CA_39C-41A | 39 | 0.27 |
|  | 41 | 0.27 |
| CA_39C-41C | 39 | 0.24 |
|  | 41 | 0.24 |
| CA_41A-42A | 41 | 0.44 |
|  | 42 | 0.54 |
| CA_41A-42C | 41 | 0.44 |
|  | 42 | 0.54 |
| CA_41C-42A | 41 | 0.44 |
|  | 42 | 0.54 |
| CA_41C-42C | 41 | 0.44 |
|  | 42 | 0.54 |
| CA_42A-46A | 42 | [0] |
| NOTE 1: The above additional tolerances are only applicable for the E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations NOTE 2: The above additional tolerances also apply in intra-band CA and non-aggregated operation for the supported E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations. NOTE 3: In case the UE supports more than one of the above 2DL inter-band carrier aggregation configurations and a E-UTRA operating band belongs to more than one 2DL inter-band carrier aggregation configurations then: - When the E-UTRA operating band frequency range is ≤ 1GHz, the applicable additional tolerance shall be the average of the 2DL tolerances in Table 7.3.3-1A, truncated to one decimal place that would apply for that operating band among the supported 2DL CA configurations. In case there is a harmonic relation between low band UL and high band DL, then the maximum tolerance among the different supported 2DL carrier aggregation configurations involving such band shall be applied - When the E-UTRA operating band frequency range is >1GHz, the applicable additional tolerance shall be the maximum 2DL tolerance in Table 7.3.3-1A that would apply for that operating band among the supported 2DL CA configurations NOTE 4: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. NOTE 5: Unless otherwise specified, in case the UE supports more than one of the above 3DL inter-band carrier aggregation configurations and a E-UTRA operating band belongs to more than one 3DL inter-band carrier aggregation configurations then: - When the E-UTRA operating band frequency range is ≤ 1GHz and the tolerances are the same, the value applies to the band. If the tolerances are different, the applicable additional 3DL tolerance is FFS. In case there is a harmonic relation between low band UL and high band DL, then the maximum tolerance among the different supported 3DL carrier aggregation configurations involving such band shall be applied. - When the E-UTRA operating band frequency range is >1GHz, the applicable additional 3DL tolerance shall be the maximum tolerance above that applies for that operating band among the supported 3DL CA configurations. NOTE 6: The above additional tolerances applicable for the E-UTRA operating bands that belong to the supported highest order inter-band carrier aggregation configuration, also applies to the same E-UTRA operating bands that belong to a supported lower order CA configuration. NOTE 7: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx. NOTE 8: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in the FDD band. NOTE 9: For Band 28, the requirements only apply for the restricted frequency range specified for this CA configuration (Table 5.2A-2). NOTE 10: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 11: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

Table 7.3.3-1B: ΔRIB,c (three bands)

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1A-3A-5A | 1 | 0 |
|  | 3 | 0 |
|  | 5 | 0 |
| CA_1A-3A-7A | 1 | 0 |
|  | 3 | 0 |
|  | 7 | 0 |
| CA_1A-3A-7C | 1 | 0 |
|  | 3 | 0 |
|  | 7 | 0 |
| CA_1A-3A-8A | 1 | 0 |
|  | 3 | 0 |
|  | 8 | 0 |
| CA_1A-3A-19A | 1 | 0 |
|  | 3 | 0 |
|  | 19 | 0 |
| CA_1A-3A-20A | 1 | 0 |
|  | 3 | 0 |
|  | 20 | 0 |
| CA_1A-3A-26A | 1 | 0 |
|  | 3 | 0 |
|  | 26 | 0 |
| CA_1A-3A-28A | 1 | 0 |
|  | 3 | 0 |
|  | 28 | 0.2 |
| CA_1A-3A-40A | 1 | 0 |
|  | 3 | 0 |
|  | 40 | 0 |
| CA_1A-3A-41A | 1 | 0 |
|  | 3 | 0 |
|  | 41 | 05/0.56 |
| CA_1A-3A-42A | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 42 | 0.5 |
| CA_1A-3A-42C | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 42 | 0.5 |
| CA_1A-5A-7A | 1 | 0 |
|  | 5 | 0 |
|  | 7 | 0 |
| CA_1A-5A-40A | 1 | 0 |
|  | 5 | 0 |
|  | 40 | 0 |
| CA_1A-7A-8A | 1 | 0 |
|  | 7 | 0 |
|  | 8 | 0.2 |
| CA_1A-7A-20A | 1 | 0 |
|  | 7 | 0 |
|  | 20 | 0 |
| CA_1A-7A-28A | 1 | 0 |
|  | 7 | 0 |
|  | 28 | 0.2 |
| CA_1A-7C-28A | 1 | 0 |
|  | 7 | 0 |
|  | 28 | 0.2 |
| CA_1A-8A-11A | 1 | 0 |
|  | 8 | 0 |
|  | 11 | 0 |
| CA_1A-8A-40A | 1 | 0 |
|  | 8 | 0 |
|  | 40 | 0 |
| CA_1A-11A-18A | 1 | 0 |
|  | 11 | 0 |
|  | 18 | 0 |
| CA_1A-18A-28A | 1 | 0 |
|  | 18 | 0 |
|  | 28 | 0 |
| CA_1A-19A-21A | 1 | 0 |
|  | 19 | 0 |
|  | 21 | 0 |
| CA_1A-19A-28A | 1 | 0 |
|  | 19 | 0 |
|  | 28 | 0 |
| CA_1A-19A-42A | 1 | 0 |
|  | 19 | 0 |
|  | 42 | 0.5 |
| CA_1A-19A-42C | 1 | 0 |
|  | 19 | 0 |
|  | 42 | 0.5 |
| CA_1A-21A-42A | 1 | 0 |
|  | 21 | 0.2 |
|  | 42 | 0.5 |
| CA_1A-21A-42C | 1 | 0 |
|  | 21 | 0.2 |
|  | 42 | 0.5 |
| CA_1A-41A-42A7 | 1 | 0 |
|  | 41 | 0 |
|  | 42 | 0.5 |
| CA_2A-2A-4A-12A | 2 | 0.3 |
|  | 4 | 0.3 |
|  | 12 | 0.5 |
| CA_2A-4A-5A | 2 | 0.3 |
|  | 4 | 0.3 |
|  | 5 | 0 |
| CA_2A-2A-4A-5A | 2 | 0.3 |
|  | 4 | 0.3 |
|  | 5 | 0 |
| CA_2A-4A-4A-5A | 2 | 0.3 |
|  | 4 | 0.3 |
|  | 5 | 0 |
| CA_2A-4A-7A | 2 | 0.3 |
|  | 4 | 0.5 |
|  | 7 | 0.5 |
| CA_2A-4A-12A | 2 | 0.3 |
|  | 4 | 0.3 |
|  | 12 | 0.5 |
| CA_2A-4A-4A-12A | 2 | 0.3 |
|  | 4 | 0.3 |
|  | 12 | 0.5 |
| CA_2A-4A-13A | 2 | 0.3 |
|  | 4 | 0.3 |
|  | 13 | 0 |
| CA_2A-4A-30A | 2 | 0.4 |
|  | 4 | 0.4 |
|  | 30 | 0.5 |
| CA_2A-5A-12A | 2 | 0 |
|  | 5 | 0.5 |
|  | 12 | 0.3 |
| CA_2A-2A-5A-12A | 2 | 0.3 |
|  | 5 | 0.5 |
|  | 12 | 0.3 |
| CA_2A-5A-12B | 2 | 0 |
|  | 5 | 0.5 |
|  | 12 | 0.3 |
| CA_2A-5A-13A | 2 | 0 |
|  | 5 | 0 |
|  | 13 | 0 |
| CA_2A-5A-30A | 2 | 0.4 |
|  | 5 | 0 |
|  | 30 | 0.5 |
| CA_2C-5A-30A | 2 | 0.4 |
|  | 5 | 0 |
|  | 30 | 0.5 |
| CA_2A-7A-12A | 2 | 0 |
|  | 7 | 0 |
|  | 12 | 0 |
| CA_2A-12A-30A | 2 | 0.4 |
|  | 12 | 0 |
|  | 30 | 0.5 |
| CA_2C-12A-30A | 2 | 0.4 |
|  | 12 | 0 |
|  | 30 | 0.5 |
| CA_3A-5A-40A | 1 | 0 |
|  | 5 | 0 |
|  | 40 | 0 |
| CA_3A-7A-8A | 3 | 0 |
|  | 7 | 0 |
|  | 8 | 0.2 |
| CA_3A-7A-20A | 3 | 0 |
|  | 7 | 0 |
|  | 20 | 0 |
| CA_3A-7A-28A | 3 | 0 |
|  | 7 | 0 |
|  | 28 | 0 |
| CA_3A-7C-28A | 3 | 0 |
|  | 7 | 0 |
|  | 28 | 0 |
| CA_3C-7A-28A | 3 | 0 |
|  | 7 | 0 |
|  | 28 | 0 |
| CA_3C-7C-28A | 3 | 0 |
|  | 7 | 0 |
|  | 28 | 0 |
| CA_3A-7A-38A | 3 | 0 |
|  | 7 | 0 |
|  | 38 | 0.2 |
| CA_3A-8A-40A | 3 | 0 |
|  | 8 | 0 |
|  | 40 | 0 |
| CA_3A-19A-42A | 3 | 0.2 |
|  | 19 | 0 |
|  | 42 | 0.5 |
| CA_3A-19A-42C | 3 | 0.2 |
|  | 19 | 0 |
|  | 42 | 0.5 |
| CA_3A-28A-40A | 3 | 0 |
|  | 28 | 0 |
|  | 40 | 0 |
| CA_3A-28A-40C | 3 | 0 |
|  | 28 | 0 |
|  | 40 | 0 |
| CA_3A-41A-42A | 3 | 0.5 |
|  | 41 | 05/0.56 |
|  | 42 | 0.5 |
| CA_4A-5A-12A | 4 | 0 |
|  | 5 | 0.5 |
|  | 12 | 0.5 |
| CA_4A-4A-5A-12A | 4 | 0 |
|  | 5 | 0.5 |
|  | 12 | 0.5 |
| CA_4A-5A-13A | 4 | 0 |
|  | 5 | 0 |
|  | 13 | 0 |
| CA_4A-5A-30A | 4 | 0.4 |
|  | 5 | 0 |
|  | 30 | 0.5 |
| CA_4A-4A-5A-30A | 4 | 0.4 |
|  | 5 | 0 |
|  | 30 | 0.5 |
| CA_4A-7A-12A | 4 | 0.5 |
|  | 7 | 0.5 |
|  | 12 | 0.5 |
| CA_4A-12A-30A | 4 | 0.4 |
|  | 12 | 0.5 |
|  | 30 | 0.5 |
| CA_4A-4A-12A-30A | 4 | 0.4 |
|  | 12 | 0.5 |
|  | 30 | 0.5 |
| CA_7A-8A-20A | 7 | 0 |
|  | 8 | 0.2 |
|  | 20 | [0.2] |
| CA_7A-20A-38A | 7 | 0 |
|  | 20 | 0 |
|  | 38 | 0.2 |
| CA_19A-21A-42A | 19 | 0 |
|  | 21 | 0.2 |
|  | 42 | 0.5 |
| CA_19A-21A-42C | 19 | 0 |
|  | 21 | 0.2 |
|  | 42 | 0.5 |
| NOTE 1: The above additional tolerances are only applicable for the E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations. NOTE 2: The above additional tolerances also apply in intra-band and non-aggregated operation for the supported E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations. NOTE 3: Unless otherwise specified, in case the UE supports more than one of the above 3DL inter-band carrier aggregation configurations and a E-UTRA operating band belongs to more than one 3DL inter-band carrier aggregation configurations then: - When the E-UTRA operating band frequency range is ≤ 1GHz and the tolerances are the same, the value applies to the band. If the tolerances are different, the applicable additional 3DL tolerance is FFS. In case there is a harmonic relation between low band UL and high band DL, then the maximum tolerance among the different supported 3DL carrier aggregation configurations involving such band shall be applied. - When the E-UTRA operating band frequency range is >1GHz, the applicable additional 3DL tolerance shall be the maximum tolerance above that applies for that operating band among the supported 3DL CA configurations. NOTE 4: The above additional tolerances applicable for the E-UTRA operating bands that belong to the supported highest order inter-band carrier aggregation configuration, also applies to the same E-UTRA operating bands that belong to a supported lower order CA configuration. NOTE 5: The requirement is specified for the frequency range of 2545-2690MHz. NOTE 6: The requirement is specified for the frequency range of 2496-2545MHz. NOTE 7: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 1 or Band 42. |  |  |

### <<Unchanged sections omitted>>