3GPP TSG RAN WG5 Meeting #73	 R5-169549

# Nevada, US 14th – 18th November 2016

| Title: | Discussion for test configuration table format of RX for 4DL with UL CA |
| --- | --- |
| Source: | SGS Wireless |
| Agenda Item: | 5.3.10.2 |
| Document for: | Discussion and endorsement |

1	Introduction

At RAN#70, Rel-13 CA WI was approved by [1] and test configuration table format for 4DL with UL CA needs to be discussed as did in 3DL/1UL case.

2	Discussion

According to the discussion paper by [2] in RAN5#70 meeting, the following 8 types of 4DL/1UL CA shown in Table 2-1 can be put in together.

Table 2-1: CA types list for 4DL/1UL CA in TS 36.101 v13.2.0

| CA types | Classes |
| --- | --- |
| All intra C | CA_XE |
| inter + intra C(3CCs) | CA_XA-YD/CA_XD-YA |
| inter + intra C(2CCs) | CA_XA-YA-ZC/CA_XA-YC-ZA/CA_XC-YA-ZA CA_XA-YA-ZB |
| All inter | CA_WA-XA-YA-ZA |
| intra C(2CCs) + intra C(2CCs) | CA_XC-YC |
| intra NC + intra NC | CA_XA-XA-YA-YA |
| intra NC + Intra C(2CCs) | CA_XA-XA-YB |
| intra NC+inter | CA_XA-XA-YA-ZA/CA_XA-YA-YA-ZA |

- Putting all the CA types in together/in the same table format:

In discussion paper [2], all intra-band contiguous (highlighted by orange in Table 2-1) needs to consider defined and make in a specified Table as Table 2-2, the types of inter-band relative and intra-band contiguous + intra-band contiguous (highlighted by green & aquamarine in Table 2-1) can be put in the same table format like Table 2-3.

- Considering a new table format for 3DL/4DL/5DL CA with/without UL CA

In last RAN5 #71 meeting the TC title for “4DL CA with UL CA” and “4DL CA without UL CA” is defined as “xxx for 4DL CA” which is meant both should consider in together, in view of this, the TC title for “3DL CA with UL CA” and “3DL CA without UL CA” is also changed to “xxx for 3DL CA”.

In this meeting, we are going to introduce a new configuration table format considering different CA types with single UL PCC allocation and UL CA.

- Older Table Format VS. New Table Format

The Table 2-2 & Table 2-4 is older format with suggestion from discussion paper [2].

In this table format the “UL Allocation” is located in the rightmost of table; the configuration table will become wider if considering with UL CA.

The Table 2-3 & Table 2-5 is a new format which is made by comment [3].

In this table all of older title/ID/Parameters are continuous to use and the difference is adding new column under each cc for UL allocation.

- Separating into two groups (Intra-band contiguous relative & Intra-band non-contiguous relative)

Finally, all intra-band contiguous & inter-band can be put in together, and intra-band non-contiguous & Inter-band can be put in together as shown in Table 2-6.

NOTE: Test points in these tables below are just example and we are not proposing default test points for each CA type.

Table 2-2: Test Configuration Table (Intra-band contiguous) (Older format)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  |  |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. For mapping within Band refer to “CC” columns |  |  |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  |  |  |  | Refer to “PCC NRB”and “SCCs NRB ” columns |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / NRB_agg |  |  |  |  |  |  |  |  |  | DL Allocation |  |  |  |  | UL Allocation (Note 2,3) |  |
|  | CA Configuration |  |  |  |  |  | PCC NRB | SCCs NRB |  |  | CC MOD | PCC & SCCs RB allocation |  |  |  | CC MOD | PCC LCRB @ RBstart |
|  | Band | Range | PCC | SCC1 | SCC2 | SCC3 |  | SCC1 | SCC2 | SCC3 |  | PCC | SCC1 | SCC2 | SCC3 |  |  |
|  |  |  | CC | CC | CC | CC |  |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a CA_XE Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Low | CC1 | CC2 | CC3 | CC4 | Highest NRB_agg |  |  |  | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| 2 | X | Low | CC1 | CC2 | CC3 | CC4 | Lowest NRB_agg |  |  |  | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| 3 | X | High | CC1 | CC2 | CC3 | CC4 | Highest NRB_agg |  |  |  | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| 4 | X | High | CC1 | CC2 | CC3 | CC4 | Lowest NRB_agg |  |  |  | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| Note: TBD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

New additional column for difference MOD

Table 2-3: Test Configuration Table (Intra-band contiguous) (New format)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  |  |  |  |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. For mapping within Band refer to “CC” columns |  |  |  |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  |  |  |  |  |  | Refer to “PCC NRB”and “SCCs NRB ” columns |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | PCC |  |  |  | SCC1 |  |  |  | SCC2 |  |  |  | SCC3 |  |  |  | SCC4 |  |  |  |
|  | Band | Range | NRB |  | Band | Range | NRB |  | Band | Range | NRB |  | Band | Range | NRB |  | Band | Range | NRB |  |
|  | UL MOD | DL MOD | ULalloc | DLalloc | UL MOD | DL MOD | ULalloc | DLalloc | UL MOD | DL MOD | ULalloc | DLalloc | UL MOD | DL MOD | ULalloc | DLalloc | UL MOD | DL MOD | ULalloc | DLalloc |
| Default Test Settings for a CA_XE Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Low/CC1 | REFSENS | All RBs | X | Low/CC2 | N/A | All RBs | X | Low/CC3 | N/A | All RBs | X | Low/CC4 | N/A | All RBs |  |  |  |  |
|  | QPSK | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg |  |  |  |  |
| 2 | X | Low/CC1 | REFSENS | All RBs | X | Low/CC2 | N/A | All RBs | X | Low/CC3 | N/A | All RBs | X | Low/CC3 | N/A | All RBs |  |  |  |  |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg |  |  |  |  |
| 3 | X | High/CC1 | REFSENS | All RBs | X | High/CC2 | N/A | All RBs | X | High/CC3 | N/A | All RBs | X | High/CC4 | N/A | All RBs |  |  |  |  |
|  | QPSK | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg |  |  |  |  |
| 4 | X | High/CC1 | REFSENS | All RBs | X | High/CC2 | N/A | All RBs | X | High/CC3 | N/A | All RBs | X | High/CC4 | N/A | All RBs |  |  |  |  |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg |  |  |  |  |
| Note: TBD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 2-4: Test Configuration Table (Inter-band) (Older format)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  |  |  |  |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. For mapping within Band refer to “CC” columns |  |  |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  |  |  |  |  |  | Refer to “PCC NRB”and “SCCs NRB ” columns |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / NRB_agg |  |  |  |  |  |  |  |  |  |  |  | DL Allocation |  |  |  |  | UL Allocation (Note 2,3) |  |
|  | CA Configuration |  |  |  |  |  |  |  | PCC NRB | SCCs NRB |  |  | CC MOD | PCC & SCCs RB allocation |  |  |  | CC MOD | PCC LCRB @ RBstart |
|  | PCC |  | SCC1 |  | SCC2 |  | SCC3 |  |  | SCC1 | SCC2 | SCC3 |  | PCC | SCC1 | SCC2 | SCC3 |  |  |
|  | Band | Range | Band | Range | Band | Range | Band | Range |  |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a CA_WA-XA-YA-ZA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | W | Mid | X | Mid | Y | Mid | Z | Mid | Highest NRB for all CCs |  |  |  | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| 2 | X | Mid | W | Mid | Y | Mid | Z | Mid | Highest NRB for all CCs |  |  |  | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| 3 | Y | Mid | W | Mid | X | Mid | Z | Mid | Highest NRB for all CCs |  |  |  | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| 4 | Z | Mid | W | Mid | X | Mid | Y | Mid | Highest NRB for all CCs |  |  |  | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| Default Test Settings for a CA_XA-YD Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Mid | Y | Mid /CC1 | Y | Mid /CC2 | Y | Mid /CC3 | Highest NRB | Highest NRB_agg |  |  | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| 2 | Y | Low /CC1 | Y | Low /CC2 | Y | Low /CC3 | X | Mid | Highest NRB_agg |  |  | Highest NRB | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| 3 | Y | High /CC1 | Y | High /CC2 | Y | High /CC3 | X | Mid | Lowest NRB_agg |  |  | Highest NRB | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| 4 | Y | Low /CC1 | Y | Low /CC2 | Y | Low /CC3 | X | Mid | Highest NRB_agg |  |  | Highest NRB | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| 5 | Y | High /CC1 | Y | High /CC2 | Y | High /CC3 | X | Mid | Lowest NRB_agg |  |  | Highest NRB | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| Default Test Settings for a CA_XA-YA-ZC and CA_XA-YA-ZB Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Mid | Y | Mid | Z | Mid /CC1 | Z | Mid /CC2 | Highest NRB | Highest NRB | Highest NRB_agg |  | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| 2 | Y | Mid | X | Mid | Z | Mid /CC1 | Z | Mid /CC2 | Highest NRB | Highest NRB | Highest NRB_agg |  | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| 3 | Z | Low /CC1 | Z | Low /CC2 | X | Mid | Y | Mid | Highest NRB_agg |  | Highest NRB | Highest NRB | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| 4 | Z | Low /CC1 | Z | Low /CC2 | Y | Mid | X | Mid | Lowest NRB_agg |  | Highest NRB | Highest NRB | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| 5 | Z | High /CC1 | Z | High /CC2 | X | Mid | Y | Mid | Highest NRB_agg |  | Highest NRB | Highest NRB | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| 6 | Z | High /CC1 | Z | High /CC2 | Y | Mid | X | Mid | Lowest NRB_agg |  | Highest NRB | Highest NRB | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| Default Test Settings for a CA_XC-YC Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Low /CC1 | X | Low /CC2 | Y | Low /CC1 | Y | Low /CC2 | Highest NRB_agg |  | Highest NRB_agg |  | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| 2 | X | Low /CC1 | X | Low /CC2 | Y | Low /CC1 | Y | Low /CC2 | Lowest NRB_agg |  | Lowest NRB_agg |  | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| 3 | Y | High /CC1 | Y | High /CC2 | X | High /CC1 | X | High /CC2 | Highest NRB_agg |  | Highest NRB_agg |  | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| 4 | Y | High /CC1 | Y | High /CC2 | X | High /CC1 | X | High /CC2 | Lowest NRB_agg |  | Lowest NRB_agg |  | QPSK | All RBs |  |  |  | QPSK | REFSENS |
| Note: TBD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 2-5: Test Configuration Table (Inter-band) (New format)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in |  |  |  |  |  |  |  |  |  |  |  | NC |  |  |  |  |  |  |  |  |
| TS 36.508[7] subclause 4.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. For mapping within Band refer to “CC” columns |  |  |  |  |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  |  |  |  |  | Refer to “PCC NRB”and “SCCs NRB ” columns |  |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | PCC |  |  |  | SCC1 |  |  |  | SCC2 |  |  |  | SCC3 |  |  |  | SCC4 |  |  |  |
|  | Band | Range | NRB |  | Band | Range | NRB |  | Band | Range | NRB |  | Band | Range | NRB |  | Band | Range | NRB |  |
|  | UL MOD | DL MOD | ULalloc | DLalloc | UL MOD | DL MOD | ULalloc | DLalloc | UL MOD | DL MOD | ULalloc | DLalloc | UL MOD | DL MOD | ULalloc | DLalloc | UL MOD | DL MOD | ULalloc | DLalloc |
| Default Test Settings for a CA_XA-YA-ZA-RA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Mid | REFSENS | ALL RBs | Y | Mid | N/A | Highest NRB | Z | Mid | N/A | Highest NRB | R | Mid | N/A | Highest NRB | 5 DL CA |  |  |  |
|  | QPSK | QPSK |  | Highest NRB | N/A | QPSK |  | ALL RBs | N/A | QPSK |  | ALL RBs | N/A | QPSK |  | ALL RBs |  |  |  |  |
| 2 | Y | Mid | REFSENS | Highest NRB | X | Mid | N/A | Highest NRB | Z | Mid | N/A | Highest NRB | R | Mid | N/A | Highest NRB |  |  |  |  |
|  | QPSK | QPSK |  | ALL RBs | N/A | QPSK |  | ALL RBs | N/A | QPSK |  | ALL RBs | N/A | QPSK |  | ALL RBs |  |  |  |  |
| 3 | Z | Mid | REFSENS | Highest NRB | X | Mid | N/A | Highest NRB | Y | Mid | N/A | Highest NRB | R | Mid | N/A | Highest NRB |  |  |  |  |
|  | QPSK | QPSK |  | ALL RBs | N/A | QPSK |  | ALL RBs | N/A | QPSK |  | ALL RBs | N/A | QPSK |  | ALL RBs |  |  |  |  |
| 4 | R | Mid | REFSENS | Highest NRB | X | Mid | N/A | Highest NRB | Y | Mid | N/A | Highest NRB | Z | Mid | N/A | Highest NRB |  |  |  |  |
|  | QPSK | QPSK |  | ALL RBs | N/A | QPSK |  | ALL RBs | N/A | QPSK |  | ALL RBs | N/A | QPSK |  | ALL RBs |  |  |  |  |
| Default Test Settings for a CA_XA-YD Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Mid | REFSENS | Highest NRB | Y | Mid/CC1 | N/A | Highest NRB_agg | Y | Mid/CC2 | N/A | Highest NRB_agg | Y | Mid/CC3 | N/A | Highest NRB_agg |  |  |  |  |
|  | QPSK | QPSK |  | ALL RBs | N/A | QPSK |  | ALL RBs | N/A | QPSK |  | ALL RBs | N/A | QPSK |  | ALL RBs | 5 DL CA |  |  |  |
| 2 | Y | Low/CC1 | REFSENS | Highest NRB_agg | Y | Low/CC2 | N/A | Highest NRB_agg | Y | Low/CC3 | N/A | Highest NRB_agg | X | Mid | N/A | Highest NRB |  |  |  |  |
|  | QPSK | QPSK |  | ALL RBs | N/A | QPSK |  | ALL RBs | N/A | QPSK |  | ALL RBs | N/A | QPSK |  | ALL RBs |  |  |  |  |
| 3 | Y | High/CC1 | REFSENS | Lowest NRB_agg | Y | High/CC2 | N/A | Lowest NRB_agg | Y | High/CC3 | N/A | Lowest NRB_agg | X | Mid | N/A | Highest NRB |  |  |  |  |
|  | QPSK | QPSK |  | ALL RBs | N/A | QPSK |  | ALL RBs | N/A | QPSK |  | ALL RBs | N/A | QPSK |  | ALL RBs |  |  |  |  |
| 4 | Y | Low/CC1 | REFSENS | Highest NRB_agg | Y | Low/CC2 | N/A | Highest NRB_agg | Y | Low/CC3 | N/A | Highest NRB_agg | X | Mid | N/A | Highest NRB |  |  |  |  |
|  | QPSK | QPSK |  | ALL RBs | N/A | QPSK |  | ALL RBs | N/A | QPSK |  | ALL RBs | N/A | QPSK |  | ALL RBs |  |  |  |  |
| 5 | Y | High/CC1 | REFSENS | Lowest NRB_agg | Y | High/CC2 | N/A | Lowest NRB_agg | Y | High/CC3 | N/A | Lowest NRB_agg | X | Mid | N/A | Highest NRB |  |  |  |  |
|  | QPSK | QPSK |  | ALL RBs | N/A | QPSK |  | ALL RBs | N/A | QPSK |  | ALL RBs | N/A | QPSK |  | ALL RBs |  |  |  |  |
| Default Test Settings for a CA_XB-YA-ZA and CA_XC-YA-ZA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Low/CC1 | REFSENS | Highest NRB_agg | X | Low/CC2 | N/A | Highest NRB_agg | Y | Mid | N/A | Highest NRB | Z | Mid | N/A | Highest NRB |  |  |  |  |
|  | QPSK | QPSK |  | ALL RBs | N/A | QPSK |  | ALL RBs | N/A | QPSK |  | ALL RBs | N/A | QPSK |  | ALL RBs |  |  |  |  |
| 2 | X | Low/CC1 | REFSENS | Lowest NRB_agg | X | Low/CC2 | N/A | Lowest NRB_agg | Y | Mid | N/A | Highest NRB | Z | Mid | N/A | Highest NRB |  |  |  |  |
|  | QPSK | QPSK |  | ALL RBs | N/A | QPSK |  | ALL RBs | N/A | QPSK |  | ALL RBs | N/A | QPSK |  | ALL RBs |  |  |  |  |
| 3 | X | High/CC1 | REFSENS | Highest NRB_agg | X | High/CC2 | N/A | Highest NRB_agg | Y | Mid | N/A | Highest NRB | Z | Mid | N/A | Highest NRB |  |  |  |  |
|  | QPSK | QPSK |  | ALL RBs | N/A | QPSK |  | ALL RBs | N/A | QPSK |  | ALL RBs | N/A | QPSK |  | ALL RBs |  |  |  |  |
| 4 | X | High/CC1 | REFSENS | Lowest NRB_agg | X | High/CC2 | N/A | Lowest NRB_agg | Y | Mid | N/A | Highest NRB | Z | Mid | N/A | Highest NRB |  |  |  |  |
|  | QPSK | QPSK |  | ALL RBs | N/A | QPSK |  | ALL RBs | N/A | QPSK |  | ALL RBs | N/A | QPSK |  | ALL RBs |  |  |  |  |
| 5 | Y | Mid | REFSENS | Highest NRB | X | Mid/CC1 | N/A | Highest NRB_agg | X | Mid/CC2 | N/A | Highest NRB_agg | Z | Mid | N/A | Highest NRB |  |  |  |  |
|  | QPSK | QPSK |  | ALL RBs | N/A | QPSK |  | ALL RBs | N/A | QPSK |  | ALL RBs | N/A | QPSK |  | ALL RBs |  |  |  |  |
| 6 | Z | Mid | REFSENS | Highest NRB | X | Mid/CC1 | N/A | Highest NRB_agg | X | Mid/CC2 | N/A | Highest NRB_agg | Y | Mid | N/A | Highest NRB |  |  |  |  |
|  | QPSK | QPSK |  | ALL RBs | N/A | QPSK |  | ALL RBs | N/A | QPSK |  | ALL RBs | N/A | QPSK |  | ALL RBs |  |  |  |  |
| Default Test Settings for a CA_XC-YC Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Low/CC1 | REFSENS | Highest NRB_agg | X | Low/CC2 | N/A | Highest NRB_agg | Y | Low/CC1 | N/A | Highest NRB_agg | Y | Low/CC2 | N/A | Highest NRB_agg |  |  |  |  |
|  | QPSK | QPSK |  | ALL RBs | N/A | QPSK |  | ALL RBs | N/A | QPSK |  | ALL RBs | N/A | QPSK |  | ALL RBs |  |  |  |  |
| 2 | X | Low/CC1 | REFSENS | Lowest NRB_agg | X | Low/CC2 | N/A | Lowest NRB_agg | Y | Low/CC1 | N/A | Lowest NRB_agg | Y | Low/CC2 | N/A | Lowest NRB_agg |  |  |  |  |
|  | QPSK | QPSK |  | ALL RBs | N/A | QPSK |  | ALL RBs | N/A | QPSK |  | ALL RBs | N/A | QPSK |  | ALL RBs |  |  |  |  |
| 3 | Y | High/CC1 | REFSENS | Highest NRB_agg | Y | High/CC2 | N/A | Highest NRB_agg | X | High/CC1 | N/A | Highest NRB_agg | X | High/CC2 | N/A | Highest NRB_agg |  |  |  |  |
|  | QPSK | QPSK |  | ALL RBs | N/A | QPSK |  | ALL RBs | N/A | QPSK |  | ALL RBs | N/A | QPSK |  | ALL RBs |  |  |  |  |
| 4 | Y | High/CC1 | REFSENS | Lowest NRB_agg | Y | High/CC2 | N/A | Lowest NRB_agg | X | High/CC1 | N/A | Lowest NRB_agg | X | High/CC2 | N/A | Lowest NRB_agg |  |  |  |  |
|  | QPSK | QPSK |  | ALL RBs | N/A | QPSK |  | ALL RBs | N/A | QPSK |  | ALL RBs | N/A | QPSK |  | ALL RBs |  |  |  |  |
| Note: TBD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 2-6: CA types list for 4DL CA

| CA types | Classes |
| --- | --- |
| All intra C | CA_XE |
| inter + intra C(3CCs) | CA_XA-YD/CA_XD-YA |
| inter + intra C(2CCs) | CA_XA-YA-ZC/CA_XA-YC-ZA/CA_XC-YA-ZA CA_XA-YA-ZB |
| All inter | CA_WA-XA-YA-ZA |
| intra C(2CCs) + intra C(2CCs) | CA_XC-YC |
| intra NC + intra NC | CA_XA-XA-YA-YA |
| intra NC + Intra C(2CCs) | CA_XA-XA-YB |
| intra NC+inter | CA_XA-XA-YA-ZA/CA_XA-YA-YA-ZA |

3	Proposal

In this document it is proposed that for 4DL/1UL CA receiver test cases

Proposal: Table 2-3 & 2-5 is new configuration table format which has considered the DL CA with UL CA and different modulation in UL/DL; also that the inter-band relative & intra-band contiguous CA can be put in together in this new configuration table.

4	References

[1]	RP-151764	New Work Item proposal: UE Conformance Test Aspects - Rel-13 LTE CA configurations

[2] R5-160353	Discussion for test configuration table format of receiver test cases for 4DL/1UL CA

[3] Comment to R5-165651 (test conf format for 4DL by SGS Wireless) by the e-mail from Jussi Kuusisto Dish