3GPP TSG-RAN WG5 Meeting #74 	R5-171582

Athens, Greece, 13th-17th Febuary 2017

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.508 | CR | 0898 | rev | 1 | Current version: | 14.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to test frequencies for Band 46 and Test frequencies for CA signalling test |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_LAA-UEConTest |  |  |  |  |  |  | Date: |  | 2017-02-02 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Test Frequencies 5535MHz and 5915MHz are not one of the allowed test frequencies as per the TS 36.104 section 5.7.3-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrected test frequencies for operating band 46 from 5535 MHz and 5915MHz to 5540 MHz and 5865MHz. Test frequencies for band 46 CA signalling test updated accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case would use invalid test frequencies. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Cl 4.3.1.2.14, 6.2.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | TTCN Impact R5-171733r1 Updated EARFCN and High Freq in Table Cl 4.3.1.2.14 R5-171374 is replacement doc for R5-170956 due to coverpage rel mismatch R5- 171582 is Rev-1 R5-171374 due to cover page issue |  |  |  |  |  |  |  |  |

<Start of first modified section>

##### 4.3.1.2.14	TDD reference test frequencies for Operating Band 46

Table 4.3.1.2.14-1: Test frequencies for E-UTRA channel bandwidth for operating band 46

| Test Frequency ID | Bandwidth [MHz] | EARFCN | Frequency (UL and DL) [MHz] |
| --- | --- | --- | --- |
| Low Range | 20 | 46890 | 5160 |
| Mid Range | 20 | 50690 | 5540 |
| High Range | 20 | 54340 | 5905 |
| NOTE 1: In Rel-13, restricted to E-UTRA operation when carrier aggregation is configured. The downlink operating band is paired with the uplink operating band (external) of the carrier aggregation configuration that is supporting the configured Pcell. |  |  |  |

<End of First modified section>

#### 6.2.3.2	Test frequencies for CA signalling test

Test frequencies for CA signalling testing are specified in Table 6.2.3.2-1 for CA Intra-Band contiguous case; in Tables 6.2.3.2-2 for CA Inter-band case and in Table 6.2.3.2-4 for Intra-Band Non-Contiguous case.

<Text Skipped>

<Start of second modified section>

Table 6.2.3.2-2: Test frequencies for E-UTRA PCell and SCell for CA Inter-band operation and E-UTRA PCell and PSCell for DC Inter-band operation (two bands)

| E-UTRA CA Configuration | Width of Operating bands [MHz+MHz] | CC Combination / NRB_agg |  |  | Test Frequency | CC NRB | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1A-3A, DC_1A-3A | 60+75 | 50 + 50 |  |  | f1 | CC 50 | 18300 | 1950 | 300 | 2140 |  |
|  |  |  |  |  | f2 | CC 50 | 18550 | 1975 | 550 | 2165 |  |
|  |  |  |  |  | f5 | CC 50 | 19575 | 1747.5 | 1575 | 1842.5 |  |
|  |  |  |  |  | f6 | CC 50 | 19900 | 1780 | 1900 | 1875 |  |
| CA_1A-5A, DC_1A-5A | 60+25 | 50 + 50 |  |  | f1 | CC 50 | 18300 | 1950 | 300 | 2140 |  |
|  |  |  |  |  | f2 | CC 50 | [18550] | [1975] | [550] | [2165] |  |
|  |  |  |  |  | f5 | CC 50 | 20450 | 829 | 2450 | 874 |  |
|  |  |  |  |  | f6 | CC 50 | 20600 | 844 | 2600 | 889 |  |
| CA_1A-7A, DC_1A-7A | 60 + 70 | 50 + 50 |  |  | f1 | CC 50 | 18300 | 1950 | 300 | 2140 |  |
|  |  |  |  |  | f2 | CC 50 | 18550 | 1975 | 550 | 2165 |  |
|  |  |  |  |  | f5 | CC 50 | 21100 | 2535 | 3100 | 2655 |  |
|  |  |  |  |  | f6 | CC 50 | 21400 | 2565 | 3400 | 2685 |  |
| CA_1A-8A, DC_1A-8A | 60 + 35 | 50 + 50 |  |  | f1 | CC 50 | 18300 | 1950 | 300 | 2140 |  |
|  |  |  |  |  | f2 | CC 50 | 18550 | 1975 | 550 | 2165 |  |
|  |  |  |  |  | f5 | CC 50 | 21625 | 897.5 | 3625 | 942.5 |  |
|  |  |  |  |  | f6 | CC 50 | 21750 | 910 | 3750 | 955 |  |
| CA_1A-11A | 60 + 20 | 100 + 50 |  |  | f1 | CC 100 | 18300 | 1950 | 300 | 2140 |  |
|  |  |  |  |  | f2 | CC 100 | 18500 | 1970 | 500 | 2160 |  |
|  |  |  |  |  | f5 | CC 50 | 22800 | 1432.9 | 4800 | 1480.9 |  |
|  |  |  |  |  | f6 | CC 50 | 22900 | 1442.9 | 4900 | 1490.9 |  |
| CA_1A-18A | 60 + 15 | 50 + 50 |  |  | f1 | CC 50 | 18300 | 1950 | 300 | 2140 |  |
|  |  |  |  |  | f2 | CC 50 | 18550 | 1975 | 550 | 2165 |  |
|  |  |  |  |  | f5 | CC 50 | 23900 | 820 | 5900 | 865 |  |
|  |  |  |  |  | f6 | N/A | N/A | N/A | N/A | N/A |  |
| CA_1A-19A, DC_1A-19A | 60+15 | 75 + 50 |  |  | f1 | CC 75 | 18324 | 1952.4 | 324 | 2142.4 |  |
|  |  |  |  |  | f2 | CC 75 | 18525 | 1972.5 | 525 | 2162.5 |  |
|  |  |  |  |  | f5 | CC 50 | 24100 | 840 | 6100 | 885 |  |
|  |  |  |  |  | f6 | N/A | N/A | N/A | N/A | N/A |  |
| CA_1A-20A | 60 + 30 | 50 + 50 |  |  | f1 | CC 50 | 18300 | 1950 | 300 | 2140 |  |
|  |  |  |  |  | f2 | CC 50 | 18550 | 1975 | 550 | 2165 |  |
|  |  |  |  |  | f5 | CC 50 | 24200 | 837 | 6200 | 796 |  |
|  |  |  |  |  | f6 | CC 50 | 24400 | 857 | 6400 | 816 |  |
| CA_1A-21A, DC_1A-21A | 60+15 | 75 + 75 |  |  | f1 | CC 75 | 18324 | 1952.4 | 324 | 2142.4 |  |
|  |  |  |  |  | f2 | CC 75 | 18525 | 1972.5 | 525 | 2162.5 |  |
|  |  |  |  |  | f5 | CC 75 | 24525 | 1455.4 | 6525 | 1503.4 |  |
|  |  |  |  |  | f6 | N/A | N/A | N/A | N/A | N/A |  |
| CA_1A-26A | 60 + 35 | 50 + 50 |  |  | f1 | CC 50 | 18300 | 1950 | 300 | 2140 |  |
|  |  |  |  |  | f2 | CC 50 | 18550 | 1975 | 550 | 2165 |  |
|  |  |  |  |  | f5 | CC 50 | 26865 | 831.5 | 8865 | 876.5 |  |
|  |  |  |  |  | f6 | CC 50 | 26990 | 844 | 8990 | 889 |  |
| CA_1A-28A | 60+45 | 50 + 50 |  |  | f1 | CC 50 | 18300 | 1950 | 300 | 2140 |  |  |
|  |  |  |  |  | f2 | CC 50 | 18550 | 1975 | 550 | 2165 |  |  |
|  |  |  |  |  | f5 | CC 50 | 27410 | 723 | 9410 | 778 |  |  |
|  |  |  |  |  | f6 | CC 50 | 27610 | 743 | 9610 | 798 |  |  |
| CA_1A-40A | 60 + 100 | 50 + 100 |  |  | f1 | CC 50 | 18300 | 1950 | 300 | 2140 |  |
|  |  |  |  |  | f2 | CC 50 | 18550 | 1975 | 550 | 2165 |  |
|  |  |  |  |  | f5 | CC 100 | 39150 | 2350 | 39150 | 2350 |  |
|  |  |  |  |  | f6 | CC 100 | 39550 | 2390 | 39550 | 2390 |  |
| CA_1A-41A | 60 + 194 |  | 50 + 100 |  | f1 | CC 50 | 18300 | 1950 | 300 | 2140 |  |  |
|  |  |  |  |  | f2 | CC 50 | 18550 | 1975 | 550 | 2165 |  |  |
|  |  |  |  |  | f5 | CC 100 | 40620 | 2593 | 40620 | 2593 |  |  |
|  |  |  |  |  | f6 | CC 100 | 41490 | 2680 | 41490 | 2680 |  |  |
| CA_1A-41C | 60+194 |  |  | 50+100+100 | f1 | CC 50 | 18300 | 1950 | 300 | 2140 |
|  |  |  |  |  | f2 | CC 50 | 18550 | 1975 | 550 | 2165 |
|  |  |  |  |  | f5 | CC 100 | 40521 | 2583.1 | 40521 | 2583.1 |
|  |  |  |  |  | f6 | CC 100 | 40719 | 2602.9 | 40719 | 2602.9 |
| CA_1A-42A | 60 + 200 |  | 75 + 100 |  | f1 | CC 75 | 18324 | 1952.4 | 324 | 2142.4 |  |  |
|  |  |  |  |  | f2 | CC 75 | 18525 | 1972.5 | 525 | 2162.5 |  |  |
|  |  |  |  |  | f5 | CC 100 | 42590 | 3500 | 42590 | 3500 |  |  |
|  |  |  |  |  | f6 | CC 100 | 43490 | 3590 | 43490 | 3590 |  |  |
| CA_1A-42C | 60+200 |  |  | 50+100+100 | f1 | CC 50 | 18300 | 1950 | 300 | 2140 |
|  |  |  |  |  | f2 | CC 50 | 18550 | 1975 | 550 | 2165 |
|  |  |  |  |  | f5 | CC 100 | 42491 | 3490.1 | 42491 | 3490.1 |
|  |  |  |  |  | f6 | CC 100 | 42689 | 3509.9 | 42689 | 3509.9 |
| CA_1A-46A | 60+775 |  | 50+100 |  | f1 | CC 50 | 18300 | 1950 | 300 | 2140 |  |
|  |  |  |  |  | f2 | CC 50 | 18550 | 1975 | 550 | 2165 |  |
|  |  |  |  |  | f5 | CC 100 | 50690 | 5540 | 50690 | 5540 |  |
|  |  |  |  |  | f6 | CC 100 | 53940 | 5865 | 53940 | 5865 |  |
| CA_2A-4A, DC_2A-4A | 60 + 45 |  | 50 + 50 |  | f1 | CC 50 | 18900 | 1880 | 900 | 1960 |  |
|  |  |  |  |  | f2 | CC 50 | 19150 | 1905 | 1150 | 1985 |  |
|  |  |  |  |  | f5 | CC 50 | 20175 | 1732.5 | 2175 | 2132.5 |  |
|  |  |  |  |  | f6 | CC 50 | 20350 | 1750 | 2350 | 2150 |  |
| CA_2A-5A | 60+25 |  | 50 + 50 |  | f1 | CC 50 | 18900 | 1880 | 900 | 1960 |  |
|  |  |  |  |  | f2 | CC 50 | 19150 | 1905 | 1150 | 1985 |  |
|  |  |  |  |  | f5 | CC 50 | 20450 | 829 | 2450 | 874 |  |
|  |  |  |  |  | f6 | CC 50 | 20600 | 844 | 2600 | 889 |  |
| CA_2A-12A | 60 + 17 |  | 50 + 50 |  | f1 | CC 50 | 18900 | 1880 | 900 | 1960 |  |
|  |  |  |  |  | f2 | CC 50 | 19150 | 1905 | 1150 | 1985 |  |
|  |  |  |  |  | f5 | CC 50 | 23060 | 704 | 5060 | 734 |  |
|  |  |  |  |  | f6 | N/A | N/A | N/A | N/A | N/A |  |
| CA_2A-13A, DC_2A-13A | 60 + 10 |  | 50 + 50 |  | f1 | CC 50 | 18900 | 1880 | 900 | 1960 |  |
|  |  |  |  |  | f2 | CC 50 | 19150 | 1905 | 1150 | 1985 |  |
|  |  |  |  |  | f5 | CC 50 | 23230 | 782 | 5230 | 751 |  |
|  |  |  |  |  | f6 | N/A | N/A | N/A | N/A | N/A |  |
| CA_2A-17A | 60+12 |  | 50 + 50 |  | f1 | CC 50 | 18900 | 1880 | 900 | 1960 |  |
|  |  |  |  |  | f2 | CC 50 | 19150 | 1905 | 1150 | 1985 |  |
|  |  |  |  |  | f5 | CC 50 | 23790 | 710 | 5790 | 740 |  |
|  |  |  |  |  | f6 | N/A | N/A | N/A | N/A | N/A |  |
| CA_2A-28A | 60+45 |  | 50 + 50 |  | f1 | CC 50 | 18900 | 1880 | 900 | 1960 |  |
|  |  |  |  |  | f2 | CC 50 | 19150 | 1905 | 1150 | 1985 |  |
|  |  |  |  |  | f5 | CC 50 | 27410 | 723 | 9410 | 778 |  |
|  |  |  |  |  | f6 | CC 50 | 27610 | 743 | 9610 | 798 |  |
| CA_2A-29A | 60+11 |  | 50 + 50 |  | f1 | CC 50 | 18900 | 1880 | 900 | 1960 |  |
|  |  |  |  |  | f2 | CC 50 | 19150 | 1905 | 1150 | 1985 |  |
|  |  |  |  |  | f5 | CC 50 | N/A | N/A | 9720 | 723 |  |
|  |  |  |  |  | f6 | N/A | N/A | N/A | N/A | N/A |  |
| CA_2A-30A | 60+10 | 50 + 50 |  |  | f1 | CC 50 | 18900 | 1880 | 900 | 1960 |  |
|  |  |  |  |  | f2 | CC 50 | 19150 | 1905 | 1150 | 1985 |  |
|  |  |  |  |  | f5 | CC 50 | 27710 | 2310 | 9820 | 2355 |  |
|  |  |  |  |  | f6 | N/A | N/A | N/A | N/A | N/A |  |
| CA_2A-46A | 60+775 | 50+100 |  |  | f1 | CC 50 | 18900 | 1880 | 900 | 1960 |  |
|  |  |  |  |  | f2 | CC 50 | 19150 | 1905 | 1150 | 1985 |  |
|  |  |  |  |  | f5 | CC 100 | 50690 | 5540 | 50690 | 5540 |  |
|  |  |  |  |  | f6 | CC 100 | 53940 | 5865 | 53940 | 5865 |  |
| CA_3A-5A, DC_3A-5A | 75+25 | 50 + 50 |  |  | f1 | CC 50 | 19575 | 1747.5 | 1575 | 1842.5 |  |
|  |  |  |  |  | f2 | CC 50 | [19900] | [1780] | [1900] | [1875] |  |
|  |  |  |  |  | f5 | CC 50 | 20450 | 829 | 2450 | 874 |  |
|  |  |  |  |  | f6 | CC 50 | 20600 | 844 | 2600 | 889 |  |
| CA_3C-5A | 75+25 | 100+100+50 |  |  | f1 | CC 100 | 19475 | 1737.5 | 1475 | 1832.5 |  |
|  |  |  |  |  | f2 | CC 100 | 19673 | 1757.3 | 1673 | 1852.3 |  |
|  |  |  |  |  | f5 | CC 50 | 20450 | 829 | 2450 | 874 |  |
|  |  |  |  |  | f6 | CC 50 | 20600 | 844 | 2600 | 889 |  |
| CA_3A-7A, DC_3A-7A | 75+70 | 50 + 50 |  |  | f1 | CC 50 | 19575 | 1747.5 | 1575 | 1842.5 |  |
|  |  |  |  |  | f2 | CC 50 | [19900] | [1780] | [1900] | [1875] |  |
|  |  |  |  |  | f5 | CC 50 | 21100 | 2535 | 3100 | 2655 |  |
|  |  |  |  |  | f6 | CC 50 | 21400 | 2565 | 3400 | 2685 |  |
| CA_3A-8A, DC_3A-8A | 75+35 | 50+50 |  |  | f1 | CC 50 | 19575 | 1747.5 | 1575 | 1842.5 |  |
|  |  |  |  |  | f2 | CC 50 | 19900 | 1780 | 1900 | 1875 |  |
|  |  |  |  |  | f5 | CC 50 | 21625 | 897.5 | 3625 | 942.5 |  |
|  |  |  |  |  | f6 | CC 50 | 21750 | 910 | 3750 | 955 |  |
| CA_3A-19A, DC_3A-19A | 75+15 | 100+50 |  |  | f1 | CC 100 | 19575 | 1747.5 | 1575 | 1842.5 |  |
|  |  |  |  |  | f2 | CC 100 | 19850 | 1775 | 1850 | 1870 |  |
|  |  |  |  |  | f5 | CC 50 | 24100 | 840 | 6100 | 885 |  |
|  |  |  |  |  | f6 | N/A | N/A | N/A | N/A | N/A |  |
| CA_3A-20A, DC_3A-20A | 75+30 |  | 50+50 |  | f1 | CC 50 | 19575 | 1747.5 | 1575 | 1842.5 |  |
|  |  |  |  |  | f2 | CC 50 | 19900 | 1780 | 1900 | 1875 |  |
|  |  |  |  |  | f5 | CC 50 | 24200 | 837 | 6200 | 796 |  |
|  |  |  |  |  | f6 | CC 50 | 24400 | 857 | 6400 | 816 |  |
| CA_3A-26A, DC_3A-26A | 75 + 35 | 50 + 50 |  |  | f1 | CC 50 | 19575 | 1747.5 | 1575 | 1842.5 |  |
|  |  |  |  |  | f2 | CC 50 | 19900 | 1780 | 1900 | 1875 |  |
|  |  |  |  |  | f5 | CC 50 | 26865 | 831.5 | 8865 | 876.5 |  |
|  |  |  |  |  | f6 | CC 50 | 26990 | 844 | 8990 | 889 |  |
| CA_3A-27A | 75 + 17 | 50 + 50 |  |  | f1 | CC 50 | 19575 | 1747.5 | 1575 | 1842.5 |  |
|  |  |  |  |  | f2 | CC 50 | 19900 | 1780 | 1900 | 1875 |  |
|  |  |  |  |  | f5 | CC 50 | 27125 | 815.5 | 9125 | 860.5 |  |
|  |  |  |  |  | f6 | CC 50 | 27160 | 819 | 9160 | 864 |  |
| CA_3A-28A | 75+45 | 50+50 |  |  | f1 | CC 50 | 19575 | 1747.5 | 1575 | 1842.5 |  |
|  |  |  |  |  | f2 | CC 50 | 19900 | 1780 | 1900 | 1875 |  |
|  |  |  |  |  | f5 | CC 50 | 27435 | 725.5 | 9435 | 780.5 |  |
|  |  |  |  |  | f6 | CC 50 | 27610 | 743 | 9610 | 798 |  |
| CA_3A-40A | 75 + 100 | 50 + 100 |  |  | f1 | CC 50 | 19575 | 1747.5 | 1575 | 1842.5 |  |
|  |  |  |  |  | f2 | CC 50 | 19900 | 1780 | 1900 | 1875 |  |
|  |  |  |  |  | f5 | CC 100 | 39150 | 2350 | 39150 | 2350 |  |
|  |  |  |  |  | f6 | CC 100 | 39550 | 2390 | 39550 | 2390 |  |
| CA_3A-41A | 75 + 194 | 100 + 100 |  |  | f1 | CC 100 | 19575 | 1747.5 | 1575 | 1842.5 |  |
|  |  |  |  |  | f2 | CC 100 | 19850 | 1775 | 1850 | 1870 |  |
|  |  |  |  |  | f5 | CC 100 | 40620 | 2593 | 40620 | 2593 |  |
|  |  |  |  |  | f6 | CC 100 | 41490 | 2680 | 41490 | 2680 |  |
| CA_3A-42A | 75 + 200 | 100 + 100 |  |  | f1 | CC 100 | 19575 | 1747.5 | 1575 | 1842.5 |  |
|  |  |  |  |  | f2 | CC 100 | 19850 | 1775 | 1850 | 1870 |  |
|  |  |  |  |  | f5 | CC 100 | 42590 | 3500 | 42590 | 3500 |  |
|  |  |  |  |  | f6 | CC 100 | 43490 | 3590 | 43490 | 3590 |  |
| CA_3A-42C | 75+200 |  |  | 50+100+100 | f1 | CC 50 | 19575 | 1747.5 | 1575 | 1842.5 |
|  |  |  |  |  | f2 | CC 50 | 19900 | 1780 | 1900 | 1875 |
|  |  |  |  |  | f5 | CC 100 | 42491 | 3490.1 | 42491 | 3490.1 |
|  |  |  |  |  | f6 | CC 100 | 42689 | 3509.9 | 42689 | 3509.9 |
| CA_3A-46A | 75+775 |  |  | 50+100 | f1 | CC 50 | 19575 | 1747.5 | 1575 | 1842.5 |
|  |  |  |  |  | f2 | CC 50 | 19900 | 1780 | 1900 | 1875 |
|  |  |  |  |  | f5 | CC 100 | 50690 | 5540 | 50690 | 5540 |
|  |  |  |  |  | f6 | CC 100 | 53940 | 5865 | 53940 | 5865 |
| CA_4A-5A | 45+25 | 50 + 50 |  |  | f1 | CC 50 | 20175 | 1732.5 | 2175 | 2132.5 |  |
|  |  |  |  |  | f2 | CC 50 | 20350 | 1750 | 2350 | 2150 |  |
|  |  |  |  |  | f5 | CC 50 | 20450 | 829 | 2450 | 874 |  |
|  |  |  |  |  | f6 | CC 50 | 20600 | 844 | 2600 | 889 |  |
| CA_4A-7A, DC_4A-7A | 45+70 |  | 50+50 |  | f1 | CC 50 | 20175 | 1732.5 | 2175 | 2132.5 |  |
|  |  |  |  |  | f2 | CC 50 | 20350 | 1750 | 2350 | 2150 |  |
|  |  |  |  |  | f5 | CC 50 | 21100 | 2535 | 3100 | 2655 |  |
|  |  |  |  |  | f6 | CC 50 | 21400 | 2565 | 3400 | 2685 |  |
| CA_4A-12A, DC_4A-12A | 45+17 | 50+50 |  |  | f1 | CC 50 | 20175 | 1732.5 | 2175 | 2132.5 |  |
|  |  |  |  |  | f2 | CC 50 | 20350 | 1750 | 2350 | 2150 |  |
|  |  |  |  |  | f5 | CC 50 | 23130 | 711 | 5130 | 741 |  |
|  |  |  |  |  | f6 | N/A | N/A | N/A | N/A | N/A |  |
| CA_4A-13A, DC_4A-13A | 45+10 | 50 + 50 |  |  | f1 | CC 50 | 20175 | 1732.5 | 2175 | 2132.5 |  |
|  |  |  |  |  | f2 | CC 50 | 20350 | 1750 | 2350 | 2150 |  |
|  |  |  |  |  | f5 | CC 50 | 23230 | 782 | 5230 | 751 |  |
|  |  |  |  |  | f6 | N/A | N/A | N/A | N/A | N/A |  |
| CA_4A-17A, DC_4A-17A | 45+12 |  | 50 + 50 |  | f1 | CC 50 | 20175 | 1732.5 | 2175 | 2132.5 |  |
|  |  |  |  |  | f2 | CC 50 | 20350 | 1750 | 2350 | 2150 |  |
|  |  |  |  |  | f5 | CC 50 | 23790 | 710 | 5790 | 740 |  |
|  |  |  |  |  | f6 | N/A | N/A | N/A | N/A | N/A |  |
| CA_4A-28A | 45+45 |  | 50 + 50 |  | f1 | CC 50 | 20175 | 1732.5 | 2175 | 2132.5 |  |
|  |  |  |  |  | f2 | CC 50 | 20350 | 1750 | 2350 | 2150 |  |
|  |  |  |  |  | f5 | CC 50 | 27410 | 723 | 9410 | 778 |  |
|  |  |  |  |  | f6 | CC 50 | 27610 | 743 | 9610 | 798 |  |
| CA_4A-29A | 45+11 |  | 50 + 50 |  | f1 | CC 50 | 20175 | 1732.5 | 2175 | 2132.5 |  |
|  |  |  |  |  | f2 | CC 50 | 20350 | 1750 | 2350 | 2150 |  |
|  |  |  |  |  | f5 | CC 50 | N/A | N/A | 9720 | 723 |  |
|  |  |  |  |  | f6 | N/A | N/A | N/A | N/A | N/A |  |
| CA_4A-30A | 45+10 | 50 + 50 |  |  | f1 | CC 50 | 20175 | 1732.5 | 2175 | 2132.5 |  |  |
|  |  |  |  |  | f2 | CC 50 | 20350 | 1750 | 2350 | 2150 |  |  |
|  |  |  |  |  | f5 | CC 50 | 27710 | 2310 | 9820 | 2355 |  |  |
|  |  |  |  |  | f6 | N/A | N/A | N/A | N/A | N/A |  |  |
| CA_4A-46A | 54+775 | 50+100 |  |  | f1 | CC 50 | 20175 | 1732.5 | 2175 | 2132.5 |  |  |
|  |  |  |  |  | f2 | CC 50 | 20350 | 1750 | 2350 | 2150 |  |  |
|  |  |  |  |  | f5 | CC 100 | 50690 | 5540 | 50690 | 5540 |  |  |
|  |  |  |  |  | f6 | CC 100 | 53940 | 5865 | 53940 | 5865 |  |  |
| CA_5A-7A, DC_5A-7A | 25+70 |  | 50 + 50 |  | f1 | CC 50 | 20450 | 829 | 2450 | 874 |  |
|  |  |  |  |  | f2 | CC 50 | 20600 | 844 | 2600 | 889 |  |
|  |  |  |  |  | f5 | CC 50 | 21100 | 2535 | 3100 | 2655 |  |
|  |  |  |  |  | f6 | CC 50 | 21400 | 2565 | 3400 | 2685 |  |
| CA_5A-12A, DC_5A-12A | 25+17 |  | 50+50 |  | f1 | CC 50 | 20525 | 836.5 | 2525 | 881.5 |  |
|  |  |  |  |  | f2 | CC 50 | 20600 | 844 | 2600 | 889 |  |
|  |  |  |  |  | f5 | CC 50 | 23130 | 711 | 5130 | 741 |  |
|  |  |  |  |  | f6 | N/A | N/A | N/A | N/A | N/A |  |
| CA_5A-13A | 25+10 |  | 50 + 50 |  | f1 | CC 50 | 20450 | 829 | 2450 | 874 |  |
|  |  |  |  |  | f2 | CC 50 | 20600 | 844 | 2600 | 889 |  |
|  |  |  |  |  | f5 | CC 50 | 23230 | 782 | 5230 | 751 |  |
|  |  |  |  |  | f6 | N/A | N/A | N/A | N/A | N/A |  |
| CA_5A-17A | 25+12 | 50 + 50 |  |  | f1 | CC 50 | 20525 | 836.5 | 2525 | 881.5 |  |
|  |  |  |  |  | f2 | CC 50 | 20600 | 844 | 2600 | 889 |  |
|  |  |  |  |  | f5 | CC 50 | 23790 | 710 | 5790 | 740 |  |
|  |  |  |  |  | f6 | N/A | N/A | N/A | N/A | N/A |  |
| CA_5A-25A | 25+65 |  | 50 + 50 |  | f1 | CC 50 | 20525 | 836.5 | 2525 | 881.5 |  |
|  |  |  |  |  | f2 | CC 50 | 20600 | 844 | 2600 | 889 |  |
|  |  |  |  |  | f5 | CC 50 | 26363 | 1882.5 | 8365 | 1962.5 |  |
|  |  |  |  |  | f6 | CC 50 | 26640 | 1910 | 8640 | 1990 |  |
| CA_5A-29A | 25 + 11 |  | 50 + 50 |  | f1 | CC 50 | 20525 | 836.5 | 2525 | 881.5 |  |
|  |  |  |  |  | f2 | CC 50 | 20600 | 844 | 2600 | 889 |  |
|  |  |  |  |  | f5 | CC 50 | N/A | N/A | 9720 | 723 |  |
|  |  |  |  |  | f6 | N/A | N/A | N/A | N/A | N/A |  |
| CA_5A-30A | 25+10 |  | 50 + 50 |  | f1 | CC 50 | 20525 | 836.5 | 2525 | 881.5 |  |
|  |  |  |  |  | f2 | CC 50 | 20600 | 844 | 2600 | 889 |  |
|  |  |  |  |  | f5 | CC 50 | 27710 | 2310 | 9820 | 2355 |  |
|  |  |  |  |  | f6 | N/A | N/A | N/A | N/A | N/A |  |
| CA_7A-8A | 70+35 |  |  | 50+50 | f1 | CC 50 | 21100 | 2535 | 3100 | 2655 |
|  |  |  |  |  | f2 | CC 50 | [21400] | [2565] | [3400] | [2685] |
|  |  |  |  |  | f5 | CC 50 | 21625 | 897.5 | 3625 | 942.5 |
|  |  |  |  |  | f6 | CC 50 | 21750 | 910 | 3750 | 955 |
| CA_7A-12A | 70+17 |  |  | 50+50 | f1 | CC 50 | 21100 | 2535 | 3100 | 2655 |
|  |  |  |  |  | f2 | CC 50 | [21400] | [2565] | [3400] | [2685] |
|  |  |  |  |  | f5 | CC 50 | 23130 | 711 | 5130 | 741 |
|  |  |  |  |  | f6 | N/A | N/A | N/A | N/A | N/A |
| CA_7A-20A, DC_7A-20A | 70+30 |  | 50 + 50 |  | f1 | CC 50 | 21100 | 2535 | 3100 | 2655 |  |
|  |  |  |  |  | f2 | CC 50 | [21400] | [2565] | [3400] | [2685] |  |
|  |  |  |  |  | f5 | CC 50 | 24200 | 837 | 6200 | 796 |  |
|  |  |  |  |  | f6 | CC 50 | 24400 | 857 | 6400 | 816 |  |
| CA_7A-22A | 70+80 |  | 50+50 |  | f1 | CC 50 | 21100 | 2535 | 3100 | 2655 |
|  |  |  |  |  | f2 | CC 50 | [21400] | [2565] | [3400] | [2685] |
|  |  |  |  |  | f1 | CC 50 | 24650 | 3415 | 6650 | 3515 |
|  |  |  |  |  | f2 | CC 50 | 25350 | 3485 | 7350 | 3585 |
| CA_7A-28A, DC_7A-28A | 70+45 | 50+50 |  |  | f1 | CC 50 | 21100 | 2535 | 3100 | 2655 |  |
|  |  |  |  |  | f2 | CC 50 | [21400] | [2565] | [3400] | [2685] |  |
|  |  |  |  |  | f5 | CC 50 | 27410 | 723 | 9410 | 778 |  |
|  |  |  |  |  | f6 | CC 50 | 27610 | 743 | 9610 | 798 |  |
| CA_7A-46A | 70+775 | 50+100 |  |  | f1 | CC 50 | 21100 | 2535 | 3100 | 2655 |  |
|  |  |  |  |  | f2 | CC 50 | [21400] | [2565] | [3400] | [2685] |  |
|  |  |  |  |  | f5 | CC 100 | 50690 | 5540 | 50690 | 5540 |  |
|  |  |  |  |  | f6 | CC 100 | 53940 | 5865 | 53940 | 5865 |  |
| CA_8A-11A | 35 + 20 |  | 50 + 50 |  | f1 | CC 50 | 21750 | 910 | 3750 | 955 |  |
|  |  |  |  |  | f2 | N/A | N/A | N/A | N/A | N/A |  |
|  |  |  |  |  | f5 | CC 50 | 22800 | 1432.9 | 4800 | 1480.9 |  |
|  |  |  |  |  | f6 | N/A | N/A | N/A | N/A | N/A |  |
| CA_8A-20A | 35 + 30 | 50 + 50 |  |  | f1 | CC 50 | 21625 | 897.5 | 3625 | 942.5 |  |
|  |  |  |  |  | f2 | CC 50 | 21750 | 910 | 3750 | 955 |  |
|  |  |  |  |  | f5 | CC 50 | 24200 | 837 | 6200 | 796 |  |
|  |  |  |  |  | f6 | CC 50 | 24400 | 857 | 6400 | 816 |  |
| CA_8A-40A | 35 + 100 | 50 + 100 |  |  | f1 | CC 50 | 21625 | 897.5 | 3625 | 942.5 |  |
|  |  |  |  |  | f2 | CC 50 | 21750 | 910 | 3750 | 955 |  |
|  |  |  |  |  | f5 | CC 100 | 39150 | 2350 | 39150 | 2350 |  |
|  |  |  |  |  | f6 | CC 100 | 39550 | 2390 | 39550 | 2390 |  |
| CA_8A_41A | 35 + 194 | 50 + 100 |  |  | f1 | CC 50 | 21651 | 900.1 | 3651 | 945.1 |  |
|  |  |  |  |  | f2 | CC 50 | 21750 | 910 | 3750 | 955 |  |
|  |  |  |  |  | f5 | CC 100 | 40521 | 2583.1 | 40521 | 2583.1 |  |
|  |  |  |  |  | f6 | CC 100 | 40719 | 2602.9 | 40719 | 2602.9 |  |
| CA_8A_41C | 35 + 194 | 50 + 100+100 |  |  | f1 | CC 50 | 21651 | 900.1 | 3651 | 945.1 |  |
|  |  |  |  |  | f2 | CC 50 | 21750 | 910 | 3750 | 955 |  |
|  |  |  |  |  | f5 | CC 100 | 40521 | 2583.1 | 40521 | 2583.1 |  |
|  |  |  |  |  | f6 | CC 100 | 40719 | 2602.9 | 40719 | 2602.9 |  |
| CA_8A-42A | 35+200 | 50 + 100 |  |  | f1 | CC 50 | 21625 | 897.5 | 3625 | 942.5 |  |
|  |  |  |  |  | f2 | CC 50 | 21750 | 910 | 3750 | 955 |  |
|  |  |  |  |  | f5 | CC 100 | 42590 | 3500 | 42590 | 3500 |  |
|  |  |  |  |  | f6 | CC 100 | 43490 | 3590 | 43490 | 3590 |  |
| CA_11A-18A | 20+15 | 50 + 50 |  |  | f1 | CC 50 | 22800 | 1432.9 | 4800 | 1480.9 |  |
|  |  |  |  |  | f2 | CC 50 | 22900 | 1442.9 | 4900 | 1490.9 |  |
|  |  |  |  |  | f5 | CC 50 | 23900 | 820 | 5900 | 865 |  |
|  |  |  |  |  | f6 | N/A | N/A | N/A | N/A | N/A |  |
| CA_12A-25A | 17+65 | 50 + 50 |  |  | f1 | CC 50 | 23060 | 704 | 5060 | 734 |  |
|  |  |  |  |  | f2 | N/A | N/A | N/A | N/A | N/A |  |
|  |  |  |  |  | f5 | CC 50 | 26363 | 1882.5 | 8365 | 1962.5 |  |
|  |  |  |  |  | f6 | CC 50 | 26640 | 1910 | 8640 | 1990 |  |
| CA_12A-30A | 17+10 |  | 50 + 50 |  | f1 | CC 50 | 23060 | 704 | 5060 | 734 |  |
|  |  |  |  |  | f2 | N/A | N/A | N/A | N/A | N/A |  |
|  |  |  |  |  | f5 | CC 50 | 27710 | 2310 | 9820 | 2355 |  |
|  |  |  |  |  | f6 | N/A | N/A | N/A | N/A | N/A |  |
| CA_18A-28A | 15+30 | 50 + 50 |  |  | f1 | CC 50 | 23900 | 820 | 5900 | 865 |  |
|  |  |  |  |  | f2 | N/A | N/A | N/A | N/A | N/A |  |
|  |  |  |  |  | f5 | CC 50 | 27360 | 718 | 9360 | 773 |  |
|  |  |  |  |  | f6 | CC 50 | 27460 | 728 | 9460 | 783 |  |
| CA_19A-21A, DC_19A-21A | 15+15 | 50+75 |  |  | f1 | CC 50 | 24075 | 837.5 | 6075 | 882.5 |  |
|  |  |  |  |  | f2 | N/A | N/A | N/A | N/A | N/A |  |
|  |  |  |  |  | f5 | CC 75 | 24525 | 1455.4 | 6525 | 1503.4 |  |
|  |  |  |  |  | f6 | N/A | N/A | N/A | N/A | N/A |  |
| CA_19A-28A | 15+30 | 50+50 |  |  | f1 | CC 50 | 24075 | 837.5 | 6075 | 882.5 |  |
|  |  |  |  |  | f2 | N/A | N/A | N/A | N/A | N/A |  |
|  |  |  |  |  | f5 | CC 50 | 27360 | 718 | 9360 | 773 |  |
|  |  |  |  |  | f6 | CC 50 | 27460 | 728 | 9460 | 783 |  |
| CA_19A-42A | 15 + 200 | 50 + 100 |  |  | f1 | CC 50 | 24075 | 837.5 | 6075 | 882.5 |  |
|  |  |  |  |  | f2 | N/A | N/A | N/A | N/A | N/A |  |
|  |  |  |  |  | f5 | CC 100 | 42590 | 3500 | 42590 | 3500 |  |
|  |  |  |  |  | f6 | CC 100 | 43490 | 3590 | 43490 | 3590 |  |
| CA_19A-42C | 15+200 |  |  | 50+100+100 | f1 | CC 50 | 24075 | 837.5 | 6075 | 882.5 |
|  |  |  |  |  | f2 | N/A | N/A | N/A | N/A | N/A |
|  |  |  |  |  | f5 | CC 100 | 42491 | 3490.1 | 42491 | 3490.1 |
|  |  |  |  |  | f6 | CC 100 | 42689 | 3509.9 | 42689 | 3509.9 |
| CA_20A-28A | 30+45 | 50 + 50 |  |  | f1 | CC 50 | 24300 | 847 | 6300 | 806 |  |
|  |  |  |  |  | f2 | CC 50 | 24400 | 867 | 6400 | 816 |  |
|  |  |  |  |  | f5 | CC 50 | 27410 | 723 | 9410 | 778 |  |
|  |  |  |  |  | f6 | CC 50 | 27610 | 743 | 9610 | 798 |  |
| CA_20A-31A | 30+5 | 50 + 25 |  |  | f1 | CC 50 | 24300 | 847 | 6300 | 806 |  |
|  |  |  |  |  | f2 | CC 50 | 24400 | 867 | 6400 | 816 |  |
|  |  |  |  |  | f5 | CC 25 | 27785 | 455 | 9895 | 465 |  |
|  |  |  |  |  | f6 | N/A | N/A | N/A | N/A | N/A |  |
| CA_20A-32A | 30+44 | 50 + 50 |  |  | f1 | CC 50 | 24300 | 847 | 6300 | 806 |  |
|  |  |  |  |  | f2 | CC 50 | 24400 | 867 | 6400 | 816 |  |
|  |  |  |  |  | f5 | CC 50 | N/A | N/A | 10140 | 1474 |  |
|  |  |  |  |  | f6 | CC 50 | N/A | N/A | 10310 | 1491 |  |
| CA_20A-40A | 30+100 |  | 50 + 100 |  | f1 | CC 50 | 24300 | 847 | 6300 | 806 |  |
|  |  |  |  |  | f2 | CC 50 | 24400 | 867 | 6400 | 816 |  |
|  |  |  |  |  | f5 | CC 100 | 39150 | 2350 | 39150 | 2350 |  |
|  |  |  |  |  | f6 | CC 100 | 39550 | 2390 | 39550 | 2390 |  |
| CA_20A-67A | 30+20 | 50 + 50 |  |  | f1 | CC 50 | 24300 | 847 | 6300 | 806 |  |
|  |  |  |  |  | f2 | CC 50 | 24400 | 867 | 6400 | 816 |  |
|  |  |  |  |  | f5 | CC 50 | N/A | N/A | 67386 | 743 |  |
|  |  |  |  |  | f6 | CC 50 | N/A | N/A | 67486 | 753 |  |
| CA_21A-42C | 15+200 | 75+100+100 |  |  | f1 | CC 75 | 24525 | 1455.4 | 6525 | 1503.4 |  |
|  |  |  |  |  | f2 | N/A | N/A | N/A | N/A | N/A |  |
|  |  |  |  |  | f5 | CC 100 | 42491 | 3490.1 | 42491 | 3490.1 |  |
|  |  |  |  |  | f6 | CC 100 | 42689 | 3509.9 | 42689 | 3509.9 |  |
| CA_23A-29A | 20+11 | 50 + 50 |  |  | f1 | CC 50 | 25550 | 2005 | 7550 | 2185 |  |
|  |  |  |  |  | f2 | CC 50 | 25650 | 2015 | 7650 | 2195 |  |
|  |  |  |  |  | f5 | CC 50 | N/A | N/A | 9720 | 723 |  |
|  |  |  |  |  | f6 | N/A | N/A | N/A | N/A | N/A |  |
| CA_25A-41A | 65+194 |  |  | 50 + 50 | f1 | CC 50 | 26363 | 1882.5 | 8365 | 1962.5 |
|  |  |  |  |  | f2 | CC 50 | 26640 | 1910 | 8640 | 1990 |
|  |  |  |  |  | f5 | CC 50 | 40620 | 2593 | 40620 | 2593 |
|  |  |  |  |  | f6 | CC 50 | 41540 | 2685 | 41540 | 2685 |
| CA_25A-41C | 65+194 |  |  | 50+100+100 | f1 | CC 50 | 26363 | 1882.5 | 8365 | 1962.5 |
|  |  |  |  |  | f2 | CC 50 | 26640 | 1910 | 8640 | 1990 |
|  |  |  |  |  | f5 | CC 100 | 40521 | 2583.1 | 40521 | 2583.1 |
|  |  |  |  |  | f6 | CC 100 | 40719 | 2602.9 | 40719 | 2602.9 |
| CA_26A-41A | 60 + 194 |  | 50 + 100 |  | f1 | CC 50 | 26865 | 831.5 | 8865 | 876.5 |  |
|  |  |  |  |  | f2 | CC 50 | 26990 | 844 | 8990 | 889 |  |
|  |  |  |  |  | f5 | CC 100 | 40620 | 2593 | 40620 | 2593 |  |
|  |  |  |  |  | f6 | CC 100 | 41490 | 2680 | 41490 | 2680 |  |
| CA_26A-41C | 60+194 |  |  | 50+100+100 | f1 | CC 50 | 26865 | 831.5 | 8865 | 876.5 |
|  |  |  |  |  | f2 | CC 50 | 26990 | 844 | 8990 | 889 |
|  |  |  |  |  | f5 | CC 100 | 40521 | 2583.1 | 40521 | 2583.1 |
|  |  |  |  |  | f6 | CC 100 | 40719 | 2602.9 | 40719 | 2602.9 |
| CA_28A-41A | 45+194 |  |  | 50 + 100 | f1 | CC 50 | 27410 | 723 | 9410 | 778 |
|  |  |  |  |  | f2 | CC 50 | 27610 | 743 | 9610 | 798 |
|  |  |  |  |  | f5 | CC 100 | 40620 | 2593 | 40620 | 2593 |
|  |  |  |  |  | f6 | CC 100 | 41490 | 2680 | 41490 | 2680 |
| CA_28A-41C | 45+194 |  |  | 50+100+100 | f1 | CC 50 | 27410 | 723 | 9410 | 778 |
|  |  |  |  |  | f2 | CC 50 | 27610 | 743 | 9610 | 798 |
|  |  |  |  |  | f5 | CC 100 | 40521 | 2583.1 | 40521 | 2583.1 |
|  |  |  |  |  | f6 | CC 100 | 40719 | 2602.9 | 40719 | 2602.9 |
| CA_28A-42A | 45+200 |  |  | 50 + 100 | f1 | CC 50 | 27410 | 723 | 9410 | 778 |
|  |  |  |  |  | f2 | CC 50 | 27610 | 743 | 9610 | 798 |
|  |  |  |  |  | f5 | CC 100 | 42590 | 3500 | 42590 | 3500 |
|  |  |  |  |  | f6 | CC 100 | 43490 | 3590 | 43490 | 3590 |
| CA_28A-42C | 45+200 |  |  | 50+100+100 | f1 | CC 50 | 27410 | 723 | 9410 | 778 |
|  |  |  |  |  | f2 | CC 50 | 27610 | 743 | 9610 | 798 |
|  |  |  |  |  | f5 | CC 100 | 42491 | 3490.1 | 42491 | 3490.1 |
|  |  |  |  |  | f6 | CC 100 | 42689 | 3509.9 | 42689 | 3509.9 |
| CA_29A-30A (Note 1) | 11+10 |  |  | 50+50 | f1 | CC 50 | N/A | N/A | 9720 | 723 |
|  |  |  |  |  | f2 | N/A | N/A | N/A | N/A | N/A |
|  |  |  |  |  | f5 | CC 50 | 27710 | 2310 | 9820 | 2355 |
|  |  |  |  |  | f6 | N/A | N/A | N/A | N/A | N/A |
| CA_39A-41A, DC_39A-41A | 40+194 | 100+100 |  |  | f1 | CC 100 | 38350 | 1890 | 38350 | 1890 |  |
|  |  |  |  |  | f2 | CC 100 | 38550 | 1910 | 38550 | 1910 |  |
|  |  |  |  |  | f5 | CC 100 | 40620 | 2593 | 40620 | 2593 |  |
|  |  |  |  |  | f6 | CC 100 | 41490 | 2680 | 41490 | 2680 |  |
| CA_41A-42A | 194+200 | 100 + 100 |  |  | f1 | CC 100 | 40620 | 2593 | 40620 | 2593 |  |
|  |  |  |  |  | f2 | CC 100 | 41490 | 2680 | 41490 | 2680 |  |
|  |  |  |  |  | f5 | CC 100 | 42590 | 3500 | 42590 | 3500 |  |
|  |  |  |  |  | f6 | CC 100 | 43490 | 3590 | 43490 | 3590 |  |
| CA_41A-42C | 194+200 | 100+100+100 |  |  | f1 | CC100 | 40620 | 2593 | 40620 | 2593 |  |
|  |  |  |  |  | f2 | CC 100 | 41490 | 2680 | 41490 | 2680 |  |
|  |  |  |  |  | f5 | CC 100 | 42491 | 3490.1 | 42491 | 3490.1 |  |
|  |  |  |  |  | f6 | CC 100 | 42689 | 3509.9 | 42689 | 3509.9 |  |
| CA_41A-46A | 194+775 | 100+100 |  |  | f1 | CC 100 | 40620 | 2593 | 40620 | 2593 |  |
|  |  |  |  |  | f2 | CC 100 | 41490 | 2680 | 41490 | 2680 |  |
|  |  |  |  |  | f5 | CC 100 | 50690 | 5540 | 50690 | 5540 |  |
|  |  |  |  |  | f6 | CC 100 | 53940 | 5865 | 53940 | 5865 |  |
| CA_41C-42A | 194+200 | 100+100+100 |  |  | f1 | CC 100 | 40521 | 2583.1 | 40521 | 2583.1 |  |
|  |  |  |  |  | f2 | CC 100 | 40719 | 2602.9 | 40719 | 2602.9 |  |
|  |  |  |  |  | f5 | CC 100 | 42590 | 3500 | 42590 | 3500 |  |
|  |  |  |  |  | f6 | CC 100 | 43490 | 3590 | 43490 | 3590 |  |
| CA_42A-46A | 200+775 | 100+100 |  |  | f1 | CC 100 | 42590 | 3500 | 42590 | 3500 |  |
|  |  |  |  |  | f2 | CC 100 | 43490 | 3590 | 43490 | 3590 |  |
|  |  |  |  |  | f5 | CC 100 | 50690 | 5540 | 50690 | 5540 |  |
|  |  |  |  |  | f6 | CC 100 | 53940 | 5865 | 53940 | 5865 |  |
| Note 1: Exceptionally the second band test frequencies of this CA combination shall be used for PCell and the first band test frequencies of this CA combination shall be used for the SCell respectively. |  |  |  |  |  |  |  |  |  |  |  |

<End of second modified section>