3GPP TSG-RAN5 Meeting #75	R5-172482

# Hangzhou, China, 15th May 2017 - 19th May 2017

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 4034 | rev | - | Current version: | 14.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NB-IoT RLC test case 22.3.1.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TDIA, Starpoint, MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOT-UEConTest |  |  |  |  |  |  | Date: |  | 2017-05-03 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - Correct TBSUL in Table 22.3.1.6.3.2-2. - Move step 1 to step 3A. Generating two segments will spend more time of step 2 - 3, so the timing of sending two segments will be late. Get start time in step 3A in every loop. - Change timing of step 4/4A/5/7. Sinse TTCN did not calculate time of transmission of NPSS, NSSS, MIB, SIB and SIs, the opportunity of sending DL data of step 4/4A perhaps meets transmission of them, SS will wait for appropriate timing for NPDCCH and NPDSCH transmission. In that case UE do not have sufficient time to handle and loopback data because time interval between SS sending DL data of step 4/4A and allocating an uplink grant of step 5 is too small. So Increase the time of SS allocating an uplink grant. And it does not impact test purpose. - Void Step 8. - Use RLC AMD PDUs instead of RLC AMD segment to reduce the RLC overhead by 2 octets for each MAC PDU in step 3. If TBSDL < 100 bits, split the DL RLC SDU into three RLC AMD PDUs, otherwise split it into two RLC AMD PDUs. Because 2*TBSDL is not sufficient for loading whole DL RLC SDU when TBSDL < 100 bits. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct TBSUL in Table 22.3.1.6.3.2-2. - Move step 1 to step 3A. - Change timing of step 4/4A/5/7. - Void step 8. - Use RLC AMD PDUs instead of RLC AMD segment. If TBSDL < 100 bits, split the DL RLC SDU into three RLC AMD PDUs, otherwise split it into two RLC AMD PDUs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE will fail this testcase. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.3.1.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< start of modified section >

22.3.1.6	NB-IoT / DL-SCH /UL-SCH transport block size selection / DCI format N1/ N0

22.3.1.6.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE on NPDCCH receives DCI format N1 indicating a resource assignment corresponding to  number of subframes and a modulation and coding scheme  }

then { UE decodes the received transport block of size corresponding to the read  and  and forwards it to higher layers }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE has pending data for transmission and receives a Resource Block Assignment corresponding to  number of subframes and a modulation and coding scheme  for NPUSCH scheduling }

then { UE transmits MAC PDU on NPUSCH on the granted resources using a transport block size corresponding to the read  and  }

22.3.1.6.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 36.212, clause 6.4.3.1, 6.4.3.2; TS 36.213, clauses 16.4.3.1, 16.4.1.5, 16.4.1.5.1; and TS 36.306 clause 4.1C.

[TS 36.212, clause 6.4.3.1]

DCI format N0 is used for the scheduling of NPUSCH in one UL cell.

The following information is transmitted by means of the DCI format N0:

-	Flag for format N0/format N1 differentiation – 1 bit, where value 0 indicates format N0 and value 1 indicates format N1

-	Subcarrier indication – 6 bits as defined in section 16.5.1.1 of [3]

-	Resource assignment – 3 bits as defined in section 16.5.1.2 of [3]

-	Scheduling delay – 2 bits as defined in section 16.5.1 of [3]

-	Modulation and coding scheme – 4 bits as defined in section 16.5.1.2 of [3]

-	Redundancy version – 1 bit as defined in section 16.5.1.2 of [3]

-	Repetition number – 3 bits as defined in section 16.5.1.2 of [3]

-	New data indicator – 1 bit

-	DCI subframe repetition number – 2 bits as defined in section 16.6 in [3]

[TS 36.212, clause 6.4.3.2]

DCI format N1 is used for the scheduling of one NPDSCH codeword in one cell and random access procedure initiated by a NPDCCH order. The DCI corresponding to a NPDCCH order is carried by NPDCCH.

The following information is transmitted by means of the DCI format N1:

-	Flag for format N0/format N1 differentiation – 1 bit, where value 0 indicates format N0 and value 1 indicates format N1

-	NPDCCH order indicator – 1 bit

Format N1 is used for random access procedure initiated by a NPDCCH order only if NPDCCH order indicator is set to ‘1’, format N1 CRC is scrambled with C-RNTI, and all the remaining fields are set as follows:

-	Starting number of NPRACH repetitions – 2 bits as defined in section 16.3.1 of [3]

-	Subcarrier indication of NPRACH – 6 bits as defined in section 16.3.1 of [3]

-	All the remaining bits in format N1 are set to one

Otherwise,

-	Scheduling delay – 3 bits as defined in section 16.4.1 of [3]

-	Resource assignment – 3 bits as defined in section 16.4.1.3 of [3]

-	Modulation and coding scheme – 4 bits as defined in section 16.4.1.5 of [3]

-	Repetition number – 4 bits as defined in section 16.4.1.3 of [3]

-	New data indicator – 1 bit

-	HARQ-ACK resource – 4 bits as defined in section 16.4.2 of [3].

-	DCI subframe repetition number – 2 bits as defined in section 16.6 in [3]

When the format N1 CRC is scrambled with a RA-RNTI, then the following fields among the fields above are reserved:

-	New data indicator

-	HARQ-ACK resource

If the number of information bits in format N1 is less than that of format N0, zeros shall be appended to format N1 until the payload size equals that of format N0.

[TS 36.306, clause 4.1C]

The field ue-Category-NB defines a combined uplink and downlink capability in NB-IoT. The parameters set by the UE Category are defined in subclause 4.2. Tables 4.1C-1 and 4.1C-2 define the downlink and, respectively, uplink physical layer parameter values for each UE Category.

Table 4.1C-1: Downlink physical layer parameter values set by the field ue-Category-NB

| UE Category | Maximum number of DL-SCH transport block bits received within a TTI | Maximum number of bits of a DL-SCH transport block received within a TTI | Total number of soft channel bits |
| --- | --- | --- | --- |
| Category NB1 | 680 | 680 | 2112 |

Table 4.1C-2: Uplink physical layer parameter values set by the field ue-Category-NB

| UE Category | Maximum number of UL-SCH transport block bits transmitted within a TTI | Maximum number of bits of an UL-SCH transport block transmitted within a TTI |
| --- | --- | --- |
| Category NB1 | 1000 | 1000 |

[TS 36.213, clause 16.4.1.3]

The resource allocation information in DCI format N1, N2 (paging) for NPDSCH indicates to a scheduled UE

-	a number of subframes () determined by the resource assignment field () in the corresponding DCI according to Table 16.4.1.3-1.

-	a repetition number () determined by the repetition number field () in the corresponding DCI according to Table 16.4.1.3-2.

Table 16.4.1.3-1: Number of subframes () for NPDSCH

| 0 | 1 |
| --- | --- |
| 1 | 2 |
| 2 | 3 |
| 3 | 4 |
| 4 | 5 |
| 5 | 6 |
| 6 | 8 |
| 7 | 10 |

Table 16.4.1.3-2: Number of repetitions () for NPDSCH

| 0 | 1 |
| --- | --- |
| 1 | 2 |
| 2 | 4 |
| 3 | 8 |
| 4 | 16 |
| 5 | 32 |
| 6 | 64 |
| 7 | 128 |
| 8 | 192 |
| 9 | 256 |
| 10 | 384 |
| 11 | 512 |
| 12 | 768 |
| 13 | 1024 |
| 14 | 1536 |
| 15 | 2048 |

The number of repetitions for the NPDSCH carrying SystemInformationBlockType1-NB is determined based on the parameter schedulingInfoSIB1 configured by higher-layers and according to Table 16.4.1.3-3.

[TS 36.213, clause 16.4.1.5]

The UE shall use modulation order, = 2.

To determine the transport block size in the NPDSCH, the UE shall first,

-	if NPDSCH carries SystemInformationBlockType1-NB

-	set  to the value of the parameter schedulingInfoSIB1 configured by higher-layers

-	otherwise

-	read the 4-bit "modulation and coding scheme" field () in the DCI and set .

and second,

-	if NPDSCH carries SystemInformationBlockType1-NB

-	use subclause 16.4.1.5.2 for determining its transport block size.

-	otherwise,

-	read the 3-bit "resource assignment" field () in the DCI and determine its TBS by the procedure in subclause 16.4.1.5.1.

The NDI as signalled on NPDCCH, and the TBS, as determined above, shall be delivered to higher layers.

[TS 36.213, clause 16.4.1.5.1]

The TBS is given by the (,) entry of Table 16.4.1.5.1-1. For the value of the higher layer parameter operationModeInfo set to ‘00’ or ‘01’, .

Table 16.4.1.5.1-1: Transport block size (TBS) table

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 16 | 32 | 56 | 88 | 120 | 152 | 208 | 256 |
| 1 | 24 | 56 | 88 | 144 | 176 | 208 | 256 | 344 |
| 2 | 32 | 72 | 144 | 176 | 208 | 256 | 328 | 424 |
| 3 | 40 | 104 | 176 | 208 | 256 | 328 | 440 | 568 |
| 4 | 56 | 120 | 208 | 256 | 328 | 408 | 552 | 680 |
| 5 | 72 | 144 | 224 | 328 | 424 | 504 | 680 |  |
| 6 | 88 | 176 | 256 | 392 | 504 | 600 |  |  |
| 7 | 104 | 224 | 328 | 472 | 584 | 680 |  |  |
| 8 | 120 | 256 | 392 | 536 | 680 |  |  |  |
| 9 | 136 | 296 | 456 | 616 |  |  |  |  |
| 10 | 144 | 328 | 504 | 680 |  |  |  |  |
| 11 | 176 | 376 | 584 |  |  |  |  |  |
| 12 | 208 | 440 | 680 |  |  |  |  |  |

[TS 36.213, clause 16.5.1.1]

The resource allocation information in uplink DCI format N0 for NPUSCH transmission indicates to a scheduled UE

- a set of contiguously allocated subcarriers () of a resource unit determined by the Subcarrier indication field in the corresponding DCI,

- a number of resource units () determined by the resource assignment field in the corresponding DCI according to Table 16.5.1.1-2,

- a repetition number () determined by the repetition number field in the corresponding DCI according to Table 16.5.1.1-3.

The subcarrier spacing  of NPUSCH transmission is determined by the uplink subcarrier spacing field in the Narrowband Random Access Response Grant according to subclause 16.3.3.

For NPUSCH transmission with subcarrier spacing, where  is the subcarrier indication field in the DCI. is reserved.

For NPUSCH transmission with subcarrier spacing, the subcarrier indication field () in the DCI determines the set of contiguously allocated subcarriers () according to Table 16.5.1.1-1.

Table 16.5.1.1-1: Allocated subcarriers for NPUSCH with

| Subcarrier indication field () | Set of Allocated subcarriers () |
| --- | --- |
| 0 – 11 |  |
| 12-15 |  |
| 16-17 |  |
| 18 |  |
| 19-63 | Reserved |

Table 16.5.1.1-2: Number of resource units () for NPUSCH

| 0 | 1 |
| --- | --- |
| 1 | 2 |
| 2 | 3 |
| 3 | 4 |
| 4 | 5 |
| 5 | 6 |
| 6 | 8 |
| 7 | 10 |

Table 16.5.1.1-3: Number of repetitions () for NPUSCH.

| 0 | 1 |
| --- | --- |
| 1 | 2 |
| 2 | 4 |
| 3 | 8 |
| 4 | 16 |
| 5 | 32 |
| 6 | 64 |
| 7 | 128 |

[TS 36.213, clause 16.5.1.2]

To determine the modulation order, redundancy version and transport block size for the NPUSCH, the UE shall first

-	read the “modulation and coding scheme” field () in the DCI, and

-	read the “redundancy version” field () in the DCI, and

-	read the "resource assignment" field () in the DCI, and

-	compute the total number of allocated subcarriers (), number of resource units (), and repetition number () according to subclause 16.5.1.1.

The UE shall use modulation order, = 2 if . The UE shall useand Table 16.5.1.2-1 to determine the modulation order to use for NPUSCH if .

Table 16.5.1.2-1: Modulation and TBS index table for NPUSCH with

| MCS Index | Modulation Order | TBS Index |
| --- | --- | --- |
| 0 | 1 | 0 |
| 1 | 1 | 2 |
| 2 | 2 | 1 |
| 3 | 2 | 3 |
| 4 | 2 | 4 |
| 5 | 2 | 5 |
| 6 | 2 | 6 |
| 7 | 2 | 7 |
| 8 | 2 | 8 |
| 9 | 2 | 9 |
| 10 | 2 | 10 |

NPUSCH is transmitted in N consecutive NB-IoT UL slots, ni , i=0,1,…,N-1. The redundancy version  of the NPUSCH transmission in jth block of B consecutive NB-IoT UL slots ni , is determined by, , where  if ,  otherwise. Portion of NPUSCH codeword with  as defined in clause 6.3.2 in [4] mapped to slot  of allocated  resource unit(s) is transmitted in NB-IoT UL slots ni. for and   for .

The UE shall use (,) and Table 16.5.1.2-2 to determine the TBS to use for the NPUSCH. is given in Table 16.5.1.2.1-1 if ,  otherwise.

Table 16.5.1.2-2: Transport block size (TBS) table for NPUSCH

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 16 | 32 | 56 | 88 | 120 | 152 | 208 | 256 |
| 1 | 24 | 56 | 88 | 144 | 176 | 208 | 256 | 344 |
| 2 | 32 | 72 | 144 | 176 | 208 | 256 | 328 | 424 |
| 3 | 40 | 104 | 176 | 208 | 256 | 328 | 440 | 568 |
| 4 | 56 | 120 | 208 | 256 | 328 | 408 | 552 | 680 |
| 5 | 72 | 144 | 224 | 328 | 424 | 504 | 680 | 872 |
| 6 | 88 | 176 | 256 | 392 | 504 | 600 | 808 | 1000 |
| 7 | 104 | 224 | 328 | 472 | 584 | 712 | 1000 |  |
| 8 | 120 | 256 | 392 | 536 | 680 | 808 |  |  |
| 9 | 136 | 296 | 456 | 616 | 776 | 936 |  |  |
| 10 | 144 | 328 | 504 | 680 | 872 | 1000 |  |  |
| 11 | 176 | 376 | 584 | 776 | 1000 |  |  |  |
| 12 | 208 | 440 | 680 | 1000 |  |  |  |  |

Table 16.4.1.5.1-1: Transport block size (TBS) table

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 16 | 32 | 56 | 88 | 120 | 152 | 208 | 256 |
| 1 | 24 | 56 | 88 | 144 | 176 | 208 | 256 | 344 |
| 2 | 32 | 72 | 144 | 176 | 208 | 256 | 328 | 424 |
| 3 | 40 | 104 | 176 | 208 | 256 | 328 | 440 | 568 |
| 4 | 56 | 120 | 208 | 256 | 328 | 408 | 552 | 680 |
| 5 | 72 | 144 | 224 | 328 | 424 | 504 | 680 |  |
| 6 | 88 | 176 | 256 | 392 | 504 | 600 |  |  |
| 7 | 104 | 224 | 328 | 472 | 584 | 680 |  |  |
| 8 | 120 | 256 | 392 | 536 | 680 |  |  |  |
| 9 | 136 | 296 | 456 | 616 |  |  |  |  |
| 10 | 144 | 328 | 504 | 680 |  |  |  |  |
| 11 | 176 | 376 | 584 |  |  |  |  |  |
| 12 | 208 | 440 | 680 |  |  |  |  |  |

The NDI as signalled on NPDCCH, and the RV and TBS, as determined above, shall be delivered to higher layers.

22.3.1.6.3	Test description

22.3.1.6.3.1	Pre-test conditions

System Simulator:

-	Ncell 1.

-	MasterInformationBlock-NB using parameters as specified in TS 36.508 [18] Table 8.1.4.3.2-1 with condition "Standalone"
NOTE: According to TS 36.213 [30] clause 16.4.1.5.1 for Inband Operation Mode ITBS is restricted to 0 ≤ ITBS ≤ 10 i.e. ISF/IMCS combinations with IMCS = 11, 12 could not be tested

UE:

None.

Preamble:

-	UE is in state NB-IoT UE Attach, Connected Mode, UE Test Loopback Activated (State 2B-NB) according to [18] in Ncell 1.

22.3.1.6.3.2	Test procedure sequence

Table 22.3.1.6.3.2-1: RLC SDU size used as test data

| TBSUL [bits] | UL RLC SDU size [bits] |
| --- | --- |
| 56 < TBsize ≤1000 (Note) | TBSUL – 56 (Note) |
| Note: UL MAC PDU contains 2 MAC SDUs (RLC STATUS PDU, RLC AMD PDU with a single RLC SDU): 2 bytes R/R/E/LCID/L header for 1st MAC SDU 1 byte R/R/E/LCID header for 2nd MAC SDU 2 bytes RLC STATUS PDU 2 bytes RLC AMD header N bytes UL RLC SDU N = (TBSUL – 56) / 8 |  |

Table 22.3.1.6.3.2-2: TB sizes and ISF/ITBS combinations used for UL and DL

|  | DL |  |  | UL |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| S.No | ISF | IMCS = ITBS | TBSDL | IRU | IMCS / ITBS | TBSUL |
| 1 | 0 | 0 | 16 | 0 | 0/0 | 16 |
| 2 | 0 | 1 | 24 | 0 | 2/1 | 24 |
| 3 | 0 | 2 | 32 | 0 | 1/2 | 32 |
| 4 | 0 | 3 | 40 | 0 | 3/3 | 40 |
| 5 | 0 | 4 | 56 | 0 | 4/4 | 56 |
| 6 | 0 | 5 | 72 | 0 | 5/5 | 72 |
| 7 | 0 | 6 | 88 | 0 | 6/6 | 88 |
| 8 | 0 | 7 | 104 | 0 | 7/7 | 104 |
| 9 | 0 | 8 | 120 | 0 | 8/8 | 120 |
| 10 | 0 | 9 | 136 | 0 | 9/9 | 136 |
| 11 | 0 | 10 | 144 | 0 | 10/10 | 144 |
| 12 | 0 | 11 | 176 | 0 | 10/10 | 144 |
| 13 | 0 | 12 | 208 | 0 | 10/10 | 144 |
| 14 | 1 | 0 | 32 | 1 | 0/0 | 32 |
| 15 | 1 | 1 | 56 | 1 | 2/1 | 56 |
| 16 | 1 | 2 | 72 | 1 | 1/2 | 72 |
| 17 | 1 | 3 | 104 | 1 | 3/3 | 104 |
| 18 | 1 | 4 | 120 | 1 | 4/4 | 120 |
| 19 | 1 | 5 | 144 | 1 | 5/5 | 144 |
| 20 | 1 | 6 | 176 | 1 | 6/6 | 176 |
| 21 | 1 | 7 | 224 | 1 | 7/7 | 224 |
| 22 | 1 | 8 | 256 | 1 | 8/8 | 256 |
| 23 | 1 | 9 | 296 | 1 | 9/9 | 296 |
| 24 | 1 | 10 | 328 | 1 | 10/10 | 328 |
| 25 | 1 | 11 | 376 | 1 | 10/10 | 328 |
| 26 | 1 | 12 | 440 | 1 | 10/10 | 328 |
| 27 | 2 | 0 | 56 | 2 | 0/0 | 56 |
| 28 | 2 | 1 | 88 | 2 | 2/1 | 88 |
| 29 | 2 | 2 | 144 | 2 | 1/2 | 144 |
| 30 | 2 | 3 | 176 | 2 | 3/3 | 176 |
| 31 | 2 | 4 | 208 | 2 | 4/4 | 208 |
| 32 | 2 | 5 | 224 | 2 | 5/5 | 224 |
| 33 | 2 | 6 | 256 | 2 | 6/6 | 256 |
| 34 | 2 | 7 | 328 | 2 | 7/7 | 328 |
| 35 | 2 | 8 | 392 | 2 | 8/8 | 392 |
| 36 | 2 | 9 | 456 | 2 | 9/9 | 456 |
| 37 | 2 | 10 | 504 | 2 | 10/10 | 504 |
| 38 | 2 | 11 | 584 | 2 | 10/10 | 504 |
| 39 | 2 | 12 | 680 | 2 | 10/10 | 504 |
| 40 | 3 | 0 | 88 | 3 | 0/0 | 88 |
| 41 | 3 | 1 | 144 | 3 | 2/1 | 144 |
| 42 | 3 | 2 | 176 | 3 | 1/2 | 176 |
| 43 | 3 | 3 | 208 | 3 | 3/3 | 208 |
| 44 | 3 | 4 | 256 | 3 | 4/4 | 256 |
| 45 | 3 | 5 | 328 | 3 | 5/5 | 328 |
| 46 | 3 | 6 | 392 | 3 | 6/6 | 392 |
| 47 | 3 | 7 | 472 | 3 | 7/7 | 472 |
| 48 | 3 | 8 | 536 | 3 | 8/8 | 536 |
| 49 | 3 | 9 | 616 | 3 | 9/9 | 616 |
| 50 | 3 | 10 | 680 | 3 | 10/10 | 680 |
| 51 | 4 | 0 | 120 | 4 | 0/0 | 120 |
| 52 | 4 | 1 | 176 | 4 | 2/1 | 176 |
| 53 | 4 | 2 | 208 | 4 | 1/2 | 208 |
| 54 | 4 | 3 | 256 | 4 | 3/3 | 256 |
| 55 | 4 | 4 | 328 | 4 | 4/4 | 328 |
| 56 | 4 | 5 | 424 | 4 | 5/5 | 424 |
| 57 | 4 | 6 | 504 | 4 | 6/6 | 504 |
| 58 | 4 | 7 | 584 | 4 | 7/7 | 584 |
| 59 | 4 | 8 | 680 | 4 | 8/8 | 680 |
| 60 | 4 | 8 | 680 | 4 | 9/9 | 776 |
| 61 | 4 | 8 | 680 | 4 | 10/10 | 872 |
| 62 | 5 | 0 | 152 | 5 | 0/0 | 152 |
| 63 | 5 | 1 | 208 | 5 | 2/1 | 208 |
| 64 | 5 | 2 | 256 | 5 | 1/2 | 256 |
| 65 | 5 | 3 | 328 | 5 | 3/3 | 328 |
| 66 | 5 | 4 | 408 | 5 | 4/4 | 408 |
| 67 | 5 | 5 | 504 | 5 | 5/5 | 504 |
| 68 | 5 | 6 | 600 | 5 | 6/6 | 600 |
| 69 | 5 | 7 | 680 | 5 | 7/7 | 712 |
| 70 | 5 | 7 | 680 | 5 | 8/8 | 808 |
| 71 | 5 | 7 | 680 | 5 | 9/9 | 936 |
| 72 | 5 | 7 | 680 | 5 | 10/10 | 1000 |
| 73 | 6 | 0 | 208 | 6 | 0/0 | 208 |
| 74 | 6 | 1 | 256 | 6 | 2/1 | 256 |
| 75 | 6 | 2 | 328 | 6 | 1/2 | 328 |
| 76 | 6 | 3 | 440 | 6 | 3/3 | 440 |
| 77 | 6 | 4 | 552 | 6 | 4/4 | 552 |
| 78 | 6 | 5 | 680 | 6 | 5/5 | 680 |
| 79 | 6 | 5 | 680 | 6 | 6/6 | 808 |
| 80 | 6 | 5 | 680 | 6 | 7/7 | 1000 |
| 81 | 7 | 0 | 256 | 7 | 0/0 | 256 |
| 82 | 7 | 1 | 344 | 7 | 2/1 | 344 |
| 83 | 7 | 2 | 424 | 7 | 1/2 | 424 |
| 84 | 7 | 3 | 568 | 7 | 3/3 | 568 |
| 85 | 7 | 4 | 680 | 7 | 4/4 | 680 |
| 86 | 7 | 4 | 680 | 7 | 5/5 | 872 |
| 87 | 7 | 4 | 680 | 7 | 6/6 | 1000 |

Table 22.3.1.6.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1 | Void. | - | - | - | - |
| - | EXCEPTION: Steps 1 to 8 are repeated as per table 22.3.1.6.3.2-2 with values of ISF, DL IMCS, IRU and UL IMCS | - | - | - | - |
| 2 | SS looks up for TBSUL in table 22.3.1.6.3.2-1 as per the execution counter | - | - | - | - |
| - | EXCEPTION: Steps 3 to 6 are performed for TBSUL>56 | - | - | - | - |
| 3 | SS creates one UL RLC SDU of size TBSUL, embeds it in an ESM DATA TRANSPORT and DLInformationTransfer-NB and splits the resulting DL RLC SDU into three RLC AMD PDUs ( if TBSDL < 100 bits ) or two equal sized RLC AMD PDUs ( if TBSDL > 100 bits ) (NOTE 1) | - | - | - | - |
| 3A | SS gets current timing and NPDCCH period N is the next NPDCCH period which can be used by the SS for a DL transmission | - | - | - | - |
| 4 | At NPDCCH period N+1 the SS sends a DCI format N1 and a MAC PDU containing an RLC AMD PDU with polling field ‘P’ set to '0' and containing the 1st AMD PDU of the RLC SDU created at step 3. | <-- | DCI N1 format with ISF, DL IMCS MAC PDU with 1st AMD PDU of the DL RLC SDU | - | - |
| 4A | IF TBSDL < 100 bits At NPDCCH period N+2 the SS sends a DCI format N1 and a MAC PDU containing an RLC AMD PDU with polling field ‘P’ set to '0' and containing the 2nd AMD PDU of the RLC SDU created at step 3. | <-- | DCI N1 format with ISF, DL IMCS MAC PDU with 2nd AMD PDU of the DL RLC SDU | - | - |
| 4B | At NPDCCH period N+i the SS sends a DCI format N1 and a MAC PDU containing an RLC AMD PDU with polling field ‘P’ set to '1' and containing the last AMD PDU of the RLC SDU created at step 3. Where i=3 if TBSDL < 100 bits, otherwise i=2. (NOTE 2) | <-- | DCI N1 format with ISF, DL IMCS MAC PDU with last AMD PDU of the DL RLC SDU | - | - |
| 5 | In the NPDCCH period N+3 after the transmission at step 4B the SS allocates an uplink grant for the UL TBS according to 22.3.1.6.3.2-2 | <-- | (UL Grant) DCI N0 format with IRU and UL IMCS | - | - |
| 6 | CHECK: Does UE return an RLC SDU with the same content as created and embedded in the ESM DATA TRANSPORT at step 3 using the Resource unit Assignment and modulation and coding scheme as configured by the SS in step 5? | --> | MAC PDU containing RLC STATUS PDU and RLC AMD PDU with UL RLC SDU | 1,2 | P |
| 7 | The SS transmits an RLC STATUS PDU to acknowledge correctly received data | <-- | RLC STATUS PDU | - | - |
| 8 | Void. |  |  | - | - |
| NOTE 1: DL RLC SDU with size n contains a DLInformationTransfer-NB with ESM_DATA_TRANSPORT: 18 or 26 bits DLInformationTransfer-NB (PER encode: ‘0000000000’B + 8 bits ( length of DedicatedInfoNAS <= 127 bytes ) or 16 bits length ( 128 bytes <= length of DedicatedInfoNAS < 16383 bytes )) 1/2 byte Protocol discriminator ( SECURITY PROTECTED NAS MESSAGE ) 1/2 byte Security header type ( SECURITY PROTECTED NAS MESSAGE ) 4 bytes Message authentication code ( SECURITY PROTECTED NAS MESSAGE ) 1 byte Sequence number ( SECURITY PROTECTED NAS MESSAGE ) 1/2 byte Protocol discriminator ( ESM_DATA_TRANSPORT ) 1/2 byte EPS bearer identity ( ESM_DATA_TRANSPORT ) 1 byte Procedure transaction identity ( ESM_DATA_TRANSPORT ) 1 byte ESM data transport message identity ( ESM_DATA_TRANSPORT ) 2 bytes length indicator of the user data container ( ESM_DATA_TRANSPORT ) N bytes User data container contents ( UL RLC SDU ) 6 bits Aligned bits ( PER encode ) n = N+14 or N+15 bytes DL RLC SDU to achieve the maximum TBS in UL (1000 bits) the DL RLC SDU needs to be segmented NOTE 2: setting of the poll bit causes the UE to insert an RLC Status PDU in the UL message of step 6 |  |  |  |  |  |

22.3.1.6.3.3	Specific message contents

None.

< end of modified section >