3GPP TSG-RAN5 Meeting #75	R5-172495

# Hangzhou, China, 15th May 2017 - 19th May 2017

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 4040 | rev | - | Current version: | 14.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to TC 22.5.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TDIA, CATT, TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOT-UEConTest |  |  |  |  |  |  | Date: |  | 2017-05-02 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The PIXIT px_SinglePLMN_Tested in LTE, specifically for test cases which only used 2/3 cells on one PLMN and a single cell on another PLMN. This was to allow testing on band that single frequency operation only. This PIXIT has no relationship with NB-IoT, since NB-IoT dosen’t have single frequency operation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the PIXIT px_SinglePLMN_Tested and relevant test steps. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may unfairly fail the test case due to incorrect implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of first modified section>

22.5.4	NB-IoT / Attach to new PLMN GUTI reallocation / Network reject with Extended Wait Timer / Paging with IMSI / Attach Rejected Illegal ME/UE / Detach upon switch-off

22.5.4.1	Test Purpose (TP)

<Unchanged skipped>

22.5.6.3	Test description

22.5.6.3.1	Pre-test conditions

System Simulator:

-	Maximum 2 Ncells are on at any given time

-	Ncell 50, Ncell 51 and Ncell 55 are defined in subclause 8.1.4.2 in TS 36.508[18]

-	NB-IOT system information combination 2 as defined in TS 36.508[18] clause 8.1.4.3.1.1 is used in all cells

-	Ncell 50 and Ncell 51  belong to PLMN1.

UE:

-	The Test UICC shall be inserted. This shall contain a USIM application on UICC.

-	the UE is previously registered on  NB-IOT , and when on NB-IOT, the UE is last authenticated and registered on Ncell 50 using default message contents according to TS 36.508 [18].

Preamble:

-	the UE is in state Switched OFF (State 1-NB) according to TS 36.508 [18].

- 	The NAS integrity algorithm shall be set to a different value than 'EPS integrity algorithm' EIA0 throughout the whole duration of the test.

22.5.6.3.2	Test procedure sequence

Table 22.5.6.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| - | The SS configures: - Ncell 50 as a "serving cell" - Ncell 51 as a "non-suitable cell". | - | - | - | - |
| 1 | Switch-on the UE. | - | - | - | - |
| 2 | The UE transmits an ATTACH REQUEST message. (Attach attempt counter = 1) | --> | ATTACH REQUEST | - | - |
| 3 | The SS waits 265 seconds (255 seconds T3410 and 10 seconds T3411). | - | - | - | - |
| 3a | Check: Does the UE transmit an RRCConnectionRequest-NB message? | - | - | 1 | P |
| 4 | Check: Does the UE transmit an ATTACH REQUEST message? (Attach attempt counter = 2) | --> | ATTACH REQUEST | 2 | P |
| 5 | The SS releases the RRC connection. | - | - | - | - |
| 6 | The SS waits 10 seconds (T3411). | - | - | - | - |
| 7 | Check: Does the UE transmit an ATTACH REQUEST message? (Attach attempt counter = 3) | --> | ATTACH REQUEST | 3 | P |
| 8 | The SS transmits an ATTACH REJECT message, EMM cause = Network failure (#17) | - |  | - | - |
| 9 | The SS releases the RRC connection. | - | - | - | - |
| 10 | The SS waits 10 seconds (T3411). | - | - | - | - |
| 11 | Check: Does the UE transmit an ATTACH REQUEST message? (Attach attempt counter = 4) | --> | ATTACH REQUEST | 4 | P |
| 12 | The SS transmits an ATTACH REJECT message, EMM cause = Network failure (#22) | <-- | ATTACH REJECT | - | - |
| 13 | The SS releases the RRC connection. | - | - | - | - |
| 14 | The SS waits 10 seconds (T3411). | - | - | - | - |
| 15 | Check: Does the UE transmit an ATTACH REQUEST message? (Attach attempt counter = 5) | --> | ATTACH REQUEST | 5 | P |
| 16 | The SS transmits an ATTACH REJECT message, EMM cause = Congestion (#22) | <-- | ATTACH REJECT | - | - |
| 17 | The SS releases the RRC connection. | - | - | - | - |
| 18 | The SS waits 12 minutes (default value of T3402). | - | - | - | - |
| 19 | Check: Does the UE transmit an ATTACH REQUEST message? | --> | ATTACH REQUEST | 6 | P |
| 20-29b1 | The attach procedure is completed by executing steps 5 to 14b1 of the UE registration procedure in TS 36.508 sub clause 8.1.5.2. | - | - | - | - |
| 30 | Switch-off the UE. | - | - | - | - |
| - | The UE transmits a DETACH REQUEST message with the Detach type IE indicating “switch off”. | --> | DETACH REQUEST | - | - |
| - | The SS responds the DETACH ACCEPT message. | <-- | DETACH ACCEPT | - | - |
| - | The SS configures: - Ncell 51 as a "serving cell" - Ncell 50 as a "non-suitable cell". |  |  |  |  |
| 31 | Switch-on the UE. |  |  |  |  |
| - | The following messages are to be observed on Ncell 51 unless explicitly stated otherwise. | - | - | - | - |
| 32 | The UE transmits an ATTACH REQUEST message in Ncell 51 including a PDN CONNECTIVITY REQUEST or ESM DUMMY MESSAGE message. | --> | ATTACH REQUEST | - | - |
| 33 | SS does not send ATTACH ACCEPT to the UE and update TAC value in SystemInformationBlockType1-NB. | - |  | - | - |
| 34 | The SS transmits a Paging message paging occasion including a systemInfoModification-r13.From the beginning of the next modification period the SS transmits a modified SystemInformationBlockType1-NB as specified. | <-- | Paging -NB | - | - |
| 35 | Check: Does the UE transmit an ATTACH REQUEST message in Ncell 51 including a PDN CONNECTIVITY REQUEST or ESM DUMMY MESSAGE message in the next 12 seconds? Note: Wait time is more than 2.1* modification period for the UE to receive system information and inferior to T3410. | --> | ATTACH REQUEST | 7 | P |
| 36-41a2 | Steps 5-10a2 of the generic procedure for UE registration specified in TS 36.508 subclause 8.1.5.2 are performed. | - | - | - | - |
| 42 | The SS configures: - Ncell 50 as a “serving cell” | - | - | - | - |
| 43 | SS is configured to not allocate any UL grant or respond to any NPRACH preambles for ATTACH COMPLETE in Ncell 51. | - | - | - | - |
| 44 | SS responds with ATTACH ACCEPT message including a valid TAI list. The ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST or ESM DUMMY MESSAGE message is piggybacked in ATTACH ACCEPT message. GUTI-1 is allocated. | <-- | ATTACH ACCEPT | - | - |
| 45 | The SS configures: - Ncell 51 as a "non-suitable cell". | - | - | - | - |
| 46 | UE transmits RRCConnectionRequest-NB message in Ncell 50 | - | - | - | - |
| 47 | SS Sends RRCConnectionSetup-NB | - | - | - | - |
| 48 | The UE transmits an RRCConnectionSetupComplete-NB message to confirm the successful completion of the connection establishment in Ncell 50 | - | - | - | - |
| - | The following messages are to be observed on Ncell 50 unless explicitly stated otherwise. | - | - | - | - |
| 49 | Check: Does the UE transmit an ATTACH REQUEST message, containing GUTI 1? | --> | ATTACH REQUEST | 8 | P |
| 50 | SS responds with ATTACH ACCEPT message including a valid TAI list. The ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST or ESM DUMMY MESSAGE message. | <-- | ATTACH ACCEPT | - | - |
| 51 | The UE transmits an ATTACH COMPLETE message including an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT or ESM DUMMY MESSAGE message . | --> | ATTACH COMPLETE | - | - |
| 52 | Switch-off the UE. | - | - | - | - |
| - | The UE transmits a DETACH REQUEST message with the Detach type IE indicating “switch off”. | --> | DETACH REQUEST | - | - |
| - | The SS responds the DETACH ACCEPT message. | <-- | DETACH ACCEPT | - | - |
| - | The SS configures: -Ncell 50 as the "Serving cell". -Ncell 51 as a " Suitable neighbour cell". -If present, Ncell 55 as a "Non-Suitable Off cell". | - | - | - | - |
| - | The following messages are to be observed on Ncell 50 unless explicitly stated otherwise. | - | - | - | - |
| 53 | Switch-on the UE. | - | - | - | - |
| 54 | The UE transmits an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST or ESM DUMMY MESSAGE message. | --> | ATTACH REQUEST | - | - |
| 55 | The SS transmits an ATTACH REJECT message with EMM cause = "EPS services not allowed". | <-- | ATTACH REJECT | - | - |
| 56 | The SS releases the RRC connection. | - | - | - | - |
| 57 | The SS reconfigures: Ncell 50 as a " Suitable neighbour cell". Ncell 51 as the "Serving cell". Note: Ncell 50 and Ncell 51 are in different TAIs – same PLMN(PLMN1). | - | - | - | - |
| - | The following messages are to be observed on Ncell 51 unless explicitly stated otherwise. | - | - | - | - |
| 58 | Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds? | --> | ATTACH REQUEST | 9 | F |
| 59 | The user initiates an attach by MMI or by AT command. | - | - | - | - |
| 60 | Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds? | --> | ATTACH REQUEST | 9 | F |
| 61 | Switch-off the UE. | - | - | - | - |
| - | The SS reconfigures: - Ncell 51 as a " Non-Suitable Off cell ". - Ncell 55 as a "Non-Suitable Off cell". - Ncell 50 as the "Serving cell". | - | - | - | - |
| 62 | Switch-on the UE. | - | - | - | - |
| 63 | The UE transmits an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message or ESM DUMMY MESSAGE. Note: The ATTACH REQUEST message shall be sent as a security protected NAS message (see TS 24.301 – clause 9.1). | --> | ATTACH REQUEST | - | - |
| 64a1-65a7 | Steps 5a1-12 of the generic procedure for UE registration specified in TS 36.508 subclause 8.1.5.2 are performed. (Control_Plane_CIoT_Optimisation are used) | - | - | - | - |
| 66 | The SS transmits an ATTACH ACCEPT although UE has not successfully completed any NAS security mode control procedure. Note: The ATTACH ACCEPT message is sent as a plain NAS message (see TS 24.301 – clause 9.1). | <-- | ATTACH ACCEPT | - | - |
| 67 | Check: Does the UE transmit an ATTACH COMPLETE message within the next 3s? Note: The UE is expected to discard the ATTACH ACCEPT message without security protection. | --> | ATTACH COMPLETE | 10 | F |
| 68 | The SS transmits an AUTHENTICATION REQUEST message to initiate the EPS authentication and AKA procedure. | <-- | AUTHENTICATION REQUEST |  |  |
| 69 | The UE transmits an AUTHENTICATION RESPONSE message to establish mutual authentication. | --> | AUTHENTICATION RESPONSE |  |  |
| 70 | The SS transmits a SECURITY MODE COMMAND message to activate NAS security. | <-- | SECURITY MODE COMMAND |  |  |
| 71 | The UE transmits a SECURITY MODE COMPLETE message and establishes the initial security configuration. | --> | SECURITY MODE COMPLETE |  |  |
| 72 | The SS transmits an ATTACH ACCEPT without integrity protection. Note: The ATTACH ACCEPT message is sent as a plain NAS message (see TS 24.301 – clause 9.1). | <-- | ATTACH ACCEPT | - | - |
| 73 | Check: Does the UE transmit an ATTACH COMPLETE message within the next 3s? Note: The UE is expected to discard the ATTACH ACCEPT message without security protection | --> | ATTACH COMPLETE | 11 | F |
| 74 | The SS transmits an ATTACH ACCEPT with integrity protection with the Message authentication code set to an incorrect value. Note: The ATTACH ACCEPT message is sent as a security protected NAS message (see TS 24.301 – clause 9.1). | <-- | ATTACH ACCEPT | - | - |
| 75 | Check: Does the UE transmit an ATTACH COMPLETE message within the next 3s? Note: The UE is expected to discard the ATTACH ACCEPT message because the integrity check is failed. | --> | ATTACH COMPLETE | 12 | F |
| 76 | The SS transmits an ATTACH ACCEPT message including an ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST or ESM DUMMY MESSAGE. Note: The ATTACH ACCEPT message is sent as a security protected NAS message (see TS 24.301 – clause 9.1). Note 1: SS allocates a PDN address of a PDN type which is compliant with the PDN type requested by the UE. | <-- | ATTACH ACCEPT | - | - |
| 77 | Check: Does the UE transmit an ATTACH COMPLETE message including an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT or ESM DUMMY MESSAGE message as specified? Note: The ATTACH COMPLETE message is sent as a security protected NAS message (see TS 24.301 – clause 9.1). | --> | ATTACH COMPLETE | 13 | P |
| 78 | Check: Does the test results of test procedure in 36.508 clause 8.1.5A.2 indicate that the UE is in E-UTRA EMM-REGISTERED state with S-TMSI-2? Note: This step verifies that the UE has correctly stored the GUTI-4 which was included in the protected ATTACH ACCEPT messages. | - | - | 13 | - |
| 79 | The SS configures: -Ncell 50 as the "Serving cell". | - | - | - | - |
| 80 | IF pc_USIM_Removal THEN cause removal of USIM from the UE without powering down . | - | - | - | - |
| 80a | ELSE switch-off the UE. | - | - | - | - |
| 81 | Check: Does the UE transmit a DETACH REQUEST with the Detach Type IE indicating "normal detach" or “power off” detach and "IMSI detach" or "EPS detach" depending on the UE configuration? | --> | DETACH REQUEST | 14 | P |
| - | EXCEPTION: Step 81a describe behaviour that depends on the UE implementation | - | - | - | - |
| 81a | If in step 81 normal detach is performed SS responds with DETACH ACCEPT message | <-- | DETACH ACCEPT | - | - |
| 82 | Check: Does the test result of generic procedure in TS 36.508 subclause 8.1.5A.2 indicate that the UE does not respond to paging when paged with S-TMSI included in GUTI-1 and with CN domain indicator set to "PS"? | - | - | 14 | - |
| 83 | IF pc_USIM_Removal THEN insert a valid USIM and switch-on the UE. | - | - | - | - |
| 84 | ELSE switch-on the UE. | - | - | - | - |
| 85 | The UE transmits an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST or ESM DUMMY MESSAGE message (NOTE 2) | --> | ATTACH REQUEST | - | - |
| 86 | The SS initiates Detach procedure with the Detach Type IE "re-attach not required" | <-- | DETACH REQUEST | - | - |
| 87 | Check: Does the UE send DETACH ACCEPT message? | --> | DETACH ACCEPT | 15 | P |
| 88 | The SS releases the RRC connection. | - | - | - | - |
| 88a | The SS waits 5 seconds (NOTE 1) | - | - | - | - |
| 89 | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |
| 89a | The UE is brought back to operation or the USIM is inserted. | - | - | - | - |
| 90 | The UE transmits an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message. (NOTE 2) | --> | ATTACH REQUEST | - | - |
| 91-96a2 | Steps 5-10a2 of the generic procedure for UE registration specified in TS 36.508 subclause 8.1.5.2 are performed. | - | - | - | - |
| 97 | The SS initiates Detach procedure with the Detach Type IE "re-attach required" | <-- | DETACH REQUEST | - | - |
| - | EXCEPTION: In parallel with steps 98 to 99, the parallel behaviour defined in table 22.5.6.3.2-2 is running. | - | - | - | - |
| 98 | SS responds with ATTACH ACCEPT message including a valid TAI list. The ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST or ESM DUMMY MESSAGE message is piggybacked in ATTACH ACCEPT message. GUTI-1 is allocated. | <-- | ATTACH ACCEPT | - | - |
| 99 | The UE transmits an ATTACH COMPLETE message including an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT or ESM DUMMY MESSAGE message? | --> | ATTACH COMPLETE | 16 | P |
| NOTE 1: The time delay is added to additionally guarantee UE has respected the content of the DETACH REQUEST message. NOTE 2: Any type of attach is acceptable. |  |  |  |  |  |

<End of modified section>