3GPP TSG-RAN5 Meeting #75	R5-172498

# Hangzhou, China, 15th May 2017 - 19th May 2017

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 4042 | rev | - | Current version: | 14.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to TC 22.5.7a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TDIA, CATT, TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOT-UEConTest |  |  |  |  |  |  | Date: |  | 2017-05-02 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - The PIXIT px_SinglePLMN_Tested in LTE, specifically for test cases which only used 2/3 cells on one PLMN and a single cell on another PLMN. This was to allow testing on band that single frequency operation only. This PIXIT has no relationship with NB-IoT, since NB-IoT dosen’t have single frequency operation. - Ncell 62 is never used in this testcase, should be remove from pre-test conditions. - The UE has just done a successful attach on Ncell 51 at steps 70 - 79b1, so it will not be able to perform another attach at steps 82 - 91b1. These steps should be removed since have no TP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove the PIXIT px_SinglePLMN_Tested reference. - Remove Ncell62 from pre-test conditions. - Remove step 81 – 91b1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may unfairly fail the test case due to incorrect implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.5.7a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of first modified section>

22.5.7a	NB-IoT / Normal tracking area update List of equivalent PLMNs in the TRACKING AREA UPDATE ACCEPT message / Normal tracking area update Rejected (IMSI invalid / Illegal ME / UE identity cannot be derived by the network / UE implicitly detached / PLMN not allowed

22.5.7a.1	Test Purpose (TP)

<Unchanged skipped>

22.5.7a.3	Test description

22.5.7a.3.1	Pre-test conditions

System Simulator:

-	Maximum 2 Ncells are on at any given time

-	Ncell 55, Ncell 56 and belong to PLMN2 and Ncell 63 belongs to PLMN3

-	Ncell 50,  Ncell 51, Ncell 55, Ncell 56 and Ncell 63 are defined in subclause 8.1.4.2 in TS 36.508[18]

-	NB-IOT system information combination 2 as defined in TS 36.508[18] clause 8.1.4.3.1.1 is used in all cells

UE:

-	The Test UICC shall be inserted. This shall contain a USIM application on UICC.

Preamble:

- 	the UE is in state Registered, Idle mode (State 3-NB) on Ncell 50 according to TS 36.508 [18] with PLMN3 on its "forbidden PLMN list".

22.5.7a.3.2	Test procedure sequence

22.5.7a.3.2	Test procedure sequence

Table 22.5.7a.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1 | Set the cell type of Ncell 51 to the ''Serving cell''. All the other cells are set to ''Non-Suitable off cell''. All TRACKING AREA UPDATE REJECT messages below shall be sent integrity protected. | - | - | - | - |
| 2 | The UE transmits a TRACKING AREA UPDATE REQUEST message on NB-IoT Ncell 51. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 3 | SS responds with a TRACKING AREA UPDATE ACCEPT message including PLMN2 and PLMN3 in the list of equivalent PLMNs. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 4 | The UE transmits a TRACKING AREA UPDATE COMPLETE message | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 5 | SS Releases the RRC Connection. | - | - | - | - |
| 6 | Set the cell type of Ncell 51 to the ''Non-Suitable off cell''. Set the cell type of Ncell 55 to the ''Suitable cell''. | - | - | - | - |
| 7 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message on Ncell 55 (PLMN2)? | --> | TRACKING AREA UPDATE REQUEST | 1 | P |
| 8 | The SS transmits a TRACKING AREA UPDATE ACCEPT message including PLMN1 and PLMN3 in the list of equivalent PLMNs. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 9 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 10 | SS Releases the RRC Connection. | - | - | - | - |
| 11 | Set the cell type of Ncell 55 to the ''Non-Suitable off cell''. Set the cell type of Ncell 63 to the '' Serving cell''. Note: Ncell 63 (PLMN3) belongs to the forbidden PLMN. | - | - | - | - |
| 12 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message on NB-IoT Ncell 63 (PLMN3) in next 30 seconds? | --> | TRACKING AREA UPDATE REQUEST | 1 | F |
| 13 | The UE is switched to manual PLMN selection mode and is made to select PLMN3 in order to remove PLMN3 in the forbidden PLMN list in the UE. | - | - | - | - |
| 14 | The UE transmits a TRACKING AREA UPDATE REQUEST message on NB-IoT Ncell 63 (PLMN3). | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 15 | The SS transmits a TRACKING AREA UPDATE ACCEPT message without the list of equivalent PLMNs. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 16 | Check: Does the UE transmit a TRACKING AREA UPDATE COMPLETE message? | --> | TRACKING AREA UPDATE COMPLETE | 1 | P |
| 17 | SS releases the RRC connection | - | - | - | - |
| 18 | The UE is switched back to automatic PLMN selection mode. | - | - | - | - |
| 19 | SS waits for 5 minutes to allow UE to complete automatic PLMN search |  |  |  |  |
| 20 | Set the cell type of Ncell 51 to ”Serving cell”. Set the cell type of Ncell 55 to ”Suitable neighbour intra-frequency cell”. Set the cell type of Ncell 63 to the ''Non-Suitable off cell''. | - | - | - | - |
| 21 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message on Ncell 51 (PLMN1)? | --> | TRACKING AREA UPDATE REQUEST | 2 | P |
| 22 | SS responds with TRACKING AREA UPDATE ACCEPT message | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 23 | The UE transmits a TRACKING AREA UPDATE COMPLETE message | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| - | EXCEPTION: Steps 24 to 47 shall be repeated for (k=0,1) |  |  |  |  |
| 24 | Set the cell type of Ncell 50 to the ''Serving cell''. Set the cell type of Ncell 51 to the ''Non-Suitable cell''. Set the cell type of Ncell 55 to the ''Non-Suitable off cell''. | - | - | - | - |
| 25 | The UE transmits a TRACKING AREA UPDATE REQUEST on Ncell 50. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 26 | The SS transmits a TRACKING AREA UPDATE REJECT message: - if k=0 with EMM cause = "Illegal UE" - if k=1 with EMM cause = "Illegal ME" | <-- | TRACKING AREA UPDATE REJECT | - | - |
| 27 | The SS releases the RRC connection. | - | - | - | - |
| 28 | Set the cell type of Ncell 50 to the ''Non-Suitable cell''. Set the cell type of Ncell 51 to ''Serving cell''. | - | - | - | - |
| 29 | Check: Does the UE transmit an ATTACH REQUEST message in the next 90 seconds on Ncell 51? Note: NB-IoT Ncell 51 belongs to the same PLMN where the UE was rejected but a different TAC | --> | ATTACH REQUEST | 3 | F |
| 30 | The user initiates an attach by MMI or by AT command. | - | - | - | - |
| 31 | Check: Does the UE transmit an ATTACH REQUEST message in the next 90 seconds on Ncell 51? | --> | ATTACH REQUEST | 3 | F |
| 32a1 | Set the cell type of Ncell 51 to the '' Non-Suitable cell''. Set the cell type of Ncell 55 to the ''Serving cell''. | - | - | - | - |
| 32a2 | Check: Does the UE transmit an ATTACH REQUEST message in the next 90 seconds on Ncell 55? Note: Ncell 55 belongs to a PLMN which is not the same like the one on which the UE was rejected. | --> | ATTACH REQUEST | 3 | F |
| 32a3 | The user initiates an attach by MMI or by AT command. | - | - | - | - |
| 32a4 | Check: Does the UE transmit an ATTACH REQUEST message in the next 90 seconds on Ncell 55? | --> | ATTACH REQUEST | 3 | F |
| 33 | Switch-off the UE | - | - | - | - |
| 34 | Set the cell type of Ncell 51 to the ''Serving cell''. Set the cell type of Ncell 50 to the ''Non-Suitable off cell''. Set the cell type of Ncell 55 to the ''Non-Suitable off cell''. | - | - | - | - |
| 35 | Switch-on the UE | - | - | - | - |
| 36 | Check: Does the UE transmit an ATTACH REQUEST message with a PDN CONNECTIVITY REQUEST message to request PDN connectivity to the default PDN or ESM DUMMY MESSAGE on Ncell 51? | --> | ATTACH REQUEST | 4 | P |
| 37-46b1 | The procedure is completed by executing steps 5 to 14b1 of the UE registration procedure in TS 36.508 subclause 8.1.5.2. |  |  |  |  |
| 47 | SS Releases the RRC Connection, and the UE is in state Registered, NB-IoT Idle Mode (N1) on Ncell 51 | - | - | - | - |
| 48 | Set the cell type of Ncell 51 to the ''Non-Suitable cell''. Set the cell type of Ncell 50 to the ''Serving cell''. | - | - | - | - |
| 49 | The UE transmits a TRACKING AREA UPDATE REQUEST on Ncell 50. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 50 | The SS transmits a TRACKING AREA UPDATE REJECT message with EMM cause = '' UE identity cannot be derived by the network '' as specified. | <-- | TRACKING AREA UPDATE REJECT | - | - |
| - | EXCEPTION: Steps 51-52a1 describes the behaviour that depends on UE behaviour (Note 1). | - | - | - | - |
| 51 | The SS releases the RRC connection. | - | - | - | - |
| - | EXCEPTION: Step 52a1 describes a behaviour which depends on the UE capability | - | - | - | - |
| 52a1 | IF NOT pc_Automatic_EPS_Re_Attach, the user initiates an attach by MMI or by AT command. | - | - | - | - |
| 53 | Check: Does the UE transmit an ATTACH REQUEST message with a PDN CONNECTIVITY REQUEST message to request PDN connectivity to the default PDN or ESM DUMMY MESSAGE message on Ncell 50? | --> | ATTACH REQUEST | 5 | P |
| 54-63b1 | The attach procedure is completed by executing steps 5 to 14b1 of the UE registration procedure in TS 36.508 subclause 8.1.5.2. |  |  |  |  |
| 64 | The SS releases the RRC connection. | - | - | - | - |
| 65 | Set the cell type of Ncell 51 to the ''Serving cell''. Set the cell type of Ncell 50 to the ''Non-Suitable cell''. | - | - | - | - |
| 66 | The UE transmits a TRACKING AREA UPDATE REQUEST on Ncell 51. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 67 | The SS transmits a TRACKING AREA UPDATE REJECT message with EMM cause = ''UE implicitly detached'' as specified. | <-- | TRACKING AREA UPDATE REJECT | - | - |
| - | EXCEPTION: Steps 68a-68a1 describes the behaviour that depends on UE behaviour (Note 1). | - | - | - | - |
| 68a | The SS releases the RRC connection. | - | - | - | - |
| - | EXCEPTION: Step 68a1 describes a behaviour which depends on the UE capability | - | - | - | - |
| 68a1 | IF NOT pc_Automatic_EPS_Re_Attach THEN the user initiates an attach by MMI or by AT command. | - | - | - | - |
| 69 | Check: Does the UE transmit an ATTACH REQUEST message with a PDN CONNECTIVITY REQUEST message to request PDN connectivity to the default PDN or ESM DUMMY MESSAGE message on Ncell 51? | --> | ATTACH REQUEST | 6 | P |
| 70-79b1 | The attach procedure is completed by executing steps 5 to 14b1 of the UE registration procedure in TS 36.508 sub clause 8.1.5.2. |  |  |  |  |
| 80 | The SS releases the RRC connection. | - | - | - | - |
| 81 | Void. |  |  |  |  |
| 82-91b1 | Void. |  |  |  |  |
| 92 | Set the cell type of Ncell 55 to the ''Non-Suitable cell''. Set the cell type of Ncell 56 to the ''Serving cell''. Note: Ncell 55 and Ncell 56 are in the same PLMN. |  |  |  |  |
| 93 | The UE transmits a TRACKING AREA UPDATE REQUEST on Ncell 56. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 94 | The SS transmits a TRACKING AREA UPDATE REJECT message with EMM cause = ''PLMN not allowed''. | <-- | TRACKING AREA UPDATE REJECT | - | - |
| 95 | The SS releases the RRC connection. | - | - | - | - |
| 96 | Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds on Ncell 56? | --> | ATTACH REQUEST | 7 | F |
| 97 | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |
| 98 | Set the cell type of Ncell 55 to the ''Serving cell''. Set the cell type of Ncell 56 to the ''Non-Suitable off cell''. Note: Ncell 55 and Ncell 56 are in the same PLMN. | - | - | - | - |
| 99 | The UE is brought back to operation or the USIM is inserted. | - | - | - | - |
| 100 | Check: Does the UE transmit an ATTACH REQUEST message in the next 90 seconds on Ncell 55? | --> | ATTACH REQUEST | 8 | F |
| 101 | The user initiates an attach by MMI or by AT command. | - | - | - | - |
| 102 | Check: Does the UE transmit an ATTACH REQUEST message in the next 90 seconds on Ncell 55? | --> | ATTACH REQUEST | 8 | F |
| 103 | The following messages are sent and shall be received on Ncell 63. | - | - | - | - |
| 104 | Set the cell type of Ncell 63 to the ''Serving cell'', Set the cell type of Ncell 55 to the ''Non-Suitable “off” cell''. | - | - | - | - |
| 105 | Check: Does the UE transmit ATTACH REQUEST message with a PDN CONNECTIVITY REQUEST message to request PDN connectivity to the default PDN or ESM DUMMY MESSAGE message on Ncell 63? | --> | ATTACH REQUEST | 9 | P |
| 106-115b1 | The attach procedure is completed by executing steps 5 to 14b1 of the UE registration procedure in TS 36.508 sub clause 8.1.5.2. | - | - | - | - |
| 116 | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |
| - | EXCEPTION: Step 117a1 describes behaviour that depends on the UE capability. |  |  |  |  |
| 117a1 | If pc_SwitchOnOff or pc_USIM_Removal then the UE transmits a DETACH REQUEST. | --> | DETACH REQUEST | - | - |
| 118 | The following messages are sent and shall be received on NB-IoT Ncell 56. | - | - | - | - |
| 119 | Set the cell type of Ncell 56 to the '' Serving cell''. Set the cell type of Ncell 63 to the ' Non-Suitable off cell''. Note: Ncell 56 belongs to the forbidden PLMN. | - | - | - | - |
| 120 | The UE is brought back to operation or the USIM is inserted. | - | - | - | - |
| 121 | The UE is switched to manual PLMN selection mode and is made to select the forbidden PLMN (PLMN2). | - | - | - | - |
| 122 | Check: Does the UE transmit ATTACH REQUEST message with a PDN CONNECTIVITY REQUEST message to request PDN connectivity to the default PDN or ESM DUMMY MESSAGE message? | --> | ATTACH REQUEST | 10 | P |
| 123-132b1 | The attach procedure is completed by executing steps 5 to 14b1 of the UE registration procedure in TS 36.508 sub clause 8.1.5.2. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state NB-IoT manual selection Mode (N5) according to TS 36.523-3. | - | - | - | - |
| 133 | The SS releases the RRC connection. | - | - | - | - |
| NOTE 1: SS waits for 1.5 second to receive the Attach Request on the existing RRC Connection. In case Attach Request is not received within 1.5 second then the existing RRC Connection is released. |  |  |  |  |  |

<End of modified section>