3GPP TSG-RAN5 Meeting #75	R5-172499

# Hangzhou, China, 15th May 2017 - 19th May 2017

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 4043 | rev | - | Current version: | 14.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Remove Multi-PLMN test for TC 22.5.7b |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TDIA, CATT, TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOT-UEConTest |  |  |  |  |  |  | Date: |  | 2017-05-02 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - The PIXIT px_SinglePLMN_Tested in LTE, specifically for test cases which only used 2/3 cells on one PLMN and a single cell on another PLMN. This was to allow testing on band that single frequency operation only. This PIXIT has no relationship with NB-IoT, since NB-IoT dosen’t have single frequency operation. - Ncell 62 is never used in this testcase, should be remove from pre-test conditions. - The statement in the Preamble is required to add PLMN3 to the forbidden list, however this isn't referred to in the expected sequence and no other cells are on PLMN3. - The IF and ELSE statements in step 68 is useless and confusing, should be removed. - Step 83 should be replaced with a paging message and then check the UE doesn't respond for an amount of time, as has been done in other NAS test cases. - The type of detach in the specific message contents at step 75 currently states EPS only or combined EPS/IMSI detach, but combined attach is not relevant for NB-IoT. - The parameter active flag and EPS bearer context status in specific message contents for step 78 is not required in this test. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove the PIXIT px_SinglePLMN_Tested and relevant conditions. - Remove Ncell62 from pre-test conditions. - Remove the requirments for adding PLMN3 to the forbidden list in the Preamble. - Remove the IF and ELSE statements in step 68. - Update step 83 to paging and check UE no response. - Remove combined type from the specific message contents at step 75. - Remove the parameter active flag and EPS bearer context status in specific message contents for step 78. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may unfairly fail the test case due to incorrect implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.5.7a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of first modified section>

22.5.7b	NB-IoT / Normal tracking area update Rejected ( Tracking area not allowed / No suitable cells in tracking area / Roaming not allowed in this tracking area / Congestion) / UE initiated detach Abnormal case Change of cell into a new tracking area

22.5.7b.1	Test Purpose (TP)

<Unchanged skipped>

22.5.7b.3	Test description

22.5.7b.3.1	Pre-test conditions

System Simulator:

-	Maximum 2 Ncells are on at any given time

-	Ncell 55, Ncell 56 belong to PLMN2

-	Ncell 50, Ncell 51, Ncell 52, Ncell 55, Ncell 56, Ncell 61are defined in subclause 8.1.4.2 in TS 36.508[18]

-	NB-IOT system information combination 2 as defined in TS 36.508[18] clause 8.1.4.3.1.1 is used in all cells

UE:

-	The Test UICC shall be inserted. This shall contain a USIM application on UICC.

Preamble:

- 	the UE is in state Registered, Idle mode (State 3-NB) on Ncell 51 according to TS 36.508 [18].

22.5.7b.3.2	Test procedure sequence

Table 22.5.7b.3.2-1: Main behaviour

| 1 | Set the cell type of Ncell 50 to the ''Serving cell''. Set the cell type of Ncell 51 to the ''Non-Suitable off cell''. Set the cell type of Ncell 52 to the ''Suitable neighbour intra-frequency cell''. | - | - | - | - |
| --- | --- | --- | --- | --- | --- |
| 2 | The UE transmits a TRACKING AREA UPDATE REQUEST on Ncell 50. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 3 | The SS transmits a TRACKING AREA UPDATE REJECT message with EMM cause = ''Tracking area not allowed'' as specified. | <-- | TRACKING AREA UPDATE REJECT | - | - |
| 4 | The SS releases the RRC connection. | - | - | - | - |
| 5 | Check: Does the UE transmit an ATTACH REQUEST message in the next 90 seconds on Ncell 52? | --> | ATTACH REQUEST | 2 | F |
| 6 | Set the cell type of Ncell 52 to the ''Non-Suitable off cell''. |  |  |  |  |
| 7 | The user initiates an attach by MMI or by AT command. | - | - | - | - |
| 8 | Check: Does the UE transmit an ATTACH REQUEST message in the next 90 seconds on Ncell 50? | --> | ATTACH REQUEST | 1 | F |
| 9 | Set the cell type of Ncell 50 to the '' Non-Suitable off cell ''. Set the cell type of Ncell 61 to the ''Serving cell''. | - | - | - | - |
| 10 | Check: Does the UE transmit an ATTACH REQUEST message in the next 90 seconds on Ncell 61? | --> | ATTACH REQUEST | 3 | F |
| 11 | Set the cell type of Ncell 52 to the ''Serving cell''. Set the cell type of Ncell 61 to the ''Non-Suitable off cell ''. | - | - | - | - |
| 12 | Check: Does the UE transmit an ATTACH REQUEST including a PDN CONNECTIVITY REQUEST or ESM DUMMY MESSAGE message on Ncell 52? | --> | ATTACH REQUEST | 4 | P |
| 13-24b1 | The attach procedure is completed by executing steps 5 to 14b1 of the UE registration procedure in TS 36.508 sub clause 8.1.5.2. | - | - | - | - |
| 25 | If possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |
| - | EXCEPTION: Step 26a1 describes behaviour that depends on the UE capability. |  |  |  |  |
| 26a1 | If pc_SwitchOnOff or pc_USIM_Removal then the UE transmits a DETACH REQUEST. | --> | DETACH REQUEST | - | - |
| 27 | The following messages are sent and shall be received on Ncell 50. | - | - | - | - |
| 28 | Set the cell type of Ncell 50 to the '' Serving cell''. Set the cell type of Ncell 52 to the '' Non-Suitable off cell''. | - | - | - | - |
| 29 | The UE is brought back to operation or the USIM is inserted. | - | - | - | - |
| 30 | Check: Does the UE transmits ATTACH REQUEST message with a PDN CONNECTIVITY REQUEST message to request PDN connectivity to the default PDN or ESM DUMMY MESSAGE message on Ncell 50? | --> | ATTACH REQUEST | 5 | P |
| 31-40b1 | The attach procedure is completed executing steps 5 to 14b1 of the UE registration procedure in TS 36.508 sub clause 8.1.5.2. | - | - | - | - |
| 41 | The SS releases the RRC connection. | - | - | - | - |
| 42 | Set the cell type of Ncell 50 to the ''Non-Suitable cell''. Set the cell type of Ncell 55 to the ''Serving cell''. | - | - | - | - |
| 43- 48 | The UE performs steps 1 to 6 of the generic procedure in TS 36.508 subclause 8.1.5A.5 to camp on Ncell 55. |  |  |  |  |
| 49 | Set the cell type of Ncell 55 to the '' Suitable neighbour intra-frequency cell ''. Set the cell type of Ncell 56 to the ''Serving cell''. | - | - | - | - |
| 50 | The UE transmits TRACKING AREA UPDATE REQUEST on Ncell 56. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 51 | Set the cell type of Ncell 55 to the ''Serving cell''. Set the cell type of Ncell 56 to the “Suitable neighbour intra-frequency cell''. | - | - | - | - |
| 52 | The SS transmits a TRACKING AREA UPDATE REJECT message with EMM cause = '"Roaming not allowed in this tracking area"' as specified. | <-- | TRACKING AREA UPDATE REJECT | - | - |
| 53 | The SS releases the RRC connection. |  |  |  |  |
| 54 | Check: Does the UE transmit TRACKING AREA UPDATE REQUEST message on Ncell 55? | --> | TRACKING AREA UPDATE REQUEST | 6, 7, 8 | P |
| 55 | The SS transmits TRACKING AREA UPDATE REJECT message with EMM cause = ''Roaming not allowed in this tracking area'' as specified. | <-- | TRACKING AREA UPDATE REJECT | - | - |
| 56 | The SS releases the RRC connection. |  |  |  |  |
| 57 a1 | Set the cell type of Ncell 55 to the ''Non-Suitable “off” cell''. Set the cell type of Ncell 56 to the ''Non-Suitable “off” cell''. Set the cell type of Ncell 50 to the ''Serving cell''. | - | - | - | - |
| 57a2 | Check: Does the UE transmit TRACKING AREA REQUEST message on Ncell 50? | --> | TRACKING AREA UPDATE REQUEST | 6, 9 | P |
| 57a3 | The SS sends TRACKING AREA ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 57a4 | The UE transmits TRACKING AREA COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 57a5 | The SS releases the RRC connection. | - | - | - | - |
| 58 | Set the cell type of Ncell 50 to the "non-Suitable cell". Set the cell type of Ncell 51 to the "Serving cell". If (px_SinglePLMN_Tested = Multi PLMN), set the cell type of Ncell 55 to the "Serving cell". | - | - | - | - |
| - | The following messages are sent and shall be received on NCell 51. | - | - | - | - |
| 59 | The UE transmits a TRACKING AREA UPDATE REQUEST message. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 60 | The SS transmits a TRACKING AREA UPDATE REJECT message with the EMM cause set to 'No Suitable Cells In tracking area'. | <-- | TRACKING AREA UPDATE REJECT | - | - |
| 61 | Set the cell type of Ncell 50 to the " Serving cell ". Set the cell type of Ncell 51 to the "Suitable neighbour Intra Frequency cell". |  |  |  |  |
| 62 | The SS releases the RRC connection. | - | - | - | - |
| - | The following messages are sent and shall be received on Ncell 50. | - | - | - | - |
| 63 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message? | --> | TRACKING AREA UPDATE REQUEST | 10 | P |
| 64 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 65 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 66 | Set the cell type of Ncell 50 to the ''Non-Suitable cell''. Set the cell type of Ncell 51 to the ''Serving cell''. If present, set the cell type of NB-IoT Ncell 55 to the ''Non-Suitable cell'' | - | - | - | - |
| 67 | The UE transmits a TRACKING AREA UPDATE REQUEST on Ncell 51. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 68 | The SS transmits a TRACKING AREA UPDATE REJECT message with EMM cause = ''Congestion '' and T3346 IE set to 5 minutes. The UE starts timer T3346. | <-- | TRACKING AREA UPDATE REJECT | - | - |
| 69 | The SS releases the RRC connection. | - | - | - | - |
| 70 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message before timer T3346 has expired? | - | - | 11 | F |
| 71 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message after timer T3346 has expired? | --> | TRACKING AREA UPDATE REQUEST | 11 | P |
| 72 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 73 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| - | The following messages are to be observed on Ncell 51 unless explicitly stated otherwise. | - | - | - | - |
| 74 | Cause UE to initiate detach. | - | - | - | - |
| 75 | The UE transmits a DETACH REQUEST message. (The UE starts timer T3421) | --> | DETACH REQUEST | - | - |
| 76 | The SS does not respond to the DETACH REQUEST message. | - | - | - | - |
| 77 | Set the cell type of Ncell 50 to the ''Serving cell'. Set the cell type of Ncell 51 to the ''Non-Suitable neighbour cell''. | - | - | - | - |
| 78 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message? | --> | TRACKING AREA UPDATE REQUEST | 12 | P |
| 79 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 80 | Check: Does the UE transmit a TRACKING AREA UPDATE COMPLETE message? | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 81 | The UE transmits a DETACH REQUEST message. | --> | DETACH REQUEST | 13 | P |
| 82 | SS responds with DETACH ACCEPT message | <-- | DETACH ACCEPT | - | - |
| 83a | SS sends a Paging message to the UE on the appropriate paging block, and including the UE identity in one entry of the IE pagingRecordLists. | <-- | RRC: Paging-NB | - | - |
| 83b | Check: Does the UE respond to paging by transmitting an RRCConnectionRequest-NB message in the next 30 seconds? | --> | -RRC: RRCConnectionRequest-NB | 13 | F |

Table 22.5.7b.3.3-10: DETACH REQUEST (step 75, Table 22.5.7b.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Detach type |  |  |  |
| Type of detach | 001 | EPS detach |  |
| Switch off | 0 | normal detach |  |
| GUTI or IMSI | GUTI-2 |  |  |

Table 22.5.7b.3.3-12: Message TRACKING AREA UPDATE REQUEST (step 78, Table 22.5.7b.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update type | "TA updating" |  |  |

<End of modified section>