3GPP TSG-RAN WG5#75	R5-172801

# Hangzhou, China, 15 - 19 May 2017

Agenda Item:	6.5.6.5

Source:	Anritsu Ltd, Qualcomm Incorporated

Title:	Disable of IMS over UTRAN in LTE Inter-RAT test cases

Document for:	Discussion and Decision

## 1.	Introduction

In RAN WG5 Meeting #69, using R5-155993, one of topic ‘IMS disabled over UTRAN” discussed to address real network deployment scenario.

Conclusion shown below is noted regarding this topic:

“No recommendation can be proposed for IMS NOT supported in WCDMA /TDSCDMA. More discussion feedback required. “

Hence, would like to discuss further on this topic on how to handle various possible scenarios for IMS disabled over UTRAN.

## 2.	Discussion

Following scenarios are possible for a UE which does not support IMS upon mobility to UTRAN:

- UE does not send IMS deregistration

- UE sends IMS deregistration

- UE sends PDP context deactivation for IMS PDN

RAN5 provided regression results for a) but not for b) or c). RAN5 have addressed above issues related to UTRAN->LTE(U2L) test cases. But, in case of LTE->UTRAN(L2U), L2U2L,U2L2U, above issues needs to be addressed.

As per 24.008, section 4.7.13.5,

If the network indicates "no follow-on proceed" in the ATTACH ACCEPT or ROUTING AREA UPDATE ACCEPT message, the MS shall not initiate the pending Service request procedure until the current PS signalling connection is released.

As per 36.508, Routing Area Accept sends default as “no follow-on-proceed”. This results UE not triggering RAU procedure until RRC connection released. In case of “follow-on-proceed”, UE triggers Service request before RRC connection release.

“no follow-on-proceed” will have preference over “follow-on-proceed” as follows setting in 36.508. Hence, signalling after RRC connection release can be considered as preference.

## 3.	Proposal

It is proposed that RAN5 endorses the following:

- Handle signaling procedure for b) or c) after RRC connection release of Routing Area Update procedure

- Handle signaling procedure for b) or c) before RRC connection release of Routing Area Update procedure

## 4.	Conclusion and Way forward

It is recommended to proposal 1) is way forward