3GPP TSG-RAN5 Meeting #75	R5-173013

# Hangzhou, China, 15th May 2017 - 19th May 2017

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 4097 | rev | 1 | Current version: | 14.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to PC2 Test Case 6.1.2.5a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | QUALCOMM Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, LTE_B41_UE_PC2-UEConTest |  |  |  |  |  | Date: |  |  | 2017-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Incorrect reference table number used from 36.508 for SystemInformationBlockType1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modified the table number from 4.4.3.3-2 to 4.4.3.2-3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect table number will be used |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.5a.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | R5-172897 is Rev1 of R5-172167 due to WIC mismatch in 3GU and minor cover page issue R5-173013 is final version R5-172897- Fixed spec version on cover page |  |  |  |  |  |  |  |  |

<Start of changed section>

#### 6.1.2.5a	Cell reselection for interband operation/ Power Class 2 UE operation/ Between FDD and TDD

6.1.2.5a.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state on a FDD band}

ensure that {

when { UE detects the power class 2 cell re-selection criteria is met for the cell on a TDD band }

then { UE reselects to the new cell }

}

6.1.2.5a.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304, clause 5.2.3.2, 5.2.4.1, 5.2.4.2 and 5.2.4.5.

[TS 36.304, clause 5.2.3.2]

The cell selection criterion S in normal coverage is fulfilled when:

Srxlev > 0  AND  Squal > 0

where:

Srxlev = Qrxlevmeas – (Qrxlevmin + Qrxlevminoffset) – Pcompensation - Qoffsettemp

Squal = Qqualmeas – (Qqualmin + Qqualminoffset) - Qoffsettemp

where:

| Srxlev | Cell selection RX level value (dB) |
| --- | --- |
| Squal | Cell selection quality value (dB) |
| Qoffsettemp | Offset temporarily applied to a cell as specified in [3] (dB) |
| Qrxlevmeas | Measured cell RX level value (RSRP) |
| Qqualmeas | Measured cell quality value (RSRQ) |
| Qrxlevmin | Minimum required RX level in the cell (dBm) |
| Qqualmin | Minimum required quality level in the cell (dB) |
| Qrxlevminoffset | Offset to the signalled Qrxlevmin taken into account in the Srxlev evaluation as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN [5] |
| Qqualminoffset | Offset to the signalled Qqualmin taken into account in the Squal evaluation as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN [5] |
| Pcompensation | If the UE supports the additionalPmax in the NS-PmaxList, if present, in SIB1, SIB3 and SIB5: max(PEMAX1 –PPowerClass, 0) – (min(PEMAX2, PPowerClass) – min(PEMAX1, PPowerClass)) (dB); else: max(PEMAX1 –PPowerClass, 0) (dB); |
| PEMAX1, PEMAX2 | Maximum TX power level an UE may use when transmitting on the uplink in the cell (dBm) defined as PEMAX in [33]. PEMAX1 and PEMAX2 are obtained from the p-Max and the NS-PmaxList respectively in SIB1, SIB3 and SIB5 as specified in TS 36.331 [3]. |
| PPowerClass | Maximum RF output power of the UE (dBm) according to the UE power class as defined in [33] |

[TS 36.304, clause 5.2.4.1]

Absolute priorities of different E-UTRAN frequencies or inter-RAT frequencies may be provided to the UE in the system information, in the RRCConnectionRelease message, or by inheriting from another RAT at inter-RAT cell (re)selection. In the case of system information, an E-UTRAN frequency or inter-RAT frequency may be listed without providing a priority (i.e. the field cellReselectionPriority is absent for that frequency). If priorities are provided in dedicated signalling, the UE shall ignore all the priorities provided in system information. If UE is in camped on any cell state, UE shall only apply the priorities provided by system information from current cell, and the UE preserves priorities provided by dedicated signalling unless specified otherwise. When the UE in camped normally state, has only dedicated priorities other than for the current frequency, the UE shall consider the current frequency to be the lowest priority frequency (i.e. lower than the eight network configured values). While the UE is camped on a suitable CSG cell, the UE shall always consider the current frequency to be the highest priority frequency (i.e. higher than the eight network configured values), irrespective of any other priority value allocated to this frequency. The UE shall delete priorities provided by dedicated signalling when:

-	the UE enters RRC_CONNECTED state; or

-	the optional validity time of dedicated priorities (T320) expires; or

-	a PLMN selection is performed on request by NAS [5].

NOTE:	Equal priorities between RATs are not supported.

The UE shall only perform cell reselection evaluation for E-UTRAN frequencies and inter-RAT frequencies that are given in system information and for which the UE has a priority provided.

The UE shall not consider any black listed cells as candidate for cell reselection.

The UE shall inherit the priorities provided by dedicated signalling and the remaining validity time (i.e., T320 in E-UTRA, T322 in UTRA and T3230 in GERAN), if configured, at inter-RAT cell (re)selection.

NOTE:	The network may assign dedicated cell reselection priorities for frequencies not configured by system information.

[TS 36.304, clause 5.2.4.2]

When evaluating for reselection purposes cell selection criterion or SnonServingCell,x, the UE shall use parameters provided by the serving cell.

Following rules are used by the UE to limit needed measurements:

-	If Sintrasearch is sent in the serving cell and SServingCell > Sintrasearch, UE may choose to not perform intra-frequency measurements.

-	If SServingCell <= Sintrasearch, or Sintrasearch is not sent in the serving cell UE shall perform intra-frequency measurements.

-	The UE shall apply the following rules for E-UTRAN inter-frequencies and inter-RAT frequencies which are indicated in system information and for which the UE has priority provided as defined in 5.2.4.1:

-	For an E-UTRAN inter-frequency or inter-RAT frequency with a reselection priority higher than the reselection priority of the current E-UTRA frequency the UE shall perform measurements of higher priority E-UTRAN inter-frequency or inter-RAT frequencies according to [10].

-	For an E-UTRAN inter-frequency with an equal or lower reselection priority than the reselection priority of the current E-UTRA frequency and for inter-RAT frequency with lower reselection priority than the reselection priority of the current E-UTRAN frequency:

-	If Snonintrasearch is sent in the serving cell and SServingCell > Snonintrasearch UE may choose not to perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority.

-	If SServingCell <= Snonintrasearchor Snonintrasearch is not sent in the serving cell  the UE shall perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority according to [10].

Where SServingCell is the Srxlev-value of the serving cell.

[TS 36.304, clause 5.2.4.5]

Criteria 1: the SnonServingCell,x of a cell on evaluated frequency is greater than Threshx, high during a time interval TreselectionRAT;

Cell reselection to a cell on a higher priority E-UTRAN frequency or inter-RAT frequency than serving frequency shall be performed if:

-	A cell of a higher priority E-UTRAN frequency or inter-RAT frequency fulfils criteria 1; and

-	more than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a cell on an equal priority E-UTRAN frequency shall be based on ranking for Intra-frequency cell reselection as defined in sub-clause 5.2.4.6.

Cell reselection to a cell on a lower priority E-UTRAN frequency or inter-RAT frequency than serving frequency shall be performed if:

-	No cell on a higher priority E-UTRAN frequency or inter-RAT frequency than serving frequency fulfils the criteria 1; and

-	No cell on serving frequency or on an equal priority E-UTRAN frequency fulfils the criteria in 5.2.4.6; and

-	SServingCell < Threshserving, low and the SnonServingCell,x of a cell of a lower priority E-UTRAN frequency or inter-RAT frequency is greater than Threshx, low during a time interval TreselectionRAT; and

-	more than 1 second has elapsed since the UE camped on the current serving cell.

For GERAN, UTRAN, and E-UTRAN, SnonServingCell,x is the Srxlev-value of an evaluated cell. For UTRAN FDD, Squal is higher than 0, as defined in [8]. For cdma2000 RATs, SnonServingCell,x is equal to FLOOR(-2 x 10 x log10 Ec/Io) in units of 0.5 dB, as defined in [18], with Ec/Io referring to the value measured from the evaluated cell.

For cdma2000 RATs, Threshx, high and Threshx, low are equal to -1 times the values signalled for the corresponding parameters in the system information.

In all the above criteria the value of TreselectionRAT is scaled when the UE is in the medium or high mobility state as defined in subclause 5.2.4.3.1. If more than one cell meets the above criteria, the UE shall reselect a cell ranked as the best cell among the cells meeting the criteria on the highest priority frequencies or the highest priority RAT if the highest priority RAT is E-UTRA according to criteria defined in 5.2.4.6.

6.1.2.5a.3	Test description

6.1.2.5a.3.1	Pre-test conditions

System Simulator:

-	Cell 1 is set to LTE-FDD mode and Cell 10 is set to LTE-TDD mode.

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) on Cell 1 (serving cell) according to [18].

6.1.2.5a.3.2	Test procedure sequence

Table 6.1.2.5a.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The configuration marked "T1" is applied at the point indicated in the Main behaviour description in Table 6.1.2.5a.3.2-2.

Table 6.1.2.5a.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 10 | Remark |
| --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | -71 | Cell 10 becomes stronger than Threshx, high. |

Table 6.1.2.5a.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell 10 power level according to the row "T1" in table 6.1.2.5a.3.2-1. | - | - | - | - |
| 2 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 10? | - | - | 1 | - |
| Note: Pcompensation is used while determining Srxlev of Cell 10. Pcomp for PC2 UE = max(Pmax – Ppowerclass, 0) i.e. max(26dBm – 26dBm, 0) which comes to 0. Pcomp for PC3 UE = max(Pmax – Ppowerclass, 0) i.e. max(26dBm – 23dBm, 0) which comes to 3. Srxlev of Cell 10 using Pcomp of PC2 is 35dB and using Pcomp of PC3 is 32dB. Threshx, high is set to 34dB in SIB5 of Cell 1, such that Srxlev of Cell 10 using Pcomp for PC2 UE is greater than that and Srxlev of Cell 10 using Pcomp of PC3 is lesser. i.e. Srxlev with PC3 < Threshx, high < Srxlev with PC2. This is to ensure that only PC2 UE will be able to do reselection. |  |  |  |  |  |

6.1.2.5a.3.3	Specific message contents

Table 6.1.2.5a.3.3-0: Conditions for specific message contents
in Tables 6.1.2.5a.3.3-1, 6.1.2.5a.3.3-2, 6.1.2.5a.3.3-3 and 6.1.2.5a.3.3-4

| Condition | Explanation |
| --- | --- |
| Cell 1 | This condition applies to system information transmitted on Cell 1. |
| Cell 10 | This condition applies to system information transmitted on Cell 10. |
| Band > 64 | If band > 64 is selected |

Table 6.1.2.5a.3.3-1: SystemInformationBlockType1 for Cell 1 and Cell 10 (preamble and all steps, Table 6.1.2.5a.3.2-2)

| Derivation path: 36.508 table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| p-Max | 23dBm | Default | Cell 1 |
|  | 26dBm |  | Cell 10 |
| } |  |  |  |

Table 6.1.2.5a.3.3-2: SystemInformationBlockType3 for Cell 1 and Cell 10 (preamble and all steps, Table 6.1.2.5a.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| cellReselectionPriority | 1 |  | Cell 1 |
|  | 5 |  | Cell 10 |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.5a.3.3-3: SystemInformationBlockType5 for Cell 1 (preamble and all steps, Table 6.1.2.5a.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 10 |  |  |
| dl-CarrierFreq | maxEARFCN |  | Band > 64 |
| p-max | 26dBm |  |  |
| t-ReselectionEUTRA | 7 |  |  |
| threshX-High | 17 | 34dB |  |
| cellReselectionPriority | 5 |  |  |
| } |  |  |  |
| SystemInformationBlockType5-v8h0-IEs SEQUENCE { |  |  | Band > 64 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| InterFreqCarrierFreqInfo-v9e0 ::= SEQUENCE { |  |  |  |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 10 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.5a.3.3-4: SystemInformationBlockType5 for Cell 10 (preamble and all steps, Table 6.1.2.5a.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 1 |  |  |
| dl-CarrierFreq | maxEARFCN |  | Band > 64 |
| t-ReselectionEUTRA | 7 |  |  |
| threshX-High | 17 | 34dB |  |
| cellReselectionPriority | 1 |  |  |
| } |  |  |  |
| SystemInformationBlockType5-v8h0-IEs SEQUENCE { |  |  | Band > 64 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| InterFreqCarrierFreqInfo-v9e0 ::= SEQUENCE { |  |  |  |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<End of changed section>