3GPP TSG-RAN5 Meeting #76	R5-174111

# Berlin, Germany, 21st August 2017 – 25th August 2017

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-3 | CR | xxxx | rev | - | Current version: | 14.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New TC: 7.1.20 HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under enhanced coverage |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom, Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOT-UEConTest ? |  |  |  |  |  | Date: |  |  | 2017-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Adding new NBIoT 7.1.20 HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under enhanced coverage |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new NBIoT RRM TC 7.1.20 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This test case will be missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (New) 7.1.20 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 36.521-2 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

7.1.20	HD-FDD Transmit Timing Accuracy Tests for Category NB1 UE in Standalone Mode under Enhanced Coverage

Editor’s notes: This test case is incomplete. The following items are missing or incomplete:

- Test tolerances and test requirement values

7.1.20.1	Test purpose

To verify that the Category NB1 UE under enhanced coverage is capable of following the frame timing change of the connected eNode B, that the UE initial transmit timing accuracy is within the specified limits and that the UE shall not adjust the uplink transmission timing autonomously during an ongoing repetition period other than at initial transmission.

7.1.20.2	Test applicability

This test applies to all types of E-UTRAN HD-FDD UE release 13 and forward of UE Category NB1.

7.1.20.3	Minimum conformance requirements

The UE initial transmission timing error shall be less than or equal to Te where the timing error limit value Te is defined in TS 36.133 [4] clause 7.20 and shown in table in Table 7.1.20.3-1. This requirement applies when it is the first transmission in a DRX cycle or the first transmission in a repetition period (R>1) for NPUSCH and NPRACH, the first transmission after an uplink transmission gap in a repetition period (R>1) for NPUSCH and NPRACH. The reference point for the UE initial transmit timing control requirement shall be the downlink timing of the serving NB-IoT cell minus . The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame is received from the serving NB-IoT cell. NTA_Ref for NPRACH is defined as 0.  (in Ts units) for other channels is the difference between UE transmission timing and the Downlink timing immediately after when the last timing advance in TS 36.133 clause 7.22 was applied. NTA_Ref for other channels is not changed until next timing advance is received.

Table 7.1.20.3-1: Te Timing Error Limit

| Downlink Bandwidth (MHz) | Te_ |
| --- | --- |
| 0.18 | 80*TS |
| Note 1: TS is the basic timing unit defined in TS 36.211 |  |

When it is not the first transmission in a DRX cycle or there is no DRX cycle, and when it is the transmission for NPUSCH the UE shall, when no repetitions are configured on the uplink or the repetition period is R=1, be capable of changing the transmission timing according to the received downlink frame of the serving NB-IoT cell except when the timing advance in TS 36.133 clause 7.22 is applied such that the UE transmission timing error shall be less than or equal to ±Te, where the timing error limit value Te is specified in Table 7.1.20.3-1.

When no repetition period is configured, or the configured repetition period is R=1, all adjustments made to the UE uplink timing shall follow these rules:

1)	The maximum amount of the magnitude of the timing change in one adjustment shall be 58.33*TS seconds.

2)	The minimum aggregate adjustment rate shall be 7*TS per second.

3)	The maximum aggregate adjustment rate shall be 58.33*TS per 200ms.

When a repetition period is configured on the uplink for which R>1, the UE shall not adjust the uplink transmission timing autonomously during an ongoing repetition period other than at initial transmission as defined above.

The normative reference for this requirement is TS 36.133 [4] clause 7.20 and A.7.1.18.

7.1.20.4	Test description

7.1.20.4.1	Initial conditions

Test Environment: Normal, as defined in TS 36.508 [7] clause 8.1.1.

Frequencies to be tested: According to Annex E table E-4 and TS 36.508 [7] clauses 8.1.3 and 8.1.4.2.

Channel Bandwidth to be tested: As specified in Table 7.1.20.5-1.

1.	Connect the SS (node B emulator) and AWGN noise sources to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.51 without faders.

2.	The general test parameter settings are set according to Table 7.1.20.4.1-1.

3.	Propagation conditions are set according to Annex B clause B.0.

4.	There is one cell specified in the test. Ncell 1 is the cell used for connection setup with the power level set according to Annex C.0 and C.1 for this test.

Table 7.1.20.4.1-1: General Test Parameters for UE Transmit Timing Accuracy Tests for HD-FDD Category NB1 UE in Standalone mode under enhanced coverage

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |
| NB-IoT Operation mode |  | Standalone | Standalone |
| DRX |  | OFF | ON |
| NPRACH configuration |  | As specified in A.10.3 |  |
| NPDCCH repetition level |  | 128 | 128 |
| NPUSCH repetition level |  | 32 | 32 |
| Note 1: DRX related parameters are defined in Table 7.1.20.5-3. |  |  |  |

7.1.20.4.2	Test procedure

The test consists of a single cell. The transmit timing accuracy is verified related to the downlink frame timing of Ncell 1. The downlink timing of Ncell 1 is changed and the changes in UE transmit timing are observed. The transmit timing is verified by the UE transmitting NPUSCH used as a measurement reference facilitating the SS timing estimation.

1.	Ensure the UE is in idle State 2A-NB with CP CIoT Optimisation according to TS 36.508 [7] clause 8.1.5.

2.	Set the parameters according to Tables 7.1.20.5-1. Propagation conditions are set according to Annex B clause B.1.1.

3.  SS sends uplink scheduling information via NPDCCH DCI format N0 for C_RNTI to schedule uplink subframes.

4.	The UE shall transmit NPUSCH and the SS shall measure the UE transmit timing offset (nTs) and verify that it is within the limits specified in Table 7.1.20.5-4 with respect to the first detected path (in time) of the corresponding downlink frame of Ncell 1.

5.	After 16ms from the initial transmission in Step 4, the test system adjusts the downlink transmit timing for the Ncell using the value of n measured in step 4.

-	if n < 0, by +(144 – |n|)TS compared to that in step 4.

-	if n ≥ 0, by -(144 – |n|)TS compared to that in step 4.

6.	Step 6 applies for Test 1, but is omitted for Test 2. The SS shall verify that for Test 1 the UE transmit timing autonomous adjustment shall not exceed ± 3×Ts during an ongoing repetition period.

7.	Step 7 applies for Test 2, but is omitted for Test 1. The SS sends NPDCCH including uplink grant for NPUSCH transmission and shall verify that the UE transmit timing offset stays within the limits specified in Table 7.1.20.5-4 with respect to the first detected path  (in time) of the corresponding downlink frame of NB-IoT Ncell 1. For test 2 the UE transmit timing offset shall be verified for the first transmission in the DRX cycle immediately after DL timing adjustment.

7.1.20.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 8.1.6 with the following exceptions:

Table 7.1.20.4.3-1: MAC-MainConfig-NB: Additional HD-FDD Category NB1 UE Transmit Timing Accuracy test requirement under Enhanced Coverage

| Derivation Path: TS 36.508 clause 8.1.8, Table 8.1.8.2.1.5-1: MAC-MainConfig-NB-SRB |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| MAC-MainConfig-NB-SRB ::= SEQUENCE { |  |  |  |  |
| drx-Config-r13 CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| onDurationTimer-r13 |  | pp1 |  |  |
| drx-InactivityTimer-r13 |  | pp0 |  |  |
| drx-RetransmissionTimer-r13 |  | pp0 |  |  |
| drx-Cycle-r13 |  | sf2048 |  |  |
| drx-StartOffset-r13 |  | 0 |  |  |
| drx-ULRetransmissionTimer-r13 |  | pp0 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.20.4.3-2: NPDCCH-ConfigDedicated-NB: Additional HD-FDD Category NB1 UE Transmit Timing Accuracy test requirement under Enhanced Coverage

| Derivation Path: TS 36.508 clause 8.1.6, Table 8.1.6.3-3: NPDCCH-ConfigDedicated-NB-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPDCCH-ConfigDedicated-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| npdcch-NumRepetitions-r13 | r128 | Set NPDCCH repetition to 128 |  |
| npdcch-StartSF-USS-r13 | v2 |  |  |
| } |  |  |  |

Table 7.1.20.4.3-3:NB-IoT Physical layer parameters for DCI format N0: Additional HD-FDD Category NB1 UE Transmit Timing Accuracy test requirement under Enhanced Coverage

| Derivation Path: TS 36.508 clause 8.1.3, Table 8.1.3.6.1.1-1: NB-IoT Physical layer parameters for DCI format N0 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Parameter | Value | Value in binary | Comment | Condition |
| Repetition number | 5 | “101” | Set NPUSCH repetition to 32 |  |

7.1.20.5	Test requirement

Tables 7.1.20.5-1, 7.1.20.5-2  define the primary settings of Ncell and Cell for UE transmit timing accuracy for HD-FDD Category NB1 UE transmit timing accuracy test.

Table 7.1.20.5-3 define the timing error limit with respect to the first detected path (in time) of the corresponding downlink frame of Ncell 1 include Test Tolerances.

Table 7.1.20.5-1: Ncell specific Test Parameters for UE Transmit Timing Accuracy Tests for HD-FDD Category NB1 UE in Standalone mode under enhanced coverage

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |
| RF Channel Number |  | 1 | 1 |
| BWchannel | kHz | 200 | 200 |
| NPDSCH parameter |  | R.19 HD-FDD | R.19 HD-FDD |
| NPDCCH parameter |  | R.33 HD-FDD | R.33 HD-FDD |
| NOCNG Patterns defined in A.3.2.3.3 |  | NOP.3 FDD | NOP.3 FDD |
| NPBCH_RA | dB | -3 | -3 |
| NPBCH_RB |  |  |  |
| NPSS_RA |  |  |  |
| NSSS_RA |  |  |  |
| NPDCCH_RA |  |  |  |
| NPDCCH_RB |  |  |  |
| NPDSCH_RA |  |  |  |
| NPDSCH_RB |  |  |  |
| NOCNG_RA Note1 |  |  |  |
| NOCNG_RB Note1 |  |  |  |
|  | dBm/15 kHz | -98+TT | -98+TT |
|  | dB | -12+TT | -12+TT |
|  | dB | -12+TT | -12+TT |
| Antenna Configuration |  | 2x1 | 2x1 |
| Propagation condition | - | AWGN | AWGN |
| Note 1 NOCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table 7.1.20.5-2: drx-Configuration to be used in UE Transmit Timing Accuracy Test 2 for E-UTRAN HD-FDD Category NB1 UE in Standalone mode under enhanced coverage

| Field | Value | Comment |
| --- | --- | --- |
|  | Test 2 |  |
| onDurationTimer | pp1 |  |
| drx-InactivityTimer | pp0 |  |
| drx-RetransmissionTimer | pp0 |  |
| longDRX-CycleStartOffset | sf2048 |  |
| shortDRX | disable |  |
| Note 1: For further information see clause 6.7.3 in TS 36.331. |  |  |

The UE transmit timing offset shall be within the requirements in Table 7.1.20.5-3. This requirement applies when it is the first transmission in a DRX cycle or the first transmission in a repetition period (R>1) for NPUSCH and NPRACH, the first transmission after an uplink transmission gap in a repetition period (R>1) for NPUSCH and NPRACH transmission.

The reference point for the UE initial transmit timing control test requirement shall be the downlink timing minus.

Table 7.1.20.5-3: Te Timing Error Limit

| Downlink Bandwidth (MHz) | Te_ |
| --- | --- |
| 0.18 | 83*TS |
| Note 1: TS is the basic timing unit defined in TS 36.211 |  |

When a repetition period is configured on the uplink for which R>1, the UE transmit timing autonomous adjustment shall not exceed ± 3×Ts during an ongoing repetition period other than at initial transmission as defined above.

< End of changes >