3GPP TSG-RAN5 Meeting #77	R5-176942

# Reno,Nevada, 27th Nov 2017  - 01st  Dec 2017

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 4363 | rev | - | Current version: | 14.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to NB-IoT PDCP RoHC test case 22.3.3.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOT-UEConTest |  |  |  |  |  |  | Date: |  | 2017-11-24 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Based on current PDCP Test Model(RoHC), SS required to perform RoHC functionality while PDCP configured in transparent model. This requirement adds complexity, but, TP can be achieved without it. As a result, RRC/NAS test model considered for this test case. - Test purpose is to check if UE sends PDCP SN with value 0 after RRC connection resume. Hence, Get/Set of PDCP SN would be sufficient to achieve TestPurspose. - D/C field set to ‘1’ when PDCP Data PDU are sent. In Table 22.3.3.5.2-1, only PDCP DATA PDU are exchanged between SS and UE. D/C field is always set ‘1’. Hence, D/C filed is not required to be controlled. - IP Packets are sent and received Hence, RRC Test model would be sufficieint as only Get/Set of PDCP SN is required to achieve TP while D/C field control is not required. |  |  |  |  |  |  |  |  |
| Summary of change: |  | PDCP Data PDU replaced with IP packets PDCP SN numbers checked at step 13 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Additional complexity on SS protocol stack implementation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.3.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of Change>

#### 22.3.3.5	NB-IoT / PDCP re-establishment / stored UE AS context is used and drb-ContinueROHC is configured

22.3.3.5.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state and upper layers indicate stored UE AS context }

ensure that {

when { UE is requested to resume a suspended RRC connection }

then { UE transmits next PDCP Data PDU with SN value 0 }

}

22.3.3.5.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 36.331 clause 5.3.3.4a and 3GPP TS 36.523 clause 5.2.1.2

[TS 36.331, clause 5.3.3.4a]

Reception of the RRCConnectionResume by the UE

1>	stop timer T300;

1>	restore the PDCP state and re-establish PDCP entities for SRB2 and all DRBs;

1>	if drb-ContinueROHC is included:

2>	indicate to lower layers that stored UE AS context is used and that drb-ContinueROHC is configured;

2>	continue the header compression protocol context for the DRBs configured with the header compression protocol;

1>	else:

2>	indicate to lower layers that stored UE AS context is used;

2>	reset the header compression protocol context for the DRBs configured with the header compression protocol;

1>	 discard the stored UE AS context and resumeIdentity;

1>	perform the radio resource configuration procedure in accordance with the received radioResourceConfigDedicated and as specified in 5.3.10;

1>	resume SRB2 and all DRBs;

…

[TS 36.323, clause 5.2.1.2]

When upper layers request a PDCP re-establishment, the UE shall:

-	reset the header compression protocol for uplink and start with an IR state in U-mode [9] [11] if the DRB is configured with the header compression protocol and drb-ContinueROHC is not configured [3];

-	set Next_PDCP_TX_SN, and TX_HFN to 0;

-	apply the ciphering algorithm and key provided by upper layers during the re-establishment procedure;

-	if connected as an RN, apply the integrity protection algorithm and key provided by upper layers (if configured) during the re-establishment procedure;

-	for each PDCP SDU already associated with a PDCP SN but for which a corresponding PDU has not previously been submitted to lower layers:

-	consider the PDCP SDUs as received from upper layer;

-	perform transmission of the PDCP SDUs in ascending order of the COUNT value associated to the PDCP SDU prior to the PDCP re-establishment, as specified in the subclause 5.1.1 without restarting the discardTimer.

22.3.3.5.3	Test description

22.3.3.5.3.1	Pre-test conditions

System Simulator:

-	Ncell 1 according to TS 36.508 [18] Table 8.1.4.2-1A.

UE:

None.

Preamble:

-	The UE is in State 2B-NB, Attach Connected Mode, UE Test Loopback Activated for UE test loop mode B on Ncell 1 (serving cell) according to TS 36.508 [18] clause 8.1.5.2A.

22.3.3.5.3.2	Test procedure sequence

Table 22.3.3.5.2-1: Main behaviour

| St | Procedure |  | Message Sequence |  |  | TP |  | Verdict |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | U - S |  | Message |  |  |  |  |
| 1 | The SS creates 3 IP Packets | - |  | - | - |  | - |  |
|  | EXCEPTION: Step 2 and 3 shall be repeated for k=0 to 1 (increment=1). | - |  | - | - |  | - |  |
| 2 | The SS transmits one IP packet to the UE on the DRB associated with the default EPS bearer context. | <- |  | IP Packet #k |  |  | - |  |
| 3 | The UE loops back the IP packet received in Step 2 on the DRB associated with the default EPS bearer context. | -> |  | IP Packet #k |  |  | - |  |
| 4 | SS transmits an RRCConnectionRelease-NB Message. (IE ReleaseCause-r13 with Value rrc-Suspend). | <- |  | RRCConnectionRelease-NB |  |  | - |  |
| 5 | Wait for 10 s for the UE to enter E-UTRA RRC_IDLE state on Ncell 1. | - |  |  |  |  | - |  |
| 6 | The SS transmits a Paging-NB message with a matching UE identity. | <- |  | Paging-NB |  |  | - |  |
| 7 | The UE transmits RRCConnectionResumeRequest-NB message. | -> |  | RRCConnectionResumeRequest-NB |  |  |  |  |
| 8 | The SS transmits RRCConnectionResume-NB message with drb-ContinueROHC-r13 present. | <- |  | RRCConnectionResume-NB |  |  |  |  |
| 9 | The UE transmits RRCConnectionResumeComplete-NB message to confirm the successful completion of an RRC connection resumption. | -> |  | RRCConnectionResumeComplete-NB |  |  |  |  |
| 10 | The SS transmits a SecurityModeCommand message to activate AS security. | <- |  | SecurityModeCommand |  |  |  |  |
| 11 | The UE transmits SecurityModeComplete message. | -> |  | SecurityModeComplete |  |  |  |  |
| 12 | The SS transmits one IP packet to the UE on the DRB associated with the default EPS bearer context. | <- |  | IP Packet |  |  |  |  |
| 13 | Check: Does the UE loop back the IP packet on the DRB associated with the default EPS bearer context with PDCP SN=0? | -> |  | IP Packet | 1 |  | P |  |

22.3.3.5.3.3	Specific message contents

Table 22.3.4.5.3.3-1: PDCP-Config-NB (step 1, Table 22.3.3.5.2-1)

| Derivation Path: 36.331 clause 6.7.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PDCP-Config-NB-r13 ::= SEQUENCE { |  |  |  |  |
| headerCompression-r13 CHOICE { |  |  |  |  |
| rohc SEQUENCE { |  |  |  |  |
| maxCID-r13 |  | 15 | Default 15 |  |
| profiles-r13 SEQUENCE { |  |  |  |  |
| profile0x0002 |  | TRUE |  |  |
| profile0x0003 |  | FALSE |  |  |
| profile0x0004 |  | FALSE |  |  |
| profile0x0006 |  | FALSE |  |  |
| profile0x0102 |  | FALSE |  |  |
| profile0x0103 |  | FALSE |  |  |
| profile0x0104 |  | FALSE |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 22.3.3.5.3.3-2: RRCConnectionRelease-NB message (step 4, Table 22.3.3.5.2-1)

| Derivation Path: 36.508 clause 8.1.6.1-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCConnectionRelease-NB ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| rrcConnectionRelease-r13 SEQUENCE { |  |  |  |  |
| releaseCause-r13 |  | rrc-suspend |  |  |
| resumeIdentity-r13 |  | BIT STRING (SIZE(40)) | Any value |  |
| extendedWaitTime-r13 |  | Not present |  |  |
| redirectedCarrierInfo |  | Not present |  |  |
| lateNonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 22.3.3.5.3.3-3: RRCConnectionResume-NB message (step 8, Table 22.3.3.5.2-1)

| Derivation Path: 36.508 table 8.1.6.1-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCConnectionRelease-NB ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| rrcConnectionResume-r13 SEQUENCE { |  |  |  |  |
| radioResourceConfigDedicated-r13 |  | RadioResourceConfigDedicated-NB-DRB(1) |  |  |
| nextHopChainingCount-r13 |  | 0 |  |  |
| drb-ContinueROHC-r13 |  | true |  |  |
| lateNonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

<End of Change>