3GPP TSG-RAN WG5 Meeting #78	R5-181165

# Athens, Greece, Feb 26th- March 2nd , 2018

# Source:	Qualcomm Incorporated

# Title:	5GS RRC Updates

# Document for:	Endorsement

Agenda Item:	6.3.18.19

## 1.	Introduction

The purpose of this paper is to provide an update on the 5GS RRC WP and new agreements in RAN2 which will have impact on RAN5 specifications.

## 2.	Discussion

### 2.1 User Plane Bearer Related Changes

One of the key changes in RAN2 final agreements was related to EN-DC DRB options. From previous agreements, as noted in [1], the new agreement simplified the User Plane bearer options.

Figure 4.2.2-1: Radio Protocol Architecture for MCG, MCG split, SCG and SCG split bearers in MR-DC with EPC (EN-DC)

Figure 1: Old Radio Protocol Architecture for MCG, SCG and split bearers from a UE perspective in MR-DC with EPC (EN-DC)

Figure 2: Latest Radio Protocol Architecture for MCG, SCG and split bearers from a UE perspective in MR-DC with EPC (EN-DC)

The previous and new agreement are compared in figure above. The key changes are –

- MCG Split and SCG Split bearers are replaced by a single “Split Bearer”.

- All bearers can either be terminated in MN or in SN, from a UE perspective, they are effectively same (besides the consideration of security keys).

- SCG and Split bearers will always use NR PDCP.

- MCG bearer can have either NR or E-UTRA PDCP.

Based on above, the combinations needed for RRC TC have also reduced significantly and after merging test cases, below is what we have considered in RRC WP –

| Test Spec | Test case | Title |
| --- | --- | --- |
| 38.523-1 | 8.2.2.2.1 | PSCell addition, modification and release / SCG DRB |
| 38.523-1 | 8.2.2.2.2 | PSCell addition, modification and release / Split DRB |
| 38.523-1 | 8.2.2.2.3 | Bearer Modification / MCG DRB |
| 38.523-1 | 8.2.2.2.4 | Bearer Modification / MCG DRB / SCG DRB Reconfiguration |
| 38.523-1 | 8.2.2.2.5 | Bearer Modification / MCG DRB / Split DRB Reconfiguration |
| 38.523-1 | 8.2.2.2.6 | Bearer Modification / SCG DRB / Split DRB Reconfiguration |
| 38.523-1 | 8.2.2.2.9 | Bearer Modification / Handling for bearer type change with security key change |

Summary of DRB coverage in above TC

- TC 8.2.2.2.1 – 6 covers all cases of addition, modification and release for MCG, SCG and Split Bearers.

- TC 8.2.2.2.9 covers all cases of handling of MN and SN terminated cases to ensure the proper handling of security key changes, as shown in the TS37.340 Table A-1 below.

- As per the table there are a total of 15 combinations, 9 combinations with security key change and 6 combinations without security key change.

- The combinations covering cases “without security key changes” are implicitly covered as part of TC 8.2.2.2.4 – 6 and the combinations covering cases “with security key changes” are covered as part of a single TC 8.2.2.2.9 which has 9 TP’s to cover each scenario.

Impact of DRB changes in other Protocol Area

- All references to MCG Split and SCG Split in current draft specs, WP and other CR’s should be removed and aligned with latest agreements and shall be referred as Split Bearer (MN or SN terminated, if applicable).

- Loopback Modes (TS 36.509, TS 38.509) should also reflect this change.

Table A-1: L2 handling for bearer type change with and without security key change

| Bearer type change from row to col | MCG |  | Split |  | SCG |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | no key change | with key change (KeNB <-> S-KgNB) | no key change | with key change (KeNB <-> S-KgNB) | no key change | with key change (KeNB <-> S-KgNB) |
| MCG | N/A | PDCP: Re-establish MCG RLC: Re-establish MCG MAC: See Note SCG RLC: No action SCG MAC: No action | PDCP: Reconfigure MCG RLC: No action MCG MAC: No action SCG RLC: Establish SCG MAC: Reconfigure | PDCP: Re-establish MCG RLC: Re-establish MCG MAC: See Note SCG RLC: Establish SCG MAC: Reconfigure | PDCP: Recovery MCG RLC: Re-est+release MCG MAC: Reconfigure SCG RLC: Establish SCG MAC: Reconfigure | PDCP: Re-establish MCG RLC: Re-est+release MCG MAC: Reconfigure SCG RLC: Establish SCG MAC: Reconfigure |
| Split | PDCP: Recovery MCG RLC: No action MCG MAC: No action SCG RLC: Release SCG MAC: Reconfigure | PDCP: Re-establish MCG RLC: Re-establish MCG MAC: See Note SCG RLC: Release SCG MAC: Reconfigure | N/A | PDCP: Re-establish MCG RLC: Re-establish MCG MAC: Reset SCG RLC: Re-establish SCG MAC: Reset | PDCP: Recovery MCG RLC: Re-est+release MCG MAC: Reconfigure SCG RLC: No action SCG MAC: No action | PDCP: Re-establish MCG RLC: Re-est+release MCG MAC: Reconfigure SCG RLC: Re-establish SCG MAC: See Note |
| SCG | PDCP: Recovery MCG RLC: Establish MCG MAC: Reconfigure SCG RLC: Release SCG MAC: Reconfigure | PDCP: Re-establish MCG RLC: Establish MCG MAC: Reconfigure SCG RLC: Release SCG MAC: Reconfigure | PDCP: Reconfigure MCG RLC: Establish MCG MAC: Reconfigure SCG RLC: No action SCG MAC: No action | PDCP: Re-establish MCG RLC: Establish MCG MAC: Reconfigure SCG RLC: Re-establish SCG MAC: See Note | N/A | PDCP: Re-establish MCG RLC: No action MCG MAC: No action SCG RLC: Re-establish SCG MAC: See note |

NOTE:	MAC behaviour depends on the solution selected by the network, e.g. MAC reset, change of LCID, etc.

### 2.2 ASN.1 Updates

Latest draft versions of TS 36.331 and TS 38.331 have ASN.1 available, however the definition of IEs and ASN.1 structure are undergoing considerable changes and still have several FFS. However, we can start working on defining the messages in TS38.508. For TS38.523-1 prose CR’s, pCR can indicate the spec version used for conformance requirement.

### 2.3 Measurements and SCG Failure

Below are 2 new updates from RAN2 Adhoc#

- There are few new changes in clauses 5.5.5.1 and 5.5.5.2 of TS 36.331, which were made in RAN2 Adhoc. Basically, the sections covers the determination and reporting of available NR measurement results including Beam results, Beam Quantities and number of beams. Currently, we haven’t considered designing TC to address multiple beams per cell, this needs to be further discussed and analyzed. We can also consider moving such conformance requirement to RRM or check if they are implicitly covered in RRM.

- Just for information, there is a new message, SCGFailureInformationNR, which got defined in previous RAN2 meeting. TS 36.331 captures this in a new clause 5.6.13a and the message is sent from UE to E-UTRA containing the NR failure type. 5GS RRC WP has already captured the failure related scenarios.

## 3.   Proposal

1. To consider above changes in RRC and align other WP and RAN5 specifications accordingly.

2. CR Authors to apply updated draft RAN2 specs and mention it accordingly in pCR.

## 4.	References

[1] R5-174497 MR-DC Impact on RAN Protocol Tests for 5G NR

[2] R5-176383 – 5GS RRC Scoping updates and challenges